3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230333

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3058 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC capability test case 8.1.5.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With current implementation for checking single mode UE, UE may fail if the UE is dual mode (FDD + TDD) but dl-SchedulingOffsetPDSCH-TypeA is supported in one mode only (e.g. FDD or TDD, but not both). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated to the logic of checking on single mode, so that it only applies to single mode |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC, 8.2.1.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function fl_Check_dl_SchedulingOffset_PDSCH_TypeA_XDD_Diff() |
| --- | --- |
| Reason for change | With current implementation for checking single mode UE, UE may fail if the UE is dual mode (FDD + TDD) but dl-SchedulingOffsetPDSCH-TypeA is only supported in one mode, e.g. FDD or TDD. |
| Summary of change | Updated to the logic of checking on single mode, so that it only applies to single mode |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change

function fl_Check_dl_SchedulingOffset_PDSCH_TypeA_XDD_Diff(UE_NR_Capability p_ReceivedCapabilityMsg) { ......... ......... // check indication of TDD IE if (ispresent(v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA.dl_SchedulingOffset_PDSCH_TypeA)) { v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD := true; } // Error cases if the feature is set if ((not f_NR_DualModeCheck()) and (not v_Phy_ParametersXDD_Diff_dl_SchedulingOffset_PDSCH_TypeA) or v_FDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD or v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD) { // fail cases on Single mode f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD UE-NR-CapabilityAddXDD IEs is inconsistent with pc_dl_SchedulingOffset_PDSCH_TypeA on Single Mode"); } else if(f_NR_DualModeCheck()) // case dual mode { if (not v_Phy_ParametersXDD_Diff_dl_SchedulingOffset_PDSCH_TypeA) { if (v_FDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD and v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD) // fail case: both FDD and TDD capabilities are reported and IE on HandoverInterF_Common is not { f_SetVerdict(fail, __FILE__, __LINE__, "reporting both FDD/TDD_Add_UE_NR_Capabilities IEs is inconsistent"); } if ((not v_FDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD) and (not v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD)) // inconclusive case: UE does not report any IE for dl_SchedulingOffset_PDSCH_TypeA but pc_dl_SchedulingOffset_PDSCH_TypeA is set { f_SetVerdict(inconc, __FILE__, __LINE__, "pc_dl_SchedulingOffset_PDSCH_TypeA is set but dl_SchedulingOffset_PDSCH_TypeA IEs are not reported by the UE"); } } else if (v_FDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD or v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD) // fail cases: if IE phy_Parameters.phy_ParametersXDD_Diff.dl_SchedulingOffset_PDSCH_TypeA is reported but FDD and/or TDD capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent"); } } } ......... ......... }

After Change

function fl_Check_dl_SchedulingOffset_PDSCH_TypeA_XDD_Diff(UE_NR_Capability p_ReceivedCapabilityMsg) { ......... ......... // check indication of TDD IE if (ispresent(v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA.dl_SchedulingOffset_PDSCH_TypeA)) { v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD := true; } // Error cases if the feature is set if ((not f_NR_DualModeCheck()) and ((not v_Phy_ParametersXDD_Diff_dl_SchedulingOffset_PDSCH_TypeA) or v_FDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD or v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD) ) { // fail cases on Single mode f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD UE-NR-CapabilityAddXDD IEs is inconsistent with pc_dl_SchedulingOffset_PDSCH_TypeA on Single Mode"); } else if(f_NR_DualModeCheck()) // case dual mode { if (not v_Phy_ParametersXDD_Diff_dl_SchedulingOffset_PDSCH_TypeA) { if (v_FDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD and v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD) // fail case: both FDD and TDD capabilities are reported and IE on HandoverInterF_Common is not { f_SetVerdict(fail, __FILE__, __LINE__, "reporting both FDD/TDD_Add_UE_NR_Capabilities IEs is inconsistent"); } if ((not v_FDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD) and (not v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD)) // inconclusive case: UE does not report any IE for dl_SchedulingOffset_PDSCH_TypeA but pc_dl_SchedulingOffset_PDSCH_TypeA is set { f_SetVerdict(inconc, __FILE__, __LINE__, "pc_dl_SchedulingOffset_PDSCH_TypeA is set but dl_SchedulingOffset_PDSCH_TypeA IEs are not reported by the UE"); } } else if (v_FDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD or v_TDD_Add_UE_NR_Cap_dl_SchedulingOffset_PDSCH_TypeA_XDD) // fail cases: if IE phy_Parameters.phy_ParametersXDD_Diff.dl_SchedulingOffset_PDSCH_TypeA is reported but FDD and/or TDD capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent"); } } } ......... ......... }