3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230341

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4726 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RRC testcase 8.5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-04-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to Table C.1-1 [36.331-1], if the UE does not support TDD, FGI 104 bit is irrelevant and shall be set to 0. But using v_FGIsFromPICS_r10_FDD[3] for validation of FGI 104 bit is incorrect and this bit should be checked only if the UE supports TDD. - Following implementation of prose CR R5-221221 and TTCN CR R5s220954, a conformant UE operating at Rel-15 or later fail the capability check as it may not include ce_Capabilities_r15 IE if pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH are not supported but may support other IEs under phyLayerParameters_v1530. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Used PICS parameter pc_eTDD in place of v_FGIsFromPICS_r10_FDD[3] for validation of FGI 104 bit. - Check for ce_Capabilities_r15 IE only if pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH are supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_EUTRA_CheckRel10_FGIs |
| --- | --- |
| Reason for change | According to Table C.1-1 [36.331-1], if the UE does not support TDD, FGI 104 bit is irrelevant and shall be set to 0. But using v_FGIsFromPICS_r10_FDD[3] for validation of FGI 104 bit is incorrect and this bit should be checked only if the UE supports TDD. |
| Summary of change | Used PICS parameter pc_eTDD in place of v_FGIsFromPICS_r10_FDD[3] for validation of FGI 104 bit. |
| TTCN module | LTE\8_5\RRC_OtherFunctions.ttcn |
| MCC160 Comment |  |

Before change:

function fl_EUTRA_CheckRel10_FGIs(UE_EUTRA_Capability p_ReceivedCapabilityMsg,

boolean p_Rel15 := false)

{

// Skipped code

if (p_Rel15) {

// check Setting of FGI 103 and FGI 104 against setting of UE categories

// IF (pc_ue_Category_8 or pc_ue_Category_11 or pc_ue_Category_12 or pc_ue_CategoryDL_11 or pc_ue_CategoryDL_12 or pc_ue_CategoryDL_14) THEN  FGI 103 resp 104 shall be set to '1' in one RAT at least

// It is assumed that the common value will always be one of the RAT specific values !

// E.g. common value of '1' and RAT specific values of '0' will never occur

// Otherwise FGI reporting and FGI PICS setting would both be consistently incorrect !

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10_FDD[2] != '1'B) and (v_FGIsFromPICS_r10_TDD[2] != '1'B)){ // @sic R5s200875, R5s210784 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 103 not set to '1' "); // @sic R5-196979 sic@

}

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10_FDD[3] != '1'B) and (v_FGIsFromPICS_r10_TDD[3] != '1'B)){ // @sic R5s200875, R5s210784 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 104 not set to '1' "); // @sic R5-196979 sic@

}

}

}

else {

// FGIs not reported: error in Rel9, all 1s in future releases assumed  (36.331 Annex B1)

f_SetVerdict(fail, __FILE__, __LINE__, "featureGroupIndRel10_r10 capabilities are not reported");  // @sic R5-133612 sic@

}

// Skipped code

}

After change:

function fl_EUTRA_CheckRel10_FGIs(UE_EUTRA_Capability p_ReceivedCapabilityMsg,

boolean p_Rel15 := false)

{

// Skipped code

if (p_Rel15) {

// check Setting of FGI 103 and FGI 104 against setting of UE categories

// IF (pc_ue_Category_8 or pc_ue_Category_11 or pc_ue_Category_12 or pc_ue_CategoryDL_11 or pc_ue_CategoryDL_12 or pc_ue_CategoryDL_14) THEN  FGI 103 resp 104 shall be set to '1' in one RAT at least

// It is assumed that the common value will always be one of the RAT specific values !

// E.g. common value of '1' and RAT specific values of '0' will never occur

// Otherwise FGI reporting and FGI PICS setting would both be consistently incorrect !

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10_FDD[2] != '1'B) and (v_FGIsFromPICS_r10_TDD[2] != '1'B)){ // @sic R5s200875, R5s210784 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 103 not set to '1' "); // @sic R5-196979 sic@

}

if (v_UE_CatsForFGI_103_104 and (pc_eTDD == true) and (v_FGIsFromPICS_r10_TDD[3] != '1'B)){ // @sic R5s200875, R5s210784 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 104 not set to '1' "); // @sic R5-196979 sic@

}

}

}

else {

// FGIs not reported: error in Rel9, all 1s in future releases assumed  (36.331 Annex B1)

f_SetVerdict(fail, __FILE__, __LINE__, "featureGroupIndRel10_r10 capabilities are not reported");  // @sic R5-133612 sic@

}

// Skipped code

}

## 2.2 Change 2

| Function name | f_Derive_phyLayerParameters_v1530 |
| --- | --- |
| Reason for change | Following implementation of prose CR R5-221221 and TTCN CR R5s220954, a conformant UE operating at Rel-15 or later fail the capability check as it may not include ce_Capabilities_r15 IE if pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH are not supported but may support other IEs under phyLayerParameters_v1530. |
| Summary of change | Check for ce_Capabilities_r15 IE only if pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH are supported. |
| TTCN module | EUTRA_AuxiliaryCapCheckFunctions.ttcn |
| MCC160 Comment |  |

Before change:

function f_Derive_phyLayerParameters_v1530() return template PhyLayerParameters_v1530_Type

{ // @sic R5s220954 sic@

var template PhyLayerParameters_v1530_Type v_PhyLayerParameters_v1530 := omit;

v_PhyLayerParameters_v1530 := {

stti_SPT_Capabilities_r15 := *,

ce_Capabilities_r15 := {

ce_CRS_IntfMitig_r15 := *,

ce_CQI_AlternativeTable_r15 := *,

ce_PDSCH_FlexibleStartPRB_CE_ModeA_r15 := f_Derive_CE_PDSCH_FlexibleStartPRB_CE_ModeA_r15(), // @sic R5-221221 sic@

ce_PDSCH_FlexibleStartPRB_CE_ModeB_r15 := *,

ce_PDSCH_64QAM_r15 := *,

ce_PUSCH_FlexibleStartPRB_CE_ModeA_r15 := f_Derive_CE_PUSCH_FlexibleStartPRB_CE_ModeA_r15(), // @sic R5-221221 sic@

ce_PUSCH_FlexibleStartPRB_CE_ModeB_r15 := *,

ce_PUSCH_SubPRB_Allocation_r15 := *,

ce_UL_HARQ_ACK_Feedback_r15 := *

},

shortCQI_ForSCellActivation_r15 := *,

mimo_CBSR_AdvancedCSI_r15 := *,

crs_IntfMitig_r15 := *,

ul_PowerControlEnhancements_r15 := *,

urllc_Capabilities_r15 := *,

altMCS_Table_r15 := *

}

if (pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH) {

return v_PhyLayerParameters_v1530;

}

else {

return v_PhyLayerParameters_v1530 ifpresent;

}

}

After change:

type UE_EUTRA_Capability_v1530_IEs.phyLayerParameters_v1530.ce_Capabilities_r15 CE_Capabilities_r15_Type;

function f_Derive_phyLayerParameters_v1530() return template PhyLayerParameters_v1530_Type

{ // @sic R5s220954 sic@

var template PhyLayerParameters_v1530_Type v_PhyLayerParameters_v1530 := omit;

var template CE_Capabilities_r15_Type v_CE_Capabilities_r15 := *;

if (pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH) {

v_CE_Capabilities_r15 := {

ce_CRS_IntfMitig_r15 := *,

ce_CQI_AlternativeTable_r15 := *,

ce_PDSCH_FlexibleStartPRB_CE_ModeA_r15 := f_Derive_CE_PDSCH_FlexibleStartPRB_CE_ModeA_r15(), // @sic R5-221221 sic@

ce_PDSCH_FlexibleStartPRB_CE_ModeB_r15 := *,

ce_PDSCH_64QAM_r15 := *,

ce_PUSCH_FlexibleStartPRB_CE_ModeA_r15 := f_Derive_CE_PUSCH_FlexibleStartPRB_CE_ModeA_r15(), // @sic R5-221221 sic@

ce_PUSCH_FlexibleStartPRB_CE_ModeB_r15 := *,

ce_PUSCH_SubPRB_Allocation_r15 := *,

ce_UL_HARQ_ACK_Feedback_r15 := *

}

}

v_PhyLayerParameters_v1530 := {

stti_SPT_Capabilities_r15 := *,

ce_Capabilities_r15 := v_CE_Capabilities_r15,

shortCQI_ForSCellActivation_r15 := *,

mimo_CBSR_AdvancedCSI_r15 := *,

crs_IntfMitig_r15 := *,

ul_PowerControlEnhancements_r15 := *,

urllc_Capabilities_r15 := *,

altMCS_Table_r15 := *

}

if (pc_FlexibleStartPRB_PDSCH or pc_FlexibleStartPRB_PUSCH) {

return v_PhyLayerParameters_v1530;

}

else {

return v_PhyLayerParameters_v1530 ifpresent;

}

}