3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230343

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0901 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS test case 10.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Two types of UE behaviours are possible in postamble depending upon whether UE is configured in voice centric or data centric. Therefore, it is proposed to execute different postamble branches depending upon UE configuration. UE configured in voice Centric: IMS voice is not available (informed via NOTIFY at step 15), then, N1 mode shall be disabled as per TS24.501, cl.4.3.2. [24.501, cl.4.3.2] When IMS voice is not available over 3GPP access, if the UE's usage setting is "voice centric", the UE operates in single-registration mode, and the UE. b) has a persistent PDU session, then the UE waits until the radio bearer associated with the persistent PDU session has been released. When the radio bearer associated with the persistent PDU session has been released, then: 1) if the UE is only registered over 3GPP access, or if the UE is registered over both 3GPP access and non-3GPP access and IMS voice is not available over non-3GPP access, the UE shall disable the N1 mode capability for 3GPP access and proceed as specified in subclause 4.9.2 with modifications described below; or As per above, UE shall wait until Radio bearer release to disable the N1 mode. Therefore, in case of voice centric, RRCRelease is sufficient (ie f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration); ) so that radio bearers are released and N1 mode is disabled following that. UE configured in data Centric: N1 mode is not disabled so postamble with emergency call release would be sufficient( ie f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService);) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following branches are proposed to handle postamble: f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration) in case of voice centric; f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService) in case of data cetric |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.14.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:                      Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_10_14_NR5GC_IMS2() |
| --- | --- |
| Reason for change | Two types of UE behaviours are possible in postamble depending upon whether UE is configured in voice centric or data centric. Therefore, it is proposed to execute different postamble branches depending upon UE configuration. UE configured in voice Centric: IMS voice is not available (informed via NOTIFY at step 15), then, N1 mode shall be disabled as per TS24.501, cl.4.3.2. [24.501, cl.4.3.2] When IMS voice is not available over 3GPP access, if the UE's usage setting is "voice centric", the UE operates in single-registration mode, and the UE. b) has a persistent PDU session, then the UE waits until the radio bearer associated with the persistent PDU session has been released. When the radio bearer associated with the persistent PDU session has been released, then: 1) if the UE is only registered over 3GPP access, or if the UE is registered over both 3GPP access and non-3GPP access and IMS voice is not available over non-3GPP access, the UE shall disable the N1 mode capability for 3GPP access and proceed as specified in subclause 4.9.2 with modifications described below; or As per above, UE shall wait until Radio bearer release to disable the N1 mode. Therefore, in case of voice centric, RRCRelease is sufficient (ie f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration); ) so that radio bearers are released and N1 mode is disabled following that. UE configured in data Centric: N1 mode is not disabled so postamble with emergency call release would be sufficient(ie f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService);) |
| Summary of change | Following branches are proposed to handle postamble: f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration) in case of voice centric; f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService) in case of data cetric |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change

import from IMS_XML_Templates all; import from CommonDefs all; function f_TC_10_14_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Steps 1 - 14" siclog@ f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService); f_IMS_EmergencyRegistration(); f_IMS_EmergencyCallSetup_NR5GC_A_6(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14"); f_IMS_IMS_SendCoOrdMsg(OtherPDN); // Send sync message to other IMS PTC (for steps 16/17) f_IMS_IMS_WaitForTrigger(OtherPDN); // Wait for sync message from other IMS PTC v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(registration); // @sic R5s221189 sic@ //@siclog "Steps 17" siclog@ //Void // @sic R5-227488 sic@ //@siclog "Steps 18" siclog@ f_Delay(5.0); // @sic R5-227488 sic@ //@siclog "Steps 19 - 20" siclog@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); }

After Change

import from CommonDefs all; import from Parameters all; function f_TC_10_14_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Steps 1 - 14" siclog@ f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService); f_IMS_EmergencyRegistration(); f_IMS_EmergencyCallSetup_NR5GC_A_6(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14"); f_IMS_IMS_SendCoOrdMsg(OtherPDN); // Send sync message to other IMS PTC (for steps 16/17) f_IMS_IMS_WaitForTrigger(OtherPDN); // Wait for sync message from other IMS PTC v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(registration); // @sic R5s221189 sic@ //@siclog "Steps 17" siclog@ //Void // @sic R5-227488 sic@ //@siclog "Steps 18" siclog@ f_Delay(5.0); // @sic R5-227488 sic@ //@siclog "Steps 19 - 20" siclog@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); f_IMS_TestBody_Set(false); if (px_UE_CS_PS_UsageSetting_Tested == VOICE_CENTRIC){ f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration); } else { f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); } }