3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230348

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3066 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC testcases 8.2.4.2.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At Step 7 – 8, the parameter to change the current PSCell (NR Cell 2) to make it a SCell is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the 4th parameter of function f_NR_RRC_PSCellChangeCA_InterCell to p_PSCell2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.2.1.1.ENDC, 8.2.4.2.1.2.ENDC, 8.2.4.2.1.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function f_TC_8_2_4_2_1_x_ENDC_NR_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_8_2_4_2_1_x_ENDC_NR_TestBody

| Function name | f_TC_8_2_4_2_1_x_ENDC_NR_TestBody |
| --- | --- |
| Reason for change | At Step 7 – 8, the parameter to change the current PSCell (NR Cell 2) to make it a SCell is missing |
| Summary of change | Set the 4th parameter of function f_NR_RRC_PSCellChangeCA_InterCell to p_PSCell2 |
| TTCN module | RRC_CarrierAggregation_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. //Check: Does the UE send an RRCConnectionReconfigurationComplete message and reconfigure the current NR SCell to PSCell? //@sic R5s220809 sic@ v_SCGConfig := f_NR_GetSCGConfigDef(p_SCell2); v_SCGConfig_SCell := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; f_NR_ReconfigSCellToNormalCell(p_SCell2, v_RadioBearerList); f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell2, p_SCell2, p_SCell2, omit, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); //@sic R5s221199 sic@ }

After Change:

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. //Check: Does the UE send an RRCConnectionReconfigurationComplete message and reconfigure the current NR SCell to PSCell? //@sic R5s220809 sic@ v_SCGConfig := f_NR_GetSCGConfigDef(p_SCell2); v_SCGConfig_SCell := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; f_NR_ReconfigSCellToNormalCell(p_SCell2, v_RadioBearerList); f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell2, p_SCell2, p_SCell2, p_PSCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); //WA# //@sic R5s221199 sic@ }