3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230355

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0903 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10 function, requirement to include 100rel tag in 180 Ringing will be set, when it is not specified to be mandatory in TS 34.229-5 Annex A.5.1a. Check of mentioned tag should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed input parameter to check for 100rel tag at 180 Ringing message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.24.NR5GC, 7.31.NR5GC, 7.32.NR5GC, 7.34.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Correction to function f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10

| Function name | f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10 |
| --- | --- |
| Reason for change | In f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10 function, requirement to include 100rel tag in 180 Ringing will be set, when it is not specified to be mandatory in TS 34.229-5 Annex A.5.1a. |
| Summary of change | Removed input parameter to check for 100rel tag at 180 Ringing message. |
| TTCN module | IMS\NR5GC_7\IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10(template (omit) SessionExpires p_SessionExpires := omit, template (omit) Allow p_Allow := omit, OptionTag_List p_OptionTagsForSupported := tsc_OptionTagList_Invite_C11) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* @sic R5-231489: use of f_IMS_MTCallSetup_BuildInviteRequestWithSpecificSdp, f_IMS_MTCallSetup_NR5GC_A_5_1x_Step2_7 sic@ */ /* @sic R5-231494, R5-231496, R5-231497: step 1 - 10 only sic@ */ var template (value) SDP_Message v_SdpOffer; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; // step 1: INVITE v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudio_TX(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, EVS_A2, tsc_SDP_WithPreconditions); v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSpecificSdp(v_SdpOffer, p_SessionExpires, p_Allow, p_OptionTagsForSupported); IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite)); // step 2 - 7: 100 Trying, 183 Session Progress, PRACK, 200 OK, UPDATE, 200 OK f_IMS_MTCallSetup_NR5GC_A_5_1x_Step2_7(v_InviteRequestWithSdp, EVS_16000, EVS_A2); // Step 8 - 10: 180 Ringing, PRACK (conditional), 200 OK (conditional) f_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp, tsc_OptionTagList_100rel, tsc_SDP_WithPreconditions); return v_InviteRequestWithSdp; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10(template (omit) SessionExpires p_SessionExpires := omit, template (omit) Allow p_Allow := omit, OptionTag_List p_OptionTagsForSupported := tsc_OptionTagList_Invite_C11) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* @sic R5-231489: use of f_IMS_MTCallSetup_BuildInviteRequestWithSpecificSdp, f_IMS_MTCallSetup_NR5GC_A_5_1x_Step2_7 sic@ */ /* @sic R5-231494, R5-231496, R5-231497: step 1 - 10 only sic@ */ var template (value) SDP_Message v_SdpOffer; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; // step 1: INVITE v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudio_TX(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, EVS_A2, tsc_SDP_WithPreconditions); v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSpecificSdp(v_SdpOffer, p_SessionExpires, p_Allow, p_OptionTagsForSupported); IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite)); // step 2 - 7: 100 Trying, 183 Session Progress, PRACK, 200 OK, UPDATE, 200 OK f_IMS_MTCallSetup_NR5GC_A_5_1x_Step2_7(v_InviteRequestWithSdp, EVS_16000, EVS_A2); // Step 8 - 10: 180 Ringing, PRACK (conditional), 200 OK (conditional) f_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp, -, tsc_SDP_WithPreconditions); return v_InviteRequestWithSdp; } <<SKIPPED CODE>>