3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230358

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0904 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS test cases 7.32 and 7.34 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In test 7.32, based on R5-231497 the steps 14-18 are performed using the A.5.1a steps 6-10. But this means that step 10A of A.5.1a is not executed, that is the call is not supposed to be answered. This is wrong for this test case, because an unanswered call will be soon terminated by the callee party (likely using 480TemporarilyUnavailable) as the call establishment timer times out due to user inactivity. - Inside the f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A there is a typo which is causing this function to call itself recursively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added step 10A of A.5.1 after steps 14-18 of the test body by replacing the f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10 with f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A in steps 9-18. Prose CR associated with this change - Corrected the typo inside f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A. This change affects 7.32 and 7.34. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.32.NR5GC and 7.34.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 34.229-5 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:                      Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_32_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Based on R5-231497 the steps 14-18 are performed using the A.5.1a steps 6-10. But this means that step 10A of A.5.1a is not executed, that is the call is not supposed to be answered. This is wrong for this test case, because an unanswered call will be soon terminated by the callee party (likely using 480TemporarilyUnavailable) as the call establishment timer times out due to user inactivity. |
| Summary of change | Replaced the f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10 with f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A in steps 9-18. |
| TTCN module | IMS\NR5GC_7\IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_32_NR5GC_IMS1() runs on IMS_PTC { // Session Timer / MT Voice Call / Remote end sends Session-Expires but does not choose refresher / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrlUE; var IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) MessageHeader v_MessageHeaderUpdate; var integer v_DeltaSec := 1800; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9 - 18" IPCAN siclog@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10(cs_SessionExpires(v_DeltaSec)); // @sic R5-231497: steps 1 - 10 of A.5.1a sic@ v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); v_RoutingInfo_DL := v_InviteRequestWithSdp.RoutingInfo; // @siclog "Steps 19 - 20" siclog@ f_IMS_MTCallSetupWithSessionTimer_Receive200OK_SendACK(v_InviteRequestWithSdp, cr_Require_timer, -, v_DeltaSec, "uac"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 19"); // @siclog "Step 21" siclog@ v_MessageHeaderUpdate := f_IMS_CallWithSessionTimer_SendUpdateAfterHalfDeltaSec(v_InviteRequestWithSdp.Invite, v_RoutingInfo_DL, -, v_ContactUrlUE, v_DeltaSec, "uac"); // @siclog "Step 22" siclog@ f_IMS_CallWithSessionTimer_Receive200OK(v_RoutingInfo_DL.Protocol, v_MessageHeaderUpdate, cr_Require_timer, -, v_DeltaSec, "uac"); // @sic R5s210467: cr_Supported_timer removed sic@ @sic R5-213533: cr_Require_timer added sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22"); // @siclog "Step 23 - 24" siclog@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MT_SpeechCall); }

After Change

function f_TC_7_32_NR5GC_IMS1() runs on IMS_PTC { // Session Timer / MT Voice Call / Remote end sends Session-Expires but does not choose refresher / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrlUE; var IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) MessageHeader v_MessageHeaderUpdate; var integer v_DeltaSec := 1800; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9 - 18" IPCAN siclog@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A(cs_SessionExpires(v_DeltaSec)); // @sic R5-231497: steps 1 - 10 of A.5.1a sic@ v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); v_RoutingInfo_DL := v_InviteRequestWithSdp.RoutingInfo; // @siclog "Steps 19 - 20" siclog@ f_IMS_MTCallSetupWithSessionTimer_Receive200OK_SendACK(v_InviteRequestWithSdp, cr_Require_timer, -, v_DeltaSec, "uac"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 19"); // @siclog "Step 21" siclog@ v_MessageHeaderUpdate := f_IMS_CallWithSessionTimer_SendUpdateAfterHalfDeltaSec(v_InviteRequestWithSdp.Invite, v_RoutingInfo_DL, -, v_ContactUrlUE, v_DeltaSec, "uac"); // @siclog "Step 22" siclog@ f_IMS_CallWithSessionTimer_Receive200OK(v_RoutingInfo_DL.Protocol, v_MessageHeaderUpdate, cr_Require_timer, -, v_DeltaSec, "uac"); // @sic R5s210467: cr_Supported_timer removed sic@ @sic R5-213533: cr_Require_timer added sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22"); // @siclog "Step 23 - 24" siclog@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MT_SpeechCall); }

## Change 2

| Function name | f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A |
| --- | --- |
| Reason for change | Inside the f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A there is a typo which is causing this function to call itself recursively. |
| Summary of change | Replaced f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A with f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10 inside declaration of the f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A. |
| TTCN module | IMS\NR5GC_7\IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change

function f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A(template (omit) SessionExpires p_SessionExpires := omit, template (omit) Allow p_Allow := omit, OptionTag_List p_OptionTagsForSupported := tsc_OptionTagList_Invite_C11) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* @sic R5-231498: A.5.1a instead of A.5.1 sic@ */ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; // step 1 - 10 v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A(p_SessionExpires, p_Allow, p_OptionTagsForSupported); // Step 10A f_UT_AnswerCall(MMI); return v_InviteRequestWithSdp; }

After Change

function f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10A(template (omit) SessionExpires p_SessionExpires := omit, template (omit) Allow p_Allow := omit, OptionTag_List p_OptionTagsForSupported := tsc_OptionTagList_Invite_C11) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* @sic R5-231498: A.5.1a instead of A.5.1 sic@ */ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; // step 1 - 10 v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1a_Step1_10(p_SessionExpires, p_Allow, p_OptionTagsForSupported); // Step 10A f_UT_AnswerCall(MMI); return v_InviteRequestWithSdp; }