3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230362

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3075 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1: Re-verification of Rel-16 DAPS RRC test case 8.1.4.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU EMEA LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Re-verification of Rel-16 DAPS RRC test case 8.1.4.3.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.4.3.5 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1	Verification Test Summary	2

2	Corrections required	2

2.1	Change 1	2

2.2	Change 2	10

3	Branches executed	15

4	Execution Log Files	15

4.1	MediaTek MT6879	15

5	References	15

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.4.3.5, which is part of the NR5GC test suite in iwd-TTCN3-B2022-09_D23wk12.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.4.3.5

ATS Version:	iwd-TTCN3-B2022-09_D23wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	MediaTek MT6879 UE

Verification Status:	PASS

# Corrections required

Changes on CR R5s230320 included except change 4.2.

## Change 1

| Function name | fl_TC_8_1_4_3_2And5_TestBody () |
| --- | --- |
| Reason for change | - function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig does not set PDCP count for DRB on target cell. - If f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig returns before actual RRCReconfiguration transmission, then, SS will turn off source cell and UE will not receive the message. So, added delay to give minimum buffer time for successful transmission of rrcreconfiguration message. - Forwarding step 13 as string to f_NR5GC_DAPS_HO_SourceRelease for better visibility of passing step. |
| Summary of change | . - Added v_NR_PdcpCountInfoList_DRB to get the DRBs PDCP count from source cell. - Added f_delay function with 300 ms delay. - Added "Step 13" on f_NR5GC_DAPS_HO_SourceRelease. |
| TTCN module | RRC_DAPS_Handover_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_4_3_2And5_TestBody(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) DL_DCCH_Message v_RRCReconfiguration; var SubFrameTiming_Type v_Timing; var NR_PdcpCountInfoList_Type v_PdcpCountInfoList1; //@sic R5s221242 sic@ var NR_PdcpCountInfoList_Type v_PdcpCountInfoList2; //@sic R5s221242 sic@ var NR_PdcpCountInfoList_Type v_PdcpCountInfoList; //@sic R5s221242 sic@ timer t_Wait := 5.0; v_CellPowerList_AtT0 := {cs_NR_CellPower(p_SourceCellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // TC n/a for FR2 cs_NR_CellPower(p_TargetCellId, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 v_CellPowerList_AtT1 := {cs_NR_CellPower(p_TargetCellId, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 v_CellPowerList_AtT2 := {cs_NR_CellPower(p_TargetCellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 v_CellPowerList_AtT3 := {cs_NR_CellPower(p_SourceCellId, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 //Set power level to T0 f_NR_SetCellPowerList (v_CellPowerList_AtT0); // Configuration of Target Cell for no RACH response transmission) f_NR_SS_RachProcedureConfig(p_TargetCellId, cs_NR_RachProcedureConfig_NoResponse, cs_TimingInfo_Now); //@siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2. // DRB#n of the first PDU session is configured as DAPS bearer. v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); v_RACH_ConfigGeneric.preambleTransMax := n200; // Table 8.1.4.3.2.3.3-4 v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount), cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourceCellId, cs_TimingInfo_Now, v_RRCReconfiguration)); //@siclog "Step 2" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList (v_CellPowerList_AtT1); //Acc. 38.331, 5.3.5.8.3: the UE shall, for each SRB, re-establish the RLC entity for the source PCell. When we release an SRB in the SS, the PDCP count is lost, so we need to set it to the original value once the SRB is configured again. //@sic R5s221242 sic@ v_PdcpCountInfoList1 := f_NR_SS_PdcpCount_Get(p_SourceCellId,tsc_NR_RbId_SRB1); v_PdcpCountInfoList2 := f_NR_SS_PdcpCount_Get(p_SourceCellId,tsc_NR_RbId_SRB2); v_PdcpCountInfoList := v_PdcpCountInfoList1 & v_PdcpCountInfoList2; f_NR_SS_SRBs_DRBs_Release(p_SourceCellId, cs_TimingInfo_Now, {}, tsc_NoCnfReq); f_NR_SS_SRBs_DRBs_Config(p_SourceCellId, cs_TimingInfo_Now, omit, tsc_NoCnfReq); //@siclog "Step 3-4" siclog@ // Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1 within 5s? // Check: Does the UE transmit a FailureInformation message with failureType-r16 set to daps-failure on NR Cell 1 within 5s? t_Wait.start; alt { [] t_Wait.timeout{} [] SRB.receive(car_NR_SRB0_RrcPdu_IND (p_SourceCellId, cr_38508_RRCReestablishmentRequest(?, ?))){ t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "RRCReestablishmentRequest received on NR Cell 1"); } [] SRB.receive(car_NR_SRB1_RrcPdu_IND (p_SourceCellId, cr_38508_FailureInformation(-, cr_FailureInformation_v1610({daps_failure})))){ //@sic R5s221242 sic@ t_Wait.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@sic R5s221242 sic@ } } //@sic R5s221242 sic@ f_NR_SS_PdcpCount_Set(p_SourceCellId, v_PdcpCountInfoList, cs_TimingInfo_Now); //@siclog "Step 5" siclog@ f_NR5GC_DAPS_HO_CheckDataPath(p_SourceCellId, "Step 5"); //@siclog "Step 6" siclog@ // The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList (v_CellPowerList_AtT2); //@siclog "Step 7" siclog@ v_Timing := f_NR_GetNextSendOccasion_DL(p_SourceCellId, 700); // 700 ms f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(p_SourceCellId, p_TargetCellId, v_Timing); //@siclog "Step 8" siclog@ // The SS changes the power level setting according to the row "T3" and waits 300ms to ensure T310 expires in NR Cell 1. // Added delay here f_NR_SetCellPowerList (v_CellPowerList_AtT3); f_Delay(0.3); //@sic R5s221242 sic@ // added DRB PDCP count here //@siclog "Steps 9-10" siclog@ // The SS transmits Random Access Response to respond to the latest preamble on NR Cell 2. // Check: Does UE transmit an RRCReconfigurationComplete message in NR Cell 2? f_NR5GC_DAPS_HO_RACHProcedure_RRCReconfigurationComplete(p_TargetCellId, "Step 10", -, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); //@siclog "Step 11" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB#n of the first PDU session on NR Cell 2? f_NR5GC_DAPS_HO_CheckDataPath(p_TargetCellId, "Step 11"); //@siclog "Steps 12-13" siclog@ // The SS transmits an RRCReconfiguration message with condition DAPS_HO_ReleaseSource in NR Cell 2. // Check: Does the UE transmits an RRCReconfigurationComplete message in NR Cell 2. f_NR5GC_DAPS_HO_SourceRelease(p_SourceCellId, p_TargetCellId, -, false); //@sic R5-230116 sic@ f_NR5GC_DAPS_HO_SourceRelease_SSConfig(p_SourceCellId); }

After Change

function fl_TC_8_1_4_3_2And5_TestBody(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) DL_DCCH_Message v_RRCReconfiguration; var SubFrameTiming_Type v_Timing; var NR_PdcpCountInfoList_Type v_PdcpCountInfoList1; //@sic R5s221242 sic@ var NR_PdcpCountInfoList_Type v_PdcpCountInfoList2; //@sic R5s221242 sic@ var NR_PdcpCountInfoList_Type v_PdcpCountInfoList; //@sic R5s221242 sic@ timer t_Wait := 5.0; var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; v_CellPowerList_AtT0 := {cs_NR_CellPower(p_SourceCellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // TC n/a for FR2 cs_NR_CellPower(p_TargetCellId, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 v_CellPowerList_AtT1 := {cs_NR_CellPower(p_TargetCellId, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 v_CellPowerList_AtT2 := {cs_NR_CellPower(p_TargetCellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 v_CellPowerList_AtT3 := {cs_NR_CellPower(p_SourceCellId, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 //Set power level to T0 f_NR_SetCellPowerList (v_CellPowerList_AtT0); // Configuration of Target Cell for no RACH response transmission) f_NR_SS_RachProcedureConfig(p_TargetCellId, cs_NR_RachProcedureConfig_NoResponse, cs_TimingInfo_Now); //@siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2. // DRB#n of the first PDU session is configured as DAPS bearer. v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); v_RACH_ConfigGeneric.preambleTransMax := n200; // Table 8.1.4.3.2.3.3-4 v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount), cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourceCellId, cs_TimingInfo_Now, v_RRCReconfiguration)); //@siclog "Step 2" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList (v_CellPowerList_AtT1); //Acc. 38.331, 5.3.5.8.3: the UE shall, for each SRB, re-establish the RLC entity for the source PCell. When we release an SRB in the SS, the PDCP count is lost, so we need to set it to the original value once the SRB is configured again. //@sic R5s221242 sic@ v_PdcpCountInfoList1 := f_NR_SS_PdcpCount_Get(p_SourceCellId,tsc_NR_RbId_SRB1); v_PdcpCountInfoList2 := f_NR_SS_PdcpCount_Get(p_SourceCellId,tsc_NR_RbId_SRB2); v_PdcpCountInfoList := v_PdcpCountInfoList1 & v_PdcpCountInfoList2; f_NR_SS_SRBs_DRBs_Release(p_SourceCellId, cs_TimingInfo_Now, {}, tsc_NoCnfReq); f_NR_SS_SRBs_DRBs_Config(p_SourceCellId, cs_TimingInfo_Now, omit, tsc_NoCnfReq); //@siclog "Step 3-4" siclog@ // Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1 within 5s? // Check: Does the UE transmit a FailureInformation message with failureType-r16 set to daps-failure on NR Cell 1 within 5s? t_Wait.start; alt { [] t_Wait.timeout{} [] SRB.receive(car_NR_SRB0_RrcPdu_IND (p_SourceCellId, cr_38508_RRCReestablishmentRequest(?, ?))){ t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "RRCReestablishmentRequest received on NR Cell 1"); } [] SRB.receive(car_NR_SRB1_RrcPdu_IND (p_SourceCellId, cr_38508_FailureInformation(-, cr_FailureInformation_v1610({daps_failure})))){ //@sic R5s221242 sic@ t_Wait.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@sic R5s221242 sic@ } } //@sic R5s221242 sic@ f_NR_SS_PdcpCount_Set(p_SourceCellId, v_PdcpCountInfoList, cs_TimingInfo_Now); //@siclog "Step 5" siclog@ f_NR5GC_DAPS_HO_CheckDataPath(p_SourceCellId, "Step 5"); //@siclog "Step 6" siclog@ // The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList (v_CellPowerList_AtT2); //@siclog "Step 7" siclog@ //v_Timing := f_NR_GetNextSendOccasion_DL(p_SourceCellId, 700); // 700 ms v_NR_PdcpCountInfoList_DRB :=f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(v_Timing, p_SourceCellId, p_TargetCellId); f_Delay(0.3); //@siclog "Step 8" siclog@ // The SS changes the power level setting according to the row "T3" and waits 300ms to ensure T310 expires in NR Cell 1. f_NR_SetCellPowerList (v_CellPowerList_AtT3); f_Delay(0.3); //@sic R5s221242 sic@ f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList_DRB, cs_TimingInfo_Now); //Set count for DAPS DRB; TS 38.523-3, 7.3.5.3.5, Step 10A //@siclog "Steps 9-10" siclog@ // The SS transmits Random Access Response to respond to the latest preamble on NR Cell 2. // Check: Does UE transmit an RRCReconfigurationComplete message in NR Cell 2? f_NR5GC_DAPS_HO_RACHProcedure_RRCReconfigurationComplete(p_TargetCellId, "Step 10", -, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); //@siclog "Step 11" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB#n of the first PDU session on NR Cell 2? f_NR5GC_DAPS_HO_CheckDataPath(p_TargetCellId, "Step 11"); //@siclog "Steps 12-13" siclog@ // The SS transmits an RRCReconfiguration message with condition DAPS_HO_ReleaseSource in NR Cell 2. // Check: Does the UE transmits an RRCReconfigurationComplete message in NR Cell 2. f_NR5GC_DAPS_HO_SourceRelease(p_SourceCellId, p_TargetCellId, "Step 13", false); //@sic R5-230116 sic@ f_NR5GC_DAPS_HO_SourceRelease_SSConfig(p_SourceCellId); }

## Change 2

| Function name | f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig () |
| --- | --- |
| Reason for change | - Within f_NR_SS_AS_ActivateSecurity_Current, f_NR_RRC_CipherActTime_GetCurrent -> f_NR_SS_PdcpCount_Get is called and will return status of DRBs and SRBs. But, DRBs are not yet active though configured as activation time for configuration of DRB is not yet reached. In such case, f_NR_SS_PdcpCount_Get does not return count for DRBs. So TTCN can avoid this issue by forwarding PdcpCountInfoList from source cell. SRBs are to set to ‘0’. |
| Summary of change | - Commented PDCP count setting on target cell for SRBs and modified v_NR_PdcpCountInfoList for SRBs as 0. And added variable v_NR_PdcpCountInfoList to be forwarded on f_NR_SS_AS_ActivateSecurity_Current. |
| TTCN module | NR5GC_Handover |
| MCC160 Comment |  |

Before Change

function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, SubFrameTiming_Type p_Timing, // timing based on Source Cell boolean p_RBConfig_KeyChange := true) runs on NR5GC_PTC return NR_PdcpCountInfoList_Type { var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (omit) MasterKeyUpdate v_MasterKeyUpdate := omit; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList; //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_SRBs; //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; //@sic R5-230116 sic@ var template(value) TimingInfo_Type v_TimingInfo; //@sic R5s230023 sic@ v_Timing1 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 5); // time for sending RRCReconfiguration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 10); // time for COUNT tranfer and configuration in target cell //Acc. Table 8.1.4.3.1.3.3-1 v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); // @sic R5s221151 sic@ Deleted "v_RACH_ConfigGeneric.preambleTransMax := n200;" if (p_RBConfig_KeyChange == true) { v_MasterKeyUpdate := cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, v_MasterKeyUpdate, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId))), -, p_RBConfig_KeyChange); // Step 0. Source Cell: Get PDCP COUNT for the DAPS DRB and SRBs v_NR_PdcpCountInfoList := f_NR_GetPdcpCountInfoListIntraNR_HO(p_SourceCellId, v_RadioBearerList); //RBConfig_KeyChange:=true, TimingInfo_Now, IncrementSRB1:=true // Step 1. Target Cell: Configuration of DRBs //v_RadioBearerList[0].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId := valueof (v_RadioBearerList[0].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId) + 1; //Configure DRB for DAPS, @sic R5s221151, R5s230023 sic@ //Calculate T in the TargetCell v_TimingInfo := f_NR_TimingOtherCell(p_SourceCellId, cs_TimingInfo_NR(p_Timing), p_TargetCellId); //@sic R5s230023 sic@ f_NR_SS_CommonRadioBearerConfig(p_TargetCellId, v_RadioBearerList, v_TimingInfo); //@sic R5s230023 sic@ // Step 2. Target Cell: Configuration of Target Cell for no RACH response transmission f_NR_SS_RachProcedureConfig(p_TargetCellId, cs_NR_RachProcedureConfig_NoResponse, v_TimingInfo); // Step 3. Source Cell: Send RRCReconfiguration SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourceCellId, cs_TimingInfo_NR(v_Timing1), v_RRCReconfiguration)); // Step 4. Transfer of the PDCP COUNT for the SRBs (if applied) from source to target cell // Target Cell: Set PDCP COUNT. // NOTE 2: For SRBs, the PDCP COUNT is maintained or reset depending on the RRCReconfiguration message content. If maintained, the DL PDCP COUNT for SRB1 is incremented by 1 to consider the RRCReconfiguration message sent on step 3. //Calculate T2 in the TargetCell v_TimingInfo := f_NR_TimingOtherCell(p_SourceCellId, cs_TimingInfo_NR(v_Timing2), p_TargetCellId); //@sic R5s230023 sic@ v_NR_PdcpCountInfoList_SRBs := substr(v_NR_PdcpCountInfoList,0,2); f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList_SRBs, v_TimingInfo); //Set count for SRB1 and SRB2 @sic R5s230023 sic@ // Step 5. Target Cell: Inform the SS about the HO and about the source cell id. f_NR_SS_PdcpHandoverCtrl(p_TargetCellId, cas_NR_PdcpHandoverInit_REQ(p_SourceCellId, p_TargetCellId)); // (Step 6. Target Cell: Activate security. // RBConfig_KeyChange =true -> ActivateSecurity_Current f_NR_SS_AS_ActivateSecurity_Current(p_TargetCellId, tsc_NR_38508_NextHopChainingCount, v_TimingInfo); //@sic R5s230023 sic@ // Step 7. Target Cell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetCellId, v_TimingInfo, -, -, cs_UL_GrantConfig_OnSR);//@sic R5s230023 sic@ // Step 8. Target Cell: Start periodic TA f_NR_ULGrantConfiguration_Common(p_TargetCellId, v_TimingInfo, -, cs_NR_UplinkTimeAlignment_Start, -);//@sic R5s230023 sic@ //return the DRB count to be used later in the procedure v_NR_PdcpCountInfoList_DRB := substr(v_NR_PdcpCountInfoList,2,1); return v_NR_PdcpCountInfoList_DRB; }

After Change

function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(inout SubFrameTiming_Type p_Timing, // timing based on Source Cell NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, boolean p_RBConfig_KeyChange := true) runs on NR5GC_PTC return NR_PdcpCountInfoList_Type { var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (omit) MasterKeyUpdate v_MasterKeyUpdate := omit; //var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList; //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_SRBs; //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; //@sic R5-230116 sic@ var template(value) TimingInfo_Type v_TimingInfo; //@sic R5s230023 sic@ //Acc. Table 8.1.4.3.1.3.3-1 v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); // @sic R5s221151 sic@ Deleted "v_RACH_ConfigGeneric.preambleTransMax := n200;" if (p_RBConfig_KeyChange == true) { v_MasterKeyUpdate := cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, v_MasterKeyUpdate, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId))), -, p_RBConfig_KeyChange); p_Timing := f_NR_GetNextSendOccasion_DL(p_SourceCellId, 700); // 700 ms v_Timing1 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 5); // time for sending RRCReconfiguration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 10); // time for COUNT tranfer and configuration in target cell // Step 0. Source Cell: Get PDCP COUNT for the DAPS DRB and SRBs v_NR_PdcpCountInfoList := f_NR_GetPdcpCountInfoListIntraNR_HO(p_SourceCellId, v_RadioBearerList, p_RBConfig_KeyChange); //RBConfig_KeyChange:=true, TimingInfo_Now, IncrementSRB1:=true // Step 1. Target Cell: Configuration of DRBs //v_RadioBearerList[0].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId := valueof (v_RadioBearerList[0].Config.AddOrReconfigure.RlcBearer.Config.LogicalChannelId) + 1; //Configure DRB for DAPS, @sic R5s221151, R5s230023 sic@ //Calculate T in the TargetCell v_TimingInfo := f_NR_TimingOtherCell(p_SourceCellId, cs_TimingInfo_NR(p_Timing), p_TargetCellId); //@sic R5s230023 sic@ f_NR_SS_CommonRadioBearerConfig(p_TargetCellId, v_RadioBearerList, v_TimingInfo); //@sic R5s230023 sic@ // Step 2. Target Cell: Configuration of Target Cell for no RACH response transmission f_NR_SS_RachProcedureConfig(p_TargetCellId, cs_NR_RachProcedureConfig_NoResponse, v_TimingInfo); // Step 3. Source Cell: Send RRCReconfiguration SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourceCellId, cs_TimingInfo_NR(v_Timing1), v_RRCReconfiguration)); // Step 4. Transfer of the PDCP COUNT for the SRBs (if applied) from source to target cell // Target Cell: Set PDCP COUNT. // NOTE 2: For SRBs, the PDCP COUNT is maintained or reset depending on the RRCReconfiguration message content. If maintained, the DL PDCP COUNT for SRB1 is incremented by 1 to consider the RRCReconfiguration message sent on step 3. //Calculate T2 in the TargetCell v_TimingInfo := f_NR_TimingOtherCell(p_SourceCellId, cs_TimingInfo_NR(v_Timing2), p_TargetCellId); //@sic R5s230023 sic@ v_NR_PdcpCountInfoList_SRBs := substr(v_NR_PdcpCountInfoList,0,2); if(p_RBConfig_KeyChange){ v_NR_PdcpCountInfoList[0].DL.Value := int2bit(0, 32); v_NR_PdcpCountInfoList[0].UL.Value := int2bit(0, 32); v_NR_PdcpCountInfoList[1].DL.Value := int2bit(0, 32); v_NR_PdcpCountInfoList[1].UL.Value := int2bit(0, 32); } f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList_SRBs, v_TimingInfo); //Set count for SRB1 and SRB2 @sic R5s230023 sic@ // Step 5. Target Cell: Inform the SS about the HO and about the source cell id. f_NR_SS_PdcpHandoverCtrl(p_TargetCellId, cas_NR_PdcpHandoverInit_REQ(p_SourceCellId, p_TargetCellId)); // (Step 6. Target Cell: Activate security. // RBConfig_KeyChange =true -> ActivateSecurity_Current f_NR_SS_AS_ActivateSecurity_Current(p_TargetCellId, tsc_NR_38508_NextHopChainingCount, v_TimingInfo,-,v_NR_PdcpCountInfoList); //@sic R5s230023 sic@ // Step 7. Target Cell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetCellId, v_TimingInfo, -, -, cs_UL_GrantConfig_OnSR);//@sic R5s230023 sic@ // Step 8. Target Cell: Start periodic TA f_NR_ULGrantConfiguration_Common(p_TargetCellId, v_TimingInfo, -, cs_NR_UplinkTimeAlignment_Start, -);//@sic R5s230023 sic@ //return the DRB count to be used later in the procedure v_NR_PdcpCountInfoList_DRB := substr(v_NR_PdcpCountInfoList,2,1); return v_NR_PdcpCountInfoList_DRB; }

# Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## MediaTek MT6879

The MediaTek MT6879UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_4_3_5_LOG.html

- PICS/PIXIT parameter file:

TC_8_1_4_3_5_PIXIT.xml

# References

[1] 230363: NR5GC FR1: Supporting information for addition of Rel-16 DAPS Mobilty Enh test case 8.1.4.3.5