3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230369

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3078 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC UAC test case 11.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, While running the test case 11.3.5 in multi PDU environment when both IMS first and Internet first configs are done, and the TC failed when the Internet PDU SESSION EST REQ was sent by the UE for a DNN mismatch. 2, At step 25, the SIB1 is modified for UAC Barring info without waiting modification period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Code change made to pass the DNN (tsc_DNNValue, i.e. "ABCD") in the function f_NR5GC_PDUSessionEstablishment( ) when it is called at step 10Fa1-10Fa8. 2, updated step 25 with f_NR_ModifySysinfo() call |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_11_3_5_TestBody() |
| --- | --- |
| Reason for change | 1, While running the test case 11.3.5 in multi PDU environment when both IMS first and Internet first configs are done, and the TC failed when the Internet PDU SESSION EST REQ was sent by the UE for a DNN mismatch. 2, At step 25, the SIB1 is modified for UAC Barring info without waiting modification period. |
| Summary of change | 1, Code change made to pass the DNN (tsc_DNNValue, i.e. "ABCD") in the function f_NR5GC_PDUSessionEstablishment( ) when it is called at step 10Fa1-10Fa8. 2, updated step 25 with f_NR_ModifySysinfo() call |
| TTCN module | UAC_NR5GC.ttcn. |
| MCC160 Comment |  |

Before Change

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell12); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_GutiParameters_Type v_GutiParams_NRCell1 := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI_NR_Cell1 := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_NRCell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (nr_Cell12); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var TypeOfRegistration_Type v_TypeOfRegistration; var float v_TimerValue1 := 10.0; timer t_WaitTime; timer t_Wait; ......... ......... if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0000'B)) { // @sic R5s230103 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); // @sic R5s221172 sic@ f_NR5GC_PDUSessionEstablishment (nr_Cell12,-,tsc_NR_RbId_SRB2); // @sic R5s230200 sic@ //@siclog "Steps 10H" siclog@ // @sic R5-223436 sic@ ......... ......... //@siclog "Step 24 siclog@ f_NR_RRCRelease(nr_Cell12); //Set SIB1 for NR cell 1 according to Table 11.3.5.3.3-1 for Step 25 and 35 f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(5,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '1111111'B))); // @sic R5-223436 sic@ //@siclog "Step 26" siclog@ //The SS adjusts the NR Cells power levels according to row "T2" in table 11.3.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T2); ......... ......... }

After Change

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell12); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_GutiParameters_Type v_GutiParams_NRCell1 := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI_NR_Cell1 := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_NRCell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (nr_Cell12); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var TypeOfRegistration_Type v_TypeOfRegistration; var float v_TimerValue1 := 10.0; timer t_WaitTime; timer t_Wait; var template (omit) DNN v_DNN; v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_DNNValue)); ......... ......... if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0000'B)) { // @sic R5s230103 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); // @sic R5s221172 sic@ f_NR5GC_PDUSessionEstablishment (nr_Cell12, 1,tsc_NR_RbId_SRB2, -, -, v_DNN, -, v_DNN); // @sic R5s230200 sic@ //@siclog "Steps 10H" siclog@ // @sic R5-223436 sic@ ......... ......... //@siclog "Step 24 siclog@ f_NR_RRCRelease(nr_Cell12); //Set SIB1 for NR cell 1 according to Table 11.3.5.3.3-1 for Step 25 and 35 f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(5,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '1111111'B))); // @sic R5-223436 sic@ f_NR_ModifySysinfo(nr_Cell1);//KS#23xxxx //@siclog "Step 26" siclog@ //The SS adjusts the NR Cells power levels according to row "T2" in table 11.3.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T2); ......... ......... }