3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230370

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3079 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC MAC test case 7.1.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE MAC is reset following reconfiguration with sync procedure, UE RLC is still in non reset mode and doesn’t flushes this DL data which remains in its buffer that has been scheduled by SS at step1. Using the grant configured during reconfiguration with sync procedure, UE may try to loop the same data back to NW. This eventually causes the TC failed due to unexpected data being received at SS. As per agreement with RAN5 group, we propose to absorb this looped back data after step4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In function f_TC_7_1_1_9_1_Step5to9A(), this data has been received as RLC pdu and handled by SS and subsequent status PDU is scheduled accordingly. - Following above change, we need to adjust the SN count for the status PDUs scheduled at step 9A and step 20A. - Cosmetic correction for siclog has been done. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.NR5GC, 7.1.1.9.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

2.3	Change 3	9

2.4	Change 4	12

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function f_TC_7_1_1_9_1_Step5to9A |
| --- | --- |
| Reason for change | UE MAC is reset following reconfiguration with sync procedure, UE RLC is still in non reset mode and doesn’t flushes this DL data which remains in its buffer that has been scheduled by SS at step1. Using the grant configured during reconfiguration with sync procedure, UE may try to loop the same data back to NW. This eventually causes the TC failed due to unexpected data being received at SS. Note: This change will be accompanied with prose change. |
| Summary of change | - This data has been received as MAC pdu including one RLC sdu and handled by SS. - Subsequent status PDU is scheduled accordingly by taking a Boolean variable IsDataRecvd which is set to TRUE in case UE loops step1 data back. This variable is used in subsequent function to update RLC status PDU sequence number. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_9_1_Step5to9A ( DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, boolean p_StopGrant:= true ) runs on NR_BASE_PTC { //@sic R5-206320 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var default v_IgnoreNRSR:= null; //@sic R5-204418 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@ // @siclog "Step 5" sigloc@ // void // @sic R5-213132 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList); //The SS gets prepared for SR indication and stops default UL grant allocation. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Step 5A" siclog@ //@sic R5-204418 sic@ //The SS ignores scheduling requests and does not allocate any uplink grant. v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5-204418 R5s220838 sic@ if(p_StopGrant){ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep);}//@sic R5s220838 sic@ //@siclog "Step 6" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 1. //PDCP SDU = (37B + 3B PDCP Header) + 3B RLC Header + 2B MAC Header = 48B = 384 bits (an allowed TB size as per 38.523-3 Annex B) f_NR_SS_CellConfig_DciDlInfo_ReOrNewtransmission(nr_Cell1,-,cs_NR_TransportBlockRetransmission(tsc_NR_DefaultGrant_Imcs), -, cs_HarqProcessConfig(5));//@sic R5s211710 sic@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1,crs_NR_PDCP_SDU_37B))); //@sic R5s201457 R5s210896 R5-227518 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID ,v_EncodedRlcPdu1)}, omit)})); //@sic R5s200855 R5s211710 sic@ // @siclog "Step 7 " siclog@ //Check: Does the UE transmit a Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7: UE sent Scheduling Request"); //@siclog "Step 8" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation, cs_HarqProcessConfig(5));//@sic R5s200855 R5s220687 sic@ //@siclog "Step 9" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})); //@siclog "Step 9A" siclog@ // The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 R5s220506 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); deactivate(v_IgnoreNRSR); //@sic R5-204418 R5s220687sic@ //@sic R5-213132 sic@ }

After Change

import from NR_RLC_Common all; …. …. function f_TC_7_1_1_9_1_Step5to9A ( DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, boolean p_StopGrant:= true ) runs on NR_BASE_PTC return boolean { //@sic R5-206320 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var default v_IgnoreNRSR:= null; //@sic R5-204418 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@ var boolean v_IsDataRecvd := false; var NR_RLC_SS_State_Type v_RLC_Rec; timer t_Watchdog; // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); v_RLC_Rec.AM_RX_Next := 1; // @siclog "Step4 added to absorb data for step1 in case UE loops back" siclog@ t_Watchdog.start(1.0); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, ?)) { f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id); // @siclog "UE has looped back step1 data" siclog@ v_IsDataRecvd := true; t_Watchdog.stop; } [] t_Watchdog.timeout { } } // @siclog "Step 5" siclog@ // void // @sic R5-213132 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList); //The SS gets prepared for SR indication and stops default UL grant allocation. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Step 5A" siclog@ //@sic R5-204418 sic@ //The SS ignores scheduling requests and does not allocate any uplink grant. v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5-204418 R5s220838 sic@ if(p_StopGrant){ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep);}//@sic R5s220838 sic@ //@siclog "Step 6" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 1. //PDCP SDU = (37B + 3B PDCP Header) + 3B RLC Header + 2B MAC Header = 48B = 384 bits (an allowed TB size as per 38.523-3 Annex B) f_NR_SS_CellConfig_DciDlInfo_ReOrNewtransmission(nr_Cell1,-,cs_NR_TransportBlockRetransmission(tsc_NR_DefaultGrant_Imcs), -, cs_HarqProcessConfig(5));//@sic R5s211710 sic@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (1,crs_NR_PDCP_SDU_37B))); //@sic R5s201457 R5s210896 R5-227518 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID ,v_EncodedRlcPdu1)}, omit)})); //@sic R5s200855 R5s211710 sic@ // @siclog "Step 7 " siclog@ //Check: Does the UE transmit a Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7: UE sent Scheduling Request"); //@siclog "Step 8" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation, cs_HarqProcessConfig(5));//@sic R5s200855 R5s220687 sic@ //@siclog "Step 9" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})); //@siclog "Step 9A" siclog@ // The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 R5s220506 sic@ if(v_IsDataRecvd) { f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 2, p_NR_DRB_Id); } else { f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); } deactivate(v_IgnoreNRSR); //@sic R5-204418 R5s220687sic@ //@sic R5-213132 sic@ return v_IsDataRecvd; }

## Change 2

| Function name | f_TC_7_1_1_9_1_Step16Ato21 |
| --- | --- |
| Reason for change | Status PDU need to be adjusted following change1. |
| Summary of change | Updated variable p_IsDataRecvd which is returned from function f_TC_7_1_1_9_1_Step5to9A is passed as an argument in f_TC_7_1_1_9_1_Step16Ato21 which is used to set updated SN for RLC status PDU scheduled at step 20A. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_9_1_Step16Ato21(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID) runs on NR_BASE_PTC { //@sic R5-206320 sic@ var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var default v_IgnoreNRSR:= null;//@sic R5-204418 sic@ //@sic R5s210896 R5s230261 sic@ // @siclog "Step 16 void" sigloc@ // @siclog "Step 16A" sigloc@ //@sic R5-204418 sic@ //The SS ignores scheduling requests and does not allocate any uplink grant. v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 17" siclog@ //PDCP SDU = (37B + 3B PDCP Header) + 3B RLC Header + 2B MAC Header = 48B = 384 bits (an allowed TB size as per 38.523-3 Annex B) //@sic R5s201457 R5s210896 R5s220687 sic@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(2,crs_NR_PDCP_SDU_37B))); //@sic R5-227518 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID,v_EncodedRlcPdu1)}, omit)},cs_NR_HarqProcessAssignment(1))); //@siclog "Step 18 " siclog@ //Indication of Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); //@siclog "Step 19" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1)); //@siclog "Step 20" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})) -> value v_NR_DRB_COMMON_IND; //@siclog "Step 20A" siclog@ //@sic R5s201457 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 2, p_NR_DRB_Id); // @sic R5s220015 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); }

After Change

function f_TC_7_1_1_9_1_Step16Ato21(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, boolean p_IsDataRecvd:= false) runs on NR_BASE_PTC { //@sic R5-206320 sic@ var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var default v_IgnoreNRSR:= null;//@sic R5-204418 sic@ //@sic R5s210896 R5s230261 sic@ // @siclog "Step 16 void" siclog@ // @siclog "Step 16A" siclog@ //@sic R5-204418 sic@ //The SS ignores scheduling requests and does not allocate any uplink grant. v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 17" siclog@ //PDCP SDU = (37B + 3B PDCP Header) + 3B RLC Header + 2B MAC Header = 48B = 384 bits (an allowed TB size as per 38.523-3 Annex B) //@sic R5s201457 R5s210896 R5s220687 sic@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(2,crs_NR_PDCP_SDU_37B))); //@sic R5-227518 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID,v_EncodedRlcPdu1)}, omit)},cs_NR_HarqProcessAssignment(1))); //@siclog "Step 18 " siclog@ //Indication of Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); //@siclog "Step 19" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1)); //@siclog "Step 20" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})) -> value v_NR_DRB_COMMON_IND; //@siclog "Step 20A" siclog@ //@sic R5s201457 sic@ if(p_IsDataRecvd) { f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 3, p_NR_DRB_Id); } else { f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 2, p_NR_DRB_Id);// @sic R5s220015 sic@ } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); }

## Change 3

| Function name | function fl_TC_7_1_1_9_1_ENDC_TestBody |
| --- | --- |
| Reason for change | Sequence number for status PDU scheduled at step 9A need to be corrected following change1. |
| Summary of change | Updated variable p_IsDataRecvd is returned from function f_TC_7_1_1_9_1_Step5to9A and passed as an argument in f_TC_7_1_1_9_1_Step16Ato21 which shall be used to set updated SN for RLC status PDU scheduled at step 20A. |
| TTCN module | MAC_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_7_1_1_9_1_ENDC_TestBody(DRB_Identity p_NR_DRB_Id) runs on ENDC_NR_PTC { var B6_Type v_LCID := tsc_LCID_SCG_Bearer; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var template (value) NR_RadioBearer_Type v_SS_DRB_Config := cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); var template (value) NR_RadioBearerList_Type v_SS_DrbConfigList_TestMode := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; var EUTRA_NR_Coordination_MSG v_Coordination_MSG_Value; var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var SubFrameTiming_Type v_Timing; f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); //Restrict the SS to use of the HARQ proccess id used by the test cases v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); //@sic R5s220805 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1,cs_TimingInfo_NR(v_Timing), cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(-,-,-,-,v_NR_HarqProcessConfig_DL), -, -)));//@sic R5s220669 sic@ //@siclog "Step 3" sigloc@ //SS transmits RRCConnectionReconfiguration message to E-UTRA PTC containing NR RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell //Send NR RRCreconfiguration message to E-UTRA PTC //Note: it is assumed that the timing can be achieved without timed sending //Set PDCP parameter discardTimer to 60ms v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly, -, true_); // @sic R5-197023 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1,{v_SS_DRB_Config});//@sic R5s200855 sic@ f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config, true, -, false); //@sic R5s200855 R5-213132 R5s210950 R5s220805 R5-227518 sic@ //f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config(-, omit), true, -, false); //EXCEPTION: Void //@sic R5-213132 R5-222044 sic@ EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value)); f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, v_SS_DrbConfigList_TestMode, false); //@sic R5s200855 R5s220838 sic@ f_TC_7_1_1_9_1_Step16Ato21(p_NR_DRB_Id,v_LCID);//@sic R5s200855 sic@ //@siclog "Step 22-23" siclog@ //The SS transmits NR RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell. //@sic R5s210950 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config}); f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config,true); //@sic R5-197023, R5s200855 R5-227518 sic@ //f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config( -, omit),true); f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,v_SS_DrbConfigList_TestMode);//@sic R5s200855 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config}); //@sic R5s220805 sic@ };

After Change

function fl_TC_7_1_1_9_1_ENDC_TestBody(DRB_Identity p_NR_DRB_Id) runs on ENDC_NR_PTC { var B6_Type v_LCID := tsc_LCID_SCG_Bearer; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var template (value) NR_RadioBearer_Type v_SS_DRB_Config := cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); var template (value) NR_RadioBearerList_Type v_SS_DrbConfigList_TestMode := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; var EUTRA_NR_Coordination_MSG v_Coordination_MSG_Value; var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var SubFrameTiming_Type v_Timing; var boolean v_IsDataRecvd := false; f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); //Restrict the SS to use of the HARQ proccess id used by the test cases v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); //@sic R5s220805 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1,cs_TimingInfo_NR(v_Timing), cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(-,-,-,-,v_NR_HarqProcessConfig_DL), -, -)));//@sic R5s220669 sic@ //@siclog "Step 3" siclog@ //SS transmits RRCConnectionReconfiguration message to E-UTRA PTC containing NR RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell //Send NR RRCreconfiguration message to E-UTRA PTC //Note: it is assumed that the timing can be achieved without timed sending //Set PDCP parameter discardTimer to 60ms v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly, -, true_); // @sic R5-197023 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1,{v_SS_DRB_Config});//@sic R5s200855 sic@ f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config, true, -, false); //@sic R5s200855 R5-213132 R5s210950 R5s220805 R5-227518 sic@ EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value)); v_IsDataRecvd := f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, v_SS_DrbConfigList_TestMode, false); //@sic R5s200855 R5s220838 sic@ f_TC_7_1_1_9_1_Step16Ato21(p_NR_DRB_Id,v_LCID,v_IsDataRecvd);//@sic R5s200855 sic@ //@siclog "Step 22-23" siclog@ //The SS transmits NR RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell. //@sic R5s210950 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config}); f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config,true); //@sic R5-197023, R5s200855 R5-227518 sic@ f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,v_SS_DrbConfigList_TestMode);//@sic R5s200855 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config}); //@sic R5s220805 sic@ };

## Change 4

| Function name | function fl_TC_7_1_1_9_1_NR_TestBody |
| --- | --- |
| Reason for change | Sequence number for status PDU scheduled at step 20A need to be corrected following change1. |
| Summary of change | Updated variable p_IsDataRecvd is returned from function f_TC_7_1_1_9_1_Step5to9A and passed as an argument in f_TC_7_1_1_9_1_Step16Ato21 which shall be used to set updated SN for RLC status PDU scheduled at step 20A. |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList; //@sic R5s201457 sic@ var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var template (value) CellGroupConfig v_CellGroupConfig; //@sic R5S210138 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_UL := { SpecificSubset := {0,2,3,4,6,7 }}; //@sic R5s220669 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }}; //@sic R5s220669 sic@ var template (value) NR_DciUlInfo_Type v_DciUlInfo := cs_NR_DciUlInfo_AllOmit; var SubFrameTiming_Type v_Timing; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); //@sic R5s221105 R5-227418 sic@ v_SS_Drb_ConfigList[0]:= cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_PollRetransmit, -, -, -, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_Reassembly, -, true_); v_NR_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //@sic R5s201457 sic@ v_NR_RLC_BearerToAddModList[2] := v_RLC_BearerConfigList[0]; //@sic R5s201457 sic@ //Initialise CellGroupConfig before sending data to UE to avoid additional delay v_CellGroupConfig := f_NR_GetCellGroupConfigHO(nr_Cell1, v_NR_RLC_BearerToAddModList); //@sic R5s210138 sic@ f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(-,-,-,-,v_NR_HarqProcessConfig_DL) , -, -))); //@sic R5s220669 sic@ //@siclog "Steps 3-4a" siclog@ //@sic R5s210950 R5s220795 R5-231905 sic@ f_NR_RLC_ReconfigurationHO_Steps1_8(nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, omit, omit), v_SS_Drb_ConfigList, true, v_CellGroupConfig, true);//@sic R5-213132 sic@ // @siclog "Step 4" sigloc@ SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));// @sic R5-222044 sic@ // @siclog "Step 4a - Void" // Steps left from HO: f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10 // Step 9 (after step 8) Re-configure RACH procedure as for initial access, enable TA transmissions. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); // enable TA transmissions f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); // @siclog "Steps 5-9A" siclog@ f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, p_NR_RadioBearerList); //@siclog "Steps 16A-21" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell f_TC_7_1_1_9_1_Step16Ato21(p_NR_DRB_Id, v_LCID); //@siclog "Steps 22" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell //@sic R5s210950 sic@ v_DciUlInfo.ResoureAssignment := cs_NR_DciFormat_0_X_ResourceAssignment(-, -, -, -,v_NR_HarqProcessConfig_UL);//@sic R5s220669 sic@ f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, v_DciUlInfo, -, -); f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, omit, omit), v_SS_Drb_ConfigList,true, v_CellGroupConfig, true); //@sic R5-204418 R5s210138 R5-227518 R5-23231905 sic@ f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,p_NR_RadioBearerList,v_NR_HarqProcessConfig_UL); //Reconfiguring MAC to normal mode //@sic R5s220687 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_Drb_ConfigList[0]}); }

After Change

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList; //@sic R5s201457 sic@ var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var template (value) CellGroupConfig v_CellGroupConfig; //@sic R5S210138 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_UL := { SpecificSubset := {0,2,3,4,6,7 }}; //@sic R5s220669 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }}; //@sic R5s220669 sic@ var template (value) NR_DciUlInfo_Type v_DciUlInfo := cs_NR_DciUlInfo_AllOmit; var SubFrameTiming_Type v_Timing; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); //@sic R5s221105 R5-227418 sic@ var boolean v_IsDataRecvd := false; v_SS_Drb_ConfigList[0]:= cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_PollRetransmit, -, -, -, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_Reassembly, -, true_); v_NR_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //@sic R5s201457 sic@ v_NR_RLC_BearerToAddModList[2] := v_RLC_BearerConfigList[0]; //@sic R5s201457 sic@ //Initialise CellGroupConfig before sending data to UE to avoid additional delay v_CellGroupConfig := f_NR_GetCellGroupConfigHO(nr_Cell1, v_NR_RLC_BearerToAddModList); //@sic R5s210138 sic@ f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(-,-,-,-,v_NR_HarqProcessConfig_DL) , -, -))); //@sic R5s220669 sic@ //@siclog "Steps 3-4a" siclog@ //@sic R5s210950 R5s220795 R5-231905 sic@ f_NR_RLC_ReconfigurationHO_Steps1_8(nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, omit, omit), v_SS_Drb_ConfigList, true, v_CellGroupConfig, true);//@sic R5-213132 sic@ // @siclog "Step 4" sigloc@ SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));// @sic R5-222044 sic@ // @siclog "Step 4a - Void" // Steps left from HO: f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10 // Step 9 (after step 8) Re-configure RACH procedure as for initial access, enable TA transmissions. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); // enable TA transmissions f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); // @siclog "Steps 5-9A" siclog@ v_IsDataRecvd := f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, p_NR_RadioBearerList); //@siclog "Steps 16A-21" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell f_TC_7_1_1_9_1_Step16Ato21(p_NR_DRB_Id, v_LCID, v_IsDataRecvd); //@siclog "Steps 22" siclog@ //SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell //@sic R5s210950 sic@ v_DciUlInfo.ResoureAssignment := cs_NR_DciFormat_0_X_ResourceAssignment(-, -, -, -,v_NR_HarqProcessConfig_UL);//@sic R5s220669 sic@ f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, v_DciUlInfo, -, -); f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, omit, omit), v_SS_Drb_ConfigList,true, v_CellGroupConfig, true); //@sic R5-204418 R5s210138 R5-227518 R5-23231905 sic@ f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,p_NR_RadioBearerList,v_NR_HarqProcessConfig_UL); //Reconfiguring MAC to normal mode //@sic R5s220687 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_Drb_ConfigList[0]}); }