3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230373

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4732 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE-IRAT test case 11.2.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2023-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the test case prose( based on R5-213131), the test shall be executed with the ciphering of utran set to uea0, currently the test relies on the pixit px_UTRAN_CipheringAlgorithm to be set correctly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified the function f_UTRAN_Init to include uea0 as a parameter input, so that uea0 can be used as the ciphering algorithm. - Modified the function f_UTRAN_Init to have p_CipheringAlgorithm_r7 as parameter to specify a different value other than px_UTRAN_CipheringAlgorithm (for test case specific values) Updated the code in f_UTRAN_SecurityInfo_Init to set the cipheringOnOff based on uea0 for both TDD and FDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required to test case 11.2.11	2

2.1	f_TC_11_2_11_UTRAN	2

2.2	f_UTRAN_Init	4

2.3	f_UTRAN_SecurityInfo_Init	7

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required to test case 11.2.11

## f_TC_11_2_11_UTRAN

| Template name | f_TC_11_2_11_UTRAN |
| --- | --- |
| Reason for change | According to the test case prose ( based on R5-213131), the test shall be executed with the ciphering of utran set to uea0, currently the test relies on the pixit px_UTRAN_CipheringAlgorithm to be set correctly. |
| Summary of change | Modified the function f_UTRAN_Init to include uea0 as a parameter input, so that uea0 can be used as the ciphering algorithm. |
| TTCN module | IMS_EmergencyCall_UGC_UTRAN |
| MCC160 Comment |  |

Before Change:

function f_TC_11_2_11_UTRAN() runs on UTRAN_IRAT_PTC { var integer v_T0_CPICH_Cell5 := tsc_AttenuationCellOFF; // @sic R5s140949 sic@ var integer v_T1_CPICH_Cell5 := -64; var integer v_T0_PCCPCH_Cell5 := tsc_AttenuationCellOFF; // @sic R5s140949 sic@ var integer v_T1_PCCPCH_Cell5 := -64; // Initialise all cells f_UTRAN_Init(EUTRA_UTRAN); // Create and configure Cell 5 f_UTRAN_SS_CreateCellDCH(utran_Cell5); // Start sending System Information f_UTRAN_SendDefSysInfo(utran_Cell5);

After Change:

function f_TC_11_2_11_UTRAN() runs on UTRAN_IRAT_PTC { var integer v_T0_CPICH_Cell5 := tsc_AttenuationCellOFF; // @sic R5s140949 sic@ var integer v_T1_CPICH_Cell5 := -64; var integer v_T0_PCCPCH_Cell5 := tsc_AttenuationCellOFF; // @sic R5s140949 sic@ var integer v_T1_PCCPCH_Cell5 := -64; // Initialise all cells f_UTRAN_Init(EUTRA_UTRAN,uea0); // added uea0, as the test case shall be executed with uea0 // Create and configure Cell 5 f_UTRAN_SS_CreateCellDCH(utran_Cell5); // Start sending System Information f_UTRAN_SendDefSysInfo(utran_Cell5);

## f_UTRAN_Init

| Template name | f_UTRAN_Init |
| --- | --- |
| Reason for change | According to the test case prose, the test shall be executed with the ciphering of utran set to uea0, currently the test relies on the pixit px_UTRAN_CipheringAlgorithm to be set correctly. |
| Summary of change | Modified the function to have p_CipheringAlgorithm_r7 as parameter to specify a different value other than px_UTRAN_CipheringAlgorithm (for test case specific values) |
| TTCN module | UTRAN_IRAT_CommonProcedures |
| MCC160 Comment |  |

Before Change:

function f_UTRAN_Init(IRAT_ConfiguredPTCs_Type p_ConfiguredPTCs := EUTRA_UTRAN) runs on UTRAN_IRAT_PTC { /* @sic R5-113805, R5-113696, R5-113734 sic@ */ /* f_UTRAN_Init is used as dummy function (without parameters) for the UTRAN PTC when the UTRAN PTC is needed just to get the UTRAN system information */ var UTRAN_CellIdList_Type v_CellIdList := {utran_Cell5, utran_Cell7, utran_Cell8, utran_Cell9}; var integer v_Index; var integer i; vc_UTRAN_Global.Ctrl.Default := null; vc_UTRAN_Global.Ctrl.TestBody := false; vc_UTRAN_Global.CellArray := f_UTRAN_CellArray_Init(); vc_UTRAN_Global.MobileInfo.ue_EUTRA_Capability := omit; // @sic R5s120641 sic@ vc_UTRAN_Global.MobileInfo.ue_OperationMode := omit; vc_UTRAN_Global.MobileInfo.ue_Release := rel_8; //Default value, will be updated upon receipt of RRC CONNECTION REQUEST // Initialise ConfigurationInfo vc_UTRAN_Global.ConfigurationInfo.PdcpConfiguration := false; // @sic R5s130195 PDCP sic@ vc_UTRAN_Global.ConfigurationInfo.TestCaseRelease := f_GetDefaultTestCaseRelease(); // @sic R5-130195 Utran Release sic@ vc_UTRAN_Global_IP.NoOfPDPContexts := 0; // @sic R5-160730 sic@ vc_UTRAN_Global_IP.PDPInfo := omit; // Now update sys info with inter RAT parameters for all cells select (p_ConfiguredPTCs) { case (EUTRA_UTRAN) { vc_UTRAN_Global.CellArray := f_UTRAN_IR_SIB19_Init(vc_UTRAN_Global.CellArray, f_IRAT_InitialCoOrd(EUTRA, f_UTRAN_IR_SysInfoInit(vc_UTRAN_Global.CellArray))); } case (EUTRA_UTRAN_GERAN) { vc_UTRAN_Global.CellArray := f_UTRAN_IR_SIB11And19_Init(vc_UTRAN_Global.CellArray, f_IRAT_InitialCoOrd_2Ports(EUTRA, GERAN, f_UTRAN_IR_SysInfoInit(vc_UTRAN_Global.CellArray))); for (i := 0; i < lengthof(v_CellIdList); i := i + 1) { // @sic R5s130433 sic@ v_Index := f_UTRAN_CellInfo_GetIndex(v_CellIdList[i]); f_UTRAN_SysInfo_SetMIB_SetSIB5bisType(vc_UTRAN_Global.CellArray[v_Index].cellId, cs_MIB_UTRAN_GERAN_Def(vc_UTRAN_Global.CellArray[i].plmnId)); //@sic R5s150943 sic@ f_UTRAN_SysInfo_SetSB1(vc_UTRAN_Global.CellArray[v_Index].cellId, cs_SB1_UTRAN_GERAN_Def); } } } //The first security initialisation is done with px_UTRAN_CipheringAlgorithm @sic R5-150124 sic@ vc_UTRAN_Global.Security := f_UTRAN_SecurityInfo_Init(px_UTRAN_CipheringAlgorithm); vc_UTRAN_Global.MsgInDefault := f_UTRAN_MsgInDefault_Init(); f_UTRAN_ActivateDefault(StandardDefault); }

After Change:

function f_UTRAN_Init(IRAT_ConfiguredPTCs_Type p_ConfiguredPTCs := EUTRA_UTRAN, template (omit) CipheringAlgorithm_r7 p_CipheringAlgorithm_r7 := px_UTRAN_CipheringAlgorithm) runs on UTRAN_IRAT_PTC //WA#WI=1088540 added parameter { /* @sic R5-113805, R5-113696, R5-113734 sic@ */ /* f_UTRAN_Init is used as dummy function (without parameters) for the UTRAN PTC when the UTRAN PTC is needed just to get the UTRAN system information */ var UTRAN_CellIdList_Type v_CellIdList := {utran_Cell5, utran_Cell7, utran_Cell8, utran_Cell9}; var integer v_Index; var integer i; vc_UTRAN_Global.Ctrl.Default := null; vc_UTRAN_Global.Ctrl.TestBody := false; vc_UTRAN_Global.CellArray := f_UTRAN_CellArray_Init(); vc_UTRAN_Global.MobileInfo.ue_EUTRA_Capability := omit; // @sic R5s120641 sic@ vc_UTRAN_Global.MobileInfo.ue_OperationMode := omit; vc_UTRAN_Global.MobileInfo.ue_Release := rel_8; //Default value, will be updated upon receipt of RRC CONNECTION REQUEST // Initialise ConfigurationInfo vc_UTRAN_Global.ConfigurationInfo.PdcpConfiguration := false; // @sic R5s130195 PDCP sic@ vc_UTRAN_Global.ConfigurationInfo.TestCaseRelease := f_GetDefaultTestCaseRelease(); // @sic R5-130195 Utran Release sic@ vc_UTRAN_Global_IP.NoOfPDPContexts := 0; // @sic R5-160730 sic@ vc_UTRAN_Global_IP.PDPInfo := omit; // Now update sys info with inter RAT parameters for all cells select (p_ConfiguredPTCs) { case (EUTRA_UTRAN) { vc_UTRAN_Global.CellArray := f_UTRAN_IR_SIB19_Init(vc_UTRAN_Global.CellArray, f_IRAT_InitialCoOrd(EUTRA, f_UTRAN_IR_SysInfoInit(vc_UTRAN_Global.CellArray))); } case (EUTRA_UTRAN_GERAN) { vc_UTRAN_Global.CellArray := f_UTRAN_IR_SIB11And19_Init(vc_UTRAN_Global.CellArray, f_IRAT_InitialCoOrd_2Ports(EUTRA, GERAN, f_UTRAN_IR_SysInfoInit(vc_UTRAN_Global.CellArray))); for (i := 0; i < lengthof(v_CellIdList); i := i + 1) { // @sic R5s130433 sic@ v_Index := f_UTRAN_CellInfo_GetIndex(v_CellIdList[i]); f_UTRAN_SysInfo_SetMIB_SetSIB5bisType(vc_UTRAN_Global.CellArray[v_Index].cellId, cs_MIB_UTRAN_GERAN_Def(vc_UTRAN_Global.CellArray[i].plmnId)); //@sic R5s150943 sic@ f_UTRAN_SysInfo_SetSB1(vc_UTRAN_Global.CellArray[v_Index].cellId, cs_SB1_UTRAN_GERAN_Def); } } } //The first security initialisation is done with px_UTRAN_CipheringAlgorithm @sic R5-150124 sic@ vc_UTRAN_Global.Security := f_UTRAN_SecurityInfo_Init(p_CipheringAlgorithm_r7); vc_UTRAN_Global.MsgInDefault := f_UTRAN_MsgInDefault_Init(); f_UTRAN_ActivateDefault(StandardDefault); }

## f_UTRAN_SecurityInfo_Init

| Template name | f_UTRAN_SecurityInfo_Init |
| --- | --- |
| Reason for change | When UEA0 is set the v_SecurityInfo.cipheringOnOff needs to be set to false (similar to TDD) |
| Summary of change | Updated the code to set the cipheringOnOff based on uea0 for both TDD and FDD |
| TTCN module | UTRAN_CellInfo |
| MCC160 Comment |  |

Before Change:

function f_UTRAN_SecurityInfo_Init(template (omit) CipheringAlgorithm_r7 p_CipheringAlgorithm_r7 := omit) runs on UTRAN_PTC return UTRAN_SecurityInfo_Type { var template (value) UTRAN_SecurityInfo_Type v_SecurityInfo; var CipheringAlgorithm_r7 v_CipheringAlgorithm_r7; // Check that for UTRAN TDD, px_UTRAN_CipheringAlgorithm=uea0 // @sic R5-123750 R5-150124 sic@ if (isvalue(p_CipheringAlgorithm_r7)){ v_CipheringAlgorithm_r7 := valueof(p_CipheringAlgorithm_r7); if ((px_UTRAN_ModeUnderTest == UTRAN_TDD) and (v_CipheringAlgorithm_r7 != uea0)) { f_UTRAN_SetVerdictFailOrInconc(__FILE__, __LINE__, "px_UTRAN_CipheringAlgorithm not set to uea0 in UTRAN TDD"); } else { v_SecurityInfo.cipheringAlgorithm := v_CipheringAlgorithm_r7; v_SecurityInfo.cipheringAlgorithm_r99 := omit; if (v_CipheringAlgorithm_r7 == uea2) { v_SecurityInfo.integrityProtectionAlgorithm := uia2; } else { v_SecurityInfo.integrityProtectionAlgorithm := uia1; } v_SecurityInfo.integrityProtectionAlgorithm_r99 := uia1; } } else { v_SecurityInfo := f_UTRAN_Security_Get(); } // In TDD uea0 is used: cipheringOnoff is set to false for SS // @sic R5s130116 sic@ if (px_UTRAN_ModeUnderTest == UTRAN_TDD) { v_SecurityInfo.cipheringOnOff := false; } else { v_SecurityInfo.cipheringOnOff := true; } v_SecurityInfo.cs_cipheringStarted := false; v_SecurityInfo.ps_cipheringStarted := false; v_SecurityInfo.recentSecureDomain := ps_domain; // the domain on which security was recently started, and hence the SRB are ciphered and Integrit protected with this domain. v_SecurityInfo.dL_CipherMode := cs_CipheringModeCmdOn(v_SecurityInfo.cipheringAlgorithm); v_SecurityInfo.dL_CipherMode_r99 := cs_Null_CipheringModeCommand; // @sic R5s120904 sic@ v_SecurityInfo.uL_CipherMode := omit; v_SecurityInfo.cipheringAlgorithmCapability := '0000000000000111'B; v_SecurityInfo.integrityStarted := false; v_SecurityInfo.dL_Integrity := cs_IntegrityProtectStart(px_FRESH, v_SecurityInfo.integrityProtectionAlgorithm); v_SecurityInfo.dL_Integrity_r99 := cs_IntegrityProtectStart_r99(px_FRESH, v_SecurityInfo.integrityProtectionAlgorithm_r99); // @sic R5s120904 sic@ v_SecurityInfo.uL_Integrity := omit; v_SecurityInfo.dl_IntegrityCheckInfo := omit; v_SecurityInfo.integrProtAlgCap := '0000000000000110'B; //@sic R5s130137 sic@ v_SecurityInfo.start_CS := '00000000000000000000'B; // Default new key v_SecurityInfo.start_PS := '00000000000000000000'B; // Default new key v_SecurityInfo.authKeys.AuthPS := cs_CommonAuthParams_Init(px_eAuthRAND); v_SecurityInfo.authKeys.HFN := int2bit (0, 20); v_SecurityInfo.authKeys.AuthCS.CKey := tsc_AuthUndefinedB128; v_SecurityInfo.authKeys.AuthCS.IKey := tsc_AuthUndefinedB128; v_SecurityInfo.authKeys.AuthCS.CKeySeq := '111'B; v_SecurityInfo.authKeys.CSinOtherRAT := false; // @sic R5s110168 change 8.6 sic@ v_SecurityInfo.authKeys.IMS_Registered_WithExtraPDP := false; // @sic R5-163054 sic@ return valueof(v_SecurityInfo); }

After Change:

function f_UTRAN_SecurityInfo_Init(template (omit) CipheringAlgorithm_r7 p_CipheringAlgorithm_r7 := omit) runs on UTRAN_PTC return UTRAN_SecurityInfo_Type { var template (value) UTRAN_SecurityInfo_Type v_SecurityInfo; var CipheringAlgorithm_r7 v_CipheringAlgorithm_r7; // Check that for UTRAN TDD, px_UTRAN_CipheringAlgorithm=uea0 // @sic R5-123750 R5-150124 sic@ if (isvalue(p_CipheringAlgorithm_r7)){ v_CipheringAlgorithm_r7 := valueof(p_CipheringAlgorithm_r7); if (v_CipheringAlgorithm_r7 == uea0) { // added uea0 to set cipheringOnOff to false v_SecurityInfo.cipheringOnOff := false;} else { v_SecurityInfo.cipheringOnOff := true; } if ((px_UTRAN_ModeUnderTest == UTRAN_TDD) and (v_CipheringAlgorithm_r7 != uea0)) { f_UTRAN_SetVerdictFailOrInconc(__FILE__, __LINE__, "px_UTRAN_CipheringAlgorithm not set to uea0 in UTRAN TDD"); } else { v_SecurityInfo.cipheringAlgorithm := v_CipheringAlgorithm_r7; v_SecurityInfo.cipheringAlgorithm_r99 := omit; if (v_CipheringAlgorithm_r7 == uea2) { v_SecurityInfo.integrityProtectionAlgorithm := uia2; } else { v_SecurityInfo.integrityProtectionAlgorithm := uia1; } v_SecurityInfo.integrityProtectionAlgorithm_r99 := uia1; } } else { v_SecurityInfo := f_UTRAN_Security_Get(); } // In TDD uea0 is used: cipheringOnoff is set to false for SS //WA#WI=1088540 moved the check above based on UEA0 // @sic R5s130116 sic@ /*if (px_UTRAN_ModeUnderTest == UTRAN_TDD) { v_SecurityInfo.cipheringOnOff := false; } else { v_SecurityInfo.cipheringOnOff := true; }*/ v_SecurityInfo.cs_cipheringStarted := false; v_SecurityInfo.ps_cipheringStarted := false; v_SecurityInfo.recentSecureDomain := ps_domain; // the domain on which security was recently started, and hence the SRB are ciphered and Integrit protected with this domain. v_SecurityInfo.dL_CipherMode := cs_CipheringModeCmdOn(v_SecurityInfo.cipheringAlgorithm); v_SecurityInfo.dL_CipherMode_r99 := cs_Null_CipheringModeCommand; // @sic R5s120904 sic@ v_SecurityInfo.uL_CipherMode := omit; v_SecurityInfo.cipheringAlgorithmCapability := '0000000000000111'B; v_SecurityInfo.integrityStarted := false; v_SecurityInfo.dL_Integrity := cs_IntegrityProtectStart(px_FRESH, v_SecurityInfo.integrityProtectionAlgorithm); v_SecurityInfo.dL_Integrity_r99 := cs_IntegrityProtectStart_r99(px_FRESH, v_SecurityInfo.integrityProtectionAlgorithm_r99); // @sic R5s120904 sic@ v_SecurityInfo.uL_Integrity := omit; v_SecurityInfo.dl_IntegrityCheckInfo := omit; v_SecurityInfo.integrProtAlgCap := '0000000000000110'B; //@sic R5s130137 sic@ v_SecurityInfo.start_CS := '00000000000000000000'B; // Default new key v_SecurityInfo.start_PS := '00000000000000000000'B; // Default new key v_SecurityInfo.authKeys.AuthPS := cs_CommonAuthParams_Init(px_eAuthRAND); v_SecurityInfo.authKeys.HFN := int2bit (0, 20); v_SecurityInfo.authKeys.AuthCS.CKey := tsc_AuthUndefinedB128; v_SecurityInfo.authKeys.AuthCS.IKey := tsc_AuthUndefinedB128; v_SecurityInfo.authKeys.AuthCS.CKeySeq := '111'B; v_SecurityInfo.authKeys.CSinOtherRAT := false; // @sic R5s110168 change 8.6 sic@ v_SecurityInfo.authKeys.IMS_Registered_WithExtraPDP := false; // @sic R5-163054 sic@ return valueof(v_SecurityInfo); }