3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230381

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3085 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Inter-RAT Test case 6.2.3.4.NR5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - This test case is unstable during step 6 due to high noise power in comparision with PDSCH power as NR Cell1’s power level of PDSCH(-83dBm/SCS) and Noise level((-85dBm/SCS) quite close. Even though SSS PBCH EPRE is -80dBm/SCS, but PDSCH would be 3dB lower than SSS PBCH EPRE ie -83dBm/SCS as per Table 6.2.1.2-1, TS38.508-1. To improve the stability, it is proposed to increase NR Cell’1 PBCH power level by 3dB. #draft proseCR would be raised during RAN5#99 - Initial max reference power needs to be modied due to change propose in change 1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adjust SS/PBCH SSS EPRE level for NR Cell 1 from -80 to -77 - Adjust Initial Max Reference Power of NR CELL1 from -80 to -77 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:                      Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_6_2_3_4_TestBody() |
| --- | --- |
| Reason for change | This test case is unstable during step 6 due to high noise power in comparision with PDSCH power as NR Cell1’s power level of PDSCH(-83dBm/SCS) and Noise level((-85dBm/SCS) quite close. Even though SSS PBCH EPRE is -80dBm/SCS, but PDSCH would be 3dB lower than SSS PBCH EPRE ie -83dBm/SCS as per Table 6.2.1.2-1, TS38.508-1. To improve the stability, it is proposed to increase NR Cell’1 PBCH power level by 3dB. #draft proseCR would be raised during RAN5#99 |
| Summary of change | Adjust SS/PBCH SSS EPRE level for NR Cell 1 from -80 to -77. |
| TTCN module | Idle_CellReSelection_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_6_2_3_4_TestBody() runs on NR5GC_PTC { var integer v_Noc := -85; // @sic R5s200723 sic@ //@siclog "Step 1" siclog@ //The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T1" in table 6.2.3.4.3.2-1 f_NR_SetCellPower (nr_Cell1, -95, -95); f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell1, v_Noc)}); // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA); // wait until power level change on Step4 //@siclog "Step 4" siclog@ // The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T2" in table 6.2.3.4.3.2-1 f_NR_SetCellPower (nr_Cell1, -80, -80); // @siclog "Step 5" siclog@ // The UE selects NR Cell 1 and performs the generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 with condition 'connected without release'. f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, noRrcConnectionRelease, false); // @siclog "Step 6" siclog@ // The SS transmits an RRCRelease message on NR Cell 1. f_NR_RRCRelease(nr_Cell1); // @sic R5s210276 sic@ // @siclog "Step 7" siclog@ // The SS changes the E-UTRA cell priority broadcast in system information on NR Cell 1. f_NR5GC_CellInfo_SetSIB5_CellReselectionPriority(nr_Cell1, 0, 5); // Table 6.2.3.4.3.3-7: SIB5 NR Cell 1 f_NR_CellInfo_SetSIB5_ThreshXHighQ(nr_Cell1, 0, cs_InterFreqCarrierFreqInfo_ThreshX_Q(12, 20)); // @sic R5-213455 sic@ f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); // @siclog "Step 8" siclog@ // Notify UE change of System Information on NR Cell 1. // Table 6.2.3.4.3.3-8: SIB1 of NR Cell 1 f_NR_IncrementValueTag(nr_Cell1, sibType5); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 9" siclog@ // Wait for 6 s for UE to receive system information. f_Delay(6.0); // @siclog "Step 10" siclog@ // The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T3" in table 6.2.3.4.3.2-1 f_NR_SetCellPower (nr_Cell1, -95, -95); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); // send coordination to move context to EUTRA // @siclog "Step 11" siclog@ // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); }

After Change

function fl_TC_6_2_3_4_TestBody() runs on NR5GC_PTC { var integer v_Noc := -85; // @sic R5s200723 sic@ //@siclog "Step 1" siclog@ //The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T1" in table 6.2.3.4.3.2-1 f_NR_SetCellPower (nr_Cell1, -95, -95); f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell1, v_Noc)}); // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA); // wait until power level change on Step4 //@siclog "Step 4" siclog@ // The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T2" in table 6.2.3.4.3.2-1 f_NR_SetCellPower (nr_Cell1, -77, -77); // @siclog "Step 5" siclog@ // The UE selects NR Cell 1 and performs the generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 with condition 'connected without release'. f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, noRrcConnectionRelease, false); // @siclog "Step 6" siclog@ // The SS transmits an RRCRelease message on NR Cell 1. f_NR_RRCRelease(nr_Cell1); // @sic R5s210276 sic@ // @siclog "Step 7" siclog@ // The SS changes the E-UTRA cell priority broadcast in system information on NR Cell 1. f_NR5GC_CellInfo_SetSIB5_CellReselectionPriority(nr_Cell1, 0, 5); // Table 6.2.3.4.3.3-7: SIB5 NR Cell 1 f_NR_CellInfo_SetSIB5_ThreshXHighQ(nr_Cell1, 0, cs_InterFreqCarrierFreqInfo_ThreshX_Q(12, 20)); // @sic R5-213455 sic@ f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); // @siclog "Step 8" siclog@ // Notify UE change of System Information on NR Cell 1. // Table 6.2.3.4.3.3-8: SIB1 of NR Cell 1 f_NR_IncrementValueTag(nr_Cell1, sibType5); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 9" siclog@ // Wait for 6 s for UE to receive system information. f_Delay(6.0); // @siclog "Step 10" siclog@ // The SS changes SS/PBCH SSS EPRE level for NR Cell 1 and Cell-specific RS EPRE level for E-UTRA Cell 1 according to row "T3" in table 6.2.3.4.3.2-1 f_NR_SetCellPower (nr_Cell1, -95, -95); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); // send coordination to move context to EUTRA // @siclog "Step 11" siclog@ // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); }

## Change 2

| Function name | f_TC_6_2_3_4_NR5GC() |
| --- | --- |
| Reason for change | Initial max reference power needs to be modied due to change propose in change 1. |
| Summary of change | Adjust Initial Max Reference Power of NR CELL1 from -80 to -77. |
| TTCN module | Idle_CellReSelection_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_6_2_3_4_NR5GC() runs on NR5GC_PTC { // Inter-RAT cell reselection / From NR RRC_Idle to E-UTRA_IDLE (lower priority & higher priority, Squal based) // @sic R5-210632 sic@ var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var boolean v_IsFR1; // @sic R5-225372 sic@ // System information combination NR-6 as defined in TS 38.508-1 clause 4.4.3.1, and message contents defined in clause 4.6.1 and clause 4.6.2 with QBASED condition is used in NR cell. f_NR5GC_IRAT_Init(NR_6); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -80, -80); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); //NR Cell 1 is configured to operate in FR1 bands as defined in TS 38.508-1 clause 6.2.3. if (not v_IsFR1) { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.2.3.4: NR Cell 1 shall operate in FR1 bands"); } v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); // E-UTRA Cell 1 information Rx. // Table 6.2.3.4.3.3-1:SIB1 for NR Cell 1 f_NR_CellInfo_SetSIB1_CellSelectionInfo(nr_Cell1, cs_38508_SIB1_CellSelectionInfo(-55,-,-25)); // Table 6.2.3.4.3.3-2:SIB2 for NR Cell 1 f_NR_CellInfo_SetSIB2_ThreshServingLowQ(nr_Cell1, 9); // @sic R5-213455 sic@ // Table 6.2.3.4.3.3-3:SIB5 NR Cell 1 f_NR_CellInfo_SetSIB5_CarrierFreq(nr_Cell1, 0, v_CoOrd_SysInfo.Eutra[0].Arfcn); f_NR_CellInfo_SetSIB5_ThreshXHighQ(nr_Cell1, 0, cs_InterFreqCarrierFreqInfo_ThreshX_Q(20, 10)); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_IRAT_PreambleEUTRA_NR (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_6_2_3_4_TestBody(); f_NR_TestBody_Set(false); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); // UE now on EUTRA f_NR_ReleaseAllCells(); }

After Change

function f_TC_6_2_3_4_NR5GC() runs on NR5GC_PTC { // Inter-RAT cell reselection / From NR RRC_Idle to E-UTRA_IDLE (lower priority & higher priority, Squal based) // @sic R5-210632 sic@ var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var boolean v_IsFR1; // @sic R5-225372 sic@ // System information combination NR-6 as defined in TS 38.508-1 clause 4.4.3.1, and message contents defined in clause 4.6.1 and clause 4.6.2 with QBASED condition is used in NR cell. f_NR5GC_IRAT_Init(NR_6); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -77, -77); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); //NR Cell 1 is configured to operate in FR1 bands as defined in TS 38.508-1 clause 6.2.3. if (not v_IsFR1) { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.2.3.4: NR Cell 1 shall operate in FR1 bands"); } v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); // E-UTRA Cell 1 information Rx. // Table 6.2.3.4.3.3-1:SIB1 for NR Cell 1 f_NR_CellInfo_SetSIB1_CellSelectionInfo(nr_Cell1, cs_38508_SIB1_CellSelectionInfo(-55,-,-25)); // Table 6.2.3.4.3.3-2:SIB2 for NR Cell 1 f_NR_CellInfo_SetSIB2_ThreshServingLowQ(nr_Cell1, 9); // @sic R5-213455 sic@ // Table 6.2.3.4.3.3-3:SIB5 NR Cell 1 f_NR_CellInfo_SetSIB5_CarrierFreq(nr_Cell1, 0, v_CoOrd_SysInfo.Eutra[0].Arfcn); f_NR_CellInfo_SetSIB5_ThreshXHighQ(nr_Cell1, 0, cs_InterFreqCarrierFreqInfo_ThreshX_Q(20, 10)); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_IRAT_PreambleEUTRA_NR (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_6_2_3_4_TestBody(); f_NR_TestBody_Set(false); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); // UE now on EUTRA f_NR_ReleaseAllCells(); }