3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230388

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4735 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for LTE EIEI test case 11.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, at Registration step 11, “a_IMS_ReceiveResponse200OK(v_MessageHeader_Notify, v_Protocol)” at the top level of the alt-statement cause runtime error, because it will capture the 200OK message in previous step, where v_MessageHeader_Notify is unintialised yet 2, v_InviteRequest is not updated for the MT call setup |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Added statement for IMS_Client to check and receive 200 OK intended for NOTIFY only. 2, updated v_InviteRequest at MT call setup sequence |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_11_3_2_IMS1() |
| --- | --- |
| Reason for change | 1, at Registration step 11, “a_IMS_ReceiveResponse200OK(v_MessageHeader_Notify, v_Protocol)” at the top level of the alt-statement cause runtime error, because it will capture the 200OK message in previous step, where v_MessageHeader_Notify is unintialised yet 2, v_InviteRequest is not updated for the MT call setup |
| Summary of change | 1, Added statement for IMS_Client to check and receive 200 OK intended for NOTIFY only. 2, updated v_InviteRequest at MT call setup sequence |
| TTCN module | IMS_ECall_IMS.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_11_3_2_IMS1() runs on IMS_PTC { var IMS_DATA_REQ v_IMS_REGISTER_REQ; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrl; var REGISTER_Request v_RegisterRequest; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader_Notify; var InternetProtocol_Type v_Protocol; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var IMS_Response_Type v_SessionProgress183; var boolean v_FinishedRegistration := false; var boolean v_RxdInvite := false; f_IMS_PTC_Init(PDN_1); // Don't need to wait for trigger as 1st message UL and won't be sent until PDN is in place // @siclog "Step 1 Parallel Behaviour" siclog@ // steps 4-7 of registration procedure v_IMS_REGISTER_REQ := f_IMS_Register(); v_RegisterRequest := v_IMS_REGISTER_REQ.Request.Register; v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterRequest.msgHeader); // @siclog "Step 2 Parallel Behaviour" siclog@ // receive INVITE in parallel to steps 8-11 of registration procedure alt { // @sic R5-227567 sic@ // Registration steps 8-9 [] IMS_Server.check(receive(car_IMS_Subscribe_Request())) { f_IMS_Register_Subscribe(); // Registration step 10 v_MessageHeader_Notify := f_IMS_NotifyRequest_MessageHeader_A_1_6(cs_SubscriptionState_Active); /* @sic R5-153756, R5-153986 sic@ */ v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); // @sic R5s230164 sic@ IMS_Client.send(cas_IMS_Notify_Request(cs_IMS_RoutingInfo(v_Protocol), cs_NOTIFY_Request(v_ContactUrl, v_MessageHeader_Notify, cs_MessageBody_RegInfo(f_IMS_XmlMessageForNotify_Active(v_RegisterRequest))))); repeat; } // Registration step 11 [] a_IMS_ReceiveResponse200OK(v_MessageHeader_Notify, v_Protocol){ v_FinishedRegistration := true; if (not v_RxdInvite){ repeat; } } // INVITE [] IMS_Server.check(receive(car_IMS_Invite_Request())) { v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(TEST, A_2_1_A4); /* @sic R5-220971, R5-221073, R5s230164 sic@ */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "INVITE Received"); v_RxdInvite := true; if (not v_FinishedRegistration) { repeat; } } // allow the Register message to be repeated [] IMS_Server.receive(cr_IMS_DATA_REQ_Any(v_IMS_REGISTER_REQ.RoutingInfo, v_IMS_REGISTER_REQ.Request)) { repeat; } } // Inform EUTRA registration finished and Invite received f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // Steps 2-3 of C.47, once dedicated bearer has been configured f_IMS_IPCAN_WaitForTrigger(IPCAN); // @sic R5-231208 sic@ f_ECall_CallSetup_Steps2_3 (v_InviteRequestWithSdp, { AMR_16000, AMR_8000 }); // Split Invite into a separate function @sic R5-227567, R5s230164 sic@ // Now call is setup, release it v_InviteRequest := v_InviteRequestWithSdp.Invite; v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_InviteRequest.msgHeader); f_IMS_CallReleaseMT(v_ContactUrl); // @sic R5-196019 sic@ // Inform EUTRA: IMS Call released. f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(); v_ResponseWithSdp := f_IMS_MTCallSetup_AnnexC11_Steps1_4(v_InviteRequestWithSdp); v_SessionProgress183 := v_ResponseWithSdp.Response; // Inform EUTRA: IMS Call established. f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequest, v_SessionProgress183.msgHeader); // @sic R5s230164 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); f_IMS_MTCallSetup_AnnexC11_Steps7_13(v_InviteRequestWithSdp, v_ResponseWithSdp); // @sic R5s230164 sic@ v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_InviteRequest.msgHeader); // Now call is setup, release it f_IMS_CallReleaseMT(v_ContactUrl); // Inform EUTRA: IMS Call released. f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change

function f_TC_11_3_2_IMS1() runs on IMS_PTC { var IMS_DATA_REQ v_IMS_REGISTER_REQ; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrl; var REGISTER_Request v_RegisterRequest; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader_Notify; var InternetProtocol_Type v_Protocol; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var IMS_Response_Type v_SessionProgress183; var boolean v_FinishedRegistration := false; var boolean v_RxdInvite := false; var template (present) MessageHeader v_MessageHeader_Response; f_IMS_PTC_Init(PDN_1); // Don't need to wait for trigger as 1st message UL and won't be sent until PDN is in place // @siclog "Step 1 Parallel Behaviour" siclog@ // steps 4-7 of registration procedure v_IMS_REGISTER_REQ := f_IMS_Register(); v_RegisterRequest := v_IMS_REGISTER_REQ.Request.Register; v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterRequest.msgHeader); // @siclog "Step 2 Parallel Behaviour" siclog@ // receive INVITE in parallel to steps 8-11 of registration procedure alt { // @sic R5-227567 sic@ // Registration steps 8-9 [] IMS_Server.check(receive(car_IMS_Subscribe_Request())) { f_IMS_Register_Subscribe(); // Registration step 10 v_MessageHeader_Notify := f_IMS_NotifyRequest_MessageHeader_A_1_6(cs_SubscriptionState_Active); /* @sic R5-153756, R5-153986 sic@ */ v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); // @sic R5s230164 sic@ IMS_Client.send(cas_IMS_Notify_Request(cs_IMS_RoutingInfo(v_Protocol), cs_NOTIFY_Request(v_ContactUrl, v_MessageHeader_Notify, cs_MessageBody_RegInfo(f_IMS_XmlMessageForNotify_Active(v_RegisterRequest))))); v_MessageHeader_Response := f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Notify); repeat; } // Registration step 11 //MOVED [] a_IMS_ReceiveResponse200OK(v_MessageHeader_Notify, v_Protocol){ [] IMS_Client.check(receive(car_IMS_DATA_RSP(cr_IMS_RoutingInfo(), cr_Response(c_statusLine200, v_MessageHeader_Response)))){ a_IMS_ReceiveResponse200OK(v_MessageHeader_Notify, v_Protocol); v_FinishedRegistration := true; if (not v_RxdInvite){ repeat; } } // INVITE [] IMS_Server.check(receive(car_IMS_Invite_Request())) { v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(TEST, A_2_1_A4); /* @sic R5-220971, R5-221073, R5s230164 sic@ */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "INVITE Received"); v_RxdInvite := true; if (not v_FinishedRegistration) { repeat; } } // allow the Register message to be repeated [] IMS_Server.receive(cr_IMS_DATA_REQ_Any(v_IMS_REGISTER_REQ.RoutingInfo, v_IMS_REGISTER_REQ.Request)) { repeat; } } // Inform EUTRA registration finished and Invite received f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // Steps 2-3 of C.47, once dedicated bearer has been configured f_IMS_IPCAN_WaitForTrigger(IPCAN); // @sic R5-231208 sic@ f_ECall_CallSetup_Steps2_3 (v_InviteRequestWithSdp, { AMR_16000, AMR_8000 }); // Split Invite into a separate function @sic R5-227567, R5s230164 sic@ // Now call is setup, release it v_InviteRequest := v_InviteRequestWithSdp.Invite; v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_InviteRequest.msgHeader); f_IMS_CallReleaseMT(v_ContactUrl); // @sic R5-196019 sic@ // Inform EUTRA: IMS Call released. f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(); v_InviteRequest := v_InviteRequestWithSdp.Invite; v_ResponseWithSdp := f_IMS_MTCallSetup_AnnexC11_Steps1_4(v_InviteRequestWithSdp); v_SessionProgress183 := v_ResponseWithSdp.Response; // Inform EUTRA: IMS Call established. f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequest, v_SessionProgress183.msgHeader); // @sic R5s230164 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); f_IMS_MTCallSetup_AnnexC11_Steps7_13(v_InviteRequestWithSdp, v_ResponseWithSdp); // @sic R5s230164 sic@ v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_InviteRequest.msgHeader); // Now call is setup, release it f_IMS_CallReleaseMT(v_ContactUrl); // Inform EUTRA: IMS Call released. f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }