3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230393

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2951 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR L2 test cases with IMS enabled UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For IMS enabled UE, when running MAC testcases where L2 test mode apply, test often fails during postamble with unexpected DRB data at Port due to IMS SIP signalling. The root cause is the route table is not in effect in test mode, and once the test loop is opened at postamble, the IMS signalling triggered from the UE will not go through the routing table and will still be delivered to TTCN in a NR_DRB_COMMON_IND ASP with type RlcPdu/PdcpPdu/PdcpSdu/SdapPdu |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adopted approach in LTE MAC testcases, that do not open the loopback during postamble |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | NR5GC 7.1.x testcases, e.g. 7.1.4.2.NR5GC, 7.1.3.5.4.NR5GC, 7.1.3.5.1.NR5GC, 7.1.3.3.3.NR5GC, 7.1.3.3.2.NR5GC, 7.1.3.3.1.NR5GC, 7.1.2.3.2.NR5GC, 7.1.2.3.1,7.1.2.2.6.NR5GC, 7.1.2.2.5.NR5GC, 7.1.2.2.2.NR5GC, 7.1.1.2.1.NR5GC, 7.1.1.2.2.NR5GC, 7.1.1.3.1.NR5GC, 7.1.2.2.1.NR5GC, 7.1.1.3.2.NR5GC, 7.1.1.3.2b.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_2_1_NR5GC |
| --- | --- |
| Reason for change | For IMS enabled UE, when running MAC testcases where L2 test mode apply, test often fails during postamble with unexpected DRB data at Port due to IMS SIP signalling. The root cause is the route table is not in effect in test mode, and once the test loop is opened at postamble, the IMS signalling triggered from the UE will not go through the routing table and will still be delivered to TTCN in a NR_DRB_COMMON_IND ASP with type RlcPdu/PdcpPdu/PdcpSdu/SdapPdu |
| Summary of change | Adopted approach in LTE MAC testcases, that do not open the loopback during postamble Note – similar change also applies to other MAC testcases eg. 7.1.4.2, 7.1.3.5.4, 7.1.3.5.1, 7.1.3.3.3, 7.1.3.3.2, 7.1.3.3.1, 7.1.2.3.2, 7.1.2.3.1,7.1.2.2.6, 7.1.2.2.5, 7.1.2.2.2, 7.1.1.2.1, 7.1.1.2.2, 7.1.1.3.1, 7.1.2.2.1, 7.1.1.3.2, 7.1.1.3.2b |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_2_1_NR5GC() runs on NR5GC_PTC { // Correct Handling of DL MAC PDU / Assignment / HARQ process var DRB_Identity v_DRBId; ................................... f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change

function f_TC_7_1_1_2_1_NR5GC() runs on NR5GC_PTC { // Correct Handling of DL MAC PDU / Assignment / HARQ process var DRB_Identity v_DRBId; ................................... f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };