3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230401

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0897 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS XCAP test case 15.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-02-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is possible for some deivces that the supplementary service flavours can only be activated in some combinations, e.g. for the case of 15.10 a device may activate rules for CFNRc (not reachable) only inconjunction with CFNL (not logged in). In this case the condition to be checked may be in the 2nd rule in a ruleset. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For CFNL service allow that additional rules may also have been activated during the service activation. Note : An associated prose CR R5-230876 on 34.229-1 is raised at RAN5#98 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformany UE may not pass this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 34.229-1 R5-230876 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_SupplementaryService_Check() |
| --- | --- |
| Reason for change | It is possible for some deivces that the supplementary service flavours can only be activated in some combinations, e.g. for the case of 15.10 a device may activate rules for CFNRc (not reachable) only inconjunction with CFNL (not logged in). In this case the condition to be checked may be in the 2nd rule in a ruleset. |
| Summary of change | For CFNL service allow that additional rules may also have been activated during the service activation. Note : An associated prose CR R5-230876 on 34.229-1 is raised at RAN5#98 |
| TTCN module | IMS_XCAP_Simserv.ttcn |
| MCC160 Comment |  |

Before Change:

function f_SupplementaryService_Check(Ruleset p_Ruleset, SS_Type p_SupplementaryService, boolean p_Deactivated := false) runs on IMS_PTC return boolean { var boolean v_RulesetOK := true; var RuleType v_Rule; var XSD.AnyURI v_TargetAddress; // check ruleset for communication forwarding, communication barring select (p_SupplementaryService) { case (SS_CFU, SS_CFNRy, SS_CFB, SS_CFNL, SS_CFNRc) { v_Rule := p_Ruleset.rule_list[0]; // @sic R5-216570..82: the ruleset has only one rule sic@ fl_IMS_XCAP_CheckRuleId(v_Rule); v_TargetAddress := px_XCAP_TargetUri; v_RulesetOK := (fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated) and fl_ForwardingRule_CheckForwardTo(v_Rule, v_TargetAddress)); } case (SS_BAIC, SS_BAIC_ANONYMOUS, SS_BIC_ROAM, SS_BAOC, SS_BOIC, SS_BOIC_EXHC, SS_BARRING_OUT_ROAM) { // @sic R5-153995 sic@ v_Rule := p_Ruleset.rule_list[0]; // @sic R5-216570..82: the ruleset has only one rule sic@ fl_IMS_XCAP_CheckRuleId(v_Rule); v_RulesetOK := fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated); } case (SS_BAIC_EXCEPT_SPECIFIC_USER) { v_RulesetOK := (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option1(p_Ruleset) or (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule1(p_Ruleset) and fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule2(p_Ruleset))); } } return v_RulesetOK;

After Change:

function f_SupplementaryService_Check(Ruleset p_Ruleset, SS_Type p_SupplementaryService, boolean p_Deactivated := false) runs on IMS_PTC return boolean { var boolean v_RulesetOK := true; var RuleType v_Rule; var integer i; var boolean v_Rulecheck; var XSD.AnyURI v_TargetAddress; // check ruleset for communication forwarding, communication barring select (p_SupplementaryService) { case (SS_CFU, SS_CFNRy, SS_CFB, SS_CFNRc) { v_Rule := p_Ruleset.rule_list[0]; // @sic R5-216570..82: the ruleset has only one rule sic@ fl_IMS_XCAP_CheckRuleId(v_Rule); v_TargetAddress := px_XCAP_TargetUri; v_RulesetOK := (fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated) and fl_ForwardingRule_CheckForwardTo(v_Rule, v_TargetAddress)); } case (SS_CFNL) { v_RulesetOK := false; v_TargetAddress := px_XCAP_TargetUri; for (i := 0; i < lengthof(p_Ruleset.rule_list); i := i + 1) { // Change for supporting multiple ruleset reception v_Rule := p_Ruleset.rule_list[i]; fl_IMS_XCAP_CheckRuleId(v_Rule); v_Rulecheck := (fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated) and fl_ForwardingRule_CheckForwardTo(v_Rule, v_TargetAddress)); if(v_Rulecheck){ v_RulesetOK := true; } } } case (SS_BAIC, SS_BAIC_ANONYMOUS, SS_BIC_ROAM, SS_BAOC, SS_BOIC, SS_BOIC_EXHC, SS_BARRING_OUT_ROAM) { // @sic R5-153995 sic@ v_Rule := p_Ruleset.rule_list[0]; // @sic R5-216570..82: the ruleset has only one rule sic@ fl_IMS_XCAP_CheckRuleId(v_Rule); v_RulesetOK := fl_Rule_CheckConditions(p_SupplementaryService, v_Rule, p_Deactivated); } case (SS_BAIC_EXCEPT_SPECIFIC_USER) { v_RulesetOK := (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option1(p_Ruleset) or (fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule1(p_Ruleset) and fl_IncomingCommunicationBarringExceptSpecificUser_CheckRuleset_Option2_Rule2(p_Ruleset))); } } return v_RulesetOK; }