3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230407

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4721 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT RRC testcase 22.4.16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the RRC TC 22.4.16, a delay of 500ms should be added just after Step 16, as is done after Step 5, so that the RRC Connection Reject is transmitted before the Msg4 configuration is modified to transmit RRC Connection Resume at Step 18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A delay of 500ms is added after Step 16. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to function f_TC_22_4_16_NBIOT	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function f_TC_22_4_16_NBIOT

| Function name | f_TC_22_4_16_NBIOT |
| --- | --- |
| Reason for change | In the RRC TC 22.4.16, a delay of 500ms should be added just after Step 16, as is done after Step 5, so that the RRC Connection Reject is transmitted before the Msg4 configuration is modified to transmit RRC Connection Resume at Step 18. |
| Summary of change | A delay of 500ms is added after Step 16. |
| TTCN module | NBIOT_RRC_UserPlane.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_22_4_16_NBIOT() runs on NBIOT_PTC { <<SKIPPED CODE>> //@siclog "Step 15" siclog@ f_NBIOT_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 16" siclog@ SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell11, cr_RRCConnectionResumeRequest_NB(v_ResumeIdentityStep14, v_ShortMAC_I, mo_Signalling))); // @sic R5s170517 change 1.6: nbiot_Cell11 sic@ //@siclog "Step 17" siclog@ // @sic R5-182302 change 4, R5w180105: RRCConnectionReject preconfigured at the SS before paging sic@ //@siclog "Step 18" siclog@ v_ShortMAC_I := f_NBIOT_Calculate_ShortMAC(nbiot_Cell1, nbiot_Cell11, tsc_VarShortMAC_ResumeDiscriminator); /* @sic R5s170517 change 1.2: tsc_VarShortMAC_ResumeDiscriminator sic@ */ f_NBIOT_SS_ConfigDRB1AndRachProcedureMsg4_RRCConnResume(nbiot_Cell11, cs_RRCConnectionResume_NB); /* @sic R5-182302 change 4, R5w180105: reestablish security and set msg 4 to RRCConnectionResume for cell 11 sic@ @sic R5s180469 additional change 2: Configuration of DRB1 inside the function sic@ */ f_NBIOT_508CheckRRCConnectionResumeOnNewCell(nbiot_Cell11, v_ResumeIdentityStep14, v_ShortMAC_I, mo_Signalling); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, " Step 18"); f_NBIOT_TestBody_Set(false); //Switch/power off UE f_NBIOT_Postamble(nbiot_Cell11, USER_PLANE, N1_IDLE); // @sic R5s170517 change 1.7: N1_IDLE instead of N2_CONNECTED sic@ }

After Change:

function f_TC_22_4_16_NBIOT() runs on NBIOT_PTC { <<SKIPPED CODE>> //@siclog "Step 15" siclog@ f_NBIOT_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 16" siclog@ SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell11, cr_RRCConnectionResumeRequest_NB(v_ResumeIdentityStep14, v_ShortMAC_I, mo_Signalling))); // @sic R5s170517 change 1.6: nbiot_Cell11 sic@ //@siclog "Step 17" siclog@ // @sic R5-182302 change 4, R5w180105: RRCConnectionReject preconfigured at the SS before paging sic@ f_Delay(0.5); //WA# //@siclog "Step 18" siclog@ v_ShortMAC_I := f_NBIOT_Calculate_ShortMAC(nbiot_Cell1, nbiot_Cell11, tsc_VarShortMAC_ResumeDiscriminator); /* @sic R5s170517 change 1.2: tsc_VarShortMAC_ResumeDiscriminator sic@ */ f_NBIOT_SS_ConfigDRB1AndRachProcedureMsg4_RRCConnResume(nbiot_Cell11, cs_RRCConnectionResume_NB); /* @sic R5-182302 change 4, R5w180105: reestablish security and set msg 4 to RRCConnectionResume for cell 11 sic@ @sic R5s180469 additional change 2: Configuration of DRB1 inside the function sic@ */ f_NBIOT_508CheckRRCConnectionResumeOnNewCell(nbiot_Cell11, v_ResumeIdentityStep14, v_ShortMAC_I, mo_Signalling); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, " Step 18"); f_NBIOT_TestBody_Set(false); //Switch/power off UE f_NBIOT_Postamble(nbiot_Cell11, USER_PLANE, N1_IDLE); // @sic R5s170517 change 1.7: N1_IDLE instead of N2_CONNECTED sic@ }