3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230421

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0898 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of geolocation handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-03-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In addition to R5-231910 there are some details in the prose which are not fully implemented in TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Necessary changes to implement R5-231910 and to align TTCN with the prose. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies in prose and TTCN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Test cases with Geolocation |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## 2.1 Changes in decoding of <geopriv>

| Template/function name | cr_Geolocation_Any, ElemList_Type, fl_Geopriv_DecodeAndMatch (new), fl_ElemList_GetFirstGeopriv (new), f_PIDF_GetFirstGeopriv (new) |
| --- | --- |
| Reason for change | - Current implementation would allow <geopriv> element with empty list of locations, what is not according to the test specs (e.g. 34.229-1 A.2.1) - According to RFC 5491 the <geopriv> element(s) may not be contained in the PIDF's <tuple> elements but also in <device> or <person> extensions; according to the test specs it is sufficient to find a single location in any of the <geopriv> elements. |
| Summary of change | - cr_Geolocation_Any changed so that list of locations have at least one element. - New functions to decode <geopriv> elements and find the first one which contains a location |
| TTCN module | IMS_XML_PIDF_Templates |

template (present) Geolocation_Type cr_Geolocation_Any := { location_info := { elem_list := { ?, * } }, usage_rules := ?, method := *, provided_by := *, elem_list := ? };

type record of XSD.String ElemList_Type;

function fl_Geopriv_DecodeAndMatch(XSD.String p_XsdString, template (present) Geolocation_Type p_Geolocation := cr_Geolocation_Any) return template (omit) Geolocation_Type { var bitstring v_Bitstring := f_XsdString2Bit(p_XsdString); var Geopriv_1 v_Geopriv; if (decvalue(v_Bitstring, v_Geopriv) == 0) { if (match(v_Geopriv, p_Geolocation)) { return v_Geopriv; } } return omit; }

function fl_ElemList_GetFirstGeopriv(ElemList_Type p_ElemList, template (present) Geolocation_Type p_Geolocation := cr_Geolocation_Any) return template (omit) Geolocation_Type { var template (omit) Geopriv_1 v_Geopriv := omit; var integer k; for (k := 0; k < lengthof(p_ElemList); k := k + 1) { v_Geopriv := fl_Geopriv_DecodeAndMatch(p_ElemList[k], p_Geolocation); if (isvalue(v_Geopriv)) { break; } } return v_Geopriv; }

function f_PIDF_GetFirstGeopriv(PIDF_Presence_Type p_Presence, template (present) Geolocation_Type p_Geolocation := cr_Geolocation_Any) return template (omit) Geolocation_Type { var template (omit) Geopriv_1 v_Geopriv := omit; var Device v_Device; var Person v_Person; var bitstring v_Bitstring; var integer i; var ElemList_Type v_ElemList; // search in tuples for (i := 0; i < lengthof(p_Presence.tuple_list); i := i + 1) { v_ElemList := p_Presence.tuple_list[i].status.elem_list; v_Geopriv := fl_ElemList_GetFirstGeopriv(v_ElemList, p_Geolocation); if (isvalue(v_Geopriv)) { return v_Geopriv; } } // search in extensions: <device> or <person> elements for (i := 0; i < lengthof(p_Presence.elem_list); i := i + 1) { v_Bitstring := f_XsdString2Bit(p_Presence.elem_list[i]); v_ElemList := {}; if (decvalue(v_Bitstring, v_Device) == 0) { v_ElemList := v_Device.elem_list; } else if (decvalue(v_Bitstring, v_Person) == 0) { v_ElemList := v_Person.elem_list; } v_Geopriv := fl_ElemList_GetFirstGeopriv(v_ElemList, p_Geolocation); if (isvalue(v_Geopriv)) { return v_Geopriv; } } return omit; }

## 2.2 Changes in decoding of MIME body containing PIDF

| Template/function name | f_IMS_DecodeAndCheckGeolocation, f_IMS_MessageBody_CheckSdpMessageAndGeolocation, cr_SipUrl_Cid (new) |
| --- | --- |
| Reason for change | - f_IMS_DecodeAndCheckGeolocation shall check that at least one location is present in a <geopriv> element of a PIDF's <tuple>, <device> or <person> element. - According to 34.229-1 A.2.1 the Geolocation's locationURI shall be the "cid-url indicating the Content-Id of the PIDF-LO within the multipart MIME body of INVITE request"; but this is not checked in TTCN yet. |
| Summary of change | - f_PIDF_GetFirstGeopriv used in f_IMS_DecodeAndCheckGeolocation - Check added to f_IMS_MessageBody_CheckSdpMessageAndGeolocation that the Geolocation's locationURI fits to the Content-Id of the MIME body part containing PIDF NOTE: This requires to replace "boolean p_GeolocationInfoAvailable" by "template (omit) Geolocation p_Geolocation", i.e. at the calling side the Geolocation header field of the INVITE or UPDATE needs to be handed over instead of the boolean. |
| TTCN module | IMS_Procedures_CallControl4G5G, IMS_SIP_Templates |

function f_IMS_DecodeAndCheckGeolocation(charstring p_XmlBody) runs on IMS_PTC { var PIDF_Presence_Type v_Presence := f_PIDF_Decode(p_XmlBody); var template (omit) Geolocation_Type v_Geolocation; if (not match (v_Presence, cr_PIDF_Presence_Any)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Presence (PIDF) does not match"); } // Check PDIF-LO see RFC 4119 cl. 2.2 // extends the 'status' element of PIDF with a complex element called 'geopriv' // There are two major subelements that are encapsulated within geopriv: one for location information, and one for usage rules // Get the first <geopriv> element which is found in a element according to RFC 4119 or in a <device> or <person> element according to RFC 4479 and RFC 5491 v_Geolocation := f_PIDF_GetFirstGeopriv(v_Presence); if (not isvalue(v_Geolocation)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "No <geopriv> element found in PIDF document"); } }

function f_IMS_MessageBody_CheckSdpMessageAndGeolocation(MessageBody p_MessageBody, template (present) SDP_Message p_SDP_MessageTemplate, template (omit) Geolocation p_Geolocation := omit ) runs on IMS_PTC return SDP_Message { var MIME_Part v_MIME_PartSDP; var MIME_Part v_MIME_PartPIDF; var charstring v_AddrSpec; var SdpBody v_SdpBody; if (isvalue(p_Geolocation)) { // if and only if there is a Geolocation header field then there is a PIDF MIME body part if (not ispresent(p_MessageBody.mimeMessageBody.mimePartList) and lengthof(p_MessageBody.mimeMessageBody.mimePartList) != 2) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid Invite body"); } v_MIME_PartSDP := f_IMS_MIME_MessageBody_GetBody(p_MessageBody.mimeMessageBody, c_sdpApplication); v_SdpBody := v_MIME_PartSDP.body.sdpMessageBody; v_MIME_PartPIDF := f_IMS_MIME_MessageBody_GetBody(p_MessageBody.mimeMessageBody, tsc_MimeType_Pidf); v_AddrSpec := regexp(v_MIME_PartPIDF.header.contentId.msgId, "<(*)>", 0); // RFC 2045 clause 7: id := "Content-ID" ":" msg-id; RFC 822: msg-id = "<" addr-spec ">" if (not match(valueof(p_Geolocation.addrSpec), cr_SipUrl_Cid(v_AddrSpec))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "locationURI is not a cid-url indicating the Content-Id of the PIDF-LO"); } // Check PDIF f_IMS_DecodeAndCheckGeolocation(v_MIME_PartPIDF.body.xmlBody); /* @sic R5s210009 - Additional MCC160 changes sic@ */ } else { v_SdpBody := p_MessageBody.sdpMessageBody; } return f_IMS_SIP_DecodeMatchAndCheckSDP(v_SdpBody, p_SDP_MessageTemplate); }

template (present) SipUrl cr_SipUrl_Cid(template (present) charstring p_ContentId := ?) := { // RFC 2392: cid-url = "cid" ":" content-id; content-id = url-addr-spec; url-addr-spec = addr-spec (URL encoding of RFC 822 addr-spec) scheme := "cid", components := { other := p_ContentId }, urlParameters := omit, headers := omit };

## 2.3 Further Improvements

| Template/function name | f_SIP_MIME_MessageBody_FindBody |
| --- | --- |
| Reason for change | - When the required MIME body has been found there is no need to continue with the loop. NOTE: it is assumed, that there is only one MIME body of the given content type. |
| Summary of change | - 'break' added when required MIME body has been found |
| TTCN module | IMS_MessageBody_Templates |

function f_SIP_MIME_MessageBody_FindBody(MIME_Message p_MIME_Message, charstring p_ContentType) return template (omit) MIME_Part { var integer i; var template (omit) MIME_Part v_MIME_Part := omit; var MIME_Part_List v_MimePartList := p_MIME_Message.mimePartList; for (i := 0; i < lengthof(v_MimePartList); i := i + 1) { if (match(v_MimePartList[i].header.contentType, cr_MimeContentType(p_ContentType))) { v_MIME_Part := v_MimePartList[i]; break; } } return v_MIME_Part; }