3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230455

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3104 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Rel-16 MDT Cases : 8.1.6.1.1.1, 8.1.6.1.3.5, 8.1.6.1.4.5, 8.1.6.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test function defined in TS 38.509 is used to provide UE location, only ellipsoidPointWithAltitude, horizontalVelocity and gnss-TOD-msec are provided in the UPDATE UE LOCATION INFORMATION message as defined in TS 38.509/36.509. Therefore UE has no locationTimestamp and locationSource. To check these parameters may fail the UE. UEInformationResponse containing CommonLocationInfo-r16 has following elements: CommonLocationInfo-r16 ::= SEQUENCE { gnss-TOD-msec-r16 OCTET STRING OPTIONAL, è Shall be present due to gnss-TOD-msec as sent in UDPATE UE Location locationTimestamp-r16 OCTET STRING OPTIONAL, è Optional locationCoordinate-r16 OCTET STRING OPTIONAL, è Shall be present due to ellipsoidPointWithAltitude as sent in UDPATE UE Location locationError-r16 OCTET STRING OPTIONAL, è Optional locationSource-r16 OCTET STRING OPTIONAL, è Optional velocityEstimate-r16 OCTET STRING OPTIONAL è Shall be present due to horizontalVelocity as sent in UDPATE UE Location #draft proseCR would be raised during RAN5#99 |  |  |  |  |  |  |  |  |
| Summary of change: |  | IE gnss-TOD-msec-r16 -- Shall be present IE locationTimestamp-r16 – Optional IE locationCoordinate-r16 – Shall be present IE locationError-r16 – Optional IE locationSource-r16 -- Optional IE velocityEstimate-r16 – Shall be present |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.1.1.NR5GC, 8.1.6.1.3.5.NR5GC, 8.1.6.1.4.5.1.NR5GC and 8.1.6.2.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	9

2.4	Change 4	12

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:                      Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_1_1_1_NR5GC_TestBody() |
| --- | --- |
| Reason for change | According to the preamble, if the test function defined in TS 38.509 is used to provide UE location, only ellipsoidPointWithAltitude, horizontalVelocity and gnss-TOD-msec are provided in the UPDATE UE LOCATION INFORMATION message as defined in TS 38.509/36.509. Therefore UE has no locationTimestamp and locationSource. To check these parameters may fail the UE. #draft proseCR would be raised during RAN5#99 |
| Summary of change | Modify the checking of Measurement Report at Step 4 using cdr_38508_MeasResults_LocationInfo to NOT check locationTimestamp and locationSource. |
| TTCN module | RRC_MDT_IntraNR_Immediate_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_1_1_1_NR5GC_TestBody() runs on NR_BASE_PTC { var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -98; var NR_AbsoluteCellPower_Type v_CellPower_T1_FR2 := -96; //@sic R5-216496 sic@ var NR_AbsoluteCellPower_Type v_CellPower_T2_FR1 := -78; var NR_AbsoluteCellPower_Type v_CellPower_T2_FR2 := -82; //@sic R5-216496 sic@ var PhysCellId v_PhysCellId; var RSRP_Range v_RsrpThres; var RSRP_Range v_AbsThreshSS_Rsrp; var Hysteresis v_Hysteresis; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (present) MeasResults v_MeasResults; var template (value) OtherConfig_v1610 v_OtherConfig_v1610; //@sic R5-231194 sic@ //@siclog "Steps 1-2" siclog@ // The SS transmits an RRCReconfiguration message including measConfig to setup intra NR measurement // and reporting for event A2 with includeLocationInfo configured. // The UE transmits an RRCReconfigurationComplete message. v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_AbsThreshSS_Rsrp := 57; v_RsrpThres := 69; v_Hysteresis := 0; //@sic R5-216496 sic@ v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, v_AbsThreshSS_Rsrp)}; v_ReportConfigList := {cs_NR_ReportConfigEventA2(tsc_NR_IdReportConfigId1, v_RsrpThres, r1, v_Hysteresis)}; //@sic R5s211595 sic@ v_ReportConfigList[0].reportConfig.reportConfigNR.reportType.eventTriggered.includeCommonLocationInfo_r16 := true_; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); v_OtherConfig_v1610 := cs_OtherConfig_v1610; //@sic R5-231194 sic@ v_OtherConfig_v1610.obtainCommonLocation_r16 := true_; //@sic R5-231194 sic@ f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig,-,-,-,-,-,-,v_OtherConfig_v1610); //@sic R5-231194 sic@ //@siclog "Step 3" siclog@ // The SS re-adjusts the SS/PBCH EPRE level according to row "T1" in Table 8.1.6.1.1.1.3.2-1. f_NR_SetCellPower (nr_Cell1, v_CellPower_T1_FR1, v_CellPower_T1_FR2); //@sic R5-216496 sic@ //@siclog "Step 4" siclog@ // Check: Does the UE transmit a MeasurementReport message to report event A2 with the UE commonLocationInfo included? v_MeasResults := cdr_38508_MeasResults_LocationInfo(-, v_PhysCellId, -, -, -, ?, ?, ?, *, ?, ?); //@sic R5s211595 sic@ f_NR_ReceiveOneMeasurementReport(nr_Cell1, v_MeasResults); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); … }

After Change

function f_TC_8_1_6_1_1_1_NR5GC_TestBody() runs on NR_BASE_PTC { var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -98; var NR_AbsoluteCellPower_Type v_CellPower_T1_FR2 := -96; //@sic R5-216496 sic@ var NR_AbsoluteCellPower_Type v_CellPower_T2_FR1 := -78; var NR_AbsoluteCellPower_Type v_CellPower_T2_FR2 := -82; //@sic R5-216496 sic@ var PhysCellId v_PhysCellId; var RSRP_Range v_RsrpThres; var RSRP_Range v_AbsThreshSS_Rsrp; var Hysteresis v_Hysteresis; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (present) MeasResults v_MeasResults; var template (value) OtherConfig_v1610 v_OtherConfig_v1610; //@sic R5-231194 sic@ //@siclog "Steps 1-2" siclog@ // The SS transmits an RRCReconfiguration message including measConfig to setup intra NR measurement // and reporting for event A2 with includeLocationInfo configured. // The UE transmits an RRCReconfigurationComplete message. v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_AbsThreshSS_Rsrp := 57; v_RsrpThres := 69; v_Hysteresis := 0; //@sic R5-216496 sic@ v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, v_AbsThreshSS_Rsrp)}; v_ReportConfigList := {cs_NR_ReportConfigEventA2(tsc_NR_IdReportConfigId1, v_RsrpThres, r1, v_Hysteresis)}; //@sic R5s211595 sic@ v_ReportConfigList[0].reportConfig.reportConfigNR.reportType.eventTriggered.includeCommonLocationInfo_r16 := true_; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); v_OtherConfig_v1610 := cs_OtherConfig_v1610; //@sic R5-231194 sic@ v_OtherConfig_v1610.obtainCommonLocation_r16 := true_; //@sic R5-231194 sic@ f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig,-,-,-,-,-,-,v_OtherConfig_v1610); //@sic R5-231194 sic@ //@siclog "Step 3" siclog@ // The SS re-adjusts the SS/PBCH EPRE level according to row "T1" in Table 8.1.6.1.1.1.3.2-1. f_NR_SetCellPower (nr_Cell1, v_CellPower_T1_FR1, v_CellPower_T1_FR2); //@sic R5-216496 sic@ //@siclog "Step 4" siclog@ // Check: Does the UE transmit a MeasurementReport message to report event A2 with the UE commonLocationInfo included? v_MeasResults := cdr_38508_MeasResults_LocationInfo(-, v_PhysCellId, -, -, -, ?, *, ?, *, *, ?); //@sic R5s211595 sic@ f_NR_ReceiveOneMeasurementReport(nr_Cell1, v_MeasResults); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); … }

## Change 2

| Function name | f_TC_8_1_6_2_1_TestBody() |
| --- | --- |
| Reason for change | According to the preamble, if the test function defined in TS 38.509 is used to provide UE location, only ellipsoidPointWithAltitude, horizontalVelocity and gnss-TOD-msec are provided in the UPDATE UE LOCATION INFORMATION message as defined in TS 38.509/36.509. Therefore UE has no locationTimestamp and locationSource. To check these parameters may fail the UE. #draft proseCR would be raised during RAN5#99 |
| Summary of change | Modify the checking of Measurement Report at Step 5 using cdr_38508_MeasResults_LocationInfo to NOT check locationTimestamp and locationSource. |
| TTCN module | RRC_MDT_InterRAT_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_2_1_TestBody() runs on NR_BASE_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0_T1; var NR_AbsoluteCellPower_Type v_CellPower_T0_T1_FR1 := -78; var NR_AbsoluteCellPower_Type v_CellPower_T0_T1_FR2 := -82; //@sic R5-220050, R5-216497 sic@ //FR2 is FFS var PhysCellId v_PhysCellId; var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var integer v_MeasurementBandwidth_int; var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth; var integer v_MaxReportCells := 1; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR; //var template (value) MeasObjectNR v_MeasObjectNR; //@sic R5-216497 sic@ //var template (omit) MeasObjectToAddModList v_MeasObjectList_NR; //@sic R5-216497 sic@ var template (omit) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (present) MeasResults.measResultNeighCells v_MeasResultNeighCells; var template (present) MeasResults v_MeasResults; var template (value) OtherConfig_v1610 v_OtherConfig_v1610; //@sic R5-231197 sic@ //Power levels of NR cell 1: Table 8.1.6.2.1.3.2-1, Table 8.1.6.2.1.3.2-2 /* ¦ FR1 ¦ FR2 * ------------------------------- * T0 ¦ -78 ¦ -82 * ------------------------------- * T1 ¦ -78 ¦ FFS * */ v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := bit2int (v_CoOrd_SysInfo.Eutra[0].CellId); v_MeasurementBandwidth_int := v_CoOrd_SysInfo.Eutra[0].DL_Bandwidth; v_MeasurementBandwidth := f_NR5GC_IRAT_ConvertIntegerToEUTRA_AllowedMeasBandwidth(v_MeasurementBandwidth_int); v_CellPowerList_AtT0_T1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T0_T1_FR1, v_CellPower_T0_T1_FR2)}; //@sic R5-216497 sic@ //FR2 is FFS f_NR_SetCellPowerList(v_CellPowerList_AtT0_T1); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT measurement and reporting for E-UTRA cell with includeLocationInfo //The UE transmits an RRCReconfigrationComplete message v_MeasuredObjectList_NR := {cs_MeasObjectId1(nr_Cell1)}; v_MeasuredObjectList_EUTRA := {cs_MeasObjectId2_EUTRA(v_EUTRA_CellId, v_ARFCN_ValueEUTRA)}; v_ReportConfigList := {cs_NR_ReportConfig_reportConfigInterRAT_Periodical(tsc_NR_IdReportConfigId1, cs_NR_ReportConfigInterRAT_Trigger_periodical_EUTRA(v_MaxReportCells, true_))}; //@sic R5s211526 sic@ v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, -, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cds_MeasGapConfig_UE(cs_38508_GapConfig), cs_38508_QuantityConfig_EUTRA); //f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); // @sic R5-224131, R5s220792 sic@ //@siclog "Step 3" siclog@ //The SS changes E-UTRA Cell 1 parameters according to row "T1" in Table 8.1.6.2.1.3.2-1/2. f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //f_NR_SetCellPowerList(v_CellPowerList_AtT0_T1); //FR2 is FFS //@sic R5s220792 sic@ v_OtherConfig_v1610 := cs_OtherConfig_v1610; //@sic R5-231197 sic@ v_OtherConfig_v1610.obtainCommonLocation_r16 := true_; //@sic R5-231197 sic@ f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig,-,-,-,-,-,-,v_OtherConfig_v1610); //@sic R5-231197, R5-224131, R5s220792 sic@ //f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); //@sic R5-224131, R5s220792 sic@ //@siclog "Steps 4" siclog@ // Wait and ignore MeasurementReport messages for 8 s to allow for trhe switching of cells and UE Measuremnet. f_NR_IgnorePeriodicalMeasurmentReport(8.0); //@siclog "Step 5" siclog@ //Check: Does the UE transmit a MeasurementReport message on NR Cell 1 to perform periodical reporting for E-UTRA Cell 1 with locationInfo? v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_MeasResultNeighCells := cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_Rsrp_Rsrq)}); //@sic R5-216497 sic@ v_MeasResults := cdr_38508_MeasResults_LocationInfo(-, v_PhysCellId, v_MeasResultNeighCells, -, -, ?, ?, ?, *, ?, ?); //@sic R5-224131, R5s220792 sic@ f_NR_ReceivePeriodicalMeasurementReports(nr_Cell1, 30.0, 10, cr_NR_MeasurementReport(v_MeasResults)); … }

After Change

function f_TC_8_1_6_2_1_TestBody() runs on NR_BASE_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0_T1; var NR_AbsoluteCellPower_Type v_CellPower_T0_T1_FR1 := -78; var NR_AbsoluteCellPower_Type v_CellPower_T0_T1_FR2 := -82; //@sic R5-220050, R5-216497 sic@ //FR2 is FFS var PhysCellId v_PhysCellId; var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var integer v_MeasurementBandwidth_int; var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth; var integer v_MaxReportCells := 1; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR; //var template (value) MeasObjectNR v_MeasObjectNR; //@sic R5-216497 sic@ //var template (omit) MeasObjectToAddModList v_MeasObjectList_NR; //@sic R5-216497 sic@ var template (omit) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (present) MeasResults.measResultNeighCells v_MeasResultNeighCells; var template (present) MeasResults v_MeasResults; var template (value) OtherConfig_v1610 v_OtherConfig_v1610; //@sic R5-231197 sic@ //Power levels of NR cell 1: Table 8.1.6.2.1.3.2-1, Table 8.1.6.2.1.3.2-2 /* ¦ FR1 ¦ FR2 * ------------------------------- * T0 ¦ -78 ¦ -82 * ------------------------------- * T1 ¦ -78 ¦ FFS * */ v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := bit2int (v_CoOrd_SysInfo.Eutra[0].CellId); v_MeasurementBandwidth_int := v_CoOrd_SysInfo.Eutra[0].DL_Bandwidth; v_MeasurementBandwidth := f_NR5GC_IRAT_ConvertIntegerToEUTRA_AllowedMeasBandwidth(v_MeasurementBandwidth_int); v_CellPowerList_AtT0_T1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T0_T1_FR1, v_CellPower_T0_T1_FR2)}; //@sic R5-216497 sic@ //FR2 is FFS f_NR_SetCellPowerList(v_CellPowerList_AtT0_T1); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including MeasConfig to setup inter-RAT measurement and reporting for E-UTRA cell with includeLocationInfo //The UE transmits an RRCReconfigrationComplete message v_MeasuredObjectList_NR := {cs_MeasObjectId1(nr_Cell1)}; v_MeasuredObjectList_EUTRA := {cs_MeasObjectId2_EUTRA(v_EUTRA_CellId, v_ARFCN_ValueEUTRA)}; v_ReportConfigList := {cs_NR_ReportConfig_reportConfigInterRAT_Periodical(tsc_NR_IdReportConfigId1, cs_NR_ReportConfigInterRAT_Trigger_periodical_EUTRA(v_MaxReportCells, true_))}; //@sic R5s211526 sic@ v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, -, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cds_MeasGapConfig_UE(cs_38508_GapConfig), cs_38508_QuantityConfig_EUTRA); //f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); // @sic R5-224131, R5s220792 sic@ //@siclog "Step 3" siclog@ //The SS changes E-UTRA Cell 1 parameters according to row "T1" in Table 8.1.6.2.1.3.2-1/2. f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //f_NR_SetCellPowerList(v_CellPowerList_AtT0_T1); //FR2 is FFS //@sic R5s220792 sic@ v_OtherConfig_v1610 := cs_OtherConfig_v1610; //@sic R5-231197 sic@ v_OtherConfig_v1610.obtainCommonLocation_r16 := true_; //@sic R5-231197 sic@ f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig,-,-,-,-,-,-,v_OtherConfig_v1610); //@sic R5-231197, R5-224131, R5s220792 sic@ //f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); //@sic R5-224131, R5s220792 sic@ //@siclog "Steps 4" siclog@ // Wait and ignore MeasurementReport messages for 8 s to allow for trhe switching of cells and UE Measuremnet. f_NR_IgnorePeriodicalMeasurmentReport(8.0); //@siclog "Step 5" siclog@ //Check: Does the UE transmit a MeasurementReport message on NR Cell 1 to perform periodical reporting for E-UTRA Cell 1 with locationInfo? v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_MeasResultNeighCells := cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_Rsrp_Rsrq)}); //@sic R5-216497 sic@ v_MeasResults := cdr_38508_MeasResults_LocationInfo(-, v_PhysCellId, v_MeasResultNeighCells, -, -, ?, *, ?, *, *, ?); //@sic R5-224131, R5s220792 sic@ f_NR_ReceivePeriodicalMeasurementReports(nr_Cell1, 30.0, 10, cr_NR_MeasurementReport(v_MeasResults)); … }

## Change 3

| Function name | f_TC_8_1_6_1_4_5_NR5GC() |
| --- | --- |
| Reason for change | According to the preamble, if the test function defined in TS 38.509 is used to provide UE location, only ellipsoidPointWithAltitude, horizontalVelocity and gnss-TOD-msec are provided in the UPDATE UE LOCATION INFORMATION message as defined in TS 38.509/36.509. Therefore UE has no locationTimestamp and locationSource. To check these parameters may fail the UE. #draft proseCR would be raised during RAN5#99 |
| Summary of change | Modify the checking of UE Information Response at Step 15 to NOT check locationTimestamp |
| TTCN module | RRC_MDT_InterRAT_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_1_4_5_NR5GC() runs on NR5GC_PTC { /* Connection Establishment Failure / Logging and reporting / Location Information */ var template (present) UL_DCCH_Message v_UEInformationResponse; f_NR5GC_Init(NR_1); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //Preamble //The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A. f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); /*The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. *This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [6], if supported by the UE according to pc_UpdateUE_LocationInformation. *Otherwise, or in addition any other suitable method may also be used.*/ f_NR_ProvideUE_LocationInformation(nr_Cell1, RRC_IDLE); f_NR_TestBody_Set(true); //@siclog "Step 1" siclog@ //The SS is configured to not transmit Msg2 in 2-step RACH procedure //Void @sic R5-216313 sic@ //@siclog "Step 2" siclog@ //The SS transmits a Paging message f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 3" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 4" siclog@ //The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. f_Delay(2.0); //@siclog "Step 5" siclog@ //The SS is configured to normal that it would transmit Msg2 in 2-step RACH procedure. //Void @sic R5-216313 sic@ //@siclog "Steps 6-13" siclog@ //Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 //The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message. //Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3. f_NR5GC_RRC_ConnectedState3N_SON_MDT(nr_Cell1, connEstFailInfoAvailable); // @sic R5s211593 sic@ //@siclog "Step 14" siclog@ //The SS sends a UEInformationRequest message to get connEstFailReportReq SRB.send(cas_NR_SRB_RrcPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, //@sic R5s211593 sic@ -, //cs_TimingInfo_Now cs_38508_UEInformationRequest)); //@sic R5-225389 sic@ //@siclog "Step 15" siclog@ //Check: Does the UE send a UEInformationResponse message with connEstFailReport with the IE commonLocationInfo-r16 is present? v_UEInformationResponse := cr_UEInformationResponse_r16_CEF(cr_ConnEstFailReport_r16(?,omit,omit,?)); //@sic R5-217923 sic@ SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, v_UEInformationResponse)); //@sic R5s211593 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 15"); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); //@sic R5s211593 sic@ }// End of f_TC_8_1_6_1_4_5_NR5GC()

After Change

function f_TC_8_1_6_1_4_5_NR5GC() runs on NR5GC_PTC { /* Connection Establishment Failure / Logging and reporting / Location Information */ var template (present) UL_DCCH_Message v_UEInformationResponse; f_NR5GC_Init(NR_1); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //Preamble //The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A. f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); /*The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. *This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [6], if supported by the UE according to pc_UpdateUE_LocationInformation. *Otherwise, or in addition any other suitable method may also be used.*/ f_NR_ProvideUE_LocationInformation(nr_Cell1, RRC_IDLE); f_NR_TestBody_Set(true); //@siclog "Step 1" siclog@ //The SS is configured to not transmit Msg2 in 2-step RACH procedure //Void @sic R5-216313 sic@ //@siclog "Step 2" siclog@ //The SS transmits a Paging message f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 3" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 4" siclog@ //The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. f_Delay(2.0); //@siclog "Step 5" siclog@ //The SS is configured to normal that it would transmit Msg2 in 2-step RACH procedure. //Void @sic R5-216313 sic@ //@siclog "Steps 6-13" siclog@ //Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 //The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message. //Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3. f_NR5GC_RRC_ConnectedState3N_SON_MDT(nr_Cell1, connEstFailInfoAvailable); // @sic R5s211593 sic@ //@siclog "Step 14" siclog@ //The SS sends a UEInformationRequest message to get connEstFailReportReq SRB.send(cas_NR_SRB_RrcPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, //@sic R5s211593 sic@ -, //cs_TimingInfo_Now cs_38508_UEInformationRequest)); //@sic R5-225389 sic@ //@siclog "Step 15" siclog@ //Check: Does the UE send a UEInformationResponse message with connEstFailReport with the IE commonLocationInfo-r16 is present? v_UEInformationResponse := cr_UEInformationResponse_r16_CEF(cr_ConnEstFailReport_r16(?,omit,omit,?)); v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.connEstFailReport_r16.locationInfo_r16.commonLocationInfo_r16.gnss_TOD_msec_r16 := ?; v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.connEstFailReport_r16.locationInfo_r16.commonLocationInfo_r16.locationTimestamp_r16 := *; v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.connEstFailReport_r16.locationInfo_r16.commonLocationInfo_r16.locationCoordinate_r16 := ?; v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.connEstFailReport_r16.locationInfo_r16.commonLocationInfo_r16.locationError_r16 := *; v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.connEstFailReport_r16.locationInfo_r16.commonLocationInfo_r16.locationSource_r16 := *; v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.connEstFailReport_r16.locationInfo_r16.commonLocationInfo_r16.velocityEstimate_r16 := ?; //@sic R5-217923 sic@ SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, v_UEInformationResponse)); //@sic R5s211593 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 15"); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); //@sic R5s211593 sic@ }// End of f_TC_8_1_6_1_4_5_NR5GC()

## Change 4

| Function name | f_TC_8_1_6_1_3_5_NR5GC() |
| --- | --- |
| Reason for change | According to the preamble, if the test function defined in TS 38.509 is used to provide UE location, only ellipsoidPointWithAltitude, horizontalVelocity and gnss-TOD-msec are provided in the UPDATE UE LOCATION INFORMATION message as defined in TS 38.509/36.509. Therefore UE has no locationTimestamp and locationSource. To check these parameters may fail the UE. #draft proseCR would be raised during RAN5#99 |
| Summary of change | Modify the checking of UE Information Response at Step 9 to NOT check locationTimestamp |
| TTCN module | RRC_MDT_IntraNR_RLF_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_1_3_Common(NR_CellId_Type p_NeighbourCell, boolean p_CheckLocationInfo := false) runs on NR5GC_PTC {//@sic R5s211638, R5-217921 sic@ var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var NR_AbsoluteCellPower_Type v_CellPower_A3_Entry_FR1; var NR_AbsoluteCellPower_Type v_CellPower_A3_Entry_FR2; var RSRP_Range v_RsrpThres; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdServing_received; var PhysCellId v_PhysCellIdNeighbour; var ARFCN_ValueNR v_ARFCN_Neighbour; var template (present) UL_DCCH_Message v_UEInformationResponse; var DeltaValues_Type v_Deltas := f_NR_MobileInfo_GetPrimaryDeltaValue(); //@sic R5-214550 sic@ var template (omit) MeasGapConfig v_MeasGapConfig := omit; var CellIdentity v_CellIdentity_Serving; var CellIdentity v_CellIdentity_Serving_received; var CellIdentity v_CellIdentity_Neighbour; var NR_SRB_COMMON_IND v_ReceivedMsg; var RLF_Report_r16 v_ReceivedReport; var template (omit) OtherConfig_v1610 v_OtherConfig_v1610 := omit; //@sic R5-231195 sic@ //Table 8.1.6.1.3.1.3.2-1/2 v_CellPower_A3_Entry_FR1 := -82; v_CellPower_A3_Entry_FR2 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2; //@sic R5-220053 sic@ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(p_NeighbourCell, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(p_NeighbourCell, v_CellPower_A3_Entry_FR1, v_CellPower_A3_Entry_FR2)}; v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; f_NR_CellInfo_InitMaxReferencePower(p_NeighbourCell, v_CellPower_A3_Entry_FR1, v_CellPower_A3_Entry_FR2); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(p_NeighbourCell); //Preamble //The UE is in state 3N-A as defined in TS 38.508-1, subclause 4.4A on NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); if(p_CheckLocationInfo) {/*The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. *This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [6], if supported by the UE according to pc_UpdateUE_LocationInformation */ f_NR_ProvideUE_LocationInformation(nr_Cell1, RRC_CONNECTED); } f_NR_TestBody_Set(true); f_NR_SetCellPowerList(v_CellPowerList_AtT0); //Table 8.1.6.1.3.1.3.3-2¦3¦4 v_RsrpThres := 2; //@sic R5-216289 sic@ if (p_NeighbourCell == nr_Cell3) //@sic R5s211638 sic@ { if (not f_NR_CellInfo_GetIsFR1(p_NeighbourCell)) { v_RsrpThres := v_RsrpThres + v_Deltas.DeltaNRf2 - v_Deltas.DeltaNRf1; //@sic R5-214550 sic@ } v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1), cs_MeasObjectId2(p_NeighbourCell)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasGapConfig := cds_MeasGapConfig_UE(f_NR_GetGapConfig_InterFreqOdd(nr_Cell1, p_NeighbourCell)); } else { v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; } v_ReportConfigList := {cs_NR_ReportConfigEventA3(tsc_NR_IdReportConfigId1, v_RsrpThres, r1, -, -, cs_NR_MeasReportQuantity_Rsrp)}; //@sic R5-217921 sic@ v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapConfig); // @sic R5s211638 sic@ //@siclog "Steps 1-2" siclog@ // The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3 (measId 1). //The UE transmits an RRCReconfigurationComplete message. if(p_CheckLocationInfo) {//@sic R5-231195 sic@ v_OtherConfig_v1610 := cs_OtherConfig_v1610; v_OtherConfig_v1610.obtainCommonLocation_r16 := true_; } f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig,-,-,-,-,-,-,v_OtherConfig_v1610); //@sic R5-231195 sic@ //@siclog "Step 3" siclog@ //The SS changes NR Cell 2 parameters according to the row "T1" in Table 8.1.6.1.3.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 4" siclog@ //The UE transmit a MeasurementReport message to report event A3 (measId 1) with the measured RSRP value for NR Cell 2. v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_NeighbourCell); f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, cr_NR_MeasQuantityResults_Rsrp)})); //@siclog "Step 5" siclog@ //The SS changes NR Cell 1 parameter according to the row "T2" in Table 8.1.6.1.3.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Steps 6-10" siclog@ //The UE transmits an RRCReestablishmentRequest message on neighblur cell. //The SS transmits an RRCReestablishment message. //The UE transmits an RRCReestablishmentComplete message. //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. //The UE transmits an RRCReconfigurationtComplete message. //The SS transmits an RRCReestablishment message. //The UE transmits an RRCReestablishmentComplete message. f_NR5GC_RRCReestablishment_SON_MDT(nr_Cell1, p_NeighbourCell, rlf_InfoAvailable); //@sic R5s211638 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 9: RRCReestablishmentComplete message with rlf-InfoAvailable included"); //@siclog "Step 11" siclog@ //The SS transmits a UEInformationRequest message. SRB.send(cas_NR_SRB_RrcPdu_REQ(p_NeighbourCell, tsc_NR_RbId_SRB1, -, //cs_TimingInfo_Now cs_NR_UEInformationRequest_RLF)); // condition RLF //@siclog "Step 12" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 2? v_ARFCN_Neighbour := f_NR_CellInfo_GetFrequencySSB(p_NeighbourCell); v_CellIdentity_Serving := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_CellIdentity_Neighbour := f_NR_CellInfo_GetCellIdentity(p_NeighbourCell); v_UEInformationResponse := cr_UEInformationResponse_r16_RLF(v_PhysCellIdNeighbour, v_ARFCN_Neighbour, v_CellIdentity_Neighbour, f_NR_CellInfo_GetRNTI(nr_Cell1)); // @sic R5s211638 sic@ if(p_CheckLocationInfo) { v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16.nr_RLF_Report_r16.locationInfo_r16 := ?; //@sic R5-217923 sic@ } SRB.receive(car_NR_SRB_RrcPdu_IND(p_NeighbourCell, tsc_NR_RbId_SRB1, v_UEInformationResponse)) -> value v_ReceivedMsg; // @sic R5s211638 sic@ v_ReceivedReport := v_ReceivedMsg.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16; … }

After Change

function f_TC_8_1_6_1_3_Common(NR_CellId_Type p_NeighbourCell, boolean p_CheckLocationInfo := false) runs on NR5GC_PTC {//@sic R5s211638, R5-217921 sic@ var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var NR_AbsoluteCellPower_Type v_CellPower_A3_Entry_FR1; var NR_AbsoluteCellPower_Type v_CellPower_A3_Entry_FR2; var RSRP_Range v_RsrpThres; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdServing_received; var PhysCellId v_PhysCellIdNeighbour; var ARFCN_ValueNR v_ARFCN_Neighbour; var template (present) UL_DCCH_Message v_UEInformationResponse; var DeltaValues_Type v_Deltas := f_NR_MobileInfo_GetPrimaryDeltaValue(); //@sic R5-214550 sic@ var template (omit) MeasGapConfig v_MeasGapConfig := omit; var CellIdentity v_CellIdentity_Serving; var CellIdentity v_CellIdentity_Serving_received; var CellIdentity v_CellIdentity_Neighbour; var NR_SRB_COMMON_IND v_ReceivedMsg; var RLF_Report_r16 v_ReceivedReport; var template (omit) OtherConfig_v1610 v_OtherConfig_v1610 := omit; //@sic R5-231195 sic@ //Table 8.1.6.1.3.1.3.2-1/2 v_CellPower_A3_Entry_FR1 := -82; v_CellPower_A3_Entry_FR2 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2; //@sic R5-220053 sic@ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(p_NeighbourCell, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(p_NeighbourCell, v_CellPower_A3_Entry_FR1, v_CellPower_A3_Entry_FR2)}; v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; f_NR_CellInfo_InitMaxReferencePower(p_NeighbourCell, v_CellPower_A3_Entry_FR1, v_CellPower_A3_Entry_FR2); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(p_NeighbourCell); //Preamble //The UE is in state 3N-A as defined in TS 38.508-1, subclause 4.4A on NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); if(p_CheckLocationInfo) {/*The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. *This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [6], if supported by the UE according to pc_UpdateUE_LocationInformation */ f_NR_ProvideUE_LocationInformation(nr_Cell1, RRC_CONNECTED); } f_NR_TestBody_Set(true); f_NR_SetCellPowerList(v_CellPowerList_AtT0); //Table 8.1.6.1.3.1.3.3-2¦3¦4 v_RsrpThres := 2; //@sic R5-216289 sic@ if (p_NeighbourCell == nr_Cell3) //@sic R5s211638 sic@ { if (not f_NR_CellInfo_GetIsFR1(p_NeighbourCell)) { v_RsrpThres := v_RsrpThres + v_Deltas.DeltaNRf2 - v_Deltas.DeltaNRf1; //@sic R5-214550 sic@ } v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1), cs_MeasObjectId2(p_NeighbourCell)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasGapConfig := cds_MeasGapConfig_UE(f_NR_GetGapConfig_InterFreqOdd(nr_Cell1, p_NeighbourCell)); } else { v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; } v_ReportConfigList := {cs_NR_ReportConfigEventA3(tsc_NR_IdReportConfigId1, v_RsrpThres, r1, -, -, cs_NR_MeasReportQuantity_Rsrp)}; //@sic R5-217921 sic@ v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapConfig); // @sic R5s211638 sic@ //@siclog "Steps 1-2" siclog@ // The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3 (measId 1). //The UE transmits an RRCReconfigurationComplete message. if(p_CheckLocationInfo) {//@sic R5-231195 sic@ v_OtherConfig_v1610 := cs_OtherConfig_v1610; v_OtherConfig_v1610.obtainCommonLocation_r16 := true_; } f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig,-,-,-,-,-,-,v_OtherConfig_v1610); //@sic R5-231195 sic@ //@siclog "Step 3" siclog@ //The SS changes NR Cell 2 parameters according to the row "T1" in Table 8.1.6.1.3.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 4" siclog@ //The UE transmit a MeasurementReport message to report event A3 (measId 1) with the measured RSRP value for NR Cell 2. v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_NeighbourCell); f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, cr_NR_MeasQuantityResults_Rsrp)})); //@siclog "Step 5" siclog@ //The SS changes NR Cell 1 parameter according to the row "T2" in Table 8.1.6.1.3.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Steps 6-10" siclog@ //The UE transmits an RRCReestablishmentRequest message on neighblur cell. //The SS transmits an RRCReestablishment message. //The UE transmits an RRCReestablishmentComplete message. //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. //The UE transmits an RRCReconfigurationtComplete message. //The SS transmits an RRCReestablishment message. //The UE transmits an RRCReestablishmentComplete message. f_NR5GC_RRCReestablishment_SON_MDT(nr_Cell1, p_NeighbourCell, rlf_InfoAvailable); //@sic R5s211638 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 9: RRCReestablishmentComplete message with rlf-InfoAvailable included"); //@siclog "Step 11" siclog@ //The SS transmits a UEInformationRequest message. SRB.send(cas_NR_SRB_RrcPdu_REQ(p_NeighbourCell, tsc_NR_RbId_SRB1, -, //cs_TimingInfo_Now cs_NR_UEInformationRequest_RLF)); // condition RLF //@siclog "Step 12" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 2? v_ARFCN_Neighbour := f_NR_CellInfo_GetFrequencySSB(p_NeighbourCell); v_CellIdentity_Serving := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_CellIdentity_Neighbour := f_NR_CellInfo_GetCellIdentity(p_NeighbourCell); v_UEInformationResponse := cr_UEInformationResponse_r16_RLF(v_PhysCellIdNeighbour, v_ARFCN_Neighbour, v_CellIdentity_Neighbour, f_NR_CellInfo_GetRNTI(nr_Cell1)); // @sic R5s211638 sic@ if(p_CheckLocationInfo) { v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16.nr_RLF_Report_r16.locationInfo_r16.commonLocationInfo_r16.gnss_TOD_msec_r16 := ?; //@sic R5-217923 sic@ v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16.nr_RLF_Report_r16.locationInfo_r16.commonLocationInfo_r16.locationTimestamp_r16 := *; v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16.nr_RLF_Report_r16.locationInfo_r16.commonLocationInfo_r16.locationCoordinate_r16 := ?; v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16.nr_RLF_Report_r16.locationInfo_r16.commonLocationInfo_r16.locationError_r16 := *; v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16.nr_RLF_Report_r16.locationInfo_r16.commonLocationInfo_r16.locationSource_r16 := *; v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16.nr_RLF_Report_r16.locationInfo_r16.commonLocationInfo_r16.velocityEstimate_r16 := ?; } SRB.receive(car_NR_SRB_RrcPdu_IND(p_NeighbourCell, tsc_NR_RbId_SRB1, v_UEInformationResponse)) -> value v_ReceivedMsg; // @sic R5s211638 sic@ v_ReceivedReport := v_ReceivedMsg.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16;