3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230456

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3105 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to template cr_UE_NR_Capability_v1640_IEs_RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per TS 38.331 MAC-Parameters-v1700 IE is an optional IE. Thus, it needs to be checked for its presence. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add ifpresent for IE mac_Parameters_v1700 in template cr_UE_NR_Capability_v1640_IEs_RedCap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

  - Correction to template cr_UE_NR_Capability_v1640_IEs_RedCap

| Function name | cr_UE_NR_Capability_v1640_IEs_RedCap |
| --- | --- |
| Reason for change | As per TS 38.331 MAC-Parameters-v1700 IE is an optional IE. Thus, it needs to be checked for its presence. |
| Summary of change | Add ifpresent for IE mac_Parameters_v1700 in template cr_UE_NR_Capability_v1640_IEs_RedCap. |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

template UE_NR_Capability_v1640 cr_UE_NR_Capability_v1640_IEs_RedCap := // @sic R5-231410 R5s211246 R5-225384 R5s221155 R5s221179 R5-227571 R5-227572 R5-227573 sic@

{ /* @status    APPROVED (ENDC, NR5GC) */

redirectAtResumeByNAS_r16                   := *,

phy_ParametersSharedSpectrumChAccess_r16    := cr_V1640_phy_ParametersSharedSpectrumChAccess_r16 ifpresent,

nonCriticalExtension                        := { // UE-NR-Capability-v1650

mpsPriorityIndication_r16     := *,

highSpeedParameters_v1650     := *,

nonCriticalExtension          := {           // UE-NR-Capability-v1690

ul_RRC_Segmentation_r16   := *,

nonCriticalExtension      := {           // UE-NR-Capability-v1700

inactiveStatePO_Determination_r17 := *,

highSpeedParameters_v1700         := *,

powSav_Parameters_v1700           := *,

mac_Parameters_v1700              := {

mac_ParametersFR2_2_r17   := {      // MAC-ParametersFR2-2-r17

directMCG_SCellActivation_r17       := f_Derive_directMCG_SCellActivation_r17() ifpresent, // pc_directMCG_SCellActivation_r17

directMCG_SCellActivationResume_r17 := *,

directSCG_SCellActivation_r17       := *,

directSCG_SCellActivationResume_r17 := *,

drx_Adaptation_r17                  := *

}

},

ims_Parameters_v1700              := *,

……

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

template UE_NR_Capability_v1640 cr_UE_NR_Capability_v1640_IEs_RedCap := // @sic R5-231410 R5s211246 R5-225384 R5s221155 R5s221179 R5-227571 R5-227572 R5-227573 sic@

{ /* @status    APPROVED (ENDC, NR5GC) */

redirectAtResumeByNAS_r16                   := *,

phy_ParametersSharedSpectrumChAccess_r16    := cr_V1640_phy_ParametersSharedSpectrumChAccess_r16 ifpresent,

nonCriticalExtension                        := { // UE-NR-Capability-v1650

mpsPriorityIndication_r16     := *,

highSpeedParameters_v1650     := *,

nonCriticalExtension          := {           // UE-NR-Capability-v1690

ul_RRC_Segmentation_r16   := *,

nonCriticalExtension      := {           // UE-NR-Capability-v1700

inactiveStatePO_Determination_r17 := *,

highSpeedParameters_v1700         := *,

powSav_Parameters_v1700           := *,

mac_Parameters_v1700              := {

mac_ParametersFR2_2_r17   := {      // MAC-ParametersFR2-2-r17

directMCG_SCellActivation_r17       := f_Derive_directMCG_SCellActivation_r17() ifpresent, // pc_directMCG_SCellActivation_r17

directMCG_SCellActivationResume_r17 := *,

directSCG_SCellActivation_r17       := *,

directSCG_SCellActivationResume_r17 := *,

drx_Adaptation_r17                  := *

}

} ifpresent,

ims_Parameters_v1700              := *,

……

<<SKIPPED CODE>>