3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230459

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3108 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC postamble function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current implementation of STATE_EMERGENCY_RELEASE_IDLE does not handle a possible IMS deregistration on switch off |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow UE to receive SERVICE REQUEST on switch off, and handle inside existing function f_NR5GC_OptionalIMSDeregisterOnSwitchOff() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE supporting IMS can fail some emergency call test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 6.1.3.7.NR5GC, 11.4.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR_Postamble() |
| --- | --- |
| Reason for change | Current implementation of STATE_EMERGENCY_RELEASE_IDLE does not handle a possible IMS deregistration on switch off |
| Summary of change | Allow UE to receive SERVICE REQUEST on switch off, and handle inside existing function f_NR5GC_OptionalIMSDeregisterOnSwitchOff() |
| TTCN module | NR5GC_NASSteps.ttcn. |
| MCC160 Comment |  |

Before Change:

function f_NR_Postamble(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_DetachReq; var boolean v_RxdMsg := false; var boolean v_RxdCnf := false; timer t_Wait := 5.0; /*Skipping the code */ else if (p_UE_State == STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE(UT, false, CNF_REQUIRED, true); // @sic R5s220417 sic@ alt { // The Setup Req may arrive before the CNF @sic R5s220417 sic@ [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest))){} [] UT.receive(car_UT_COMMON_CNF){ v_RxdCnf := true; } } t_Wait.start; // Still need to start timer incase Setup Req arrives after CNF // @sic R5s211344 sic@ // Steps 1a3a1 - 1a3a4 (Steps 1a3a2, 1a3b1 done inside f_NR_RRC_SetupRequest_Common) v_RxdMsg := f_NR_RRC_SetupRequest_Common(p_NR_CellId, t_Wait); if (v_RxdMsg) { f_NR_RRCSetup_Def (p_NR_CellId); v_DetachReq := f_NR_RRCSetupComplete_Def(p_NR_CellId, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); if (not f_Check_NG_DeregistrationReqMOMsg (v_DetachReq.Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } // Step 1a3a5 f_NR_RRCRelease(p_NR_CellId); } if (not v_RxdCnf) { // If Setup is already there, still need to wait for CNF @sic R5s220417 sic@ UT.receive(car_UT_COMMON_CNF); } } else { f_UT_PowerOffUE(UT); } } else { // All other options already handled in Deregister function f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State, p_SRB); // @sic R5-195241, R5s21163 sic@ } /*Skipping the code */ }

After Change:

function f_NR_Postamble(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_DetachReq; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var boolean v_RxdMsg := false; var boolean v_RxdCnf := false; timer t_Wait := 5.0; /*Skipping the code */ else if (p_UE_State == STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE(UT, false, CNF_REQUIRED, true); // @sic R5s220417 sic@ alt { // The Setup Req may arrive before the CNF @sic R5s220417 sic@ [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest))){} [] UT.receive(car_UT_COMMON_CNF){ v_RxdCnf := true; } } t_Wait.start; // Still need to start timer incase Setup Req arrives after CNF // @sic R5s211344 sic@ // Steps 1a3a1 - 1a3a4 (Steps 1a3a2, 1a3b1 done inside f_NR_RRC_SetupRequest_Common) v_RxdMsg := f_NR_RRC_SetupRequest_Common(p_NR_CellId, t_Wait); if (v_RxdMsg) { f_NR_RRCSetup_Def (p_NR_CellId); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(p_NR_CellId, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); v_DetachReq := f_NR5GC_OptionalIMSDeregisterOnSwitchOff(p_NR_CellId, v_ReceivedMsg); // Step 1a3a5 f_NR_RRCRelease(p_NR_CellId); } if (not v_RxdCnf) { // If Setup is already there, still need to wait for CNF @sic R5s220417 sic@ UT.receive(car_UT_COMMON_CNF); } } else { f_UT_PowerOffUE(UT); } } else { // All other options already handled in Deregister function f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State, p_SRB); // @sic R5-195241, R5s21163 sic@ } /*Skipping the code */ }