3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230460

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4739 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NBIOT_UE_Page |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU EMEA LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Due to Incorrect Implementation of TTCN CR R5s230317, accessing the variable v_PagingCarrierID is resulting in access violation error in TTCN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the change present in R5s230317 after v_PagingCarrierID assignment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail many NB-IOT cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.1,22.3.1.1,22.3.1.10,22.3.3.1,22.3.3.2,22.3.3.3,22.3.3.4,22.3.3.6,22.4.15,22.4.16,22.4.19a,22.4.2,22.4.20a,22.4.23, and sevearl other 22.4.x and 22.5.x tests |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NBIOT_UE_Page |
| --- | --- |
| Reason for change | Due to Incorrect Implementation of TTCN CR R5s230317, accessing the variable v_PagingCarrierID is resulting in access violation error in TTCN |
| Summary of change | Added the change present in R5s230317 after v_PagingCarrierID assignment. |
| TTCN module | NBIOT_Paging |
| MCC160 Comment |  |

Before Change

function f_NBIOT_UE_Page(NBIOT_CellId_Type p_CellId, template (omit) PagingRecordList_NB_r13 p_PagingRecordList := omit, template (omit) integer p_PagingCarrierId := omit) runs on NBIOT_PTC { var PCCH_Config_NB_r13 v_SI_Pcch_Configuration; var NB_DefaultPagingCycle_Type v_PagingCycle; var SubFrameTiming_Type v_CurrentTiming; var SubFrameTimingList_Type v_PoList; var hexstring v_Imsi := px_IMSI_Def; var GutiParameters_Type v_GutiParameters; var template (value) PagingRecordList_NB_r13 v_PagingRecordList; var template (omit) integer v_PagingCarrierID := p_PagingCarrierId; var NBIOT_CellInfo_Type v_CellInfo; var charstring v_testcasename := testcasename(); if (isvalue(p_PagingRecordList)) { v_PagingRecordList := valueof(p_PagingRecordList); } else { v_GutiParameters := f_NBIOT_CellInfo_GetGuti(p_CellId); v_PagingRecordList := cs_NB_Paging_OneRecord(cs_PagingUE_Identity_S_TMSI(v_GutiParameters.MME_Code, v_GutiParameters.M_TMSI)); } f_Delay(tsc_NBIOT_DelayForCellSelection); //Derive DRX params from SYS Info v_SI_Pcch_Configuration := f_NBIOT_CellInfo_GetPCCH_ConfigInSYSINFO(p_CellId); //Set the Pagingcycle v_PagingCycle := v_SI_Pcch_Configuration.defaultPagingCycle_r13; //In case paging carrier ID is not given, check if it could be derrived based on SIB22 configuration if (not(ispresent(p_PagingCarrierId))) {//@sic R5s221226 sic@ v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); if(isvalue(v_CellInfo.Sysinfo.BCCH_Info.SIs[1].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[0].sib22_v1430)) { v_PagingCarrierID := f_NBIOT_Calculate_PagingID(v_PagingCycle, v_SI_Pcch_Configuration.nB_r13, f_NBIOT_CellInfo_GetSIB22(p_CellId), v_Imsi ); } } if(valueof(v_PagingCarrierID) == 0 and v_testcasename == "TC_22_3_1_8") { v_PagingCarrierID := omit; } // R5s230317 //Get current SFN and subframe v_CurrentTiming := f_NBIOT_GetCurrentTiming(p_CellId); //Calculate next Paging Ocassion v_PoList := f_NBIOT_Calculate_PF_PO(v_CurrentTiming.SFN.Number, v_CurrentTiming.HSFN.Number, v_PagingCycle, v_SI_Pcch_Configuration.nB_r13, v_Imsi); SYS.send(cas_NB_CellConfig_Paging_REQ(p_CellId, v_PoList[0], v_PagingRecordList, v_PagingCarrierID)); /* @sic R5w180306: Support of non-anchor carrier, R5s221226 sic@ */ }

After Change

function f_NBIOT_UE_Page(NBIOT_CellId_Type p_CellId, template (omit) PagingRecordList_NB_r13 p_PagingRecordList := omit, template (omit) integer p_PagingCarrierId := omit) runs on NBIOT_PTC { var PCCH_Config_NB_r13 v_SI_Pcch_Configuration; var NB_DefaultPagingCycle_Type v_PagingCycle; var SubFrameTiming_Type v_CurrentTiming; var SubFrameTimingList_Type v_PoList; var hexstring v_Imsi := px_IMSI_Def; var GutiParameters_Type v_GutiParameters; var template (value) PagingRecordList_NB_r13 v_PagingRecordList; var template (omit) integer v_PagingCarrierID := p_PagingCarrierId; var NBIOT_CellInfo_Type v_CellInfo; var charstring v_testcasename := testcasename(); if (isvalue(p_PagingRecordList)) { v_PagingRecordList := valueof(p_PagingRecordList); } else { v_GutiParameters := f_NBIOT_CellInfo_GetGuti(p_CellId); v_PagingRecordList := cs_NB_Paging_OneRecord(cs_PagingUE_Identity_S_TMSI(v_GutiParameters.MME_Code, v_GutiParameters.M_TMSI)); } f_Delay(tsc_NBIOT_DelayForCellSelection); //Derive DRX params from SYS Info v_SI_Pcch_Configuration := f_NBIOT_CellInfo_GetPCCH_ConfigInSYSINFO(p_CellId); //Set the Pagingcycle v_PagingCycle := v_SI_Pcch_Configuration.defaultPagingCycle_r13; //In case paging carrier ID is not given, check if it could be derrived based on SIB22 configuration if (not(ispresent(p_PagingCarrierId))) {//@sic R5s221226 sic@ v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); if(isvalue(v_CellInfo.Sysinfo.BCCH_Info.SIs[1].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[0].sib22_v1430)) { v_PagingCarrierID := f_NBIOT_Calculate_PagingID(v_PagingCycle, v_SI_Pcch_Configuration.nB_r13, f_NBIOT_CellInfo_GetSIB22(p_CellId), v_Imsi ); if(valueof(v_PagingCarrierID) == 0 and v_testcasename == "TC_22_3_1_8") { v_PagingCarrierID := omit; } // R5s230317 } } //Get current SFN and subframe v_CurrentTiming := f_NBIOT_GetCurrentTiming(p_CellId); //Calculate next Paging Ocassion v_PoList := f_NBIOT_Calculate_PF_PO(v_CurrentTiming.SFN.Number, v_CurrentTiming.HSFN.Number, v_PagingCycle, v_SI_Pcch_Configuration.nB_r13, v_Imsi); SYS.send(cas_NB_CellConfig_Paging_REQ(p_CellId, v_PoList[0], v_PagingRecordList, v_PagingCarrierID)); /* @sic R5w180306: Support of non-anchor carrier, R5s221226 sic@ */ }