3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230474

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3110 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR emergency services test case 11.4.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At the step 2-4, when UE send TAU request to the LTE, the count is incorrectly calculated as previous count is not updated from NR. Function f_EUTRA38_TAUReqFromN1 returns NAS count which is return value from f_EUTRA_RRC_ConnEst_DefWithNas and also, 5G context is mapped to 4G at the end of the f_EUTRA38_TAUReqFromN1. Therefore, at the step 5-8, latest NAS count is applied to generate based 5G context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At the step 5-8 use latest NAS count instead of old LTE value from preamble |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:                      Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_11_4_12_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | At the step 2-4, when UE send TAU request to the LTE, the count is incorrectly calculated as previous count is not updated from NR. Function f_EUTRA38_TAUReqFromN1 returns NAS count which is return value from f_EUTRA_RRC_ConnEst_DefWithNas and also, 5G context is mapped to 4G at the end of the f_EUTRA38_TAUReqFromN1. Therefore, at the step 5-8, latest NAS count is applied to generate based 5G context. |
| Summary of change | At the step 5-8 use latest NAS count instead of old LTE value from preamble |
| TTCN module | Emergency_IRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_4_12_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var NasCount_Type v_NasCount; //@siclog "Step 0" siclog@ //@sic R5-223438 sic@ //Set the power levels according to T0 as per Table 11.4.12.3.1-1/2 f_EUTRA38_SetCellPower(eutra_Cell1, -91, -91); //@siclog "Step 1" siclog@ //Make the UE initiate an Emergency call. f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 2 - 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? //SS transmits an RRCConnectionSetup message. //Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. //For some consequences related to the content of the TRACKING AREA UPDATE REQUEST message see the Notes in TS.36.508 [7], Table 4.5A.2.1-1. v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); if (not match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap_AnyproSeCap('58'O))){ // @sic R5s221058 sic@ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability indication is not correct at step 4"); } else { f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); }; v_NasCount := v_NasInd.SecurityProtection.NasCount; //@siclog "Steps 5-8" siclog@ //Steps 5-8 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); //@siclog "Step 9" siclog@ //SS responds with TRACKING AREA UPDATE ACCEPT message. //@siclog "Step 10" siclog@ //The UE transmits a TRACKING AREA UPDATE COMPLETE message. f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true, -, -, true); //@siclog "Steps 11-17" siclog@ //Steps 9-15 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1, -, -, -, '100'B, omit, true); // @sic R5s211357 R5-231416 sic@ // @siclog "Step 19-20" siclog@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s221058 sic@ // Emergency call release in IMS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Call Released"); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); // @sic R5s211357 R5s221058 sic@ } }

After Change

function fl_TC_11_4_12_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var NasCount_Type v_NasCount; var NasCountInfo_Type v_NASCountInfo; //@siclog "Step 0" siclog@ //@sic R5-223438 sic@ //Set the power levels according to T0 as per Table 11.4.12.3.1-1/2 f_EUTRA38_SetCellPower(eutra_Cell1, -91, -91); //@siclog "Step 1" siclog@ //Make the UE initiate an Emergency call. f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 2 - 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? //SS transmits an RRCConnectionSetup message. //Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. //For some consequences related to the content of the TRACKING AREA UPDATE REQUEST message see the Notes in TS.36.508 [7], Table 4.5A.2.1-1. v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); if (not match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap_AnyproSeCap('58'O))){ // @sic R5s221058 sic@ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability indication is not correct at step 4"); } else { f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); }; v_NASCountInfo := f_EUTRA_NB_SS_NAS_CountGet(NASCTRL); //@siclog "Steps 5-8" siclog@ //Steps 5-8 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NASCountInfo.UL); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); //@siclog "Step 9" siclog@ //SS responds with TRACKING AREA UPDATE ACCEPT message. //@siclog "Step 10" siclog@ //The UE transmits a TRACKING AREA UPDATE COMPLETE message. f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true, -, -, true); //@siclog "Steps 11-17" siclog@ //Steps 9-15 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1, -, -, -, '100'B, omit, true); // @sic R5s211357 R5-231416 sic@ // @siclog "Step 19-20" siclog@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s221058 sic@ // Emergency call release in IMS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Call Released"); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); // @sic R5s211357 R5s221058 sic@ } }