3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230480

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0911 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS eCall over IMS testcase 11.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify IMS 5GS test case 11.1 using IWD 23wk24 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS test case 11.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	New template cr_MIME_Part_CommonBinary	3

2.2	Correction to template cr_MIME_Message_SDP_XML	4

2.3	Correction to funcion f_SIP_MessageBodySDP_RX	5

2.4	Correction to funcion f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData	5

2.5	Correction to funcion f_ECall_CallSetup_Common	7

2.6	Correction to funcion f_IMS_ECallOverIMS_IMS1	8

2.7	Correction to funcion f_IMS_ECallOverIMS_IMS2	8

3	Branches executed	10

4	Execution Log Files	10

4.1	HiSilicon Balong 5000	10

5	References	10

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 11.1 which is part of the iwd-TTCN3-B2022-09_D23wk24.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Parikshit Bhise

Parikshit.Bhise@rohde-schwarz.com

## Verification Test Summary

Test Case:	11.1.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk24

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## New template cr_MIME_Part_CommonBinary

| Template name | cr_MIME_Part_CommonBinary |
| --- | --- |
| Reason for change | The MSD included in INVITE SIP message for eCall test cases should be encoded as specified in TS 34.229-1 Annex A.2.1 in ASN.1 PER. That will require a CommonBinary body when current implementation uses a Xml body. |
| Summary of change | Created new template for required commonBinary body. |
| TTCN module | Common\IMS\IMS_MessageBody_Templates.ttcn |
| MCC160 Comment |  |

template (present) MIME_Part cr_MIME_Part_CommonBinary(template (present) MIME_ContentType p_ContentType := ?, template MIME_ContentDisposition p_ContentDisposition := *, template MIME_ContentId p_ContentId := *) := { header := cr_MIME_Part_Header(p_ContentType, p_ContentDisposition, p_ContentId), body := { commonBinary := ? } };

## Correction to template cr_MIME_Message_SDP_XML

| Template name | cr_MIME_Message_SDP_XML |
| --- | --- |
| Reason for change | Currently while checking the validity of a MIME message, will only consider that MIME will be composed with an SDP and a xml part due to support of geolocation services. As specified at TS 34.229-1 Annex A.2.1 message body. Implementation seems incomplete as also conditions A20, A21 and A25 will include contents on MIME message of CommonBinary nature, that can be, in addition of a xml part due to support of geolocation services. |
| Summary of change | Added the possibility to check that a message body with multipart/mixed content type is correct as it includes a Common Binary body, xml body or both. |
| TTCN module | Common\IMS\IMS_MessageBody_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> template (present) MIME_Message cr_MIME_Message_SDP_XML(template (present) MIME_ContentType p_SdpContentType := ?, template (present) MIME_ContentType p_XmlContentType := ?) := { /* MIME encapsulation acc. to condition A8 in A.2.1 */ /* @status APPROVED (IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R12, LTE_IRAT, NR5GC, NR5GC_IRAT) */ boundary := ?, mimePartList := { /* @sic R5w180006 sic@ */ cr_MIME_Part_SDP(p_SdpContentType), /* @sic R5w140112 sic@ */ cr_MIME_Part_XML(p_XmlContentType) } }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (present) MIME_Message cr_MIME_Message_SDP_Any(template (present) MIME_ContentType p_SdpContentType := ?, template (present) MIME_ContentType p_CommonBinaryContentType := ?, template (present) MIME_ContentType p_XmlContentType := ?) := { /* MIME encapsulation acc. to condition A8 AND/OR ( A20 OR A21 OR A25 ) in A.2.1 */ boundary := ?, mimePartList := { /* @sic R5w180006 sic@ */ cr_MIME_Part_SDP(p_SdpContentType), /* @sic R5w140112 sic@ */ (cr_MIME_Part_CommonBinary(p_CommonBinaryContentType),cr_MIME_Part_XML(p_XmlContentType)), * } }; <<SKIPPED CODE>>

## Correction to funcion f_SIP_MessageBodySDP_RX

| Function name | f_SIP_MessageBodySDP_RX |
| --- | --- |
| Reason for change | Currently while checking the validity of a MIME message, will only consider that MIME will be composed with an SDP and a xml part due to support of geolocation services. As specified at TS 34.229-1 Annex A.2.1 message body. Implementation seems incomplete as also conditions A20, A21 and A25 will include contents on MIME message of CommonBinary nature, that can be, in addition of a xml part due to support of geolocation services. |
| Summary of change | Added the possibility to check that a message body with multipart/mixed content type is correct as it includes a Common Binary body, xml body or both. |
| TTCN module | Common\IMS\IMS_MessageBody_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_SIP_MessageBodySDP_RX(boolean p_UseMIME) return template (present) MessageBody { /* to be used e.g. for INVITE when message body depends on condition A8 (UE uses Geolocation header to provide its geographical location for emergency session setup, has obtained its location and is setting up an emergency session) */ var template (present) MessageBody v_MessageBody; if (p_UseMIME) { v_MessageBody := cr_MessageBody_MIME(cr_MIME_Message_SDP_XML); } else { v_MessageBody := cr_MessageBody_SDP; } return v_MessageBody; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_SIP_MessageBodySDP_RX(boolean p_UseMIME) return template (present) MessageBody { /* to be used e.g. for INVITE when message body depends on condition A8 (UE uses Geolocation header to provide its geographical location for emergency session setup, has obtained its location and is setting up an emergency session. This will also apply when the UE sets an eCall session independent on location services) */ var template (present) MessageBody v_MessageBody; if (p_UseMIME) { v_MessageBody := cr_MessageBody_MIME(cr_MIME_Message_SDP_Any); } else { v_MessageBody := cr_MessageBody_SDP; } return v_MessageBody; } <<SKIPPED CODE>>

## Correction to funcion f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData

| Function name | f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData |
| --- | --- |
| Reason for change | Implementation expects the same contents for CallInfo header-cid url and ContentID in message body. This is incorrect as specified in RFC3261 while CallInfo should contain an absoluteUri, ContentID should just contain its url. Estructure of ContentID is specified at RFC2392 Chapter 2: The URLs take the form content-id = url-addr-spec message-id = url-addr-spec url-addr-spec = addr-spec ; URL encoding of RFC 822 addr-spec cid-url = "cid" ":" content-id |
| Summary of change | Modified ContentId field on receiving template to adjust to RFC2392. |
| TTCN module | Common\IMS\IMS_eCall_Functions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_IMS_ECall_EmergencyCallData_Control_DecodeAndMatch(charstring p_XmlBody, template EmergencyCallData_Control p_EmergencyCallData_Control) return boolean { var bitstring v_Bitstring := f_Char2Bit(p_XmlBody); var EmergencyCallData_Control v_EmergencyCallData_Control; var boolean v_Result := false; if (decvalue(v_Bitstring, v_EmergencyCallData_Control) == 0) { v_Result := match(v_EmergencyCallData_Control, p_EmergencyCallData_Control); } return v_Result; } SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(MIME_Message p_MIME_Message, charstring p_ContentType, charstring p_ContentId, template SemicolonParam_List p_DispositionParams := *, template EmergencyCallData_Control p_EmergencyCallDataXML := omit) runs on IMS_PTC { var MIME_Part v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(p_MIME_Message, p_ContentType); var charstring v_ContentId := replace(p_ContentId, 1, 4, ""); var template (present) MIME_Part_Header v_HeaderTemplate := cr_MIME_Part_Header(cr_MimeContentType(p_ContentType), cr_MimeContentDisposition("by-reference", p_DispositionParams), cr_MimeContentId(v_ContentId)); if (not match(v_MIME_Part.header, v_HeaderTemplate)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid MIME header"); } if (ispresent(p_EmergencyCallDataXML)) { if (not ischosen(v_MIME_Part.body.xmlBody) or not f_IMS_ECall_EmergencyCallData_Control_DecodeAndMatch(v_MIME_Part.body.xmlBody, p_EmergencyCallDataXML)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid MIME body"); } } } <<SKIPPED CODE>>

## Correction to funcion f_ECall_CallSetup_Common

| Function name | f_ECall_CallSetup_Common |
| --- | --- |
| Reason for change | EUTRA and NR IMS emergency calls follow different establishment procedures, current implementation is missing the addition of Voice bearer for NR case. |
| Summary of change | Corrected eCall call setup adding voice bearers for NR case. |
| TTCN module | Common\IMS\IMS_eCall_Functions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_ECall_CallSetup_Common(ECall_Type p_ECallType, IMS_SDP_AudioCodecList_Type p_AudioCodecList, INVITE_A_2_1_Context_Type p_Context := A_2_1_A7) runs on IMS_PTC { //Allowed to be re-used for 5G with different codec @sic R5-223392 sic@ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; // Step 1. Receive INVITE for an eCall v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(p_ECallType, p_Context); // @sic R5s230115 sic@ f_ECall_CallSetup_Steps2_3 (v_InviteRequestWithSdp, p_AudioCodecList); // Split Invite into a separate function @sic R5-227567 sic@ }SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_ECall_CallSetup_Common(ECall_Type p_ECallType, IMS_SDP_AudioCodecList_Type p_AudioCodecList, INVITE_A_2_1_Context_Type p_Context := A_2_1_A7) runs on IMS_PTC { //Allowed to be re-used for 5G with different codec @sic R5-223392 sic@ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; // Step 1. Receive INVITE for an eCall v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(p_ECallType, p_Context); // @sic R5s230115 sic@ if (f_IMS_PTC_RanTypeIsNR5GC()){ f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); f_ECall_CallSetup_Steps2_3 (v_InviteRequestWithSdp, p_AudioCodecList); f_IMS_IPCAN_WaitForTrigger(IPCAN, "Voice Bearer Added"); } else { f_ECall_CallSetup_Steps2_3 (v_InviteRequestWithSdp, p_AudioCodecList); // Split Invite into a separate function @sic R5-227567 sic@ } } <<SKIPPED CODE>>

## Correction to funcion f_IMS_ECallOverIMS_IMS1

| Function name | f_IMS_ECallOverIMS_IMS1 |
| --- | --- |
| Reason for change | The f_IMS_CC_Postamble must be called from IMS2 PTC instead of IMS1 PTC for NR flavour because f_NR5GC_EmergencyCallRelease is interacting explicitly with IMS2 (and not with the component where the coordination command comes from). |
| Summary of change | Called f_IMS_CC_Postamble function from IMS2 PTC instead of IMS1 PTC for NR eCall Test cases. |
| TTCN module | Common\IMS\IMS_CC_TestcaseImplementation_eCall.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_IMS_ECallOverIMS_IMS1(charstring p_USIM_ToInsert) runs on IMS_PTC { // @sic R5-221598 re-used common code with 4G and 5G sic@ /* IMS implementation for PDN1 - IMS normal registration */ f_UT_USIM_Insert(MMI, p_USIM_ToInsert); f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_ECallOverIMS_IMS1(charstring p_USIM_ToInsert) runs on IMS_PTC { // @sic R5-221598 re-used common code with 4G and 5G sic@ /* IMS implementation for PDN1 - IMS normal registration */ f_UT_USIM_Insert(MMI, p_USIM_ToInsert); f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); f_IMS_IMS_SendCoOrdMsg(OtherPDN); //Postamble of Legacy IMS sos calls are handled in IMS1 PTC while postbamble for NR IMS sos calls is handled in IMS2 PTC if (not f_IMS_PTC_RanTypeIsNR5GC()){ f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); } }<<SKIPPED CODE>>

## Correction to funcion f_IMS_ECallOverIMS_IMS2

| Function name | f_IMS_ECallOverIMS_IMS2 |
| --- | --- |
| Reason for change | The f_IMS_CC_Postamble must be called from IMS2 PTC instead of IMS1 PTC for NR flavour because f_NR5GC_EmergencyCallRelease is interacting explicitly with IMS2 (and not with the component where the coordination command comes from). |
| Summary of change | Called f_IMS_CC_Postamble function from IMS2 PTC instead of IMS1 PTC for NR eCall Test cases. |
| TTCN module | Common\IMS\IMS_CC_TestcaseImplementation_eCall.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_IMS_ECallOverIMS_IMS1(charstring p_USIM_ToInsert) runs on IMS_PTC { // @sic R5-221598 re-used common code with 4G and 5G sic@ /* IMS implementation for PDN1 - IMS normal registration */ f_UT_USIM_Insert(MMI, p_USIM_ToInsert); f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_ECallOverIMS_IMS2(ECall_Type p_CallType, ECall_MSD_Update_Type p_MSDUpdate, template (value) EmergencyCallData_Control p_EmergencyCallData_Control_Step4 := cs_EmergencyCallData_Control_Request) runs on IMS_PTC { // @sic R5-221598 re-used common code with 4G and 5G sic@ /* IMS implementation for PDN2 - IMS normal registration */ var SipUrl v_ContactUrl; f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); // Trigger eCall initiation, PDN and DRB establishment according to TS 36.508/4.5A.26.3-1 // @siclog "Steps 1" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_EmergencyCall_NormalService); f_UT_InitiateECall(MMI, p_CallType); //@siclog "Steps 2-5 (21.*)" siclog@ //@siclog "Steps 2-11 (11.*)" siclog@ f_IMS_EmergencyRegistration(); // @siclog "Step 6-8 (21.1, 21.2)" siclog@ // @siclog "Step 6-12 (21.4, 21.5, 21.6)" siclog@ // @siclog "Step 12-18 (11.4, 11.5, 11.6)" siclog@ f_ECall_CallSetup_AnnexC47(p_CallType); if (p_MSDUpdate == msdUpdateRequired) { f_ECall_MSDUpdate_AnnexC47(p_EmergencyCallData_Control_Step4); } // @siclog "Step 9-12 (21.1, 21.2)" siclog@ // @siclog "Step 13-14 (21.4, 21.5, 21.6)" siclog@ // @siclog "Step 19-20 (11.4, 11.5, 11.6)" siclog@ // @sic R5-221598 removed option for MO release as this is never used sic@ v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrl); f_IMS_TestBody_Set(false); //Postamble of Legacy IMS sos calls are handled in IMS1 PTC while postbamble for NR IMS sos calls is handled in IMS2 PTC if (f_IMS_PTC_RanTypeIsNR5GC()){ f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); } else { f_IMS_IMS_SendCoOrdMsg(OtherPDN); } <<SKIPPED CODE>>

# Branches executed

The test case was executed on NR5GC band n41 with and using 128-IEA1 Integrity and 128-NEA1 ciphering algorithms.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_11_1_NR5GC_HiSi.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s230481: Supporting information for addition of IMS 5GS eCall over IMS testcase 11.1 |
| --- | --- |