3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230508

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3125 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1: Re-verification of Rel-16 Power Saving test case 7.1.1.12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, R&S |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 7.1.1.12.3 to the 5G/NR5GC ATS for FR1 path |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.1.12.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary Test Case:  7.1.1.12.3.NR5GC	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

3	Branches executed	6

4	Execution Log Files	6

4.1	MediaTek MT6895	6

4.2	MediaTek MT6893	6

5	References	6

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 7.1.1.12.3, which is part of the NR5GC test suite in iwd-TTCN3-B2022-09_D23wk24.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Parikshit Bhise
	parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

## Test Case: 	7.1.1.12.3.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset, R&S 5G Conformance Test Platform

UE used:	MediaTek MT6895, MediaTek MT6893

Verification Status:	PASS

# Corrections required

## Change 1

| Template name | cads_NR_DL_BWP_ConfigCommon_REQ () |
| --- | --- |
| Reason for change | The IEs FrequencyInfoDL, SSPbchBlock, PdschCellLevelConfig, CsiConfig have not been initialised to Omit in template cads_NR_DL_BWP_ConfigCommon_REQ and this causeds RUNTIME errors. |
| Summary of change | IEs FrequencyInfoDL, SSPbchBlock, PdschCellLevelConfig, CsiConfig have been initialised to Omit |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_DL_BWP_ConfigCommon_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_DownlinkBWP_List_Type p_NR_DownlinkBWP_List_Type) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status */ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink:= { BWPs := { ActiveBWP := omit, BwpArray := p_NR_DownlinkBWP_List_Type, InitialBWP_RedCap := omit } }, Uplink := omit } } } } };

After Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_DL_BWP_ConfigCommon_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_DownlinkBWP_List_Type p_NR_DownlinkBWP_List_Type) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status */ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink:= { //WA# FrequencyInfoDL := omit, SSPbchBlock := omit, PdschCellLevelConfig := omit, BWPs := { ActiveBWP := omit, BwpArray := p_NR_DownlinkBWP_List_Type, InitialBWP_RedCap := omit }, CsiConfig := omit //WA# }, Uplink := omit } } } } };

## Change 2

| Function name | f_TC_7_1_1_12_3_NR5GC () |
| --- | --- |
| Reason for change | The MAC_CellGroupConfig parameter containing the DRX parameters should be set to Omit at Step 20 as these are not required to be sent to the SS stack |
| Summary of change | Set the MAC_CellGroupConfig parameter to Omit. |
| TTCN module | MAC_PowerSaving_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_12_3_NR5GC() runs on NR5GC_PTC { //DRX adaptation / UE wakeup indication <<SKIPPED CODE>> //@siclog Step 20 siclog@ //SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) SRB.send(cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_NR(v_NextOccasion), cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, v_MeasConfig, v_V1530Ext)), -, -, -, cs_RlcBearerRouting_NR(nr_Cell1))); //f_NR_SS_MeasGapCtrlConfig(nr_Cell1, cs_NR_MeasGapCtrl_Config(v_MeasGapUE), cs_TimingInfo_NR(v_Timing)); @sic R5s230309 Ch 4.3 sic@ f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig))); //@sic R5s230001 Ch1.6, R5s230309 Ch4.1 sic@ f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); //@sic R5s230001 Ch1.8 sic@ <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_12_3_NR5GC() runs on NR5GC_PTC { //DRX adaptation / UE wakeup indication <<SKIPPED CODE>> //@siclog Step 20 siclog@ //SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) SRB.send(cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_NR(v_NextOccasion), cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, v_MeasConfig, v_V1530Ext)), -, -, -, cs_RlcBearerRouting_NR(nr_Cell1))); //f_NR_SS_MeasGapCtrlConfig(nr_Cell1, cs_NR_MeasGapCtrl_Config(v_MeasGapUE), cs_TimingInfo_NR(v_Timing)); @sic R5s230309 Ch 4.3 sic@ f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); //@sic R5s230001 Ch1.6, R5s230309 Ch4.1 sic@ WA# f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); //@sic R5s230001 Ch1.8 sic@ <<SKIPPED CODE>>

# Branches executed

The test case was executed on NR band n7 using NR ciphering nea2 and integrity algorithm nia2 on Keysight platform and on NR band n78 using NR ciphering nea1 and integrity algorithm nia1 on R&S platform.

# Execution Log Files

## MediaTek MT6895

The MediaTek MT6895 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_1_12_3_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_1_12_3_PIXIT.xml

## MediaTek MT6893

The MediaTek MT6893 UE passed this test case on R&S 5G Conformance Test Platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log and PICS/PIXIT parameter file:

ttcn_tli.log

# References

| [1] | R5s230509: NR5GC FR1: NR5GC FR1: Re-verification of Rel-16 Power Saving test case 7.1.1.12.3 |
| --- | --- |