3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230510

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3126 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR RRC testcases 8.1.5.8.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - As agreed in MCC160 comment for TTCN CR R5s230375, the implementation is done in IWD 23wk24. However, the if condition is not correct. It is implemented as below: if (not(f_NR_CellInfo_GetSCS(p_PCell) != kHz15)) { //@sic R5s230375 sic@ The *not* at the beginning of the condition reverses the logic of testing the PCell for not being in 15 Khz. The condition should have been as follows: if (f_NR_CellInfo_GetSCS(p_PCell) != kHz15) { - The above logic should also be implemented before Steps 6a5 - 6a7 for the RRC Resume procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected the IF condition. - Implemented the same logic for the RRC Resume procedure before Steps 6a5 – 6a7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case in FDD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.2.1.NR5GC, 8.1.5.8.2.2.NR5GC, 8.1.5.8.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_5_8_2_x_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - As agreed in MCC160 comment for TTCN CR R5s230375, the implementation is done in IWD 23wk24. However, the if condition is not correct. It is implemented as below: if (not(f_NR_CellInfo_GetSCS(p_PCell) != kHz15)) { //@sic R5s230375 sic@ The *not* at the beginning of the condition reverses the logic of testing the PCell for not being in 15 Khz. The condition should have been as follows: if (f_NR_CellInfo_GetSCS(p_PCell) != kHz15) { - The above logic should also be implemented before Steps 6a5 - 6a7 for the RRC Resume procedure |
| Summary of change | - Corrected the IF condition. - Implemented the same logic for the RRC Resume procedure before Steps 6a5 – 6a7. |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { <<SKIPPED CODE>> v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); if (not(f_NR_CellInfo_GetSCS(p_PCell) != kHz15)) { //@sic R5s230375 sic@ v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later } else { //Allow more time for FDD/TDD SCS=15kHz - k1=7 - reconfigure SS after UL HARQ feedback is received by SS v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 10); // 10 ms later } f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); <<SKIPPED CODE>> // @siclog "Step6a4" siclog@ // The UE transmits an RRCResumeRequest message. f_NR_RRC_ResumeRequest_Def(p_PCell); v_SCGConfig := f_NR_GetSCGConfigResume(p_PCell); // condition RESUME v_SCGConfig.mac_CellGroupConfig := v_SCGConfigCA.mac_CellGroupConfig; // condition SCell_Add v_SCGConfig.sCellToAddModList := v_SCGConfigCA.sCellToAddModList; // condition SCell_Add for (i:=0; i < lengthof(v_ExistingDRBs); i:=i+1) { v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit); } v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList); f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Config, cs_TimingInfo_Now); //@sic R5s220835 sic@ //@sic R5s210620 R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); <<SKIPPED CODE>>

After Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { <<SKIPPED CODE>> v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); if (f_NR_CellInfo_GetSCS(p_PCell) != kHz15) { //@sic R5s230375 sic@ WA# v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later } else { //Allow more time for FDD/TDD SCS=15kHz - k1=7 - reconfigure SS after UL HARQ feedback is received by SS v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 10); // 10 ms later } f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); <<SKIPPED CODE>> // @siclog "Step6a4" siclog@ // The UE transmits an RRCResumeRequest message. f_NR_RRC_ResumeRequest_Def(p_PCell); v_SCGConfig := f_NR_GetSCGConfigResume(p_PCell); // condition RESUME v_SCGConfig.mac_CellGroupConfig := v_SCGConfigCA.mac_CellGroupConfig; // condition SCell_Add v_SCGConfig.sCellToAddModList := v_SCGConfigCA.sCellToAddModList; // condition SCell_Add for (i:=0; i < lengthof(v_ExistingDRBs); i:=i+1) { v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit); } v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList); f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Config, cs_TimingInfo_Now); //@sic R5s220835 sic@ //@sic R5s210620 R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); if (f_NR_CellInfo_GetSCS(p_PCell) != kHz15) { //@sic R5s230375 sic@ WA# v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later } else { //Allow more time for FDD/TDD SCS=15kHz - k1=7 - reconfigure SS after UL HARQ feedback is received by SS v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 10); // 10 ms later } f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); <<SKIPPED CODE>>