3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230516

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3130 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC testcase 8.2.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of the function fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody , at step 4 cr_508_CounterCheckResponse is expected to have the entries for v_MCG_DRB_Id and v_SCG_DRB_Id. In the counter check DL message , count is provided as “0” for both MCG and SCG DRB. - COUNT value sent in cs_508_CounterCheck message is “0” for MCG DRB - There is no DRB tx/rx happen on MCG DRB , so UE side COUNT value also will remain as “0” for MCG DRB - Due to COUNT value being same between the ones indicated in drb_CountMSB_InfoList and the one in UE side , UE will send counter response without filling drb-CountInfoList as the highlighted condition is failing in below spec snippet . As per TS 136 331 - V17.4.0 section 5.3.6.3 Reception of the CounterCheck message by the UE Upon receiving the CounterCheck message, the UE shall: 1> for each DRB that is established: … <<skipped text>>… 2> else if, for at least one direction, the most significant bits of the COUNT are different from the value indicated in the drb-CountMSB-InfoList: 3> if the DRB is configured with E-UTRA PDCP: 4> include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of the corresponding COUNT; 3> else if the DRB is configured with NR PDCP: 4> include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 (specified in TS 38.323 [83]), respectively; It is proposed not to check the value of drb_CountInfoList , when receiving Counter Check Response |  |  |  |  |  |  |  |  |
| Summary of change: |  | When receiving Counter Check Response , drb_CountInfoList value is not checked by TTCN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody

| Function name | fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody , at step 4 cr_508_CounterCheckResponse is expected to have the entries for v_MCG_DRB_Id and v_SCG_DRB_Id. In the counter check DL message , count is provided as “0” for both MCG and SCG DRB. - COUNT value sent in cs_508_CounterCheck message is “0” for MCG DRB - There is no DRB tx/rx happen on MCG DRB , so UE side COUNT value also will remain as “0” for MCG DRB - Due to COUNT value being same between the ones indicated in drb_CountMSB_InfoList and the one in UE side , UE will send counter response without filling drb-CountInfoList as the highlighted condition is failing in below spec snippet . As per TS 136 331 - V17.4.0 section 5.3.6.3 Reception of the CounterCheck message by the UE Upon receiving the CounterCheck message, the UE shall: - for each DRB that is established: … <<skipped text>> … 2> else if, for at least one direction, the most significant bits of the COUNT are different from the value indicated in the drb-CountMSB-InfoList: 3> if the DRB is configured with E-UTRA PDCP: 4> include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of the corresponding COUNT; 3> else if the DRB is configured with NR PDCP: 4> include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 (specified in TS 38.323 [83]), respectively; It is proposed not to check the value of drb_CountInfoList , when receiving Counter Check Response |
| Summary of change | When receiving Counter Check Response , drb_CountInfoList value is not checked by TTCN |
| TTCN module | ttcn/develop/ENDC/8_2_2/RRCReconfig_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { // @sic R5-191203 R5-201204 R5-213472 R5s210730 sic@ << SKIPPED CODE >> interleave { []SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1, // Table 8.2.2.2.1.3.2-3: Parallel behaviour tsc_SRB1, cr_508_CounterCheckResponse(-, {cr_DRB_Count_Info(v_MCG_DRB_Id), cr_DRB_Count_Info(v_SCG_DRB_Id)}), -, crs_RlcBearerRouting_Eutra(eutra_Cell1))) {}; []SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,// Table 8.2.2.2.1.3.3-9 tsc_SRB1, cr_508_CounterCheckResponse(-, {cr_DRB_Count_Info(v_MCG_DRB_Id), cr_DRB_Count_Info(v_SCG_DRB_Id)}), -, cr_RlcBearerRouting_NR(nr_Cell1))) {}; } << SKIPPED CODE >> }

After Change:

function fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { // @sic R5-191203 R5-201204 R5-213472 R5s210730 sic@ << SKIPPED CODE >> interleave { []SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1, // Table 8.2.2.2.1.3.2-3: Parallel behaviour tsc_SRB1, cr_508_CounterCheckResponse(-, /*{cr_DRB_Count_Info(v_MCG_DRB_Id), cr_DRB_Count_Info(v_SCG_DRB_Id)}*/?), //WA#WI=1106046 -, crs_RlcBearerRouting_Eutra(eutra_Cell1))) {}; []SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,// Table 8.2.2.2.1.3.3-9 tsc_SRB1, cr_508_CounterCheckResponse(-, /*{cr_DRB_Count_Info(v_MCG_DRB_Id), cr_DRB_Count_Info(v_SCG_DRB_Id)}*/?), //WA#WI=1106046 -, cr_RlcBearerRouting_NR(nr_Cell1))) {}; << SKIPPED CODE >> }