3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230521

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0914 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS eCall over IMS testcase 11.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify IMS 5GS test case 11.4 using IWD 23wk24 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS test case 11.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to function f_ECall_MSDUpdate_AnnexC47	3

3	Branches executed	4

4	Execution Log Files	5

4.1	HiSilicon Balong 5000	5

5	References	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 11.4 which is part of the iwd-TTCN3-B2022-09_D23wk24.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

## Verification Test Summary

Test Case:	11.4.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk24

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Correction to function f_ECall_MSDUpdate_AnnexC47

| Template name | f_ECall_MSDUpdate_AnnexC47 |
| --- | --- |
| Reason for change | In function f_IMS_MessageHeader_Request_CheckVia an input parameter referring to a BYE SIP message is used. Inputparameter used shold be INFO instead that is the SIP message just received and target of the content to be checked. |
| Summary of change | Changed the input paramenter to be checked inside function f_IMS_MessageHeader_Request_CheckVia. |
| TTCN module | Common\IMS\IMS_eCall_Functions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> var charstring v_CidUrl_Step6; var boolean v_ConditionA4 := match(tsc_EmergencyCallData_DataType_invalidMSD, p_EmergencyCallData_Control_Step4.choice.request.datatype); // Step 4. SS sends SIP_INFO to request MSD update v_MessageHeader_Step4 := f_IMS_ECall_InfoRequest_MessageHeaderTX(v_BoundaryStep4, v_CidUrl_Step4); v_MessageBody_Step4 := f_IMS_ECall_MIME_MessageBodyTX(v_BoundaryStep4, v_ContentId, p_EmergencyCallData_Control_Step4); IMS_Client.send(cas_IMS_Info_Request(v_RoutingInfo_DL, cs_INFO_Request(v_ContactUrlUE, v_MessageHeader_Step4, v_MessageBody_Step4))); //Step 5. UE responds with 200 OK v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); f_IMS_ReceiveResponse(v_MessageHeader_Step4, v_Protocol, cr_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Step4))); // Step 6. UE sends SIP_INFO with MSD update. v_MessageHeader_Step6 := f_IMS_ECall_InfoRequest_MessageHeaderRX(v_ConditionA4); /* @sic R5-216339: v_ConditionA4 sic@ */ v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Info_Request(cr_INFO_Request(p_ContactUriSS, v_MessageHeader_Step6, cr_MessageBody_MIME))); /* @sic R5-216339: p_ContactUriSS sic@ */ v_CidUrl_Step6 := v_IMS_DATA_REQ.Request.Info.msgHeader.callInfo.callInfoBody[0].url; f_IMS_MessageHeader_Request_CheckVia(v_IMS_DATA_REQ.Request.Bye.msgHeader, v_IMS_DATA_REQ.RoutingInfo.Protocol); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> var charstring v_CidUrl_Step6; var boolean v_ConditionA4 := match(tsc_EmergencyCallData_DataType_invalidMSD, p_EmergencyCallData_Control_Step4.choice.request.datatype); // Step 4. SS sends SIP_INFO to request MSD update v_MessageHeader_Step4 := f_IMS_ECall_InfoRequest_MessageHeaderTX(v_BoundaryStep4, v_CidUrl_Step4); v_MessageBody_Step4 := f_IMS_ECall_MIME_MessageBodyTX(v_BoundaryStep4, v_ContentId, p_EmergencyCallData_Control_Step4); IMS_Client.send(cas_IMS_Info_Request(v_RoutingInfo_DL, cs_INFO_Request(v_ContactUrlUE, v_MessageHeader_Step4, v_MessageBody_Step4))); //Step 5. UE responds with 200 OK v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); f_IMS_ReceiveResponse(v_MessageHeader_Step4, v_Protocol, cr_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Step4))); // Step 6. UE sends SIP_INFO with MSD update. v_MessageHeader_Step6 := f_IMS_ECall_InfoRequest_MessageHeaderRX(v_ConditionA4); /* @sic R5-216339: v_ConditionA4 sic@ */ v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Info_Request(cr_INFO_Request(p_ContactUriSS, v_MessageHeader_Step6, cr_MessageBody_MIME))); /* @sic R5-216339: p_ContactUriSS sic@ */ v_CidUrl_Step6 := v_IMS_DATA_REQ.Request.Info.msgHeader.callInfo.callInfoBody[0].url; f_IMS_MessageHeader_Request_CheckVia(v_IMS_DATA_REQ.Request.Info.msgHeader, v_IMS_DATA_REQ.RoutingInfo.Protocol); <<SKIPPED CODE>>

# Branches executed

The test case was executed on NR5GC band n41 with and using 128-IEA1 Integrity and 128-NEA1 ciphering algorithms.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_11_4_NR5GC_Hisilicon.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s230522: Supporting information for addition of IMS 5GS eCall over IMS testcase 11.4 |
| --- | --- |