3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230547

# Online, , 13th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3146 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NR5GC testcases 6.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This test may FAIL at Step 17 with cause “Registration Complete Message Failed” upon reception of Registration Complete message containing steering of roaming information. As specified in TS 38.523-1 Table 6.3.1.1.5-3: REGISTRATION COMPLETE for NR Cell 13 (step 17, Table 6.3.1.1.4-1) The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information.and in TS 38.508-1 Table 4.7.1-8: REGISTRATION COMPLETE, the contents of the SOR Container are not checked. But in current TTCN Implementation, listInd_MSSI is hardcoded to be checked as ‘0’B. Additionally, as per TS 24.501: 5.5.1.2.4, MSSI bit is set according to UE support for SOR-CMCI. Therefore, any value should be acceptable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified expected SOR Header Container values in the Test Body of 6.3.1.1, to support processing of both ‘0’b or ‘1’b as value for listInd_MSSI IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 of test case 7.1.1.2.3 which is part of the NR5GC test suite.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_6_3_1_1_TestBody() |
| --- | --- |
| Reason for change | This test may FAIL at Step 17 with cause “Registration Complete Message Failed” upon reception of Registration Complete message containing steering of roaming information. As specified in TS 38.523-1 Table 6.3.1.1.5-3: REGISTRATION COMPLETE for NR Cell 13 (step 17, Table 6.3.1.1.4-1) The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information.and in TS 38.508-1 Table 4.7.1-8: REGISTRATION COMPLETE, the contents of the SOR Container are not checked. But in current TTCN Implementation, listInd_MSSI is hardcoded to be checked as ‘0’B. Additionally, as per TS 24.501: 5.5.1.2.4, MSSI bit is set according to UE support for SOR-CMCI. Therefore, any value should be acceptable. |
| Summary of change | Modified expected SOR Header Container values in the Test Body of 6.3.1.1 at Step 17, to support processing of both ‘0’b or ‘1’b as value for listInd_MSSI IE. |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_3_1_1_TestBody() runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202596 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202596 sic@ var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202596 sic@ var O16_Type v_Mac_UE; var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O;//@sic R5s200663 sic@ var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); timer t_Tmax; timer t_Wait:= 10.0; //@siclog "Step 1" siclog@ //Power on the UE f_UT_SwitchOnUE(UT); //@siclog "Steps 2-14" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing //steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception v_Mac_UE:=f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header,-,-,(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));//@sic R5s210658 R5s211298 sic@ //@siclog "Step 16" siclog@ //SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3) t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0)); //@siclog "Step 17" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message carrying acknowledgement of successful reception of the steering of roaming information SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(crs_NG_SOR_Header('0'B,'0'B, ‘0’B,'1'B),v_Mac_UE)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } //EXCEPTION: Step 18a1 is performed in pc_noOf_PDUsSameConnection > 0.//@sic R5-211379 sic@ //@siclog "Step 18a1" siclog@//@sic R5-211379 sic@ // The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2, //takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (nr_Cell13,-,tsc_NR_RbId_SRB1);//@sic R5s210576 sic@ } //@siclog "Steps 18a2-18a5a1" void siclog@//@sic R5-211379 sic@ //@siclog "Step 18A" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell13); //EXCEPTION: Step 18Ba1 is performed if pc_noOf_PDUsNewConnection > 0 and the UE starts PDU session establishment automatically //@siclog "Step 18Ba1" siclog@//@sic R5-211379 sic@ //The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5.2.2-4, takes place from step 2 to 9a1 performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection and then release the RRC Connection. if (pc_noOf_PDUsNewConnection > 0 ){ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Wait.stop; f_NR5GC_ExIdlePDUsession(nr_Cell13); } [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Wait.stop; } [] t_Wait.timeout{} } } //@siclog "Step 19" siclog@ //Check: Does the UE transmits an RRCSetupRequest on NR Cell 11 before tmax expires? alt { //@sic R5s201466 sic@ [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 19 PASS"); } [] t_Tmax.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 19 Failed"); } } //@siclog "Step 20-21" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 11. //@sic R5-202596 sic@ v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell11); if (v_TypeOfRegistration == Initial_Secure) { // @siclog "Steps 22a1 - 22a16a1" siclog@//@sic R5-202596 sic@ // IF 5GS registration type is set as Initial Registration in step 21, THEN steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 11. //@sic R5-204407 R5s201043 sic@ f_NR5GC_SorRegistration_Step5_20(nr_Cell11, v_Data, v_Header, crs_NG_SOR_Header('0'B,'0'B,'0'B,'1'B), //@sic R5-206314 sic@ -, Initial_Secure, v_GMM_MobilityInfo, v_ReceivedMsg); } else if(v_TypeOfRegistration == Mobility){ // @siclog "Steps 22b1 - 22b2a1" siclog@//@sic R5-202596 sic@ //ELSE IF 5GS registration type is set as Mobility Registration in step 21, THEN steps 5 to 6a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 11. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11,v_GMM_MobilityInfo); } }//End of fl_TC_6_3_1_1_TestBody

After Change:

function fl_TC_6_3_1_1_TestBody() runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202596 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202596 sic@ var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202596 sic@ var O16_Type v_Mac_UE; var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O;//@sic R5s200663 sic@ var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); timer t_Tmax; timer t_Wait:= 10.0; //@siclog "Step 1" siclog@ //Power on the UE f_UT_SwitchOnUE(UT); //@siclog "Steps 2-14" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing //steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception v_Mac_UE:=f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header,-,-,(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));//@sic R5s210658 R5s211298 sic@ //@siclog "Step 16" siclog@ //SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3) t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0)); //@siclog "Step 17" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message carrying acknowledgement of successful reception of the steering of roaming information SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(crs_NG_SOR_Header('0'B,'0'B, ?,'1'B),v_Mac_UE)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } //EXCEPTION: Step 18a1 is performed in pc_noOf_PDUsSameConnection > 0.//@sic R5-211379 sic@ //@siclog "Step 18a1" siclog@//@sic R5-211379 sic@ // The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2, //takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (nr_Cell13,-,tsc_NR_RbId_SRB1);//@sic R5s210576 sic@ } //@siclog "Steps 18a2-18a5a1" void siclog@//@sic R5-211379 sic@ //@siclog "Step 18A" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell13); //EXCEPTION: Step 18Ba1 is performed if pc_noOf_PDUsNewConnection > 0 and the UE starts PDU session establishment automatically //@siclog "Step 18Ba1" siclog@//@sic R5-211379 sic@ //The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5.2.2-4, takes place from step 2 to 9a1 performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection and then release the RRC Connection. if (pc_noOf_PDUsNewConnection > 0 ){ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Wait.stop; f_NR5GC_ExIdlePDUsession(nr_Cell13); } [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Wait.stop; } [] t_Wait.timeout{} } } //@siclog "Step 19" siclog@ //Check: Does the UE transmits an RRCSetupRequest on NR Cell 11 before tmax expires? alt { //@sic R5s201466 sic@ [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 19 PASS"); } [] t_Tmax.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 19 Failed"); } } //@siclog "Step 20-21" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 11. //@sic R5-202596 sic@ v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell11); if (v_TypeOfRegistration == Initial_Secure) { // @siclog "Steps 22a1 - 22a16a1" siclog@//@sic R5-202596 sic@ // IF 5GS registration type is set as Initial Registration in step 21, THEN steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 11. //@sic R5-204407 R5s201043 sic@ f_NR5GC_SorRegistration_Step5_20(nr_Cell11, v_Data, v_Header, crs_NG_SOR_Header('0'B,'0'B,'0'B,'1'B), //@sic R5-206314 sic@ -, Initial_Secure, v_GMM_MobilityInfo, v_ReceivedMsg); } else if(v_TypeOfRegistration == Mobility){ // @siclog "Steps 22b1 - 22b2a1" siclog@//@sic R5-202596 sic@ //ELSE IF 5GS registration type is set as Mobility Registration in step 21, THEN steps 5 to 6a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 11. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11,v_GMM_MobilityInfo); } }//End of fl_TC_6_3_1_1_TestBody