3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230570

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3102 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RACS test case 9.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the UE performs IMS registration during preamble, the IMS PTC will not be ready for a new IMS registration when UE is switched on again at step1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use function f_NR5GC_SwitchOnAndResetIMS() instead of f_UT_SwitchOnUE(UT) at step 1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE supporting IMS can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Correction to function fl_TC_9_1_9_1_Body

| Function name | fl_TC_9_1_9_1_Body() |
| --- | --- |
| Reason for change | If the UE performs IMS registration during preamble, the IMS PTC will not be ready for a new IMS registration when UE is switched on again at step1 |
| Summary of change | Use function f_NR5GC_SwitchOnAndResetIMS() instead of f_UT_SwitchOnUE(UT) at step 1 |
| TTCN module | RACS_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_9_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; //@siclog "Step 1" siclog@ f_UT_SwitchOnUE(UT); //@siclog "Step 2" siclog@ //Check: Does the UE transmit REGISTRATION REQUEST message including the UE radio capability ID 1? <<Skipped code>>

After Change:

function fl_TC_9_1_9_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ //Check: Does the UE transmit REGISTRATION REQUEST message including the UE radio capability ID 1? <<Skipped code>>