3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230578

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0095 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCPTT test case 5.8 to the MCX ATS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UPV/EHU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2023-06-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add MCPTT TC 5.8 to the MCX ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to the TTCN code required to obtain a pass using the MCX IPCAN test model. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to the MCX ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.MCPTT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Logs in R5s230485 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	4

4	Execution Log Files	4

4.1	Nemergent Solutions Client v7.0	4

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 5.8.MCPTT which is part of the ATS iwd-TTCN3-B2022-09_D23wk12 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Endika Aldecoa
	endika.aldecoa@ehu.eus

## Verification Test Summary

Test Case: 	5.8.MCPTT

ATS Version:	ATS iwd-TTCN3-B2022-09_D23wk12

System Simulator used:	MCSTaaSting Tester

UE used:	Nemergent Client v7.0 on COTS LTE UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | cr_MCX_OpenIdConnect_AuthenticationRequest |
| --- | --- |
| Reason for change | TS 33.180 Annex B.4.2.2 Authentication Request specifies the required parameters in the OpenId Connect Authentication request. TTCN code expects exactly those parameters to be present and no more parameters/tokens are allowed. This is in our view not correct since since Annex B.4.2.2 states “Other parameters defined by the OpenID Connect specification are optional.” Note: The corresponding prose CR has already been submitted and approved in Ran5#99 R5-232215. |
| Summary of change | Mandatory parameters are checked, but additional parameters sent by the Client are allowed. |
| TTCN module | MCX_OpenIdConnect.ttcn |
| MCC160 Comment |  |

- Before Change:

template (present) MCX_OpenIdConnect_ParameterList_Type cr_MCX_OpenIdConnect_AuthenticationRequest(MCX_ServiceType_Type p_ServiceType, charstring p_RedirectURI, charstring p_ClientId) := { /* Authentication Request (36.579-1 Table 5.5.4.10.1-1 */ /* @status APPROVED (MCX) */ cr_MCX_OpenIdConnect_NameValueParameter("response_type", "code"), cr_MCX_OpenIdConnect_NameValueParameter("client_id", p_ClientId), cr_MCX_OpenIdConnect_NameValueListParameter("scope", superset(all from f_MCX_OpenIdConnect_ScopeList(p_ServiceType))), cr_MCX_OpenIdConnect_NameValueParameter("redirect_uri", p_RedirectURI), cr_MCX_OpenIdConnect_NameValueParameter("state", ?), cr_MCX_OpenIdConnect_NameValueParameter("acr_values", "3gpp:acr:password"), cr_MCX_OpenIdConnect_NameValueParameter("code_challenge", ?), cr_MCX_OpenIdConnect_NameValueParameter("code_challenge_method", "S256") };

- After Change:

- template (present) MCX_OpenIdConnect_ParameterList_Type cr_MCX_OpenIdConnect_AuthenticationRequest(MCX_ServiceType_Type p_ServiceType, - charstring p_RedirectURI, - charstring p_ClientId) := - { /* Authentication Request (36.579-1 Table 5.5.4.10.1-1 */ - /* @status APPROVED (MCX) */ - cr_MCX_OpenIdConnect_NameValueParameter("response_type", "code"), - cr_MCX_OpenIdConnect_NameValueParameter("client_id", p_ClientId), - cr_MCX_OpenIdConnect_NameValueListParameter("scope", superset(all from f_MCX_OpenIdConnect_ScopeList(p_ServiceType))), - cr_MCX_OpenIdConnect_NameValueParameter("redirect_uri", p_RedirectURI), - cr_MCX_OpenIdConnect_NameValueParameter("state", ?), - cr_MCX_OpenIdConnect_NameValueParameter("acr_values", "3gpp:acr:password"), - cr_MCX_OpenIdConnect_NameValueParameter("code_challenge", ?), - cr_MCX_OpenIdConnect_NameValueParameter("code_challenge_method", "S256"), - * - };

# Branches executed

- TLSv1.2

- IPSec ipsec-3gpp;alg=hmac-sha-1-96;ealg=aes-cbc

- XML integrity and encrpytion according to  http://www.w3.org/2009/xmlenc11#aes128-gcm

- XML signatures according to  http://www.w3.org/2001/04/xmlenc#sha256

- MIKEY SAKKE/ECCSI (RFC 6507, 6508, 6509)

- LTE Band 1

- EEA2/EIA2 RRC/NAS ciphering/integrity algorithms

# Execution Log Files

## Nemergent Solutions Client v7.0

The Nemergent Solutions Client v7.0 on COTS LTE UE passed this test case against the MCSTaaSting Tester. The documentation below is enclosed as evidence of the successful test case run [1]:

- SS_Log_Coversheet.txt

- Signalling-TC5.8.MCPTT.html (RRC/NAS signalling, IP flow decrypted/decoded)

- MCX-PIXIT.cfg (Titan configuration and PIXITs)

- TLI Interface log

# References

| [1] | R5s230485: Supporting information for the addition of MCPTT test case 5.8 to the MCX ATS |
| --- | --- |