3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230595

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0187 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for GNSS 15s test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2023-07-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Incorrect template used for GPS Navigation model in the case of px_GnssScenario2012 set to false |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected template for GPS Navigation model |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Function f_POS_SelectTemplate_GenericAssistDataElement_GPS() |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_POS_SelectTemplate_GenericAssistDataElement_GPS() |
| --- | --- |
| Reason for change | Incorrect template used for GPS Navigation model in the case of px_GnssScenario2012 set to false |
| Summary of change | Corrected template for GPS Navigation model |
| TTCN module | CommonPOS_Functions.ttcn |
| MCC160 Comment |  |

Before change

function f_POS_SelectTemplate_GenericAssistDataElement_GPS(GNSS_GenericAssistData p_RetrievedGenericAssistData,

boolean p_UEB) return template (value) GNSS_GenericAssistDataElement

{

var template (value) GNSS_GenericAssistDataElement v_GenericAssistDataElement;

var integer i := f_POS_FindIndexForGNSSMethod(p_RetrievedGenericAssistData, cs_GNSS_ID_Gps); //Indicates the position of the GPS data in the retrieved data

if (i == -1) {//GPS element was not found in the list

FatalError (__FILE__, __LINE__, "Invalid CellId"); // This function is only called if PICS indicate GPS support. If GPS assistance data is not there, it is a FatalError

}

else {

//Include all possible elements in the template

if (px_GnssScenario2012) { // @sic R5-217256, R5s217257 sic@

v_GenericAssistDataElement := cs_GNSS_GenericAssistDataElement(cs_GNSS_ID_Gps,

-,

cs_NavigationModel_Gps2012(p_RetrievedGenericAssistData[i].gnss_NavigationModel),

cs_GNSS_AcquisitionAssistance(cs_GNSS_SignalID(fl_DetermineGPS_SignalID_GNSS_AcquAssist()), // GPS L1 C/A or GPS L5 @sic R5-186628 sic@

fl_Build_cs_GNSS_AcquisitionAssistList(tsc_IntList_1_11_17_20_23_28,

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.gnss_AcquisitionAssistList),

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.confidence_r10),

cs_GNSS_Almanac_GPS(p_RetrievedGenericAssistData[i].gnss_Almanac),

cs_GNSS_UTC_Model_1(p_RetrievedGenericAssistData[i].gnss_UTC_Model),

fl_Build_cs_GNSS_AuxiliaryInformation(tsc_IntList_1_11_17_20_23_28,

p_RetrievedGenericAssistData[i].gnss_AuxiliaryInformation));

}

else {

v_GenericAssistDataElement := cs_GNSS_GenericAssistDataElement(cs_GNSS_ID_Gps,

-,

cs_NavigationModel_Gps2012(p_RetrievedGenericAssistData[i].gnss_NavigationModel),

cs_GNSS_AcquisitionAssistance(cs_GNSS_SignalID(fl_DetermineGPS_SignalID_GNSS_AcquAssist()), // GPS L1 C/A or GPS L5 @sic R5-186628 sic@

fl_Build_cs_GNSS_AcquisitionAssistList(tsc_IntList_3_4_6_17_19_28, // @sic R5-215711 sic@

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.gnss_AcquisitionAssistList),

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.confidence_r10),

cs_GNSS_Almanac_GPS(p_RetrievedGenericAssistData[i].gnss_Almanac),

cs_GNSS_UTC_Model_1(p_RetrievedGenericAssistData[i].gnss_UTC_Model),

fl_Build_cs_GNSS_AuxiliaryInformation(tsc_IntList_3_4_6_17_19_28, // @sic R5-215711 sic@

p_RetrievedGenericAssistData[i].gnss_AuxiliaryInformation));

}

//Now remove those elements not needed depending on the case

if (not pc_A_GLONASS) {// UTC-Model provided only if GLONASS and at least one other GNSS supported by the UE.

v_GenericAssistDataElement.gnss_UTC_Model := omit;

}

if (not pc_A_GPS_L1C_A_MGPS) { // UE does not supports multiple GPS signals -> Do not include Auxiliary Information (37.571-2, Table 5.4.1.1-1)

v_GenericAssistDataElement.gnss_AuxiliaryInformation := omit;

}

if (p_UEB) { // UE-Based case

v_GenericAssistDataElement.gnss_AcquisitionAssistance := omit;

v_GenericAssistDataElement.gnss_Almanac := omit;

}

else { //UE-Assisted case

if (pc_GNSS_AA) { // UE supports GNSS Acquisition Assistance

v_GenericAssistDataElement.gnss_NavigationModel := omit;

v_GenericAssistDataElement.gnss_Almanac := omit;

}

else { // UE does not supports GNSS Acquisition Assistance

v_GenericAssistDataElement.gnss_AcquisitionAssistance := omit;

}

}

}

return v_GenericAssistDataElement;

}

After change

function f_POS_SelectTemplate_GenericAssistDataElement_GPS(GNSS_GenericAssistData p_RetrievedGenericAssistData,

boolean p_UEB) return template (value) GNSS_GenericAssistDataElement

{

var template (value) GNSS_GenericAssistDataElement v_GenericAssistDataElement;

var integer i := f_POS_FindIndexForGNSSMethod(p_RetrievedGenericAssistData, cs_GNSS_ID_Gps); //Indicates the position of the GPS data in the retrieved data

if (i == -1) {//GPS element was not found in the list

FatalError (__FILE__, __LINE__, "Invalid CellId"); // This function is only called if PICS indicate GPS support. If GPS assistance data is not there, it is a FatalError

}

else {

//Include all possible elements in the template

if (px_GnssScenario2012) { // @sic R5-217256, R5s217257 sic@

v_GenericAssistDataElement := cs_GNSS_GenericAssistDataElement(cs_GNSS_ID_Gps,

-,

cs_NavigationModel_Gps2012(p_RetrievedGenericAssistData[i].gnss_NavigationModel),

cs_GNSS_AcquisitionAssistance(cs_GNSS_SignalID(fl_DetermineGPS_SignalID_GNSS_AcquAssist()), // GPS L1 C/A or GPS L5 @sic R5-186628 sic@

fl_Build_cs_GNSS_AcquisitionAssistList(tsc_IntList_1_11_17_20_23_28,

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.gnss_AcquisitionAssistList),

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.confidence_r10),

cs_GNSS_Almanac_GPS(p_RetrievedGenericAssistData[i].gnss_Almanac),

cs_GNSS_UTC_Model_1(p_RetrievedGenericAssistData[i].gnss_UTC_Model),

fl_Build_cs_GNSS_AuxiliaryInformation(tsc_IntList_1_11_17_20_23_28,

p_RetrievedGenericAssistData[i].gnss_AuxiliaryInformation));

}

else {

v_GenericAssistDataElement := cs_GNSS_GenericAssistDataElement(cs_GNSS_ID_Gps,

-,

cs_NavigationModel_Gps(p_RetrievedGenericAssistData[i].gnss_NavigationModel),

cs_GNSS_AcquisitionAssistance(cs_GNSS_SignalID(fl_DetermineGPS_SignalID_GNSS_AcquAssist()), // GPS L1 C/A or GPS L5 @sic R5-186628 sic@

fl_Build_cs_GNSS_AcquisitionAssistList(tsc_IntList_3_4_6_17_19_28, // @sic R5-215711 sic@

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.gnss_AcquisitionAssistList),

p_RetrievedGenericAssistData[i].gnss_AcquisitionAssistance.confidence_r10),

cs_GNSS_Almanac_GPS(p_RetrievedGenericAssistData[i].gnss_Almanac),

cs_GNSS_UTC_Model_1(p_RetrievedGenericAssistData[i].gnss_UTC_Model),

fl_Build_cs_GNSS_AuxiliaryInformation(tsc_IntList_3_4_6_17_19_28, // @sic R5-215711 sic@

p_RetrievedGenericAssistData[i].gnss_AuxiliaryInformation));

}

//Now remove those elements not needed depending on the case

if (not pc_A_GLONASS) {// UTC-Model provided only if GLONASS and at least one other GNSS supported by the UE.

v_GenericAssistDataElement.gnss_UTC_Model := omit;

}

if (not pc_A_GPS_L1C_A_MGPS) { // UE does not supports multiple GPS signals -> Do not include Auxiliary Information (37.571-2, Table 5.4.1.1-1)

v_GenericAssistDataElement.gnss_AuxiliaryInformation := omit;

}

if (p_UEB) { // UE-Based case

v_GenericAssistDataElement.gnss_AcquisitionAssistance := omit;

v_GenericAssistDataElement.gnss_Almanac := omit;

}

else { //UE-Assisted case

if (pc_GNSS_AA) { // UE supports GNSS Acquisition Assistance

v_GenericAssistDataElement.gnss_NavigationModel := omit;

v_GenericAssistDataElement.gnss_Almanac := omit;

}

else { // UE does not supports GNSS Acquisition Assistance

v_GenericAssistDataElement.gnss_AcquisitionAssistance := omit;

}

}

}

return v_GenericAssistDataElement;

}