3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230596

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3159 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC UAC test case 11.3.9a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE will have no existing security context for the initial registration procedure at steps 4-22a1 in the case of test case 11.3.9a |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use condition Initial_NoSecurity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.9a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_11_3_9_TestBody() |
| --- | --- |
| Reason for change | UE will have no existing security context for the initial registration procedure at steps 4-22a1 in the case of test case 11.3.9a |
| Summary of change | Use condition Initial_NoSecurity |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_3_9_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_RadioBearerId_Type v_SRB:= tsc_NR_RbId_SRB2; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (omit) DNN v_DNN; var boolean v_ReceiveRRCSetupRequest; var default v_MyDefaultVar:= null; // @sic R5-224717 sic@ timer t_Wait := 11.2; // @sic R5-213507 sic@ timer t_WaitTime := 5.0; // @sic R5-214783 sic@ v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_DNNValue)); // @sic R5s210307 sic@ //@siclog "Step 3" siclog@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Step 4" siclog@ // Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is the same value in UL NAS TRANSPORT message in preamble within 20s? t_Wait.start; alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // @sic R5s210307 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } } // @siclog "Step 5" siclog@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.9.3.2-1/2. v_CellPowerList_T1 := { //@sic R5-205388 sic@ cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; f_NR_SetCellPowerList(v_CellPowerList_T1); if( f_GetTestcaseAttrib_NR_SNPN(testcasename())) //@sic R5-233351 sic@ { //@siclog "Step 4-22a1" siclog@ //Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR5GC_MobileInfo_Init(); // Need to 'reset' all stored lists as all DRBs and PDU sessions do not exist in the new SNPN f_NR5GC_RRC_IdleState1N_Def (nr_Cell12); } else { // @siclog "Step 6" siclog@ //The mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 12 and release RRC connection (Note 1) //Note 1: SS ignore the PDU SESSION ESTABLISHMENT REQUEST sent before SS release RRC connection. @sic R5-224717 sic@ v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(nr_Cell12)); f_NR5GC_MobilityRegistration(nr_Cell12, -, -, -, -, noRrcConnectionRelease); f_NR_RRCRelease(nr_Cell12); deactivate(v_MyDefaultVar); } // @sic R5-214783 sic@ // @siclog "Steps 6A - 6Bb1" siclog@ t_WaitTime.start; alt{ [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_MoData, ?))) { t_WaitTime.stop; f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221054 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Ba1"); } [] t_WaitTime.timeout { //@siclog "Step 6Bb2" siclog@ // Make the UE attempt to establish a PDU session for the DNN defined in Table 11.3.9.3.3-1. This can be done by an AT/MMI command. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Step 6Bb3" siclog@ //Check: Does the UE transmit a RRCSetupRequest message and establishmentCause is set to mo-Data? v_ReceiveRRCSetupRequest := f_NR_RRC_SetupRequest_Common( nr_Cell12, t_Wait, cr_NR_EstablishmentCause_MoData); if (v_ReceiveRRCSetupRequest) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Bb3"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6Bb3"); } } } //@siclog "Steps 9 - 14" siclog@ //Steps 3-8 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0000'B)) { // @sic R5s200654 R5s210307 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); //@siclog "Steps 15 - 16" siclog@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is set to the same value with the DNN value of Operator-defined access category definitions defined in Table 11.3.9.3.3-1 //The SS transmits PDU SESSION ESTABLISHMENT ACCEPT message. f_NR5GC_PDUSessionEstablishment (nr_Cell12, 1, -, -, - , v_DNN, -, v_DNN); }

After Change:

function fl_TC_11_3_9_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_RadioBearerId_Type v_SRB:= tsc_NR_RbId_SRB2; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (omit) DNN v_DNN; var boolean v_ReceiveRRCSetupRequest; var default v_MyDefaultVar:= null; // @sic R5-224717 sic@ timer t_Wait := 11.2; // @sic R5-213507 sic@ timer t_WaitTime := 5.0; // @sic R5-214783 sic@ v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_DNNValue)); // @sic R5s210307 sic@ //@siclog "Step 3" siclog@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Step 4" siclog@ // Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is the same value in UL NAS TRANSPORT message in preamble within 20s? t_Wait.start; alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // @sic R5s210307 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } } // @siclog "Step 5" siclog@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.9.3.2-1/2. v_CellPowerList_T1 := { //@sic R5-205388 sic@ cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; f_NR_SetCellPowerList(v_CellPowerList_T1); if( f_GetTestcaseAttrib_NR_SNPN(testcasename())) //@sic R5-233351 sic@ { //@siclog "Step 4-22a1" siclog@ //Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR5GC_MobileInfo_Init(); // Need to 'reset' all stored lists as all DRBs and PDU sessions do not exist in the new SNPN f_NR5GC_RRC_IdleState1N_Def (nr_Cell12,-,-, Initial_NoSecurity); } else { // @siclog "Step 6" siclog@ //The mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 12 and release RRC connection (Note 1) //Note 1: SS ignore the PDU SESSION ESTABLISHMENT REQUEST sent before SS release RRC connection. @sic R5-224717 sic@ v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(nr_Cell12)); f_NR5GC_MobilityRegistration(nr_Cell12, -, -, -, -, noRrcConnectionRelease); f_NR_RRCRelease(nr_Cell12); deactivate(v_MyDefaultVar); } // @sic R5-214783 sic@ // @siclog "Steps 6A - 6Bb1" siclog@ t_WaitTime.start; alt{ [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_MoData, ?))) { t_WaitTime.stop; f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221054 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Ba1"); } [] t_WaitTime.timeout { //@siclog "Step 6Bb2" siclog@ // Make the UE attempt to establish a PDU session for the DNN defined in Table 11.3.9.3.3-1. This can be done by an AT/MMI command. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Step 6Bb3" siclog@ //Check: Does the UE transmit a RRCSetupRequest message and establishmentCause is set to mo-Data? v_ReceiveRRCSetupRequest := f_NR_RRC_SetupRequest_Common( nr_Cell12, t_Wait, cr_NR_EstablishmentCause_MoData); if (v_ReceiveRRCSetupRequest) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Bb3"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6Bb3"); } } } //@siclog "Steps 9 - 14" siclog@ //Steps 3-8 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0000'B)) { // @sic R5s200654 R5s210307 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); //@siclog "Steps 15 - 16" siclog@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is set to the same value with the DNN value of Operator-defined access category definitions defined in Table 11.3.9.3.3-1 //The SS transmits PDU SESSION ESTABLISHMENT ACCEPT message. f_NR5GC_PDUSessionEstablishment (nr_Cell12, 1, -, -, - , v_DNN, -, v_DNN); }