3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230598

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0917 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS 5GS conferencing test case 8.34 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current TTCN implementation of function f_IMS_ThreeWaySession_SubscribeInvite() has no handling for expiry of the timer t_WaitForSubscribe, leading to test case failure if the UE does not subscribe to the conference event package. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add handling for timeout event |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may not pass this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.34.NR5GC, 8.35.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_IMS_ThreeWaySession_SubscribeInvite() |
| --- | --- |
| Reason for change | The current TTCN implementation has no handling for expiry of the timer t_WaitForSubscribe, leading to test case failure if the UE does not subscribe to the conference event package. |
| Summary of change | Add handling for timeout event |
| TTCN module | IMS_Procedures_Conferencing.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_ThreeWaySession_SubscribeInvite(ThreeWaySession_SessionInfo_Type p_SessionInfo_Dialog1, ThreeWaySession_SessionInfo_Type p_SessionInfo_Dialog2, InviteUserToConference_StateInfo_Type p_Dialog1_StateInfo, InviteUserToConference_StateInfo_Type p_Dialog2_StateInfo, charstring p_FinalConferenceUri, CharStringList_Type p_MediaTypeList) runs on IMS_PTC return template (omit) SUBSCRIBE_Request { ………………………………………………… //-------------------------------------------- // DIALOG 2 //-------------------------------------------- // Terminate dialog 2 [v_WaitForBYE_Dialog2] a_IMS_ReceiveByeSendOK(secondDialog, p_SessionInfo_Dialog2.ContactUrl, v_MessageHeader_Bye_Dialog2) { v_WaitForBYE_Dialog2 := false; v_Dialog2_Terminated := true; /* @sic R5s150910 sic@ */ } } v_Continue := v_WaitForBYE_Dialog1 or v_WaitForBYE_Dialog2 or (v_Dialog1_StateInfo_ByRef.State != finished) or (v_Dialog2_StateInfo_ByRef.State != finished) or (isvalue(v_SubscribeRequest) and (v_ConferenceEventPackage_SubscribeInfo_ByRef.State != subscribed)) // isvalue(v_SubscribeRequest): UE has sent the (optional) SUBSCRIBE or t_WaitForSubscribe.running; // @sic R5s221272: t_WaitForSubscribe sic@ } while (v_Continue); return v_SubscribeRequest; }

After Change:

function f_IMS_ThreeWaySession_SubscribeInvite(ThreeWaySession_SessionInfo_Type p_SessionInfo_Dialog1, ThreeWaySession_SessionInfo_Type p_SessionInfo_Dialog2, InviteUserToConference_StateInfo_Type p_Dialog1_StateInfo, InviteUserToConference_StateInfo_Type p_Dialog2_StateInfo, charstring p_FinalConferenceUri, CharStringList_Type p_MediaTypeList) runs on IMS_PTC return template (omit) SUBSCRIBE_Request { ……………………………………………………… //-------------------------------------------- // DIALOG 2 //-------------------------------------------- // Terminate dialog 2 [v_WaitForBYE_Dialog2] a_IMS_ReceiveByeSendOK(secondDialog, p_SessionInfo_Dialog2.ContactUrl, v_MessageHeader_Bye_Dialog2) { v_WaitForBYE_Dialog2 := false; v_Dialog2_Terminated := true; /* @sic R5s150910 sic@ */ } [] t_WaitForSubscribe.timeout {} } v_Continue := v_WaitForBYE_Dialog1 or v_WaitForBYE_Dialog2 or (v_Dialog1_StateInfo_ByRef.State != finished) or (v_Dialog2_StateInfo_ByRef.State != finished) or (isvalue(v_SubscribeRequest) and (v_ConferenceEventPackage_SubscribeInfo_ByRef.State != subscribed)) // isvalue(v_SubscribeRequest): UE has sent the (optional) SUBSCRIBE or t_WaitForSubscribe.running; // @sic R5s221272: t_WaitForSubscribe sic@ } while (v_Continue); return v_SubscribeRequest; }