3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230637

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3176 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the CA Functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the current TTCN implementation of the CA scenarios , - In case of DL-CA : - when adding the Scell , Peer Reconfiguration message is sent to UE by excluding dedicated PUCCH,PUSCH configurations . - But on the SS side, when DL only capable Scell is created, dedicated PUCCH,PUSCH configurations are provided . - This results in the mismatch in the configuration of Scell between SS and UE - In case of UL-CA : - when adding the Scell , Peer Reconfiguration message is sent to UE by excluding dedicated PUCCH configuration . - But on the SS side, when ULCA capable Scell is created, dedicated PUCCH,PUSCH configurations are provided . - This results in the mismatch in the configuration of Scell between SS and UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | To match the Scell configurations between SS and the UE - In case of DL-CA , dedicated PUCCH and PUSCH are set to Omit when creating the Scell on SS side. - In case of UL-CA , dedicated PUCCH is set to Omit when creating the Scell on SS side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Scell configurations provided will be different between SS and the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All the CA test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This proposal does not fully solve the mismatch in Scell configurations for all CA cases. - There are Flexible CA cases where the Scell becomes the Pcell during the test body (8.1.4.1.7.x.NR5GC, 8.1.4.1.9.x.NR5GC, 8.2.4.2.1.x.ENDC). - In these cases, the Scell is created as an Spcell (with dedicated PUSCH and PUCCH). When the Spcell is converted to an Scell during the test body, we need to find a solution to match the Scell configuration between the SS and the UE. Further discussion is needed to address this topic. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to the Function f_NR_CellConfig_Def	3

1.2	Correction to the Function f_NR_CellConfig_SCell	6

1.3	Correction to the Function f_TC_7_1_1_3_8_ENDC_Common	7

1.4	Correction to the Function f_TC_7_1_1_3_8_NR_Common	9

1.5	Correction to the Function f_TC_8_1_5_7_1_X_NR_Common	10

1.6	Correction to the Function f_TC_7_1_1_7_1_ENDC_Common	12

1.7	Correction to the Function f_TC_7_1_1_7_1_NR_Common	14

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_NR_CellConfig_Def

| Function name | f_NR_CellConfig_Def |
| --- | --- |
| Reason for change | When Creating Scell , using the function f_NR_CellConfig_Def should have the information about whether it is UL CA capable Scell (and) if it UL CA capable Scell, whether it supports PUCCH . |
| Summary of change | New parameters introduced in the function f_NR_CellConfig_Def (boolean p_UL_CA and boolean p_UL_CA_ConfigPUCCH ). These new parameters help to provide the information whether it is UL CA capable Scell (and) if it UL CA capable Scell, whether it supports PUCCH |
| TTCN module | ttcn/develop/Common/NR/NR_ConfigurationSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_CellConfig_Def(NR_CellId_Type p_NR_CellId, template (omit) NR_CellCA_Capability_Type p_CA_CellCapability := None, boolean p_NRDC_SCG_Cell := false, boolean p_ENDC_SpecificCell := false, //Set to true ONLY for a cell to be configured as ENDC in an NR5GC test environment integer p_SIB1_Offset := tsc_NR_SIB1_Offset1 ) runs on NR_BASE_PTC { // @sic R5s200038: set default value of p_CA_CellCapability to None sic@ // @sic R5s210891: added p_NRDC_SCG_Cell sic@ // @sic R5s220853: added p_ENDC_SpecificCell sic@ //@sic R5-233776: added p_SIB1_Offset - temporary parameter FFS sic@ var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var template(omit) NR_CellConfigPhysicalLayerDownlink_Type v_NR_CellConfigPhysicalLayerDownlink := cs_NR_CellConfigPhysicalLayerDownlink(v_NR_CellInfo); var template(omit) NR_CellConfigPhysicalLayerUplink_Type v_NR_CellConfigPhysicalLayerUplink := omit; var template(omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(p_NR_CellId, -, p_SIB1_Offset); var template(omit) NR_PcchConfig_Type v_NR_PcchConfig := f_NR_InitialisePcchConfig(p_NR_CellId); var template(omit) NR_RachProcedureConfig_Type v_NR_RachProcedure; if (not(f_NR_IsUL_FrequencyAvailable(p_NR_CellId))) { //SDL or SCell with no UL @sic R5-226041 sic@ v_NR_CellConfigPhysicalLayerUplink := omit; v_NR_RachProcedure := omit } else { v_NR_CellConfigPhysicalLayerUplink := cs_NR_CellConfigPhysicalLayerUplink(v_NR_CellInfo); v_NR_RachProcedure := f_NR_InitialiseRachProcedure(p_NR_CellId, p_NRDC_SCG_Cell); << SKIPPED CODE >> }

After Change:

function f_NR_CellConfig_Def(NR_CellId_Type p_NR_CellId, template (omit) NR_CellCA_Capability_Type p_CA_CellCapability := None, boolean p_NRDC_SCG_Cell := false, boolean p_ENDC_SpecificCell := false, //Set to true ONLY for a cell to be configured as ENDC in an NR5GC test environment integer p_SIB1_Offset := tsc_NR_SIB1_Offset1, boolean p_UL_CA := false, boolean p_UL_CA_ConfigPUCCH := true ) runs on NR_BASE_PTC { // @sic R5s200038: set default value of p_CA_CellCapability to None sic@ // @sic R5s210891: added p_NRDC_SCG_Cell sic@ // @sic R5s220853: added p_ENDC_SpecificCell sic@ //@sic R5-233776: added p_SIB1_Offset - temporary parameter FFS sic@ var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var template(omit) NR_CellConfigPhysicalLayerDownlink_Type v_NR_CellConfigPhysicalLayerDownlink := cs_NR_CellConfigPhysicalLayerDownlink(v_NR_CellInfo); var template(omit) NR_CellConfigPhysicalLayerUplink_Type v_NR_CellConfigPhysicalLayerUplink := omit; var template(omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(p_NR_CellId, -, p_SIB1_Offset); var template(omit) NR_PcchConfig_Type v_NR_PcchConfig := f_NR_InitialisePcchConfig(p_NR_CellId); var template(omit) NR_RachProcedureConfig_Type v_NR_RachProcedure; var boolean p_Scell_Present := false; if(match(Scell_Active,p_CA_CellCapability) or match(Scell_Inactive,p_CA_CellCapability)) { p_Scell_Present := true; } if (not(f_NR_IsUL_FrequencyAvailable(p_NR_CellId))) { //SDL or SCell with no UL @sic R5-226041 sic@ v_NR_CellConfigPhysicalLayerUplink := omit; v_NR_RachProcedure := omit } else { v_NR_CellConfigPhysicalLayerUplink := cs_NR_CellConfigPhysicalLayerUplink(v_NR_CellInfo); v_NR_RachProcedure := f_NR_InitialiseRachProcedure(p_NR_CellId, p_NRDC_SCG_Cell); } //UL CA present but with NO PUCCH if( ( p_UL_CA and (not(p_UL_CA_ConfigPUCCH)) ) ) { v_NR_CellConfigPhysicalLayerUplink.Uplink.Config.BWPs.BwpArray[tsc_NR_BWP_Id].Dedicated.R15.pucch_Config := omit; //UL CONFIG DEDICATED } //DL CA present if( (not(p_UL_CA)) and (p_Scell_Present) ) { v_NR_CellConfigPhysicalLayerUplink.Uplink.Config.BWPs.BwpArray[tsc_NR_BWP_Id].Dedicated.R15.pucch_Config := omit; //UL CONFIG DEDICATED PUCCH v_NR_CellConfigPhysicalLayerUplink.Uplink.Config.BWPs.BwpArray[tsc_NR_BWP_Id].Dedicated.R15.pusch_Config := omit; //UL CONFIG DEDICATED PUCCH } << SKIPPED CODE >> }

## Correction to the Function f_NR_CellConfig_SCell

| Function name | f_NR_CellConfig_SCell |
| --- | --- |
| Reason for change | When Creating Scell , using the function f_NR_CellConfig_Def should have the information about whether it is UL CA capable Scell (and) if it UL CA capable Scell, whether it supports PUCCH |
| Summary of change | New parameters introduced in the function f_NR_CellConfig_Def (boolean p_UL_CA and boolean p_UL_CA_ConfigPUCCH ). These new parameters help to provide the information whether it is UL CA capable Scell (and) if it UL CA capable Scell, whether it supports PUCCH |
| TTCN module | ttcn/develop/Common/NR/NR_CommonFunctions_CA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_CellConfig_SCell(NR_CellId_Type p_SCellId, template (value) NR_CellCA_Capability_Type p_CA_CellCapability := Scell_Active) runs on NR_BASE_PTC { var template (value) NR_RadioBearerList_Type v_SrbList := {cs_NR_SS_SRB0_Config}; f_NR_CellConfig_Def(p_SCellId, p_CA_CellCapability); if (not f_GetTestcaseAttrib_ENDC(testcasename()) and f_NR_IsUL_FrequencyAvailable(p_SCellId)) { //@sic R5-226041 sic@ f_NR_SS_CommonRadioBearerConfig(p_SCellId, v_SrbList); }

After Change:

function f_NR_CellConfig_SCell(NR_CellId_Type p_SCellId, template (value) NR_CellCA_Capability_Type p_CA_CellCapability := Scell_Active, boolean p_UL_CA := false, boolean p_UL_CA_ConfigPUCCH := true) runs on NR_BASE_PTC { var template (value) NR_RadioBearerList_Type v_SrbList := {cs_NR_SS_SRB0_Config}; f_NR_CellConfig_Def(p_SCellId, p_CA_CellCapability, -, -,-, p_UL_CA, p_UL_CA_ConfigPUCCH); if (not f_GetTestcaseAttrib_ENDC(testcasename()) and f_NR_IsUL_FrequencyAvailable(p_SCellId)) { //@sic R5-226041 sic@ f_NR_SS_CommonRadioBearerConfig(p_SCellId, v_SrbList); }

## Correction to the Function f_TC_7_1_1_3_8_ENDC_Common

| Function name | f_TC_7_1_1_3_8_ENDC_Common |
| --- | --- |
| Reason for change | When Creating Scell using the function f_NR_CellConfig_Def , third parameter (p_UL_CA) has to be “True” because pc_EN_DC_NR_UL_2CC is mandatory for this test case and 4th parameter (p_UL_CA_ConfigPUCCH) has to be “False” as UL CA capable Scell is configured without PUCCH on the Peer reconfiguration message. |
| Summary of change | Scell is created with UL_CA enabled but without PUCCH configured |
| TTCN module | ttcn/develop/ENDC/7_1_1/MAC_CA_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_8_ENDC_Common(NR_CellId_Type p_PSCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested) runs on ENDC_NR_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP; var SubFrameTiming_Type v_TimingUL; // Multi PR initialisation is taken care in Test body. f_ENDC_NR_Init_CA(p_NR_CA_Tested, {p_PSCell, p_SCell}, false);//@sic R5s211385 R5s210894 sic@ //Create and configure NR cell1s f_NR_CellConfig_PCell_Def(p_PSCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(p_PSCell, MCG_Only); << SKIPPED CODE >> }

After Change:

NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested) runs on ENDC_NR_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP; var SubFrameTiming_Type v_TimingUL; // Multi PR initialisation is taken care in Test body. f_ENDC_NR_Init_CA(p_NR_CA_Tested, {p_PSCell, p_SCell}, false);//@sic R5s211385 R5s210894 sic@ //Create and configure NR cell1s f_NR_CellConfig_PCell_Def(p_PSCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active, true, false); //WA#WI=1184162 // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(p_PSCell, MCG_Only); << SKIPPED CODE >> }

## Correction to the Function f_TC_7_1_1_3_8_NR_Common

| Function name | f_TC_7_1_1_3_8_NR_Common |
| --- | --- |
| Reason for change | When Creating Scell using the function f_NR_CellConfig_Def , third parameter (p_UL_CA) has to be “True” because pc_UL_NR_CA_2CC is mandatory for this test case and 4th parameter (p_UL_CA_ConfigPUCCH) has to be “False” as UL CA capable Scell is configured without PUCCH on the Peer reconfiguration message. |
| Summary of change | Scell is created with UL_CA enabled but without PUCCH configured |
| TTCN module | ttcn/develop/NR5GC/7_1_1/MAC_CA_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_8_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//UE power headroom reporting / SCell activation / DL pathloss change reporting var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var boolean v_RRC_MsgRcvd; var boolean v_PHR_Rcvd; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; //@sic R5s220848 sic@ var SubcarrierSpacing v_SCS;// // Multi PR initialisation is taken care in Test body. f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested); //@sic R5s211385 sic@ f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCell); //@sic R5s210315 sic@ // Create and configure cells f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); << SKIPPED CODE >> }

After Change:

function f_TC_7_1_1_3_8_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//UE power headroom reporting / SCell activation / DL pathloss change reporting var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var boolean v_RRC_MsgRcvd; var boolean v_PHR_Rcvd; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; //@sic R5s220848 sic@ var SubcarrierSpacing v_SCS;// // Multi PR initialisation is taken care in Test body. f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested); //@sic R5s211385 sic@ f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCell); //@sic R5s210315 sic@ // Create and configure cells f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active,true,false); //WA#WI=1184162 << SKIPPED CODE >> }

## Correction to the Function f_TC_8_1_5_7_1_X_NR_Common

| Function name | f_TC_8_1_5_7_1_X_NR_Common |
| --- | --- |
| Reason for change | When Creating Scell using the function f_NR_CellConfig_Def , third parameter (p_UL_CA) has to be “True” because pc_UL_NR_CA_2CC is mandatory for this test case and 4th parameter (p_UL_CA_ConfigPUCCH) has to be “False” as UL CA capable Scell is configured without PUCCH on the Peer reconfiguration message. |
| Summary of change | Scell is created with UL_CA enabled but without PUCCH configured |
| TTCN module | ttcn/develop/NR5GC/8_1_5/RRC_Others_CA_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_5_7_1_X_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested) runs on NR5GC_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_PCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var template (value) NR_RadioBearer_Type v_SS_Drb_Config; var template (value) DRB_ToAddMod v_DRB_ToAddMod; var DRB_Identity v_DrbId; f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested,true); // Create and configure cell f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell); << SKIPPED CODE >> }

After Change:

function f_TC_8_1_5_7_1_X_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested) runs on NR5GC_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_PCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var template (value) NR_RadioBearer_Type v_SS_Drb_Config; var template (value) DRB_ToAddMod v_DRB_ToAddMod; var DRB_Identity v_DrbId; f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested,true); // Create and configure cell f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell,-,true,false); //WA#WI=1184162 << SKIPPED CODE >> }

## Correction to the Function f_TC_7_1_1_7_1_ENDC_Common

| Function name | f_TC_7_1_1_7_1_ENDC_Common |
| --- | --- |
| Reason for change | When Creating Scell using the function f_NR_CellConfig_Def , third parameter (p_UL_CA) has to be “True” when pc_EN_DC_NR_UL_2CC is enabled for this test case and 4th parameter (p_UL_CA_ConfigPUCCH) has to be “False” as UL CA capable Scell is configured without PUCCH on the Peer reconfiguration message. |
| Summary of change | Scell is created with UL_CA enabled but without PUCCH configured |
| TTCN module | ttcn/develop/ENDC/7_1_1/MAC_CA_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_7_1_ENDC_Common(NR_CellId_Type p_PSCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested) runs on ENDC_NR_PTC { //Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer var boolean v_UL_CA := f_NR5GC_Get_UL_CA(p_NR_CA_Tested) ; //@sic R5-226021 sic@ var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)}; //@sic R5s210855 R5-214587 sic@ var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var T_PollRetransmit v_T_PollRetransmit := ms80; var DL_t_Reassembly_Type v_T_Reassembly; f_ENDC_NR_Init_CA(p_NR_CA_Tested, {p_PSCell, p_SCell}, v_UL_CA); //Create and configure NR cell1s f_NR_CellConfig_PCell_Def(p_PSCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); << SKIPPED CODE >> }

After Change:

function f_TC_7_1_1_7_1_ENDC_Common(NR_CellId_Type p_PSCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested) runs on ENDC_NR_PTC { //Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer var boolean v_UL_CA := f_NR5GC_Get_UL_CA(p_NR_CA_Tested) ; //@sic R5-226021 sic@ var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)}; //@sic R5s210855 R5-214587 sic@ var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var T_PollRetransmit v_T_PollRetransmit := ms80; var DL_t_Reassembly_Type v_T_Reassembly; f_ENDC_NR_Init_CA(p_NR_CA_Tested, {p_PSCell, p_SCell}, v_UL_CA); //Create and configure NR cell1s f_NR_CellConfig_PCell_Def(p_PSCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active,v_UL_CA,false); //WA#WI=1184162 << SKIPPED CODE >> }

## Correction to the Function f_TC_7_1_1_7_1_NR_Common

| Function name | f_TC_7_1_1_7_1_NR_Common |
| --- | --- |
| Reason for change | When Creating Scell using the function f_NR_CellConfig_Def , third parameter (p_UL_CA) has to be “True” when pc_UL_NR_CA_2CC is enabled for this test case and 4th parameter (p_UL_CA_ConfigPUCCH) has to be “False” as UL CA capable Scell is configured without PUCCH on the Peer reconfiguration message. |
| Summary of change | Scell is created with UL_CA enabled but without PUCCH configured |
| TTCN module | ttcn/develop/NR5GC/7_1_1/MAC_CA_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_7_1_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / var boolean v_UL_CA := f_NR5GC_Get_UL_CA(p_NR_CA_Tested) ; //@sic R5-226021 sic@ var DRB_Identity v_DRBId; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s210855 R5-214587 sic@ var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var T_PollRetransmit v_T_PollRetransmit := ms80; var DL_t_Reassembly_Type v_T_Reassembly; f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested, v_UL_CA); f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCell); //@sic R5s210315 sic@ v_T_Reassembly := f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(p_PCell)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(tsc_NR_DRB2, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly); // Create and configure cells f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); << SKIPPED CODE >> }

After Change:

function f_TC_7_1_1_7_1_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / var boolean v_UL_CA := f_NR5GC_Get_UL_CA(p_NR_CA_Tested) ; //@sic R5-226021 sic@ var DRB_Identity v_DRBId; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s210855 R5-214587 sic@ var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var T_PollRetransmit v_T_PollRetransmit := ms80; var DL_t_Reassembly_Type v_T_Reassembly; f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested, v_UL_CA); f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCell); //@sic R5s210315 sic@ v_T_Reassembly := f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(p_PCell)); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(tsc_NR_DRB2, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly); // Create and configure cells f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active,v_UL_CA,false); //WA#WI=1184162 << SKIPPED CODE >> }