3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230639

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3178 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_38508_SetPDCP_SN_UL and f_NR_38508_SetPDCP_SN_DL for a Redcap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current TTCN implementation doesn’t consider a Redcap UE capable of supporting 18 bit pdcp SN based on longSN-RedCap-r17 UE capability support indication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added pc_longSN support in addition to pc_supportOfRedCap_r17 for enabling 18 bit PDCP SN support for a Redcap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.1.2 NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	4

2.1	Function f_NR_38508_SetPDCP_SN_UL	4

2.2	Function f_NR_38508_SetPDCP_SN_DL	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function f_NR_38508_SetPDCP_SN_UL

| Function name | function f_NR_38508_SetPDCP_SN_UL |
| --- | --- |
| Reason for change | 1. Current TTCN implementation doesn’t consider a Redcap UE capable of supporting 18 bit pdcp SN based on longSN-RedCap-r17 UE capability support indication. |
| Summary of change | 1. Added pc_longSN support in addition to pc_supportOfRedCap_r17 for enabling 18 bit PDCP SN support for a Redcap UE. |
| TTCN module | ..\Common\NR\NR_RRC_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_38508_SetPDCP_SN_UL() return NR_PDCP_SN_UL_Type { //@sic R5-227518 sic@ if ( f_GetTestcaseAttrib_ENDC(testcasename()) or not(pc_supportOfRedCap_r17)) { return len18bits; } else { return len12bits; } }

After Change:

function f_NR_38508_SetPDCP_SN_UL() return NR_PDCP_SN_UL_Type { //@sic R5-227518 sic@ if ( f_GetTestcaseAttrib_ENDC(testcasename()) or not(pc_supportOfRedCap_r17)or (pc_supportOfRedCap_r17 and pc_longSN) ) { //WA#WI=1088023 return len18bits; } else { return len12bits; } }

## Function f_NR_38508_SetPDCP_SN_DL

| Function name | function f_NR_38508_SetPDCP_SN_DL |
| --- | --- |
| Reason for change | 1. Current TTCN implementation doesn’t consider a Redcap UE capable of supporting 18 bit pdcp SN based on longSN-RedCap-r17 UE capability support indication. |
| Summary of change | 1. Added pc_longSN support in addition to pc_supportOfRedCap_r17 for enabling 18 bit PDCP SN support for a Redcap UE. |
| TTCN module | ..\Common\NR\NR_RRC_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_38508_SetPDCP_SN_DL() return NR_PDCP_SN_DL_Type { if ( f_GetTestcaseAttrib_ENDC(testcasename()) or not(pc_supportOfRedCap_r17)) { return len18bits; } else { return len12bits; } }

After Change:

function f_NR_38508_SetPDCP_SN_DL() return NR_PDCP_SN_DL_Type { if ( f_GetTestcaseAttrib_ENDC(testcasename()) or not(pc_supportOfRedCap_r17)or (pc_supportOfRedCap_r17 and pc_longSN )) { //WA#WI=1088023 return len18bits; } else { return len12bits; } }