3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230641

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3180 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.6.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an instability observed with different UEs for the initial RRC Reconfiguration setting up the configuration at Step 1 – 2 in that the UE has adopted the new configuration transmitted in the signalling message and is ready to send the RLC ACK as well as the RRC Reconfiguration Complete to the SS. However, the SS local end configuration adopting the new configuration has not completed yet resulting in the failure to decode the uplink transmissions ( RLC ACK and RRC Reconfiguration Complete ) from the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is recommended to stop the default OnSR UL grant configuration before the transmission of the RRC Reconfiguration message at Step 1 and re-start it after the local end SS configuration is completed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Instability of the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Function f_TC_7_1_1_6_3_NR5GC	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function f_TC_7_1_1_6_3_NR5GC

| Function name | f_TC_7_1_1_6_3_NR5GC |
| --- | --- |
| Reason for change | There is an instability observed with different UEs for the initial RRC Reconfiguration setting up the configuration at Step 1 – 2 in that the UE has adopted the new configuration transmitted in the signalling message and is ready to send the RLC ACK as well as the RRC Reconfiguration Complete to the SS. However, the SS local end configuration adopting the new configuration has not completed yet resulting in the failure to decode the uplink transmissions ( RLC ACK and RRC Reconfiguration Complete ) from the UE |
| Summary of change | It is recommended to stop the default OnSR UL grant configuration before the transmission of the RRC Reconfiguration message at Step 1 and re-start it after the local end SS configuration is completed. |
| TTCN module | MAC_SPS_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_6_3_NR5GC() runs on NR5GC_PTC { // Correct handling of UL grant / configured grant Type 2 <<SKIPPED CODE>> f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_3_NR5GC() runs on NR5GC_PTC { // Correct handling of UL grant / configured grant Type 2 <<SKIPPED CODE>> f_NR_ULGrantConfiguration_Stop(nr_Cell1); //WA# f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); f_NR_ULGrantConfiguration_Start(nr_Cell1); //WA# SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); <<SKIPPED CODE>>