3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230671

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4758 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TTCN type definitions, Annexes D and F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2023-09-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Annexes D and F for the TTCN type definitions should be updated to document the changes in the TTCN delivery iwd-23wk37 (Friday 15th September 2023). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Annexes D and F. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ASP changes will not be correctly documented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex D, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex D (informative)
TTCN-3 Definitions

# D.1	EUTRA_ASP_TypeDefs

Type definitions for configuration of the system simulator;
Common design principles:
Semantics of OMIT: for all TTCN-3 type definitions used in ASPs omit means "keep as it is" =>
- on initial configuration in general all fields shall be provided
- no default values for fields are foreseen
- if necessary non-existence of information shall be explicitly configured
  (e.g. with a union of "no configuration" and "configuration parameters"
- fields within structures imported from the core spec are excepted from this rule

## D.1.1	ASN1_Container

Definitions containing ASN.1 types for backward compatibility;
NOTE 1: PCCH_Message and BCCH_DL_SCH_Message already have a critical extension mechanism by RRC type definition
NOTE 2: BCCH_BCH_Message contains the MIB and therefore is considered to be not extendable
NOTE 3: "simple types" are not considered: C_RNTI, PhysCellId, CellIdentity, ARFCN_ValueEUTRA

AntennaInfoDedicated_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | AntennaInfoDedicated_R8andLater_Type |  |  |
| Comment |  |  |  |
| antennaInfo | AntennaInfoDedicated |  |  |
| antennaInfo_v920 | AntennaInfoDedicated_v920 | opt |  |

AntennaInfoDedicated_R10andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | AntennaInfoDedicated_R10andLater_Type |  |  |
| Comment |  |  |  |
| antennaInfo | AntennaInfoDedicated_r10 |  |  |
| antennaInfoUL | AntennaInfoUL_r10 | opt |  |
| antennaInfoDedicated_v10i0 | AntennaInfoDedicated_v10i0 | opt |  |
| antennaInfoDedicated_v1250 | AntennaInfoDedicated_v1250 | opt |  |
| antennaInfoDedicated_v1430 | AntennaInfoDedicated_v1430 | opt |  |

CQI_ReportConfig_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CQI_ReportConfig_R8andLater_Type |  |  |
| Comment |  |  |  |
| cqi_ReportConfig | CQI_ReportConfig |  |  |
| cqi_ReportConfig_v920 | CQI_ReportConfig_v920 | opt |  |

CQI_ReportConfig_R10andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CQI_ReportConfig_R10andLater_Type |  |  |
| Comment |  |  |  |
| cqi_ReportConfig_r10 | CQI_ReportConfig_r10 |  | NOTE: field 'csi-SubframePatternConfig-r10' is not relevant as long as a cell is configured as SCell |
| cqi_ReportConfig_v1130 | CQI_ReportConfig_v1130 | opt |  |
| cqi_ReportConfig_v1250 | CQI_ReportConfig_v1250 | opt |  |

PUCCH_ConfigDedicated_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PUCCH_ConfigDedicated_R8andLater_Type |  |  |
| Comment |  |  |  |
| pucch_ConfigDedicated | PUCCH_ConfigDedicated |  |  |
| pucch_ConfigDedicated_v1020 | PUCCH_ConfigDedicated_v1020 | opt |  |
| pucch_ConfigDedicated_v1130 | PUCCH_ConfigDedicated_v1130 | opt |  |
| pucch_ConfigDedicated_v1250 | PUCCH_ConfigDedicated_v1250 | opt |  |

PUSCH_ConfigDedicated_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PUSCH_ConfigDedicated_R8andLater_Type |  |  |
| Comment |  |  |  |
| pusch_ConfigDedicated | PUSCH_ConfigDedicated |  | in case of CA beta offset shall be the same for the PCell and the associated SCells |
| pusch_ConfigDedicated_v1020 | PUSCH_ConfigDedicated_v1020 | opt | optionally present for Rel 10 cells (normal or CA); in case of CA beta offset shall be the same for the PCell and the associated SCells |
| pusch_ConfigDedicated_v1130 | PUSCH_ConfigDedicated_v1130 | opt |  |
| pusch_ConfigDedicated_v1250 | PUSCH_ConfigDedicated_v1250 | opt |  |

UplinkPowerControlCommon_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UplinkPowerControlCommon_R8andLater_Type |  |  |
| Comment |  |  |  |
| uplinkPowerControlCommon | UplinkPowerControlCommon |  |  |
| uplinkPowerControlCommon_v1020 | UplinkPowerControlCommon_v1020 | opt | optionally present for Rel 10 cells (normal or CA); NOTE: 'p0-NominalPUCCH', 'deltaFList-PUCCH', 'deltaPreambleMsg3' and 'UplinkPowerControlCommon-v1020' are not relevant as long as a cell is configured as SCell |
| uplinkPowerControlCommonSCell_v1130 | UplinkPowerControlCommonSCell_v1130 | opt | only present when cell is configured as Rel 11 or later Scell |
| uplinkPowerControlCommonPSCell_r12 | UplinkPowerControlCommonPSCell_r12 | opt |  |

UplinkPowerControlDedicated_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UplinkPowerControlDedicated_R8andLater_Type |  |  |
| Comment |  |  |  |
| uplinkPowerControlDedicated | UplinkPowerControlDedicated |  |  |
| uplinkPowerControlDedicated_v1020 | UplinkPowerControlDedicated_v1020 | opt | optionally present for Rel 10 cells (normal or CA); NOTE: field 'p0-UE-PUCCH' is not relevant as long as a cell is configured as SCell |
| pathlossReferenceLinking | SCellPathlossReferenceLinking_Type | opt | NOTE: relevant only as long as a cell is configured as SCell |
| uplinkPowerControlDedicated_v1130 | UplinkPowerControlDedicated_v1130 | opt |  |
| uplinkPowerControlDedicated_v1250 | UplinkPowerControlDedicated_v1250 | opt |  |

SoundingRS_UL_ConfigDedicated_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SoundingRS_UL_ConfigDedicated_R8andLater_Type |  |  |
| Comment |  |  |  |
| soundingRS_UL_ConfigDedicated | SoundingRS_UL_ConfigDedicated | opt |  |
| soundingRS_UL_ConfigDedicated_v1020 | SoundingRS_UL_ConfigDedicated_v1020 | opt |  |
| soundingRS_UL_ConfigDedicatedAperiodic_r10 | SoundingRS_UL_ConfigDedicatedAperiodic_r10 | opt |  |

SchedulingRequestConfig_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SchedulingRequestConfig_R8andLater_Type |  |  |
| Comment |  |  |  |
| schedulingRequestConfig | SchedulingRequestConfig |  |  |
| schedulingRequestConfig_v1020 | SchedulingRequestConfig_v1020 | opt |  |

TDD_Config_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TDD_Config_R8andLater_Type |  |  |
| Comment |  |  |  |
| R8 | TDD_Config |  |  |
| tdd_Config_v1130 | TDD_Config_v1130 | opt |  |

RACH_ConfigCommon_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RACH_ConfigCommon_R8andLater_Type |  |  |
| Comment |  |  |  |
| rach_ConfigCommon | RACH_ConfigCommon |  | All parameters in RACH_ConfigCommonScell_r11 are covered by RACH_ConfigCommon; RACH_ConfigCommon will be used in Scell also for a complete set of RACH parameters both when configured as a Scell or normal RACH procedure when it seazes to exist as Scell; when cell is configured as Scell, additional parameters shall be ignored by SS |
| rach_ConfigCommon_v1250 | RACH_ConfigCommon_v1250 | opt |  |

TDD_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TDD_Config_Type |  |
| Comment |  |  |
| R8andLater | TDD_Config_R8andLater_Type |  |

AntennaInfoCommon_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | AntennaInfoCommon_Type |  |
| Comment |  |  |
| R8 | AntennaInfoCommon |  |

AntennaInfoDedicated_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | AntennaInfoDedicated_Type |  |
| Comment | NOTE: acc. to Cond AI-r8/AI-r10 of PhysicalConfigDedicated 'antennaInfo'/'antennaInfo-v920' and 'antennaInfo-r10' are mutual exclusive |  |
| R8andLater | AntennaInfoDedicated_R8andLater_Type |  |
| R10andLater | AntennaInfoDedicated_R10andLater_Type |  |

PHICH_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PHICH_Config_Type |  |
| Comment |  |  |
| R8 | PHICH_Config |  |

PRACH_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PRACH_Config_Type |  |
| Comment |  |  |
| R8andLater | PRACH_Config_R8andLater_Type |  |

PRACH_Config_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PRACH_Config_R8andLater_Type |  |  |
| Comment | SS shall ignore IE PRACH_Config if PRACH_Config_v1430 is present and preamble set to be used as per parameters in PRACH_Config_v1430 |  |  |
| prach_Config | PRACH_Config |  |  |
| prach_Config_v1310 | PRACH_Config_v1310 | opt |  |
| prach_Config_v1430 | PRACH_Config_v1430 | opt |  |

PUCCH_ConfigCommon_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PUCCH_ConfigCommon_Type |  |
| Comment |  |  |
| R8andLater | PUCCH_ConfigCommon_R8andLater_Type |  |

PUCCH_ConfigCommon_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PUCCH_ConfigCommon_R8andLater_Type |  |  |
| Comment |  |  |  |
| pucch_ConfigCommon | PUCCH_ConfigCommon |  |  |
| pucch_ConfigCommon_v1310 | PUCCH_ConfigCommon_v1310 | opt |  |

PUCCH_ConfigDedicated_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PUCCH_ConfigDedicated_Type |  |
| Comment |  |  |
| R8andLater | PUCCH_ConfigDedicated_R8andLater_Type |  |
| R13andLater | PUCCH_ConfigDedicated_R13andLater_Type |  |

PUCCH_ConfigDedicated_R13andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PUCCH_ConfigDedicated_R13andLater_Type |  |  |
| Comment |  |  |  |
| pucch_ConfigDedicated_r13 | PUCCH_ConfigDedicated_r13 |  | E-UTRAN configures pucch-ConfigDedicated-r13 only if pucch-ConfigDedicated is not configured |

PUSCH_ConfigCommon_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PUSCH_ConfigCommon_Type |  |
| Comment |  |  |
| R8andLater | PUSCH_ConfigCommon_R8andLater_Type |  |

PUSCH_ConfigCommon_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PUSCH_ConfigCommon_R8andLater_Type |  |  |
| Comment |  |  |  |
| pusch_ConfigCommon | PUSCH_ConfigCommon |  |  |
| pusch_ConfigCommon_v1270 | PUSCH_ConfigCommon_v1270 | opt |  |
| pusch_ConfigCommon_v1310 | PUSCH_ConfigCommon_v1310 | opt |  |

PUSCH_ConfigDedicated_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PUSCH_ConfigDedicated_Type |  |
| Comment |  |  |
| R8andLater | PUSCH_ConfigDedicated_R8andLater_Type |  |
| R13andLater | PUSCH_ConfigDedicated_R13andLater_Type |  |

PUSCH_ConfigDedicated_R13andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PUSCH_ConfigDedicated_R13andLater_Type |  |  |
| Comment |  |  |  |
| pusch_ConfigDedicated_r13 | PUSCH_ConfigDedicated_r13 |  | E-UTRAN configures pusch-ConfigDedicated-r13 only if pusch-ConfigDedicated is not configured |
| pusch_ConfigDedicated_v1430 | PUSCH_ConfigDedicated_v1430 | opt |  |

SoundingRS_UL_ConfigCommon_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SoundingRS_UL_ConfigCommon_Type |  |
| Comment |  |  |
| R8 | SoundingRS_UL_ConfigCommon |  |

SoundingRS_UL_ConfigDedicated_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SoundingRS_UL_ConfigDedicated_Type |  |
| Comment |  |  |
| R8andLater | SoundingRS_UL_ConfigDedicated_R8andLater_Type |  |

SchedulingRequestConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SchedulingRequestConfig_Type |  |
| Comment |  |  |
| R8andLater | SchedulingRequestConfig_R8andLater_Type |  |

CQI_ReportConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CQI_ReportConfig_Type |  |
| Comment | NOTE: acc. to Cond CQI-r8/CQI-r10 of PhysicalConfigDedicated 'cqi-ReportConfig'/'cqi-ReportConfig-v920' and 'cqi-ReportConfig-r10' are mutual exclusive |  |
| R8andLater | CQI_ReportConfig_R8andLater_Type |  |
| R10andLater | CQI_ReportConfig_R10andLater_Type |  |

RACH_ConfigCommon_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RACH_ConfigCommon_Type |  |
| Comment |  |  |
| R8andLater | RACH_ConfigCommon_R8andLater_Type |  |

RACH_ConfigDedicated_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RACH_ConfigDedicated_Type |  |
| Comment |  |  |
| R8 | RACH_ConfigDedicated |  |

MeasGapConfig_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MeasGapConfig_R8andLater_Type |  |  |
| Comment |  |  |  |
| measGapConfig | MeasGapConfig |  |  |
| mgta | boolean | opt | measurement gap timing advance for ENDC according to TS 36.331: omit means false at initial configuration and "keep as it is" otherwise |

MeasGapConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MeasGapConfig_Type |  |
| Comment |  |  |
| R8andLater | MeasGapConfig_R8andLater_Type |  |

PDCP_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_Config_Type |  |
| Comment |  |  |
| R8 | PDCP_Config |  |

UL_AM_RLC_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UL_AM_RLC_Type |  |
| Comment |  |  |
| R8 | UL_AM_RLC |  |

DL_AM_RLC_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DL_AM_RLC_Type |  |
| Comment |  |  |
| R8 | DL_AM_RLC |  |

UL_UM_RLC_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UL_UM_RLC_Type |  |
| Comment |  |  |
| R8 | UL_UM_RLC |  |

DL_UM_RLC_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DL_UM_RLC_Type |  |
| Comment |  |  |
| R8 | DL_UM_RLC |  |

TTI_BundlingConfig_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TTI_BundlingConfig_R8andLater_Type |  |  |
| Comment |  |  |  |
| TTI_BundlingEnabled | TTI_Bundling_Type |  |  |
| E_HARQ_Pattern | E_HARQ_Pattern_r12_Type | opt |  |

TTI_BundlingConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TTI_BundlingConfig_Type |  |
| Comment |  |  |
| R8 | boolean |  |
| R8andLater | TTI_BundlingConfig_R8andLater_Type |  |

DRX_Config_R8andLater_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DRX_Config_R8andLater_Type |  |  |
| Comment |  |  |  |
| R8 | DRX_Config | opt |  |
| R13 | DRX_Config_r13_Type | opt |  |
| EDRX_CycleStartOffset_r13 | EDRX_CycleStartOffset_r13_Type | opt |  |

DRX_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DRX_Config_Type |  |
| Comment |  |  |
| R8andLater | DRX_Config_R8andLater_Type |  |

SpsConfigurationDL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SpsConfigurationDL_Type |  |
| Comment |  |  |
| R8 | SPS_ConfigDL.setup |  |

SpsConfigurationUL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SpsConfigurationUL_Type |  |
| Comment |  |  |
| R8 | SPS_ConfigUL.setup |  |

UplinkPowerControlCommon_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UplinkPowerControlCommon_Type |  |
| Comment |  |  |
| R8andLater | UplinkPowerControlCommon_R8andLater_Type |  |

UplinkPowerControlDedicated_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UplinkPowerControlDedicated_Type |  |
| Comment |  |  |
| R8andLater | UplinkPowerControlDedicated_R8andLater_Type |  |

## D.1.2	System_Configuration

Formal ASP Definitions for system configuration

SystemRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SystemRequest_Type |  |
| Comment |  |  |
| Cell | CellConfigRequest_Type | configure/release a cell |
| CellAttenuationList | CellAttenuationList_Type | power attenuation for one or several cells; all cells included in the list shall be changed at the same time; all cells in the list shall reach the new cell power within a maximum of 100ms (10 frames) acc. to the tolerances given in TS 36.508 NOTE: In the common ASP part the CellId shall be set - to the cell the timing information refers to if activation time shall be applied - to eutra_Cell_NonSpecific when there is no activation time |
| RadioBearerList | RadioBearerList_Type | configure/release one or several SRBs and/or DRBs NOTE: RBs are not configured in an SCell |
| EnquireTiming | Null_Type | get SFN and sub-frame number for this cell |
| AS_Security | AS_Security_Type | StartRestart/Release of AS security |
| Sps | SpsConfig_Type | to configure/activate or release semi-persistent scheduling |
| Paging | PagingTrigger_Type | to trigger SS to send paging at the given paging occasion (as calculated in TTCN) |
| L1MacIndCtrl | L1Mac_IndicationControl_Type | to configure SS to generate indications for L1/MAC events |
| RlcIndCtrl | Rlc_IndicationControl_Type | to configure SS to generate indications for RLC events |
| PdcpCount | PDCP_CountReq_Type | to set or enquire PDCP COUNT for one ore more RBs |
| PdcpHandoverControl | PDCP_HandoverControlReq_Type | to inform the target cell about the handover |
| L1_TestMode | L1_TestMode_Type | To Set L1/MAC in special Test modes e.g. DL CRC, PHICH etc |
| PdcchOrder | RA_PDCCH_Order_Type | to configure SS to transmit a PDCCH order with configured C-RNTI to the UE to trigger RA procedure; result in DCI Format 1A transmission as in TS 36.212, clause 5.3.3.1.3 |
| ActivateScell | ActivateScell_Type | to configure SS to transmit a MAC control Element to activate an Scell |
| MbmsConfig | MBMS_Config_Type | Configuration of PMCH/MCCH/MTCH for eMBMS |
| PDCCH_MCCH_ChangeNotification | PDCCH_MCCH_ChangeNotification_Type | To trigger SS to send MCCH change notification at the given SFN/subframe (as calculated in TTCN) |
| MSI_Config | MSI_Config_Type | Configuration of MSI in MBMS normal mode to be included by the SS in the first subframe allocated to the MCH within the MCH scheduling period; uses MRB_ID option in routing info and logical channel ID is omitted |
| UE_Cat_Info | UE_CategoryInfo_Type | provides UE category info to SS; to be used e.g. for determination of Nsoft bits for rate matching To be configured in SS after preamble in MAC TBS test cases |
| OCNG_Config | OCNG_Config_Type | Provides SS Mode for Activating/Releasing OCNG configuration in the cell |
| PdcchTddConfigOrder | PDCCH_TDD_ConfigOrder_Type | To trigger SS to send DCI 1C message(s) from a given SFN/subframe (as calculated in TTCN) over PDCCH with CRC scrambled by the eIMTA-RNTI, to indicate the eIMTA-UL/DL-configuration. The SS shall set the DCI 1C content according to eimta_UL_DL_ConfigIndex_r12 provided in EIMTA_Config |
| DirectIndicationInfo | DirectIndicationInfoTrigger_Type | To Trigger SS to transmit a Direct Indication information using DCI command 6_2 and including the 8 bits in the DCI command |
| LAA_SubframeConfiguration | LAA_SubframeConfiguration_Type | to trigger transmission of DCI format 1C transmitted for CC-RNTI to configure subframe configuration for frame structure type 3 |
| MultipleUlSps | MultipleUlSpsConfig_Type | to configure/activate or release multiple semi-persistent scheduling |
| WakeUpSignal | Null_Type | To trigger SS to send Wake Up Signal; TimingInfo : Wake Up Signal transmission start time, calculated by TTCN |

SystemConfirm_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SystemConfirm_Type |  |
| Comment | confirmations for system configuration; in general to be sent after the configuration has been done |  |
| Cell | Null_Type | (no further parameters from SS) |
| CellAttenuationList | Null_Type | (no further parameters from SS) NOTE 1: the confirmation shall be sent when all cells have changed power levels NOTE 2: for the CellId in the common ASP part the same rules are applied as for the SYSTEM REQ |
| RadioBearerList | Null_Type | (no further parameters from SS) |
| EnquireTiming | Null_Type | SFN and sub-frame number are included in the TimingInfo |
| AS_Security | Null_Type | (no further parameters from SS) |
| Sps | Null_Type | (no further parameters from SS) |
| Paging | Null_Type | normally not needed but defined for completeness |
| L1MacIndCtrl | Null_Type | (no further parameters from SS) |
| RlcIndCtrl | Null_Type | (no further parameters from SS) |
| PdcpCount | PDCP_CountCnf_Type | as response to 'Get' a list is returned containing COUNT information for the requested RBs |
| PdcpHandoverControl | Null_Type | confirmation for PDCP handover control |
| L1_TestMode | Null_Type | confirmation for L1 test mode |
| PdcchOrder | Null_Type | confirmation for PDCCH Order |
| ActivateScell | Null_Type | confirmation for ActivateScell |
| MbmsConfig | Null_Type | confirmation for MbmsConfig |
| PDCCH_MCCH_ChangeNotification | Null_Type | normally not needed but defined for completeness |
| MSI_Config | Null_Type | confirmation for explicit MSI Configuration |
| UE_Cat_Info | Null_Type | confirmation for UE CAT info |
| OCNG_Config | Null_Type | Confirmation of OCNG configuration |
| DirectIndicationInfo | Null_Type | Confirmation of DirectIndicationInfo |
| WakeUpSignal | Null_Type | Confirmation for WUS being triggered |

SystemIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SystemIndication_Type |  |
| Comment |  |  |
| Error | charstring | indicates an error situation in SS; is not explicitly handled in TTCN but causes an INCONC due to default behaviour; an additional error code can be signalled in the common part of the ASP; SS shall raise an error in case of - Invalid TimingInfo for TDD - Contradiction of periodic UL grants and TDD configuration - Data scheduled for the same TTI does not fit into an available transport block (NOTE: additional cases may occur) |
| RachPreamble | RachPreamble_Type | RACH preamble being sent by the UE In case of Preamble repetition (BL/CE UE under test) the SS sends one indication after reception of all preamble repetitions of a preamble attempt based on the CE level of UE. |
| SchedReq | Null_Type | indication for scheduling request sent by the UE In case of HARQ repetition (PUCCH Repetition of BL/CE UE under test) the SS sends one SI indication after reception of all PUCCH repetitions based on the CE level of UE |
| BSR | BSR_Type | to report the Buffer/Extended Buffer status report, Sidelink Buffer Status Report or Truncated Sidelink Buffer Status Report being received |
| UL_HARQ | HARQ_Type | to report the UL HARQ as received on PUCCH[TTI] for corresponding DL transmission in TTI-x, where x is normally 4. In case of HARQ repetition (PUCCH/PUSCH Repetition of BL/CE UE under test) the SS sends one HARQ indication after reception of all PUCCH/PUSCH repetitions based on the CE level of UE |
| C_RNTI | C_RNTI | indicates C-RNTI being contained in a MAC PDU sent by the UE |
| PHR | PHR_Type | to report the Power headroom report received |
| HarqError | HarqError_Type | indicates detection of HARQ error: 1. HARQ CRC error for UL data 2. HARQ NACK from the UE unless SS is configured to report HARQ ACK/NACK In case of HARQ repetition (PUCCH/PUSCH Repetition of BL/CE UE under test) the SS sends one HARQ indication after reception of all PUCCH/PUSCH repetitions based on the CE level of UE |
| RlcDiscardInd | RlcDiscardInd_Type | indicates e.g. discarded PDUs |
| PeriodicRI | RI_Type | indicates periodic Rank Indicator (RI) reported by the UE on PUCCH or PUSCH; periodic CQI/PMI/RI Reporting is semi-statically configured at the UE by higher layers (see TS 36.213 clause 7.2.2); aperiodic reporting acc. to TS 36.213 clause 7.2.1 shall not be indicated NOTE: Acc. to TS 36.213 clause 7.2 aperiodic reporting has higher precedence than periodic reporting; => as working assumption the CQI request field in DCI format 0 is expected to be 0 for UL grants assigned by the SS i.e. aperiodic reporting acc. to TS 213 clause 7.2.1 does not happen In case of RI repetition (PUCCH/PUSCH Repetition of BL/CE UE under test) the SS sends one RI indication after reception of all PUCCH/PUSCH repetitions based on the CE level of UE |
| EPHR | MAC_CTRL_ExtPowerHeadRoom_Type | indicates Extended Power headroom report reported by the UE |
| CqiInd | Null_Type | indicates periodic CQI reported by the UE - NOTE: Report CQI value is currently not required |
| SrsInd | SRSInd_Type | Indicates start or stop of Type 0 SRS reception on configured periodic SRS resources |
| DC_PHR | MAC_CTRL_DC_PowerHeadRoom_Type | indicates Dual Connectivity Power headroom report reported by the UE |

## D.1.3	Cell_Configuration

Specific Info for Cell Configuration Primitive

### D.1.3.1	Cell_Configuration_Common

Cell_Configuration_Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| EUTRA_FDD_Info_Type | Null_Type | no further parameters defined for FDD |
| CfiValue_Type | integer (1..3) |  |

CellConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CellConfigRequest_Type |  |
| Comment |  |  |
| AddOrReconfigure | CellConfigInfo_Type | for cell configuration: CellId : identifier of the cell to be configured RoutingInfo : None TimingInfo : Now (for initial configuration and for reconfiguration in general) ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Release | Null_Type | to remove a cell completely - CellId : identifier of the cell to be released; eutra_Cell_NonSpecific, in case all cells shall be released RoutingInfo : None TimingInfo : Now ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

CellConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CellConfigInfo_Type |  |  |
| Comment | common information for initial cell configuration or reconfiguration; in case of reconfiguration OMIT means 'keep configuration as it is' |  |  |
| Basic | BasicCellConfig_Type | opt | basic information for a cell (e.g. broadcasting) |
| Active | ActiveCellConfig_Type | opt | add. configuration for active cell (i.e. cell being capable to receive RACH preamble) |

CellConfigCapability_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | CellConfigCapability_Type |
| Comment | capabilities of a cell acc. to the initial condition of a test case |
| broadcastOnlyCell | no detection of RACH preambles required; cell is only broadcasting |
| minimumUplinkCell | detection of RACH preambles required but not any further RX capability |
| fullCell | full TX and RX capabilities |

CA_CellInitialConfig_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | CA_CellInitialConfig_Type |
| Comment |  |
| PCell | The cell when added as a component carrier (CC) in CA scenario for first time will be configured as an PCell |
| Scell_Active | The cell when added as a component carrier (CC) in CA scenario for first time will be configured as an SCell, and when configured as Scell, it may be activated, 36.508 clause 6.3.4 |
| Scell_Inactive | The cell when added as a component carrier (CC) in CA scenario for first time will be configured as an SCell, and when configured as Scell, it will never be activated, 36.508 clause 6.3.4 |
| PScell | The cell when added as a component carrier (CC) in CA-Dual connectivity scenario for first time will be configured as a PSCell. PSCell is always active like PCell |

BasicCellConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | BasicCellConfig_Type |  |  |
| Comment |  |  |  |
| ConfigCapability | CellConfigCapability_Type | opt | mandatory for the initial configuration; to be omitted afterwards |
| StaticCellInfo | StaticCellInfo_Type | opt | Common information which does not change during a test |
| PhysicalLayerConfigDL | PhysicalLayerConfigDL_Type | opt | default settings regarding physical control channels: PCFICH, PHICH, PDCCH |
| InitialCellPower | InitialCellPower_Type | opt | reference cell power for the RS of each antenna in DL NOTE 1: the power of the RS of an antenna may be reduced by antenna specific configuration NOTE 2: in general the power may be adjusted on a per resource element basis => all physical channel/signal power settings shall be adjusted relatively to the RS; if there are more than one TX antennas each one may have its own attenuation; independently from those relative power settings the cell power can easily be adjusted by just changing the reference power |
| BcchConfig | BcchConfig_Type | opt | configuration of BCCH/BCH; SS is triggered to configure RLC/MAC regardingly; BCCH data on the PDSCH is distinguished by the SI-RNTI PBCH: MIB; PDSCH: scheduling and resource allocation; SIBs |
| PcchConfig | PcchConfig_Type | opt | configuration of PCCH/PCH; SS is triggered to configure RLC/MAC regardingly; PCCH data on the PDSCH is distinguished by the P-RNTI (needed even to modify SI => shall be configured for CELL_BROADCASTING) |
| CA_CellInitialConfig | CA_CellInitialConfig_Type | opt | capability of a cell when added as a CC in CA scenario. 1. Provided at the initial configuration of a cell in CA test cases; to be omitted afterwards; 2. Always omit for a cell which remains normal non CA cell |
| BRBcchConfig | Bcch_BRConfig_Type | opt | configuration of BCCH_BR/BCH; SS is triggered to configure RLC/MAC regardingly; BCCH_BR data on the PDSCH is follows fixed schedule hence no SI-RNTI DCI sent PBCH: MIB; PDSCH: scheduling and resource allocation; SIBs |
| LAA_SCellConfiguration | LAA_SCellConfiguration_r13 | opt | LAA specific information provided if the cell is being configured as Scell with frame structure 3 i.e. in Unlicensed band |
| LAA_SCellConfiguration_v1430 | LAA_SCellConfiguration_v1430 | opt | LAA specific information provided if the cell is being configured as Scell with frame structure 3 i.e. in Unlicensed band with ULLAA . It is optionally present if CrossCarrierScheduledCellsList_Type is not present or configured as own, other wise not present |

ActiveCellConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ActiveCellConfig_Type |  |  |
| Comment |  |  |  |
| C_RNTI | C_RNTI | opt | (pre-)configured C-RNTI; affects scrambling of PDSCH/PUSCH and CRC of PDCCH(s); shall be used implicitly in RACH procedure (i.e. as CE in RAR) |
| PhysicalLayerConfigUL | PhysicalLayerConfigUL_Type | opt | parameters for PRACH, PUCCH, PUSCH Omitted when no UL frequency is applied to the band |
| RachProcedureConfig | RachProcedureConfig_Type | opt | to configure the SS's behaviour for the RACH procedure Omitted when no UL frequency is applied to the band |
| CcchDcchDtchConfig | CcchDcchDtchConfig_Type | opt | Parameters related to CCCH/DCCH/DTCH in UL and DL |
| ServingCellConfig | ServingCellConfig_Type | opt | not present as long as the cell is 'normal' cell (i.e. does not act as a carrier component in CA); present to configure cell for CA (Pcell or SCell); in general at initial configuration 'ServingCellInfo' is omit; after sending/scheduling the RRCConnectionReconfiguration adding 1 or more cells for CA 'ServingCellInfo' is provided for the cell which gets Pcell and for the cell which gets SCell |
| SciSchedulingConfig | SciSchedulingConfig_Type | opt | Parameters related to SideLink scheduling SCI format 0 |
| EIMTA_Config | EIMTA_Config_Type | opt | Parameters related to eIMTA in UL and DL |
| CE_Mode | CE_Mode_r13_Type | opt | Indicates the CE mode as specified in TS 36.213 It is FFS if this can be ignored by SS |

StaticCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | StaticCellInfo_Type |  |  |
| Comment | Common information which (normally) does not change during a test; therefore all fields are mandatory |  |  |
| Common | CommonStaticCellInfo_Type |  |  |
| Downlink | DownlinkStaticCellInfo_Type |  |  |
| Uplink | UplinkStaticCellInfo_Type | opt | NOTE: for TDD UL and DL are using the same parameters Omitted when no UL frequency is applied to the band |

CommonStaticCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CommonStaticCellInfo_Type |  |  |
| Comment | information common for UL and DL; all fields are mandatory |  |  |
| RAT | EUTRA_RAT_Type |  | FDD or TDD; FDD/TDD specific parameters |
| PhysicalCellId | PhysCellId |  | N(cell, ID): imported from core spec; -> cell specific reference signals (non-MBSFN) -> scrambling of all DL physical channels: PBCH, PCFICH, PDCCH, PHICH and PDSCH (together with nRNTI) |
| eNB_CellId | CellIdentity | opt | Placeholder for Cell identity (28 bits): eNB (20bits) and cell identity (8bits). The use of that field is for future usage and omit for the time being |
| EutraBand | integer |  | NOTE: in 3G there are overlapping bands therefore the band needs to be provided; in EUTRA it is provided as well to be extendable in the future |
| CellTimingInfo | CellTimingInfo_Type |  |  |

EUTRA_TDD_Info_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | EUTRA_TDD_Info_Type |  |  |
| Comment |  |  |  |
| Configuration | TDD_Config_Type |  | TDD_Config acc. to RRC ASN.1 (acc. TS 36.331, clause 6.3.2 ) |

EUTRA_HalfDuplexFDD_Info_Type

| TTCN-3 Record Type |  |
| --- | --- |
| Name | EUTRA_HalfDuplexFDD_Info_Type |
| Comment | NOTE: No specific elements required When UE is FDD Type B Half-Duplex SS shall restrict non-explicitly scheduled UL data transmission in subframe 2 and DL transmission in subframe 8. This results in UL HARQ feedback in subframe 2, DL HARQ feedback in subframe 6, UL grant allocation in subframe 8. SS shall obey explicit scheduling. |

EUTRA_RAT_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | EUTRA_RAT_Type |  |
| Comment | specifies RAT type and frame structure (TS 36.211, clause 4) |  |
| FDD | EUTRA_FDD_Info_Type |  |
| TDD | EUTRA_TDD_Info_Type |  |
| HalfDuplexFDD | EUTRA_HalfDuplexFDD_Info_Type |  |

DownlinkStaticCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DownlinkStaticCellInfo_Type |  |  |
| Comment | DL Static Info |  |  |
| Earfcn | integer |  | DL-EARFCN as defined in TS 36.101 |
| Bandwidth | Dl_Bandwidth_Type |  | N(DL, RB) = 6..110 (6, 15, 25, 50, 75, 100) |
| RBSize | EUTRA_RBSize_Type |  | may be skipped assuming normal sub-carrier spacing => N(RB, SC) = 12 |
| CyclicPrefix | EUTRA_CyclicPrefix_Type |  |  |

UplinkStaticCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UplinkStaticCellInfo_Type |  |  |
| Comment | UL Static Info |  |  |
| Earfcn | integer |  | UL-EARFCN as defined in TS 36.101 |
| Bandwidth | Ul_Bandwidth_Type |  | N(DL, RB) = 6..110 (6, 15, 25, 50, 75, 100) |
| CyclicPrefix | EUTRA_CyclicPrefix_Type |  |  |

EUTRA_RBSize_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | EUTRA_RBSize_Type |
| Comment | Resource Block Size in freq domain; N(RB,SC) is 12 for normal sub-carrier spacing |
| n_RB_SC_12 |  |
| n_RB_SC_24 |  |

Modulation_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | Modulation_Type |
| Comment | 'unused' e.g. for 2nd codeword when there is no spatial multiplexing |
| unused |  |
| qpsk |  |
| qam16 |  |
| qam64 |  |

ToRS_EPRE_Ratios_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ToRS_EPRE_Ratios_Type |  |  |
| Comment | RA and RB ratios according to see TS 36.213, clause 5.2 |  |  |
| RA | ToRS_EPRE_Ratio_Type | opt |  |
| RB | ToRS_EPRE_Ratio_Type | opt |  |

UE_CategoryInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UE_CategoryInfo_Type |  |  |
| Comment |  |  |  |
| ue_Category | UE_Category_Type |  |  |
| ue_Category_V1020 | UE_Category_v1020_Type | opt |  |
| ue_Category_v1170 | UE_Category_v1170_Type | opt |  |
| ue_Category_v11a0 | UE_Category_v11a0_Type | opt |  |
| ue_Category_v1250 | UE_Category_v1250_Type | opt | If UE category is 0 then 1. SS shall forward UL CCCH SDU only when indicated with LCHID '01011'B. 2. SS shall restrict DL UE-dedicated MAC TBS to 1000. |
| ue_CategoryDL_r12 | UE_CategoryDL_r12_Type | opt |  |
| ue_CategoryDL_v1260 | UE_CategoryDL_v1260_Type | opt |  |
| ue_CategoryDL_v1310 | UE_CategoryDL_v1310_Type | opt |  |
| ue_CategoryDL_v1330 | UE_CategoryDL_v1330_Type | opt |  |
| ue_CategoryDL_v1350 | UE_CategoryDL_v1350_Type | opt |  |
| ue_CategoryDL_v1450 | UE_CategoryDL_v1450_Type | opt |  |
| ue_CategoryDL_v1460 | UE_CategoryDL_v1460_Type | opt |  |

### D.1.3.2	Downlink_Physical_Layer_Configuration

Downlink physical layer configuration:
- DL antenna configuration
- control region (PCFICH, PHICH, PDCCH)
- primary/secondary sync signals
- power control for physical channels and signals

PhysicalLayerConfigDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PhysicalLayerConfigDL_Type |  |  |
| Comment | all fields are declared as optional to allow single reconfigurations; in this case omit means "keep as it is" |  |  |
| AntennaGroup | DownlinkAntennaGroupConfig_Type | opt |  |
| Pbch | PbchConfig_Type | opt |  |
| Pcfich | PcfichConfig_Type | opt |  |
| Phich | PhichConfig_Type | opt |  |
| Pdcch | PdcchConfig_Type | opt |  |
| Pdsch | PdschConfig_Type | opt |  |
| Pss | PrimarySyncSignal_Type | opt |  |
| Sss | SecondarySyncSignal_Type | opt |  |
| CSI_RS_Config | CSI_RS_Config_Type | opt |  |
| Pmch | PmchConfig_Type | opt | Same power offset for all PMCH carrying MCCH or MTCH |
| Epdcch | EpdcchConfig_Type | opt |  |
| DS_RS_Config | DS_RS_Config_Type | opt |  |
| Wus | WUS_Config_Type | opt | if omitted in initial configuration, the Wake-Up Signal shall be considered as being not present; if is present WUS shall be transmitted by the SS at the activation time given by WakeUpSignal trigger |

#### D.1.3.2.1	Antenna_Configuration

Antenna_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| AntennaPortId_Type | integer (0, 1, 2, 3, 4, 5, 6, 7, 8) | Antenna port 0..3: Cell specific reference signals Antenna port 4: MBSFN reference signals Antenna port 5: UE specific reference signals Antenna port 6: Positioning reference signals Antenna port 7: UE specific reference signals for dual layer beamforming Antenna port 8: UE specific reference signals for dual layer beamforming (Antenna ports in DL acc. to 36.211 cl. 6.2.1) |

AntennaPortInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | AntennaPortInfo_Type |  |  |
| Comment | NOTE: for conformance tests it may not be necessary to consider propagation pathes for different antennas; => fields of AntennaPortInfo_Type are used as place holders for future usage and are of 'Dummy_Type' for the time being |  |  |
| PowerAttenuation | Dummy_Type |  | even though eNb shall send with the same power on all antennas at the UE there may be different signal strength => RS will have reduced power NOTE: the EPRE ratios (e.g. PDSCH-to-RS ratio) are assumed to be equal for all antennas |
| PropagationDelay | Dummy_Type |  | signal from different antennas may have different propagation delay |

AntennaPortConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | AntennaPortConfig_Type |  |
| Comment |  |  |
| AddOrReconfigure | AntennaPortInfo_Type | add / re-configure antenna port |
| Release | Null_Type | release antenna port |

AntennaPort_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | AntennaPort_Type |  |  |
| Comment |  |  |  |
| Id | AntennaPortId_Type |  |  |
| Config | AntennaPortConfig_Type |  |  |

DownlinkAntennaGroupConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DownlinkAntennaGroupConfig_Type |  |  |
| Comment |  |  |  |
| AntennaInfoCommon | AntennaInfoCommon_Type |  | acc. to TS 36.331, clause 6.3.2; contains antennaPortsCount = an1, an2, an4; static parameter; will (normally) not be modified whilst a test; NOTE: information is redundant since number of antenna ports may implicitly be determined by the number of ports being configured |
| AntennaPort | record length (1..4) of AntennaPort_Type |  | 1, 2 or 4 antennas; from the UE's point of view each antenna may have a different power level and a different propagation delay |

#### D.1.3.2.2	Physical_Channels

PbchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PbchConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | ToRS_EPRE_Ratios_Type | opt | power ratio for PBCH's resource elements relative to the RS |

PcfichConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PcfichConfig_Type |  |  |
| Comment |  |  |  |
| CfiValue | CfiValue_Type | opt | control format indicator signalled on PCFICH |
| RelativeTxPower | ToRS_EPRE_Ratios_Type | opt | power ratio for PFCICH's resource elements relative to the RS |

PhichConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PhichConfig_Type |  |  |
| Comment |  |  |  |
| PhichConfig | PHICH_Config_Type | opt | parameters acc. TS 36.331, clause 6.3.2: phich-Duration, phich-Resource; may have impact on Cfi |
| RelativeTxPower | ToRS_EPRE_Ratios_Type | opt | power ratio for PHICH's resource elements relative to the RS |

CCE_StartIndex_DL_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CCE_StartIndex_DL_UL_Type |  |  |
| Comment | CCE_St_Ind' or CCE_St_Ind'' acc. to table 7.1.1-1 in TS 36.523-3 |  |  |
| CCE_StartIndex_DL | integer |  |  |
| CCE_StartIndex_UL | integer |  |  |

CCE_StartIndexList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | CCE_StartIndexList_Type |
| Comment | describes PDCCH candidates for all sub-frames |
| record length(10) of CCE_StartIndex_DL_UL_Type |  |

PdcchCandidate_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PdcchCandidate_Type |  |  |
| Comment | CCE start indeces for a given RNTI value acc. to table 7.1.1-1 in TS 36.523-3 |  |  |
| RNTI | C_RNTI |  | RNTI value as per table 7.1.1-1 |
| CCE_StartIndexList | CCE_StartIndexList_Type |  | CCE Start Indices corresponding to the RNTI |

PdcchCandidateList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PdcchCandidateList_Type |
| Comment | list of RNTIs and their corresponding CCE Start Indices |
| record of PdcchCandidate_Type |  |

PdcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PdcchConfig_Type |  |  |
| Comment | UE performs blind detection for common and UE specific search spaces for different aggregation levels (PDCCH formats acc. TS 36.211, clause 6.8.1) content of the PDCCHs (DCI formats acc. TS 36.212, clause 5.3.3) shall be controlled together with scheduling and resource allocation |  |  |
| CommonSearchSpaceFormat | integer (2, 3) | opt | PDCCH format for common search space; acc. to TS 36.213, clause 9.1.1 only aggregation level 4 and 8 are allowed (i.e. PDCCH format 2 and 3 |
| UeSpecificSearchSpaceFormat | integer (0, 1, 2, 3) | opt | UE specific search space: corresponding aggregation levels 1, 2, 4, 8 |
| PdcchCandidateList | PdcchCandidateList_Type | opt | PDCCH candidate list acc. to table 7.1.1-1 in TS 36.523-3 |
| RelativeTxPower | ToRS_EPRE_Ratios_Type | opt | power ratio for PDCCH's resource elements relative to the RS |

PdschRelativeTxPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PdschRelativeTxPower_Type |  |  |
| Comment | NOTE 1: the power control for the PDSCH is assumed to be (semi-)static for signalling conformance tests acc. to TS 36.323; nevertheless for different channels and purposes with the PDSCH there may be different power settings; NOTE 2: acc. to TS 36.213, clause 5.2 the EPRE ratio is different in time domain for OFDM symbols containing or not containing reference signals; this needs to be considered by SS |  |  |
| RachResponse | ToRS_EPRE_Ratios_Type | opt |  |
| BcchOnPdsch | ToRS_EPRE_Ratios_Type | opt |  |
| PcchOnPdsch | ToRS_EPRE_Ratios_Type | opt |  |
| CcchOnPdsch | ToRS_EPRE_Ratios_Type | opt |  |
| DcchDtchOnPdsch | ToRS_EPRE_Ratios_Type | opt |  |
| OcngOnPdsch | ToRS_EPRE_Ratios_Type | opt | the power offset used for modeling allocations to virtual UE's (different from the UE under test) |

PdschConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PdschConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | PdschRelativeTxPower_Type | opt |  |
| PDSCH_ConfigCommon | PDSCH_ConfigCommon_v1310 | opt |  |

EpdcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | EpdcchConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | ToRS_EPRE_Ratios_Type | opt | power ratio for ePDCCH's/MPDCCH(if configured) resource elements relative to the RS |
| ePdcchCandidateList | PdcchCandidateList_Type | opt | ePDCCH candidate list acc. to tables in TS 36.523-3 clause 7.1.2 |
| EPDCCH_Config | EPDCCH_Config_r11 | opt | If present with choice 'setup' SS starts using ePDCCH in subframes reserved for ePDCCH for UL/DL grant transmission on C-RNTI/SPS-RNTI and PDCCH in non reserved sub framesor MPDCCH (if present) in all sub frames when using DCI formats 6-xx If present with choice 'release' SS releases ePDCCH/MPDCCH configuration and uses PDCCH for UL/DL grant transmission in all sub frames 'omit' is no change from previous configuration |
| mPDCCH_FreqHoppingParameters_r13 | FreqHoppingParameters_r13 | opt | Provides frequency hopping parameters for MPDCCH/PDSCH/PUSCH/PUSCH. It is optional present conditional to MPDCCH is configured in EPDCCH_Config otherwise it shall not be present. |

#### D.1.3.2.3	Physical_Signals

PrimarySyncSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PrimarySyncSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | ToRS_EPRE_Ratios_Type | opt | power ratio for PSS's resource elements relative to the RS |

SecondarySyncSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SecondarySyncSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | ToRS_EPRE_Ratios_Type | opt | power ratio for SSS's resource elements relative to the RS |

SRS_UL_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SRS_UL_Config_Type |  |  |
| Comment |  |  |  |
| Common | SoundingRS_UL_ConfigCommon_Type |  |  |
| Dedicated | SoundingRS_UL_ConfigDedicated_Type |  |  |

CSI_RS_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CSI_RS_Config_Type |  |  |
| Comment |  |  |  |
| CSI_RS_Config | CSI_RS_Config_r10 | opt | The IE is used for TM1-9 |
| CSI_RS_ConfigNZPToAddModList | CSI_RS_ConfigNZPToAddModList_r11 | opt | The IE is to configure the non-zero power CSI-RS resource configuration |
| CSI_RS_ConfigNZPToReleaseModList | CSI_RS_ConfigNZPToReleaseList_r11 | opt | The IE is to release the non-zero power CSI-RS resource configuration |
| CSI_RS_ConfigZPToAddModList | CSI_RS_ConfigZPToAddModList_r11 | opt | The IE is to configure the zero power CSI-RS resource configuration |
| CSI_RS_ConfigZPToReleaseModList | CSI_RS_ConfigZPToReleaseList_r11 | opt | The IE is to release the zero power CSI-RS resource configuration |
| RelativeTxPower | ToRS_EPRE_Ratios_Type | opt | power ratio for CSI-RS resource elements relative to the CRS |

DS_RS_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DS_RS_Config_Type |  |  |
| Comment | These IE's are to simulate the discovery of partial off cell as per 36.300 clause 16.1.10. The SS shall transmit discovery signals only in the timing enabled by the IE's DMTC_PeriodOffset and DS_OccasionDuration. The SS shall stop transmitting discovery signal in other periods in the cell. Other signals and scheduled transmissions (like SysInfo) are not restricted. |  |  |
| DMTC_PeriodOffset | DMTC_PeriodOffset_r12_Type | opt | Indicates the discovery signals measurement timing configuration (DMTC) periodicity (dmtc-Periodicity) and offset (dmtc-Offset) |
| DS_OccasionDuration_r12 | DS_OccasionDuration_r12_Type | opt | Indicates the duration of discovery signal occasion |

MWUS_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MWUS_Config_Type |  |  |
| Comment |  |  |  |
| TimeOffset | TimeOffsetInFrames |  | Number of frames used to calculate start of the first PO to which the NWUS is associated as per 36.211 cl 6.11B.1 and 36.304 cl 7.4 |
| MWUS_Duration | WUS_Durations_Type |  | Actual duration of MWUS in frames, corresponds to M in 36.211 cl. 6.11B.1 |
| FreqLocation | WUS_Config_r15.freqLocation_r15 |  | Frequency location of WUS within paging narrowband. Value n0 corresponds to WUS in the 1st and 2nd PRB, value n2 represents the 3rd and 4th PRB, and value n4 represents the 5th and 6th PRB. |
| WUS_PowerBoost | WUS_PowerBoost_Type | opt | Power offset of WUS relative to CRS in dB as per TS 36.213 cl 5.2. If omit use 0dB |

WUS_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WUS_Config_Type |  |  |
| Comment |  |  |  |
| WUS_Config | MWUS_Config_Type | opt |  |

### D.1.3.3	Uplink_Physical_Layer_Configuration

Uplink physical channel configuration: PRACH, PUCCH, PUSCH and UL RS

PUCCH_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PUCCH_Configuration_Type |  |  |
| Comment |  |  |  |
| Common | PUCCH_ConfigCommon_Type | opt |  |
| Dedicated | PUCCH_ConfigDedicated_Type | opt |  |

PUSCH_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PUSCH_Configuration_Type |  |  |
| Comment |  |  |  |
| Common | PUSCH_ConfigCommon_Type | opt |  |
| Dedicated | PUSCH_ConfigDedicated_Type | opt |  |

PhysicalLayerConfigUL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PhysicalLayerConfigUL_Type |  |  |
| Comment | NOTE: For the time being there is no requirement to configure the SS with TPC-PDCCH-Config; In general SS is required to keep the UE's UL power constant |  |  |
| Prach | PRACH_Config_Type | opt | parameters acc. TS 36.331, clause 6.3.2; in general depending on FDD/TDD (see TS 36.211, clause 5.7) |
| Pucch | PUCCH_Configuration_Type | opt | parameters acc. TS 36.331, clause 6.3.2 |
| Pusch | PUSCH_Configuration_Type | opt | parameters acc. TS 36.331, clause 6.3.2 (including configuration of RS) |
| TimingAdvance | SS_TimingAdvanceConfig_Type | opt | to adjust timing advance; normally timing advance is configured as 0 at the beginning and never changed during the test case; in some MAC test cases timing advance may be configured to a non-zero (11 bit value) at the beginning and modified by (6 bit) timing advance commands during the test |
| SRS_UL_Config | SRS_UL_Config_Type | opt | sounding reference symbol (SRS); -> TS 36.213, clause 8.2, TS 36.211, clause 5.5.3 |
| SR_Config | SchedulingRequestConfig_Type | opt | PUCCH resources for scheduling requests acc. to TS 36.213 table 10.15; as signalled to the UE acc. to TS 36.331, clause 6.3.2 |
| CQI_ReportConfig | CQI_ReportConfig_Type | opt |  |
| UplinkPowerControlCommon | UplinkPowerControlCommon_Type | opt |  |
| UplinkPowerControlDedicated | UplinkPowerControlDedicated_Type | opt |  |

### D.1.3.4	Common_MAC_Configuration

Transport channel and MAC related procedures and configuration

Common_MAC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| ImcsValue_Type | integer (0..31) | Modulation and coding scheme index coding |
| RepetitionNumber_Type | integer (0..7) | DCI Format 6-0A, value 0..3 as per 36.213 table 8.2b DCI Format 6-0B, value 0..7 as per 36.213 table 8.2c DCI Format 6-1A, value 0..3 as per 36.213 table 7.1.11-1 DCI Format 6-1B, 6_2, value 0..7 as per 36.213 table 7.1.11-2 |
| DCISubframeRepetitionNumber_Type | integer (0..3) | 36.213 clause as defined in section 9.1.5 |

RedundancyVersionListDL_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RedundancyVersionListDL_Type |
| Comment | NOTE: in general the list shall contain maxHARQ-Tx elements; if there are not enough elements specified SS shall raise an error; per default the list is configured to 0,2,3,1,0 (TS 36.321, clause 5.4.2.2) |
| record length (1..28) of RedundancyVersion_Type |  |

UL_TransRetransmission_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UL_TransRetransmission_Type |  |
| Comment |  |  |
| NewTransmission | Null_Type | new transmission of data with redundancy version RV=0 (acc. to TS 36.321 clause 5.4.2.2); NDI is toggled |
| ReTransmissionAdaptive | RedundancyVersion_Type | SS assigns grant to request retransmission of data with given redundancy version; NDI is not toggled |
| ReTransmissionNonAdaptive | Null_Type | non-adaptive retransmission; SS does not send any grant; SS shall use redundancy version acc. to sequence in TS 36.321 clause 5.4.2.2 |
| ReTransmissionNonAdaptiveFixedRV | Null_Type | non-adaptive retransmission; SS does not send any grant; SS shall use redundancy version RV=0 |
| RetransmissionAsynchronous | RetransmissionAsynchronous_Type | Used to schedule time controlled asynchronous UL grant for BL/CE UEs |

RetransmissionAsynchronous_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RetransmissionAsynchronous_Type |  |  |
| Comment |  |  |  |
| RedundencyVer | RedundancyVersion_Type |  | SS assigns grant to request retransmission of data with given redundancy version; NDI is not toggled |
| SubframeOffset | integer |  | the subframe offset since previous grant in the list, shall include all repetitions |
| RepetitionNumber | RepetitionNumber_Type |  |  |
| DCISubframeRepetitionNumber | DCISubframeRepetitionNumber_Type |  |  |

UL_TransRetransmissionList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | UL_TransRetransmissionList_Type |
| Comment | list of transmission and subsequent retransmissions: in UL retransmissions are synchronous (every 8 TTIs for FDD); independent from the HARQ_ModeList SS shall send grants for every adaptive retransmissions; in case of non-adaptive retransmissions SS simply does not sent a grant (i.e. ReTransmissionNonAdaptive elements are used to adjust timing of the adaptive retransmissions only) |
| record length (1..28) of UL_TransRetransmission_Type |  |

Imcs_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | Imcs_Type |  |
| Comment |  |  |
| Value | ImcsValue_Type |  |
| NotUsed | Null_Type |  |

FreqDomainSchedulCommon_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | FreqDomainSchedulCommon_Type |  |  |
| Comment | common type to specify restrictions for frequency domain scheduling by a start index and a maximum range of RBs; in general the resource allocation refers to virtual resource blocks: - format 1A (localised): FirstRbIndex refers to the first physical RB; the RBs are subsequent (upto MaxRbCnt RBs); may be applied for all kind of channels - format 1C (distributed): FirstRbIndex refers to the first virtual RB; the virtual RBs are subsequent (upto MaxRbCnt RBs) but mapped (distributed) to physical resource; typically applied on BCCH, PCCH and RAR - format 1 (localised): FirstRbIndex refers to the first physical RB; RBs are not consecutive; SS needs to provided bitmap of RBs (see TS 36.523-3) to cope with mapping of virtual resource allocation (format 1C) applied on other channels; typically there are either - all channels having format 1A (localised) - BCCH, PCCH and RAR having format 1C (distributed) + DTCH/DCCH having format 1 |  |  |
| FirstRbIndex | integer |  | index of the first (vitual) resource block in frequency domain; 0 .. N(UL/DL, RB) - 1; NOTE: DCI format 1C refers to a virtual RB allocation i.e. the resource block index; differs from the physical resource allocation where the RBs are distributed over the whole frequency bandwidth (TS 36.213, clause 7.1.6.3) |
| MaxRbCnt | integer |  | max. number of resource blocks to be assigned; FirstRbIndex + MaxRbCnt <= N(UL/DL, RB); SS shall not assigned more than the given resource blocks to the respective channel (i.e. MaxRbCnt is the upper bound); if the the configuration for a channel exceeds the total bandwidth this is a TTCN error (=> SS shall raise an error) |

FreqDomainSchedulExplicit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | FreqDomainSchedulExplicit_Type |  |  |
| Comment | type used for explicit DL scheduling; Nprb is the exact number of RBs whereas in FreqDomainSchedulCommon_Type MaxRbCnt is the upper bound |  |  |
| FirstRbIndex | integer |  | index of the first resource block in frequency domain; 0 .. N(UL/DL, RB) - 1 |
| Nprb | integer |  | number of resource blocks to be assigned; |

PdcchDciFormat_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | PdcchDciFormat_Type |
| Comment | DCI format acc. to TS 36.212, clause 5.3.3.1; SS shall apply physical parameters accordingly as specified in TS 36.508, clause 4.3.6 |
| dci_0 | physical layer parameters acc. TS 36.508 Table 4.3.6.1.1-1 |
| dci_0A | physical layer parameters acc. TS 36.508 Table 4.3.6.1.1B-1 |
| dci_0B | physical layer parameters acc. TS 36.508 Table 4.3.6.1.1C-1 |
| dci_1 | physical layer parameters acc. TS 36.508 Table 4.3.6.1.2-1 |
| dci_1A | physical layer parameters acc. TS 36.508 Table 4.3.6.1.3-1 |
| dci_1B |  |
| dci_1C | physical layer parameters acc. TS 36.508 Table 4.3.6.1.4-1 |
| dci_1D |  |
| dci_2 | physical layer parameters acc. TS 36.508 Table 4.3.6.1.5-1 |
| dci_2A | physical layer parameters acc. TS 36.508 Table 4.3.6.1.6-1 |
| dci_2B |  |
| dci_2C |  |
| dci_2D |  |
| dci_3 |  |
| dci_3A |  |
| dci_4 |  |
| dci_4A |  |
| dci_4B |  |
| dci_6_0A | DCI format 6-0A is used for the scheduling of PUSCH in one UL cell |
| dci_6_0B | DCI format 6-0B is used for the scheduling of PUSCH in one UL cell |
| dci_6_1A | DCI format 6-1A is used for the compact scheduling of one PDSCH codeword in one cell and random access procedure initiated by a PDCCH order. The DCI corresponding to a PDCCH order can be carried by MPDCCH |
| dci_6_1B | DCI format 6-1B is used for the scheduling of one PDSCH codeword in one cell |
| dci_6_2 | DCI format 6-2 is used for for paging and direct indication |

PdcchResourceAllocation_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | PdcchResourceAllocation_Type |
| Comment | Resource allocation acc. TS 36.213, clause 7.1.6 |
| ra_0 |  |
| ra_1 |  |
| ra_2_Localised | => physical and virtual RB index are identical |
| ra_2_Distributed | => virtual resource allocation |
| ra_Unspecified | => None of above resource allocations and as specified by DCI format in 36.211. For example used f0r DCI 6_1B |

MIMO_PrecodingBits_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MIMO_PrecodingBits_Type |  |
| Comment | Number of bits for precoding information acc. TS 36.212, table 5.3.3.1.5-3 and 5.3.3.1.5A-1 |  |
| None | Null_Type | DCI 2A: 2 antenna ports at eNodeB (table 5.3.3.1.5A-1) |
| Bit2 | B2_Type | DCI 2A: 4 antenna ports at eNodeB (table 5.3.3.1.5A-1) |
| Bit3 | B3_Type | DCI 2: 2 antenna ports at eNodeB (table 5.3.3.1.5-3) |
| Bit6 | B6_Type | DCI 2: 4 antenna ports at eNodeB (table 5.3.3.1.5-3) |

MIMO_DciDlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MIMO_DciDlInfo_Type |  |  |
| Comment | additional information for DL DCI in case of MIMO (i.e. when a 2nd CW is specified) |  |  |
| RedundancyVersionList_2ndCW | RedundancyVersionListDL_Type | opt | list of Redundancy version for 2nd code word; shall have the same length as RedundancyVersionList_1stCW; if omit, for the 2nd CW the same RedundancyVersionList shall be applied as for the 1st CW |
| CodeWordSwapFlag | B1_Type |  | transport block to codeword mapping acc. to TS 36.212 Table 5.3.3.1.5-1 |
| PrecodingBits | MIMO_PrecodingBits_Type |  | precoding information acc. TS 36.212, table 5.3.3.1.5-3 and 5.3.3.1.5A-1 |

NumSchedSubframes_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NumSchedSubframes_Type |  |
| Comment |  |  |
| MaxTwoSubframes | B1_Type | when maxNumberOfSchedSubframes-Format0B/4B-r14 is configured by higher layers to two |
| MaxFourSubframes | B2_Type | otherwise |

DciDlInfoCommon_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DciDlInfoCommon_Type |  |  |
| Comment | used for normal DL scheduling acc. to TS 36.523-3, clause 7.3 |  |  |
| Format | PdcchDciFormat_Type |  | BCCH, PCCH and RACH Response: 1A or 1C (TS 36.213, clause 7.1) CCCH: 1A since transmission mode is not (may not be) configured at the UE yet (TS 36.213, clause 7.1) DTCH/DCCH: depending on transmission mode |
| ResourceAllocType | PdcchResourceAllocation_Type |  | depends on DCI format, e.g. ra_2_Localised or ra_2_Distributed for DCI format 1A |
| Modulation_1stCW | Modulation_Type |  | max. modulation scheme for the 1st code word; depending on the amount of data a lower modulation scheme may be by SS but not a higher one; BCCH, PCCH and RACH Response: QPSK only |
| Modulation_2ndCW | Modulation_Type |  | modulation scheme for 2nd code word in case of spatial multiplexing; can be different than 1st code word (see TS 36.211, clause 6.3.2; TS 36.212, clause 5.3.3.1.5); 'unused' when there is no spatial multiplexing; NOTE: Acc. to 36.523-3 cl. 7.3.3.4 in normal mode MIMO shall not be used => for the time being Modulation_2ndCW is always "unused" |
| FreqDomainSchedul | FreqDomainSchedulCommon_Type |  | index of 1st RB; max. number of RBs per TTI; NOTE: in case of DCI format 1C the first RB index has no meaning since distributed virtual resource blocks assigned in this case (TS 36.213, clause 7.1.6.3) NOTE: For DCI formats used for BL/CE UEs i.e DCIs 6_X, this IE identifies the Narrow band of 6 PRBs |
| RedundancyVersionList | RedundancyVersionListDL_Type |  | list of Redundancy version to be used in case of retransmission; the number of elements in the list provides the maxHARQ-Tx |
| RepetitionNumber | RepetitionNumber_Type | opt | used only for DCI format 6_X for BL/CE UEs |
| DCISubframeRepetitionNumber | DCISubframeRepetitionNumber_Type | opt | used only for DCI format 6_X for BL/CE UEs |

DciDlInfoExplicit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DciDlInfoExplicit_Type |  |  |
| Comment | used for explicit DL scheduling acc. to TS 36.523-3, clause 7.3 |  |  |
| Imcs_1stCW | Imcs_Type |  | MCS index of table 7.1.7.1-1 of TS 36.213 |
| Imcs_2ndCW | Imcs_Type |  | MCS index for the 2nd code word in case of MIMO; 'NotUsed' when MIMO is not used |
| Format | PdcchDciFormat_Type |  |  |
| ResourceAllocType | PdcchResourceAllocation_Type |  |  |
| FreqDomainSchedul | FreqDomainSchedulExplicit_Type |  | For DCI formats used for BL/CE UEs i.e DCIs 6_X, this IE identifies the resources in Narrow band of 6 PRBs |
| RedundancyVersionList | RedundancyVersionListDL_Type |  | list of Redundancy version to be used in case of retransmission the number of elements in the list provides the maxHARQ-Tx |
| MimoInfo | MIMO_DciDlInfo_Type | opt | shall be present when Imcs_2ndCW specifies a 2nd CW to be used; shall be omit when Imcs_2ndCW is 'NotUsed' |
| RepetitionNumber | RepetitionNumber_Type | opt | used only for DCI format 6_X for BL/CE UEs |
| DCISubframeRepetitionNumber | DCISubframeRepetitionNumber_Type | opt | used only for DCI format 6_X for BL/CE UEs |

DciDlInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DciDlInfo_Type |  |
| Comment |  |  |
| Auto | DciDlInfoCommon_Type | SS shall chose the appropriate TBS up to the maximum number of resource blocks |
| Explicit | DciDlInfoExplicit_Type | used in MAC or RAB tests where exact TBS needs to be specified |

DciUlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DciUlInfo_Type |  |  |
| Comment |  |  |  |
| Imcs | Imcs_Type |  | MCS index of table 8.6.1-1 of TS 36.213 |
| TransRetransmissionList | UL_TransRetransmissionList_Type |  | list of possible retransmissions and their redundancy versions (depending on being adapive or non-adaptive); the list shall - start with - "New Transmission" (normal case) or - "Adaptive Retransmission" (e.g. to request a retransmission even when the data has been acknowledged with a HARQ ACK) - end with "Adaptive Retransmission" (if there are retransmissions) NOTE1: TTCN implementation shall ensure that a reconfiguration is done not before the previous list has been fully processed NOTE2: for normal operation the list contains only one NewTransmission element (i.e. possible retransmissions are non-adaptive) |
| FreqDomainSchedul | FreqDomainSchedulExplicit_Type |  | For DCI formats used for BL/CE UEs i.e DCIs 6_0X, this IE identifies the resources in Narrow band of 6 PRBs |
| Format | PdcchDciFormat_Type | opt | If omit, default DCI format 0 will be applied; for BL/CE UEs 6_0X shall be specified |
| HarqProcessId | integer | opt | Can be present in DCI format is not 6_0X; If present SS Uses the specified HARQ process for all UL transmissions/retransmissions; |
| RepetitionNumber | RepetitionNumber_Type | opt | used only for DCI format 6_X for BL/CE UEs |
| DCISubframeRepetitionNumber | DCISubframeRepetitionNumber_Type | opt | used only for DCI format 6_X for BL/CE UEs |
| PUSCH_TriggerA | B1_Type | opt | used in DCI formats 0A/0B/4A/4B |
| TimingOffset | B4_Type | opt | used in DCI formats 0A/0B/4A/4B |
| NumberOfScheduledSubframes | NumSchedSubframes_Type | opt | used in DCI formats 0B/4B |

### D.1.3.5	Random_Access_Procedure

UplinkGrant_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UplinkGrant_Type |  |  |
| Comment | TS 36.213, clause 6.2 |  |  |
| HoppingFlag | B1_Type |  | Hopping flag |
| RB_Allocation | B10_Type |  | Fixed size resource block assignment |
| ModAndCodScheme | B4_Type |  | Truncated modulation and coding scheme |
| TPC_Command | B3_Type |  | TPC command for scheduled PUSCH |
| UL_Delay | B1_Type |  | UL delay |
| CQI_Req | B1_Type |  | CQI request |

CRNTI_ContentionResolutionCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CRNTI_ContentionResolutionCtrl_Type |  |
| Comment | configuration for Random Access Procedure in RRC_CONNECTED (see TS 36.300, clause 10.1.5.1); when SS receives C-RNTI MAC element sent by the UE after Random Access Response, SS shall deal with the C-RNTI as specified in this structure |  |
| AutomaticGrant | DciUlInfo_Type | before expiry of the contention resolution timer SS shall automatically address PDCCH using C-RNTI as sent by the UE; the UL grant is specified acc. to DciUlInfo_Type |
| None | Null_Type | Used in case of dedicated preamble transmission (Non-contention based random access procedure, see TS 36.300 clause 10.1.5.2) or to simulate failure cases for contention based random access procedure; SS shall not address PDCCH using C-RNTI => in case of contention based random access procedure: expiry of contention resolution timer on UE side |

ContentionResolutionCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ContentionResolutionCtrl_Type |  |
| Comment | NOTE: SS only needs to consider one kind of contention resolution at one time; in the initial configuration of a cell TCRNTI_Based shall be configured and the common assuption is that in RRC_CONNECTED normally there are no RACH procedures (i.e. no CRNTI_Based configuration needed) whereas e.g. in case of handover scenarios CRNTI_Based shall be configured |  |
| TCRNTI_Based | TCRNTI_ContentionResolutionCtrl_Type | TCRNTI based contention resolution (e.g. initial access), hence involves inclusion contention resolution identity in DL message 4 of RACH procedure |
| CRNTI_Based | CRNTI_ContentionResolutionCtrl_Type | CRNTI based contention resolution (e.g. in case UE is being in RRC_CONNECTED) or non-contention based random access procedure: in case of CRNTI based contention resolution the uplink message in step 3 (of RACH procedure) is followed by PDCCH transmission with UE C-RNTI to end procedure |

RandomAccessResponseParameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RandomAccessResponseParameters_Type |  |  |
| Comment | paramenters to control content of RAR sent to the UE |  |  |
| RapId | RAR_RapIdCtrl_Type |  | to control Random Access Preamble Id to be sent back to the UE; used in RAR MAC sub-header |
| InitialGrant | UplinkGrant_Type |  | initial UL grant; May be ignored if UplinkGrantCE_Mode_Type is configured |
| TimingAdvance | RACH_TimingAdvance_Type |  | timing advance: granularity of 0.52 micro sec (16*Ts); see TS 36.300, clause 5.2.7.3, TS 36.321, clause 6.1.3.5; NOTE: timing advance has impact not only on the RA procedure; SS in general needs to adjust its timing accordingly |
| TempC_RNTI | TempC_RNTI_Type |  | NOTE: For initial Random Access Procedure at network (SS) side there is no temporary C-RNTI: network assigns the C-RNTI which is used by any UE as being temporary; the UE which 'wins' the contention resolution keeps the (temporary) C-RNTI; other UEs need to repeat the RACH procedure; => at the SS the TempC_RNTI shall be 'SameAsC_RNTI' For Random Access Procedure in RRC_CONNECTED state the NW assigns a temporary C-RNTI which is replaced by the one stored at the UE; => TempC_RNTI may be 'SameAsC_RNTI' (in this case temp. C-RNTI and C-RNTI are equal what is not likely in a real network), or there is an explicit temp. C-RNTI what is used during RA procedure only (as in a real network) |
| InitialGrantCE_Mode | UplinkGrantCE_Mode_Type | opt | used by SS based on PRACH coverage enhancement levels |

RarList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RarList_Type |
| Comment | in general MAC PDU may contain one or several RARs; normally only one RAR is contained |
| record of RandomAccessResponseParameters_Type |  |

UplinkGrantCE_ModeA_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UplinkGrantCE_ModeA_Type |  |  |
| Comment | TS 36.213, clause 6.2 table 6-2 |  |  |
| Msg3_PUSCH_NB_Index | B4_Type |  | Msg3 PUSCH narrowband index |
| Msg3_PUSCH_ResAllocation | B4_Type |  | Msg3 PUSCH Resource allocation |
| Msg3_PUSCH_Repetitions | B2_Type |  | Number of Repetitions for Msg3 PUSCH |
| MCS | B3_Type |  |  |
| TPC_Command | B3_Type |  | TPC command for scheduled PUSCH |
| CSI_Req | B1_Type |  | CSI request |
| UL_Delay | B1_Type |  | UL delay |
| Msg3_4_MPDCCH_NB_Index | B2_Type |  | Msg3/4 MPDCCH narrowband index |

UplinkGrantCE_ModeB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UplinkGrantCE_ModeB_Type |  |  |
| Comment | TS 36.213, clause 6.2 table 6-2 |  |  |
| Msg3_PUSCH_NB_Index | B2_Type |  | Msg3 PUSCH narrowband index |
| Msg3_PUSCH_ResAllocation | B3_Type |  | Msg3 PUSCH Resource allocation |
| Msg3_PUSCH_Repetitions | B3_Type |  | Number of Repetitions for Msg3 PUSCH |
| TBS | B2_Type |  | TBS |
| Msg3_4_MPDCCH_NB_Index | B2_Type |  | Msg3/4 MPDCCH narrowband index |

UplinkGrantCE_Mode_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UplinkGrantCE_Mode_Type |  |  |
| Comment |  |  |  |
| CE_ModeA | UplinkGrantCE_ModeA_Type |  | If the most recent PRACH coverage enhancement level for the UE is 0 or 1, the contents of the Random Access Response Grant are interpreted according to CEModeA. |
| CE_ModeB | UplinkGrantCE_ModeB_Type |  | If the most recent PRACH coverage enhancement level for the UE is 2 or 3, the contents of the Random Access Response Grant are interpreted according to CEModeB |

RandomAccessResponse_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RandomAccessResponse_Type |  |
| Comment |  |  |
| None | Null_Type | used for unsuccessful RA procedure |
| List | RarList_Type | normally one RAR to be sent to the UE; in general there can be more than one RAR |

RandomAccessResponseCtrl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RandomAccessResponseCtrl_Type |  |  |
| Comment | configuration for Random Access Response mapped to DL-SCH mapped to PDSCH TransmissionMode: single antenna mode when there is only one antenna configured, transmit diversity else; RNTI: RA-RNTI (TS 36.321, clause 7.1); if both RAR msg and backoff indicator are 'None' SS shall not respond on random access preamble |  |  |
| DciInfo | DciDlInfoCommon_Type |  | DCI format: 1A or 1C (TS 36.213, clause 7.1) ResourceAllocType: 2 (acc. to DCI format) Modulation: QPSK Frequency domain schedule: index of 1st RB; max. number of RBs per TTI |
| Rar | RandomAccessResponse_Type |  | RAR to be sent to the UE |
| BackoffInd | RandomAccessBackoffIndicator_Type |  | possible backoff indicator; 'None' for normal cases |

RandomAccessResponseConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RandomAccessResponseConfig_Type |  |
| Comment |  |  |
| Ctrl | RandomAccessResponseCtrl_Type | contains information to control sending of RAR |
| Ctrl_CRC_Error | RandomAccessResponseCtrl_Type | same as Ctrl (see above), but MAC PDU transmitted will contain CRC bits (0-3) being toggled; no retransmissions shall be made as UE shall not send a NACK |
| None | Null_Type | to be used when there is no RAR to be sent at all |

RachProcedure_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RachProcedure_Type |  |  |
| Comment |  |  |  |
| RAResponse | RandomAccessResponseConfig_Type |  | control of how the SS shall react on RA preamble; this may be - the RAP id as expected by the UE - a RAP id not matching to the UE's RAP - a backoff indicator - nothing at all |
| ContentionResolutionCtrl | ContentionResolutionCtrl_Type |  |  |

RachProcedureList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RachProcedureList_Type |
| Comment | to simulate RACH procedure with one or more than one attempt by the UE: There is one RachProcedure_Type in the list per PRACH Preamble attempt. When PRACH preamble is repeated within a Preamble attempt (BL/CE UE), the SS shall wait for all PRACH preamble repetitions before acting as per configured RAR. The discrete set of RAPIDs to be used by UE in each CE level is provided in RACH_ConfigCommon->RACH_CE_LevelInfoList_r13 and the repetitions of preamble based on CE level is provided in PRACH_Config_v1310-> prach_ParametersListCE_r13. If UE is not repeating preambles as per its detected CE level the SS shall raises an error. The same applies to contention resolution, one per RACH procedure Msg 3 transmission plus its repetitions. 1. Normal cases: one single RandomAccessResponse is sent to the UE matching the UE's RACH preamble; contention resolution is successful immediately => list contains only one element which is used for any RA procedure (Even if a RACH procedure is repeated by the UE for any reason this element shall be used; e.g. it needs not to be handled as error when the UE sends another RACH preamble instead of the RRC connection request message) 2. Special cases: there are upto tsc_RandomAccessResponseListSize preambles sent by the UE => there are upto tsc_RandomAccessResponseListSize responses to be configured as elements of the list; SS shall start with the first element in the list and use the RAR as specified in this element; if the RAR matches at the UE side the UE will send UL data and contention resolution is performed as configured for this element; if the RAR does not match the UE sends another RAP and SS continues with the next element in the list; in this case the contention resolution of the respective element is not used; if the end of the list is reached and further RACH preambles are sent by the UE SS shall repeatively apply the last element of the list (this is necessary because there might be not enough time to reconfigure SS after the end of the list has been reached and there shall be well-defined behaviour after the list has been processed); to change from a special mode to normal mode the RachProcedureList is reconfigured by TTCN to achieve transparency and readability of the code; NOTE: when there are RACH_ConfigDedicated configured (see below) and the RA preamble matches with one the configured ones the contention resolution ctrl is obsolete (non contention based random access procedure) |
| record length(1..tsc_RandomAccessResponseListSize) of RachProcedure_Type |  |

RachProcedureConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RachProcedureConfig_Type |  |  |
| Comment | parameters to control the random access procedure; TS 36.321, clause 5.1 |  |  |
| RACH_ConfigCommon | RACH_ConfigCommon_Type | opt | acc. TS 36.331, clause 6.3.2; may not be necessary for SS; omit: "keep as it is" |
| RACH_ConfigDedicated | RACH_ConfigDedicated_Type | opt | acc. TS 36.331, clause 6.3.2; when random access preamble sent by the UE matches with the configured one, SS shall assume the random access procedure being non-contention based; initial configuration: no RACH_ConfigDedicated are configured; omit means "keep as it is" |
| RachProcedureList | RachProcedureList_Type | opt | in normal cases there is one element which is used for any RA procedure; special cases are used in MAC test cases; omit means "keep as it is" If the cell is an Active Scell, the RACH procedure configuration in the Active Scell controls the RAR transmission in the associated Pcell, for received PRACH preamble in Scell |

### D.1.3.6	System_Information_Control

Primitive to configuration BCCH/BCH

System_Information_Control: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| BcchToPbchConfig_Type | Null_Type | place holder for BCCH mapped to BCH mapped to PBCH: MIB using fixed scheduling (periodicity: 40ms); transmission mode: single antenna port configuration (layer mapping acc. TS 36.211, clause 6.3.3.1) or transmit diversity (layer mapping acc. TS 36.211, clause 6.3.3.3) depending on antenna configuration |

Sib1Schedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Sib1Schedul_Type |  |  |
| Comment | SIB1: fixed scheduling in time domain acc. TS 36.331, clause 5.2.1.2 (periodicity: 80ms; repetitions every 20ms) |  |  |
| DciInfo | DciDlInfoCommon_Type | opt | DCI format: 1A or 1C (TS 36.213, clause 7.1) ResourceAllocType: 2 (acc. to DCI format) Modulation: QPSK Frequency domain schedule: index of 1st RB; max. number of RBs per TTI |

SingleSiSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SingleSiSchedul_Type |  |  |
| Comment | specifies scheduling for a single SI in freq and time domain |  |  |
| DciInfo | DciDlInfoCommon_Type | opt | DCI format: 1A or 1C (TS 36.213, clause 7.1) ResourceAllocType: 2 (acc. to DCI format) Modulation: QPSK Frequency domain schedule: index of 1st RB; max. number of RBs per TTI |
| SubframeOffset | integer | opt | offset within the SI-window; NOTE: SI-window may span more than one frame |

SiSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SiSchedul_Type |  |  |
| Comment | specifies for a specific SI scheduling and repetitions within an SI window |  |  |
| Periodicity | SI_Periodicity_r12 | opt |  |
| Window | record of SingleSiSchedul_Type | opt | NOTE: acc. to TS 36.331, clause 5.2.1.2 the same SI may occur more than once in an SI-window; to allow this there is a "record of" even though acc. to TS 36.508, clause 4.4.3.3 all SIs are sent only once within the window |

SiSchedulList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SiSchedulList_Type |
| Comment |  |
| record length(1..maxSI_Message) of SiSchedul_Type |  |

AllSiSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | AllSiSchedul_Type |  |  |
| Comment |  |  |  |
| WindowLength | SiWindowLength_Type | opt | to calculate start of each SI window acc. TS 36.331, clause 5.2.3 |
| SiList | SiSchedulList_Type | opt | list of scheduling info for the SIs containing one ore more SIBs |
| SegmentedSiList | SiSchedulList_Type | opt | list of scheduling info for segmented SIs (e.g. SI containing SIB11); corresponds to SegmentedSIs in BcchInfo_Type: SS shall subsequently schedule the elements of the corresponding SegmentedSIs (BcchInfo_Type); e.g. SegmentedSiList[i] provided scheduling info for BcchInfo_Type's SegmentedSIs[i] and the kth element of SegmentedSIs[i] is sent at T0 + ((K * N) + k) * periodicity with K: number for segments k = 0 .. K-1 N = 0, 1, 2, ... T0, periodicity: scheduling info as given by SegmentedSiList[i] |

BcchToPdschConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | BcchToPdschConfig_Type |  |  |
| Comment | configuration for BCCH mapped to DL-SCH mapped to PDSCH TransmissionMode: single antenna mode when there is only one antenna configured, transmit diversity else; RNTI: SI-RNTI (TS 36.321, clause 7.1) |  |  |
| Sib1Schedul | Sib1Schedul_Type | opt | scheduling of SIB1 in frequency domain |
| SiSchedul | AllSiSchedul_Type | opt | scheduling of SIs in frequency and time domain |

SI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SI_List_Type |
| Comment | TS 36.331, clause 6.2.1 BCCH-DL-SCH-Message and clause 6.2.2 SystemInformation |
| record of BCCH_DL_SCH_Message |  |

SegmentedSI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SegmentedSI_List_Type |
| Comment | Each element is a list of segments; used for SIB11/12 segmentation |
| record of SI_List_Type |  |

BcchInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | BcchInfo_Type |  |  |
| Comment | all fields are declared as optional to allow modification of single field; acc. to TS 36.331, clause 9.1.1.1 "RRC will perform padding, if required due to the granularity of the TF signalling, as defined in 8.5."; therefore this needs to be done by the system simulator |  |  |
| MIB | BCCH_BCH_Message | opt | TS 36.331, clause 6.2.1 BCCH-BCH-Message and clause 6.2.2 MasterInformationBlock; NOTE: the sequence number included in MIB needs to be handled and maintained by the system simulator; that means that the sequence number being setup by TTCN will be overwritten by SS |
| SIB1 | BCCH_DL_SCH_Message | opt | TS 36.331, clause 6.2.1 BCCH-DL-SCH-Message and clause 6.2.2 SystemInformationBlockType1 |
| SIs | SI_List_Type | opt | list of SIs corresponding to SiList of AllSiSchedul_Type (i.e. element i of AllSiSchedul_Type's SiList specifies the scheduling for SIs[i]) |
| SegmentedSIs | SegmentedSI_List_Type | opt | list of SIs containing segmented SIBs; corresponds to SegmentedSiList in AllSiSchedul_Type |

BcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | BcchConfig_Type |  |  |
| Comment | all fields are optional to allow single modifications; activation time may be applied in the common part of the ASP; NOTE 1: acc. to TS 36.331, clause 9.1.1.1 there is no PDCP and RLC/MAC are in TM NOTE 2: mapping/scheduling and contents of the System Information in general is done in one go (i.e. there are no separate ports for SIB data and configuration) |  |  |
| Pbch | BcchToPbchConfig_Type | opt |  |
| Pdsch | BcchToPdschConfig_Type | opt |  |
| BcchInfo | BcchInfo_Type | opt |  |
| StopSib1Transmission | Null_Type | opt | if omitted: SS transmits SIB1 last provided in BcchInfo and associated DCI. if set: SS shall stop transmission of SIB1 and associated DCI. To resume SIB1 transmission, this flag shall be omitted and SIB1 shall be provided in BcchInfo. |

Bcch_BRConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Bcch_BRConfig_Type |  |  |
| Comment | all fields are optional to allow single modifications; activation time may be applied in the common part of the ASP; NOTE 1: acc. to TS 36.331, clause 9.1.1.8 there is no PDCP and RLC/MAC are in TM NOTE 2: mapping/scheduling and contents of the System Information in general is done in one go (i.e. there are no separate ports for SIB data and configuration) |  |  |
| Pbch | Bcch_BRToPbchConfig_Type | opt |  |
| Pdsch | Bcch_BRToPdschConfig_Type | opt |  |
| BcchInfo | Bcch_BRInfo_Type | opt |  |
| StopSib1Transmission | Null_Type | opt | if omitted: SS transmits SIB1 last provided in BcchInfo and associated DCI. if set: SS shall stop transmission of SIB1 and associated DCI. To resume SIB1 transmission, this flag shall be omitted and SIB1 shall be provided in BcchInfo. |

Bcch_BRToPbchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Bcch_BRToPbchConfig_Type |  |  |
| Comment | BCCH_BL mapped to BCH mapped to PBCH: MIB using fixed scheduling (periodicity: 40ms); transmission mode: single antenna port configuration (layer mapping acc. TS 36.211, clause 6.3.3.1) or transmit diversity (layer mapping acc. TS 36.211, clause 6.3.3.3) depending on antenna configuration |  |  |
| EnableMIB_Repetition | boolean |  | If true MIB transmission shall be repeated in subframe#9 of the previous radio frame for FDD and subframe #5 of the same radio frame for TDD. If false, no MIB repetition shall occur. |

SI_SubframeOffsetList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SI_SubframeOffsetList_Type |
| Comment |  |
| record of integer |  |

Bcch_BRToPdschConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Bcch_BRToPdschConfig_Type |  |  |
| Comment | Configuration for BCCH_BR mapped to DL-SCH mapped to PDSCH TransmissionMode: single antenna mode when there is only one antenna configured, transmit diversity else; RNTI: No RNTI as BR SIs are sent without DCI info and scheduling for SIB-1 is provided in MIB and for other SIBs in SIB 1 |  |  |
| Sib1_BRSchedul | Sib1_BRSchedul_Type | opt | scheduling of SIB1 in frequency & time domain |
| SiSchedul | BandwidthReducedAccessRelatedInfo_Type | opt | scheduling of SIs in frequency and time domain. SS shall ignore si-ValidityTime-r13 and systemInfoValueTagList-r13 and TTCN shall omit them. SS shall restrict all non-explicitly scheduled DL transmissions (TimingInfo = now) respecting subframe restriction configured by IE fdd-DownlinkOrTddSubframeBitmapBR-r13. SS shall allocate autonomous UL grants (not explicitly scheduled, eg. On SR reception) for UL transmission respecting subframe restriction configured by IEs fdd-DownlinkOrTddSubframeBitmapBR-r13 & fdd-UplinkSubframeBitmapBR-r13. |
| SubframeOffsetList | SI_SubframeOffsetList_Type | opt | offset within the SI-window; NOTE: SI-window may span more than one frame. There shall be 1 to 1 mapping with BandwidthReducedAccessRelatedInfo_Type.schedulingInfoList_BR_r13 elements |

Sib1_BRSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Sib1_BRSchedul_Type |  |  |
| Comment | SIB1: fixed scheduling in time domain acc. TS 36.331, clause 5.2.1.2 (periodicity: 80ms) For a BL/CE UE, the resource allocation for PDSCH carrying SystemInformationBlockType1-BR and SI messages is a set of six contiguously allocated localized virtual resource blocks within a narrowband . the Narrow band index used for a transmission is as per 36.211 clause 6.4.1 |  |  |
| SchedulingInfoSIB1_BR_r13 | SchedulingInfoSIB1_BR_r13_Type |  | TBS for SystemInformationBlockType1-BR and the repetitions made within 80ms as indicated in MIB |

BR_SI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | BR_SI_List_Type |
| Comment | TS 36.331, clause 6.2.1 BCCH-DL-SCH-Message-BR and clause 6.2.2 SystemInformation |
| record of BCCH_DL_SCH_Message_BR |  |

BR_SegmentedSI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | BR_SegmentedSI_List_Type |
| Comment | Each element is a list of segments; used for SIB11/12 segmentation |
| record of BR_SI_List_Type |  |

Bcch_BRInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Bcch_BRInfo_Type |  |  |
| Comment | all fields are declared as optional to allow modification of single field; acc. to TS 36.331, clause 9.1.1.1 "RRC will perform padding, if required due to the granularity of the TF signalling, as defined in 8.5."; therefore this needs to be done by the system simulator |  |  |
| MIB | BCCH_BCH_Message | opt | The MIB shall not be present if transmission of MIB is configured by BcchInfo_Type. TS 36.331, clause 6.2.1 BCCH-BCH-Message and clause 6.2.2 MasterInformationBlock; NOTE: the sequence number included in MIB needs to be handled and maintained by the system simulator; that means that the sequence number being setup by TTCN will be overwritten by SS |
| SIB1 | BCCH_DL_SCH_Message_BR | opt | TS 36.331, clause 6.2.1 BCCH-DL-SCH-Message-BR and clause 6.2.2 SystemInformationBlockType1 |
| SIs | BR_SI_List_Type | opt | list of SIs corresponding to SiList of AllSiSchedul_Type (i.e. element i of AllSiSchedul_Type's SiList specifies the scheduling for SIs[i]) |
| SegmentedSIs | BR_SegmentedSI_List_Type | opt | list of SIs containing segmented SIBs; corresponds to SegmentedSiList in AllSiSchedul_Type |

### D.1.3.7	Paging_Control

Primitive to configuration PCCH/PCH

PcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PcchConfig_Type |  |  |
| Comment | configuration for PCCH mapped to PCH mapped to PDSCH TransmissionMode: single antenna mode when there is only one antenna configured, transmit diversity else; RNTI: P-RNTI (TS 36.321, clause 7.1) NOTE: acc. to TS 36.331, clause 9.1.1.3 there is no PDCP and RLC/MAC are in TM |  |  |
| DciInfo | DciDlInfoCommon_Type | opt | DCI format: 1A or 1C (TS 36.213, clause 7.1) ResourceAllocType: 2 (acc. to DCI format) Modulation: QPSK Frequency domain schedule: index of 1st RB; max. number of RBs per TTI |
| PCCH_Config_v13 | PCCH_Config_v1310 | opt | provides MPDCCH repetition for PCCH etc |

### D.1.3.8	UE_Specific_Channel_Configuration

UE_Specific_Channel_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| N_PSCCH_Type | B6_Type |  |
| Itrp_Type | integer (0..127) |  |

DrxCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DrxCtrl_Type |  |
| Comment | DRX configuration for connected mode (TS 36.321, clause 5.7) |  |
| None | Null_Type | DRX not configured |
| Config | DRX_Config_Type | DRX is configured as signalled to the UE; NOTE: the release branch of DRX-Config in general is not used for configuration of the SS |

MeasGapCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MeasGapCtrl_Type |  |
| Comment | support of measurement gap configuration |  |
| None | Null_Type | no measurement gap configuration |
| Config | MeasGapConfig_Type | measurement gap configuration acc. to TS 36.331, clause 6.3.5 and gap pattern acc. TS 36.133 Table 8.1.2.1-1; NOTE: the release branch of MeasGapConfig in general is not used for configuration of the SS |

CcchDcchDtchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CcchDcchDtchConfig_Type |  |  |
| Comment |  |  |  |
| MeasGapCtrl | MeasGapCtrl_Type | opt | to tell the SS when no assignments/grants shall be assigned to the UE |
| DL | CcchDcchDtchConfigDL_Type | opt | Scheduling, parameters related to CCCH, DCCH and DTCH in DL |
| UL | CcchDcchDtchConfigUL_Type | opt | Scheduling, parameters related to CCCH, DCCH and DTCH in UL |
| DrxCtrl | DrxCtrl_Type | opt | DRX configuration as sent to the UE (or 'None' when the UE does not support connected mode DRX) |
| TtiBundling | TTI_BundlingConfig_Type | opt | TTI bundling as configured at the UE |
| CifPresence | boolean | opt | corresponds to PhysicalConfigDedicated.cif_Presence_r10: The CIF field is applied for dedicated search space scheduling i.e. DCCH/DTCH. Not present for common search space scheduling. CIF indicator as true may be configured even in non CA cell, to facilitate the future false: no serving cell is cross scheduled by this cell true: carrier indicator field is present when the PDCCH CRC is scrambled by C-RNTI or SPS C-RNTI omit means "keep as it is" |

SciSchedulingConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SciSchedulingConfig_Type |  |
| Comment |  |  |
| SCI0 | Sci0SchedulingConfig_Type |  |
| SCI1 | Sci1SchedulingConfig_Type |  |

Sci0SchedulingConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Sci0SchedulingConfig_Type |  |  |
| Comment | Scheduling for STCH mapped to SL-SCH mapped to PSSCH DPCCH DCI format 5 (TS 36.213, clause 14.2) UL grant allocation to be applied: On SL-BSR Reception For the time being there is no requirement to configure the SS with TPC-PDCCH-Config; In general SS is required to keep the UE's UL power constant |  |  |
| SL_RNTI_r12 | C_RNTI | opt |  |
| N_PSCCH | N_PSCCH_Type | opt | Resource for PSCCH - 6 bits |
| SL_Sheduling | SL_Sci0Config_Type | opt | SCI format 0 configuration |

SL_Sci0Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_Sci0Config_Type |  |  |
| Comment | SCI format 0 (TS 36.213 clause 14.2 TS 36.212 clause 5.3.3.1.9) ResourceAllocType: 0 (36.213 cl 14.1.1.2.1 and 8.1.1) Frequency domain schedule: index of 1st RB; max. number of RBs per TTI (upper bound up to which SS may assign grants to the UE) |  |  |
| Hopping | UplinkHoppingControl_Type |  | Frequency hopping flag - bit |
| FreqDomainSchedul | FreqDomainSchedulExplicit_Type |  | Resource block assignment |
| Itrp | Itrp_Type |  | Time resource pattern of 14.1.1.1.1 of TS 36.213 |

Sci1SchedulingConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Sci1SchedulingConfig_Type |  |  |
| Comment | Scheduling for STCH mapped to SL-SCH mapped to PSSCH DPCCH DCI format 5A (TS 36.213 clause 14.2, TS 36.212 clause 5.3.3.1.9A) UL grant allocation to be applied on SL-BSR Reception |  |  |
| SL_V_RNTI_r12 | C_RNTI | opt | Set to SL-V-RNTI or SL-SPS-V-RNTI when SL-SPS is configured |
| CarrierIndicator | B3_Type | opt |  |
| LowestIndexOfChannelAllocation | bitstring | opt | Lowest index of the subchannel allocation to the initial transmission |
| SL_Index | B2_Type | opt | For TDD |
| SL_SPS_ConfigurationIndex | B3_Type | opt | Applied for SL-SPS |
| SL_SPS_ActivationRelease | B1_Type | opt | Applied for SL-SPS - Activation/release indication as defined in section 14.2.1 of TS 36.213 |
| SL_SCI1_Scheduling | SL_Sci1Config_Type | opt | SCI format 1 configuration |

SL_Sci1Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_Sci1Config_Type |  |  |
| Comment | SCI format 1 parameters (TS 36.213 clause 14.2.1, TS 36.212 clause 5.3.3.1.9A) |  |  |
| FreqResourceAllocation | bitstring |  | Frequency resource location of initial transmission and retransmission |
| TimeGap | B4_Type |  | Time gap between initial transmission and retransmission |

#### D.1.3.8.1	UE_Specific_Channel_Configuration_DL

Scheduling and other information for CCCH/DCCH/DTCH mapped to DL-SCH mapped to PDSCH

HarqProcessConfigDL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | HarqProcessConfigDL_Type |  |
| Comment | HARQ processes to be used automatically for DL assignments |  |
| AllProcesses | Null_Type | all HARQ processes shall be used for automatic assignmnet; this is the normal case |
| SpecificSubset | HarqProcessList_Type | only the HARQ processes of this list shall be used automatically, other processes are excluded from automatic assignments; nevertheless all HARQ processes may be addressed explicitly by DRB_DataPerSubframe_DL_Type.HarqProcess |

CcchDcchDtchConfigDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CcchDcchDtchConfigDL_Type |  |  |
| Comment | configuration for CCCH/DCCH/DTCH mapped to DL-SCH mapped to PDSCH TransmissionMode: as signalled to the UE (AntennaInfoDedicated in RRCConnectionSetup); RNTI: C-RNTI (TS 36.321, clause 7.1); all fields optional (omit = "keep as it is") since DCI format and modulation may be changed during a test; for initial configuration all fields are mandatory |  |  |
| DciInfo | DciDlInfo_Type | opt | DCI format: 1A per default since for CCCH mimo cannot be applied in general ResourceAllocType: (depending on DCI format) Modulation: QPSK for signalling Frequency domain schedule: index of 1st RB; max. number of RBs per TTI; in case of spatial multiplexing if there are 2 code words FreqDomainSchedul shall be applied to both |
| AntennaInfo | AntennaInfoDedicated_Type | opt | as signalled to the UE (TS 36.331, clause 6.3.2): transmissionMode, codebookSubsetRestriction |
| HarqProcessConfig | HarqProcessConfigDL_Type | opt | HARQ processes automatically used by the SS in DL |

#### D.1.3.8.2	UE_Specific_Channel_Configuration_UL

Scheduling information for CCCH/DCCH/DTCH mapped to UL-SCH mapped to PUSCH

PucchHoppingBits_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PucchHoppingBits_Type |  |
| Comment | Number of hopping bits acc. to TS 36.213 table 8.4-2 |  |
| OneBit | B1_Type | N(UL, RB) = 6..49 i.e. default system bandwid this less than 10 MHz (does not include 10 MHz) |
| TwoBits | B2_Type | N(UL, RB) = 50..110 i.e. default system bandwidth is 10 MHz or above |

UplinkHoppingResourceParameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UplinkHoppingResourceParameters_Type |  |  |
| Comment |  |  |  |
| PucchHopping | PucchHoppingBits_Type |  | to control hopping resource allocation as signalled in DCI format 0 (TS 36.212, clause 5.3.3.1.1) |

UplinkHoppingControl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UplinkHoppingControl_Type |  |
| Comment | shall be considered by SS to fill in the information needed for DCI format 0 (TS 36.213, clause 7.1) |  |
| Deactivated | Null_Type |  |
| Activated | UplinkHoppingResourceParameters_Type |  |

CcchDcchDtchConfigUL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CcchDcchDtchConfigUL_Type |  |  |
| Comment | scheduling for CCCH/DCCH/DTCH mapped to UL-SCH mapped to PUSCH NOTE 1: for definition of the possible UL grants the location of the PUCCH (TS 36.211, clause 5.4.3) and the PRACH (TS 36.211, clause 5.7.3) need to be taken into account; NOTE 2: In contrast to the DL where the scheduling can be done (with consideration of some restrictions) by SS on a per need basis in the UL the scheduling depends on information provided by the UE: e.g. BSR (buffer status report), SR (scheduling request) see TS 36.523-3 clause 7.2 for further information. |  |  |
| DciInfo | DciUlInfo_Type | opt | DCI format: 0 (TS 36.213, clause 7.1) ResourceAllocType: 2 (acc. to DCI format) Modulation: QPSK per default Frequency domain schedule: index of 1st RB; max. number of RBs per TTI (upper bound up to which SS may assign grants to the UE) |
| Hopping | UplinkHoppingControl_Type | opt | when Hopping = 'Activated' SS shall set hopping flag in DCI format 0 |
| PUCCH_Synch | UplinkTimeAlignment_Synch_Type | opt | parameters to control automatic control of timing advance |
| UL_GrantConfig | UL_GrantConfig_Type | opt | UL grant allocation to be applied |

### D.1.3.9	Carrier_Aggregation

ActivateScell_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ActivateScell_Type |  |  |
| Comment |  |  |  |
| ScellActivation | ScellBitMap_Type |  | 36.321 clause 6.1.3.8; B0=C7, B1=C6 .. B6=C1, B7 is reserved. B0 to B6, 1 means Activate associated Scell |
| SendMCE | boolean |  | If true the SS sends a MAC Control Element to the UE |

Scell_Capability_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | Scell_Capability_Type |
| Comment |  |
| DlOnly | the CC is configured in DL only, no aggregation in this cell in UL |
| UL_DL | the aggregation is configured in both UL and DL |

ScellDeactivationTimer_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ScellDeactivationTimer_Type |  |
| Comment | NOTE: this type is a union to allow semantic of "keep as it is" for optional fields of this type |  |
| NumberOfRadioFrames | MAC_MainConfig_ScellDeactivationTimer_Type | SCell deactivation timer acc. to TS 36.321 |
| Infinity | Null_Type | infinity as when 'sCellDeactivationTimer' is omitted in 'MAC-MainConfig' sent to the UE |

CrossCarrierScheduledCellsList_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CrossCarrierScheduledCellsList_Type |  |  |
| Comment |  |  |  |
| crossCarrierSchedulingConfig_r10 | CrossCarrierSchedulingConfig_r10 | opt | present if cross carrier scheduling needs to be configured and CrossCarrierSchedulingConfig_r13 is not present |
| crossCarrierSchedulingConfig_r13 | CrossCarrierSchedulingConfig_r13 | opt | present if cross carrier scheduling needs to be configured and CrossCarrierSchedulingConfig_r10 is not present |

SchedulingCarrierConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SchedulingCarrierConfig_Type |  |
| Comment |  |  |
| Own | CrossCarrierScheduledCellsList_Type | Cell is scheduled by itself and possible cross schedules other Scells |
| CrossScheduled | CrossSchedulingCarrierInfo_Type | Cell is cross Scheduled by other carrier; the CIF field shall be configured in the serving cell scheduling this scell |

CrossCarrierSchedulingConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CrossCarrierSchedulingConfig_Type |  |
| Comment |  |  |
| Config | SchedulingCarrierConfig_Type | When cross carrier scheduling is enabled then the CIF field shall be configured in the serving cell scheduling this scell |
| None | Null_Type |  |

PrimaryCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PrimaryCellInfo_Type |  |  |
| Comment |  |  |  |
| AssociatedScellList | EUTRA_CellIdList_Type |  | List of Scells associated with the Pcell/PScells (needs to be consistent with AssociatedPcellId in Scell/PScell ) |
| MeasSubframePatternPCell | MeasSubframePatternPCell_r10 | opt |  |
| CrossCarrierScheduledCellsList | CrossCarrierScheduledCellsList_Type | opt | Information of possible Cells Cross Scheduled by this cell |

Pcell_Mode_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | Pcell_Mode_Type |  |
| Comment |  |  |
| FDD | Null_Type |  |
| TDD | TDD_SubframeAssignment_Type |  |

SecondaryCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SecondaryCellInfo_Type |  |  |
| Comment |  |  |  |
| AssociatedPcellId | EUTRA_CellId_Type |  | cell ID of associated Pcell/PSCell (eutra_Cell_NonSpecific is not allowed) |
| SCellIndex | SCellIndex_r10 |  |  |
| Scell_Capability | Scell_Capability_Type | opt | if DL only or both UL and DL (omit means "keep as it is") |
| ScellDeactivationTimer | ScellDeactivationTimer_Type | opt | SCell deactivation timer; omit means "keep as it is"; when there is more than one SCell associated to the same PCell this field shall be set to the same value for each SCell |
| CrossCarrierSchedulingConfig | CrossCarrierSchedulingConfig_Type | opt | omit means "keep as it is" |
| STAG_Id | STAG_Id_r11 | opt | if IE is never configured then scell is part of PTAG. Uses the STAG_ID or PTAG in Auto PUCCH synchronization when transmitting periodically Timing Advance MCE |
| Pcell_Mode | Pcell_Mode_Type | opt | omit means Pcell/PScell and Scell mode are same. To be provide for FDD-TDD mixed mode CA/Dual connectivity configurations |

ServingCellConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ServingCellConfig_Type |  |
| Comment |  |  |
| PCell | PrimaryCellInfo_Type | cell shall become PCell |
| SCell | SecondaryCellInfo_Type | cell shall become SCell |
| PSCell | PSCellInfo_Type | cell shall become PSCell |
| Release | Null_Type | cell is changed back to normal non CA cell |

PSCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PSCellInfo_Type |  |  |
| Comment |  |  |  |
| AssociatedPcellId | EUTRA_CellId_Type |  | cell ID of associated Pcell (eutra_Cell_NonSpecific is not allowed) |
| SCellIndex | SCellIndex_r10 |  |  |
| AssociatedScellList | EUTRA_CellIdList_Type |  | List of Scells associated with the PScell (needs to be consistent with AssociatedPcellId in Scell) |
| CrossCarrierSchedulingConfig | CrossCarrierSchedulingConfig_Type | opt | omit means "keep as it is" |
| Pcell_Mode | Pcell_Mode_Type | opt | omit means PCell and PSCell mode are same. To be provide for FDD-TDD mixed mode Dual connectivity configurations |

### D.1.3.10	OCNG_Config

OCNG_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | OCNG_Config_Type |  |
| Comment |  |  |
| Activate | OCNG_ActivateInfo_Type | CellId : identifier of the cell where the OCNG needs to be activated RoutingInfo : None TimingInfo : activation time from when the SS starts transmission of uncorrelated pseudo random MAC TB's addressed to Virtual UE's in non ABS subframes. ControlInfo : CnfFlag:=false; FollowOnFlag:=false |
| Deactivate | Null_Type | CellId : identifier of the cell where the OCNG needs to be deactivated RoutingInfo : None TimingInfo : activation time from when the SS stops transmission of MAC TB's addressed to Virtual UE's in non ABS subframes. ControlInfo : CnfFlag:=false; FollowOnFlag:=false |

OCNG_ActivateInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | OCNG_ActivateInfo_Type |  |  |
| Comment | OFDMA Channel Noise Generator (OCNG): SS for transmitting uncorrelated pseudo random data in MAC TB's addressed to Virtual UE's, will not do any retransmissions due to no HARQ ACK/NACK reception being considered as NACK. |  |  |
| MeasSubFramePattern | MeasSubframePattern_r10 |  | provides the bit string for x (size of bit string divided by 10) frames to be repeated continuously from provided timing info; 1 represents ABS subframe hence no pseudo random MAC TB is scheduled 0 represent non-ABS subframe and SS automatically transmits MAC TB's consisting of pseudo random data in each non-ABS subframe |

### D.1.3.11	EIMTA_Config

EIMTA_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | EIMTA_Config_Type |  |  |
| Comment |  |  |  |
| EIMTA_MainConfig_r12 | EIMTA_MainConfig_r12 |  | The IE EIMTA-MainConfig is used to specify the eIMTA-RNTI used for eIMTA and the subframes used for PDCCH monitoring by the UE with eIMTA-RNTI |
| EIMTA_MainConfigServCell_r12 | EIMTA_MainConfigServCell_r12 |  | The IE EIMTA-MainConfigServCell is used to specify the eIMTA related parameters applicable for the cell |

PeriodicTransmission_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PeriodicTransmission_Type |  |  |
| Comment |  |  |  |
| Period | integer (1..infinity) |  | DCI 1C transmission period (TTI=1ms); the starting time, Period and NumRepetitions need to be chosen in TTCN so that the DCI 1C transmissions are assigned at valid subframes according to EIMTA_MainConfig_r12 settings; otherwise SS shall raise an error |
| NumRepetitions | TransmissionRepetition_Type |  | number of DCI 1C repetitions to be automatically transmitted or 'Continuous' |

PDCCH_TDD_ConfigOrder_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCCH_TDD_ConfigOrder_Type |  |
| Comment |  |  |
| OnlyOnce | Null_Type | DCI 1C is sent out on PDCCH with CRC scrambled by eIMTA-RNTI only once; no period |
| Periodic | PeriodicTransmission_Type | SS transmits DCI 1C periodically as configured by EIMTA_MainConfig_Type |
| None | Null_Type | stop DCI 1C transmission |

## D.1.4	Cell_Power_Attenuation

CellAttenuationConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CellAttenuationConfig_Type |  |  |
| Comment |  |  |  |
| CellId | EUTRA_CellId_Type |  |  |
| Attenuation | Attenuation_Type |  |  |
| TimingInfo | TimingInfo_Type | opt |  |

CellAttenuationList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | CellAttenuationList_Type |
| Comment |  |
| record length(1..tsc_EUTRA_MaxNumberOfCells) of CellAttenuationConfig_Type |  |

## D.1.5	Radio_Bearer_Configuration

Radio Bearer Configuration: SRBs/DRBs

Radio_Bearer_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| TestLogicalChannelId_Type | integer (0..31) | To be used in MAC test mode for reserved values of Logical channels; |

RadioBearerConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RadioBearerConfigInfo_Type |  |  |
| Comment | semantics of omit: "keep as it is" |  |  |
| Sdap | SDAP_Configuration_Type | opt | only relevant for NE-DC, otherwise omitted; for NE-DC: mandatory if PDCP configured on EUTRA e.g. set to omit for MN terminated SCG omit means "keep as it is" |
| Pdcp | PDCP_Configuration_Type | opt | for SRB0: "Pdcp.None:=true" mandatory for initial configuration; omit means "keep as it is" |
| Rlc | RLC_Configuration_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| LogicalChannelId | LogicalChannelId_Type | opt | DRBs: DTCH-LogicalChannelIdentity as for rb-MappingInfo in DRB-ToAddModifyList; SRBs: for SRBs specified configurations acc. to TS 36.331, clause 9.1.2 shall be applied: SRB1: ul-LogicalChannel-Identity = dl-LogicalChannel-Identity = 1 SRB2: ul-LogicalChannel-Identity = dl-LogicalChannel-Identity = 2 for SRB0 being mapped to CCCH the LCID is '00000'B acc. to TS 36.321, clause 6.2.1; mandatory for initial configuration; omit means "keep as it is" |
| Mac | MAC_Configuration_Type | opt |  |
| DiscardULData | boolean | opt | if omitted: initial configuration: data is handed over to TTCN as usual re-configuration: "keep as it is" if set: true - SS shall discard any data in UL for this radio bearer false - (re)configuration back to normal mode NOTE: typically applicable for UM DRBs only |

RadioBearerConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RadioBearerConfig_Type |  |
| Comment |  |  |
| AddOrReconfigure | RadioBearerConfigInfo_Type | add / re-configure RB - CellId : identifier of the cell being configured RoutingInfo : None TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Release | Null_Type | release RB - CellId : identifier of the cell being configured RoutingInfo : None TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

RadioBearer_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RadioBearer_Type |  |  |
| Comment |  |  |  |
| Id | RadioBearerId_Type |  | either for SRB or DRB |
| Config | RadioBearerConfig_Type |  |  |

RadioBearerList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RadioBearerList_Type |
| Comment | array of SRBs and/or DRBs (DRBs + 3 SRBs) |
| record length (1..tsc_MaxRB) of RadioBearer_Type |  |

### D.1.5.1	PDCP_Configuration

PDCP_SNLength_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | PDCP_SNLength_Type |
| Comment | PDCP Sequence Number |
| PDCP_SNLength5 | TS 36.323 clause 6.2.2 |
| PDCP_SNLength7 | TS 36.323 clause 6.2.3 |
| PDCP_SNLength12 | TS 36.323 clause 6.2.4 |
| PDCP_SNLength16 | TS 36.323 clause 6.2.10 |

PDCP_ROHC_Mode_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_ROHC_Mode_Type |  |  |
| Comment |  |  |  |
| SN_Size | PDCP_SNLength_Type |  |  |

PDCP_NonROHC_Mode_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_NonROHC_Mode_Type |  |  |
| Comment |  |  |  |
| SN_Size | PDCP_SNLength_Type |  |  |

PDCP_TestModeInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_TestModeInfo_Type |  |
| Comment |  |  |
| PDCP_ROHC_Mode | PDCP_ROHC_Mode_Type | ROHC test mode acc. to TS 36.523-3, clause 4.2.1.3.1; requires PDCP to be configured for this RB => - SS applies ciphering in UL and DL - SS maintains PDCP sequence numbers and state variables Furthermore in this mode - SS does not add/remove PDCP headers (in UL the PDCP PDUs are decoded depending on SN_Size) - SS applies ROHC in DL only |
| PDCP_NonROHC_Mode | PDCP_NonROHC_Mode_Type | PDCP test mode acc. to TS 36.523-3, clause 4.2.1.3.2 (non-ROCH test mode); requires PDCP to be configured as transparent => - SS does not apply ciphering in UL and DL - SS does not interpret, insert or remove PDCP headers (in UL PDCP PDUs are decoded depending on SN_Size) - SS does not maintain PDCP sequence numbers and state variables |

PDCP_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Info | PDCP_TestModeInfo_Type |  |

PDCP_RbConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_RbConfig_Type |  |
| Comment |  |  |
| Srb | Null_Type | for SRB1/2 there are no PDCP_Parameters; SN is always 5 bits |
| Drb | PDCP_Config_Type | PDCP-Configuration acc. to TS 36.331, clause 6.3.2; among others for UM here pdcp-SN-Size is configured to be either len7bits or len12bits; for AM it always is 12bit |
| Transparent | Null_Type | used for PDCP tests (TS 36.523-3, clause 4.2.1.3.2): the SS does not apply ciphering and does not maintain PDCP sequence numbers and state variables; in UL the PDCP PDUs are decoded acc. to the TestMode; Note: a reconfiguration of a RB from transparent mode to 'normal' mode is not foreseen (i.e. there is no mechanism to restore Ciphering, PDCP sequence numbers and state variables at the SS) |

PDCP_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_ConfigInfo_Type |  |  |
| Comment |  |  |  |
| Rb | PDCP_RbConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | PDCP_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

PDCP_Configuration_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_Configuration_Type |  |
| Comment |  |  |
| None | Null_Type | for SRB0 no PDCP is configured; furthermore the PDCP may not be configured e.g. for DRBs tested in MAC test cases |
| Config | PDCP_ConfigInfo_Type | 4G PDCP configuration |
| NR_PDCP | NR_PDCP_RBTerminating_Type | 5G PDCP configuration |
| Proxy | NR_PDCP_Proxy_Type | PDCP Proxy to be used for (MR)-DC |

### D.1.5.2	RLC_Configuration

RLC configuration: radio bearer specific

RLC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_AM_SequenceNumber_Type | integer (0..1023) | RLC AM sequence number |
| SS_RLC_TM_Type | Null_Type | TM to configure SRB0; no parameters to be defined |

RLC_ACK_Prohibit_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | RLC_ACK_Prohibit_Type |
| Comment |  |
| Prohibit | cause SS RLC layer to stop any ACK transmission for UL PDU's received from UE |
| Continue | bring back the SS RLC in normal mode, where ACK/NACK are transmitted at polling |

RLC_NotACK_NextRLC_PDU_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | RLC_NotACK_NextRLC_PDU_Type |
| Comment |  |
| Start | cause SS RLC layer not to ACK the next received RLC PDU; this is done regardless of whether the poll bit is set or not; Example [from UMTS]: when the UE gets new security information in a SECURITY MODE COMMAND the response (SECURITY MODE COMPLETE) sent by the UE is not acknowledged at the RLC level; this causes the UE to continue using the "old" security information |

RLC_TestModeInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_TestModeInfo_Type |  |
| Comment |  |  |
| AckProhibit | RLC_ACK_Prohibit_Type | valid only when the RLC is configured in AM |
| NotACK_NextRLC_PDU | RLC_NotACK_NextRLC_PDU_Type | valid only when the RLC is configured in AM |
| ModifyVTS | RLC_AM_SequenceNumber_Type | to modify the VT(S) at SS: VT(S) at the SS side is set to this (absolute) value; valid only when the RLC is configured in AM |
| TransparentMode_UMDwith5BitSN | Null_Type | shall be set when TTCN expects RLC PDUs as UMD in UL with an SN of 5 bits; valid only when the RLC is configured in TM |
| TransparentMode_UMDwith10BitSN | Null_Type | shall be set when TTCN expects RLC PDUs as UMD in UL with an SN of 10 bits; valid only when the RLC is configured in TM |
| TransparentMode_AMD | Null_Type | shall be set when TTCN expects RLC PDUs as AMD in UL; valid only when the RLC is configured in TM |

RLC_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Info | RLC_TestModeInfo_Type |  |

SS_RLC_AM_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SS_RLC_AM_Type |  |  |
| Comment |  |  |  |
| Tx | UL_AM_RLC_Type | opt | the UE's UL setting to be used in SS's tx direction |
| Rx | DL_AM_RLC_Type | opt | the UE's DL setting to be used in SS's rx direction |
| ExtendedLI | RLC_Config_v1250 | opt | Indicates the RLC LI field size. Value TRUE means that 15 bit LI length shall be used, otherwise 11 bit LI length shall be used |

SS_RLC_UM_Bi_Directional_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SS_RLC_UM_Bi_Directional_Type |  |  |
| Comment |  |  |  |
| Tx | UL_UM_RLC_Type | opt | the UE's UL setting to be used in SS's tx direction |
| Rx | DL_UM_RLC_Type | opt | the UE's DL setting to be used in SS's rx direction |

SS_RLC_UM_Uni_Directional_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SS_RLC_UM_Uni_Directional_UL_Type |  |  |
| Comment |  |  |  |
| Rx | DL_UM_RLC_Type | opt | the UE's DL setting to be used in SS's rx direction |

SS_RLC_UM_Uni_Directional_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SS_RLC_UM_Uni_Directional_DL_Type |  |  |
| Comment |  |  |  |
| Tx | UL_UM_RLC_Type | opt | the UE's UL setting to be used in SS's tx direction |

RLC_RbConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_RbConfig_Type |  |
| Comment |  |  |
| AM | SS_RLC_AM_Type |  |
| UM | SS_RLC_UM_Bi_Directional_Type |  |
| UM_OnlyUL | SS_RLC_UM_Uni_Directional_UL_Type |  |
| UM_OnlyDL | SS_RLC_UM_Uni_Directional_DL_Type |  |
| TM | SS_RLC_TM_Type | normally SRB0 only; may be used for test purposes also |

RLC_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Configuration_Type |  |  |
| Comment |  |  |  |
| Rb | RLC_RbConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | RLC_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

### D.1.5.3	MAC_Configuration

MAC configuration: radio bearer specific configuration

EUTRA_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_MaxHarqRetransmission | integer | 28 | maximum value for maxHARQ-Msg3Tx as being signalled to the UE |

MAC_Test_DLLogChID_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MAC_Test_DLLogChID_Type |  |
| Comment |  |  |
| LogChId | TestLogicalChannelId_Type | Specifies to over write the logical channel ID in MAC header in all the DL messages sent on the configured logical channel |
| ConfigLchId | Null_Type | Specifies that the normal mode of correct logical channel ID to be used in DL MAc header. This will be the default mode, when SS is initially configured. |

MAC_Test_DL_SCH_CRC_Mode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | MAC_Test_DL_SCH_CRC_Mode_Type |
| Comment |  |
| Normal | default mode, the CRC generation is correct |
| Erroneous | SS shall generate CRC error by toggling CRC bits; the CRC error shall be applied for all PDUs of the given RNTI and their retransmission until SS is configured back to 'normal' operation |
| Error1AndNormal | the SS generates wrong CRC for first transmission and correct CRC on first retransmission. Later SS operates in normal mode. The retransmission is automatically triggered by reception of HARQ NACK |

MAC_Test_SCH_NoHeaderManipulation_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | MAC_Test_SCH_NoHeaderManipulation_Type |
| Comment |  |
| NormalMode | MAC header is fully controlled by the SS |
| DL_SCH_Only | TTCN can submit a final MAC PDU including header and payloads; SS does not do anything with this MAC PDU i.e. no header is added for the DL SCH transport channel. It is possible that data belonging to multiple DRBs is sent in one MAC PDU and from one special RB configured. NOTE: SRBs shall work as in normal mode and data can be sent/received on SRBs but sending on SRBs shall be in different TTIs than sending data PDUs. |
| DL_UL_SCH | In UL and DL the SS' MAC layer is transparent i.e. SS does not add or remove any MAC header |

HARQ_ModeList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | HARQ_ModeList_Type |
| Comment |  |
| record length (1..tsc_MaxHarqRetransmission) of HARQ_Type |  |

PhichTestMode_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PhichTestMode_Type |  |
| Comment |  |  |
| NormalMode | Null_Type | PHICH is configured to operate in normal mode |
| ExplicitMode | HARQ_ModeList_Type | the number of elements in explicit list shall match the number of retransmissions being expected |

MAC_TestModeInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_TestModeInfo_Type |  |  |
| Comment | Parameters/Configuration for MAC tests |  |  |
| DiffLogChId | MAC_Test_DLLogChID_Type |  | to be used in test cases 7.1.1.1 and 7.1.1.2 for using a different logical channel ID in MAC-header on DL-SCH channel |
| No_HeaderManipulation | MAC_Test_SCH_NoHeaderManipulation_Type |  | to configure mode for no header manipulation in SS MAC layer for DL/UL SCH |

MAC_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MAC_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Info | MAC_TestModeInfo_Type |  |

MAC_LogicalChannelConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_LogicalChannelConfig_Type |  |  |
| Comment |  |  |  |
| Priority | integer |  | logical channel priority for the DL as described in TS 36.321, clause 5.4.3.1 for the UL |
| PrioritizedBitRate | PrioritizedBitRate_Type |  | PBR as described for the UL; probably not needed at SS |
| LAA_UL_Allowed | LAA_UL_Allowed_Type | opt | Indicates whether the data of a logical channel is allowed to be transmitted via UL of LAA SCells |

MAC_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_Configuration_Type |  |  |
| Comment |  |  |  |
| LogicalChannel | MAC_LogicalChannelConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | MAC_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is"; for none MAC tests "TestMode.None:=true" |

## D.1.6	AS_Security

Primitive for control of AS security

PdcpSQN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PdcpSQN_Type |  |  |
| Comment |  |  |  |
| Format | PdcpCountFormat_Type |  | 5 bit, 7 bit or 12 bit SQN |
| Value | integer |  | SQN value (5 bit, 7 bit or 12 bit SQN) NOTE: in TTCN the test case writer is responsible to deal with potential overflows (e.g. there shall be a "mod 32", "mod 128" or "mod 4096" according to the format) |

PDCP_ActTime_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_ActTime_Type |  |
| Comment | The sequence number in UL and DL for SRB1 should be one more than the present SQN, as Ciphering starts in UL and DL soon after SMC and SMComp; For other SRB/DRB it should be the present SQN. |  |
| None | Null_Type | No Activation time; to be used if Ciphering is not applied |
| SQN | PdcpSQN_Type | PDCP sequence number |

SecurityActTime_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SecurityActTime_Type |  |  |
| Comment |  |  |  |
| RadioBearerId | RadioBearerId_Type |  |  |
| UL | PDCP_ActTime_Type |  |  |
| DL | PDCP_ActTime_Type |  |  |

SecurityActTimeList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SecurityActTimeList_Type |
| Comment |  |
| record length (1..tsc_MaxRB) of SecurityActTime_Type |  |

AS_IntegrityInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | AS_IntegrityInfo_Type |  |  |
| Comment | for initial configuration activation time is not needed for integrity protection as all messages in DL after security activation are integrity protected; this means this ASP is invoked before transmission of Security mode command; if there is a integrity violation in UL SS shall set the IndicationStatus in the common ASP part to flag the integrity error (IndicationStatus.Error.Integrity.Pdcp := true); integrity to be provided for each SRB as per core spec |  |  |
| Algorithm | IntegrityProtAlgorithm_Type |  | IntegrityProtAlgorithm_Type being defined in RRC ASN.1 |
| KRRCint | B128_Key_Type |  |  |
| KUPint | B128_Key_Type | opt | Only used for UPIP |
| ActTimeList | SecurityActTimeList_Type | opt | omit for initial configuration (i.e. all SRBs to be integrity protected immediately); in HO scenarios activation time may be needed e.g. for SRB1 |

AS_CipheringInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | AS_CipheringInfo_Type |  |  |
| Comment |  |  |  |
| Algorithm | CipheringAlgorithm_r12 |  | CipheringAlgorithm_Type being defined in RRC ASN.1 |
| KRRCenc | B128_Key_Type |  |  |
| KUPenc | B128_Key_Type |  | KUPenc is mandatory; and SS uses it when DRB are configured |
| ActTimeList | SecurityActTimeList_Type |  |  |

AS_SecStartRestart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | AS_SecStartRestart_Type |  |  |
| Comment |  |  |  |
| Integrity | AS_IntegrityInfo_Type | opt | optional to allow separated activation of integrity and ciphering; omit: keep as it is |
| Ciphering | AS_CipheringInfo_Type | opt | optional to allow separated activation of integrity and ciphering; omit: keep as it is |

AS_Security_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | AS_Security_Type |  |
| Comment | Security mode command procedure (TS 36.331, clause 5.3.4): both SMC and SMComp are integrity protected (nevertheless SS shall be able to cope with unprotected SM reject); ciphering is started just after SMComp (acc. to TS 36.331, clause 5.3.4.3 and 5.3.1.1) |  |
| StartRestart | AS_SecStartRestart_Type | information to start/restart AS security protection in the PDCP |
| Release | Null_Type | to release AS security protection in the PDCP |

## D.1.7	Semi_Persistent_Scheduling

Semi-persistent scheduling (SPS)
NOTE 1:
configuration of SPS cannot be done completely in advance but needs to be activated by PDCCH signalling
=> SPS is configured/activated in an own primitive which may be sent to SS during RBs are being configured
NOTE 2:
semi-persistent (configured) scheduling is per UE (as well as 'normal' scheduling; see e.g. TS 36.300, clause 11.1)

SpsAssignmentUL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SpsAssignmentUL_Type |  |  |
| Comment | information to assign semi-persistent scheduls in UL |  |  |
| DciInfo | DciUlInfo_Type | opt | to apply a grant |
| SchedulInterval | SpsConfigurationUL_Type | opt | as in TS 36.331, clause 6.3.2 SPS-ConfigUL |
| SetNDI_1 | Null_Type | opt | if present then NDI is set as 1 indicating a retransmission; If absent then NDI is set as 0 indicating a new transmission |

SpsAssignmentDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SpsAssignmentDL_Type |  |  |
| Comment | information to assign semi-persistent scheduls in DL |  |  |
| DciInfo | DciDlInfo_Type | opt | to apply a assignment |
| SchedulInterval | SpsConfigurationDL_Type | opt | as in TS 36.331, clause 6.3.2 SPS-ConfigDL |
| SetNDI_1 | Null_Type | opt | if present then NDI is set as 1 indicating a retransmission; If absent then NDI is set as 0 indicating a new transmission |

SpsActivateInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SpsActivateInfo_Type |  |  |
| Comment | Semi-persistent scheduling (SPS): Even though SPS is pre-configured at the UE (e.g. RRCConnectionSetup->RadioResourceConfiguration->MAC_MainConfig) it needs to be activated by L1 signalling => SS shall 'activate' SPS by sending appropriate assignments/grants to the UE; this shall be done with an activation time. If SPS is already configured and new Activate command is received, at the activation time SS locally deactivates old SPS configuration, sends UE an PDCCH assignment for new SPS assignment and locally activates new SPS configuration. In DL, in addition to SS SPS assignment configuration with activation time 'T', TTCN writer shall also schedule a DL MAC PDU with same activation time 'T' and at every SPS ScheduleInterval (NOTE: in general it is an error when TTCN does not provide data for a SchedulInterval; SS shall send no data in this case). Special fields of PDCCH assignment are filled as per table 9.2-1 of 36.213 |  |  |
| SPS_C_RNTI | C_RNTI |  | SPS C-RNTI as signalled to UE |
| UplinkGrant | SpsAssignmentUL_Type | opt |  |
| DownlinkAssignment | SpsAssignmentDL_Type | opt |  |

SpsPdcchRelease_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SpsPdcchRelease_Type |  |  |
| Comment | On reception of this information SS shall send an SPS release indicated by PDCCH transmission with indicated DCI format (0 or 1A) at the activation time. Special fields of PDCCH assignment are filled as per table 9.2-1A of 36.213 |  |  |
| SPS_C_RNTI | C_RNTI |  |  |
| DCI_Format | PdcchDciFormat_Type |  | only formats 0 (UL release) and 1A (DL release) are applicable. It is a TTCN error if any other formats are used. |

SpsDeactivateInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SpsDeactivateInfo_Type |  |
| Comment |  |  |
| LocalRelease | Null_Type | SPS configuration shall be released at the SS, that means as well that the SS shall not address SPS_C_RNTI anymore from the given TimingInfo onward; NOTE: there is no SPS release to be signalled on PDCCH (this is done with PdcchExplicitRelease - see below) |
| PdcchExplicitRelease | SpsPdcchRelease_Type | SS transmits PDCCH content indicating SPS release but holds the local SPS configuration until it is locally released |

SpsConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SpsConfig_Type |  |
| Comment |  |  |
| Activate | SpsActivateInfo_Type | CellId : identifier of the cell where the UE is active RoutingInfo : None TimingInfo : activation time for SPS assignment/grant transmission; NOTE: the first SPS DL data packet shall be sent with the same timing information ControlInfo : CnfFlag:=false; FollowOnFlag:=false |
| Deactivate | SpsDeactivateInfo_Type | CellId : identifier of the cell where the UE is active RoutingInfo : None TimingInfo : activation time for SPS release indicated by PDCCH transmission or SS local deactivation ControlInfo : CnfFlag:=false; FollowOnFlag:=false |

## D.1.8	Paging_Trigger

SubframeOffsetList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SubframeOffsetList_Type |
| Comment |  |
| record length (1..infinity) of integer |  |

PagingTrigger_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PagingTrigger_Type |  |  |
| Comment | CellId : identifier of the cell where the UE is active RoutingInfo : None TimingInfo : Calculated paging occasion ControlInfo : CnfFlag:=false; FollowOnFlag:=false primitive to trigger transmission of a paging on the PCCH at a calculated paging occasion (TS 36.304, clause 7); the paging occasion is calculated by TTCN and activation time is applied; as for BCCH Infor acc. to TS 36.331, clause 9.1.1.3 "RRC will perform padding, if required due to the granularity of the TF signalling, as defined in 8.5."; therefore this needs to be done by the system simulator |  |  |
| Paging | PCCH_Message |  | paging to be send out at paging occasion and being announced on PDCCH using P-RNTI |
| SubframeOffsetList | SubframeOffsetList_Type | opt | list of subframe offsets relative to the absolute timing information given in the common part of the ASP; if present, multiple pagings are sent out at all occasions given by the list; if omitted only a single paging is sent at the occasion given by the timing information in the common part of the ASP |

## D.1.9	L1_MAC_Indication_Control

Primitive for control of L1/MAC indication for special purposes

L1Mac_IndicationControl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | L1Mac_IndicationControl_Type |  |  |
| Comment | NOTE: Initially all indications are disabled in SS (i.e. it shall not be necessary in 'normal' test cases to use this primitive but only if a specific indication is needed); omit means indication mode is not changed |  |  |
| RachPreamble | IndicationAndControlMode_Type | opt | To enable/disable reporting of PRACH preamble received. |
| SchedReq | IndicationAndControlMode_Type | opt | To enable/disable reporting of reception of Scheduling Request on PUCCH. |
| BSR | IndicationAndControlMode_Type | opt | To enable/disable reporting of Buffer/Extended Buffer Status Report, Sidelink Buffer Status report or Truncated Sidelink Buffer Status Report. NOTE: this is applicable only when MAC is configured in normal mode in UL; MAC configured in test mode, results in over writing the report. |
| UL_HARQ | IndicationAndControlMode_Type | opt | To enable/disable reporting of reception of HARQ ACK/NACK. |
| C_RNTI | IndicationAndControlMode_Type | opt | To enable/disable reporting of C-RNTI sent by the UE within MAC PDU |
| PHR | IndicationAndControlMode_Type | opt | To enable/disable reporting of Power Headroom Report. NOTE: this is applicable only when MAC is configured in normal mode in UL; MAC configured in test mode, results in over writing the report. |
| HarqError | IndicationAndControlMode_Type | opt | To enable/disable reporting of HARQ errors |
| PeriodicRI | IndicationAndControlMode_Type | opt | To enable/disable reporting of reception of periodic Rank Indicators |
| EPHR | IndicationAndControlMode_Type | opt | To enable/disable reporting of Extended Power Headroom Report. NOTE: this is applicable only when MAC is configured in normal mode for UL; MAC configured in test mode, results in overwriting the report. |
| PeriodicCQI | IndicationAndControlMode_Type | opt | To enable/disable reporting of reception of periodic CQI |
| SRS | IndicationAndControlMode_Type | opt | To enable/disable reporting of start and stop of reception of Type 0 (Configured by Higher Layer) SRS in frames meant for periodic SRS transmission by UE. SS reports an indication every time when SRS reception from UE has toggled in the frames configured for periodic SRS reception i.e. 'No periodic SRS' reception -> 'periodic SRS' reception or 'periodic SRS' reception -> 'No periodic' SRS reception. |
| DC_PHR | IndicationAndControlMode_Type | opt | To enable/disable reporting of Dual Connectivity Power Headroom Report. NOTE: this is applicable only when MAC is configured in normal mode for UL; MAC configured in test mode, results in overwriting the report. |

## D.1.10	Rlc_Indication_Control

Primitive for control of RLC indication for special purposes

Rlc_IndicationMode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | Rlc_IndicationMode_Type |
| Comment |  |
| enable |  |
| disable |  |

Rlc_IndicationControl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Rlc_IndicationControl_Type |  |  |
| Comment |  |  |  |
| Discard | Rlc_IndicationMode_Type | opt | To enable/disable reporting of discarded RLC PDUs |

## D.1.11	PDCP_Count

Primitives to enquire PDCP COUNT

PdcpCountFormat_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | PdcpCountFormat_Type |
| Comment |  |
| PdcpCount_Srb | 27 bit HFN; 5 bit SQN |
| PdcpCount_DrbLongSQN | 20 bit HFN; 12 bit SQN |
| PdcpCount_DrbShortSQN | 25 bit HFN; 7 bit SQN |
| NrPdcpCount_Srb | 20 bit HFN; 12 bit SQN |
| NrPdcpCount_DrbSQN12 | 20 bit HFN; 12 bit SQN |
| NrPdcpCount_DrbSQN18 | 14 bit HFN; 18 bit SQN |

PdcpCount_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PdcpCount_Type |  |  |
| Comment |  |  |  |
| Format | PdcpCountFormat_Type |  |  |
| Value | PdcpCountValue_Type |  |  |

PdcpCountInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PdcpCountInfo_Type |  |  |
| Comment |  |  |  |
| RadioBearerId | RadioBearerId_Type |  |  |
| UL | PdcpCount_Type | opt | omit: keep as it is |
| DL | PdcpCount_Type | opt | omit: keep as it is |

PdcpCountInfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PdcpCountInfoList_Type |
| Comment |  |
| record length (1..tsc_MaxRB) of PdcpCountInfo_Type |  |

PdcpCountGetReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PdcpCountGetReq_Type |  |
| Comment |  |  |
| AllRBs | Null_Type | return COUNT values for all RBs being configured with either Pdcp.Config or Pdcp.NR_PDCP in the E-UTRA cell. |
| SingleRB | RadioBearerId_Type |  |

PDCP_CountReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_CountReq_Type |  |
| Comment |  |  |
| Get | PdcpCountGetReq_Type | Request PDCP count for one or all RBs being configured at the PDCP |
| Set | PdcpCountInfoList_Type | Set PDCP count for one or all RBs being configured at the PDCP; list for RBs which's COUNT shall be manipulated |

PDCP_CountCnf_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_CountCnf_Type |  |
| Comment |  |  |
| Get | PdcpCountInfoList_Type | RBs in ascending order; SRBs first |
| Set | Null_Type |  |

## D.1.12	PDCP_Handover

Primitives to control PDCP regarding handover

PDCP_HandoverInit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_HandoverInit_Type |  |  |
| Comment |  |  |  |
| SourceCellId | EUTRA_CellId_Type |  |  |

PDCP_HandoverControlReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_HandoverControlReq_Type |  |
| Comment |  |  |
| HandoverInit | PDCP_HandoverInit_Type | to inform SS that a handover will follow: in the common ASP part the CellId shall be set to the id of the target cell |
| HandoverComplete | Null_Type | to inform SS that the handover has successfully been performed by the UE; this shall trigger the SS to sent a PDCP Status Report to the UE; in the common ASP part the CellId shall be set to the id of the target cell |

## D.1.13	L1_MAC_Test_Mode

Primitive for control of L1/MAC Test Modes

L1_TestMode_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | L1_TestMode_Type |  |  |
| Comment | L1 test mode; in general RACH is handled separately |  |  |
| DL_SCH_CRC | DL_SCH_CRC_Type |  | Manipulation of CRC bit generation for DL-SCH |
| Phich | PhichTestMode_Type |  | HARQ feedback mode on the PHICH |

DL_SCH_CRC_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DL_SCH_CRC_Type |  |
| Comment | NOTE: CRC error mode for RA_RNTI is not addressed as it will be configured in RACHProcedureConfig |  |
| C_RNTI | MAC_Test_DL_SCH_CRC_Mode_Type | to configure mode for CRC bit for all MAC PDU's for which C-RNTI is used in PDCCH transmission |
| SI_RNTI | MAC_Test_DL_SCH_CRC_Mode_Type | to configure mode for CRC bit for all MAC PDU's for which SI-RNTI is used in PDCCH transmission |
| SPS_RNTI | MAC_Test_DL_SCH_CRC_Mode_Type | to configure mode for CRC bit for all MAC PDU's for which SPS-RNTI is used in PDCCH transmission |

## D.1.14	PDCCH_Order

Primitive to trigger SS to send PDCCH order to initiate RA procedure (TS 36.321, clause 5.1.1)

PDCCH_Order: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| PrachPreambleIndex_Type | Ra_PreambleIndex_Type |  |
| PrachMaskIndex_Type | integer (0..15) | TS 36.321, clause 7.3 |

RA_PDCCH_Order_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RA_PDCCH_Order_Type |  |  |
| Comment | see also TS 36.212, clause 5.3.3.1.3 |  |  |
| PreambleIndex | PrachPreambleIndex_Type |  | naming acc. TS 36.212, clause 5.3.3.1.3 |
| PrachMaskIndex | PrachMaskIndex_Type |  | naming acc. TS 36.212, clause 5.3.3.1.3 |

## D.1.15	System_Indications

Primitives for System indications

System_Indications: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| PRTPower_Type | Dummy_Type | needs to define appropriately the power level report of PREAMBLE_RECEIVED_TARGET_POWER; NOTE: for the time being this is just a place holder for enhancements in the future. |
| LogicalChannelGroup_Type | integer (0..3) |  |
| BSR_Value_Type | integer (0..63) |  |
| PHR_Type | integer (0..63) |  |
| RI_Type | integer (1..4) | Rank indicator reported acc. to TS 36.212 Table 5.2.2.6-6 |

HarqProcessInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HarqProcessInfo_Type |  |  |
| Comment |  |  |  |
| Id | HarqProcessId_Type |  |  |
| CURRENT_TX_NB | integer |  | acc. to TS 36.321 clause 5.4.2.2 |

HarqError_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | HarqError_Type |  |
| Comment |  |  |
| UL | HarqProcessInfo_Type | indicates HARQ error detected at the SS side (error at UL transmission) |
| DL | HarqProcessInfo_Type | indicates HARQ NACK sent by the UE (error at DL transmission) |

RachPreamble_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RachPreamble_Type |  |  |
| Comment |  |  |  |
| RAPID | PrachPreambleIndex_Type |  | indicates the RAPID of the preamble used (integer (0..63)) |
| PRTPower | PRTPower_Type |  | represents the PREAMBLE_RECEIVED_TARGET_POWER |
| RepetitionsPerPreambleAttempt | integer | opt | Applicable when numRepetitionPerPreambleAttempt-r13 is configured |

Short_BSR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Short_BSR_Type |  |  |
| Comment |  |  |  |
| LCG | LogicalChannelGroup_Type |  | Logical channel Group |
| Value | BSR_Value_Type |  | BSR or Extended BSR value |

Long_BSR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Long_BSR_Type |  |  |
| Comment |  |  |  |
| Value_LCG1 | BSR_Value_Type |  | BSR or Extended BSR value for LCG 1 |
| Value_LCG2 | BSR_Value_Type |  | BSR or Extended BSR value for LCG 2 |
| Value_LCG3 | BSR_Value_Type |  | BSR or Extended BSR value for LCG 3 |
| Value_LCG4 | BSR_Value_Type |  | BSR or Extended BSR value for LCG 4 |

BSR_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | BSR_Type |  |
| Comment |  |  |
| Short | Short_BSR_Type |  |
| Truncated | Short_BSR_Type |  |
| Long | Long_BSR_Type |  |
| Sidelink | SL_BSR_MACHeader_Type |  |

RlcDiscardInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RlcDiscardInd_Type |  |  |
| Comment | SS shall send this indication if it discards a received RLC AMD PDU as specified in TS 36.322 cl. 5.1.3.2.2. |  |  |
| SequenceNumber | integer |  | sequence number of the PDU being discarded |

SRSInd_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | SRSInd_Type |
| Comment |  |
| SRS_Start | SS was not receiving SRS in the frames configured for periodic SRS, and SS has started reception of higher layer configured SRS in frame meant for periodic SRS transmission |
| SRS_Stop | SS was receiving SRS in the frames configured for periodic SRS, and SS has stopped reception of higher layer configured SRS in frame meant for periodic SRS transmission |

SL_BSR_Value_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_BSR_Value_Type |  |  |
| Comment | According to TS 36.5321 clause 6.1.3.1a |  |  |
| DestinationIndex | B4_Type |  |  |
| LCG_ID | B2_Type |  |  |
| BufferSize | B6_Type |  |  |

SL_BSR_ValueList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SL_BSR_ValueList_Type |
| Comment |  |
| record of SL_BSR_Value_Type |  |

SL_BSR_MACHeader_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_BSR_MACHeader_Type |  |  |
| Comment |  |  |  |
| HeaderFieldLCID | B5_Type |  | Sidelink BSR LCID='10111' Truncated Sidelink BSR LCID='10110' |
| HeaderFieldE | B1_Type |  | E field in the MAC subheader |
| HeaderFieldF | B1_Type |  | F field in the MAC subheader |
| SL_BSR_Value | SL_BSR_ValueList_Type |  |  |

## D.1.16	System_Interface

SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SYSTEM_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : 'none' TimingInfo : depends on respective primitive (see below) ControlInfo : CnfFlag depends on use case; in general 'false' when for scheduled configuration, 'true' for 'now' FollowOnFlag 'false' RlcBearerRouting : omit |
| Request | SystemRequest_Type |  | - Cell TimingInfo: 'now' (in general) - CellAttenuationList TimingInfo: 'now' (in general, but activation time may be used also) - RadioBearerList TimingInfo: 'now' in general; activation time may be used in special case for release and/or reconfiguration of one or several RBs; the following rules shall be considered: - release/Reconfiguration of an RB shall not be scheduled ealier than 5ms after a previous data transmission on this RB - subsequent release and reconfiguration(s) shall be scheduled with an interval of at least 5ms - a subsequent data transmission on an RB shall not be scheduled ealier than 5ms after the last reconfiguration of the RB the configuration shall be performed exactly at the given time - EnquireTiming TimingInfo: 'now' - AS_Security TimingInfo: 'now'; NOTE: "activation time" may be specified in the primitive based on PDCP SQN - Sps TimingInfo: activation time for SPS assignment transmission - Paging TimingInfo: Calculated paging occasion - L1MacIndCtrl TimingInfo: 'now' (in general) - PdcpCount TimingInfo: 'now' (in general) activation time may be used in case of CA inter cell handover to set the PdcpCount - L1_TestMode TimingInfo: depends on the test mode; activation time is used e.g. for manipulation of the CRC - PdcchOrder TimingInfo: 'now' (in general) |

SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SYSTEM_CTRL_CNF |  |  |
| Comment |  |  |  |
| Common | CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN (apart from EnquireTiming) => SS may set TimingInfo to "None" |
| Confirm | SystemConfirm_Type |  |  |

SYSTEM_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SYSTEM_IND |  |  |
| Comment |  |  |  |
| Common | IndAspCommonPart_Type |  | CellId identifier of the cell RoutingInfo 'none' TimingInfo The SS shall provide TimingInfo (HSFN + SFN + subframe number) depending on the respective indication (see below) RlcBearerRouting not checked |
| Indication | SystemIndication_Type |  | - Error/HarqError TimingInfo: related to the error (if available) - RachPreamble TimingInfo: shall indicate start of the RACH preamble. In case of Preamble repetition (BL/CE UE under test), indicates the timing of the first Preamble transmission. - SchedReq TimingInfo: subframe containing the SR. In case of SR repetition (BL/CE UE under test), indicates the timing of the first SR transmission - BSR TimingInfo: subframe in which the MAC PDU contains the BSR - UL_HARQ/HARQ Error TimingInfo: subframe containing the UL HARQ. In case of HARQ repetition (BL/CE UE under test), indicates the timing of the first HARQ transmission - C_RNTI TimingInfo: subframe in which the MAC PDU contains the C_RNTI - PHR TimingInfo: subframe in which the MAC PDU contains the PHR - Periodic RI TimingInfo: subframe containing the RI. In case of RI repetition (BL/CE UE under test), indicates the timing of the first RI transmission |

EUTRA_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | EUTRA_SYSTEM_PORT |  |
| Comment | EUTRA PTC: Port for system configuration |  |
| out | SYSTEM_CTRL_REQ |  |
| in | SYSTEM_CTRL_CNF |  |

EUTRA_SYSIND_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | EUTRA_SYSIND_PORT |  |
| Comment | EUTRA PTC: Port for system indications |  |
| in | SYSTEM_IND |  |

## D.1.17	MBMS_Configuration

EUTRA_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_MaxMRB | integer | 32 | maximum MRB used in test cases - Value arbitrarily set to 32 (5bits) Theorically the maximum value is maxPMCH-PerMBSFN * maxSessionPerPMCH i.e. 15*29 |

MBMS_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MBMS_Config_Type |  |  |
| Comment | all fields are optional to allow single modifications; activation time is applied in the common part of the ASP |  |  |
| Mbsfn_SubframeConfigList | MBSFN_SubframeConfigList | opt | Configure subframes reserved for MBSFN |
| MbsfnAreaList | MbsfnAreaList_Type | opt | Configure all MBSFN areas |
| ScptmConfig | SCPTM_Config_Type | opt | If this IE is present, MBMS services are provided via SC-PTM |

MbsfnAreaList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MbsfnAreaList_Type |
| Comment |  |
| record length(1..maxMBSFN_Area) of MbsfnArea_Type |  |

MbsfnArea_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MbsfnArea_Type |  |  |
| Comment |  |  |  |
| Mbsfn_AreaInfo | MBSFN_AreaInfo_r9 | opt | Configure MCCH scheduling acc. to TS 36. 331 cl 9.1.1.4 there is no PDCP and MCCH use the RLC-UM mode configuration/scheduling and contents of the MCCH Information is done in one go (i.e. there are no separate ports for MCCH data and configuration) |
| McchData | MCCH_Data_Type | opt | MCCH information to be broadcasted |
| MbsfnAreaConfiguration | MBSFNAreaConfiguration_r9 | opt | Configuration parameters for CommonCSA/PMCH/MTCH |
| MRB_List | MRB_List_Type | opt | Configure/release MTCH MRBs |

MCCH_Data_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCCH_Data_Type |  |  |
| Comment |  |  |  |
| Mbsfn_AreaId | MBSFN_AreaId_r12 | opt | Used only for modification of MCCH info. Omitted if Mbsfn_AreaInfo is present and mandatory present otherwise |
| MsgList | MCCH_MessageList_Type |  |  |

MCCH_MessageList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MCCH_MessageList_Type |
| Comment | Includes mbsfnAreaConfiguration-r9 and optionally mbmsCountingRequest-r10. |
| record length(1..2) of MCCH_Message |  |

MRB_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MRB_List_Type |
| Comment |  |
| record length(1.. tsc_MaxMRB) of MRB_Type |  |

MRB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MRB_Type |  |  |
| Comment |  |  |  |
| PmchLogicalChannel | PmchLogicalChannel_Type |  |  |
| Config | MTCH_Config_Type | opt | present for MRB_COMMON_REQ for MTCH data scheduling; not present for SystemRequest_Type MSI_Conf |

MSI_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MSI_Config_Type |  |
| Comment |  |  |
| None | Null_Type | no MSI MCE included |
| Auto | MSI_MCE_Type | SS automatically includes configured MSI in the first subframes allocated to the MCH within the MCH scheduling period |

MSI_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MSI_Type |  |  |
| Comment | 36.321 clause 6.1.3.7 |  |  |
| LchID | integer (0..31) |  | MRB Logical Channel ID |
| StopMTCH | integer (0..2047) |  | Stop MTCH |

MSI_MCE_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MSI_MCE_Type |
| Comment |  |
| record of MSI_Type |  |

MTCH_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MTCH_Config_Type |  |
| Comment |  |  |
| AddOrReconfigure | MTCH_ConfigInfo_Type | Add/re-configure RB - CellId : identifier of the cell being configured RoutingInfo : none TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Release | Null_Type | release RB - CellId : identifier of the cell being configured RoutingInfo : none TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

MTCH_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MTCH_ConfigInfo_Type |  |  |
| Comment | Acc. to TS 36. 331 cl 9.1.1.4 there is no PDCP and MTCH use the RLC-UM mode |  |  |
| Rlc | RLC_Configuration_Type | opt | Mandatory for initial configuration; omit means "keep as it is" Note RLC DL only UM mode with SN 5 bits is only valid |
| Mac | MAC_MCH_TestModeConfig_Type | opt |  |

MAC_MCH_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MAC_MCH_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type | MAC operation in normal mode for MCH |
| Config | Null_Type | MAC is configured in no header manipulation in DL [MTCH is DL only channel] |

PDCCH_MCCH_ChangeNotification_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCCH_MCCH_ChangeNotification_Type |  |  |
| Comment |  |  |  |
| NotificationIndicator_r9 | NotificationIndicator_r9_Type | opt | MCCH change notification: shall be present, PDCCH shall be scrambled with M-RNTI SC-MCCH change notification: shall be omitted. PDCCH shall be scrambled with SC-N-RTNI |
| SubframeOffsetList | SubframeOffsetList_Type | opt | list of subframe offsets relative to the absolute timing information given in the common part of the ASP, this IE is MCCH-relevant and may be present only if NotificationIndicator_r9 is present; if present, multiple MCCH Change Notifications are sent out at all occasions given by the list; if omitted only a single MCCH Change Notifications is sent at the occasion given timing information given in the common part of the ASP |

PmchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PmchConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | ToRS_EPRE_Ratios_Type | opt | power ratio for MBSFN-RS and PMCH's resource elements relative to the CRS |

## D.1.18	SCPTM_Configuration

EUTRA_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_MaxSC_MRB | integer | 1023 | maximum SC-MRBs used in test cases arbitrarly set to 32 (5bits), but can set to max number of SC-MTCHs in one cell that is 1023(10bits) |

SCPTM_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SCPTM_Config_Type |  |  |
| Comment | all fields are optional to allow single modifications; if IE is set to "omit" - no change from previous configuration activation time is applied in the common part of the ASP |  |  |
| ScMcchScheduling | SC_MCCH_Scheduling_Type | opt | SC-MCCH scheduling configuration on PDCCH as per SystemInformationBlockType20 acc. to TS 36. 331 cl 9.1.1.4 there is no PDCP and SC-MCCH uses the RLC-UM mode configuration/scheduling and contents of the SC-MCCH Information is done in one go (i.e. there are no separate ports for SC-MCCH data and configuration) |
| ScptmConfiguration | SCPTMConfiguration_r13 | opt | Message containing SC-MTCH configuration to be broadcasted on SC-MCCH |
| ScMrbList | SC_MRB_List_Type | opt | Configure/release SC-MTCH SC-MRB |

SC_MCCH_Scheduling_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SC_MCCH_Scheduling_Type |  |  |
| Comment |  |  |  |
| ScMcchRepetitionPeriod | SC_MCCH_RepetitionPeriod_Type |  |  |
| ScMcchOffset | SC_MCCH_Offset_Type |  |  |
| ScMcchFirstSubframe | SC_MCCH_FirstSubframe_Type |  |  |
| ScMcchDuration | SC_MCCH_Duration_Type | opt |  |
| ScMcchModificationPeriod | SC_MCCH_ModificationPeriod_Type |  |  |

SC_MCCH_ModificationPeriod_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SC_MCCH_ModificationPeriod_Type |  |
| Comment |  |  |
| ScMcchModificationPeriod_r13 | SC_MCCH_ModificationPeriod_r13_Type |  |
| ScMcchModificationPeriod_v1470 | SC_MCCH_ModificationPeriod_v1470_Type |  |

SC_MCCH_RepetitionPeriod_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SC_MCCH_RepetitionPeriod_Type |  |
| Comment |  |  |
| ScMcchRepetitionPeriod_r13 | SC_MCCH_RepetitionPeriod_r13_Type |  |
| ScMcchRepetitionPeriod_v1470 | SC_MCCH_RepetitionPeriod_v1470_Type |  |

SC_MRB_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SC_MRB_List_Type |
| Comment |  |
| record length(1.. tsc_MaxSC_MRB) of SC_MRB_Type |  |

SC_MRB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SC_MRB_Type |  |  |
| Comment |  |  |  |
| ScMrbId | SC_MRB_Identity_Type |  |  |
| Config | SC_MTCH_Config_Type |  |  |

SC_MTCH_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SC_MTCH_Config_Type |  |  |
| Comment |  |  |  |
| GRnti | B16_Type |  |  |
| ScMtchConfig | MTCH_Config_Type |  |  |
| ScMtchSchedulingInfo | SC_MTCH_SchedulingInfo_r13 | opt |  |

## D.1.19	DirectIndicationInfo_Trigger

DirectIndicationInfoTrigger_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DirectIndicationInfoTrigger_Type |  |  |
| Comment | CellId : identifier of the cell where the UE is active RoutingInfo : None TimingInfo : Calculated paging occasion ControlInfo : CnfFlag:=false; FollowOnFlag:=false primitive to trigger transmission of a Direct Indication Information on MPDCCH at a calculated paging occasion (TS 36.304, clause 7) using DCI command 6_2 with Flag=0 and including the 8 bits in the DCI command (TS 36.212 clause 5.3.3.1.14); the paging occasion is calculated by TTCN and activation time is applied; |  |  |
| DirectIndicationInfo | B8_Type |  | Direct Indication Information |
| SubframeOffsetList | SubframeOffsetList_Type | opt | list of subframe offsets relative to the absolute timing information given in the common part of the ASP; if present, multiple direct indications are sent out at all occasions given by the list; if omitted only a single direct indication is sent at the occasion given timing information given in the common part of the ASP |

## D.1.20	Multiple_Uplink_Sps

MultipleUlSpsConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MultipleUlSpsConfig_Type |  |
| Comment |  |  |
| Activate | MultipleUlSpsActivateInfoList_Type | CellId : identifier of the cell where the UE is active RoutingInfo : None TimingInfo : activation time for SPS assignment/grant transmission ControlInfo : CnfFlag:=false; FollowOnFlag:=false |
| Deactivate | MultipleUlSpsDeactivateInfoList_Type | CellId : identifier of the cell where the UE is active RoutingInfo : None TimingInfo : activation time for SPS release indicated by PDCCH transmission or SS local deactivation ControlInfo : CnfFlag:=false; FollowOnFlag:=false |

MultipleUlSpsActivateInfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MultipleUlSpsActivateInfoList_Type |
| Comment |  |
| record of MultipleUlSpsActivateInfo_Type |  |

MultipleUlSpsActivateInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MultipleUlSpsActivateInfo_Type |  |  |
| Comment | UL Semi-persistent scheduling (SPS): Even though SPS is pre-configured at the UE (e.g. RRCConnectionSetup->RadioResourceConfiguration->sps-ConfigUL-ToAddModList-r14) it needs to be activated by L1 signalling => SS shall 'activate' SPS by sending appropriate assignments/grants with the appropriate UL SPS index to the UE; this shall be done with an activation time. If SPS with the same index is already configured and new Activate command is received, at the activation time SS locally deactivates old SPS configuration, sends UE an PDCCH assignment for new SPS assignment and locally activates new SPS configuration. Special fields of PDCCH assignment are filled as per table 9.2-1 of 36.213 |  |  |
| UlSpsConfigurationIndex | B3_Type |  | Indicates the SPS configuration indicated by the UL SPS configuration index field - included in DCI format 0 |
| UL_SPS_V_RNTI | C_RNTI |  | ul-SPS-V-RNTI as signalled to UE |
| UplinkGrant | SpsAssignmentUL_Type |  |  |

MultipleUlSpsDeactivateInfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MultipleUlSpsDeactivateInfoList_Type |
| Comment |  |
| record of MultipleUlSpsDeactivateInfo_Type |  |

MultipleUlSpsDeactivateInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MultipleUlSpsDeactivateInfo_Type |  |
| Comment | SS transmits PDCCH content indicating SPS release but holds the local SPS configuration until it is locally released |  |
| UlSpsConfigurationIndex | B3_Type | Indicates the SPS configuration indicated by the UL SPS configuration index field - included in DCI format 0 |
| UL_SPS_V_RNTI | C_RNTI | ul-SPS-V-RNTI as signalled to UE |
| DCI_Format | PdcchDciFormat_Type | only formats 0 (UL release) is applicable. It is a TTCN error if any other formats are used. |

## D.1.21	Licensed_Assisted_Access

LAA_SubframeConfiguration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | LAA_SubframeConfiguration_Type |  |  |
| Comment |  |  |  |
| SubframeConfig | B4_Type |  | as per 36.213 Table 13A-1 |
| ULDurationAndOffset | B5_Type | opt | only for UL LAA, as per 36.213 Table 13A-2 |
| PUSCH_TriggerB | B1_Type | opt | only for UL LAA |

# D.2	EUTRA_ASP_DrbDefs

ASP interface for DRBs

## D.2.1	MBMS_MRB_Primitive_Definitions

EUTRA_MRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | EUTRA_MRB_PORT |  |
| Comment |  |  |
| out | MRB_COMMON_REQ |  |

MRB_COMMON_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MRB_COMMON_REQ |  |  |
| Comment | common ASP to send PDUs to MRBs |  |  |
| Common | ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : set to Mrb TimingInfo : starting point when to start sending sequence of data PDUs e.g. SFN = X, subframe number = x; U_Plane.SubframeDataList[i].SubframeOffset := offset_i; => U_Plane.SubframeDataList[i].PduSduList shall be sent out at SFN = X + ((x + offset_i) / 10); subframe number = (x + offset_i) % 10 ControlInfo : CnfFlag:=false; FollowOnFlag:=false RlcBearerRouting : omit |
| U_Plane | L2Data_Request_Type |  |  |

## D.2.2	System_Interface

DRB_COMMON_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DRB_COMMON_REQ |  |  |
| Comment | common ASP to send PDUs to DRBs |  |  |
| Common | ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : DRB id TimingInfo : starting point when to start sending sequence of data PDUs e.g. SFN = X, subframe number = x; U_Plane.SubframeDataList[i].SubframeOffset := offset_i; => U_Plane.SubframeDataList[i].PduSduList shall be sent out at SFN = X + ((x + offset_i) / 10); subframe number = (x + offset_i) % 10 ControlInfo : CnfFlag:=false; FollowOnFlag:=false RlcBearerRouting : If omitted the SS shall assume the data to be routed to the same cell as indicated by CellId; when - e.g. in case of dual connectivity - a specific routing is provided by TTCN the SS shall route the data accordingly. NOTE: TTCN may provide a specific RCL bearer routing even when the data is routed to the same cell as given by CellId. |
| U_Plane | L2Data_Request_Type |  |  |
| SuppressPdcchForC_RNTI | Null_Type | opt | By default all DRB_COMMON_REQ scheduled DL PDU's are associated with an appropriate explicit configured or SS selected DL assignment allocation on PDCCH. For SuppressPdcch:=true in the sub frame in which DL PDU's are transmitted, there is no associated DL assignment allocation for configured C-RNTI. This will be used for SPS assignment based transmission or in any error scenarios; NOTE: this flag has no impact on PDCCH messages required for SPS activation |

DRB_COMMON_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DRB_COMMON_IND |  |  |
| Comment | common ASP to receive PDUs from DRBs |  |  |
| Common | IndAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : DRB id TimingInfo : time when message has been received RlcBearerRouting : For dual connectivity (e.g. ENDC), i.e. when there is more than one path for a given DRB or the RLC bearer is at a different cell as indicated by CellId, the SS shall provide id of the cell in which the data has been sent by the UE; otherwise the SS may provide the cell id but TTCN does not check it. NOTE 1: For MAC and RCL PDUs per definition L2Data_Indication_Type corresponse to exactly one subframe => TimingInfo refers to this subframe NOTE 2: For PDCP a single PDU or SDU may take more than one TTI => TimingInfo refers to the end of the PDU/SDU and the length is given by NoOfTTIs in L2Data_Indication_Type (the end of the PDU/SDU is the last RLC PDU being received; in case of retransmissins this is not necessarily the RLC PDU with the last SN) |
| U_Plane | L2Data_Indication_Type |  |  |

EUTRA_DRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | EUTRA_DRB_PORT |  |
| Comment |  |  |
| out | DRB_COMMON_REQ |  |
| in | DRB_COMMON_IND |  |

# D.3	EUTRA_NB_ASP_L2DataDefs

ASP interface for DRBs

## D.3.1	PDU_TypeDefs

### D.3.1.1	MAC_PDU

MAC_PDU: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| MAC_CTRL_C_RNTI_Type | C_RNTI | TS 36.321, clause 6.1.3.2 |
| MAC_CTRL_ContentionResolutionId_Type | ContentionResolutionId_Type | TS 36.321, clause 6.1.3.4 fix 48-bit size; consists of a single field defined UE Contention Resolution Identity (first 48 bits of the uplink CCCH SDU transmitted by MAC) |
| MAC_CTRL_TimingAdvance_Type | B8_Type | TS 36.321, clause 6.1.3.5 indicates the amount of timing adjustment in 0.5 ms that the UE has to apply; the length of the field is 8 bits |
| MAC_SDU_Type | octetstring |  |

MAC_PDU_Length_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_PDU_Length_Type |  |  |
| Comment | NOTE: since F and L field are either both present or both omitted they are put into this record; to allow homogeneous (direct) encoding the PDU length is not defined as union; TTCN-3 does allow length restrictions to one lenght or a range of length but not to two specific lengthes; further restriction may be achieved by appropriate templates (parameter either 7 or 15 bit) |  |  |
| Format | B1_Type |  | F: The Format field indicates the size of the Length field as indicated in table 6.2.1-3. There is one F field per MAC PDU subheader except for the last subheader and sub-headers corresponding to fixed-sized MAC control elements. The size of the F field is 1 bit. If the size of the MAC SDU or MAC control element is less than 128 bytes, the UE shall set the value of the F field to 0, otherwise the UE shall set it to 1 |
| Value | B7_15_Type |  | L: The Length field indicates the length of the corresponding MAC SDU or MAC control element in bytes. There is one L field per MAC PDU subheader except for the last subheader and sub-headers corresponding to fixed-sized MAC control elements. The size of the L field is indicated by the F field |

MAC_PDU_SubHeader_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_PDU_SubHeader_Type |  |  |
| Comment |  |  |  |
| Reserved | B2_Type |  | Reserved bits |
| Extension | B1_Type |  | E: The Extension field is a flag indicating if more fields are present in the MAC header or not. The E field is set to "1" to indicate another set of at least R/R/E/LCID fields. The E field is set to "0" to indicate that either a MAC SDU, a MAC control element or padding starts at the next byte |
| LCID | B5_Type |  | LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC control element or padding as described in tables 6.2.1-1 and 6.2.1-2 for the DL and UL-SCH respectively. There is one LCID field for each MAC SDU, MAC control element or padding included in the MAC PDU. The LCID field size is 5 bits; NOTE: In case of DRX command the sub-header corresponds to a control element of length zero (i.e. there is no control element) |
| Length | MAC_PDU_Length_Type | opt |  |

MAC_Header_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MAC_Header_Type |
| Comment |  |
| record of MAC_PDU_SubHeader_Type |  |

MAC_CTRL_ShortBSR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_ShortBSR_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.1 |  |  |
| LCG | B2_Type |  |  |
| Value | B6_Type |  |  |

MAC_CTRL_LongBSR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_LongBSR_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.1 |  |  |
| Value_LCG1 | B6_Type |  |  |
| Value_LCG2 | B6_Type |  |  |
| Value_LCG3 | B6_Type |  |  |
| Value_LCG4 | B6_Type |  |  |

MAC_CTRL_PowerHeadRoom_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_PowerHeadRoom_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.6 |  |  |
| Reserved | B2_Type |  |  |
| Value | B6_Type |  |  |

MAC_CTRL_ElementList_Type

| TTCN-3 Set Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_ElementList_Type |  |  |
| Comment | NOTE 1: for simplicication UL and DL are not distinguished even though the control elements are either UL or DL NOTE 2: type is defined as set: the ordering is not signifficant; nevertheless the ordering is well-defined by the sub-headers; for codec implementations it is in any case necessary to evaluate the sub-header information in order to encode/decode the payload |  |  |
| ShortBSR | MAC_CTRL_ShortBSR_Type | opt | UL only |
| LongBSR | MAC_CTRL_LongBSR_Type | opt | UL only |
| C_RNTI | MAC_CTRL_C_RNTI_Type | opt | UL only |
| ContentionResolutionID | MAC_CTRL_ContentionResolutionId_Type | opt | DL only |
| TimingAdvance | MAC_CTRL_TimingAdvance_Type | opt | DL only |
| PowerHeadRoom | MAC_CTRL_PowerHeadRoom_Type | opt | UL only |
| ScellActDeact | MAC_CTRL_ScellActDeact_Type | opt | DL only |
| ExtPowerHeadRoom | MAC_CTRL_ExtPowerHeadRoom_Type | opt | UL only. Only one among PowerHeadroom, ExtPowerHeadroom and DC_PowerHeadRoom may be present |
| DC_PowerHeadRoom | MAC_CTRL_DC_PowerHeadRoom_Type | opt | UL only. Only one among PowerHeadroom, ExtPowerHeadroom and DC_PowerHeadRoom may be present |
| TimingAdvanceReport | MAC_CTRL_TimingAdvanceReport_Type | opt | UL only |
| DifferentialKoffset | MAC_CTRL_DifferentialKoffset_Type | opt | DL only |

MAC_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MAC_SDUList_Type |
| Comment |  |
| record of MAC_SDU_Type |  |

MAC_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_PDU_Type |  |  |
| Comment |  |  |  |
| Header | MAC_Header_Type |  | list of MAC PDU SubHeaders corresponding to MAC control elements and MAC SDUs |
| CtrlElementList | MAC_CTRL_ElementList_Type | opt | Mac control elements; acc. to TS 36.321, clause 6.1.2 "MAC control elements, are always placed before any MAC SDU." |
| SduList | MAC_SDUList_Type | opt | MAC SDUs, which can typically be RLC PDUs |
| Padding | octetstring | opt | Octet aligned Padding if more than or equal to 2 bytes |

MAC_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MAC_PDUList_Type |
| Comment |  |
| record of MAC_PDU_Type |  |

### D.3.1.2	RLC_PDU

RLC_PDU: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_SDU_Type | octetstring |  |

RLC_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_PDU_Type |  |
| Comment |  |  |
| TMD | RLC_TMD_PDU_Type |  |
| UMD | RLC_UMD_PDU_Type |  |
| AMD | RLC_AMD_PDU_Type |  |
| AMD_Ext | RLC_AMD_PDU_Ext_Type |  |
| AMD_SegExt | RLC_AMD_PDU_SegExt_Type |  |
| Status | RLC_AM_StatusPDU_Type |  |
| Status_Ext | RLC_AM_StatusPDU_Ext_Type |  |

RLC_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_PDUList_Type |
| Comment |  |
| record of RLC_PDU_Type |  |

RLC_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_SDUList_Type |
| Comment |  |
| record of RLC_SDU_Type |  |

#### D.3.1.2.1	Common

RLC PDU definition: common AM/UM field definitions

Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_FramingInfo_Type | B2_Type | 00 - First byte of the Data field corresponds to the first byte of a RLC SDU. Last byte of the Data field corresponds to the last byte of a RLC SDU. 01 - First byte of the Data field corresponds to the first byte of a RLC SDU. Last byte of the Data field does not correspond to the last byte of a RLC SDU. 10 - First byte of the Data field does not correspond to the first byte of a RLC SDU. Last byte of the Data field corresponds to the last byte of a RLC SDU. 11 - First byte of the Data field does not correspond to the first byte of a RLC SDU. Last byte of the Data field does not correspond to the last byte of a RLC SDU. |

RLC_LengthIndicator_LI11_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_LengthIndicator_LI11_Type |  |  |
| Comment |  |  |  |
| Extension | B1_Type |  | 0 - Data field follows from the octet following the LI field following this E field 1 - A set of E field and LI field follows from the bit following the LI field following this E field |
| LengthIndicator | B11_Type |  | Length Indicator |

RLC_LengthIndicator_LI15_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_LengthIndicator_LI15_Type |  |  |
| Comment |  |  |  |
| Extension | B1_Type |  | 0 - Data field follows from the octet following the LI field following this E field 1 - A set of E field and LI field follows from the bit following the LI field following this E field |
| LengthIndicator | B15_Type |  | 15 bit Length Indicator |

RLC_LI11_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_LI11_List_Type |
| Comment |  |
| record of RLC_LengthIndicator_LI11_Type |  |

RLC_LI15_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_LI15_List_Type |
| Comment |  |
| record of RLC_LengthIndicator_LI15_Type |  |

RLC_LI_List_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_LI_List_Type |  |
| Comment |  |  |
| LI11 | RLC_LI11_List_Type |  |
| LI15 | RLC_LI15_List_Type |  |

RLC_PDU_Header_FlexPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_PDU_Header_FlexPart_Type |  |  |
| Comment | Flexible part of the header with a number of K LIs |  |  |
| LengthIndicator | RLC_LI_List_Type |  | List of E, LI fields |
| Padding | B4_Type | opt | optional 4 bit padding present in case of odd number of LI's |

#### D.3.1.2.2	TM_Data

RLC PDU definition: UM  (TS 36.322, clause 6.2.1.2)

TM_Data: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_TMD_PDU_Type | octetstring | TS 36.322, clause 6.2.1.2 |

#### D.3.1.2.3	UM_Data

RLC PDU definition: UM  (TS 36.322, clause 6.2.1.3)
NOTE:
To allow direct encoding the definition for RLC UM Data PDU is split into data PDU with 5/10 bit sequence number

UM_Data: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_DataField_Type | octetstring | restrictions imposed from LI size of 11 bits is not applicable when the LI's are not present |

RLC_UMD_Header_FixPartShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_Header_FixPartShortSN_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.3 Figure 6.2.1.3-1, 6.2.1.3-3 and 6.2.1.3-4); one octet |  |  |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bits FI |
| Extension | B1_Type |  | 1 bit E |
| SequenceNumber | B5_Type |  | 5 bits SN |

RLC_UMD_Header_FixPartLongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_Header_FixPartLongSN_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.3 Figure 6.2.1.3-2, 6.2.1.3-5 and 6.2.1.3-6); two octets |  |  |
| Reserved | B3_Type |  | 3 bits reserved |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bits FI |
| Extension | B1_Type |  | 1 bit E |
| SequenceNumber | B10_Type |  | 10 bits SN |

RLC_UMD_HeaderShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_HeaderShortSN_Type |  |  |
| Comment |  |  |  |
| FixPart | RLC_UMD_Header_FixPartShortSN_Type |  |  |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_UMD_HeaderLongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_HeaderLongSN_Type |  |  |
| Comment |  |  |  |
| FixPart | RLC_UMD_Header_FixPartLongSN_Type |  |  |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_DataFieldList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_DataFieldList_Type |
| Comment | One to one correspondence with sub headers (LengthIndicatorList_Type) |
| record of RLC_DataField_Type |  |

RLC_UMD_PDU_ShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_PDU_ShortSN_Type |  |  |
| Comment |  |  |  |
| Header | RLC_UMD_HeaderShortSN_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_UMD_PDU_LongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_PDU_LongSN_Type |  |  |
| Comment |  |  |  |
| Header | RLC_UMD_HeaderLongSN_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_UMD_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_UMD_PDU_Type |  |
| Comment |  |  |
| ShortSN | RLC_UMD_PDU_ShortSN_Type |  |
| LongSN | RLC_UMD_PDU_LongSN_Type |  |

#### D.3.1.2.4	AM_Data

RLC PDU definition: AM (TS 36.322, clause 6.2.1.4 and 6.2.1.5)

RLC_AMD_Header_FixPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_FixPart_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.4 Figure 6.2.1.4-1, 6.2.1.4-2 and 6.2.1.4-3); 2 or 4 octets (AMD PDU or AMD PDU segment) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| ReSeg | B1_Type |  | 0 - AMD PDU 1 - AMD PDU segment |
| Poll | B1_Type |  | 0 - Status report not requested 1 - Status report is requested |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bit FI |
| Extension | B1_Type |  | 1 bit E |
| SN | B10_Type |  | Sequence number |

RLC_AMD_Header_FixPartExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_FixPartExt_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.4 Figure 6.2.1.4-1a, 6.2.1.4-2a and 6.2.1.4-3a); 3 octets |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| ReSeg | B1_Type |  | 0 - AMD PDU 1 - AMD PDU segment |
| Poll | B1_Type |  | 0 - Status report not requested 1 - Status report is requested |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bit FI |
| Extension | B1_Type |  | 1 bit E |
| Reserved | B2_Type |  | 2 reserved bits |
| SN_Ext | B16_Type |  | Sequence number |

RLC_AMD_Header_FixPartSegExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_FixPartSegExt_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.5 Figure 6.2.1.5-1a); 5 octets |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| ReSeg | B1_Type |  | 0 - AMD PDU 1 - AMD PDU segment |
| Poll | B1_Type |  | 0 - Status report not requested 1 - Status report is requested |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bit FI |
| Extension | B1_Type |  | 1 bit E |
| LastSegmentFlag | B1_Type |  | 0 - Last byte of the AMD PDU segment does not correspond to the last byte of an AMD PDU 1 - Last byte of the AMD PDU segment corresponds to the last byte of an AMD PDU |
| Reserved | B1_Type |  | 1 reserved bit |
| SN_Ext | B16_Type |  | Sequence number |

RLC_AMD_Header_SegmentPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_SegmentPart_Type |  |  |
| Comment | AMD PDU segment related info in PDU header acc. TS 36.322, clause 6.2.1.5 |  |  |
| LastSegmentFlag | B1_Type |  | 0 - Last byte of the AMD PDU segment does not correspond to the last byte of an AMD PDU 1 - Last byte of the AMD PDU segment corresponds to the last byte of an AMD PDU |
| SegOffset | B15_Type |  | The SO field indicates the position of the AMD PDU segment in bytes within the original AMD PDU. Specifically, the SO field indicates the position within the Data field of the original AMD PDU to which the first byte of the Data field of the AMD PDU segment corresponds to. |

RLC_AMD_Header_SegmentPartExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_SegmentPartExt_Type |  |  |
| Comment | AMD PDU segment related info in PDU header acc. TS 36.322, clause 6.2.1.5 |  |  |
| SegOffset | B16_Type |  | The SO field indicates the position of the AMD PDU segment in bytes within the original AMD PDU. Specifically, the SO field indicates the position within the Data field of the original AMD PDU to which the first byte of the Data field of the AMD PDU segment corresponds to. |

RLC_AMD_Header_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_Type |  |  |
| Comment |  |  |  |
| FixPart | RLC_AMD_Header_FixPart_Type |  |  |
| SegmentPart | RLC_AMD_Header_SegmentPart_Type | opt | present in case of AMD Seg PDU only |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_AMD_HeaderExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_HeaderExt_Type |  |  |
| Comment |  |  |  |
| FixPartExt | RLC_AMD_Header_FixPartExt_Type |  |  |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_AMD_HeaderSegExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_HeaderSegExt_Type |  |  |
| Comment |  |  |  |
| FixPartSegExt | RLC_AMD_Header_FixPartSegExt_Type |  |  |
| SegmentPart | RLC_AMD_Header_SegmentPartExt_Type | opt | present in case of AMD Seg PDU only |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_AMD_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_PDU_Type |  |  |
| Comment |  |  |  |
| Header | RLC_AMD_Header_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_AMD_PDU_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_PDU_Ext_Type |  |  |
| Comment |  |  |  |
| HeaderExt | RLC_AMD_HeaderExt_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_AMD_PDU_SegExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_PDU_SegExt_Type |  |  |
| Comment |  |  |  |
| HeaderSegExt | RLC_AMD_HeaderSegExt_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

#### D.3.1.2.5	AM_Status

AM Status PDU (TS 36.322, clause 6.2.1.6)

AM_Status: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_Status_Padding_Type | bitstring length (1..7) | NOTE: in TTCN-3 length restriction cannot be done inline in record definition => explicit type definition necessary |

RLC_Status_ACK_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_ACK_Type |  |  |
| Comment |  |  |  |
| ACK_SN | B10_Type |  | Acknowledgement SN (TS 36.322, clause 6.2.2.14) |
| Extn1 | B1_Type |  | 0 - a set of NACK_SN, E1 and E2 does not follow. 1 - a set of NACK_SN, E1 and E2 follows. |

RLC_Status_ACK_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_ACK_Ext_Type |  |  |
| Comment |  |  |  |
| ACK_SN_Ext | B16_Type |  | Acknowledgement SN (TS 36.322, clause 6.2.2.14) |
| Extn1 | B1_Type |  | 0 - a set of NACK_SN, E1 and E2 does not follow. 1 - a set of NACK_SN, E1 and E2 follows. |

RLC_Status_SegOffset_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_SegOffset_Type |  |  |
| Comment |  |  |  |
| Start | B15_Type |  | SOstart field indicates the position of the first byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU |
| End | B15_Type |  | SOend field indicates the position of the last byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU. The special SOend value '111111111111111'B is used to indicate that the missing portion of the AMD PDU includes all bytes to the last byte of the AMD PDU |

RLC_Status_SegOffset_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_SegOffset_Ext_Type |  |  |
| Comment |  |  |  |
| Start | B16_Type |  | SOstart field indicates the position of the first byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU |
| End | B16_Type |  | SOend field indicates the position of the last byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU. The special SOend value '111111111111111'B is used to indicate that the missing portion of the AMD PDU includes all bytes to the last byte of the AMD PDU |

RLC_Status_NACK_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_NACK_Type |  |  |
| Comment |  |  |  |
| NACK_SN | B10_Type |  |  |
| Extn1 | B1_Type |  | 0 - A set of NACK_SN, E1 and E2 does not follow. 1 - A set of NACK_SN, E1 and E2 follows. |
| Extn2 | B1_Type |  | 0 - A set of SOstart and SOend does not follow for this NACK_SN. 1 - A set of SOstart and SOend follows for this NACK_SN. |
| SO | RLC_Status_SegOffset_Type | opt |  |

RLC_Status_NACK_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_Status_NACK_List_Type |
| Comment |  |
| record of RLC_Status_NACK_Type |  |

RLC_Status_NACK_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_NACK_Ext_Type |  |  |
| Comment |  |  |  |
| NACK_SN_Ext | B16_Type |  |  |
| Extn1 | B1_Type |  | 0 - A set of NACK_SN, E1 and E2 does not follow. 1 - A set of NACK_SN, E1 and E2 follows. |
| Extn2 | B1_Type |  | 0 - A set of SOstart and SOend does not follow for this NACK_SN. 1 - A set of SOstart and SOend follows for this NACK_SN. |
| SO | RLC_Status_SegOffset_Ext_Type | opt |  |

RLC_Status_NACK_Ext_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_Status_NACK_Ext_List_Type |
| Comment |  |
| record of RLC_Status_NACK_Ext_Type |  |

RLC_AM_StatusPDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AM_StatusPDU_Type |  |  |
| Comment | RLC STATUS PDU (TS 36.322, clause 6.2.1.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| CP_Type | B3_Type |  | 000 - STATUS PDU 001..111 - Reserved (=> PDU to be discarded by the receiving entity for this release of the protocol) |
| Ack | RLC_Status_ACK_Type |  | ACK_SN and E1 bit |
| NackList | RLC_Status_NACK_List_Type | opt | presence depends on Extn1 bit of Ack filed (RLC_Status_ACK_Type) |
| Padding | RLC_Status_Padding_Type | opt | 1..7 bit padding if needed for octet alignment |

RLC_AM_StatusPDU_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AM_StatusPDU_Ext_Type |  |  |
| Comment | RLC STATUS PDU (TS 36.322, clause 6.2.1.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| CP_Type | B3_Type |  | 000 - STATUS PDU 001..111 - Reserved (=> PDU to be discarded by the receiving entity for this release of the protocol) |
| Ack_Ext | RLC_Status_ACK_Ext_Type |  | ACK_SN and E1 bit |
| Nack_Ext_List | RLC_Status_NACK_Ext_List_Type | opt | presence depends on Extn1 bit of Ack field (RLC_Status_ACK_Type) |
| Padding | RLC_Status_Padding_Type | opt | 1..7 bit padding if needed for octet alignment |

### D.3.1.3	PDCP

PDCP user plane SDU and PDU definitions
NOTE:
To allow direct encoding the definition for PDCP Data PDU is split into data PDU with long/short sequence number

PDCP: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| PDCP_SDU_Type | octetstring |  |

PDCP_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PDCP_SDUList_Type |
| Comment |  |
| record of PDCP_SDU_Type |  |

PDCP_DataPdu_LongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_LongSN_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number (TS 36.323, clause 6.2.3) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| Reserved | B3_Type |  | 3 reserved bits |
| SequenceNumber | B12_Type |  | 12 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_ShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_ShortSN_Type |  |  |
| Comment | User plane PDCP Data PDU with short sequence number (TS 36.323, clause 6.2.4) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| SequenceNumber | B7_Type |  | 7 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_ExtSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_ExtSN_Type |  |  |
| Comment | User plane PDCP Data PDU with extended sequence number (TS 36.323, clause 6.2.9) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| SequenceNumber | B15_Type |  | 15 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_18bitSN_Type |  |  |
| Comment | User plane PDCP Data PDU with 18 bit sequence number (TS 36.323, clause 6.2.11) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PollingBit | B1_Type |  | 1 - indicates the UE is requested to send a PDCP status report or a LWA status report for LWA |
| Reserved | B4_Type |  | 4 reserved bits |
| SequenceNumber | B18_Type |  | 18 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_LongSN_UDC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_LongSN_UDC_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number for UDC (TS 36.323, clause 6.2.14) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| Reserved | B3_Type |  | 3 reserved bits |
| SequenceNumber | B12_Type |  | 12 bit sequence number |
| FU | B1_Type |  | 0 - Packet is not compressed using UDC protocol 1 - Packet is compressed using UDC protocol |
| FR | B1_Type |  | 0 - Compression buffer is not reset 1 - Compression buffer has been reset |
| Reserved2 | B2_Type |  | 2 reserved bits |
| Checksum | B4_Type |  | 4 validation bits for the compression buffer content |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_ExtSN_UDC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_ExtSN_UDC_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number for UDC (TS 36.323, clause 6.2.15) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| SequenceNumber | B15_Type |  | 15 bit sequence number |
| FU | B1_Type |  | 0 - Packet is not compressed using UDC protocol 1 - Packet is compressed using UDC protocol |
| FR | B1_Type |  | 0 - Compression buffer is not reset 1 - Compression buffer has been reset |
| Reserved2 | B2_Type |  | 2 reserved bits |
| Checksum | B4_Type |  | 4 validation bits for the compression buffer content |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_18bitSN_UDC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_18bitSN_UDC_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number for UDC (TS 36.323, clause 6.2.16) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PollingBit | B1_Type |  | 1 - indicates the UE is requested to send a PDCP status report or a LWA status report for LWA |
| Reserved | B4_Type |  | 4 reserved bits |
| SequenceNumber | B18_Type |  | 18 bit sequence number |
| FU | B1_Type |  | 0 - Packet is not compressed using UDC protocol 1 - Packet is compressed using UDC protocol |
| FR | B1_Type |  | 0 - Compression buffer is not reset 1 - Compression buffer has been reset |
| Reserved2 | B2_Type |  | 2 reserved bits |
| Checksum | B4_Type |  | 4 validation bits for the compression buffer content |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_SLRB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_SLRB_Type |  |  |
| Comment | User plane PDCP Data PDU for SLRB (TS 36.323, clause 6.2.10.1) |  |  |
| SDU_Type | B3_Type |  | 000 - IP 001 - ARP 010 - PC5 Signaling 011 - Non-IP 100 - 111 reserved |
| PGK_Index | B5_Type |  | 5 LSBs of PGK Identity (TS 33.303, clause 6.2.3) |
| PTK_Identity | B16_Type |  | PTK Identity (TS 33.303, clause 6.2.3) |
| SequenceNumber | B16_Type |  | 16 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_SLRB_1to1_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_SLRB_1to1_Type |  |  |
| Comment | User plane PDCP Data PDU for SLRB (TS 36.323, clause 6.2.10.2) |  |  |
| SDU_Type | B3_Type |  | 000 - IP 001 - ARP 010 - PC5 Signaling 011 - Non-IP 100 - 111 reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| KD_sess_ID | B16_Type |  | KD_sess Identity (TS 33.303, clause 6.5.3.1) |
| SequenceNumber | B16_Type |  | 16 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |
| MAC_I | O4_Type |  | MAC-I |

PDCP_Ctrl_ROHC_FB_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_ROHC_FB_PDU_Type |  |  |
| Comment | PDCP Control PDU for interspersed ROHC feedback packet (TS 36.323, clause 6.2.5) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B4_Type |  |  |
| ROHC_FB | octetstring |  | Contains one ROHC packet with only feedback, i.e. a ROHC packet that is not associated with a PDCP |

PDCP_Ctrl_StatusReport_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_StatusReport_Type |  |  |
| Comment | PDCP Control PDU for PDCP status report (TS 36.323, clause 6.2.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| FMS | B12_Type |  | PDCP SN of the first missing PDCP SDU. |
| Bitmap | octetstring | opt | The MSB of the first octet of the type "Bitmap" indicates whether or not the PDCP SDU with the SN (FMS + 1) modulo 4096 has been received and, optionally decompressed correctly. 0 - PDCP SDU with PDCP SN = (FMS + bit position) modulo 4096 is missing in the receiver. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. 1 - PDCP SDU with PDCP SN = (FMS + bit position) modulo 4096 does not need to be retransmitted. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. |

PDCP_Ctrl_StatusReportExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_StatusReportExt_Type |  |  |
| Comment | PDCP Control PDU for PDCP status report using a 15 bit SN (TS 36.323, clause 6.2.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| FMS_Ext | B15_Type |  | PDCP SN of the first missing PDCP SDU. |
| Bitmap | octetstring | opt | The MSB of the first octet of the type "Bitmap" indicates whether or not the PDCP SDU with the SN (FMS + 1) modulo (Maximum_PDCP_SN + 1) has been received and, optionally decompressed correctly. 0 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) is missing in the receiver. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. 1 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) does not need to be retransmitted. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. |

PDCP_Ctrl_StatusReport_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_StatusReport_18bitSN_Type |  |  |
| Comment | PDCP Control PDU for PDCP status report using a 18 bit SN (TS 36.323, clause 6.2.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B2_Type |  | 2 reserved bits |
| FMS_18bitSN | B18_Type |  | PDCP SN of the first missing PDCP SDU. |
| Bitmap | octetstring | opt | The MSB of the first octet of the type "Bitmap" indicates whether or not the PDCP SDU with the SN (FMS + 1) modulo (Maximum_PDCP_SN + 1) has been received and, optionally decompressed correctly. 0 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) is missing in the receiver. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. 1 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) does not need to be retransmitted. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. |

PDCP_Ctrl_LWA_StatusReport_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_StatusReport_Type |  |  |
| Comment | PDCP Control PDU for LWA status report (TS 36.323, clause 6.2.12) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| FMS | B12_Type |  | PDCP SN of the first missing PDCP SDU. |
| HRW | B12_Type |  | PDCP SN of the PDCP SDU received on WLAN with highest associated PDCP COUNT value. |
| NMP | B12_Type |  | Number of Missing PDCP PDUs with PDCP SNs below HRW starting from and including FMS. |

PDCP_Ctrl_LWA_StatusReportExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_StatusReportExt_Type |  |  |
| Comment | PDCP Control PDU for LWA status report (TS 36.323, clause 6.2.12) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| FMS_Ext | B15_Type |  | PDCP SN of the first missing PDCP SDU. |
| Reserved2 | B1_Type |  | 1 reserved bit |
| HRW_Ext | B15_Type |  | PDCP SN of the PDCP SDU received on WLAN with highest associated PDCP COUNT value. |
| Reserved3 | B1_Type |  | 1 reserved bit |
| NMP_Ext | B15_Type |  | Number of Missing PDCP PDUs with PDCP SNs below HRW starting from and including FMS. |

PDCP_Ctrl_LWA_StatusReport_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_StatusReport_18bitSN_Type |  |  |
| Comment | PDCP Control PDU for LWA status report (TS 36.323, clause 6.2.12) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B2_Type |  | 2 reserved bits |
| FMS_18bitSN | B18_Type |  | PDCP SN of the first missing PDCP SDU. |
| HRW_18bitSN | B18_Type |  | PDCP SN of the PDCP SDU received on WLAN with highest associated PDCP COUNT value. |
| Reserved2 | B4_Type |  | 4 reserved bits |
| NMP_18bitSN | B18_Type |  | Number of Missing PDCP PDUs with PDCP SNs below HRW starting from and including FMS. |

PDCP_Ctrl_LWA_EndMarker_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_EndMarker_Type |  |  |
| Comment | PDCP Control PDU for LWA end marker (TS 36.323, clause 6.2.13) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| LSN | B12_Type |  | PDCP SN of the last PDCP PDU for which the data part is ciphered with the key used before PDCP re-establishment. |

PDCP_Ctrl_LWA_EndMarkerExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_EndMarkerExt_Type |  |  |
| Comment | PDCP Control PDU for LWA end marker (TS 36.323, clause 6.2.13) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| LSN_Ext | B15_Type |  | PDCP SN of the last PDCP PDU for which the data part is ciphered with the key used before PDCP re-establishment. |

PDCP_Ctrl_LWA_EndMarker_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_EndMarker_18bitSN_Type |  |  |
| Comment | PDCP Control PDU for LWA end marker (TS 36.323, clause 6.2.13) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B2_Type |  | 2 reserved bits |
| LSN_18bitSN | B18_Type |  | PDCP SN of the last PDCP PDU for which the data part is ciphered with the key used before PDCP re-establishment. |

PDCP_Ctrl_UDC_FB_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_UDC_FB_PDU_Type |  |  |
| Comment | PDCP Control PDU for UDC feedback packet (TS 36.323, clause 6.2.17) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 100 - UDC feedback packet |
| FE | B1_Type |  | 0 - No Error 1 - Checksum Error Notification |
| Reserved | B3_Type |  |  |

PDCP_Ctrl_EHC_FB_PDU_ShortCID_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_EHC_FB_PDU_ShortCID_Type |  |  |
| Comment |  |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 101 - EHC feedback packet |
| Reserved | B4_Type |  | 4 bits reserved |
| Reserved2 | B1_Type |  | 1 bit reserved |
| CID | B7_Type |  | 7 bits CID |

PDCP_Ctrl_EHC_FB_PDU_LongCID_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_EHC_FB_PDU_LongCID_Type |  |  |
| Comment |  |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 101 - EHC feedback packet |
| Reserved | B4_Type |  | 4 bits reserved |
| Reserved2 | B1_Type |  | 1 bit reserved |
| CID | B15_Type |  | 15 bits CID |

PDCP_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_PDU_Type |  |
| Comment |  |  |
| DataLongSN | PDCP_DataPdu_LongSN_Type | user plane PDCP data PDU with 12 Bit Seq Number |
| DataShortSN | PDCP_DataPdu_ShortSN_Type | user plane PDCP data PDU with 7 Bit Seq Number |
| DataExtSN | PDCP_DataPdu_ExtSN_Type | user plane PDCP data PDU with 15 Bit Seq Number |
| Data_18bitSN | PDCP_DataPdu_18bitSN_Type | user plane PDCP data PDU with 18 Bit Seq Number |
| RohcFeedback | PDCP_Ctrl_ROHC_FB_PDU_Type | PDCP Control PDU for interspersed ROHC feedback packet |
| StatusReport | PDCP_Ctrl_StatusReport_Type | PDCP Control PDU for PDCP status report |
| StatusReportExt | PDCP_Ctrl_StatusReportExt_Type | PDCP Control PDU for PDCP status report using a 15 bit SN |
| StatusReport_18bitSN | PDCP_Ctrl_StatusReport_18bitSN_Type | PDCP Control PDU for PDCP status report using a 18 bit SN |
| LWA_StatusReport | PDCP_Ctrl_LWA_StatusReport_Type | PDCP Control PDU for LWA status report |
| LWA_StatusReportExt | PDCP_Ctrl_LWA_StatusReportExt_Type | PDCP Control PDU for LWA status report using a 15 bit SN |
| LWA_StatusReport_18bitSN | PDCP_Ctrl_LWA_StatusReport_18bitSN_Type | PDCP Control PDU for LWA status report using a 18 bit SN |
| DataSLRB | PDCP_DataPdu_SLRB_Type | user plane PDCP Data PDU for SLRB |
| DataSLRB_1to1 | PDCP_DataPdu_SLRB_1to1_Type | user plane PDCP Data PDU for SLRB one to one communication |
| LWA_EndMarker | PDCP_Ctrl_LWA_EndMarker_Type | PDCP Control PDU for LWA end marker |
| LWA_EndMarkerExt | PDCP_Ctrl_LWA_EndMarkerExt_Type | PDCP Control PDU for LWA end marker using a 15 bit SN |
| LWA_EndMarker_18bitSN | PDCP_Ctrl_LWA_EndMarker_18bitSN_Type | PDCP Control PDU for LWA end marker using a 18 bit SN |
| DataLongSN_UDC | PDCP_DataPdu_LongSN_UDC_Type | user plane PDCP data PDU with 12 Bit Seq Number for UDC |
| DataExtSN_UDC | PDCP_DataPdu_ExtSN_UDC_Type | user plane PDCP data PDU with 15 Bit Seq Number for UDC |
| Data18bitSN_UDC | PDCP_DataPdu_18bitSN_UDC_Type | user plane PDCP data PDU with 18 Bit Seq Number for UDC |
| UdcFeedback | PDCP_Ctrl_UDC_FB_PDU_Type | PDCP Control PDU for UDC feedback packet |
| EhcFeedback_ShortCID | PDCP_Ctrl_EHC_FB_PDU_ShortCID_Type | PDCP Control PDU for EHC feedback packet Short CID |
| EhcFeedback_LongCID | PDCP_Ctrl_EHC_FB_PDU_LongCID_Type | PDCP Control PDU for EHC feedback packet Long CID |

PDCP_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PDCP_PDUList_Type |
| Comment |  |
| record of PDCP_PDU_Type |  |

## D.3.2	DRB_Primitive_Definitions

Primitive definitions to send/receive data PDUs over DRB's

### D.3.2.1	DRB_Common

L2DataList_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | L2DataList_Type |  |
| Comment | MAC: acc. to rel-8 protocols there is not more than one MAC PDU per TTI; any MAC PDU is completely included in one subframe RLC: one or more RLC PDUs per TTI (e.g. RLC Data + Status PDU on a logical channel; more than one RLC Data PDU in one MAC PDU is valid too) any RLC PDU is completely included in one subframe PDCP: one or more PDUs per TTI; one PDCP PDU may be included in more than one subframe |  |
| MacPdu | MAC_PDUList_Type | SS configuration: RLC TM mode, MAC no header removal (PDCP is not configured) |
| RlcPdu | RLC_PDUList_Type | SS configuration: RLC TM mode, MAC header removal (PDCP is not configured) |
| PdcpPdu | PDCP_PDUList_Type | SS configuration: RLC AM/UM mode, PDCP no header removal |
| PdcpSdu | PDCP_SDUList_Type | SS configuration: RLC AM/UM mode, PDCP header removal |
| NrPdcpSdu | PDCP_SDUList_Type | SS configuration: RLC AM/UM mode, PDCP header removal |
| RlcSdu | RLC_SDUList_Type | SS configuration: RLC UM mode with no PDCP, for example MRB |
| SdapPdu | SDAP_PDUList_Type | SS configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header), no handling of SDAP header |
| SdapSdu | SDAP_SDUList_Type | SS configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header), automatic handling of SDAP header |

HarqProcessAssignment_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | HarqProcessAssignment_Type |  |
| Comment | in DL the HARQ process id may be specified by the test case or automatically assigned by SS |  |
| Id | HarqProcessId_Type | HARQ process as specified by the test case NOTE1: the scope of this type is only for data being sent in one TTI; if data needs more than one TTI the HarqProcessId is undefined for the 2nd TTI onward what shall be handled as an error at the SS; SS may send a SYSTEM_IND indicating an error in this case; NOTE2: The initial value of the NDI shall be the same for all HARQ processes and cells |
| Automatic | Null_Type | HARQ process id automatically assigned by SS |

### D.3.2.2	Downlink

DRB_DataPerSubframe_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DRB_DataPerSubframe_DL_Type |  |  |
| Comment | common definition for one or several PDUs/SDUs; in EUTRA the DL data is sent in the subframe given by the subframe offset; in NBIOT the DL transmission is scheduled in the search space candidate given by "SubframeOffset" for NBIOT it specifies the NOTE 1: For MAC and RLC PDUs a single PDU is always sent in one subframe; SS shall raise an error indication (using SYSTEM_IND) when that is not possible NOTE 2: For PDCP the data may be spread over more than one subframe (segmented by the RLC); the TTCN implemetation is responsible to calculate appropriate offsets accordingly; the exact timing depends on (and is exactly specified by) configuration of the DL scheduling; SS shall raise an error when there is any conflict |  |  |
| SubframeOffset | integer |  | EUTRA: Subframe offset relative to the absolute timing information given in the common part of the ASP; NBIOT: Offset of the search space candidate relative to the (default) search space candidate as configured for the NPDCCH UE specific search space; the search space candidate is in the next search space starting at or after the point in time given by the timing information of the common part of the ASP NOTE 1: Notes: Acc. to TS 36.523-3, clause 7.3.3 in case of TDD or half-duplex configuration only subframes available for DL are taken into consideration NOTE 2: if a PDCP PDU or SDU takes more than one subframe, SubframeOffset specifies the first TTI |
| HarqProcess | HarqProcessAssignment_Type | opt | HARQ process to be used: specific value (0..7) or automatically assigned by SS; in automatic mode SS chooses HARQ process out of the set configured by CcchDcchDtchConfigDL_Type.HarqProcessConfig NOTE 1: for PDCP SDUs or PDUs automatic mode shall be used; otherwise SS shall raise an error NOTE 2: for NB-IoT the HarqProcess shall be omitted by TTCN and ignored by the SS |
| PduSduList | L2DataList_Type |  | list of PDUs/SDUs to be sent in one TTI |

DRB_DataPerSubframeList_DL_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | DRB_DataPerSubframeList_DL_Type |
| Comment | list of user plane data to be sent in sub-frames given by the SubframeOffset in the single elements of the list; Timing: the start time for the whole sequence is given by the timing info of the ASP (common information); the timing for the respective data pdus is given by the SubframeOffset relative to the common timing info; design consideration: repetitions of this sequence are not foreseen (in which case the subframe offset could not be related to the timing info of the ASP) |
| record of DRB_DataPerSubframe_DL_Type |  |

L2Data_Request_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | L2Data_Request_Type |  |  |
| Comment | NOTE: formal type definition to allow later enhancements; L2Data_Request_Type defines a sequence of subframes in which data shall be sent |  |  |
| SubframeDataList | DRB_DataPerSubframeList_DL_Type |  |  |

### D.3.2.3	Uplink

DRB_DataPerSubframe_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DRB_DataPerSubframe_UL_Type |  |  |
| Comment | common definition for one or several PDUs/SDUs being received in one subframe or to receive one PDCP PDU or SDU being spread over more than one TTI; NOTE: There is a fix relation between HARQ process id and subframe in UL => it is not necessary to include HARQ process id for UL data |  |  |
| PduSduList | L2DataList_Type |  | list of PDUs/SDUs being received in one TTI; elements of the list appear in the same order as the PDUs/SDUs in the MAC PDU; for PDCP when a PDU or SDU takes more than one TTI the list only contains this PDU or SDU |
| NoOfTTIs | integer |  | in case of PDCP: number of TTIs the SDU or PDU has taken NOTE 1: for the time being the NoOfTTIs is not checked by TTCN-3 and may be set to 1 by SS; NOTE 2: the timing info in common part of the ASP refers to the last TTI NOTE 3: when NoOfTTIs > 1 => PduSduList shall only contain one PDCP PDU or SDU in case of MAC or RLC PDUs: NoOfTTIs shall always be 1 (acc. to TS 36.321 MAC is not doing segmentation of RLC PDUs and acc. to TS 36.322, clause 6.2.2.2 the maximum RLC data is calculated to fit into a MAC PDU and RLC does segmentation accordingly) |

L2Data_Indication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | L2Data_Indication_Type |  |  |
| Comment | NOTE: formal type definition to allow later enhancements; L2Data_Indication_Type defines data being received in a single subframe i.e. PDUs of subsequent TTIs are indicated in separated ASPs |  |  |
| SubframeData | DRB_DataPerSubframe_UL_Type |  |  |

# D.4	EUTRA_ASP_SrbDefs

## D.4.1	SRB_DATA_ASPs

ASP Definitions to send/receive peer-to-peer messages on SRBs

C_Plane_Request_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | C_Plane_Request_Type |  |  |
| Comment | RRC and/or NAS PDU to be send to the UE; Note: it may be necessary to allow more than one NAS PDU (-> "record of") |  |  |
| Rrc | RRC_MSG_Request_Type | opt | omit: NAS message shall be present; NAS message shall be sent in DLInformationTransfer present: if NAS message is present also, (piggybacked) NAS PDU shall be security protected (if necessary) and inserted in RRC PDU's DedicatedInfoNAS |
| Nas | NAS_MSG_RequestList_Type | opt | omit: RRC message shall be present; RRC message does not contain (piggybacked) NAS PDU present: if RRC message is omitted => NAS message shall be sent embedded in DLInformationTransfer if RRC message is present => NAS message is piggybacked in RRC message in case of RRC message is sent on CCCH, NAS message shall be omitted NOTE: acc. DEC 08 ASN.1 RRCConnectionReconfiguration may contain DedicatedInfoNAS several times |

C_Plane_Indication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | C_Plane_Indication_Type |  |  |
| Comment | RRC and/or NAS PDU to be received from the UE; Note: it may be necessary to allow more than one NAS PDU (-> "record of") |  |  |
| Rrc | RRC_MSG_Indication_Type | opt | omit: NAS message shall be present; NAS message is received in ULInformationTransfer present: if NAS message is present also, DedicatedInfoNAS contains unstructured and ciphered NAS message and the NAS message is the deciphered message in structured format |
| Nas | NAS_MSG_IndicationList_Type | opt | omit: RRC message shall be present; RRC message does not contain (piggybacked) NAS PDU present: if RRC message is omitted => NAS message has been received in ULInformationTransfer if RRC message is present => NAS message has been piggybacked in RRC message NOTE: even though currently (DEC 08 ASN.1) there is no RRC PDU in UL containing more than one DedicatedInfoNAS we provide a list to allow extendability |

SRB_COMMON_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SRB_COMMON_REQ |  |  |
| Comment | common ASP to send PDUs to SRB0, SRB1 or SRB2 |  |  |
| Common | ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB0, SRB1, SRB2 TimingInfo : Now in normal cases; For latency tests TimingInfo can be set to the SFN/subframe in which the RRC messages shall be sent out (in this case and if the RRC PDU is too long to be sent in one TTI the TimingInfo corresponds to the first TTI) ControlInfo CnfFlag:=false; FollowOnFlag true: Indicates that the message(s) to be sent on the same TTI will follow NOTE 1: When FollowOnFlag is true, TimingInfo shall always be "Now". Otherwise SS shall produce an error NOTE 2: the follow on flag applies only for messages of the same SRB false: Indicates that no more message(s) will follow RlcBearerRouting : If omitted the SS shall assume the data to be routed to the same cell as indicated by CellId; when - e.g. in case of split SRB - a specific routing is provided by TTCN the SS shall route the data accordingly. |
| Signalling | C_Plane_Request_Type |  |  |

SRB_COMMON_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SRB_COMMON_IND |  |  |
| Comment | common ASP to receive PDUs from SRB0, SRB1 or SRB2 |  |  |
| Common | IndAspCommonPart_Type |  | CellId identifier of the cell RoutingInfo SRB0, SRB1, SRB2 TimingInfo time when message has been received (as received from the SS by the NAS emulator) RlcBearerRouting : In case of split SRB the SS shall provide the id of the cell in which the data has been sent from the UE; otherwise the SS may provide the cell id but it is not checked by TTCN |
| Signalling | C_Plane_Indication_Type |  |  |

## D.4.2	Port_Definitions

EUTRA_SRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | EUTRA_SRB_PORT |  |
| Comment | EUTRA PTC: Port for Sending/Receiving data on SRBs |  |
| out | SRB_COMMON_REQ |  |
| in | SRB_COMMON_IND |  |

NASEMU_EUTRA_SRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NASEMU_EUTRA_SRB_PORT |  |
| Comment | NASEMU PTC: Port for Sending/Receiving data on SRBs (interface to EUTRA PTC) |  |
| out | SRB_COMMON_IND |  |
| in | SRB_COMMON_REQ |  |

# D.5	IP_ASP_TypeDefs

General Notes:
NOTE 1:
In general the handling of IP data shall be independent from the RAT being used on lower layers.
NOTE 2:
It shall be possible for SS implementation to reuse existing IP stack implementations in the system adaptor;
therefore the well-known concept of socket programming shall be supported
(regardless of whether those are used in the system adaptor implementation or not)
NOTE 3:
Since in general at the network side there are several different IP addresses the SS needs to simulate more than one IP address;
that can be based on a concept of multiple virtual network adaptors
NOTE 4:
There is no easy way to control the routing of IP data for an IP connection from above the IP stack
i.e. there are no parameters at the socket interface to determine e.g. cell id and DRB id
=> another independent logical entity (DRB-MUX) is needed below the IP stack which is responsible to control the routing of IP packets from/to DRBs in different cells of different RATs

Reference:
An introduction to socket programming can be found in
UNIX Network Programming Volume 1, Third Edition: The Sockets Networking API
by W. Richard Stevens, Bill Fenner, Andrew M. Rudoff

## D.5.1	IP_Common

IP_Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| PortNumber_Type | UInt16_Type |  |

IPv4_AddrInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPv4_AddrInfo_Type |  |  |
| Comment | IPv4 specific info of the socket addr (AF_INET) |  |  |
| Addr | charstring |  | IP Address as string (IP v4 dot notation) to be converted to 32-bit unsigned integer |

IPv6_AddrInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPv6_AddrInfo_Type |  |  |
| Comment | IPv6 specific info of the socket addr (AF_INET6); NOTE: sin6_flowinfo can be ignored and set to 0 |  |  |
| Addr | charstring |  | to be converted to sin6_addr |
| ScopeId | UInt32_Type | opt | sin6_scope_id in general an IPv6 address is like "fe80::1%eth0" with eth0 being the network adaptor mapped to a scope id (Unix) assumption: for UE conformance testing it is not necessary to distinguish different scopes and the scope id in general can be determined by the system adaptor => omit |

IP_AddrInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_AddrInfo_Type |  |
| Comment |  |  |
| V4 | IPv4_AddrInfo_Type |  |
| V6 | IPv6_AddrInfo_Type |  |

IP_Socket_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_Socket_Type |  |  |
| Comment | Socket |  |  |
| IpAddr | IP_AddrInfo_Type | opt | IP address |
| Port | PortNumber_Type | opt | port number |

InternetProtocol_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | InternetProtocol_Type |
| Comment |  |
| udp |  |
| tcp |  |
| icmp |  |
| icmpv6 |  |

IP_Connection_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_Connection_Type |  |  |
| Comment | A connection between peer-to-peer entities is unambiguously defined by the protocol (udp/tcp/icmp/icmpv4), the local socket and the remote socket |  |  |
| Protocol | InternetProtocol_Type |  |  |
| Local | IP_Socket_Type | opt |  |
| Remote | IP_Socket_Type | opt |  |

## D.5.2	IP_Config

Configuration of the routing table managed be the system adaptor's DRB-MUX:
foreach IP connection it is specified which
- RAT
- Cell
- DRB
to be used.
The IP connection does not need to be fully specified depending on the role SS plays (e.g. in case of a server role the port number of the remote side is not known in advance).
The configurations of DRBs within the same cell shall be mutual exclusive.

With the configuration of the IP routing the DRB is configured either in IP or in raw mode:
either there are entries for the DRB in the routing table (IP mode) or not (raw mode)
=> It is not necessary to reconfigure this for the respective RAT.

Behaviour of the DRB-MUX in UL:
- SS gets data packet from the lower layers (e.g. PDCP SDU)
- SS checks whether there is any IP connection configured for this DRB (identified by {RAT, CellId, DrbId})
  if YES => packet is routed to the IP stack (IP mode)
  if NO  => packet is handed over to the DRB port (raw mode)
NOTE 1:
If there is any entry for a DRB in the routing table this DRB is considered as being in IP mode and all UL IP packets are sent to the IP stack regardless of whether their addresses match the DRB's routing entries or not (in general 'unknown' packets are discarded by the IP stack)
=> a DRB can be either in IP or in raw mode
NOTE 2:
=> The SS does not need to evaluate any IP headers to decide whether data shall be routed to the DRB port or to the IP stack (i.e. there is no conflict with unstructured loopback data)

Behaviour of the DRB-MUX in DL:
- SS gets IP packets from the IP stack for an IP connection
- SS compares the IP connection (protocol, local/remote IP Addr) against the IP routing table and
  checks whether the corresponding protocol stack is configured at the lower layers  =>
  1. no match:
     no entry in the routing table fits to the address in the IP packet
     or the corresponding RB is not configured
     => SS shall raise an error (DRBMUX_COMMON_IND_CNF.Error)
  2. one match:
     There is exactly one possibility to route the IP packet
     => SS shall send the packet to this RB
  3. several matches:
     There are more than one DRBs, cells or RATs to which the packet may be routed
     => SS shall raise an error if there is more than one DRB in one cell matching;
        if the DRBs belong to different cells or RATs SS shall send the data to all of them
        (whether this may occur in test cases is FFS)

General notes:
NOTE 1:
SS may use the information of the routing table to determine which network adaptors it needs to simulate (implementation dependent);
in general there will be more than one IP address at the network side.
NOTE 2:
In general the routing table is a simplified DL TFT implementation
NOTE 3:
When the routing table is empty all DRBs are in raw mode; this shall be the initial condition at the DRB-MUX;
=> for L2 testing in general (and apart from the preamble) there is no need to use/configure the IP_PTC; the configuration of the RAT specific U-plane stacks is not affected

IP_DataMode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IP_DataMode_Type |
| Comment |  |
| discard |  |
| loopbackRTP |  |
| loopbackRTCP |  |
| IPsecTunnel |  |

IP_RoutingInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_RoutingInfo_Type |  |  |
| Comment |  |  |  |
| IpInfo | IP_Connection_Type |  | IP connection tuple: protocol, local socket, remote socket depending on the role the SS plays the following information may be provided (informative; even less information can be sufficient): 1. TCP/UDP server - local IP addr -- provided - local port -- provided - remote IP addr -- omit - remote port -- omit 2. TCP/UDP client - local IP addr -- provided (to inform SS about the local IP addr for this service) - local port -- omit; for UDP a well-defined port may be defined (protocol dependent, e.g. DHCP) - remote IP addr -- provided - remote port -- provided 3. ICMP (in general ICMP may be mapped only to a single DRB) - local IP addr -- provided (to inform SS about the local IP addr for this service) - local port -- n/a (shall be set to omit) - remote IP addr -- omit - remote port -- n/a (shall be set to omit) NOTE: In case of broadcasts in UL the broadcast address shall match any local IP address; in DL for broadcast services typically no remote IP address is specified in the routing table |
| DRB | IP_DrbInfo_Type |  |  |
| DataMode | IP_DataMode_Type | opt | present when IP packets matching this entry shall be discarded or be looped back to the UE as defined for RTP or RTCP |

IP_RoutingTable_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | IP_RoutingTable_Type |
| Comment | NOTE: configurations of DRBs within the same cell shall be mutual exclusive |
| record of IP_RoutingInfo_Type |  |

## D.5.3	IPsec_Config

IP_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_IPsec_SPI_Max | integer | 4294967295 |  |

IPsec_Config: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| IPsec_SPI_Type | integer (0..tsc_IPsec_SPI_Max) | security parameter index for IPsec; According to RFC 2406, SPI values from 0 to 255 are reserved |

IPsec_IntegrityAlgorithm_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IPsec_IntegrityAlgorithm_Type |
| Comment |  |
| hmac_md5_96 |  |
| hmac_sha_1_96 |  |

IPsec_CipheringAlgorithm_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IPsec_CipheringAlgorithm_Type |
| Comment |  |
| des_ede3_cbc |  |
| aes_cbc |  |
| nociph | no ciphering |

IPsec_SecurityKeys_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPsec_SecurityKeys_Type |  |  |
| Comment | to install the security keys |  |  |
| MD5_96Key | bitstring length (128) |  |  |
| SHA_1_96Key | bitstring length (160) |  |  |
| DES_EDE3_CBCKey | bitstring length (192) |  |  |
| AES_CBCKey | bitstring length (128) |  |  |

IPsec_SecurityAssociation_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPsec_SecurityAssociation_Type |  |  |
| Comment | single security association (SA); for configuration of an SA at the SS all fields are mandatory; to release an SA the optional information is omitted |  |  |
| SPI | IPsec_SPI_Type |  |  |
| SrcAddress | charstring |  |  |
| DestAddress | charstring |  |  |
| SrcPort | UInt16_Type |  |  |
| DestPort | UInt16_Type |  |  |
| IntegrityAlgorithm | IPsec_IntegrityAlgorithm_Type | opt | mandatory to set-up an SA |
| CipheringAlgorithm | IPsec_CipheringAlgorithm_Type | opt | mandatory to set-up an SA |

IPsec_SecurityAssociationList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | IPsec_SecurityAssociationList_Type |
| Comment |  |
| record of IPsec_SecurityAssociation_Type |  |

IPsec_Configure_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPsec_Configure_Type |  |  |
| Comment | add new security associations; existing SAs are not affected |  |  |
| SA_List | IPsec_SecurityAssociationList_Type |  |  |
| SecurityKeys | IPsec_SecurityKeys_Type |  |  |

IPsec_Release_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPsec_Release_Type |  |  |
| Comment | release security associations; NOTE: in context with multiple PDNs it cannot be ensured that all SPIs are unique; e.g. the UE may use the same SPI values in different PDNs in which case uniqueness cannot be achieved furthermore it depends on the system implementation how entries in the IPsec SAD and SPD are administrated => to release SAs the SS gets the same information as for configuration but without the security algorithms |  |  |
| SA_List | IPsec_SecurityAssociationList_Type |  |  |

## D.5.4	IP_SocketHandling

Handling of IP data and IP connections
NOTE 1:
In general IP connections are distinguished by the tuple {protocol, local socket, remote socket};
this information is used at the interface between TTCN and the system adaptor.
It is up the system adaptor implementation to associate the IP connection with the internal socket (file descriptor; implementation dependent)
NOTE 2:
In general the association of the IP connections to (internal) sockets and the routing table for the DRB mapping (as configured with IP_RoutingTable_Type) are independent from each other

### D.5.4.1	Socket_Common

IP_SockOpt_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_SockOpt_Type |  |
| Comment | socket options acc. to the setsockopt system call (i.e. for level=SOL_SOCKET in case of Berkeley socket API); NOTE: only options being relevant for a specific applications (upon a socket) are configured by TTCN other options (e.g. SO_REUSEADDR) are out of TTCN and therefore a matter of system adaptor implementation |  |
| SO_BROADCAST | boolean | set to true when IP broadcast messages shall be allowed for a port; this is required e.g. in case of DHCP |
| IP_MTU_SIZE | integer | MTU size to be used for IP data; NOTEs: - Even though the MTU size is defined as socket option it shall be the same for all sockets of a given interface (i.e. at least within one PDN the MTU size shall be the same) - in general a PIXIT is used as constant value for all sockets |

IP_SockOptList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | IP_SockOptList_Type |
| Comment |  |
| record of IP_SockOpt_Type |  |

IP_SocketError_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_SocketError_Type |  |
| Comment | used to indicate errors related to sockets; the IP_Connection shall contain as much address information as available at the system adaptor |  |
| InvalidAddress | Null_Type | TTCN error: e.g. invalid or incomplete address information |
| System | integer | system error caused by system call; the integer value may be used for validation but shall not be evaluated by TTCN |

### D.5.4.2	Socket_Datagram

Socket_Datagram: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| Datagram_Content_Type | octetstring | data as sent/received with sendto()/recvfrom() on UDP or ICMP socket; NOTE: For ICMP the data may depend on the socket options; in general it does not include the IP header and the checksum of the ICMP packet needs to be calculated/checked in TTCN |

Datagram_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Datagram_DL_Type |  |  |
| Comment | datagram to be sent at a UDP or ICMP socket |  |  |
| Buffer | Datagram_Content_Type |  | content of the IP packet |

Datagram_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Datagram_UL_Type |  |  |
| Comment | datagram as received on a UDP or ICMP socket |  |  |
| Buffer | Datagram_Content_Type |  | content of the IP packet |
| DrbInfo | IP_DrbInfo_Type | opt | "interface id" where the data comes from in case of broadcast or multicast packets: for the LTE test model this is the DRB on which the IP packet has been received; the information is necessary when the SS cannot resolve an IP address being assigned to that DRB. => when the SS provides a broadcast or multicast address as local address in the ConnectionId of the ASP, the SS shall provide the DRB information in this field When the ConnectionId of the ASP is fully specified and unique (unicast address at least for local address) the DrbId is ignored by TTCN |

### D.5.4.3	TCP_Socket

TCP primitives used on the IP port

TCP_Socket: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| TCP_Data_Type | octetstring | data as sent/received with send()/recv() on a TCP socket |

InternetApplication_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | InternetApplication_Type |
| Comment | as TCP is stream oriented SS may need information about which criteria to be applied to get start/end of an application message |
| ims |  |
| http |  |
| msrp |  |

TLS_CIPHER_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | TLS_CIPHER_Type |
| Comment | Cipher suite to be used for TLS |
| TLS_PSK_WITH_RC4_128_SHA | RFC 4279 |
| TLS_PSK_WITH_3DES_EDE_CBC_SHA | RFC 4279 |
| TLS_PSK_WITH_AES_128_CBC_SHA | RFC 4279 |
| TLS_PSK_WITH_AES_256_CBC_SHA | RFC 4279 |
| TLS_PSK_WITH_AES_128_CBC_SHA256 | RFC 5487 |
| TLS_RSA_WITH_NULL_MD5 | RFC 5246 |
| TLS_RSA_WITH_NULL_SHA | RFC 5246 |
| TLS_RSA_WITH_NULL_SHA256 | RFC 5246 |
| TLS_RSA_WITH_RC4_128_MD5 | RFC 5246 |
| TLS_RSA_WITH_RC4_128_SHA | RFC 5246 |
| TLS_RSA_WITH_3DES_EDE_CBC_SHA | RFC 5246 |
| TLS_RSA_WITH_AES_128_CBC_SHA | RFC 5246 |
| TLS_RSA_WITH_AES_256_CBC_SHA | RFC 5246 |
| TLS_RSA_WITH_AES_128_CBC_SHA256 | RFC 5246 |
| TLS_RSA_WITH_AES_256_CBC_SHA256 | RFC 5246 |

PSK_BootstrappingInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PSK_BootstrappingInfo_Type |  |  |
| Comment | bootstrapping information as defined in 24.109 |  |  |
| BTid | charstring |  |  |
| Ks_NAF | bitstring |  |  |

TLS_PSK_Info_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TLS_PSK_Info_Type |  |  |
| Comment | configuration information for PSK TLS |  |  |
| IdentityHint | charstring |  |  |
| BootstrappingInfo | PSK_BootstrappingInfo_Type |  |  |

TLS_CipherSuiteInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TLS_CipherSuiteInfo_Type |  |
| Comment |  |  |
| psk | TLS_PSK_Info_Type |  |

TLSPSKInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TLSPSKInfo_Type |  |  |
| Comment |  |  |  |
| cipherSuite | TLS_CIPHER_Type |  | Cipher suite to be used |
| cipherSuiteInfo | TLS_CipherSuiteInfo_Type |  | parameters for the respective cipher suite |

TLSCertificateInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TLSCertificateInfo_Type |  |  |
| Comment |  |  |  |
| certificate | charstring |  | name or file location of the certificate to be used by the SS; in general provided by a PIXIT: the PIXIT contains the SS implementation specific information for the SS to identify the certificate to be used for a particular TLS connection; in case of only one certificate being used by TTCN, the string may be empty |
| cipherSuite | TLS_CIPHER_Type |  | (non-PSK) cipher suite to be used for the TLS tunnel; the SS shall raise an error when it detects that the UE does not support the configured cipher suite |

TLSConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TLSConfig_Type |  |
| Comment |  |  |
| pskInfo | TLSPSKInfo_Type | Used in the case of PSK |
| certificateInfo | TLSCertificateInfo_Type | Used in the case of certificate based TLS |

TCP_ConnectRequest_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TCP_ConnectRequest_Type |  |  |
| Comment | TCP client: -> 'connect' system call |  |  |
| SockOptList | IP_SockOptList_Type |  | when there are no options to configure the list is empty |
| Application | InternetApplication_Type |  | to specify start/end criteria for application messages |

TCP_Listen_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TCP_Listen_Type |  |  |
| Comment | TCP server: -> 'listen' system call |  |  |
| SockOptList | IP_SockOptList_Type |  | when there are no options to configure the list is empty |
| Application | InternetApplication_Type |  | to specify start/end criteria for application messages |
| TLSConfig | TLSConfig_Type | opt | to support TLS for HTTP server implementation |

TCP_CtrlRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TCP_CtrlRequest_Type |  |
| Comment |  |  |
| ConnectReq | TCP_ConnectRequest_Type | request a 'connect' to a remote server system calls (informative) socket -- get file descriptor (setsockopt) -- normally not needed bind -- assign local IP addr (to cope with multiple IP addresses) and dedicated port number (if local port is given) connect -- connect to the client IP_Connection: protocol -- tcp local IP addr -- mandatory to distinguish different network adaptors local port -- omit (ephemeral port will be assigned by the system) or specific port to be used for this connection (e.g. to bind a given port number to the IMS client) remote IP addr -- mandatory remote port -- mandatory |
| Listen | TCP_Listen_Type | establish a server at the local (SS) side system calls (informative) socket -- get file descriptor (setsockopt) -- if needed bind -- assign local IP addr and port listen -- await incoming connection IP_Connection: protocol -- tcp local IP addr -- mandatory to distinguish different network adaptors local port -- mandatory remote IP add -- omit remote port -- omit |
| Close | Null_Type | close a connection system calls (informative): close IP_Connection: protocol -- tcp local IP addr -- mandatory local port -- mandatory remote IP addr -- mandatory for TCP connections, omit for TCP server remote port -- mandatory for TCP connections, omit for TCP server |

TCP_DataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TCP_DataRequest_Type |  |
| Comment |  |  |
| Send | TCP_Data_Type | send data system calls (informative): send or write IP_Connection: protocol -- tcp local IP addr -- mandatory local port -- mandatory remote IP addr -- mandatory remote port -- mandatory |

TCP_CtrlIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TCP_CtrlIndication_Type |  |
| Comment |  |  |
| ConnectCnf | Null_Type | confirm a 'connect' to a remote server system calls (informative): getsockname -- get local port (ephemeral port assigned by the system) IP_Connection: protocol -- tcp local IP addr -- mandatory (as in corresponding TCP_ConnectRequest) local port -- mandatory (if there is more than one connection to the same server the local port is necessary to distinguish the connections) remote IP addr -- mandatory (as in corresponding TCP_ConnectRequest) remote port -- mandatory (as in corresponding TCP_ConnectRequest) |
| Accept | Null_Type | sent by the SS when it 'accepts' an incoming connection system calls (informative): accept IP_Connection: protocol -- tcp local IP addr -- mandatory (as in corresponding TCP_ListenRequest) local port -- mandatory (as in corresponding TCP_ListenRequest) remote IP addr -- mandatory (as gotten from 'accept') remote port -- mandatory (as gotten from 'accept') |
| Close | Null_Type | indicate 'close' by the remote side system calls (informative): indicated by recv or read IP_Connection: protocol -- tcp local IP addr -- mandatory local port -- mandatory remote IP addr -- mandatory remote port -- mandatory |
| CloseCnf | Null_Type | Confirmation for 'close' request; necessary since for TCP there are IP packets to release the connection system calls (informative): close IP_Connection: protocol -- tcp local IP addr -- mandatory local port -- mandatory remote IP addr -- mandatory for TCP connections, omit for TCP server remote port -- mandatory for TCP connections, omit for TCP server |

TCP_DataIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TCP_DataIndication_Type |  |
| Comment |  |  |
| Recv | TCP_Data_Type | receive data system calls (informative): recv or read IP_Connection: protocol -- tcp local IP addr -- mandatory local port -- mandatory remote IP addr -- mandatory remote port -- mandatory |

### D.5.4.4	UDP_Socket

UDP primitives used on the IP port
NOTE:
In principle a UDP socket may communicate with different remote entities;
therefore the system adaptor may associate the socket handle with the local socket only
(local IP address and local port)

UDP_SocketReq_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UDP_SocketReq_Type |  |  |
| Comment | to establish a UDP server or to bind local port number |  |  |
| SockOptList | IP_SockOptList_Type |  | e.g. to allow broadcast messages; when there are no options to configure the list is empty |

UDP_CtrlRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UDP_CtrlRequest_Type |  |
| Comment |  |  |
| SocketReq | UDP_SocketReq_Type | request the system adaptor to bind a socket to a local address; this is needed in general when the system adaptor acts as 1. UDP server 2. UDP client when it uses a well-known port rather than an ephemeral port (this is e.g. for DHCP) 3. UDP client when a local address needs to be bond (e.g. when there are several local addresses) system calls (informative): socket -- get file descriptor (setsockopt) -- needed e.g. to allow broad cast message bind -- assign local IP address (to cope with multiple IP addresses) and local port (in case of well-known local port) IP_Connection: protocol -- udp local IP addr -- mandatory (to distinguish multiple IP addresses) local port -- optional (mandatory in case of a UDP server) remote IP addr -- omit remote port -- omit |
| Close | Null_Type | release local socket system calls (informative): close IP_Connection: protocol -- udp local IP addr -- mandatory (to identify local socket) local port -- mandatory (to identify local socket) remote IP addr -- omit remote port -- omit |

UDP_DataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UDP_DataRequest_Type |  |
| Comment |  |  |
| SendTo | Datagram_DL_Type | send data to (any) remote socket; NOTE: To simplify implementation of the system adaptor the local socket shall be bond in any case (using 'SocketReq') to specify the local IP address before sending data; (in general the sendto system call can be used without explicitly binding the socket before; in this case the port gets implicitly bond to an ephemeral port and the default IP address is used) system calls (informative): sendto IP_Connection: protocol -- udp local IP addr -- mandatory (to identify local socket) local port -- mandatory (to identify local socket) remote IP addr -- mandatory (to address remote socket) remote port -- mandatory (to address remote socket) |

UDP_CtrlIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UDP_CtrlIndication_Type |  |
| Comment |  |  |
| SocketCnf | Null_Type | confirm 'SocketReq' and tell TTCN about assignment of ephemeral port; system calls (informative): getsockname -- get local port (ephemeral port assigned by the system; not needed if local port is well-known) IP_Connection: protocol -- udp local IP addr -- mandatory local port -- mandatory (well-known or ephemeral port assigned by the system) remote IP addr -- omit remote port -- omit |

UDP_DataIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UDP_DataIndication_Type |  |
| Comment |  |  |
| RecvFrom | Datagram_UL_Type | receive data; system calls (informative): recvfrom -- get data and src addr IP_Connection: protocol -- udp local IP addr -- mandatory (see note) local port -- mandatory remote IP addr -- mandatory (as gotten from recvfrom) remote port -- mandatory (as gotten from recvfrom) NOTE: The UE may send a UDP packet as broadcast (IP Addr 255.255.255.255 - e.g. in case of DHCP) or multicast (e.g. ICMPv6) SS shall consider a broadcast address as matching every IP for UL and DL; the SS shall not replace the broadcast/multicast address by the local unicast address, but shall provide DRB information in RecvFrom; example: - SS gets DHCPDISCOVER with DEST_Addr=255.255.255.255 DEST_Port=67, SRC_Addr=0.0.0.0 SRC_Port=68 - TTCN gets DHCPDISCOVER with local Addr=(255.255.255.255 Port=67), remote Addr=(0.0.0.0 Port=68), DrbId=(LTE, cell1, DRB1) - TTCN sends DHCPOFFER with local Addr=(local IP Addr Port=67), remote Addr=(255.255.255.255 Port=68) |

### D.5.4.5	ICMP_Socket

ICMP primitives used on the IP port
NOTE:
the local side is identified by the protocol and in general by the local IP address

ICMP_SocketReq_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ICMP_SocketReq_Type |  |  |
| Comment | to establish a raw socket to send/receive ICMP packets |  |  |
| SockOptList | IP_SockOptList_Type |  | e.g. to set the IP_HDRINCL socket option (to include the IP header in the data buffer) when there are no options to configure the list is empty |

ICMP_CtrlRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ICMP_CtrlRequest_Type |  |
| Comment |  |  |
| SocketReq | ICMP_SocketReq_Type | request the system adaptor to open a raw socket (IPv4 or IPv6) system calls (informative): socket -- get file descriptor (IPPROTO_ICMP or IPPROTO_IPV6); (setsockopt) -- optional; to set socket options bind -- assign local IP address (to cope with multiple IP addresses) IP_Connection: protocol -- icmp or icmpv6 local IP addr -- mandatory (to distinguish multiple IP addresses) local port -- omit (not applicable for ICMP) remote IP addr -- omit remote port -- omit (not applicable for ICMP) |
| Close | Null_Type | release local socket system calls (informative): close IP_Connection: protocol -- icmp or icmpv6 local IP addr -- mandatory (to identify local socket) local port -- omit remote IP addr -- omit remote port -- omit |

ICMP_DataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ICMP_DataRequest_Type |  |
| Comment |  |  |
| SendTo | Datagram_DL_Type | send datagram system calls (informative): sendto IP_Connection: protocol -- icmp or icmpv6 local IP addr -- mandatory (to identify local socket) local port -- omit remote IP addr -- mandatory remote port -- omit |

ICMP_CtrlIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ICMP_CtrlIndication_Type |  |
| Comment |  |  |
| SocketCnf | Null_Type | confirm 'SocketReq' system calls (informative): (SocketCnf is sent when all system calls for SocketReq have been successful) IP_Connection: protocol -- icmp or icmpv6 local IP addr -- mandatory local port -- omit remote IP addr -- omit remote port -- omit |

ICMP_DataIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ICMP_DataIndication_Type |  |
| Comment |  |  |
| RecvFrom | Datagram_UL_Type | receive datagram system calls (informative): recvfrom -- get data and src addr IP_Connection: protocol -- icmp or icmpv6 local IP addr -- mandatory (see note) local port -- omit remote IP addr -- mandatory (as gotten from recvfrom) remote port -- omit NOTE: As for UDP there may be multicast/broadcast packets. In this case - as for UDP - the SS shall provide the DRB information in RecvFrom. |

### D.5.4.6	Socket_Primitives

IP_CtrlRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_CtrlRequest_Type |  |
| Comment |  |  |
| TCP | TCP_CtrlRequest_Type |  |
| UDP | UDP_CtrlRequest_Type |  |
| ICMP | ICMP_CtrlRequest_Type |  |

IP_DataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_DataRequest_Type |  |
| Comment |  |  |
| TCP | TCP_DataRequest_Type |  |
| UDP | UDP_DataRequest_Type |  |
| ICMP | ICMP_DataRequest_Type |  |

IP_CtrlIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_CtrlIndication_Type |  |
| Comment |  |  |
| TCP | TCP_CtrlIndication_Type |  |
| UDP | UDP_CtrlIndication_Type |  |
| ICMP | ICMP_CtrlIndication_Type |  |
| Error | IP_SocketError_Type |  |

IP_DataIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_DataIndication_Type |  |
| Comment |  |  |
| TCP | TCP_DataIndication_Type |  |
| UDP | UDP_DataIndication_Type |  |
| ICMP | ICMP_DataIndication_Type |  |

## D.5.5	System_Interface

DRBMUX_CONFIG_REQ

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DRBMUX_CONFIG_REQ |  |
| Comment | NOTE 1: There is just one primitive to configure the whole routing table. It is not foreseen to add, remove or manipulate single entries but the table is managed in TTCN and completely configured on any change; (otherwise it might get complicated to identify single entries) NOTE 2: the SS's routing table shall be empty at the beginning and can be cleared by an empty record (DRBMUX_CONFIG_REQ.RoutingInfo = {}) NOTE 3: In general a reconfiguration of the routing table during a test case would be necessary only if an ephemeral port is needed to distinguish different routing (e.g. when there are several TCP connections of the same service routed to different DRBs) |  |
| RoutingInfo | IP_RoutingTable_Type |  |

DRBMUX_COMMON_IND_CNF

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DRBMUX_COMMON_IND_CNF |  |
| Comment |  |  |
| Confirm | Null_Type | confirm DRBMUX_CONFIG_REQ |
| Error | Null_Type | indication of errors at the DRB-MUX: An Error shall be raised by the DRB-MUX e.g. in the following cases: - in DL when there are IP packets which cannot be routed to any DRB i.e. the IP packet does not match to any entry in the routing table or the corresponding RB is not configured - in DL when there are several DRBs possible for routing in the same cell |

IPSEC_CONFIG_REQ

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IPSEC_CONFIG_REQ |  |
| Comment |  |  |
| Configure | IPsec_Configure_Type |  |
| Release | IPsec_Release_Type |  |

IPSEC_CONFIG_CNF

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IPSEC_CONFIG_CNF |  |
| Comment |  |  |
| Confirm | Null_Type | confirm IPSEC_CONFIG_REQ |
| Error | Null_Type | to indicate invalid configuration of IPsec |

IP_SOCKET_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_SOCKET_CTRL_REQ |  |  |
| Comment |  |  |  |
| ConnectionId | IP_Connection_Type |  |  |
| Req | IP_CtrlRequest_Type |  |  |

IP_SOCKET_DATA_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_SOCKET_DATA_REQ |  |  |
| Comment |  |  |  |
| ConnectionId | IP_Connection_Type |  |  |
| Req | IP_DataRequest_Type |  |  |

IP_SOCKET_CTRL_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_SOCKET_CTRL_IND |  |  |
| Comment |  |  |  |
| ConnectionId | IP_Connection_Type | opt | shall be present at the system interface; maybe omit only intenally in inter-PTC coordination |
| Ind | IP_CtrlIndication_Type | opt | shall be present at the system interface; maybe omit only intenally in inter-PTC coordination |

IP_SOCKET_DATA_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_SOCKET_DATA_IND |  |  |
| Comment |  |  |  |
| ConnectionId | IP_Connection_Type |  |  |
| Ind | IP_DataIndication_Type |  |  |

IP_SOCKET_REQ

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_SOCKET_REQ |  |
| Comment |  |  |
| CTRL | IP_SOCKET_CTRL_REQ |  |
| DATA | IP_SOCKET_DATA_REQ |  |

IP_SOCKET_IND

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_SOCKET_IND |  |
| Comment |  |  |
| CTRL | IP_SOCKET_CTRL_IND |  |
| DATA | IP_SOCKET_DATA_IND |  |

IP_CONTROL_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | IP_CONTROL_PORT |  |
| Comment |  |  |
| out | DRBMUX_CONFIG_REQ |  |
| in | DRBMUX_COMMON_IND_CNF |  |

IPSEC_CONTROL_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | IPSEC_CONTROL_PORT |  |
| Comment |  |  |
| out | IPSEC_CONFIG_REQ |  |
| in | IPSEC_CONFIG_CNF |  |

IP_SOCKET_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | IP_SOCKET_PORT |  |
| Comment |  |  |
| out | IP_SOCKET_REQ |  |
| in | IP_SOCKET_IND |  |

# D.6	NasEmu_AspTypes_EUTRA

System interface between NAS emulation and system adaptor

## D.6.1	System_Interface

EUTRA_RRC_PDU_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | EUTRA_RRC_PDU_REQ |  |  |
| Comment |  |  |  |
| Common | ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB0, SRB1, SRB2 TimingInfo : Now in normal cases; For latency tests TimingInfo can be set to the SFN/subframe in which the RRC messages shall be sent out NOTE 1: if the RRC PDU is too long to be sent in one TTI the TimingInfo corresponds to the first TTI NOTE 2: the TimingInfo is not changed by the NAS Emu (i.e. the timing info as coming from the test case (SRB_COMMON_REQ) is handed through by the NAS Emu) ControlInfo CnfFlag:=false; FollowOnFlag true: Indicates that the message(s) to be sent on the same TTI will follow NOTE 1: If the TimingInfo is not the same for messages to be sent on the same TTI, the SS shall produce an error NOTE 2: the follow on flag applies only for messages of the same SRB false: Indicates that no more message(s) will follow RlcBearerRouting : If omitted the SS shall assume the data to be routed to the same cell as indicated by CellId; when - e.g. in case of split SRB - a specific routing is provided by TTCN the SS shall route the data accordingly. |
| RrcPdu | RRC_MSG_Request_Type |  |  |

EUTRA_RRC_PDU_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | EUTRA_RRC_PDU_IND |  |  |
| Comment | common ASP to receive PDUs from SRB0, SRB1 or SRB2 |  |  |
| Common | IndAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB0, SRB1, SRB2 TimingInfo : time when message has been received (frame and sub-frame number); this is handed through to the test case by the NAS emulation NOTE: normally an RRC PDU is expected in one TTI; nevertheless if it is spread over more than one TTIs TimingInfo shall refer to the end of the PDU i.e. to the last RLC PDU being received; Status : OK or RRC integrity error RlcBearerRouting : In case of split SRB the SS shall provide the id of the cell in which the data has been sent from the UE; otherwise the SS may provide the cell id but it is not checked by TTCN |
| RrcPdu | RRC_MSG_Indication_Type |  |  |

NASEMU_EUTRA_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NASEMU_EUTRA_SYSTEM_PORT |  |
| Comment | NASEMU PTC: Port for Sending/Receiving data to/from the SYSTEM Interface |  |
| out | EUTRA_RRC_PDU_REQ |  |
| in | EUTRA_RRC_PDU_IND |  |

# D.7	EUTRA_CommonDefs

## D.7.1	Common_Types

Common_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RedundancyVersion_Type | integer (0..3) | used in EUTRA_ASP_DrbDefs and EUTRA_ASP_Typedefs |

HarqProcessList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | HarqProcessList_Type |
| Comment | list of HARQ processes: each element shall be unique |
| record length(0..14) of HarqProcessId_Type |  |

RRC_MSG_Request_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RRC_MSG_Request_Type |  |
| Comment | DL RRC PDU on CCCH or DCCH |  |
| Ccch | DL_CCCH_Message |  |
| Dcch | DL_DCCH_Message |  |

RRC_MSG_Indication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RRC_MSG_Indication_Type |  |
| Comment | UL RRC PDU on CCCH or DCCH |  |
| Ccch | UL_CCCH_Message |  |
| Dcch | UL_DCCH_Message |  |

## D.7.2	Common_Constants

EUTRA_CommonDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_EUTRA_MaxNumberOfCells | integer | 20 | Maximum number of cells; in TS 36.508 in, clause 4.4.2 and 6.3.2.2 there are tables for cells being used in non-NAS and NAS test cases; in both cases less than 20 cells are listed |

## D.7.3	RRC_Nested_Types

RRC_Nested_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SchedulingInfoSIB1_BR_r13_Type | MasterInformationBlock.schedulingInfoSIB1_BR_r13 |  |
| SiWindowLength_Type | SystemInformationBlockType1.si_WindowLength |  |
| BandwidthReducedAccessRelatedInfo_Type | SystemInformationBlockType1_v1310_IEs.bandwidthReducedAccessRelatedInfo_r13 |  |
| PrioritizedBitRate_Type | LogicalChannelConfig.ul_SpecificParameters.prioritisedBitRate |  |
| Ra_PreambleIndex_Type | RACH_ConfigDedicated.ra_PreambleIndex |  |
| EDRX_CycleStartOffset_r13_Type | MAC_MainConfig.eDRX_Config_CycleStartOffset_r13 |  |
| UE_Category_Type | UE_EUTRA_Capability.ue_Category |  |
| DRX_Config_r13_Type | MAC_MainConfig.drx_Config_r13 |  |
| TDD_SubframeAssignment_Type | TDD_Config.subframeAssignment |  |
| SCellPathlossReferenceLinking_Type | UplinkPowerControlDedicatedSCell_r10.pathlossReferenceLinking_r10 |  |
| MAC_MainConfig_ScellDeactivationTimer_Type | MAC_MainConfig.mac_MainConfig_v1020.sCellDeactivationTimer_r10 |  |
| CrossSchedulingCarrierInfo_Type | CrossCarrierSchedulingConfig_r10.schedulingCellInfo_r10.other_r10 |  |
| NotificationIndicator_r9_Type | MBSFN_AreaInfo_r9.notificationIndicator_r9 |  |
| LogicalChannelIdentity_r9_Type | MBMS_SessionInfo_r9.logicalChannelIdentity_r9 |  |
| UE_Category_v1020_Type | UE_EUTRA_Capability_v1020_IEs.ue_Category_v1020 |  |
| UE_Category_v1170_Type | UE_EUTRA_Capability_v1170_IEs.ue_Category_v1170 |  |
| UE_Category_v11a0_Type | UE_EUTRA_Capability_v11a0_IEs.ue_Category_v11a0 |  |
| UE_CategoryDL_v1310_Type | UE_EUTRA_Capability_v1310_IEs.ue_CategoryDL_v1310 |  |
| UE_Category_v1250_Type | UE_RadioPagingInfo_r12.ue_Category_v1250 |  |
| UE_CategoryDL_r12_Type | UE_EUTRA_Capability_v1250_IEs.ue_CategoryDL_r12 |  |
| UE_CategoryDL_v1260_Type | UE_EUTRA_Capability_v1260_IEs.ue_CategoryDL_v1260 |  |
| UE_CategoryDL_v1330_Type | UE_EUTRA_Capability_v1330_IEs.ue_CategoryDL_v1330 |  |
| UE_CategoryDL_v1350_Type | UE_EUTRA_Capability_v1350_IEs.ue_CategoryDL_v1350 |  |
| UE_CategoryDL_v1450_Type | UE_EUTRA_Capability_v1450_IEs.ue_CategoryDL_v1450 |  |
| UE_CategoryDL_v1460_Type | UE_EUTRA_Capability_v1460_IEs.ue_CategoryDL_v1460 |  |
| E_HARQ_Pattern_r12_Type | MAC_MainConfig.e_HARQ_Pattern_r12 |  |
| TTI_Bundling_Type | MAC_MainConfig.ul_SCH_Config.ttiBundling |  |
| CE_Mode_r13_Type | PhysicalConfigDedicated.ce_Mode_r13 |  |
| SC_MCCH_Duration_Type | SystemInformationBlockType20_r13.sc_mcch_duration_r13 |  |
| SC_MCCH_FirstSubframe_Type | SystemInformationBlockType20_r13.sc_mcch_FirstSubframe_r13 |  |
| SC_MCCH_ModificationPeriod_r13_Type | SystemInformationBlockType20_r13.sc_mcch_ModificationPeriod_r13 |  |
| SC_MCCH_Offset_Type | SystemInformationBlockType20_r13.sc_mcch_Offset_r13 |  |
| SC_MCCH_RepetitionPeriod_r13_Type | SystemInformationBlockType20_r13.sc_mcch_RepetitionPeriod_r13 |  |
| DMTC_PeriodOffset_r12_Type | MeasDS_Config_r12.setup.dmtc_PeriodOffset_r12 |  |
| DS_OccasionDuration_r12_Type | MeasDS_Config_r12.setup.ds_OccasionDuration_r12 |  |
| SC_MCCH_ModificationPeriod_v1470_Type | SystemInformationBlockType20_r13.sc_mcch_ModificationPeriod_v1470 |  |
| SC_MCCH_RepetitionPeriod_v1470_Type | SystemInformationBlockType20_r13.sc_mcch_RepetitionPeriod_v1470 |  |
| LAA_UL_Allowed_Type | LogicalChannelConfig.laa_UL_Allowed_r14 |  |

## D.7.4	ASP_CommonPart

Definition of ASP common parts for REQ-, CNF- and IND-ASPs

### D.7.4.1	ASP_CommonPart_Definitions

#### D.7.4.1.1	Routing_Info

EUTRA_CommonDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_MaxRB | integer | maxDRB + 3 | DRBs + 3 SRBs |

RadioBearerId_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RadioBearerId_Type |  |
| Comment |  |  |
| Srb | SRB_Identity_Type |  |
| Drb | DRB_Identity |  |
| Mrb | MRB_Identity_Type |  |
| ScMrb | SC_MRB_Identity_Type |  |

RoutingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RoutingInfo_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| RadioBearerId | RadioBearerId_Type |  |
| QosFlow | QosFlow_Identification_Type | routing of user plane data for 5GC with SDAP being configured in non-transparent mode at the SS |

### D.7.4.2	REQ_ASP_CommonPart

ReqAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ReqAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | EUTRA_CellId_Type |  |  |
| RoutingInfo | RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| ControlInfo | ReqAspControlInfo_Type |  |  |
| RlcBearerRouting | RlcBearerRouting_Type | opt |  |

### D.7.4.3	CNF_ASP_CommonPart

CnfAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CnfAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | EUTRA_CellId_Type |  |  |
| RoutingInfo | RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| Result | ConfirmationResult_Type |  |  |

### D.7.4.4	IND_ASP_CommonPart

IndAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IndAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | EUTRA_CellId_Type |  |  |
| RoutingInfo | RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| Status | IndicationStatus_Type |  |  |
| RlcBearerRouting | RlcBearerRouting_Type | opt |  |

## D.7.5	MBMS_CommonDefs

Common definitions for MBMS needed for configuration of the SS

MBMS_CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| Pmch_InfoIndex_Type | integer (0..maxPMCH_PerMBSFN) |  |

MRB_Identity_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MRB_Identity_Type |  |  |
| Comment | MTCH is uniquely identified by: - the logicalChannelIdentity in the mbms-SessionInfoList-r9 - the MBSFN area identity, - the MCH index in the pmch-InfoList-r9 |  |  |
| Mbsfn_AreaId | MBSFN_AreaId_r12 |  |  |
| PmchLogicalChannel | PmchLogicalChannel_Type |  |  |

PmchLogicalChannel_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PmchLogicalChannel_Type |  |  |
| Comment |  |  |  |
| Pmch_InfoIndex | Pmch_InfoIndex_Type |  |  |
| LogicalChannelIdentity | LogicalChannelIdentity_r9_Type |  |  |

# D.8	CDMA2000_ASP_TypeDefs

## D.8.1	CDMA2000_Common

Common definitions for CDMA2000 and CDMA2000 ASPs

### D.8.1.1	CDMA2000_SystemContants

CDMA2000_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_CDMA2000_MaxNumberOfCells | integer | 8 | Maximum number of CDMA2000 cells; in TS 36.508 in, clause 6.3.1.5 and 6.3.1.6 define 4 cells each for HRPD and 1XRTT; hence total is 8 |

### D.8.1.2	CDMA2000_Routing

CDMA2000_Routing: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLP_FlowId_Type | integer (0..30) | As per S.0024, clause 4.8.2.10 both MaxNumRLPFlowsFwd and MaxNumRLPFlowsRvs need to be in the range of 0x06(6) to 0x1F(31) As per x.s007 clause 10, the PDN ID and Flow ID identify a flow |

RLP_FlowIdList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLP_FlowIdList_Type |
| Comment |  |
| record of RLP_FlowId_Type |  |

CDMA2000_RoutingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_RoutingInfo_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| RLP_FlowId | RLP_FlowId_Type |  |

### D.8.1.3	CDMA2000_TimingInfo

CDMA2000_TimingInfo: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| HRPD_FrameNumber_Type | integer | CDMA system time specified in units of 16 slots i.e. 26.66 ms. |
| HRPD_SubFrameNumber_Type | integer (0..3) | A sub-frame is a group of four contiguous slots. The start of a sub-frame is specified by (T - FrameOffset) mod 4 = 0, where T is the CDMA System Time in slots |
| RTT1X_FrameNumber_Type | integer | CDMA System Time, in integer multiples of 20 ms |

HRPD_SubFrameInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | HRPD_SubFrameInfo_Type |  |
| Comment |  |  |
| Number | HRPD_SubFrameNumber_Type |  |
| Any | Null_Type | no specific sub-frame (valid for REQ ASPs only) |

HRPD_Frame_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | HRPD_Frame_Type |  |
| Comment |  |  |
| Number | HRPD_FrameNumber_Type |  |
| Any | Null_Type | no specific TimeStamp (valid for REQ ASPs only) |

RTT1X_Frame_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RTT1X_Frame_Type |  |
| Comment |  |  |
| Number | RTT1X_FrameNumber_Type |  |
| Any | Null_Type | no specific TimeStamp (valid for REQ ASPs only) |

HRPD_SubFrameTiming_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HRPD_SubFrameTiming_Type |  |  |
| Comment |  |  |  |
| Frame | HRPD_Frame_Type |  |  |
| Subframe | HRPD_SubFrameInfo_Type |  |  |

CDMA2000_SubFrameTiming_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_SubFrameTiming_Type |  |
| Comment |  |  |
| HRPD | HRPD_SubFrameTiming_Type | HRPD Timing |
| RTT1X | RTT1X_Frame_Type | RTT1X Timing specified in terms of Frames only |

CDMA2000_TimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_TimingInfo_Type |  |
| Comment |  |  |
| SubFrame | CDMA2000_SubFrameTiming_Type |  |
| Now | Null_Type | to be used in REQ ASPs when there is no 'activation time' |
| None | Null_Type | only to be used in SYSTEM_CTRL_CNF but not for EnquireTiming |

### D.8.1.4	CDMA2000_ReqAspCommonPart

CDMA2000_ReqAspControlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_ReqAspControlInfo_Type |  |  |
| Comment |  |  |  |
| CnfFlag | boolean |  | true => SS shall send CNF: when the REQ is with no timing information (no activation time), SS shall send the confirmation when the configuration is done, i.e. when the test case may continue. Example: when there is a configuration followed by a send event it shall not be necessary to have a wait timer in between but the CNF triggers the send event or system Command. If there are other triggers e.g. like the UE sending a message, CnfFlag shall be set to false by the test case to avoid racing conditions with the CNF and the signalling message. When there is an activation time SS shall send the CNF after the configuration has been scheduled; that means SS shall not wait until the activation time has been expired. |
| FollowOnFlag | boolean |  | false => no further (related) information true: further related information will be sent to SS ; Currently this value is not foreseen to be used. |

CDMA2000_ReqAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_ReqAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | CDMA2000_CellId_Type |  |  |
| RoutingInfo | CDMA2000_RoutingInfo_Type |  |  |
| TimingInfo | CDMA2000_TimingInfo_Type |  |  |
| ControlInfo | CDMA2000_ReqAspControlInfo_Type |  |  |

### D.8.1.5	CDMA2000_IndAspCommonPart

CDMA2000_ErrorIndication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_ErrorIndication_Type |  |  |
| Comment |  |  |  |
| System | integer |  | any other error: may be SS specific error code; this will not be evaluated by TTCN; e.g. an error shall be raised when the UE requests retransmission of an RLC PDU |

CDMA2000_IndicationStatus_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_IndicationStatus_Type |  |
| Comment |  |  |
| Ok | Null_Type |  |
| Error | CDMA2000_ErrorIndication_Type |  |

CDMA2000_IndAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_IndAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | CDMA2000_CellId_Type |  |  |
| RoutingInfo | CDMA2000_RoutingInfo_Type |  |  |
| TimingInfo | CDMA2000_TimingInfo_Type |  |  |
| Status | CDMA2000_IndicationStatus_Type |  |  |

### D.8.1.6	CDMA2000_CnfAspCommonPart

CDMA2000_ConfirmationResult_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_ConfirmationResult_Type |  |
| Comment |  |  |
| Success | Null_Type |  |
| Error | integer | may contain SS specific error code; this will not be evaluated by TTCN |

CDMA2000_CnfAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_CnfAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | CDMA2000_CellId_Type |  |  |
| RoutingInfo | CDMA2000_RoutingInfo_Type |  |  |
| TimingInfo | CDMA2000_TimingInfo_Type |  |  |
| Result | CDMA2000_ConfirmationResult_Type |  | Similar definition as EUTRA |

## D.8.2	CDMA2000_PowerLevel

CDMA2000_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_CDMA2000_Attenuation_Off | CDMA2000_Attenuation_Type | {Off:=true} |  |

CDMA2000_PowerLevel: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| CDMA2000_InitialAttenuation_Type | CDMA2000_Attenuation_Type (tsc_CDMA2000_Attenuation_Off) | Attenuation restricted to 'Off' |

CDMA2000_Attenuation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_Attenuation_Type |  |
| Comment | attenuation of the reference power |  |
| Value | CDMA2000_AttenuationValue_Type | cell power reference power reduced by the given attenuation (value is in dB) |
| Off | Null_Type | for non suitable off cell we specify an explicit "Off" value here |

CDMA2000_CellAttenuation_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_CellAttenuation_Type |  |  |
| Comment |  |  |  |
| CellId | CDMA2000_CellId_Type |  |  |
| Attenuation | CDMA2000_Attenuation_Type |  |  |

CDMA2000_CellAttenuationList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | CDMA2000_CellAttenuationList_Type |
| Comment |  |
| record length(1..tsc_CDMA2000_MaxNumberOfCells) of CDMA2000_CellAttenuation_Type |  |

CDMA2000_AbsoluteCellPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_AbsoluteCellPower_Type |  |  |
| Comment |  |  |  |
| PowerIoc | PowerIoc_Type |  | TTCN writer Shall set same vale in all cells; SS shall have only one AWGN channel for all configured cells per frequency SS shall create a AWGN channel in first cell per frequency and ignore this in later cell creations on the same frequency; i.e. this channel is created along once for Cell 15 or 16 and one each per 17 and 19 similary for RTT1X once for 19 or 20 and one each per 21 and 22 |
| PowerIor | PowerIor_Type |  | Total Transmit power in cell before attenuation |
| PilotOffset | PilotOffset_Type |  | Default -7 |

CDMA2000_InitialCellPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_InitialCellPower_Type |  |  |
| Comment |  |  |  |
| MaxReferencePower | CDMA2000_AbsoluteCellPower_Type |  | maximum value of cell reference power corresponding to Max Ior/Ioc in power level table; a cell is initialised with this reference power; its value is the upper bound of the cell power during the test case |
| Attenuation | CDMA2000_InitialAttenuation_Type |  | initial attenuation Cell is off |

## D.8.3	CDMA2000_Data

Data primitives sent/received at CDMA2000_RLP_FLOW_PORT

CDMA2000_Data: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLP_SDU_Type | octetstring |  |

RLP_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLP_SDUList_Type |
| Comment |  |
| record of RLP_SDU_Type |  |

CDMA2000_U_PlaneData_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_U_PlaneData_Type |  |
| Comment | Union structure is provided for future possible enhancements |  |
| RLP_Sdu | RLP_SDUList_Type | RLP SDU's |

RLPFlow_DataPerSubframe_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLPFlow_DataPerSubframe_DL_Type |  |  |
| Comment | common definition for one or several SDUs to be sent in the subframe given by the subframe offset; SS shall raise an error indication (using SYSTEM_IND) when that is not possible NOTE 1: For RLP the data may be spread over more than one subframe ; the TTCN implemetation is responsible to calculate appropriate offsets accordingly |  |  |
| SubframeOffset | integer |  | subframe offset relative to the absolute timing information given in the common part of the ASP; NOTE : if a RLP SDU takes more than one subframe, SubframeOffset specifies the first TTI |
| SduList | CDMA2000_U_PlaneData_Type |  | list of PDUs/SDUs to be sent in one subframe |

RLPFlow_DataPerSubframeList_DL_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLPFlow_DataPerSubframeList_DL_Type |
| Comment | list of user plane data to be sent in sub-frames given by the SubframeOffset in the single elements of the list; Timing: the start time for the whole sequence is given by the timing info of the ASP (common information); the timing for the respective data pdus is given by the SubframeOffset relative to the common timing info; design consideration: repetitions of this sequence are not foreseen (in which case the subframe offset could not be related to the timing info of the ASP) |
| record of RLPFlow_DataPerSubframe_DL_Type |  |

CDMA2000_U_Plane_Request_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_U_Plane_Request_Type |  |  |
| Comment | NOTE: formal type definition to allow later enhancements; CDMA2000_U_Plane_Request_Type defines a sequence of subframes in which data shall be sent |  |  |
| SubframeDataList | RLPFlow_DataPerSubframeList_DL_Type |  |  |

## D.8.4	CDMA2000_CellConfiguration

HRPD_CellParameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HRPD_CellParameters_Type |  |  |
| Comment | Parameters specific to HRPD |  |  |
| SystemType | SystemType_Type |  | Specifies the sytem type of Channel As per Table 13.1-1 of C.S0024 0, 1, 2 are defined values and 3 to 255 are reserved |
| SubNetMask | B8_Type |  | 7.11.6.2.2 of C.S0024 Sector Subnet identifier set this field to the number of consecutive 1s in the subnet mask of the subnet to which this sector belongs |
| ColorCode | ColorCode_Type |  | 7.11.6.2.1 of C.S0024 set to the color code corresponding to this sector part of QuickConfig Over head message |
| CountryCode | MCC_Type |  | 7.11.6.2.2 of C.S0024 three-digit BCD (binary coded decimal) encoded representation of the Mobile Country Code associated with this sector |
| OpenLoopAdjust | OpenLoopAdjust_Type |  | 9.4.6.2.6 of C.S0024; The negative of the nominal power to be used by access terminals in the open loop power estimate, expressed as an unsigned value in units of 1 dB. The value used by the access terminal is -1 times the value of this field |
| ReverseRateLimit | ReverseRateLimit_Type |  | Table 9.9.6.3-2 of C.S0024; set to the highest data rate that the access terminal is allowed to use on the Reverse Traffic Channel |
| MACIndex | ReverseLinkMACIndex_Type |  | C.S0024 clause 12.4.1.3.2.2 Forward channel MAC is derivered from this based on table 12.4.1.3.2.2-1 |
| PacketApp | PacketApplication_Type |  | Multi Flow Packet Application to be used Enhanced Multi-Flow Packet Application subtype(0x0009) defined in C.S0087 or as per C.S0087, clause 2.3 the UE Shalll not propose AEMPA during stream protocol negotiation (0xFFFE) in C.R1001 |
| ControlChannelRate | ControlChannelRate_Type |  | MAC index to be used for the Control Channel |
| PDN_Id | PDN_Id_Type |  | PDN_ID of the bearer |
| PDN_Address | PDN_Address_Type |  | the PDN Address to be provided to the UE in VSNCP ConfigAck |
| UATI | UATI_Type |  | UATI to be Assigned to the UE |

RTT1X_CellParameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RTT1X_CellParameters_Type |  |  |
| Comment | Parameters specific to 1XRTT |  |  |
| Reg_Zone | B12_Type |  | C.S005 clause 3.7.2.3.2.1 and 2.6.5.1.5 Registration Zone of the base station Reg_Zone, SID and NID shall be unique for each base station |
| Base_Class | B4_Type |  | C.S0005 clause 3.7.2.3.2.1 Base station class. The base station shall set this field as follows: For Band Class 1 and 4, the base station shall set this field to '0001'; otherwise, the base station shall set this field to '0000' |
| MCC | B10_Type |  | 3.7.2.3.2.13 and 2.3.1.1 of C.S0005 encoding is int2bit (100*D1+10*D2+D3 -111,10) with digit 0 being maped to 10 binary representation of the Mobile Country Code associated with this sector |
| IMSI_11_12 | B7_Type |  | 3.7.2.3.2.13 and 2.3.1.2 of C.S0005 encoding is int2bit (10*D2+D3 -11,7) with digit 0 being maped to 10 binary representation of the Mobile Network Code associated with this sector |
| TMSI | TMSI_Type |  | the TMSI to be assigned to the MS |
| ProtRev | ProtRev_Type |  | Protocol Revision |
| Min_ProtRev | ProtRev_Type |  | the minimum protocol revision supported by Base station |
| Sig_EncryptionMode | EncryptionMode_Type |  | Encryption mode for Common and dedicated signalling |
| USerInfo_EncryptionMode | EncryptionMode_Type |  | User information Encryption mode |

ModeSpecificCellParams_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ModeSpecificCellParams_Type |  |
| Comment |  |  |
| RTT1X | RTT1X_CellParameters_Type |  |
| HRPD | HRPD_CellParameters_Type |  |

CDMA2000_CellParameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_CellParameters_Type |  |  |
| Comment |  |  |  |
| Type | CDMA2K_Type |  | Gives if cell is EHRPD or RTT1X |
| CarrierFreq | CarrierFreqCDMA2000_Type |  | Contains bandclass (5 bit) and arfcn i.e. 11 bit channel number |
| PhysCellId | PhysCellIdCDMA2000_Type |  | PN offset of pilot 0..511 |
| CellGlobalId | CellGlobalIdCDMA2000_Type |  | Contains the 128 bit cell ID for HRPD and 47 bit cell ID for 1XRTT |
| SearchWindow | SearchWindowSizeRecord_Type |  | contains the SearchWindow for Active, Neighbor & Remaining cells |

CDMA2000_CellConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_CellConfigInfo_Type |  |  |
| Comment |  |  |  |
| CellParameters | CDMA2000_CellParameters_Type |  | Parameters common to HRPD and RTT1X |
| InitialCellPower | CDMA2000_InitialCellPower_Type |  | Power level parameters |
| ModeSpecificCellParams | ModeSpecificCellParams_Type |  | Parameters specific to RTT1X or HRPD |

CDMA2000_CellConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_CellConfigRequest_Type |  |
| Comment |  |  |
| AddOrReconfigure | CDMA2000_CellConfigInfo_Type | for cell configuration: CellId : identifier of the cell to be configured RoutingInfo : None TimingInfo : Now (for initial configuration and for reconfiguration in general) ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Release | Null_Type | to remove a cell completely - CellId : identifier of the cell to be released; eutra_Cell_NonSpecific, in case all cells shall be released RoutingInfo : None TimingInfo : Now ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

## D.8.5	CDMA2000_HRPD

### D.8.5.1	CDMA2000_PDN_Defs

CDMA2000_PDN_Defs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| CDMA2000_AttachType | O3_Type | Defined values: 1: Initial Attach to a PDN, 3: Handover attach to a PDN. Rest undefined and not used |
| IPv4_Address_Type | O4_Type | represents the IPv4 address as per 24.301 clause 9.9.4.9 |
| IPv6_Address_Type | O8_Type | represents the IPv6 interface identifier as per 24.301 clause 9.9.4.9 |
| PDN_Id_Type | B4_Type | indicates the PDN Id associated with the bearer PDN Identifier of the PDN for which the user data is sent. it is the low order 4 bits of, containing the PDN-ID identifies the PDN (i.e. one per default bearer) Reference x.s0057 clause 10.1.5; gives only low order 4 bits, and high order 4 bits are added as all zero's |
| Flow_Id_Type | B4_Type | the lower 4 bits of the Flow Identifier, as defined in Table 15 of x.s0057 identify each reservation that is requested to be added or deleted the complete 8 bit flow Identifier is formed by PDN-ID and Flow-Id |

IPv4v6_Address_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPv4v6_Address_Type |  |  |
| Comment |  |  |  |
| IPv4 | IPv4_Address_Type |  | IP v4 address to be allocated |
| IPv6 | IPv6_Address_Type |  | IP v6 interface identifier to be allocated |

PDN_Address_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDN_Address_Type |  |
| Comment | based on 24.301 cl. 9.9.4.9 |  |
| IPv4 | IPv4_Address_Type | only IP v4 address to be allocated |
| IPv6 | IPv6_Address_Type | only IP v6 interface identifier to be allocated |
| IPv4v6 | IPv4v6_Address_Type | both IP v4 address and IP v6 interface identifier to be allocated |

Flow_IdList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | Flow_IdList_Type |
| Comment |  |
| record of Flow_Id_Type |  |

### D.8.5.2	CDMA2000_SubProtocols

LCP_DetachInit_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | LCP_DetachInit_Type |
| Comment |  |
| networkInitiated | x.s0057 clause 11.2 |
| UEInitiated | x.s0057 clause 11.1.2 |

DHCP_Ind_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DHCP_Ind_Type |  |  |
| Comment |  |  |  |
| RapidCommit | boolean |  | indicates if Rapid Comit option of DHCP is used |

UATI104_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UATI104_Type |  |
| Comment |  |  |
| Value | O13_Type |  |
| None | Null_Type |  |

UATI_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UATI_Type |  |  |
| Comment |  |  |  |
| UATI24 | O3_Type |  | Represents UATI (0:23), as per clause 6.3.7.2.2 of C.S0024 |
| UATI104 | UATI104_Type |  | Represents UATI (127:24), as per clause 6.3.7.2.2 of C.S0024 if has to be assigned |

### D.8.5.3	HRPD_Indications

RegAndDefBearerEstInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RegAndDefBearerEstInd_Type |  |  |
| Comment |  |  |  |
| UATI_AssignmentCmpl | Null_Type |  | UATIAssignment is received UATIComplete is received |
| InitialChAssignCmpl | Null_Type |  | Initial Traffic/Extended Channel/AlternateLink(Pre-registration) Assignment procedure started UE has sent ConnectionRequest/AlternateLinkOpen message Traffic/Extended Channel /AlternateLink(Pre-registration) assignment is completedUE has sent TrafficChannelComplete( Route update protocol)/ AlternateLinkOpenComplete. In the registration and Default bearer establishment procedure, UE initiated Channel/Alternate Link can be released and configured, only first assignment is reported. |
| SCP_ConfigCmpl | Null_Type |  | SCP (Session Configuration Protocol )ConfigurationRequest mesage is received SCP (Session Configuration Protocol )ConfigurationResponse mesage is transmitted |
| Stream_ConfigCmpl | Null_Type |  | Stream Protocol Configuration ConfigurationRequest mesage is received Stream Protocol Configuration ConfigurationResponse mesage is transmitted |
| EMPA_MMPA_ConfigCmpl | Null_Type |  | Enhanced Multi flow/Multi flow Packet application ConfigurationRequest mesage is received Enhanced Multi flow/Multi flow Packet application ConfigurationComplete mesage is received EMPA ConfigurationResponse message or MMPA ConfigurationResponse is received corresponding to steps 30A TO 30C of table 4.5.2B.3-2 |
| SessionNegotiationCmpl | Null_Type | opt | SS initiated Session Negotiation has started; Session Negotiation has completed |
| DeviceAuthCmpl | Null_Type | opt | Device level authentication has started; Device level authentication has completed |
| LocationUpdateCmpl | Null_Type | opt | Location Update started; Location Update completed |
| EAP_AKA_Cmpl | Null_Type |  | Improved Extensible Authentication protocol for Authentication and Key agreement started RFC 5448 * Message flow in x.s0057 clause 5.2.5.1 Authentication and Key agreement Completed optionally After entering PPP LCP Open State, PPP Version Capability Indicaiton and/or Max PPP Inactivity Timer negotiation are completed |
| VSNCP_ConfigCmpl | Null_Type |  | PDN connection establishment started and UE has sent PPP Vendor Specific Network Control Protocol Configuration Request PDN Connection and default bearer establishment is completed with possible IPV4 address (optional) and or IPv6 interface ID (Mandatory) provided Attach type shall be Handover Attach |
| DHCP_ConfigCmpl | DHCP_Ind_Type | opt | UE and SS decided for IPv4 address allocation by DHCP IPv4 address allocation completed by UE and SS Completion of IP Address through DHCP |
| ICMPv6_ConfigCmpl | Null_Type | opt | UE optionally sent IPv6 stateless autoconfiguration Router solitation message and SS has responded with IPv6 Router Advertisement message |

HRPD_ZoneRegistrationInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | HRPD_ZoneRegistrationInd_Type |  |  |
| Comment |  |  |  |
| UATI_AssignmentCmpl | Null_Type |  | UATIAssignment is received UATIComplete is received |
| EAP_AKA_Cmpl | Null_Type |  | Improved Extensible Authentication protocol for Authentication and Key agreement started RFC 5448 Message flow in x.s0057 clause 5.2.5.1 Authentication and Key agreement Completed optionally After entering PPP LCP Open State, PPP Version Capability Indicaiton and/or Max PPP Inactivity Timer negotiation are completed |
| VSNCP_ConfigCmpl | Null_Type |  | PDN connection establishment started and UE has sent PPP Vendor Specific Network Control Protocol Configuration Request PDN Connection and default bearer establishment is completed with possible IPV4 address (optional) and or IPv6 interface ID (Mandatory) provided Attach type shall be Handover Attach |
| DHCP_ConfigCmpl | DHCP_Ind_Type | opt | UE and SS decided for IPv4 address allocation by DHCP IPv4 address allocation completed by UE and SS Completion of IP Address through DHCP |
| ICMPv6_ConfigCmpl | Null_Type | opt | UE optionally sent ICMPv6 Router solitation message and SS has responded with IPv6 Router Advertisement message |

DedicatedBearerRelInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DedicatedBearerRelInd_Type |  |  |
| Comment |  |  |  |
| VSNP_TerminateCmpl | Null_Type |  | Dedicated bearers are deactivated/ released |
| SCP_ReleaseCmpl | Null_Type | opt | Session Configuration Protocol to relase the reservations exclusively associated with the deleated bearer Reservation deletion completed |

DefaultBearerRelDetachInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DefaultBearerRelDetachInd_Type |  |  |
| Comment |  |  |  |
| VSNCP_TerminateCmpl | Null_Type | opt | To Released configured default bearer and hense associated Dedicated bearer x.s0057 clause 11.3 and 11.1.1 To indicate the default bearer is released |
| LCP_TerminateCmpl | Null_Type |  | To detach the UE x.s0057 clause 11.2 Detach completed |

MobilityFromEUTRACmpl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MobilityFromEUTRACmpl_Type |  |  |
| Comment |  |  |  |
| ConnectionReqRcvd | Null_Type |  | Received Tunneled HRPD Connection Request Message |
| RouteUpdateInd | Null_Type |  | Received Tunneled HRPD Route Update Message |
| TrafficChCmplRcvd | Null_Type |  | Received HRPD Traffic Channel Complete in HRPD RAT, after transmission of tunneled Traffic Channel Assignment, HRPD Silence Parameters and HRPD Open Loop Parameters |

AdditionalDefBearerEstInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | AdditionalDefBearerEstInd_Type |  |  |
| Comment |  |  |  |
| VSNCP_ConfigCmpl | Null_Type |  | PDN connection establishment started and UE has sent PPP Vendor Specific Network Control Protocol Configuration Request PDN Connection and default bearer establishment is completed with possible IPV4 address (optional) and or IPv6 interface ID (Mandatory) provided Attach type shall be Handover Attach(pre-registration) or Initial Attach (if normal registration in HRPD cell) |
| DHCP_ConfigCmpl | DHCP_Ind_Type | opt | UE and SS decided for IPv4 address allocation by DHCP IPv4 address allocation completed by UE and SS Completion of IP Address through DHCP |
| ICMPv6_ConfigCmpl | Null_Type | opt | UE optionally sent ICMPv6 Router solitation message and SS has responded with IPv6 Router Advertisement message |

HRPD_SystemIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | HRPD_SystemIndication_Type |  |
| Comment |  |  |
| Error | Null_Type | Used by SS to indicate any error; the Actual Error types reported in ASP common part in CDMA2000_IndicationStatus_Type |
| InitialAccessProbeRcvd | Null_Type | Initial Access probe is received; |
| RegAndDefBearerEstInd | RegAndDefBearerEstInd_Type | UE has succesfully performed registration and default bearer esablishment |
| DedicatedBearerEstInd | Null_Type | Vendor specific network protocol (RFC 3772) procedures to re-establish Dedicated bearer as defined in S.0057 clause 5.5.3.1 (BCM is MS/NW) or clause 5.5.4.1.1 (BCM = MS-Only) Bearer Configuration Mode Dedicated bearers are (re) established |
| DedicatedBearerRelInd | DedicatedBearerRelInd_Type | To indicate the Dedicated bearer is released |
| DefaultBearerRelDetachInd | DefaultBearerRelDetachInd_Type | To Release configured default bearer and hense associated Dedicated bearer x.s0057 clause 11.3 and 11.1.1 Dedicated bearers are deactivated/released To detach the UE x.s0057 clause 11.2 Detach completed |
| MovedToDormantMode | Null_Type | The channels are released and UE is moved to PPP dormant mode/Air interface Idle. |
| MobilityFromEUTRACmpl | MobilityFromEUTRACmpl_Type | To confirm that Handover from EUTRAN is completed by receiving Traffic Channel Complete and the MessageSequence is same as in Traffic Channel Assignment |
| AdditionalDefBearerEstInd | AdditionalDefBearerEstInd_Type | UE has successfully performed additional default bearer establishment (additional PDN) |
| HRPD_ZoneRegistrationInd | HRPD_ZoneRegistrationInd_Type | Initially pre-registered UE detects change in HRPD Zone ID in SIB and hence updates registration |

### D.8.5.4	HRPD_Commands

HRPD_UE_InitStateType

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | HRPD_UE_InitStateType |
| Comment | HRPD UE states as defined in C.S0057 clause 3.1 |
| idle_Null | In the Inactive/Null State, 1. there is no physical traffic channel between the UE and the eAN, and no connection exists between the eAN and the ePCF 2. no PPP link between the UE and the HSGW . 3. The UE may have a Universal Access Terminal Identifier (UATI) that has been assigned by an eHRPD eAN |
| dormant | In the Dormant State, 1. no physical traffic channel exists between the UE and the eAN and no connection exists between the eAN and the ePCF. 2. PPP link between the UE and the HSGW 3. eHRPD DORMANT state equates to the "idle" state referred to in TS 23.402 |
| active_Connected | In the Active/Connected State, 1. a physical traffic channel exists between the UE and the eAN over which data may be sent. A connection exists between the eAN and the ePCF, and between the ePCF and the HSGW, 2. there is a PPP link between the UE and the HSGW |
| preregister | The UE is performing pre-register though a different Access network |

RegAndDefBearerEst_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RegAndDefBearerEst_Type |  |  |
| Comment |  |  |  |
| InitState | HRPD_UE_InitStateType |  |  |
| RLP_FlowId | RLP_FlowId_Type |  | Associated RLP Flow ID |
| AttachType | CDMA2000_AttachType |  | The Attach Type to be expected in VSNCP procedure |

DefaultBearerRelDetach_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DefaultBearerRelDetach_Type |  |  |
| Comment |  |  |  |
| InitState | HRPD_UE_InitStateType |  |  |
| PDN_Id | PDN_Id_Type |  | PDN_ID of the bearer |
| RLP_FlowId | RLP_FlowId_Type |  | Associated RLP Folw ID |
| UE_NW_Initiated | LCP_DetachInit_Type |  | If initiated by UE or Network |

DedicatedBearerEstRel_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DedicatedBearerEstRel_Type |  |  |
| Comment |  |  |  |
| InitState | HRPD_UE_InitStateType |  | PPP and Air Interface state of UE when the procedure is being executed |
| AssociatedDefaultBearer | PDN_Id_Type |  | the PDN ID of the associated default bearer; Gives the APN with which addititonal Dedicated Bearer needs to be established |
| Flow_Ids | Flow_IdList_Type |  | Flow_ID's of the multiple dedicated bearers to be Activated/Deactivated |
| RLP_FlowIds | RLP_FlowIdList_Type |  | Associated RLP Folw ID; There is one to one association between elements in Flow_IdList_Type and RLP_FlowIdList_Type; ITs a TTCN programing error otherwise |

AdditionalDefBearerEst_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | AdditionalDefBearerEst_Type |  |  |
| Comment | used for multiple PDN connections |  |  |
| InitState | HRPD_UE_InitStateType |  |  |
| RLP_FlowId | RLP_FlowId_Type |  | Associated RLP Flow ID |

HRPD_SystemCommand_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | HRPD_SystemCommand_Type |  |
| Comment |  |  |
| ReportInitialAccesProbe | Null_Type | SS is expected to report any possible Access probes received on HRPD Cell; will be used in situations where UE is not expected to camp on a HRPD Cell |
| RegAndDefBearerEst | RegAndDefBearerEst_Type | To complete registeration and establish Default bearer; Initial UE State is Idle_Null State Indications upto VSNCP protocol and possible IP signalling over DHCPv4 and/or ICMPv6 is performed At the end of procedure, UE is still in Active/Connected state (not pre-registration); SS is expected to send InitialAccessProbeRcvd(only if initial state is not Active and not pre-registration) and RegAndDefBearerEstInd as an indication for succesful completion of procedure |
| DedicatedBearerEst | DedicatedBearerEstRel_Type | Dedicated bearers are established/Activated by VSNP/EMPA protocol; PDN ID and RLP flow ID pairs are provided for each Dedicated bearer At the end of procedure, UE is still in Active/Connected state SS is expected to send InitialAccessProbeRcvd (only if initial state is not Active) and DedicatedBearerEstInd as an indication for succesful completion of procedure |
| MoveToDormantState | Null_Type | UE is Active_Connected state and is moved to Dormant state SS is expected to send MovedToDormantMode |
| MoveToActiveState | RLP_FlowIdList_Type | UE is initially Dormant state; UE is made to Move to Active_Connected State List of RLP flow Id's (associated with default + dedicated bearer), need to be established are provided SS is expected to send InitialAccessProbeRcvd |
| DedicatedBearerRel | DedicatedBearerEstRel_Type | Dedicated bearers are released/De-Activated by VSNP terminate and SCP release protocol; At the end of procedure, UE is still in Active/Connected state (not pre-registration) SS is expected to send InitialAccessProbeRcvd (only if initial state is not Active and not pre-registration) and DedicatedBearerRelInd as an indication for succesful completion of procedure |
| DefaultBearerRelDetach | DefaultBearerRelDetach_Type | Default bearer is released by VSNCP terminate and SCP release protocol UE is made to detach by LCP protocol and Possible Channels are released At the end of procedure, UE is in Idle_Null state Notes: When Detach is network initiated the sequence is 1. Default bearer (and hence all associated Dedicated bearers) released by VSNCP termintate 2. UE is detached by LCP terminate procedure When Detach is UE initiatated, UE may only perform LCP terminate procedure SS is expected to send InitialAccessProbeRcvd (only if initial state is not Active) and DefaultBearerRelDetachInd as an indication for succesful completion of procedure |
| MobilityFromEUTRA | Null_Type | Prepare CDMA SS for receiving tunneled HRPD Connection Request and Route Update tunneled in ULHandoverPreparationTransfer Respond with GCSNA encapsulated HRPD Silence Parameters and HRPD Open Loop Parameters, HRPD Traffic Channel Assignment to be sent tunneled in MobilityFromEUTRACommand Receive Traffic Channel Complete in the HRPD Cell; After Receiving Traffic Channel Assignment , HRPD Silence Parameters and HRPD Open Loop Parameters embedded in EUTRA message MobilityFromEUTRACommand, UE has Tuned to HRPD Radio and transmitted Traffic Channel Complete in the HRPD Cell SS is expected to send MobilityFromEUTRACmpl as an indication for succesful completion of procedure |
| AdditionalDefBearerEst | AdditionalDefBearerEst_Type | To establish an additional PDN connection Initial UE State is Idle_Null State or procedure performed through pre-registration Indications upto VSNCP protocol and possible IP signaling over DHCPv4 and/or ICMPv6 is performed At the end of procedure, UE is still in Active/Connected state(not pre-registration); SS is expected to send InitialAccessProbeRcvd (only if initial state is not Active and not pre-registration) and AdditionalDefBearerEstInd as an indication for successful completion of procedure |
| HRPDZoneRegister | RegAndDefBearerEst_Type | To update registeration by a UE already registered and established Default bearer; Initial UE State is Idle_Null State Indications upto VSNCP protocol and possible IP signalling over DHCPv4 and/or ICMPv6 is performed At the end of procedure, UE is still in Active/Connected state (not pre-registration); SS is expected to send InitialAccessProbeRcvd(only if initial state is not Active and not pre-registration) and RegAndDefBearerEstInd as an indication for succesful completion of procedure |

## D.8.6	CDMA2000_RTT1X

### D.8.6.1	RTT1X_Indications

RTT1X call flows in RTT1x cell
Expected Sequence for Attach (Power Up Attach)
1. Initial AccessProbeRcvd
2. CS_RegistrationStart (Powerup)
3. CS_RegistrationCmpl
Expected Sequence for Detach (Power Down Attach)
1. Initial AccessProbeRcvd
2. CS_RegistrationStart (PowerDown)
3. CS_RegistrationCmpl
Expected Sequence for CSFB Call Establishment
1. Initial AccessProbeRcvd
2. CS_CallEstStart (Origination/ PageResponse)
3. ChAssignCmpl (Extended Channel Assignment is sent)
4. CS_CallEstCompleted (Acknowledgement Order Sent, Service Connect sent, Service Connect Completion received, Alert Sent/Received and ConnectOrder is received)
Expected Sequence for SRVCC call handover
1. HandoffCmpl

RTT1X_CS_CallType

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | RTT1X_CS_CallType |
| Comment |  |
| mo | Call is UE oringinated |
| mt | Call is UE Terminated |
| mo_Emergency | UE originated Emergency Call |

RTT1XAttachType

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | RTT1XAttachType |
| Comment | Ref C.S005 Table 2.7.1.3.2.1-1 |
| powerUpAttach | UE is doing Power up attach REG_Type = '0001'B; it was not previously attached |
| powerDownAttach | UE is doing power down attach REG_Type = '0011'B; it was previously attached |
| zoneBasedAttach | UE is doing Zone based attach REG_Type = '0010'B |
| orderedAttach | UE is doing Ordered attach REG_Type = '0101'B |
| OtherAttach | Any other Attach REG_Type does not equal above values |

CS_RegCmplInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CS_RegCmplInd_Type |  |  |
| Comment |  |  |  |
| CS_RegistrationCmpl | RTT1XAttachType |  | CS power up/down registration is completed UE Sent Registration message and received an L2 Acknowledgement Optionally SS can perform Authentication and and has sent Registration Accepted order |

CS_Reg_CallCmplInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CS_Reg_CallCmplInd_Type |  |  |
| Comment |  |  |  |
| CS_RegistrationCmpl | RTT1XAttachType | opt | CS power up/down registration is completed; This is omit if implicit registration is done UE Sent Registration message and received an L2 Acknowledgement Optionally SS can perform Authentication and and has sent Registration Accepted order UE can also do a implicit registration; i.e. reception of Origination/reconnect/CallRecovery/Page message by Base station is treated as implicit registration |
| CS_CallEstStarted | RTT1X_CS_CallType |  | Received Origination message for MO and Page Response for MT |
| ChAssignCmpl | Null_Type |  | (Extended) Channel Assignment procedure started UE has sent ConnectionRequestTraffic Extended Channel assignment is completedUE has sent TrafficChannelComplete |
| CS_CallEstCompleted | Null_Type |  | SS received Service Connect Completion (Mo) or ConnectOrder(MT) (i.e User Accepted call) |

CS_CallCmplInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CS_CallCmplInd_Type |  |  |
| Comment |  |  |  |
| CS_CallEstStarted | RTT1X_CS_CallType |  | Received Origination message for MO and Page Response for MT |
| ChAssignCmpl | Null_Type |  | (Extended) Channel Assignment procedure started completed |
| CS_CallEstCompleted | Null_Type |  | SS received Service Connect Completion (MO) or ConnectOrder (MT) (i.e User Accepted call) |
| IsEmergencyCall | boolean |  | True indicates the established call is emergency call, false indicates a normal voice has been established |

ECSFB_CallCmplInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ECSFB_CallCmplInd_Type |  |  |
| Comment |  |  |  |
| CS_CallEstStarted | RTT1X_CS_CallType |  | Received Tunneled GCSNA encapsulated Origination message for MO and Page Response for MT call |
| HandoffCmpl | Null_Type |  | indicates SS has received HandoffComplete message and the call is established |
| CS_CallEstCompleted | Null_Type |  | SS received Service Connect Completion (MO) or ConnectOrder (MT) (i.e User Accepted call) |
| IsEmergencyCall | boolean |  | True indicates the established call is emergency call, false indicates a normal voice has been established |

ECAM_CS_CallCmplInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ECAM_CS_CallCmplInd_Type |  |  |
| Comment |  |  |  |
| CS_TunneledCallEstStarted | RTT1X_CS_CallType |  | Received Tunneled GCSNA encapsulated Origination message for MO and Page Response for MT call |
| CS_CallEstStarted | RTT1X_CS_CallType |  | Received Origination message for MO and Page Response for MT |
| ChAssignCmpl | Null_Type |  | Extended Channel Assignment procedure started completed |
| CS_CallEstCompleted | Null_Type |  | SS received Service Connect[MO] or ConnectOrder[MT] (i.e User Accepted call) |
| IsEmergencyCall | boolean |  | True indicates the established call is emergency call, false indicates a normal voice has been established |

ECSFB_CallRejInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ECSFB_CallRejInd_Type |  |  |
| Comment |  |  |  |
| CS_CallEstRejected | RTT1X_CS_CallType |  | SS Received tunneled 1xRTT Origination (M0)/Page Response (MT) message Respond with 1xRTT Release Order message |
| IsEmergencyCall | boolean |  | True indicates the established call is emergency call, false indicates a normal voice has been established |

RTT1X_SystemIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RTT1X_SystemIndication_Type |  |
| Comment |  |  |
| Error | Null_Type | Used by SS to indicate any error; the Actual Error types reported in ASP common part in CDMA2000_IndicationStatus_Type |
| InitialAccessProbeRcvd | Null_Type | Initial Access probe is received |
| CS_RegistrationCmpl | CS_RegCmplInd_Type | CS power up/down registration is completed As registration message, and possible Authentication Registration accepted order are all sent received on f/r-csch UE at end is in Idle state |
| CS_Reg_CallCmplInd | CS_Reg_CallCmplInd_Type | CS Registration /implicit registration and Call Indication MO or MT UE is in connected state with f/r dtch configured |
| CS_CallCmplInd | CS_CallCmplInd_Type | CS Call Indication MO or MT UE is in connected state with f/r dtch configured |
| HandoffCmpl | Null_Type | needed for SRVCC handover of an IMS voice call on LTE to 1XRTT indicates SS has received HandoffComplete message and the call is established |
| MovedToIdleState | Null_Type | The channels are released and UE is moved to Idle state. CS Call is released by exchange of Release order in both directions C.S0005 figure B3 and B4 |
| ECSFB_CallCmplInd | ECSFB_CallCmplInd_Type | eCSFB Call Indication MO or MT UE is in connected state with f/r dtch configured |
| ECSFB_ECAM_CallCmplInd | ECAM_CS_CallCmplInd_Type | eCSFB ECAM based Call Indication MO or MT UE is in connected state with f/r dtch configured |
| ECSFB_CallRejInd | ECSFB_CallRejInd_Type | eCSFB Call Rejected MO or MT UE is not camping in 1xRTT |

### D.8.6.2	RTT1X_Commands

CS_Registration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CS_Registration_Type |  |  |
| Comment |  |  |  |
| AttachType | RTT1XAttachType |  |  |
| IsPreRegistration | boolean |  | Indicates if it is done as pre registration Value is ignored if Attach Type is Power down (Assumption detach happens only in 1XRTT cell) |
| RAND | B32_Type | opt | RAND [From eNB] to be included in CSFBParametersResponseCDMA2000, HandoverFromEUTRAPreparationRequest Value not present for power down registration |

RTT1X_SystemCommand_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RTT1X_SystemCommand_Type |  |
| Comment |  |  |
| ReportInitialAccesProbe | Null_Type | SS is expected to report any possible Access probes received on 1XRTT Cell; will be used in situations where UE is not expected to camp on a 1XRTT Cell |
| CS_Registration | CS_Registration_Type | Power up attach/ power down attach in 1xRTT cell or Pre registration (Power up attach) tunelled through a different RAT in case of pre-registration attach, the CDMA SS starts by sending mobilityParameters to be tunneled in CSFBParametersRequestCDMA2000 SS is expected to send InitialAccessProbeRcvd (only if initial state is not pre-registration) and CS_RegistrationCmpl as an indication for succesful completion of procedure |
| CSFB_Call | RTT1X_CS_CallType | CSFB by a (pre-)registered UE If the call Type is mt, CDMA SS sends 1x RTT GCSNA encapsulated General Page to be tunneled in DLInformation Transfer SS is expected to send InitialAccessProbeRcvd and CS_CallCmplInd as an indication for succesful completion of procedure |
| CS_Reg_CSFB_Call | RTT1X_CS_CallType | UE not previously pre-registered hence performs registration (Power up attach) and CSFB call Registration can be implicit registration SS is expected to send InitialAccessProbeRcvd and CS_Reg_CallCmplInd as an indication for succesful completion of procedure |
| MobilityFromEUTRA_1XRTT | Null_Type | Prepare SS for Mobility from Eutra CDMA SS sends mobilityParameters to be tunneled in HandoverFromEUTRAPreparationRequest Receive tunneled 1xRTT GCSNA Encapsulated Origination message and MEID in ULHandoverPreparationTransfer Respond with GCSNA encapsulated 1xRTT Handoff Direction message to be sent tunneled in MobilityFromEUTRACommand Receive HandoffCmpl in the 1xRTT Cell; SS is expected to send HandoffCmpl as an indication for succesful completion of procedure |
| CS_OrderedRegistration | CS_Registration_Type | ordered registration on (already registered) 1xRTT cell or Pre registration Power up attach) through a different RAT. SS triggers the procedure by sending Registration Request order (GCSNA encapsulated in case of pre-registration) and sends mobilityParameters to be tunneled in CSFBParametersRequestCDMA2000 SS is expected to send InitialAccessProbeRcvd (only if initial state is not pre-registration) and CS_RegistrationCmpl as an indication for succesful completion of procedure |
| E_CSFB_Call | RTT1X_CS_CallType | Prepare SS for Enhanced CSFB call If the call Type is mt, CDMA SS sends 1x RTT GCSNA encapsulated General Page to be tunneled in DLInformation Transfer CDMA SS sends mobilityParameters to be tunneled in HandoverFromEUTRAPreparationRequest Receive tunneled 1xRTT GCSNA Encapsulated Origination (M0)/Page Response (MT) message and MEID in ULHandoverPreparationTransfer Respond with GCSNA encapsulated 1xRTT Handoff Direction message to be sent tunneled in MobilityFromEUTRACommand Receive HandoffCmpl in the 1xRTT Cell; SS responds with Alert With Information (MT)/Sevice connect (MO) in the 1xRTT cell. SS receives Connect order (MT)/Service Connect Completion (MO) SS is expected to send ECSFB_CallCmplInd as an indication for succesful completion of procedure |
| E_CSFB_Call_ECAM | RTT1X_CS_CallType | Prepare SS for Enhanced CSFB call with Extended channel assignment If the call Type is mt, CDMA SS sends 1x RTT GCSNA encapsulated General Page to be tunneled in DLInformation Transfer CDMA SS sends mobilityParameters to be tunneled in HandoverFromEUTRAPreparationRequest Receive tunneled 1xRTT GCSNA Encapsulated Origination (M0)/Page Response (MT) message and MEID in ULHandoverPreparationTransfer Respond with GCSNA Encapsulated ECAM message to be sent tunneled in MobilityFromEUTRACommand Receive Origination in the 1xRTT Cell; After extended channel assignment, SS receives Connect order (MT)/Service Connect Completion (MO) SS is expected to send ECSFB_ECAM_CallCmplInd as an indication for succesful completion of procedure |
| E_CSFB_Call_Reject | RTT1X_CS_CallType | Prepare SS for Enhanced CSFB call, which needs to be rejected If the call Type is mt, CDMA SS sends 1x RTT GCSNA encapsulated General Page to be tunneled in DLInformation Transfer CDMA SS sends mobilityParameters to be tunneled in HandoverFromEUTRAPreparationRequest Receive tunneled 1xRTT GCSNA Encapsulated Origination (M0)/Page Response (MT) message and MEID in ULHandoverPreparationTransfer Respond with GCSNA encapsulated 1xRTT Release Order message to be sent tunneled in DLInformation Transfer SS is expected to send ECSFB_CallRejInd as an indication for succesful completion of procedure |

## D.8.7	System_Interface

CDMA2000_SystemRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_SystemRequest_Type |  |
| Comment |  |  |
| Cell | CDMA2000_CellConfigRequest_Type | configure/release a cell |
| CellAttenuationList | CDMA2000_CellAttenuationList_Type |  |

CDMA2000_SystemConfirm_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_SystemConfirm_Type |  |
| Comment | confirmations for system configuration; in general to be sent after the configuration has been done |  |
| Cell | Null_Type | (no further parameters from SS) |
| CellAttenuationList | Null_Type | (no further parameters from SS) NOTE 1: the confirmation shall be sent when all cells have changed power levels NOTE 2: for the CellId in the common ASP part the same rules are applied as for the CDMA2000 SYSTEM REQ |

CDMA2000_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_SYSTEM_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | CDMA2000_ReqAspCommonPart_Type |  | TimingInfo depends on respective primitive: |
| Request | CDMA2000_SystemRequest_Type |  | - Cell TimingInfo: 'now' (in general) - CellAttenuationList TimingInfo: 'now' (in general, but activation time may be used also) |

CDMA2000_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_SYSTEM_CTRL_CNF |  |  |
| Comment |  |  |  |
| Common | CDMA2000_CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN => SS may set TimingInfo to "None" |
| Confirm | CDMA2000_SystemConfirm_Type |  |  |

CDMA2000_SystemCommand_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_SystemCommand_Type |  |
| Comment |  |  |
| HRPD | HRPD_SystemCommand_Type | HRPD Specific System commands |
| RTT1X | RTT1X_SystemCommand_Type | 1XRTT specific System commands |

CDMA2000_SYSTEM_CMD

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_SYSTEM_CMD |  |  |
| Comment |  |  |  |
| Common | CDMA2000_ReqAspCommonPart_Type |  | Routing info will be none generally; TimingInfo is generally now but activation time may be used also for all System commands Cnf and Follow on flags are both false |
| Command | CDMA2000_SystemCommand_Type |  | HRPD or 1XRTT System commands |

CDMA2000_SystemIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_SystemIndication_Type |  |
| Comment |  |  |
| HRPD | HRPD_SystemIndication_Type |  |
| RTT1X | RTT1X_SystemIndication_Type |  |

CDMA2000_SYSTEM_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_SYSTEM_IND |  |  |
| Comment |  |  |  |
| Common | CDMA2000_IndAspCommonPart_Type |  | The SS shall provide TimingInfo depending on the respective indication: |
| Indication | CDMA2000_SystemIndication_Type |  | - Error TimingInfo: related to the error (if available) - HRPD/RTT1X Procedure completion The timing info corresponding to logical completion of the complete procedure includes completion of all sub protocols |

CDMA2000_RLP_FLOW_COMMON_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_RLP_FLOW_COMMON_IND |  |  |
| Comment | ASP to receive PDUs from RLP Packet Flows |  |  |
| Common | CDMA2000_IndAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : RLP Flow id TimingInfo : time when RLP SDU's has been completely received |
| Data | CDMA2000_U_PlaneData_Type |  |  |

CDMA2000_RLP_FLOW_COMMON_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_RLP_FLOW_COMMON_REQ |  |  |
| Comment | ASP to send PDUs to RLP Packet flows |  |  |
| Common | CDMA2000_ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : RLP Flow id TimingInfo : starting point when to start sending sequence of data PDUs e.g. TimeStampLong_Type = X, subframe number = x; U_Plane.SubframeDataList (i).SubframeOffset := offset_i; => U_Plane.SubframeDataList(i).PduSduList shall be sent out at TimeStampLong_Type = X + ((x + offset_i) / 4); subframe number = (x + offset_i) mod 4 ControlInfo : CnfFlag:=false; FollowOnFlag:=false |
| U_Plane | CDMA2000_U_Plane_Request_Type |  |  |

CDMA2000_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_SYSTEM_PORT |  |
| Comment | CDMA2000 PTC: Port for system configuration |  |
| out | CDMA2000_SYSTEM_CTRL_REQ |  |
| in | CDMA2000_SYSTEM_CTRL_CNF |  |

CDMA2000_SYSCMD_IND_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_SYSCMD_IND_PORT |  |
| Comment | CDMA2000 PTC: Port for system indications/Commands |  |
| out | CDMA2000_SYSTEM_CMD |  |
| in | CDMA2000_SYSTEM_IND |  |

CDMA2000_RLP_FLOW_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_RLP_FLOW_PORT |  |
| Comment | CDMA2000 PTC: Port for RLP SDU's to be sent on RLP packet data streams |  |
| out | CDMA2000_RLP_FLOW_COMMON_REQ |  |
| in | CDMA2000_RLP_FLOW_COMMON_IND |  |

# D.9	CDMA2000_CommonDefs

type definitions used by CDMA2000 and EUTRA

CDMA2000_CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SearchWindowSize_Type | integer (0..15) | Search Window as in 36.331 ASN.1 definition for SystemInformationBlockType8.searchWindowSize |
| BandclassCDMA2000_Type | integer (0..31) | Band class defined as in 36.331 ASN.1 definition for BandclassCDMA2000 |
| ARFCN_ValueCDMA2000_Type | integer (0..2047) | ARFCN for CDMA2000 cell as in 36.331 ASN.1 definition for ARFCN_ValueCDMA2000 |
| PhysCellIdCDMA2000_Type | integer (0..511) | PN offset for CDMA2000 cell as in 36.331 ASN.1 definition for PhysCellIdCDMA2000 |
| ProtRev_Type | integer (0..255) | protocol revision |
| OpenLoopAdjust_Type | integer (0..255) | 9.4.6.2.6 of C.S0024 |
| BCD_Digit_Type | integer (0..9) | To represent BCD digit of MCC |
| TMSI_Code_Type | O4_Type |  |
| EncryptionMode_Type | integer (0..7) | C.S0005 table 3.7.4.5-1 & 3.7.5.7-3 0 ... Encryption disabled 1 ... Encryption with ORYX algorithm for User Info and Enhanced Cellular Msg Encryption Algorithm for Signalling 2 ... Encryption with Rijndael algorithm 3-7 ... reserved |
| TMSI_ZoneLen_Type | integer (1..8) | TMSI Zone Lenght; On encoding this is encoded to B4_Type |
| SectorID_HRPD_Type | B128_Type | Sector ID for HRPD as in 36.331 ASN.1 definition for CellGlobalIdCDMA2000.cellGlobalIdHRPD |
| PilotOffset_Type | integer (-31..0) | Represents the offset i.e. Pilot Channel power to total cell power(dB); By default shall be set to -7 127 selected Max value by 7 bits |
| PowerIor_Type | integer (-127..0) | Represets the cell total Tx power Ior (dBm/1.23 MHz) |
| PowerIoc_Type | integer (-127..0) | Represets the cell total AWGN power Ioc (dBm/1.23 MHz) which is independent of cell |
| CDMA2000_AttenuationValue_Type | integer (0..127) |  |
| SystemType_Type | integer (0..255) | 0 to 2 are allowed and 3 to 255 are reserved 13.1 of C.S0024 |
| ColorCode_Type | integer (0..255) | 7.11.6.2.1 of C.S0024 |
| ReverseLinkMACIndex_Type | integer (0..383) | C.S0024 clause 12.4.1.3.2.2 |

MCC_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MCC_Type |
| Comment | Represents Mobile Country Code |
| record length (3) of BCD_Digit_Type |  |

TMSI_Zone_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | TMSI_Zone_Type |
| Comment | TMSI Zone 1 to 8 octets |
| record length (1..8) of B8_Type |  |

TMSI_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TMSI_Type |  |  |
| Comment | Globally unique TMSI as defined in C.s0005 clause 3.7.2.3.2.19 |  |  |
| TMSI_ZoneLen | TMSI_ZoneLen_Type |  | Length of TMSI_Zone 1..8 |
| TMSI_Zone | TMSI_Zone_Type |  | TMSI_ZoneLen octets of TMSI_Zone |
| TMSI_Code | TMSI_Code_Type |  | TMSI code |

SectorID_RTT1X_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SectorID_RTT1X_Type |  |  |
| Comment | Sector ID for 1XRTT acc. to C.S0005 clause 3.7.2.3.2.1 and as in 36.331 ASN.1 clause 6.3.4, definition of CellGlobalIdCDMA2000.cellGlobalId1XRTT |  |  |
| BaseId | B16_Type |  | Base station identification. The base station shall set this field to its identification number |
| NID | B16_Type |  | Network identification This field serves as a sub-identifier of a system as defined by the owner of the SID. The base station shall set this field to the network identification number for this network |
| SID | B15_Type |  | System identification. set to the system identification number for this system |

CarrierFreqCDMA2000_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CarrierFreqCDMA2000_Type |  |  |
| Comment | Carrier Frequency for CDMA2000 cell as in 36.331 ASN.1 definition for CarrierFreqCDMA2000; contains Band class 5 bit and Channel number 11 bit part of Sector Channel over head message contained in 24 bit Channel IE |  |  |
| BandClass | BandclassCDMA2000_Type |  |  |
| ARFCN | ARFCN_ValueCDMA2000_Type |  |  |

CDMA2K_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | CDMA2K_Type |
| Comment | CDMA 2000 Type for CDMA2000 cell as in 36.331 ASN.1 definition for CDMA2000-Type |
| type1XRTT |  |
| typeHRPD |  |

CellGlobalIdCDMA2000_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CellGlobalIdCDMA2000_Type |  |
| Comment | CDMA 2000 Type Sector ID of the Cell as in 36.331 ASN.1 definition CellGlobalIdCDMA2000 |  |
| RTT1X | SectorID_RTT1X_Type |  |
| HRPD | SectorID_HRPD_Type |  |

ReverseRateLimit_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | ReverseRateLimit_Type |
| Comment | Table 9.9.6.3-2 of C.S0024; set to the highest data rate that the access terminal is allowed to use on the Reverse Traffic Channel; 10 Reserved values |
| kbps0 |  |
| kbps9_6 |  |
| kbps19_2 |  |
| kbps38_4 |  |
| kbps76_8 |  |
| kbps153_6 |  |
| resrv1 |  |
| resrv2 |  |
| resrv3 |  |
| resrv4 |  |
| resrv5 |  |
| resrv6 |  |
| resrv7 |  |
| resrv8 |  |
| resrv9 |  |
| resrv10 |  |

PacketApplication_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | PacketApplication_Type |
| Comment | Type of Packet Application to be used in Stream protocol |
| enhMultiFlowPacketApp |  |

ControlChannelRate_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | ControlChannelRate_Type |
| Comment | Determines the MAC configuration for Control Channel |
| macIndex2 |  |
| macIndex3 |  |

CDMA2000_CellId_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | CDMA2000_CellId_Type |
| Comment |  |
| cdma2000_Cell_NonSpecific |  |
| cdma2000_Cell15 | HRDP Cell |
| cdma2000_Cell16 | HRDP Cell |
| cdma2000_Cell17 | HRDP Cell |
| cdma2000_Cell18 | HRDP Cell |
| cdma2000_Cell19 | RTT1X Cell |
| cdma2000_Cell20 | RTT1X Cell |
| cdma2000_Cell21 | RTT1X Cell |
| cdma2000_Cell22 | RTT1X Cell |

SearchWindowSizeRecord_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SearchWindowSizeRecord_Type |  |  |
| Comment |  |  |  |
| SearchWindow_Active | SearchWindowSize_Type |  | Search Window for Active Cells |
| SearchWindow_Neighbor | SearchWindowSize_Type |  | Search Window for Neighbor Cells |
| SearchWindow_Remaining | SearchWindowSize_Type |  | Search Window for Rest Cells |

# D.10	EUTRA_ASP_CDMA2000TunnellingDefs

ASP definitions for tunnelling of CDMA2000 messages e.g. during CDMA2000 pre-registration

EUTRA_ASP_CDMA2000TunnellingDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| TunneledInfoCDMA2000 | octetstring |  |
| MEID_Type | ULHandoverPreparationTransfer_r8_IEs.meid |  |
| CDMA2000_MSG_REQ | CDMA2000_UL_Container_Type |  |
| CDMA2000_MSG_IND | CDMA2000_DL_Container_Type |  |

UL_TunneledInfoCDMA2000

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UL_TunneledInfoCDMA2000 |  |  |
| Comment |  |  |  |
| Msg | TunneledInfoCDMA2000 |  | OCTET STRING |
| Meid | MEID_Type | opt | ASN.1 type: BIT STRING (SIZE (56)) used to tunnel meid received from UE in ULHandoverPreparationTransfer for 1xRTT, not present other wise |

CDMA2000_UL_Container_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_UL_Container_Type |  |  |
| Comment |  |  |  |
| CDMA2000Type | CDMA2000_Type |  | ASN.1 type: type1XRTT, typeHRPD |
| UL_Msg | UL_TunneledInfoCDMA2000 |  |  |

DL_TunneledInfoCDMA2000

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DL_TunneledInfoCDMA2000 |  |  |
| Comment |  |  |  |
| Msg | TunneledInfoCDMA2000 |  | OCTET STRING |

CDMA2000_DL_Container_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CDMA2000_DL_Container_Type |  |  |
| Comment |  |  |  |
| CDMA2000Type | CDMA2000_Type |  | ASN.1 type: type1XRTT, typeHRPD |
| DL_Msg | DL_TunneledInfoCDMA2000 |  | OCTET STRING |

CDMA2000_TUNNELLING_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | CDMA2000_TUNNELLING_PORT |  |
| Comment | EUTRA PTC: Port to deal with tunnelling of CDMA2000 messages |  |
| out | CDMA2000_MSG_REQ |  |
| in | CDMA2000_MSG_IND |  |

# D.11	EUTRA_ASP_VirtualNoiseDefs

ASP definitions for virtual noise generation in EUTRA cells.
The noise is configured for an already existing EUTRA cell.
For UEs with 2 antenna connectors the AWGN (Additive white Gaussian noise) signals applied to each receiver antenna connector shall be uncorrelated.

EUTRA_ASP_VirtualNoiseDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| EUTRA_VngConfigConfirm_Type | Null_Type |  |

EUTRA_VngConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | EUTRA_VngConfigInfo_Type |  |  |
| Comment |  |  |  |
| Bandwidth | Dl_Bandwidth_Type |  | Bandwidth to be used for the noise (in general the same bandwidth as for the associated EUTRA cell) |
| NocLevel | integer |  | Noc level; calculation is according to 36.523-3 cl 7.22 |

EUTRA_VngConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | EUTRA_VngConfigRequest_Type |  |
| Comment | configure/activate noise for a given cell; NOTE: it is assumed the the associated EUTRA cell has been created beforehand |  |
| Configure | EUTRA_VngConfigInfo_Type | configuration of the virtual noise generator; regardless of the power level the noise generator is off before it gets activated for this cell; in case the configuration needs to be changed during a test, the noise generator shall be deactivated for this cell |
| Activate | Null_Type | noise is activated (switched on) for the given cell acc. to the previous configuration; while being active the configuration shall not be modified |
| Deactivate | Null_Type | deactivate noise for given cell |

EUTRA_VNG_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | EUTRA_VNG_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | ReqAspCommonPart_Type |  | CellId : as for the associated EUTRA cell RoutingInfo : None TimingInfo : Now ControlInfo : CnfFlag:=true; FollowOnFlag:=false RlcBearerRouting : omit |
| Request | EUTRA_VngConfigRequest_Type |  |  |

EUTRA_VNG_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | EUTRA_VNG_CTRL_CNF |  |  |
| Comment |  |  |  |
| Common | CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN (apart from EnquireTiming) => SS may set TimingInfo to "None" |
| Confirm | EUTRA_VngConfigConfirm_Type |  |  |

EUTRA_VNG_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | EUTRA_VNG_PORT |  |
| Comment | EUTRA PTC: Port for virtual noise generator |  |
| out | EUTRA_VNG_CTRL_REQ |  |
| in | EUTRA_VNG_CTRL_CNF |  |

# D.12	UTRAN_ASP_VirtualNoiseDefs

ASP definitions for virtual noise generation in UTRAN cells.
The noise is configurated for an already existing UTRAN cell.
NOTE: For the time being VNG is applicable for UTRAN FDD only as acc. to TS 36.304 clause 5.2.4.5 there is no quality based measurement for UTRAN TDD, GERAN or CDMA2000

UTRAN_ASP_VirtualNoiseDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| UTRAN_VngConfigConfirm_Type | Null_Type |  |

UTRAN_VngConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTRAN_VngConfigInfo_Type |  |  |
| Comment |  |  |  |
| IocLevel | integer |  | Ioc level; calculation is testcase specific |

UTRAN_VngConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UTRAN_VngConfigRequest_Type |  |
| Comment | configure/activate noise for a given cell; NOTE: it is assumed the the associated UTRAN cell has been created beforehand |  |
| Configure | UTRAN_VngConfigInfo_Type | configuration of the virtual noise generator; regardless of the power level the noise generator is off before it gets activated for this cell; in case the configuration needs to be changed during a test, the noise generator shall be deactivated for this cell |
| Activate | Null_Type | noise is activated (switched on) for the given cell acc. to the previous configuration; while being active the configuration shall not be modified |
| Deactivate | Null_Type | deactivate noise for given cell |

UTRAN_VNG_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTRAN_VNG_CTRL_REQ |  |  |
| Comment |  |  |  |
| CellId | integer |  | id of associated UTRAN cell |
| Request | UTRAN_VngConfigRequest_Type |  |  |

UTRAN_VNG_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTRAN_VNG_CTRL_CNF |  |  |
| Comment |  |  |  |
| CellId | integer |  | id of associated UTRAN cell |
| Confirm | UTRAN_VngConfigConfirm_Type |  |  |

UTRAN_VNG_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | UTRAN_VNG_PORT |  |
| Comment | UTRAN PTC: Port for virtual noise generator |  |
| out | UTRAN_VNG_CTRL_REQ |  |
| in | UTRAN_VNG_CTRL_CNF |  |

# D.13	WLAN_ASP_TypeDefs

## D.13.1	Common

Type definitions being common for WLAN AP and ePDG

WLAN_ReqAspControlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ReqAspControlInfo_Type |  |  |
| Comment |  |  |  |
| CnfFlag | boolean |  | true => SS shall send CNF: when the REQ is with no timing information (no activation time), SS shall send the confirmation when the configuration is done, i.e. when the test case may continue. Example: when there is a configuration followed by a send event it shall not be necessary to have a wait timer in between but the CNF triggers the send event or system Command. If there are other triggers e.g. like the UE sending a message, CnfFlag shall be set to false by the test case to avoid racing conditions with the CNF and the signalling message. When there is an activation time SS shall send the CNF after the configuration has been scheduled; that means SS shall not wait until the activation time has been expired. |
| FollowOnFlag | boolean |  | false => no further (related) information true: further related information will be sent to SS; currently this value is not foreseen to be used. |

WLAN_ErrorIndication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ErrorIndication_Type |  |  |
| Comment |  |  |  |
| System | integer |  | any other error: may be SS specific error code; this will not be evaluated by TTCN; e.g. an error shall be raised when the UE performs an out of sequence signalling |

WLAN_IndicationStatus_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_IndicationStatus_Type |  |
| Comment |  |  |
| Ok | Null_Type |  |
| Error | WLAN_ErrorIndication_Type |  |

WLAN_ConfirmationResult_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_ConfirmationResult_Type |  |
| Comment |  |  |
| Success | Null_Type |  |
| Error | integer | may contain SS specific error code; this will not be evaluated by TTCN |

## D.13.2	WLAN_AP

WLAN_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_WLAN_MaxNumberOfCells | integer | 3 | Only one cell is foreseen to be used. defined 3 just for completeness. |

WLAN_AP: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| WLAN_ChannelNumber_Type | integer (1..2047) | Channel Number of WLAN Channel 1 to 14 belong to 2.4 Ghz ISM band 131-138 belong to 3.6 Ghz ISM band 36,40,44,48,52,56,60,64,100,104,108,112,116,120,124,128,132,136,140,149,153,157,161&165 belong to 5Ghz ISM band Other channel numbers may be defined in future for other bands and are reserved until defined Channel number uniquely identifies the frequency |
| WLAN_BeaconRSSI_Type | integer (0..255) | WLAN Beacon RSSI signal strength in dBm is defined in IEEE 802.11-2012, Part 11 table 6.7 as integer -100..40, but defined in TS 36.331/25.331 in ASN.1 as integer(0..255). So actual beacon RSSI(dBm) = value -128. Note: Values 0..27 and 169..255 are not used or spares |
| WLAN_ChannelUtilizationType | integer (0..255) | As defined in in IEEE 802.11-2012, Part 11 clause 8.4.2.20 used in BSS load element |
| WLAN_AttenuationValue_Type | integer (0..127) |  |
| WLAN_TimeStamp_Type | O8_Type | TimeStamp field (TSF) timer in microseconds as defined in IEEE 802.11-2012 clause 8.4.1.10 |

WLAN_Band_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | WLAN_Band_Type |
| Comment |  |
| Band2_4Ghz | Band2_4Ghz = 2.4 Ghz ISM band |
| Band5Ghz | Band5Ghz = 5Ghz ISM band |
| Band3_6Ghz | Band3_6Ghz = 3.6 Ghz ISM band |

WLAN_RoutingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_RoutingInfo_Type |  |
| Comment |  |  |
| None | Null_Type | place holder for any possible extensions, currently only none defined |

WLAN_TimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_TimingInfo_Type |  |
| Comment |  |  |
| TimeStamp | WLAN_TimeStamp_Type |  |
| Now | Null_Type | to be used in REQ ASPs when there is no 'activation time' |
| None | Null_Type | only to be used in WLAN_AP_SYSTEM_CTRL_CNF |

WLAN_ReqAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ReqAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | WLAN_CellId_Type |  |  |
| RoutingInfo | WLAN_RoutingInfo_Type |  |  |
| TimingInfo | WLAN_TimingInfo_Type |  |  |
| ControlInfo | WLAN_ReqAspControlInfo_Type |  |  |

WLAN_IndAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_IndAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | WLAN_CellId_Type |  |  |
| RoutingInfo | WLAN_RoutingInfo_Type |  |  |
| TimingInfo | WLAN_TimingInfo_Type |  |  |
| Status | WLAN_IndicationStatus_Type |  |  |

WLAN_CnfAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_CnfAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | WLAN_CellId_Type |  |  |
| RoutingInfo | WLAN_RoutingInfo_Type |  |  |
| TimingInfo | WLAN_TimingInfo_Type |  |  |
| Result | WLAN_ConfirmationResult_Type |  | Similar definition as EUTRA |

WLAN_Attenuation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_Attenuation_Type |  |
| Comment | attenuation of the reference power |  |
| Value | WLAN_AttenuationValue_Type | cell power reference power reduced by the given attenuation (value is in dB) |
| Off | Null_Type | for non suitable off cell we specify an explicit "Off" value here |

WLAN_CellAttenuation_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_CellAttenuation_Type |  |  |
| Comment |  |  |  |
| CellId | WLAN_CellId_Type |  |  |
| Attenuation | WLAN_Attenuation_Type |  |  |

WLAN_CellAttenuationList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | WLAN_CellAttenuationList_Type |
| Comment |  |
| record length(1..tsc_WLAN_MaxNumberOfCells) of WLAN_CellAttenuation_Type |  |

WLAN_InitialCellPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_InitialCellPower_Type |  |  |
| Comment |  |  |  |
| BeaconRSSI | WLAN_BeaconRSSI_Type |  | maximum value of beacon RSSI a cell is initialised with this Beacon RSSI; its value is the upper bound of the Beacon RSSI during the test case |
| Attenuation | WLAN_Attenuation_Type |  | initial attenuation Cell is off |

WLAN_Revision_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | WLAN_Revision_Type |
| Comment |  |
| Rev_2012 | 802.11-2012, specified to be used in corespecs |
| Rev_n | 802.11n |

WLAN_ChannelBandwidth_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | WLAN_ChannelBandwidth_Type |
| Comment |  |
| bw5 | 5 Mhz |
| bw10 | 10 Mhz |
| bw20 | 20 Mhz |
| bw40 | 40 Mhz |
| bw80 | 80 Mhz |

WLAN_Identifiers_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_Identifiers_Type |  |  |
| Comment |  |  |  |
| SSID | octetstring length (1..32) | opt |  |
| BSSID | O6_Type | opt |  |
| HESSID | O6_Type | opt |  |

WLAN_Hotspot_Version_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | WLAN_Hotspot_Version_Type |
| Comment |  |
| None | no hotspot certification |
| Rev_2 | Refered in core spec; Hotspot Revision 2.0 release 2 |
| Rev_1 | Hotspot Revision 1.0 |

WLAN_AP_Parameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_AP_Parameters_Type |  |  |
| Comment | Parameters specific to WLAN AP |  |  |
| WLAN_Band | WLAN_Band_Type |  |  |
| ChannelNumber | WLAN_ChannelNumber_Type |  | WLAN band and channel number identify the frequency |
| ChannelBandwidth | WLAN_ChannelBandwidth_Type |  | Channel Bandwidth of 802.11 |
| WLAN_Identifiers | WLAN_Identifiers_Type |  | defined in 36.331, need to figure out how to import or define an equivalent |
| WLAN_Revision | WLAN_Revision_Type |  | Version of WLAN AP, core specs specify it to be 802.11-2012 |
| WLAN_Hotspot_Version | WLAN_Hotspot_Version_Type |  | Hotspot certification version, Core spec specify it is Hotspot 2.0 |

WLAN_AP_AddressInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_AP_AddressInfo_Type |  |  |
| Comment |  |  |  |
| UE_IPAddressIPv4 | charstring |  | IP Address of the UE in the WLAN |
| UE_IPAddressIPv6 | charstring |  | IP Address of the UE in the WLAN |
| DNS_ServerAddressIPv4 | charstring |  | IP address of the DNS server (WLAN) |
| DNS_ServerAddressIPv6 | charstring |  | IP address of the DNS server (WLAN) |

WLAN_AP_CellConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_AP_CellConfigInfo_Type |  |  |
| Comment |  |  |  |
| AP_AddressInfo | WLAN_AP_AddressInfo_Type |  | IP addresses to be assigned to the UE while it associates to the WLAN AP |
| WLAN_AP_Parameters | WLAN_AP_Parameters_Type |  | Parameters for WLAN AP |
| InitialCellPower | WLAN_InitialCellPower_Type |  | Power level parameters |

WLAN_AP_CellConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_AP_CellConfigRequest_Type |  |
| Comment |  |  |
| AddOrReconfigure | WLAN_AP_CellConfigInfo_Type | for cell configuration: CellId : identifier of the cell to be configured RoutingInfo : None TimingInfo : Now (for initial configuration and for reconfiguration in general) ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Release | Null_Type | to remove a cell completely - CellId : identifier of the cell to be released; Cell_NonSpecific, in case all cells shall be released RoutingInfo : None TimingInfo : Now ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

WLAN_OffloadSpecificParams_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_OffloadSpecificParams_Type |  |  |
| Comment | parameters that are used for offload to/from WLAN and controlled by Hotspot 2.0 version 2 |  |  |
| ChannelUtilisation | WLAN_ChannelUtilizationType | opt | omit means unchanged, if present AP transmits this parameter in BSS Load element IEEE 802.11-2012 clause 8.4.2.30 |
| BackHaulDLBandwidth | O4_Type | opt | Omit means unchanged, DL WAN bandwidth in kbps If present AP hotspot uses this value in WAN Metrics element Hotspot 2.0 release 2 clause 4.4 |
| BackHaulULBandwidth | O4_Type | opt | Omit means unchanged, UL WAN bandwidth in kbps If present AP hotspot uses this value in WAN Metrics element Hotspot 2.0 release 2 clause 4.4 |

WLAN_AP_SystemRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_AP_SystemRequest_Type |  |
| Comment |  |  |
| Cell | WLAN_AP_CellConfigRequest_Type | configure/release a cell |
| CellAttenuationList | WLAN_CellAttenuationList_Type | power attenuation for one or several cells, WLAN AP; all cells included in the list shall be changed at the same time; all cells in the list shall reach the new cell power within a maximum of 100ms (10 frames) |
| OffloadSpecificParams | WLAN_OffloadSpecificParams_Type | parameter used in offload decisions |

WLAN_AP_SystemConfirm_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_AP_SystemConfirm_Type |  |
| Comment | confirmations for system configuration; in general to be sent after the configuration has been done |  |
| Cell | Null_Type | (no further parameters from SS) |
| CellAttenuationList | Null_Type | (no further parameters from SS) NOTE 1: the confirmation shall be sent when all cells have changed power levels NOTE 2: for the CellId in the common ASP part the same rules are applied as for theWLAN SYSTEM REQ |
| OffloadSpecificParams | Null_Type | (no further parameters from SS) |

WLAN_AP_SystemCommand_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_AP_SystemCommand_Type |  |
| Comment |  |  |
| AssociateWLANAP | Null_Type | To prepare SS to expect WLAN association by the UE: SS shall indicate when the UE has already associated or if not already associated, indicate when the UE associates |
| DisassociateWLANAP | Null_Type | To prepare SS to expect WLAN disassociation by the UE: SS shall indicate when the UE has already disassociated or if not already disassociated, indicate when the UE disassociates |

WLAN_AP_SystemIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_AP_SystemIndication_Type |  |
| Comment |  |  |
| Error | Null_Type | Used by SS to indicate any error; the Actual Error types reported in ASP common part in WLAN_IndicationStatus_Type |
| AssociateWLANAP_Ind | Null_Type | UE has associated to WLAN as per IEEE 802.11-2012 clause 4.5.3.3 |
| DisassociateWLANAP_Ind | Null_Type | UE has disassociated from WLAN as per IEEE 802.11-2012 clause 4.5.3.5 |

WLAN_AP_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_AP_SYSTEM_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | WLAN_ReqAspCommonPart_Type |  | CellId : Cell Id of the WLAN AP RoutingInfo : None TimingInfo : Now ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Request | WLAN_AP_SystemRequest_Type |  |  |

WLAN_AP_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_AP_SYSTEM_CTRL_CNF |  |  |
| Comment |  |  |  |
| Common | WLAN_CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN => SS may set TimingInfo to "None" |
| Confirm | WLAN_AP_SystemConfirm_Type |  |  |

WLAN_AP_SYSTEM_CMD

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_AP_SYSTEM_CMD |  |  |
| Comment |  |  |  |
| Common | WLAN_ReqAspCommonPart_Type |  | CellId : Cell Id of the WLAN AP RoutingInfo : None TimingInfo : Now (in general) ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Command | WLAN_AP_SystemCommand_Type |  | WLAN AP System commands |

WLAN_AP_SYSTEM_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_AP_SYSTEM_IND |  |  |
| Comment |  |  |  |
| Common | WLAN_IndAspCommonPart_Type |  | The SS shall provide TimingInfo depending on the respective indication: |
| Indication | WLAN_AP_SystemIndication_Type |  | - Error TimingInfo: related to the error (if available) - WLAN offload Procedure completion The timing info corresponding to logical completion of the complete procedure includes completion of all sub protocols |

WLAN_AP_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | WLAN_AP_SYSTEM_PORT |  |
| Comment | WLAN PTC: Port for WLAN AP system configuration |  |
| out | WLAN_AP_SYSTEM_CTRL_REQ |  |
| in | WLAN_AP_SYSTEM_CTRL_CNF |  |

WLAN_AP_SYSCMD_IND_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | WLAN_AP_SYSCMD_IND_PORT |  |
| Comment | WLAN PTC: Port for WLAN AP system indications/Commands |  |
| out | WLAN_AP_SYSTEM_CMD |  |
| in | WLAN_AP_SYSTEM_IND |  |

## D.13.3	WLAN_ePDG

WLAN_PDN_AddressInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_PDN_AddressInfo_Type |  |  |
| Comment | Identifier of a specific IPsec tunnel |  |  |
| UE_IPAddressIPv4 | charstring |  | PDN IP Address of the UE in the 3GPP network |
| UE_IPAddressIPv6 | charstring |  | PDN IP Address of the UE in the 3GPP network |
| PCSCF_IPAddressIPv4 | charstring | opt | IP address of the P-CSCF (3GPP network); may be omitted in case of non-IMS PDN |
| PCSCF_IPAddressIPv6 | charstring | opt | IP address of the P-CSCF (3GPP network); may be omitted in case of non-IMS PDN |
| DNS_ServerAddressIPv4 | charstring | opt | IP address of DNS-Server (3GPP network);may be omitted in case of IMS PDN DNS-Server shall resolve the IP address of any other server (e.g. XCAP) located in the 3GPP network for this PDN |
| DNS_ServerAddressIPv6 | charstring | opt | IP address of DNS-Server (3GPP network);may be omitted in case of IMS PDN DNS-Server shall resolve the IP address of any other server (e.g. XCAP) located in the 3GPP network for this PDN |

WLAN_ePDG_RoutingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_ePDG_RoutingInfo_Type |  |
| Comment |  |  |
| None | Null_Type | shall be used in configuration requests and configuration confirmations which are not specific for a particular PDN (IPsec tunnel) |
| PdnId | PDN_Index_Type | shall be used in IPsec tunnel specific commands and indications |

WLAN_ePDG_ReqAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ePDG_ReqAspCommonPart_Type |  |  |
| Comment |  |  |  |
| RoutingInfo | WLAN_ePDG_RoutingInfo_Type |  | None" in case of configuration of ePDG;specific PdnId in case of commands sent to a particular IPsec tunnel (PDN) |
| ControlInfo | WLAN_ReqAspControlInfo_Type |  |  |

WLAN_ePDG_IndAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ePDG_IndAspCommonPart_Type |  |  |
| Comment |  |  |  |
| RoutingInfo | WLAN_ePDG_RoutingInfo_Type |  | PdnId as configured for the IPsec tunnel (PDN) for which an indication is raised |
| Status | WLAN_IndicationStatus_Type |  |  |

WLAN_ePDG_CnfAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ePDG_CnfAspCommonPart_Type |  |  |
| Comment |  |  |  |
| RoutingInfo | WLAN_ePDG_RoutingInfo_Type |  | As in the corresponding request |
| Result | WLAN_ConfirmationResult_Type |  |  |

WLAN_EAP_AKA_Cntrl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_EAP_AKA_Cntrl_Type |  |  |
| Comment | configurable parameters for EAP AKA |  |  |
| IMSI | hexstring |  | IMSI/USIM of UE |
| RAND | B128_Type |  | RAND to be used in initial key derivation |
| AuthK | B128_Type |  | Authentication secret key shared between AAA-server and USIM |

IPsec_ePDG_Cntrl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPsec_ePDG_Cntrl_Type |  |  |
| Comment | Parameters to be used for all IPsec security associations between UE and ePDG |  |  |
| IntegrityAlgorithm | IPsec_IntegrityAlgorithm_Type |  |  |
| CipheringAlgorithm | IPsec_CipheringAlgorithm_Type |  |  |

WLAN_ePDG_PDN_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ePDG_PDN_ConfigInfo_Type |  |  |
| Comment | Data set of PDN specific information being used for establishment of an IPsec tunnel |  |  |
| PdnId | PDN_Index_Type |  | PdnId to be used in commands and indications referring to a respective IPsec tunnel |
| PDN_AddressInfo | WLAN_PDN_AddressInfo_Type |  | Various IP Addresses to be used |
| IPsec_SA | IPsec_ePDG_Cntrl_Type |  | Parameters for ePDG secure tunnels |
| EAP_AKA_Cntrl | WLAN_EAP_AKA_Cntrl_Type |  | Parameters for EAP AKA control |

WLAN_ePDG_PDN_ConfigInfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | WLAN_ePDG_PDN_ConfigInfoList_Type |
| Comment |  |
| record of WLAN_ePDG_PDN_ConfigInfo_Type |  |

WLAN_ePDG_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ePDG_ConfigInfo_Type |  |  |
| Comment |  |  |  |
| ePDG_IPAddressIPv4 | charstring |  | IP address of the ePDG in the non-3GPP network |
| ePDG_IPAddressIPv6 | charstring |  | IP address of the ePDG in the non-3GPP network |
| IPsecTunnelList | WLAN_ePDG_PDN_ConfigInfoList_Type |  | In general there is just one IPsec tunnel configured (e.g. for IMS in case of IMSoWLAN); in case of IMSoWLAN XCAP scenarios there may be a second IPsec tunnel in which case the same working assumption is applied as for IMSoLTE according to which connectivity for IMS is established first and connectivity to XCAP is using the second IPsec tunnel => according to this working assumption the data set for the IMS IPsec tunnel shall be the first one and the data set for the XCAP IPsec tunnel shall be the second one. |

WLAN_ePDG_ConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_ePDG_ConfigRequest_Type |  |
| Comment | Configuration of ePDG and AAA-server emulation |  |
| AddOrReconfigure | WLAN_ePDG_ConfigInfo_Type | to setup configuration of ePDG and AAA-server emulation |
| Release | Null_Type | to release the ePDG/AAA-server emulation |

WLAN_ePDG_SystemRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_ePDG_SystemRequest_Type |  |
| Comment |  |  |
| ePDG | WLAN_ePDG_ConfigRequest_Type | setup/release configuration of ePDG and AAA-server emulation |

WLAN_ePDG_SystemConfirm_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_ePDG_SystemConfirm_Type |  |
| Comment | confirmations for system configuration; in general to be sent after the configuration has been done |  |
| ePDG | Null_Type | (no further parameters from SS) |

WLAN_ePDG_SystemCommand_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_ePDG_SystemCommand_Type |  |
| Comment |  |  |
| IpsecTunnelDiscCmd | Null_Type | To trigger SS for ePDG initiated tunnel disconnection procedure according to TS 24.302 clause 7.4.3.1 (and IETF RFC 5996 clause 1.4) |

EAP_AKA_Cmpl_Status_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | EAP_AKA_Cmpl_Status_Type |
| Comment |  |
| success |  |
| failure |  |

EAP_AKA_Cmpl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | EAP_AKA_Cmpl_Type |  |  |
| Comment |  |  |  |
| EAP_AKA_Status | EAP_AKA_Cmpl_Status_Type |  |  |
| XRES_Length | integer | opt | provide XRES_Length for success status |

IKE_AUTH_Req_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IKE_AUTH_Req_Type |  |  |
| Comment |  |  |  |
| AccessPointName | charstring | opt | Indication raised by the SS when the UE sends APN in IKE_AUTH_Req_Type. APN might be omitted when the UE establishes IPSec Tunnel for the Default APN acc. to 24.302 clause 7.2.2. |

IPsecTunnelEstabInd_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IPsecTunnelEstabInd_Type |  |
| Comment |  |  |
| IKE_SA_INIT_Ind | Null_Type | indication by the SS that the UE has started the attempt to establish an IPsec tunnel to the ePDG by sending an initial IKEv2 IKE_SA_INIT (step 1 of Figure 8.2.2-1 in 33.402) NOTE: the major purpose of this primitive is to get an early indication when the UE shall not attempt to establish the IPsec tunnel |
| IKE_AUTH_Req | IKE_AUTH_Req_Type | Indication by the SS that the UE has started the attempt to establish an IPsec tunnel with a specific PDN.NOTE: The purpose of this indication is to get the APN of the PDN requested by the UE. |
| EAP_AKA_Cmpl | EAP_AKA_Cmpl_Type | indication by the SS that the UE has provided all necessary parameters for EAP-AKA and the SS has sent an EAP Success/Failure message to UE according to step 11 of Figure 8.2.2-1 in 33.402 |
| IPSecTunnelEstabCmpl | Null_Type | indication by the SS that IPsec tunnel is established: SS shall indicate this immediately after it has sent out the final IKE_AUTH response (containing the CFG_REPLY) at step 15 of Figure 8.2.2-1 in 33.402 NOTE: Typically the UE gets all relevant IP addresses of the 3GPP network in the configuration payload (CFG_REPLY) |

IPsecTunnelDiscInd_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IPsecTunnelDiscInd_Type |  |
| Comment |  |  |
| IPSecTunnelRelCmpl | Null_Type | Indication raised by the SS when the SS has received INFORMATIONAL response from the UE according to TS 24.302 clause 7.2.4.2 (ePDG initiated tunnel disconnection) or SS has sent INFORMATIONAL response according to TS 24.302 clause 7.4.3.2 (UE initiated tunnel disconnection) |

WLAN_ePDG_SystemIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | WLAN_ePDG_SystemIndication_Type |  |
| Comment |  |  |
| Error | Null_Type | Used by SS to indicate any error; the Actual Error types reported in ASP common part in WLAN_IndicationStatus_Type |
| IPsecTunnelEstabInd | IPsecTunnelEstabInd_Type | To indicate progress for IPsec tunnel establishment |
| IPsecTunnelDiscInd | IPsecTunnelDiscInd_Type | To indicate progress for release of the IPsec tunnel |

WLAN_ePDG_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ePDG_SYSTEM_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | WLAN_ePDG_ReqAspCommonPart_Type |  | RoutingInfo : None ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Request | WLAN_ePDG_SystemRequest_Type |  |  |

WLAN_ePDG_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ePDG_SYSTEM_CTRL_CNF |  |  |
| Comment |  |  |  |
| Common | WLAN_ePDG_CnfAspCommonPart_Type |  | RoutingInfo : As in the corresponding request |
| Confirm | WLAN_ePDG_SystemConfirm_Type |  |  |

WLAN_ePDG_SYSTEM_CMD

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ePDG_SYSTEM_CMD |  |  |
| Comment |  |  |  |
| Common | WLAN_ePDG_ReqAspCommonPart_Type |  | RoutingInfo: PdnId addressing the IPsec tunnel (PDN) for which the command shall be applied ControlInfo : CnfFlag:=false; FollowOnFlag:=false |
| Command | WLAN_ePDG_SystemCommand_Type |  | WLAN ePDG System commands |

WLAN_ePDG_SYSTEM_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | WLAN_ePDG_SYSTEM_IND |  |  |
| Comment |  |  |  |
| Common | WLAN_ePDG_IndAspCommonPart_Type |  | RoutingInfo : PdnId of the IPsec tunnel (PDN) the indication is coming from. Status : Ok ¦ Error |
| Indication | WLAN_ePDG_SystemIndication_Type |  |  |

WLAN_ePDG_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | WLAN_ePDG_SYSTEM_PORT |  |
| Comment | WLAN PTC: Port for WLAN ePDG/AAA-Server system configuration |  |
| out | WLAN_ePDG_SYSTEM_CTRL_REQ |  |
| in | WLAN_ePDG_SYSTEM_CTRL_CNF |  |

WLAN_ePDG_SYSCMD_IND_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | WLAN_ePDG_SYSCMD_IND_PORT |  |
| Comment | WLAN PTC: Port for WLAN ePDG/AAA-Server emulation system indications/Commands |  |
| out | WLAN_ePDG_SYSTEM_CMD |  |
| in | WLAN_ePDG_SYSTEM_IND |  |

# D.14	SideLinkUE_ASP_TypeDefs

## D.14.1	SideLinkUE_Data

SideLinkUE_Data: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SFN_DFN_SubframeOffset_Type | integer | Subframe offset relative to the absolute timing information given in the common part of the ASP |

SL_DATA_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_DATA_REQ |  |  |
| Comment | Common ASP to send PDUs on PC5 channels SL-DCH or SLRB |  |  |
| Common | SL_ReqAspCommonPart_Type |  | SS_UE_Id : identifier of the SL UE * RoutingInfo : SL-DCH or SLRB identity * TimingInfo : For Direct Discovery see description in DiscPeriodResourceList_Type; For communication and V2X: SFN/Subframe or DFN/DirectSubframe to use for transmission of the first data PDUs. * ControlInfo : CnfFlag:=false; FollowOnFlag:=false |
| U_Plane | SL_U_PlaneTx_Type |  |  |

SL_DATA_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_DATA_IND |  |  |
| Comment | Common ASP to receive PDUs from PC5 channels SL-DCH or SLRB |  |  |
| Common | SL_IndAspCommonPart_Type |  | SS_UE_Id : identifier of the SL UE RoutingInfo : SL-DCH or SLRB identity TimingInfo : SFN/Subframe or DFN/DirectSubframe when message has been received |
| U_Plane | SL_U_PlaneRx_Type |  |  |

SL_U_PlaneTx_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_U_PlaneTx_Type |  |  |
| Comment |  |  |  |
| U_Plane | SL_U_Plane_Type |  |  |
| SL_DataScheduling | SL_DataScheduling_Type | opt | Applied on SL-DCH and PSCCH/PSSCH |

SL_U_PlaneRx_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_U_PlaneRx_Type |  |  |
| Comment | SS-UE shall send to TTCN all PDCP data packets received on PSSCH including all retransmissions |  |  |
| U_Plane | SL_U_Plane_Type |  |  |
| FreqDomainSchedul | FreqDomainSchedulExplicit_Type | opt | Applied on SL-RB and SL-DCH |
| SCIformat | SCIformat_Type | opt | Applied when V2X_SCI_Reporting is enabled |

SL_U_Plane_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_U_Plane_Type |  |
| Comment |  |  |
| Discovery | SL_DCH_Message_Type | SL-DCH message |
| PdcpPdu | PDCP_PDUList_Type | SS configuration: PDCP no header removal - In Rx only one PDCP PDU is expected |
| PdcpSdu | PDCP_SDUList_Type | SS configuration: PDCP header removal - In Rx only one PDCP SDU is expected |

SL_DCH_Message_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_DCH_Message_Type |  |
| Comment | SL-DCH message |  |
| pC5_DISCOVERY | PC5_DISCOVERY |  |

SL_DataScheduling_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_DataScheduling_Type |  |
| Comment |  |  |
| DiscPeriodResource | DiscPeriodResourceList_Type | Applied for SS-UE transmission on SL-DCH/PSDCH |
| CommPeriodResource | CommPeriodResourceList_Type | Applied for SS-UE transmission on PSCCH for direct communication |
| V2X_Resource | V2X_SchedulingList_Type | Applied for SS-UE transmission on PSCCH/PSSCH for V2X |

DiscPeriodResourceList_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DiscPeriodResourceList_Type |  |  |
| Comment | Parameters for transmission of the PC5_DISCOVERY message during NumberOfTransmissionPeriod discovery Periods TimingInfo in Common indicates the SFN/subframe of the first discovery period |  |  |
| NumberOfTransmissionPeriod | integer |  | Number of consecutive discovery period |
| N_PDSCH | integer |  | Resource number in the range 0..(Nt*Nf-1) (see TS 36.213 cl 14.3.1) Multiple PC5_DISCOVERY messages can be transmitted within a discovery period, different SL_DATA_REQ and N_PDSCH values will be provided |

CommPeriodResourceList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | CommPeriodResourceList_Type |
| Comment | One entry in the list per STCH packets provided in the U_Plane field One entry per communication period |
| record of CommPeriodResource_Type |  |

CommPeriodResource_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CommPeriodResource_Type |  |  |
| Comment | Parameters for transmission of the STCH packets TimingInfo in Common indicates the SFN/subframe of the first communication period |  |  |
| N_PSCCH | integer |  | PSCCH Resource number (see TS 36.213 cl 14.2.1.1) Multiple STCH packets can be transmitted within a communication period, different SL_DATA_REQ and N_PSCCH values will be provided |

SCIformat_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SCIformat_Type |  |
| Comment |  |  |
| SciFormat1 | SciFormat1_Type |  |

SciFormat1_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SciFormat1_Type |  |  |
| Comment | SCI format 1 acc to TS 36.212 clause 5.4.3.1.2 |  |  |
| Priority | integer |  |  |
| ResourceReservation | integer |  |  |
| FrequencyResourceLocation | integer |  | Frequency resource location of initial transmission and retransmission |
| TimeGap | integer |  | Time gap between initial transmission and retransmission |
| MCS | integer |  | Modulation and coding scheme |
| RetransmissionIndex | integer |  |  |

V2X_SchedulingList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | V2X_SchedulingList_Type |
| Comment | Subframe offset relative to the absolute timing information given in the common part of the ASP. Applied from the second data PDU. |
| record of SFN_DFN_SubframeOffset_Type |  |

SL_DATA_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | SL_DATA_PORT |  |
| Comment |  |  |
| out | SL_DATA_REQ |  |
| in | SL_DATA_IND |  |

## D.14.2	SideLinkUE_Configuration

### D.14.2.1	SL_Routing_Timing

SL_Routing_Timing: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SLRB_Identity_Type | integer |  |

SL_RoutingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_RoutingInfo_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| ChannelRadioBearerId | SL_ChannelRadioBearerId_Type |  |

SL_ChannelRadioBearerId_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_ChannelRadioBearerId_Type |  |
| Comment |  |  |
| SLDCH | Null_Type | Discovery SL-DCH |
| SLRB | SLRB_Identity_Type | Communication SL-RB |

SL_TimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_TimingInfo_Type |  |
| Comment |  |  |
| InCoverageTiming | TimingInfo_Type | SFN/subframe |
| OutOfCoverageTiming | TimingInfo_Type | Out of coverage timing info: - SFN is equivalent to DFN. - HSFN is not relevant on PC5 interface. It is not checked in IND and CNF ASPs and it set to Any in REQ ASPs. |
| UTC_SynchronisedTiming | TimingInfo_Type | Out of coverage timing info: : - SFN is equivalent to DFN. - HSFN is not relevant on PC5 interface. It is not checked in IND and CNF ASPs and it set to Any in REQ ASPs. |

### D.14.2.2	SL_SystemRequestAsp

#### D.14.2.2.1	SL_RequestAspCommon_Part

SL_RequestAspCommon_Part: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SL_Bandwidth_Type | MasterInformationBlock_SL.sl_Bandwidth_r12 |  |
| V2X_OffsetDFN_Type | SL_V2X_ConfigCommon_r14.offsetDFN_r14 |  |

SL_ReqAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_ReqAspCommonPart_Type |  |  |
| Comment |  |  |  |
| SS_UE_Id | SS_UE_Id_Type |  | SS-UE ID |
| RoutingInfo | SL_RoutingInfo_Type |  | Channel or RB Id |
| TimingInfo | SL_TimingInfo_Type |  | In coverage or Out of coverage timing |
| ControlInfo | ReqAspControlInfo_Type |  |  |

SL_SystemRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_SystemRequest_Type |  |
| Comment |  |  |
| SS_UE_Config | SS_UE_ConfigRequest_Type | Configure/release a SS-UE |
| EnquireTiming | Null_Type | Get SFN/subframe number or DFN/Direct subframe time for this SS-UE |
| L1MacIndCtrl | SL_L1Mac_IndicationControl_Type | To configure SS_UE to generate indications for L1/MAC events |
| SL_Security | SL_Security_Type | StartRestart/Release of SLRB security |
| CongestionConfig | CongestionConfig_Type | Configure V2X congestion |

SL_L1Mac_IndicationControl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_L1Mac_IndicationControl_Type |  |  |
| Comment | NOTE: Initially all indications are disabled in SS (i.e. it shall not be necessary in 'normal' test cases to use this primitive but only if a specific indication is needed); omit means indication mode is not changed |  |  |
| DiscSync | IndicationAndControlMode_Type | opt | To enable/disable reporting of start and stop of reception of Discovery SLSS as configured. NOTE: this is applicable when SS_UE is configured with RxDiscSyncRef SS reports an indication every time when SLSS reception from UE has toggled in the subframes configured for SLSS reception |
| CommSLSS_MIBSL | IndicationAndControlMode_Type | opt | To enable/disable reporting of start and stop of reception of communication SLSS and MIB-SL as configured. NOTE: this is applicable when SS_UE is configured with CommRxSyncRef SS reports an indication every time when SLSS/MIB-SL reception from UE has toggled in the subframes configured for SLSS/MIB-SL reception |
| V2X_CommSLSS_MIBSL | IndicationAndControlMode_Type | opt | To enable/disable reporting of start and stop of reception of V2X communication SLSS and MIB-SL-V2X as configured. NOTE: this is applicable when SS_UE is configured with V2X_CommRxSyncRef SS reports an indication every time when SLSS/MIB-SL-V2X reception from UE has toggled in the subframes configured for SLSS/MIB-SL-V2X reception |
| V2X_SCI_Reporting | IndicationAndControlMode_Type | opt | To enable/disable reporting of SCI in SL_DATA_IND upon reception V2X communication. NOTE: this is applicable when SS-UE is configured in reception with V2X_CommRxConfig SS-UE includes the SCI in the DATA-IND for every STCH data received |

SS_UE_ConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SS_UE_ConfigRequest_Type |  |
| Comment |  |  |
| AddOrReconfigure | SS_UE_ConfigInfo_Type | For one SS-UE configuration: SS_UE_Id : identifier of the SS-UE to be configured RoutingInfo : None TimingInfo : Now (for initial configuration) ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Release | Null_Type | To remove all SS-UEs completely SS_UE_Id : ss_UE_NonSpecific RoutingInfo : None TimingInfo : Now ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

SS_UE_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SS_UE_ConfigInfo_Type |  |  |
| Comment |  |  |  |
| SS_UE_CoverageSynchronisation | SS_UE_CoverageSynchronisation_Type | opt | Mandatory in the first configuration - omit means no change InCoverage for direct discovery InCoverage or OutOfCoverage for direct communication InCoverage, OutOfCoverage or GNSS/UTC synchronised for V2X |
| Power | SS_UE_Power_Type | opt | Reference power for the RS of each antenna in Tx Initially configured as switched ON |
| DiscConfig | DiscConfig_Type | opt |  |
| CommConfig | CommConfig_Type | opt |  |
| V2X_CommConfig | V2X_CommConfig_Type | opt |  |

SS_UE_Power_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SS_UE_Power_Type |  |  |
| Comment |  |  |  |
| MaxReferencePower | AbsoluteCellPower_Type | opt | Maximum value of SS-UE reference power (in dBm/15kHz as per TS 36.508, clause 4.3.4.1); SS-UE is initialised with this reference power; its value can't be reconfigured during test execution, attenuation is used to change SS-UE power level its value is the upper bound of the SS-UE power during the test case |
| Attenuation | Attenuation_Type |  | Initial attenuation: ON |

InCoverageConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | InCoverageConfig_Type |  |  |
| Comment |  |  |  |
| UL_CarrierFreq | ARFCN_ValueEUTRA_r9 |  |  |
| UL_bandwidth | Ul_Bandwidth_Type |  |  |
| TDD_Config | TDD_Config | opt | omitted for FDD and for V2X frame structure type 1 |
| CellTimingInfo | CellTimingInfo_Type |  |  |

OutOfCoverageConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | OutOfCoverageConfig_Type |  |  |
| Comment | To configure SS-UE in Out of coverage |  |  |
| CarrierFreq_r12 | ARFCN_ValueEUTRA_r9 |  |  |
| SL_bandwidth_r12 | SL_Bandwidth_Type |  |  |
| TDD_ConfigSL_r12 | TDD_ConfigSL_r12 |  |  |
| SS_UE_TimingInfo | SS_UE_TimingInfo_Type |  |  |

UTC_SynchronisedConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UTC_SynchronisedConfig_Type |  |  |
| Comment | To configure V2X SS-UE in GNSS/UTC synchronised |  |  |
| CarrierFreq_r12 | ARFCN_ValueEUTRA_r9 |  |  |
| SL_bandwidth_r12 | SL_Bandwidth_Type |  |  |
| TDD_ConfigSL_r12 | TDD_ConfigSL_r12 |  |  |
| OffsetDFN_r14 | V2X_OffsetDFN_Type | opt |  |

SS_UE_TimingInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SS_UE_TimingInfo_Type |  |  |
| Comment | SS_UE Timing |  |  |
| DSubframeOffset | integer (0..307199) |  | frame duration Tf = 307200 * Ts = 10ms; System Time Unit Ts = 1/(15000 * 2048) |
| DfnOffset | integer (0..1023) |  | (assuming 10 bit DFN) |

SS_UE_CoverageSynchronisation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SS_UE_CoverageSynchronisation_Type |  |
| Comment | For Discovery only InCoverage is applied |  |
| InCoverage | InCoverageConfig_Type | In coverage SS-UE shall use Band/Bandwidth/UL frequency/Cell timing/Frame structure/CP of the associated EUTRA cell |
| OutOfCoverage | OutOfCoverageConfig_Type |  |
| UTC_Synchronised | UTC_SynchronisedConfig_Type | V2X SS-UE configured in GNSS/UTC synchronised mode |

PrimarySideLinkSyncSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PrimarySideLinkSyncSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | SideLinkChannelPower_Type | opt | Power for PSSS relative to the SS-UE power level - value in dB |

SecondarySideLinkSyncSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SecondarySideLinkSyncSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | SideLinkChannelPower_Type | opt | Power for SSSS relative to the SS-UE power level - value in dB |

#### D.14.2.2.2	Discovery_Specific

Discovery_Specific: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| DiscPeriod_r12_Type | SL_DiscResourcePool_r12.discPeriod_r12 |  |
| DiscNumRetx_r12_Type | SL_DiscResourcePool_r12.numRetx_r12 |  |
| DiscNumRepetition_r12_Type | SL_DiscResourcePool_r12.numRepetition_r12 |  |
| SideLinkChannelPower_Type | integer | see TS 36.213, clause 14 |

DiscConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DiscConfig_Type |  |  |
| Comment | Acc. to 36.321: HARQ configured with fixed TBS=232 - MAC configured in TM PC5_DISCOVERY msg = 232 bits, transmitted in one MAC TM PDU. The PSDCH is transmitted on two contiguous physical resource blocks per slot. The SideLink process is configured with a maximum number of HARQ retransmissions by RRC: numRetx. |  |  |
| SL_DCH_Tx | SL_DCH_TxConfig_Type | opt | Corresponding to UE Rx resource pool |
| SL_DCH_Rx | SL_DCH_RxConfig_Type | opt | Corresponding to UE Tx resource allocation |

SL_DCH_TxConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_DCH_TxConfig_Type |  |
| Comment |  |  |
| AddOrReconfigure | SL_DCH_TxConfigInfo_Type |  |
| Release | Null_Type |  |

SL_DCH_TxConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_DCH_TxConfigInfo_Type |  |  |
| Comment |  |  |  |
| SS_UE_Tx | DiscResourcePool_Type | opt |  |
| DiscTxSyncRef | DiscTxSyncRefConfig_Type | opt | Configure SS_UE to transmit Sync SLSS with PC5_DISCOVERY Tx No prose yet - See 36.331 clause 5.10.5 Note 4 |
| Psdch | PsdchConfig_Type | opt |  |

SL_DCH_RxConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_DCH_RxConfig_Type |  |
| Comment |  |  |
| AddOrReconfigure | SL_DCH_RxConfigInfo_Type |  |
| Release | Null_Type |  |

SL_DCH_RxConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_DCH_RxConfigInfo_Type |  |  |
| Comment |  |  |  |
| SS_UE_Rx | DiscResourcePoolList_Type | opt |  |
| DiscRxSyncRef | DiscRxSyncRefConfig_Type | opt | For SS_UE to detect SLSS transmitted by UE under test It is assumed that only a single Rx resource pool will be specified for the receipt of SLSS by SS-UE |

DiscResourcePool_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DiscResourcePool_Type |  |  |
| Comment | All fields are mandatory for initial configuration; omit means "keep as it is" |  |  |
| cp_Len_r12 | SL_CP_Len_r12 | opt |  |
| discPeriod_r12 | DiscPeriod_r12_Type | opt |  |
| numRetx_r12 | DiscNumRetx_r12_Type | opt |  |
| numRepetition_r12 | DiscNumRepetition_r12_Type | opt |  |
| tf_ResourceConfig_r12 | SL_TF_ResourceConfig_r12 | opt |  |
| discTF_IndexList_r12b | SL_TF_IndexPairList_r12b | opt | Applied in SL-DCH Rx when UE is configured in transmission mode 2B To ensure SS-UE and UE are timing aligned, the indexes are provided to SS-UE using the same activation as the one used to send the signalling reconfiguration message to UE |

DiscResourcePoolList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | DiscResourcePoolList_Type |
| Comment |  |
| record length(1..maxSL_TxPool_r12) of DiscResourcePool_Type |  |

DiscTxSyncRefConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DiscTxSyncRefConfig_Type |  |
| Comment | To start/reconfigure or stop transmitting SLSS (irrespective of any measurement) |  |
| AddOrReconfigure | DiscTxSyncRefConfigInfo_Type |  |
| Release | Null_Type |  |

DiscRxSyncRefConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DiscRxSyncRefConfig_Type |  |
| Comment |  |  |
| AddOrReconfigure | DiscSyncConfigParam_Type |  |
| Release | Null_Type |  |

DiscTxSyncRefConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DiscTxSyncRefConfigInfo_Type |  |  |
| Comment | All fields are mandatory for initial configuration; omit means "keep as it is" |  |  |
| SyncConfig | DiscSyncConfigParam_Type | opt |  |
| Psss | PrimarySideLinkSyncSignal_Type | opt |  |
| Ssss | SecondarySideLinkSyncSignal_Type | opt |  |

DiscSyncConfigParam_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DiscSyncConfigParam_Type |  |  |
| Comment | All fields are mandatory for initial configuration; omit means "keep as it is" |  |  |
| SyncCP_Len_r12 | SL_CP_Len_r12 | opt |  |
| SyncOffsetIndicator_r12 | SL_OffsetIndicatorSync_r12 | opt |  |
| Slssid_r12 | SLSSID_r12 | opt |  |

PsdchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PsdchConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | SideLinkChannelPower_Type | opt | Power for PSDCH relative to the SS-UE power level - value in dB |

#### D.14.2.2.3	Communication_Specific

SideLinkUE_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_MaxNumberSL_STCH | integer | 1 | Acc to 36.508 cl 4.2.2.3.1.1 |

Communication_Specific: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SLSSID_Type | integer (0..335) | Covering values for In and Out of coverage |
| Mcs_Type | SL_CommConfig_r12.commTxResources_r12.setup.scheduled_r12.mcs_r12 |  |
| SL_Rohc_Profiles_Type | SL_PreconfigGeneral_r12.rohc_Profiles_r12 |  |

CommConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CommConfig_Type |  |  |
| Comment |  |  |  |
| CommSyncRef | CommSyncConfig_Type | opt |  |
| CommRxConfig | CommRxConfig_Type | opt |  |
| CommTxConfig | CommTxConfig_Type | opt |  |

CommSyncConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CommSyncConfig_Type |  |  |
| Comment |  |  |  |
| CommTxSyncRef | CommTxSyncRefConfig_Type | opt |  |
| CommRxSyncRef | CommRxSyncRefConfig_Type | opt |  |
| SynchronisationSource | SL_SyncRef_Type | opt | Applied when OoC - When In coverage: omit ueSyncRef UE under test is SyncRef UE SS_UE shall synchronise from received SLSS/MIB-SL configured in CommRxSyncRef UE shall be transmitting SL-SS when SS_UE is switched on sS_UE_SyncRef SS_UE is SyncRef UE SS_UE shall initiate DFN/DirectSubrame and transmit SLSS/MIB-SL acc to CommTxSyncRef |

CommTxSyncRefConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CommTxSyncRefConfig_Type |  |
| Comment | To start/reconfigure or stop transmitting SLSS/MIB-SL (irrespective of any measurement) |  |
| AddOrReconfigure | CommTxSyncRef_Type | Add/re-configure SLSS or SBCCH |
| Release | Null_Type | Stop SLSS and SBCCH transmission |

CommRxSyncRefConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CommRxSyncRefConfig_Type |  |
| Comment |  |  |
| AddOrReconfigure | CommRxSyncRef_Type | Add/re-configure reception of SLSS or SBCCH |
| Release | Null_Type | Release/stop reception of SLSS and SBCCH transmission |

CommRxSyncRef_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CommRxSyncRef_Type |  |  |
| Comment |  |  |  |
| SyncCP_Len_r12 | SL_CP_Len_r12 |  |  |
| Slssid | SLSSID_Type | opt | Omit means any value is expected |
| SyncOffsetIndicator1_r12 | SL_OffsetIndicatorSync_r12 |  |  |
| SyncOffsetIndicator2_r12 | SL_OffsetIndicatorSync_r12 | opt | When present both SyncOffsetIndicator1_r12 and SyncOffsetIndicator2_r12 shall ne monitored |

CommTxSyncRef_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CommTxSyncRef_Type |  |  |
| Comment | SBCCH - RLC TM - MAC TM SS will perform padding, if required due to the granularity of the TF signalling, as defined in 36.331 cl 8.5. |  |  |
| TxSyncConfig | CommTxSyncRefParam_Type | opt |  |
| MIB_SL | MasterInformationBlock_SL | opt | DFN and Direct subframe set to a dummy value - to be set by SS-UE |
| Psss | PrimarySideLinkSyncSignal_Type | opt |  |
| Ssss | SecondarySideLinkSyncSignal_Type | opt |  |
| Psbch | PsbchConfig_Type | opt | The transmit power of PSBCH is same as the transmit power of primary sidelink synchronisation signal |

CommTxSyncRefParam_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CommTxSyncRefParam_Type |  |  |
| Comment |  |  |  |
| SyncCP_Len_r12 | SL_CP_Len_r12 |  |  |
| Slssid | SLSSID_Type | opt |  |
| SyncOffsetIndicator1_r12 | SL_OffsetIndicatorSync_r12 |  |  |

SL_SyncRef_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | SL_SyncRef_Type |
| Comment |  |
| sS_UE_SyncRef |  |
| ueSyncRef |  |

CommRxConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CommRxConfig_Type |  |
| Comment | SS-UE ID : identifier of the simulated UE being configured RoutingInfo : none TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |  |
| AddOrReconfigure | CommRx_ConfigInfo_Type | Add/re-configure SideLink PSCCH/PSSCH/SL-SCH/STCH/RLC/PDCP |
| Release | Null_Type | Release SideLink PSCCH/PSSCH/SL-SCH/STCH/RLC/PDCP |

CommTxConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CommTxConfig_Type |  |
| Comment | SS-UE ID : identifier of the simulated UE being configured RoutingInfo : none TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |  |
| AddOrReconfigure | CommTx_ConfigInfo_Type | Add/re-configure SideLink PSCCH/PSSCH/SL-SCH/STCH/RLC/PDCP |
| Release | Null_Type | Release SideLink PSCCH/PSSCH/SL-SCH/STCH/RLC/PDCP |

CommRx_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CommRx_ConfigInfo_Type |  |  |
| Comment | It is assumed that one SSUE layer 2 ID/one SLRB needs to be handled SS-UE is configured with a ProSe layer 2 identity and with the UE ProSe layer 2 identity. SS-UE shall send to TTCN only the data packets carrying the correct layer 2 destination and source identities |  |  |
| ResourcePoolList | CommResourcePoolList_Type | opt | Configure subframes reserved for PSSCH and PSCCH Present for first configuration omit means no change |
| SL_SCH_Config | SL_SCH_Config_Type | opt | SL-SCH/MAC configuration Present for first configuration omit means no change |
| STCH_ConfigList | STCH_ConfigList_Type | opt |  |

CommResourcePoolList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | CommResourcePoolList_Type |
| Comment |  |
| record length(1..maxSL_TxPool_r12) of CommResourcePool_Type |  |

CommResourcePool_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CommResourcePool_Type |  |  |
| Comment |  |  |  |
| sc_CP_Len_r12 | SL_CP_Len_r12 | opt |  |
| sc_Period_r12 | SL_PeriodComm_r12 | opt |  |
| sc_TF_ResourceConfig_r12 | SL_TF_ResourceConfig_r12 | opt |  |
| data_CP_Len_r12 | SL_CP_Len_r12 | opt |  |
| dataHoppingConfig_r12 | SL_HoppingConfigComm_r12 | opt |  |
| data_TF_ResourceConfig_r12 | SL_TF_ResourceConfig_r12 | opt |  |
| trpt_Subset_r12 | SL_TRPT_Subset_r12 | opt |  |

CommTx_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CommTx_ConfigInfo_Type |  |  |
| Comment | All fields are optional: present for first configuration omit means no change SS-UE is configured with a ProSe layer 2 identity and with the UE ProSe layer 2 identity. The SS-UE shall construct the MAC packets using these identities |  |  |
| ResourcePoolList | CommResourcePoolList_Type | opt | Configure pool of resources for PSSCH and PSCCH |
| Mcs_r12 | Mcs_Type | opt | Indicates the Modulation and Coding Scheme as defined in TS 36.212 [23, 14.2.1] |
| SL_SCH_Config | SL_SCH_Config_Type | opt | SL-SCH/MAC configuration |
| STCH_ConfigList | STCH_ConfigList_Type | opt | RLC PDCP |
| SL_Sheduling | SSUE_SciConfig_Type | opt | SCI fixed grant used |
| PscchConfig | PscchConfig_Type | opt |  |
| PsschConfig | PsschConfig_Type | opt |  |

SSUE_SciConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SSUE_SciConfig_Type |  |  |
| Comment |  |  |  |
| Imcs | ImcsValue_Type |  | Modulation and coding scheme - 5 bits as defined in section 14.1.1 of [3] |
| SL_Sheduling | SL_Sci0Config_Type |  | SCI fixed grant used |

SL_SCH_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_SCH_Config_Type |  |  |
| Comment | All fields are mandatory for initial configuration; omit means "keep as it is" |  |  |
| SS_UE_Layer2Id | B24_Type | opt |  |

STCH_ConfigList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | STCH_ConfigList_Type |
| Comment |  |
| record length(1..tsc_MaxNumberSL_STCH) of STCH_Config_Type |  |

STCH_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | STCH_Config_Type |  |  |
| Comment | All fields are mandatory for initial configuration; omit means "keep as it is" |  |  |
| SLRB | SLRB_Identity_Type | opt |  |
| SL_RBIDConfig | SLRB_Config_Type | opt |  |

SLRB_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SLRB_Config_Type |  |
| Comment |  |  |
| AddOrReconfigure | SLRB_ConfigInfo_Type | Add/re-configure SideLink RB |
| Release | Null_Type | Release SideLink RB |

SLRB_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SLRB_ConfigInfo_Type |  |  |
| Comment | All fields are mandatory for initial configuration; omit means "keep as it is" |  |  |
| Rlc | STCH_RLC_Type | opt |  |
| Pdcp | SL_PDCP_ConfigInfo_Type | opt |  |

STCH_RLC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | STCH_RLC_Type |  |  |
| Comment | One RLC entity per Tx peer UE Uni-directional UM RLC sn-fieldLength = 5 LCG = 3 SN is set to the first received UMD PDU in Rx UM_Window size = 0 |  |  |
| Layer2Id | B24_Type | opt | Layer-2 Id of RLC entity: SourceLayer2-Id in Rx - DestinationLayer2Id in Tx |
| LCID | B5_Type | opt | Applied in Tx, in Rx SS shall use the value received in the first STCH data PDU |

SL_PDCP_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_PDCP_ConfigInfo_Type |  |  |
| Comment |  |  |  |
| SlRb | PDCP_SLRB_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | PDCP_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

PDCP_SLRB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_SLRB_Type |  |  |
| Comment | PDCP SN length = 16b discard timer = undefined maxCID=15 |  |  |
| headerCompression | SL_Rohc_Profiles_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

PsbchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PsbchConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | SideLinkChannelPower_Type | opt | Power for PSBCH relative to the SS-UE power level - value in dB |

PsschConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PsschConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | SideLinkChannelPower_Type | opt | Power for PSSCH relative to the SS-UE power level - value in dB |

PscchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PscchConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | SideLinkChannelPower_Type | opt | Power for PSCCH relative to the SS-UE power level - value in dB |

#### D.14.2.2.4	SL_Security

SL_Security_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_Security_Type |  |
| Comment | When not configured ciphering is considered as off |  |
| StartRestart | SL_CipheringStartRestart_Type | Information to start/restart SL security protection in the PDCP SLRB |
| Release | Null_Type | To release SL security protection in the PDCP |

SL_CipheringStartRestart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_CipheringStartRestart_Type |  |  |
| Comment | Acc to 33.303 NOTE: Group Member Id (i.e. the Layer 2 source address of the sending UE) and Group Id (i.e. the destination Layer 2 identity of the group) are configured in MAC and RLC (SL_SCH_Config_Type and STCH_RLC_Type). Expiry time to be handled by TTCN |  |  |
| GroupId | B24_Type |  | ProSe Layer-2 Group Id |
| AlgorithmInfo | B3_Type |  | Security algorithm acc to 33.303 Table E.5.2.2.7-1 |
| PGK | B256_Type |  |  |
| PGK_Id | B64_Type |  | PGK identity |
| PTK_Id | B16_Type |  | For SS_UE Tx - For PTK derivation at SS-UE side. The value may be different from UE PTK-Id |
| Counter | B16_Type |  | For SS_UE Tx - For PTK derivation |

#### D.14.2.2.5	V2X_Specific

V2X_Specific: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SizeSubchannel_r14_Type | SL_CommResourcePoolV2X_r14.sizeSubchannel_r14 |  |
| NumSubchannel_r14_Type | SL_CommResourcePoolV2X_r14.numSubchannel_r14 |  |
| StartRB_Subchannel_r14_Type | SL_CommResourcePoolV2X_r14.startRB_Subchannel_r14 |  |
| StartRB_PSCCH_Pool_r14_Type | SL_CommResourcePoolV2X_r14.startRB_PSCCH_Pool_r14 |  |
| Mcs_r14_Type | SL_V2X_ConfigDedicated_r14.commTxResources_r14.setup.scheduled_r14.mcs_r14 |  |

V2X_CommConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | V2X_CommConfig_Type |  |  |
| Comment |  |  |  |
| V2X_CommRxConfig | V2X_CommRxConfig_Type | opt |  |
| V2X_CommTxConfig | V2X_CommTxConfig_Type | opt |  |
| V2X_CommSyncRef | V2X_CommSyncConfig_Type | opt |  |

V2X_CommRxConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | V2X_CommRxConfig_Type |  |
| Comment | SS-UE ID : identifier of the simulated UE being configured RoutingInfo : none TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |  |
| AddOrReconfigure | V2X_CommRx_ConfigInfo_Type | Add/re-configure SideLink PSCCH/PSSCH/SL-SCH/STCH/RLC/PDCP |
| Release | Null_Type | Release SideLink PSCCH/PSSCH/SL-SCH/STCH/RLC/PDCP |

V2X_CommTxConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | V2X_CommTxConfig_Type |  |
| Comment | SS-UE ID : identifier of the simulated UE being configured RoutingInfo : none TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false |  |
| AddOrReconfigure | V2X_CommTx_ConfigInfo_Type | Add/re-configure SideLink PSCCH/PSSCH/SL-SCH/STCH/RLC/PDCP for V2X |
| Release | Null_Type | Release SideLink PSCCH/PSSCH/SL-SCH/STCH/RLC/PDCP for V2X |

V2X_CommTx_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | V2X_CommTx_ConfigInfo_Type |  |  |
| Comment | All fields are optional: present for first configuration omit means no change SS-UE is configured with a SS-UE layer 2 identity, corresponding to the source layer 2 identity and with the UE layer 2 identity corresponding to the service destination layer 2 identity; the SS-UE shall construct the MAC packets using these identities. |  |  |
| ResourcePool | V2X_CommResourcePool_Type | opt | Configure the pool of resources for PSSCH and PSCCH |
| Mcs_r14 | Mcs_r14_Type | opt | Indicates the Modulation and Coding Scheme - Omitted when congestion is configured |
| SL_SCH_Config | SL_SCH_Config_Type | opt | SL-SCH/MAC configuration - Omitted when congestion is configured |
| STCH_ConfigList | STCH_ConfigList_Type | opt | RLC and PDCP configuration - Omitted when congestion is configured |
| SL_Scheduling | SSUE_Sci1Config_Type | opt | Omitted when congestion is configured |
| PscchConfig | PscchConfig_Type | opt |  |
| PsschConfig | PsschConfig_Type | opt |  |

V2X_CommResourcePool_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | V2X_CommResourcePool_Type |  |  |
| Comment |  |  |  |
| sl_OffsetIndicator_r14 | SL_OffsetIndicator_r12 | opt |  |
| sl_Subframe_r14 | SubframeBitmapSL_r14 |  |  |
| adjacencyPSCCH_PSSCH_r14 | boolean |  |  |
| sizeSubchannel_r14 | SizeSubchannel_r14_Type |  |  |
| numSubchannel_r14 | NumSubchannel_r14_Type |  |  |
| startRB_Subchannel_r14 | StartRB_Subchannel_r14_Type |  |  |
| startRB_PSCCH_Pool_r14 | StartRB_PSCCH_Pool_r14_Type | opt |  |

SSUE_Sci1Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SSUE_Sci1Config_Type |  |  |
| Comment |  |  |  |
| Priority | B3_Type |  | Priority - as defined in section 4.4.5.1 of TS 23.285 |
| ResourceReservation | B4_Type |  | Resource reservation as defined in section 14.2.1 of TS 36.213 |
| FrequencyResourceLocation | bitstring |  | Frequency resource location of initial transmission and retransmission as defined in section 14.1.1.4C of TS 36.213 |
| Timegap | B4_Type |  | Time gap between initial transmission and retransmission - 4 bits as defined in section 14.1.1.4C of TS 36.213 |
| RetransmissionIndex | B1_Type |  | Retransmission index - 1 bit as defined in section 14.2.1 of TS 36.213 |

V2X_CommRx_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | V2X_CommRx_ConfigInfo_Type |  |  |
| Comment | It is assumed that one SS-UE layer 2 ID/one SLRB needs to be handled. SS-UE is configured with a layer 2 identity, corresponding to the service destination layer 2 identity; layer2Id in STCH_RLC_Type is omitted. SS-UE shall send to TTCN only the PDCP data packets received with the correct destination layer 2 identities in the MAC header. The source layer 2 identity, randomly and uniquely self-selected by UE, can be any value. |  |  |
| ResourcePool | V2X_CommResourcePool_Type | opt | Configure subframes reserved for PSSCH and PSCCH Present for first configuration omit means no change |
| SL_SCH_Config | SL_SCH_Config_Type | opt | SL-SCH/MAC configuration Present for first configuration omit means no change |
| STCH_ConfigList | STCH_ConfigList_Type | opt |  |

V2X_CommSyncConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | V2X_CommSyncConfig_Type |  |  |
| Comment | To configure SS-UE for synRef transmission/reception and synchronisation |  |  |
| V2X_CommTxSyncRef | V2X_CommTxSyncRefConfig_Type | opt |  |
| V2X_CommRxSyncRef | V2X_CommRxSyncRefConfig_Type | opt |  |
| V2X_SynchronisationSource | SL_SyncRef_Type | opt | Applied when OoC - Omitted when In coverage or UTC synchronised ueSyncRef UE under test is SyncRef UE SS_UE shall synchronise from received SLSS/MIB-SL-V2X configured in V2X_CommRxSyncRef UE shall be transmitting SL-SS when SS_UE is switched on sS_UE_SyncRef SS_UE is SyncRef UE SS_UE shall initiate DFN/DirectSubrame and transmit SLSS/MIB-SL-V2X acc to CommTxSyncRef |

V2X_CommTxSyncRefConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | V2X_CommTxSyncRefConfig_Type |  |
| Comment | To configure/reconfigure SS-UE to start or stop transmitting SLSS/MIB-SL-V2X (irrespective of any measurement) |  |
| AddOrReconfigure | V2X_CommTxSyncRef_Type | Add/re-configure SLSS or SBCCH |
| Release | Null_Type | Stop SLSS and SBCCH transmission |

V2X_CommRxSyncRefConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | V2X_CommRxSyncRefConfig_Type |  |
| Comment | To configure/reconfigure SS-UE to start or stop receiving SLSS/MIB-SL-V2X (irrespective of any measurement) |  |
| AddOrReconfigure | V2X_CommRxSyncRef_Type | Add/re-configure reception of SLSS or SBCCH |
| Release | Null_Type | Release/stop reception of SLSS and SBCCH transmission |

V2X_CommRxSyncRef_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | V2X_CommRxSyncRef_Type |  |  |
| Comment |  |  |  |
| Slssid | SLSSID_Type | opt | Omit means any value is expected |
| SyncOffsetIndicator1_r14 | SL_OffsetIndicatorSync_r14 |  |  |
| SyncOffsetIndicator2_r14 | SL_OffsetIndicatorSync_r14 | opt |  |

V2X_CommTxSyncRef_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | V2X_CommTxSyncRef_Type |  |  |
| Comment | SBCCH - RLC TM - MAC TM SS will perform padding, if required due to the granularity of the TF signalling, as defined in 36.331 cl 8.5. |  |  |
| V2X_TxSyncConfig | V2X_CommTxSyncRefParam_Type | opt |  |
| Psss | PrimarySideLinkSyncSignal_Type | opt |  |
| Ssss | SecondarySideLinkSyncSignal_Type | opt |  |
| Psbch | PsbchConfig_Type | opt | The transmit power of PSBCH is same as the transmit power of primary sidelink synchronisation signal |

V2X_CommTxSyncRefParam_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | V2X_CommTxSyncRefParam_Type |  |  |
| Comment |  |  |  |
| Slssid | SLSSID_Type |  |  |
| SyncOffsetIndicator_r14 | SL_OffsetIndicatorSync_r14 |  |  |
| MIB_SL_V2X | MasterInformationBlock_SL_V2X_r14 |  | DFN and Direct subframe set to a dummy value - to be set by SS-UE |

CongestionConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CongestionConfig_Type |  |
| Comment | Configure SS-UE to start or stop generating congestion in all RBs of specific subchannels on specific subframes. NOTE: SS-UE already configured in Tx (Timing reference, power, frequency and subchannels are already configured). The configured SCI grant for transmission does not apply. |  |
| Activate | SubchannelOffsetConfig_Type | For one SS-UE configuration: SS_UE_Id : identifier of the SS-UE to be configured RoutingInfo : None TimingInfo : activation time from when the SS starts transmission of random data. ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Deactivate | Null_Type | For one SS-UE configuration: SS_UE_Id : identifier of the SS-UE to be configured RoutingInfo : None TimingInfo : activation time from when the SS stops transmitting random data. ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

SubchannelList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SubchannelList_Type |
| Comment |  |
| record of integer |  |

SubchannelOffset_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SubchannelOffset_Type |  |  |
| Comment |  |  |  |
| Subchannels | SubchannelList_Type |  | List of subchannels on which the bitmap applies |
| OffsetBitmap | B10_Type |  | Bitmap indicating the subframes on which congestion is transmitted. Value 0 in the bitmap indicates that the corresponding subframe is invalid for congestion transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for congestion transmission. |

SubchannelOffsetPerFrameList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SubchannelOffsetPerFrameList_Type |
| Comment |  |
| record of SubchannelOffset_Type |  |

SubchannelOffsetConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SubchannelOffsetConfig_Type |  |  |
| Comment | SS-UE shall transmit, over PSCCH/PSSCH, QPSK modulated uncorrelated pseudo random data in all RBs on the provided subchannel list and in subframes indicated as '1' in the OffsetBitmap The congestion is repeated a number of frames or continuously |  |  |
| CongestionRepetition | CongestionRepetition_Type |  |  |
| SubchannelOffsetPerFrameList | SubchannelOffsetPerFrameList_Type |  | Congestion configuration for one frame |

### D.14.2.3	SL_SystemConfirmAsp

SL_CnfAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_CnfAspCommonPart_Type |  |  |
| Comment |  |  |  |
| SS_UE_Id | SS_UE_Id_Type |  |  |
| RoutingInfo | SL_RoutingInfo_Type |  |  |
| TimingInfo | SL_TimingInfo_Type |  |  |
| Result | ConfirmationResult_Type |  |  |

SL_SystemConfirm_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_SystemConfirm_Type |  |
| Comment | Confirmations for SL system configuration; in general to be sent after the configuration has been done |  |
| SS_UE_Config | Null_Type | Confirmation for SS-UE configuration |
| EnquireTiming | Null_Type | Confirmation for EnquireTiming |
| L1MacIndCtrl | Null_Type | Confirmation for L1MacIndCtrl |
| SL_Security | Null_Type | (no further parameters from SS) |

### D.14.2.4	SL_SystemIndicationAsp

SL_IndAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_IndAspCommonPart_Type |  |  |
| Comment |  |  |  |
| SS_UE_Id | SS_UE_Id_Type |  | Simulated SS-UE ID |
| RoutingInfo | SL_RoutingInfo_Type |  | Channel ID or RB Id for STCH |
| TimingInfo | SL_TimingInfo_Type |  | In coverage or Out of coverage timing |
| Status | IndicationStatus_Type |  |  |

SL_SystemIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_SystemIndication_Type |  |
| Comment |  |  |
| Error | charstring | Indicates an error situation in SS-UE; is not explicitly handled in TTCN but causes an INCONC due to default behaviour; an additional error code can be signalled in the common part of the ASP; SS shall raise an error in case of - Invalid TimingInfo - Data scheduled does not fit into an available resource in a resource pool (NOTE: additional cases may occur) |
| DiscSync | DiscSLSS_Ind_Type | Indication for Discovery SLSS sent by UE |
| CommSLSS_MIBSL | CommSLSS_MIBSL_Ind_Type | Indication for Communication SLSSID and MIB-SL sent by UE |
| V2X_CommSLSS_MIBSL | V2X_CommSLSS_MIBSL_Ind_Type | Indication for V2X Communication SLSSID and MIB-SL-V2X sent by UE |

DiscSLSS_Ind_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | DiscSLSS_Ind_Type |
| Comment | When transmitted SLSS is associated with a PC5_DISCOVERY message according to RxDiscSyncRef. Start is triggered in SS_UE when - the first SLSS associated with a PC5_Discovery message is received by SS-UE since enabling of SLSS reporting - the previous SLSS indication was 'stop' or 'spurious' and an SLSS associated with a PC5_Discovery message is received by SS-UE Stop is triggered in SS_UE when - the first PC5_DISCOVERY message is received without SLSS - the previous SLSS indication was 'start' or 'spurious' and PC5_DISCOVERY message is received without SLSS Spurious is triggered in SS_UE when - the first SLSS is received without PC5_DISCOVERY message since enabling of SLSS reporting - the previous SLSS indication was 'start' or 'stop' and SLSS is received without associated PC5_DISCOVERY message |
| DiscSLSS_Start |  |
| DiscSLSS_Stop |  |
| DiscSLSS_Spurious |  |

CommSLSS_MIBSL_Ind_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CommSLSS_MIBSL_Ind_Type |  |
| Comment | Start is triggered in SS_UE when - the first SLSS associated with a MIB-SL is received in the same subframe by SS-UE since enabling of reporting - the previous indication was 'stop' or 'spurious' and an SLSS associated with a MIB-SL is received by SS-UE Stop is triggered in SS_UE when - no SLSS/MIB-SL is transmitted in the period - the previous indication was 'start' or 'spurious' and no SLSS/MIB-SL is transmitted in the period Spurious is triggered in SS_UE when - the first SLSS is received without MIB-SL or MIB-SL is received without SLSS since enabling of SLSS reporting - the previous indication was 'start' or 'stop' and SLSS is received without MIB-SL or MIB-SL is received without SLSS |  |
| CommSLSS_Start | CommSyncParamInd_Type |  |
| CommSLSS_Stop | Null_Type |  |
| CommSLSS_Spurious | Null_Type |  |

CommSyncParamInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CommSyncParamInd_Type |  |  |
| Comment |  |  |  |
| Slssid | SLSSID_Type |  |  |
| MIB_SL | MasterInformationBlock_SL |  |  |

V2X_CommSLSS_MIBSL_Ind_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | V2X_CommSLSS_MIBSL_Ind_Type |  |
| Comment | Start is triggered in SS_UE when - the first SLSS associated with a MIB-SL-V2X is received in the same subframe by SS-UE since enabling of reporting - the previous indication was 'stop' or 'spurious' and an SLSS associated with a MIB-SL-V2X is received by SS-UE Stop is triggered in SS_UE when - no SLSS/MIB-SL-V2X is transmitted in the period - the previous indication was 'start' or 'spurious' and no SLSS/MIB-SL-V2X is transmitted in the period Spurious is triggered in SS_UE when - the first SLSS is received without MIB-SL-V2X or MIB-SL-V2X is received without SLSS since enabling of SLSS reporting - the previous indication was 'start' or 'stop' and SLSS is received without MIB-SL or MIB-SL-V2X is received without SLSS |  |
| V2X_CommSLSS_Start | V2X_CommSyncParamInd_Type |  |
| CommSLSS_Stop | Null_Type |  |
| CommSLSS_Spurious | Null_Type |  |

V2X_CommSyncParamInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | V2X_CommSyncParamInd_Type |  |  |
| Comment |  |  |  |
| Slssid | SLSSID_Type |  |  |
| MIB_SL_V2X | MasterInformationBlock_SL_V2X_r14 |  |  |

### D.14.2.5	SL_System_Interface

SL_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_SYSTEM_CTRL_REQ |  |  |
| Comment | ASP to request/control SideLink system configuration |  |  |
| Common | SL_ReqAspCommonPart_Type |  | TimingInfo depends on respective primitive: |
| Request | SL_SystemRequest_Type |  | - SS_UE_Config TimingInfo: depends on the request - EnquireTiming TimingInfo: 'now' - L1MacIndCtrl TimingInfo: 'now' - SL_Security RoutingInfo: SLRB TimingInfo: 'now' |

SL_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_SYSTEM_CTRL_CNF |  |  |
| Comment | ASP to confirm a SideLink system configuration request |  |  |
| Common | SL_CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN (apart from EnquireTiming): SS may set TimingInfo to "None" -EnquireTiming TimingInfo InCoverage: SFN/Subframe |
| Confirm | SL_SystemConfirm_Type |  |  |

SL_SYSTEM_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_SYSTEM_IND |  |  |
| Comment | ASP to report errors or system indications |  |  |
| Common | SL_IndAspCommonPart_Type |  | The SS shall provide TimingInfo (SFN + subframe number) depending on the respective indication: |
| Indication | SL_SystemIndication_Type |  | - Error TimingInfo: related to the error (if available) - DiscSync TimingInfo: SFN/Subframe related to the SLSS - CommSLSS TimingInfo: SFN/Subframe or DFN/DirectSubframe related to the SLSS - CommMIB_SL TimingInfo: SFN/Subframe or DFN/DirectSubframe related to the MIB-SL |

SL_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | SL_SYSTEM_PORT |  |
| Comment | SideLink UE PTC: Port for system configuration |  |
| out | SL_SYSTEM_CTRL_REQ |  |
| in | SL_SYSTEM_CTRL_CNF |  |

SL_SYSIND_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | SL_SYSIND_PORT |  |
| Comment | SideLink UE PTC: Port for system indications |  |
| in | SL_SYSTEM_IND |  |

# D.15	CommonDefs

CommonDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_UInt16Max | integer | 65535 |  |
| tsc_UInt32Max | integer | 4294967295 |  |

CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| B1_Type | bitstring length(1) |  |
| B2_Type | bitstring length(2) |  |
| B3_Type | bitstring length(3) |  |
| B4_Type | bitstring length(4) |  |
| B5_Type | bitstring length(5) |  |
| B6_Type | bitstring length(6) |  |
| B7_Type | bitstring length(7) |  |
| B7_15_Type | bitstring length(7..15) | NOTE: length restriction can only be a range but not two destinct lengths |
| B8_Type | bitstring length(8) |  |
| B10_Type | bitstring length(10) |  |
| B11_Type | bitstring length(11) |  |
| B12_Type | bitstring length(12) |  |
| B14_Type | bitstring length(14) |  |
| B15_Type | bitstring length(15) |  |
| B16_Type | bitstring length(16) |  |
| B18_Type | bitstring length(18) |  |
| B24_Type | bitstring length(24) |  |
| B32_Type | bitstring length(32) |  |
| B64_Type | bitstring length(64) |  |
| B128_Type | bitstring length(128) |  |
| B256_Type | bitstring length(256) |  |
| B128_Key_Type | B128_Type | 128 bit security key |
| O3_Type | octetstring length(3) |  |
| O4_Type | octetstring length(4) |  |
| O6_Type | octetstring length(6) |  |
| O8_Type | octetstring length(8) |  |
| O13_Type | octetstring length(13) |  |
| Null_Type | boolean (true) | dummy type for 'typeless' fields in unions |
| Dummy_Type | boolean (true) | dummy type for temporary purposes only |
| UInt16_Type | integer (0 .. tsc_UInt16Max) |  |
| UInt32_Type | integer (0 .. tsc_UInt32Max) |  |
| IP_DrbId_Type | integer | DRB identity type common for all RATs: - for EUTRA it corresponds to the ASN.1 type DRB-Identity - for UTRAN it corresponds to the ASN.1 type RB-Identity and values are as defined in TS 34.123-3 Table 8.2.4.1 - for GERAN the NSAPI value (type record NSAPI) may be used NOTE: this is introduced to simplify the dependencies (i.e. to keep IP_ASP_TypeDefs independent from any RAT specific type definitions) |
| PdcpCountValue_Type | B32_Type |  |

IndicationAndControlMode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IndicationAndControlMode_Type |
| Comment |  |
| enable |  |
| disable |  |

NR_CellId_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_CellId_Type |
| Comment |  |
| nr_Cell_NonSpecific |  |
| nr_Cell1 |  |
| nr_Cell2 |  |
| nr_Cell3 |  |
| nr_Cell4 |  |
| nr_Cell6 |  |
| nr_Cell10 |  |
| nr_Cell11 |  |
| nr_Cell12 |  |
| nr_Cell13 |  |
| nr_Cell14 |  |
| nr_Cell23 |  |
| nr_Cell28 |  |
| nr_Cell29 |  |
| nr_Cell30 |  |
| nr_Cell31 |  |

EUTRA_CellId_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | EUTRA_CellId_Type |
| Comment |  |
| eutra_Cell_NonSpecific |  |
| eutra_Cell1 |  |
| eutra_Cell2 |  |
| eutra_Cell3 |  |
| eutra_Cell4 |  |
| eutra_Cell6 |  |
| eutra_Cell10 |  |
| eutra_Cell11 |  |
| eutra_Cell12 |  |
| eutra_Cell13 |  |
| eutra_Cell14 |  |
| eutra_Cell23 |  |
| eutra_Cell28 |  |
| eutra_Cell29 |  |
| eutra_Cell30 |  |
| eutra_Cell31 |  |
| eutra_CellA |  |
| eutra_CellB |  |
| eutra_CellC |  |
| eutra_CellD |  |
| eutra_CellE |  |
| eutra_CellG |  |
| eutra_CellH |  |
| eutra_CellI |  |
| eutra_CellJ |  |
| eutra_CellK |  |
| eutra_CellL |  |
| eutra_CellM |  |

EUTRA_CellIdList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | EUTRA_CellIdList_Type |
| Comment |  |
| record length (0..9) of EUTRA_CellId_Type |  |

WLAN_CellId_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | WLAN_CellId_Type |
| Comment |  |
| wlan_Cell_NonSpecific |  |
| wlan_Cell27 | WLAN AP in EUTRA environment |
| wlan_Cell27b | WLAN AP in EUTRA environment |
| wlan_Cell39 | WLAN AP in UTRA environment |

IP_EUTRA_DrbInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_EUTRA_DrbInfo_Type |  |  |
| Comment |  |  |  |
| CellId | EUTRA_CellId_Type |  | data is routed to a specific cell regardless of whether the same DRB is configured in any other cell |
| DrbId | IP_DrbId_Type | opt | mandatory at the system interface |

IP_UTRAN_GERAN_DrbInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_UTRAN_GERAN_DrbInfo_Type |  |  |
| Comment |  |  |  |
| CellId | integer |  |  |
| DrbId | IP_DrbId_Type | opt | mandatory at the system interface |

IP_WLAN_DrbInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_WLAN_DrbInfo_Type |  |  |
| Comment |  |  |  |
| CellId | integer |  |  |

IP_ePDG_IPsecTunnelInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_ePDG_IPsecTunnelInfo_Type |  |  |
| Comment |  |  |  |
| PdnId | PDN_Index_Type |  | 'index name' of PDN associated to the IPsec tunnel, e.g. for SS to distinguish routing of IP packets in case of more than one IPsec tunnel NOTE: In general only 'ePDG_XXX' values shall be used |

QosFlow_Identification_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | QosFlow_Identification_Type |  |  |
| Comment |  |  |  |
| PDU_SessionId | integer |  | TS 24.007 clause 11.2.3.1b |
| QFI | integer |  | TS 24.501 Table 11.2.3.1c.1 |

IP_NR_QosFlowInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_NR_QosFlowInfo_Type |  |  |
| Comment |  |  |  |
| CellId | NR_CellId_Type |  | data is routed to a specific cell regardless of whether the same DRB is configured in any other cell |
| QosFlow | QosFlow_Identification_Type | opt | mandatory at the system interface |

IP_EUTRA_QosFlowInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_EUTRA_QosFlowInfo_Type |  |  |
| Comment |  |  |  |
| CellId | EUTRA_CellId_Type |  | data is routed to a specific cell regardless of whether the same DRB is configured in any other cell |
| QosFlow | QosFlow_Identification_Type | opt | mandatory at the system interface |

IP_NR_DrbInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_NR_DrbInfo_Type |  |  |
| Comment |  |  |  |
| CellId | NR_CellId_Type |  |  |
| DrbId | IP_DrbId_Type | opt | mandatory at the system interface |

IP_DrbInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_DrbInfo_Type |  |
| Comment |  |  |
| Eutra | IP_EUTRA_DrbInfo_Type |  |
| Eutra5GC | IP_EUTRA_QosFlowInfo_Type | used when SDAP is configured in non-transparent mode at the SS |
| Utran | IP_UTRAN_GERAN_DrbInfo_Type |  |
| Geran | IP_UTRAN_GERAN_DrbInfo_Type |  |
| Nr5GC | IP_NR_QosFlowInfo_Type | used when SDAP is configured in non-transparent mode at the SS |
| NrEPC | IP_NR_DrbInfo_Type | used when NR is connected to EPC (S1-U interface) or when SDAP is configured in transparent mode at the SS |
| Wlan | IP_WLAN_DrbInfo_Type |  |
| IPsecTunnel | IP_ePDG_IPsecTunnelInfo_Type |  |

PDN_Index_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | PDN_Index_Type |
| Comment | 'index name' associated to a PDN: The major purpose is to associate a PDN specific set of IP addresses to the given PDN (e.g. UE addresses, P-CSCF address etc.); in general there are one or two PDNs configured at the same time and - from TTCN point of view - the IMS PDN is considered to be the first one; a second PDN may be configured in case of emergency call or e.g. for XCAP signalling; the 'internet PDN' (according to TS 36.508 clause 4.5.2) is considered as (optional) second PDN during initial registration and gets released after initial registration; in case of WLAN a separate group of index names is used to distinguish the different configuration of the emulated IP network |
| PDN_1 | "default" PDN being kept connected to during a test case (in case of LTE in general the IMS PDN) |
| PDN_2 | second PDN: during initial registration (TS 36.508 clause 4.5.2) for LTE and "multiple PDN' this is the internet PDN; after initial registration it is used if needed according to the test purpose (e.g. emergency call) |
| PDN_2a | used for the special case when the UE IP address of the second PDN changes in a test case |
| PDN_Internet | mainly used as alias for PDN2 during initial registration |
| ePDG_IMS1 | WLAN: PDN for 'normal' IMS |
| ePDG_IMS2 | WLAN: PDN for emergency IMS (in general) |
| ePDG_XCAP | WLAN: PDN for XCAP in case of XCAP server being part of 3GPP-network NOTE: In contrast to LTE for WLAN there is a different IP architecture to be consider by TTCN for XCAP and IMS emergency |
| ePDG_Internet | place-holder for WLAN-offload scenarios |
| nrPDN_Internet | mainly used as alias during initial registration in ENDC |
| PDN_3 | used in ENDC and 5GC for the third PDN |
| PDN_4 | used in 5GC for the URLLC PDN |
| PDN_5 | used in 5GC for the MIoT PDN |
| PDN_6 | used in 5GC for the V2X PDN |

# D.16	CommonAspDefs

## D.16.1	Cell_Configuration_Common

CellTimingInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CellTimingInfo_Type |  |  |
| Comment | Cell Timing |  |  |
| TcOffset | integer (0..63) | opt | For NR according to TS 38.211 clause 4.1 Ts/Tc = 64 with Tc = 1/(480000 * 4096) and Ts = 1/(15000 * 2048) as for EUTRA; => for NR to specify granularity per Tc; for EUTRA to be set to 0 (and/or to be ignored by the SS) |
| Tcell | integer (0..307199) |  | frame duration Tf = 307200 * Ts = 10ms; System Time Unit Ts = 1/(15000 * 2048) |
| SfnOffset | integer (0..1023) |  |  |
| HsfnOffset | integer (0..1023) |  |  |

## D.16.2	MAC_Layer

ULGrant_Period_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ULGrant_Period_Type |  |
| Comment |  |  |
| OnlyOnce | Null_Type | grant is sent out only once; no period |
| Duration | integer (1..infinity) | duration of the grant period in number of sub-frames (1ms) for EUTRA and number of slots for NR |

TransmissionRepetition_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TransmissionRepetition_Type |  |
| Comment |  |  |
| Continuous | Null_Type |  |
| NumOfCycles | integer (1..infinity) |  |

PeriodicGrant_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PeriodicGrant_Type |  |  |
| Comment |  |  |  |
| Period | ULGrant_Period_Type |  | time period after which UL Grant need to be automatically transmitted or 'OnlyOnce' |
| NoOfRepetitions | TransmissionRepetition_Type |  | number of UL Grant repetitions to be automatically transmitted or continuous repetition |

UL_GrantConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UL_GrantConfig_Type |  |
| Comment |  |  |
| OnSR_Reception | Null_Type | SS transmits UL Grant as configured by DciInfoUL_Type at every reception of SR; to be used in non L2 Test |
| Periodic | PeriodicGrant_Type | SS transmits UL Grant as configured by DciInfoUL_Type periodically; to be used in L2 tests; MAC tests testing Grants might set the period as infinite and num grant as 1 |
| PeriodicOnSR_Reception | PeriodicGrant_Type | SS transmits UL Grant as configured by DciInfoUL_Type periodically; the periodic grant transmission is started/restarted on reception of a SR from UE to be used in non L2 Test to enable large UL data transmission for lower category UEs (Cat<=1) |
| None | Null_Type | disable any grant transmission |

RAR_RapIdCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RAR_RapIdCtrl_Type |  |
| Comment |  |  |
| Automatic | Null_Type | SS shall automatically use same RAPID as received from the UE |
| Unmatched | Null_Type | SS shall use RAPID being different from preamble sent by the UE; SS shall calculate this RAPID acc. to RAPID := (RAPID + 3..63) mod 64 if single RAR is transmitted in a MAC PDU then only 3 is added if multiple RAR's are transmitted in MAC PDU, then for first unmatched RAR 3 is added, second unmatched 4 is added, third unmatched 5 is added and so on |

## D.16.3	System_Indications

HARQ_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | HARQ_Type |
| Comment | ack represents HARQ ACK; nack represents HARQ NACK |
| ack |  |
| nack |  |

## D.16.4	ASP_CommonPart

Definition of ASP common parts for REQ-, CNF- and IND-ASPs

### D.16.4.1	ASP_CommonPart_Definitions

#### D.16.4.1.1	Routing_Info

CommonAspDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_SRB0 | integer | 0 |  |
| tsc_SRB1 | integer | 1 |  |
| tsc_SRB2 | integer | 2 |  |
| tsc_SRB3 | integer | 3 |  |
| tsc_SRB4 | integer | 4 |  |

Routing_Info: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SRB_Identity_Type | integer (tsc_SRB0, tsc_SRB1, tsc_SRB2, tsc_SRB3, tsc_SRB4) |  |

RlcBearerRouting_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RlcBearerRouting_Type |  |
| Comment | data routing e.g. in case of split bearer (split DRB or split SRB); applicable for multi-RAT Dual Connectivity (MR-DC) as well as single-RAT Dual Connectivity |  |
| EUTRA | EUTRA_CellId_Type |  |
| NR | NR_CellId_Type |  |
| None | Null_Type | normal case: PDCP and RLC are configured at the same cell |

#### D.16.4.1.2	Timing_Info

Timing_Info: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SystemFrameNumber_Type | integer (0..1023) |  |
| SubFrameNumber_Type | integer (0..9) |  |
| SymbolNumber_Type | integer (0..13) |  |
| HyperSystemFrameNumberInfo_Type | SystemFrameNumberInfo_Type |  |

SubFrameInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SubFrameInfo_Type |  |
| Comment |  |  |
| Number | SubFrameNumber_Type |  |
| Any | Null_Type | no specific sub-frame, in which case the SS may choose the next available subframe (valid for REQ ASPs only) |

SystemFrameNumberInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SystemFrameNumberInfo_Type |  |
| Comment |  |  |
| Number | SystemFrameNumber_Type |  |
| Any | Null_Type | no specific frame number, in which case the SS may choose the next available SFN (valid for REQ ASPs only) |

SlotOffset_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SlotOffset_Type |  |
| Comment | slots per subframe according to TS 38.211 Table 4.3.2-1 |  |
| Numerology0 | Null_Type | mu=0; only one slot per subframe |
| Numerology1 | integer (0..1) | mu=1; 2 slots per subframe |
| Numerology2 | integer (0..3) | mu=2; 4 slots per subframe |
| Numerology3 | integer (0..7) | mu=3; 8 slots per subframe |
| Numerology4 | integer (0..15) | mu=4; 16 slots per subframe |

SlotTimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SlotTimingInfo_Type |  |
| Comment | EUTRA, NBIOT: REQ ASPs: TTCN shall set the SlotTimingInfo to "FirstSlot" for EUTRA, NBIOT addressing the whole subframe IND ASPs: TTCN shall ignore the SlotTimingInfo sent by the SS for EUTRA, NBIOT NR: REQ ASPs: Any:=true only if the slot number is not relevant, in which case the SS may choose the next available slot of the given subframe IND ASPs: Any:=true only if there is no slot information available for the particular kind of indication |  |
| SlotOffset | SlotOffset_Type | to address a particular slot in a subframe |
| FirstSlot | Null_Type | to address the first slot independent from the numerology (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT |
| Any | Null_Type | for IND ASPs in EUTRA and NBIOT or if slot number is not relevant or not available |

SymbolTimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SymbolTimingInfo_Type |  |
| Comment | EUTRA, NBIOT: REQ ASPs: TTCN shall set the SymbolTimingInfo to "FirstSymbol" for EUTRA, NBIOT addressing the whole subframe IND ASPs: TTCN shall ignore the SymbolTimingInfo sent by the SS for EUTRA, NBIOT NR: REQ ASPs: Any:=true only if the symbol number is not relevant, in which case the SS may choose the next available symbol of the given slot IND ASPs: Any:=true only if there is no symbol information available for the particular kind of indication |  |
| SymbolOffset | SymbolNumber_Type | to address a particular symbol in a slot |
| FirstSymbol | Null_Type | to address the first symbol (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT |
| Any | Null_Type | for IND ASPs in EUTRA and NBIOT or if symbol number is not relevant or not available |

SubFrameTiming_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SubFrameTiming_Type |  |  |
| Comment |  |  |  |
| SFN | SystemFrameNumberInfo_Type |  |  |
| Subframe | SubFrameInfo_Type |  |  |
| HSFN | HyperSystemFrameNumberInfo_Type |  |  |
| Slot | SlotTimingInfo_Type |  |  |
| Symbol | SymbolTimingInfo_Type |  |  |

TimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TimingInfo_Type |  |
| Comment |  |  |
| SubFrame | SubFrameTiming_Type |  |
| Now | Null_Type | to be used in REQ ASPs when there is no 'activation time' |
| None | Null_Type | only to be used in SYSTEM_CTRL_CNF or NR_SYSTEM_CTRL_CNF but not for EnquireTiming |

### D.16.4.2	REQ_ASP_CommonPart

ReqAspControlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ReqAspControlInfo_Type |  |  |
| Comment |  |  |  |
| CnfFlag | boolean |  | true => SS shall send CNF: when the REQ is with no timing information (no activation time), SS shall send the confirmation when the configuration is done, i.e. when the test case may continue. Example: when there is a configuration followed by a send event it shall not be necessary to have a wait timer in between but the CNF triggers the send event. If there are other triggers e.g. like the UE sending a message, CnfFlag shall be set to false by the test case to avoid racing conditions with the CNF and the signalling message. When there is an activation time SS shall send the CNF after the configuration has been scheduled; that means SS shall not wait until the activation time has been expired. When FollowOnFlag is true, CnfFlag shall be set to false. |
| FollowOnFlag | boolean |  | false => no further (related) information true: further related information will be sent to SS (semantics depending on respective ASP) |

### D.16.4.3	CNF_ASP_CommonPart

ConfirmationResult_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ConfirmationResult_Type |  |
| Comment |  |  |
| Success | Null_Type |  |
| Error | integer | may contain SS specific error code; this will not be evaluated by TTCN |

### D.16.4.4	IND_ASP_CommonPart

IntegrityErrorIndication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IntegrityErrorIndication_Type |  |  |
| Comment |  |  |  |
| Nas | boolean |  | NAS Integrity: set to true when received MAC does not match calculated MAC |
| Pdcp | boolean |  | PDCP Integrity: set to true when received MAC does not match calculated MAC |

ErrorIndication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ErrorIndication_Type |  |  |
| Comment |  |  |  |
| Integrity | IntegrityErrorIndication_Type |  | Integrity error: received MAC does not match calculated MAC |
| System | integer |  | any other error: may be SS specific error code; this will not be evaluated by TTCN; e.g. an error shall be raised when the UE requests retransmission of an RLC PDU |

IndicationStatus_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IndicationStatus_Type |  |
| Comment |  |  |
| Ok | Null_Type |  |
| Error | ErrorIndication_Type |  |

# D.17	EUTRA_NB_ASP_TypeDefs

Type definitions for configuration of the system simulator;
Common design principles:
Semantics of OMIT: for all TTCN-3 type definitions used in ASPs omit means "keep as it is" =>
- on initial configuration in general all fields shall be provided
- no default values for fields are foreseen
- if necessary non-existence of information shall be explicitly configured
  (e.g. with a union of "no configuration" and "configuration parameters"
- fields within structures imported from the core spec are excepted from this rule

## D.17.1	Cell_Configuration

Specific Info for Cell Configuration Primitive

### D.17.1.1	Cell_Configuration_Common

EUTRA_NB_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_CellAttenuation_Off | Attenuation_Type | {Off:=true} |  |

Cell_Configuration_Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| InitialAttenuation_Type | Attenuation_Type (tsc_CellAttenuation_Off) | Attenuation restricted to 'Off' |
| ToRS_EPRE_Ratio_Type | integer (-35..0) | any-resource-element to RS ratio in dB (e.g. PDSCH-to-RS ratio; see TS 36.213, clause 5.2) |

Attenuation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | Attenuation_Type |  |
| Comment | attenuation of the reference power |  |
| Value | integer (0..149) | cell power reference power reduced by the given attenuation (value is in dB) |
| Off | Null_Type | even though in TS 36.508 -145dBm for LTE and -150dBm for NB-IoT is given for a non suitable cell we specify an explicit "Off" value here |

InitialCellPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | InitialCellPower_Type |  |  |
| Comment |  |  |  |
| MaxReferencePower | AbsoluteCellPower_Type |  | maximum value of cell reference power (RS EPRE in dBm/15kHz as per TS 36.508, clause 4.3.4.1); a cell is initialised with this reference power; its value is the upper bound of the cell power during the test case |
| Attenuation | InitialAttenuation_Type |  | initial attenuation |

EUTRA_CyclicPrefix_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | EUTRA_CyclicPrefix_Type |
| Comment | NOTE: in DL extended cyclic prefix depends on sub-carrier spacing |
| normal |  |
| extended |  |

### D.17.1.2	Downlink_Physical_Layer_Configuration

Downlink_Physical_Layer_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| WUS_Durations_Type | integer (1,2,4,8,16,32,64,128,256,512,1024) | In 36.213 Table 16.9-1 in cl 16.9 for NB-IoT and Table 17-1 in cl 17 for BL/CE UE, values are given in subframes |
| TimeOffsetInFrames | integer (4,8,16,24,100,200) | Corresponds to timeOffsetDRX/timeOffset-eDRX-Short-r15/ timeOffset-eDRX-Long-r15 as defined in WUS-Config-NB-r15 for NB-IoT or WUS-Config-r15 for BL/CE UE |

WUS_PowerBoost_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | WUS_PowerBoost_Type |
| Comment |  |
| db0 |  |
| dB1dot8 |  |
| dB3 |  |
| dB4dot8 |  |

### D.17.1.3	Uplink_Physical_Layer_Configuration

SS_TimingAdvanceConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SS_TimingAdvanceConfig_Type |  |
| Comment |  |  |
| InitialValue | RACH_TimingAdvance_Type | initial value corresponding to what is sent to the UE in RACH response (range acc. 11 bit value; 0 in normal cases) |
| Relative | TimingAdvanceIndex_Type | timing advance command to adjust changes of timing advance acc. to TS 36.213, clause 4.2.3; (range acc. 6 bit value: -31..32) |
| NtnTA | NTN_TimingAdvanceConfig_Type | timing advance configuration for NTN operation |

### D.17.1.4	Common_MAC_Configuration

Transport channel and MAC related procedures and configuration

Common_MAC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| TimingAdvanceIndex_Type | integer (0..63) | acc. to TS 36.321, clause 6.1.3.5 "Timing Advance Command MAC Control Element" and TS 36.213, clause 4.2.3 "Transmission timing adjustments" |
| TimingAdvance_Period_Type | integer (400, 600, 1020, 1530, 2040, 4090, 8190) | the values correspond to 80 % of TimeAlignmentTimer (acc. to TS 36.523-3, clause 7.2) (TS 36.331, clause 6.3.2: sf500, sf750, sf1280, sf1920, sf2560, sf5120, sf10240) rounded to nearest multiple of 10 |

UplinkTimeAlignment_AutoSynch_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UplinkTimeAlignment_AutoSynch_Type |  |  |
| Comment |  |  |  |
| TimingAdvance | TimingAdvanceIndex_Type |  |  |
| TA_Period | TimingAdvance_Period_Type |  | time period after which TA MAC control elements need to be automatically transmitted |
| TA_Repetition | TransmissionRepetition_Type |  | number of TA MAC control element repetitions to be automatically transmitted or 'Continuous' |

UplinkTimeAlignment_Synch_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UplinkTimeAlignment_Synch_Type |  |
| Comment |  |  |
| None | Null_Type | no PUCCH Synchronisation applied |
| Auto | UplinkTimeAlignment_AutoSynch_Type | SS automatically maintains PUCCH synchronization at UE If the cell is a - Rel 10 or earlier cell - or Rel 11 or later Pcell - or the Rel-11 or later scell with no STAG-ID configured, the TAG-ID is set to '00' i.e. P-TAG in Timing advance MCE If the cell is Rel 11 scell with STAG-ID configured, the configured stag-ID is used as TAG-ID in Timing advance MCE |

### D.17.1.5	Random_Access_Procedure

EUTRA_NB_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_RandomAccessResponseListSize | integer | 10 | arbitrary value (needs to be extended, if necessary); in case of RACH in idle, UE will keep on making RACH attempts until t300 expires => number of PRACH preambles maybe even greater than maximum value of PREAMBLE_TRANS_MAX |

Random_Access_Procedure: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RACH_TimingAdvance_Type | integer (0..2047) | 11 bit timing advance as used in RACH response (absolute value) |

ContentionResolution_ContainedDlschSdu_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ContentionResolution_ContainedDlschSdu_Type |  |
| Comment |  |  |
| RlcPduCCCH | octetstring | octetstring of an RLC PDU containing e.g. the RRC Connection Setup; to be sent in the same MAC PDU as the MAC Contention Resolution Control Element (Msg4) |
| PdcpSduDCCH | octetstring | octetstring of a PDCP SDU containing e.g. the RRC Connection Resume; to be sent in the same MAC PDU as the MAC Contention Resolution Control Element (Msg4); the SS shall - apply integrity protection, - add a PDCP header accordingly, - add an AM RLC header, according to TS 36.523-3 clause 7A.8a and clause 7A.13 The logical channel id to be used for the MAC DL-SCH SDU shall be '00001'B corresponding to SRB1 |
| None | Null_Type | MAC PDU containing the MAC Contention Resolution Control Element does not contain an RLC PDU (i.e. RRC Connection Setup is sent in another PDU) |

ContentionResolution_ContainedId_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ContentionResolution_ContainedId_Type |  |
| Comment |  |  |
| XorMask | ContentionResolutionId_Type | When SS receives Contention Resolution ID from the UE, SS shall XOR it with the given mask and use this as Contention Resolution ID; this allows to get an unmatching Contention Resolution ID; in normal cases mask shall be set to tsc_ContentionResolutionId_Unchanged (i.e. the Contention Resolution ID remains unchanged) |
| None | Null_Type | MAC Contention Resolution Control Element is not contained in the MAC PDU sent out as response on Msg3 |

TCRNTI_ContentionResolutionMacPdu_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TCRNTI_ContentionResolutionMacPdu_Type |  |  |
| Comment | NOTE: Either ContainedId or ContainedRlcPdu (or both) shall not be 'none'; (if no Contention Resolution Mac Pdu shall be sent, TCRNTI_ContentionResolutionCtrl_Type.NoContResolID shall be used instead) |  |  |
| ContainedId | ContentionResolution_ContainedId_Type |  | Either the Contention Resolution ID as received from the UE or a modified Contention Resolution ID (XorMask != tsc_ContentionResolutionId_Unchanged) or no Contention Resolution ID at all |
| ContainedRlcPdu | ContentionResolution_ContainedDlschSdu_Type |  | the MAC PDU containing the MAC Contention Resolution Control Element may contain the RRC Connection Setup; in this case the RRC PDU shall be completely encoded been contained in an RLC PDU |

TCRNTI_ContentionResolutionCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TCRNTI_ContentionResolutionCtrl_Type |  |
| Comment | when the UE responds on a Random Access Response with a RRC Connection Request on CCCH and not with a C-RNTI SS shall assume initial Random Access Procedure (TS 36.300, clause 10.1.5.1), i.e. sends a ContentionResolutionId back to the UE |  |
| MacPdu | TCRNTI_ContentionResolutionMacPdu_Type | MAC PDU containing the Contention Resolution ID and optionally an RRC PDU (RRC Connection Setup) |
| MacPdu_CRC_Error | TCRNTI_ContentionResolutionMacPdu_Type | same as MacPdu (see above), but SS shall generate CRC error by toggling CRC bits; no retransmissions shall be made as UE shall not send a NACK |
| NoContResolID | Null_Type | SS shall not include contention resolution ID (i.e. no MAC PDU shall be sent); used for contention resolution fail case |

TempC_RNTI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TempC_RNTI_Type |  |
| Comment |  |  |
| SameAsC_RNTI | Null_Type | in the RA response SS shall use the same C-RNTI as configured in ActiveCellConfig_Type; this is useful for initial random access |
| Explicit | C_RNTI | in the RA response SS shall use different value as configured in ActiveCellConfig_Type; this can be used when the UE already is in RRC_CONNECTED to have a temporary C-RNTI different from the one used by the UE; NOTE: when the UE is not in RRC_CONNECTED there shall be no explicit temp. C-RNTI since then the UE would assume this value as C-RNTI |

RandomAccessBackoffIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RandomAccessBackoffIndicator_Type |  |
| Comment |  |  |
| None | Null_Type | normal case, no back off indicator included |
| Index | integer (0..15) | Backoff Parameter values acc. TS 36.321, clause 7.2; values 0..12 are defined, 13..15 may be used in error case |

### D.17.1.6	NTN_Configuration

NTN_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NTN_T_TA_Type | ServingSatelliteInfo_r17.nta_CommonParameters_17.nta_Common_r17 | acc. to TS 36.331 clause 6.3.1 |

NTN_SemistaticTA_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NTN_SemistaticTA_Type |  |  |
| Comment | see Table 7A.14.2.2-1 for further information |  |  |
| InitialValue | RACH_TimingAdvance_Type |  | initial value corresponding to what is sent to the UE in RACH response; (range acc. 11 bit value; 0 in normal cases) |
| NtnCommonTA | NTN_T_TA_Type |  | round trip time (RTT) between the UL time synchronisation reference point and the serving satellite |
| NtnUeSpecificTA | NTN_T_TA_Type |  | round trip time (RTT) between the UE and the serving satellite |

NTN_TimingAdvanceConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NTN_TimingAdvanceConfig_Type |  |
| Comment |  |  |
| Semistatic | NTN_SemistaticTA_Type | semi-static configuration of the NTN cell's Timing Advance (TA); SS shall not autonomously change the value of TA over time; TTCN may re-configure the TA value |

# D.18	EUTRA_NB_CommonDefs

## D.18.1	Common_Types

Common_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| LogicalChannelId_Type | integer (0..10) | acc. TS 36.331, clause 6.3.2 and clause 6.7.3.2 for DRBs DTCH-LogicalChannelIdentity is INTEGER (3..10); additionally we have 0..2 for the SRBs |
| HarqProcessId_Type | integer (0..14) | The values 0..7 represent the ID of HARQ process ID; value range 0..14 is for TDD |
| ContentionResolutionId_Type | bitstring length(48) | used in EUTRA_ASP_DrbDefs and EUTRA_ASP_Typedefs |
| AbsoluteCellPower_Type | integer (-150..0) | absolute cell power (dBm) |

## D.18.2	RRC_Nested_Types

RRC_Nested_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| IntegrityProtAlgorithm_Type | SecurityAlgorithmConfig.integrityProtAlgorithm |  |
| Dl_Bandwidth_Type | CarrierBandwidthEUTRA.dl_Bandwidth |  |
| Ul_Bandwidth_Type | CarrierBandwidthEUTRA.ul_Bandwidth |  |

## D.18.3	L2Data_CommonDefs

Common definitions of L2 control elements used in L2 messages (EUTRA_NB_ASP_L2DataDefs) as well as for SS configuration (EUTRA_ASP_TypeDefs)

L2Data_CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| MAC_CTRL_ScellActDeact_Type | ScellBitMap_Type | 36.321 clause 6.1.3.8 |

ScellBitMap_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ScellBitMap_Type |  |  |
| Comment |  |  |  |
| Value | B7_Type |  | 7 MSB bits the C Fields C7 to C1. 1 => the corresponding Scell is Active 0 => Inactive |
| Reserved | B1_Type |  | LSBit Reserved. Shall be set to 0 |

PH_Record_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PH_Record_Type |  |  |
| Comment | 36.321 clause 6.1.3.6a |  |  |
| P_Bit | B1_Type |  | P bit: 1 indicates the UE applies power backoff due to power management |
| V_Bit | B1_Type |  | V bit: Indicates if the PH value is based on a real transmission or a reference format. For Type 1 PH, V=0 indicates real transmission on PUSCH and V=1 indicates that a PUSCH reference format is used |
| Valve | B6_Type |  | The power headroom level. Ph Type 2 (if configured) for Pcell and Type 1 for Pcell and Scell |
| Reserved | B2_Type | opt | 2 reservid bits. Present if V=1 |
| PCMaxc | B6_Type | opt | Present if V=1 |

PH_RecordList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PH_RecordList_Type |
| Comment | If simultaneousPUCCH-PUSCH is not configured at least oneType 1 PH record for Pcell is present. Additional one record per Scell for which respective 'C' bit is set as 1. If simultaneousPUCCH-PUSCH is configured then one PH Type 2 record for P cell followed by PH Type 1 record for P cell is present. Additional one record per Scell for which respective 'C' bit is set as 1 |
| record length(1..9) of PH_Record_Type |  |

MAC_CTRL_ExtPowerHeadRoom_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_ExtPowerHeadRoom_Type |  |  |
| Comment |  |  |  |
| EPH_Octet1 | ScellBitMap_Type |  |  |
| PH_RecordList | PH_RecordList_Type |  | At least one record for Pcell is present. Additional one record per Scell for which respective 'C' bit is set as 1 |

DC_PH_RecordList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | DC_PH_RecordList_Type |
| Comment | If simultaneousPUCCH-PUSCH is not configured at least oneType 1 PH record for Pcell is present. Additional one record per PScell/Scell for which respective 'C' bit is set as 1. If simultaneousPUCCH-PUSCH is configured then one PH Type 2 record for Pcell followed one PH Type 2 record for PScell followed by PH Type 1 record for Pcell is present. Additional one record per PSCell/Scell for which respective 'C' bit is set as 1 |
| record length(1..10) of PH_Record_Type |  |

MAC_CTRL_DC_PowerHeadRoom_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_DC_PowerHeadRoom_Type |  |  |
| Comment |  |  |  |
| DC_PH_Octet1 | ScellBitMap_Type |  |  |
| DC_PH_RecordList | DC_PH_RecordList_Type |  | At least one record for Pcell is present. Additional one record per PScell/Scell for which respective 'C' bit is set as 1 |

MAC_CTRL_DifferentialKoffset_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_DifferentialKoffset_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.22 |  |  |
| Reserved | B2_Type |  | 2 bits are reserved |
| Value | B6_Type |  | the differential Koffset in subframes |

MAC_CTRL_TimingAdvanceReport_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_TimingAdvanceReport_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.20 |  |  |
| Reserved | B2_Type |  | 2 bits are reserved |
| Value | B14_Type |  | the least integer number of subframes greater than or equal to the Timing Advance value |

## D.18.4	SC_PTM_Types

SC_MRB_Identity_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SC_MRB_Identity_Type |  |  |
| Comment |  |  |  |
| MbmsSessionInfo | MBMSSessionInfo_r13 |  |  |

# D.19	SidelinkUE_Common_TypeDefs

SS_UE_Id_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | SS_UE_Id_Type |
| Comment | Simulated UE-ID |
| ss_UE_NonSpecific | Represents all Simulated UEs |
| ss_UE_Id1 |  |
| ss_UE_Id2 |  |
| ss_UE_Id3 |  |
| ss_UE_Id4 |  |
| ss_UE_Id5 |  |

CongestionRepetition_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CongestionRepetition_Type |  |
| Comment |  |  |
| Continuous | Null_Type | Send random data until Deactivation command is received |
| NumberOfFrames | integer | Number of consecutive frames the congestion is repeated |

# D.20	NR_PDCP_TypeDefs

## D.20.1	NR_PDCP_Config_Parameters

Parameters defined in or related to NR ASN.1 type PDCP-Config

NR_PDCP_SN_Size_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_PDCP_SN_Size_Type |
| Comment | PDCP Sequence Number |
| PDCP_SNLength12 | TS 38.323 clause 6.2.2.1 and clause 6.2.2.2 |
| PDCP_SNLength18 | TS 38.323 clause 6.2.2.3 |

NR_PDCP_DRB_HeaderCompression_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_DRB_HeaderCompression_Type |  |
| Comment | place holder for header compression |  |
| None | Null_Type |  |

NR_PDCP_DRB_Config_Parameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_DRB_Config_Parameters_Type |  |  |
| Comment | parameters corresponding to NR ASN.1 PDCP-Config.drb; the following parameter are not included and may be added if needed: - integer DiscardTimer (timer value in milliseconds) - boolean StatusReportRequired |  |  |
| SN_SizeUL | NR_PDCP_SN_Size_Type |  | PDCP-Config.drb.pdcp-SN-SizeUL |
| SN_SizeDL | NR_PDCP_SN_Size_Type |  | PDCP-Config.drb.pdcp-SN-SizeDL |
| HeaderCompression | NR_PDCP_DRB_HeaderCompression_Type |  | PDCP-Config.drb.headerCompression |
| IntegrityProtectionEnabled | boolean |  | PDCP-Config.drb.integrityProtection |
| OutOfOrderDelivery | boolean |  | PDCP-Config.drb.outOfOrderDelivery Set to true by default (regardless of the UE configuration), unless specifically specified otherwise in a test case. |

NR_PDCP_RB_Config_Parameters_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_RB_Config_Parameters_Type |  |
| Comment |  |  |
| Srb | Null_Type | no SRB specific parameters in NR ASN.1 PDCP-Config |
| Drb | NR_PDCP_DRB_Config_Parameters_Type | DRB specific parameters corresponding to NR ASN.1 PDCP-Config.drb |

NR_PDCP_Config_Parameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_Config_Parameters_Type |  |  |
| Comment | parameters corresponding to NR ASN.1 PDCP-Config: the following parameter are not included and may be added if needed: - integer TReorderingTimer (timer value in milliseconds) |  |  |
| Rb | NR_PDCP_RB_Config_Parameters_Type |  |  |
| DuplicateDiscarding | boolean | opt | if omitted: initial configuration: duplicate discarding function operates as normal re-configuration: "keep as it is" if set: false - SS shall disable the duplicate discarding function of the receiving PDCP entity: a PDCP PDU, using the same SN as a previously received PDCP PDU, shall not be discarded and the resulting PDCP SDU shall be delivered to the upper layer. true - (re)configuration of duplicate discarding function back to normal operation. |

## D.20.2	NR_PDCP_Configuration

NR_PDCP_TransparentMode

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_TransparentMode |  |  |
| Comment |  |  |  |
| SN_Size | NR_PDCP_SN_Size_Type |  |  |

NR_PDCP_RbConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_RbConfig_Type |  |
| Comment |  |  |
| Params | NR_PDCP_Config_Parameters_Type | PDCP configuration parameters corresponding to UE configuration |
| TransparentMode | NR_PDCP_TransparentMode | PDCP configuration for transparent (test) mode: used for PDCP tests (TS 38.523-3, cl. 5.1.2.1): the SS does not apply ciphering, not apply integrity protection and does not maintain PDCP sequence numbers and state variables; ROHC is not not applied by the SS. Note: a reconfiguration of a RB from transparent mode to 'normal' mode is not foreseen (i.e. there is no mechanism to restore Ciphering, PDCP sequence numbers and state variables at the SS) (in UL PDCP PDUs are decoded depending on SN_Size) |

NR_PDCP_RBTerminating_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_RBTerminating_Type |  |  |
| Comment | RB terminating PDCP configuration: the PDCP may be linked - to a local RLC bearer: RLC bearer is configured for the same cell - to the RLC bearer of some other cell group: LinkToOtherCellGroup is not "None" - the both (in case of split bearer): RLC bearer is configured for the same cell and LinkToOtherCellGroup is not "None" |  |  |
| RbConfig | NR_PDCP_RbConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| LinkToOtherCellGroup | RlcBearerRouting_Type | opt | mandatory for initial configuration; omit means "keep as it is" None: no link to other cell group (normal case, non-split bearer) RAT/cellId: PDCP is linked to RLC bearer of another cell group (same or other RAT): split bearer or PDCP and RLC bearer being configured at different cells NOTE: applicable also for PDCP split bearer test cases when PDCP is in transparent mode => test case body may be implemented at one PTC |

NR_PDCP_Proxy_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_Proxy_Type |  |  |
| Comment | proxy to route PDCP data between terminating PDCP entity and RLC bearer of another cell (group) |  |  |
| LinkToOtherNode | RlcBearerRouting_Type |  | RAT/cellId to address the radio bearer terminating node (PDCP) (None is not applicable) |

# D.21	SDAP_TypeDefs

## D.21.1	SDAP_Configuration

SDAP_Header_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | SDAP_Header_Type |
| Comment |  |
| Present |  |
| Absent |  |

QFI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | QFI_List_Type |
| Comment |  |
| record of integer |  |

SdapConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SdapConfig_Type |  |  |
| Comment |  |  |  |
| Pdu_SessionId | integer |  |  |
| Sdap_HeaderDL | SDAP_Header_Type | opt | mandatory for initial configuration; omit means "keep as it is" when set to 'Present', unless specifically triggered otherwise, the SS shall set the RDI and RQI fields to 0 |
| Sdap_HeaderUL | SDAP_Header_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| MappedQoS_Flows | QFI_List_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

SdapTransparentMode_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SdapTransparentMode_Type |  |  |
| Comment |  |  |  |
| Sdap_HeaderUL | SDAP_Header_Type |  | Indicates to the SS if the UE has been configured to include SDAP UL header |

SdapConfigInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SdapConfigInfo_Type |  |
| Comment |  |  |
| SdapConfig | SdapConfig_Type | SDAP configuration parameters for the DRB |
| TransparentMode | SdapTransparentMode_Type | SDAP configuration for transparent (test) mode, used for SDAP tests: SS does not add any SDAP headers in DL and does not remove any SDAP headers in UL |

SDAP_Configuration_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SDAP_Configuration_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Config | SdapConfigInfo_Type |  |

## D.21.2	SDAP_DrbDefs

SDAP_DrbDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SDAP_SDU_Type | octetstring |  |

SDAP_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SDAP_SDUList_Type |
| Comment |  |
| record of SDAP_SDU_Type |  |

SDAP_DL_PduHeader_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SDAP_DL_PduHeader_Type |  |  |
| Comment | TS 37.324 Figure 6.2.2.2-1 |  |  |
| RDI | B1_Type |  | TS 37.324 Figure 6.3.7: The RDI bit indicates whether QoS flow to DRB mapping rule should be updated 1 bit: '0'B No action '1'B To store QoS flow to DRB mapping rule |
| RQI | B1_Type |  | TS 37.324 clause 6.3.6: The RQI bit indicates whether NAS should be informed of the updated of SDF to QoS flow mapping rules 1 bit: '0'B No action '1'B To inform NAS that RQI bit is set to 1 |
| QFI | B6_Type |  | TS 37.324 clause 6.3.4: The QFI field indicates the ID of the QoS flow to which the SDAP SDU belongs |

SDAP_UL_PduHeader_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SDAP_UL_PduHeader_Type |  |  |
| Comment | TS 37.324 Figure 6.2.2.3-1 or 6.2.3-1 |  |  |
| DC | B1_Type |  | TS 37.324 clause 6.3.3: The D/C bit indicates whether the SDAP PDU is an SDAP Data PDU or an SDAP Control PDU 1 bit: '0'B Control PDU '1'B Data PDU |
| R | B1_Type |  | TS 37.324 clause 6.3.5: Reserved. In this version of the specification reserved bits shall be set to 0. Reserved bits shall be ignored by the receiver |
| QFI | B6_Type |  | QFI (UL) or PQFI (SL) TS 37.324 clause 6.3.4: The QFI field indicates the ID of the QoS flow to which the SDAP SDU belongs, or TS 37.324 clause 6.3.8: The PQFI field indicates the ID of the PC5 QoS flow to which the SDAP PDU belongs |

SDAP_PDU_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SDAP_PDU_DL_Type |  |  |
| Comment | TS 37.324 clause 6.2.2 Data PDU |  |  |
| Header | SDAP_DL_PduHeader_Type | opt | present for DL Data PDU with SDAP header (clause 6.2.2.2), omitted for Data PDU without SDAP header (clause 6.2.2.1) |
| Data | SDAP_SDU_Type |  |  |

SDAP_PDU_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SDAP_PDU_UL_Type |  |  |
| Comment | TS 37.324 clause 6.2.2 Data PDU or clause 6.2.3 End-Marker Control PDU |  |  |
| Header | SDAP_UL_PduHeader_Type | opt | present for: UL Data PDU in default DRB or UL Data PDU with SDAP header in a non-default DRB (clause 6.2.2.3) or End-Marker Control PDU (clause 6.2.3); omitted for UL Data PDU without SDAP header in a non-default DRB (clause 6.2.2.1) |
| Data | SDAP_SDU_Type | opt | present for UL Data PDU (clause 6.2.2.1 or 6.2.2.3), omitted for End-Marker Control PDU (clause 6.2.3) |

SDAP_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SDAP_PDU_Type |  |
| Comment | TS 37.324 clause 6.2.2 Data PDU or 6.2.3 End-Marker Control PDU |  |
| DL | SDAP_PDU_DL_Type | Data PDU in DL |
| UL | SDAP_PDU_UL_Type | Data PDU in UL or End-Marker Control PDU |

SDAP_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SDAP_PDUList_Type |
| Comment |  |
| record of SDAP_PDU_Type |  |

# D.22	References to TTCN-3

| References to TTCN-3 |  |  |
| --- | --- | --- |
| EUTRA_ASP_TypeDefs | EUTRA_Defs/EUTRA_ASP_TypeDefs.ttcn | Rev 36634 |
| EUTRA_ASP_DrbDefs | EUTRA_Defs/EUTRA_ASP_DrbDefs.ttcn | Rev 25117 |
| EUTRA_NB_ASP_L2DataDefs | Common4G/EUTRA_NB_ASP_L2DataDefs.ttcn | Rev 36615 |
| EUTRA_ASP_SrbDefs | EUTRA_Defs/EUTRA_ASP_SrbDefs.ttcn | Rev 24710 |
| IP_ASP_TypeDefs | IP_PTC/IP_ASP_TypeDefs.ttcn | Rev 32541 |
| NasEmu_AspTypes_EUTRA | NasEmulation/NasEmu_AspTypes_EUTRA.ttcn | Rev 25117 |
| EUTRA_CommonDefs | EUTRA_Defs/EUTRA_CommonDefs.ttcn | Rev 29778 |
| CDMA2000_ASP_TypeDefs | C2K/CDMA2000_ASP_TypeDefs.ttcn | Rev 27659 |
| CDMA2000_CommonDefs | C2K/CDMA2000_CommonDefs.ttcn | Rev 19076 |
| EUTRA_ASP_CDMA2000TunnellingDefs | EUTRA_Defs/EUTRA_ASP_CDMA2000TunnellingDefs.ttcn | Rev 20254 |
| EUTRA_ASP_VirtualNoiseDefs | EUTRA_Defs/EUTRA_ASP_VirtualNoiseDefs.ttcn | Rev 24458 |
| UTRAN_ASP_VirtualNoiseDefs | UTRAN/UTRAN_ASP_VirtualNoiseDefs.ttcn | Rev 15724 |
| WLAN_ASP_TypeDefs | WLAN/WLAN_ASP_TypeDefs.ttcn | Rev 34258 |
| SideLinkUE_ASP_TypeDefs | EUTRA_SL/SideLinkUE_ASP_TypeDefs.ttcn | Rev 35210 |
| CommonDefs | Common/CommonDefs.ttcn | Rev 36499 |
| CommonAspDefs | Common/CommonAspDefs.ttcn | Rev 34526 |
| EUTRA_NB_ASP_TypeDefs | Common4G/EUTRA_NB_ASP_TypeDefs.ttcn | Rev 36032 |
| EUTRA_NB_CommonDefs | Common4G/EUTRA_NB_CommonDefs.ttcn | Rev 36616 |
| SidelinkUE_Common_TypeDefs | Common4G5G/SidelinkUE_Common_TypeDefs.ttcn | Rev 35279 |
| NR_PDCP_TypeDefs | Common4G5G/NR_PDCP_TypeDefs.ttcn | Rev 36711 |
| SDAP_TypeDefs | Common4G5G/SDAP_TypeDefs.ttcn | Rev 29170 |

Annex F (informative)
TTCN-3 Definitions

# F.1	NBIOT_ASP_TypeDefs

Type definitions for configuration of the system simulator;
Common design principles:
Semantics of OMIT: for all TTCN-3 type definitions used in ASPs omit means "keep as it is" =>
- on initial configuration in general all fields shall be provided
- no default values for fields are foreseen
- if necessary non-existence of information shall be explicitly configured
  (e.g. with a union of "no configuration" and "configuration parameters"
- fields within structures imported from the core spec are excepted from this rule

## F.1.1	ASN1_Container

Definitions containing ASN.1 types for backward compatibility;
NOTE 1: PCCH_Message and BCCH_DL_SCH_Message already have a critical extension mechanism by RRC type definition
NOTE 2: BCCH_BCH_Message contains the MIB and therefore is considered to be not extendable
NOTE 3: "simple types" are not considered: C_RNTI, PhysCellId, CellIdentity, ARFCN_ValueEUTRA

NPUSCH_ConfigDedicated_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPUSCH_ConfigDedicated_Type |  |  |
| Comment |  |  |  |
| R13 | NPUSCH_ConfigDedicated_NB_r13 |  |  |
| Npusch_TxDuration_r17 | NPUSCH_TxDuration_NB_r17 | opt | used in NTN cell |

NPRACH_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPRACH_Config_Type |  |  |
| Comment |  |  |  |
| R13 | NPRACH_ConfigSIB_NB_r13 |  |  |
| R17 | NRACH_TxDuration_r17_Type | opt | used in NTN cell |

NRACH_TxDuration_r17_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NRACH_TxDuration_r17_Type |  |  |
| Comment |  |  |  |
| Nprach_TxDurationFmt01_r17 | NPRACH_TxDurationFmt01_NB_r17 | opt |  |
| Nprach_TxDurationFmt2_r17 | NPRACH_TxDurationFmt2_NB_r17 | opt |  |

NB_RACH_ConfigCommon_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RACH_ConfigCommon_Type |  |  |
| Comment |  |  |  |
| R13 | RACH_ConfigCommon_NB_r13 |  |  |

NB_PDCP_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_Config_Type |  |
| Comment |  |  |
| R13 | PDCP_Config_NB_r13 |  |

NB_UL_AM_RLC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_UL_AM_RLC_Type |  |  |
| Comment |  |  |  |
| R13 | UL_AM_RLC_NB_r13 |  |  |

NB_DL_AM_RLC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DL_AM_RLC_Type |  |  |
| Comment |  |  |  |
| R13 | DL_AM_RLC_NB_r13 |  |  |

NPUSCH_ConfigCommon_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPUSCH_ConfigCommon_Type |  |  |
| Comment |  |  |  |
| R13 | NPUSCH_ConfigCommon_NB_r13 |  |  |
| Npusch_TxDuration_r17 | NPUSCH_TxDuration_NB_r17 | opt | used in NTN cell |

NB_DRX_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DRX_Config_Type |  |  |
| Comment |  |  |  |
| R13 | DRX_Config_NB_r13 |  |  |

CarrierConfigDedicated_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CarrierConfigDedicated_Type |  |  |
| Comment |  |  |  |
| R13 | CarrierConfigDedicated_NB_r13 |  |  |

DL_GapConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DL_GapConfig_Type |  |  |
| Comment |  |  |  |
| R13 | DL_GapConfig_NB_r13 |  |  |

DL_Bitmap_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DL_Bitmap_Type |  |  |
| Comment |  |  |  |
| R13 | DL_Bitmap_NB_r13 |  |  |

NB_DL_ConfigCommon_NB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DL_ConfigCommon_NB_Type |  |  |
| Comment |  |  |  |
| R14 | DL_ConfigCommon_NB_r14 |  |  |

NB_UL_ConfigCommon_NB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_UL_ConfigCommon_NB_Type |  |  |
| Comment |  |  |  |
| R14 | UL_ConfigCommon_NB_r14 |  |  |
| R15 | UL_ConfigCommon_NB_v1530 | opt | if present, configures NPRACH parameters Fmt2 for the UL non-anchor carrier configured by R14 |

## F.1.2	System_Configuration

Formal ASP Definitions for system configuration

System_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_DPR_DataVolumnIndex_Type | integer (0..15) |  |
| NB_DPR_PowerHeadroomLevel_Type | integer (0..3) |  |

NB_SystemRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_SystemRequest_Type |  |
| Comment |  |  |
| Cell | NB_CellConfigRequest_Type | configure/release a cell |
| CellAttenuationList | NB_CellAttenuationList_Type | power attenuation for one or several cells; all cells included in the list shall be changed at the same time; all cells in the list shall reach the new cell power within a maximum of 100ms (10 frames) acc. to the tolerances given in TS 36.508 NOTE: In the common ASP part the CellId shall be set - to the cell the timing information refers to if activation time shall be applied - to nbiot_Cell_NonSpecific when there is no activation time |
| RadioBearerList | NB_RadioBearerList_Type | configure/release one or several SRBs and/or DRBs |
| EnquireTiming | Null_Type | get HSFN, SFN and sub-frame number for this cell |
| AS_Security | NB_AS_Security_Type | StartRestart/Release of AS security |
| Paging | NB_PagingTrigger_Type | to trigger SS to send paging at the given paging occasion (as calculated in TTCN) or a Direct Indication |
| L1MacIndCtrl | NB_L1Mac_IndicationControl_Type | to configure SS to generate indications for L1/MAC events |
| PdcpCount | NB_PDCP_CountReq_Type | to set or enquire PDCP COUNT for one or more RBs |
| RlcCounts | NB_RLC_CountsReq_Type | to set or enquire RLC Counts VTS and VRR |
| NpdcchOrder | NB_RA_NPDCCH_Order_Type | to configure SS to transmit a NPDCCH order with configured C-RNTI to the UE to trigger RA procedure; result in DCI Format N1 transmission as in TS 36.212, clause 6.4.3.2; the SS shall schedule the NPDCCH order in the same way as DCI for DL transmissions |
| PtmConfig | NB_PTM_Config_Type | Configuration of Point To Multipoint (PTM) |
| WakeUpSignal | Null_Type | To trigger SS to send Wake Up Signal; CellId : identifier of the cell where the UE is active RoutingInfo : None TimingInfo : Wake Up Signal transmission start time, calculated by TTCN ControlInfo : CnfFlag:=false; FollowOnFlag:=false |

NB_SystemConfirm_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_SystemConfirm_Type |  |
| Comment | confirmations for system configuration; in general to be sent after the configuration has been done |  |
| Cell | Null_Type | (no further parameters from SS) |
| CellAttenuationList | Null_Type | (no further parameters from SS) NOTE 1: the confirmation shall be sent when all cells have changed power levels NOTE 2: for the CellId in the common ASP part the same rules are applied as for the SYSTEM REQ |
| RadioBearerList | Null_Type | (no further parameters from SS) |
| EnquireTiming | Null_Type | SFN and sub-frame number are included in the TimingInfo |
| AS_Security | Null_Type | (no further parameters from SS) |
| Paging | Null_Type | normally not needed but defined for completeness |
| L1MacIndCtrl | Null_Type | (no further parameters from SS) |
| PdcpCount | NB_PDCP_CountCnf_Type | as response to 'Get' a list is returned containing COUNT information for the requested RBs |
| RlcCounts | NB_RLC_CountsCnf_Type | as response to 'Get' a list is returned containing RLC Counts VTS and VRR |
| NpdcchOrder | Null_Type | confirmation for NPDCCH Order |
| WakeUpSignal | Null_Type | confirmation for activation of wake up signal |

NB_HarqError_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_HarqError_Type |  |
| Comment |  |  |
| UL | Null_Type | there is just one HARQ process for NB-IoT and uplink HARQ operation is asynchronous (see TS 36.321 clause 5.4.2) |
| DL | Null_Type |  |

NB_RachPreamble_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RachPreamble_Type |  |  |
| Comment |  |  |  |
| RAPID | integer |  | indicates the RAPID of the preamble used (integer (0..63)); according to TS 36.321 clause 6.2.2 "For NB-IoT, the Random Access Preamble IDentifier field corresponds to the start subcarrier index"; in case of NPDCCH order it is calculated according to TS 36.321 clause 5.2.1 for CBRA or CFRA |
| RepetitionsPerPreambleAttempt | integer |  | number of preambles the SS has detected for the preamble attempt |
| NonAnchorCarrierId | integer | opt | Id of the non-anchor carrier (-> NB_NonAnchorCarrierCommonUL_Type.Id) on which the UE as sent the RACH preamble; omit when the RACH preamble has been sent on the anchor carrier |

NB_DPR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DPR_Type |  |  |
| Comment |  |  |  |
| DataVolumnIndex | NB_DPR_DataVolumnIndex_Type |  |  |
| PowerHeadroomLevel | NB_DPR_PowerHeadroomLevel_Type |  |  |

NB_SystemIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_SystemIndication_Type |  |
| Comment |  |  |
| Error | charstring | indicates an error situation in SS; is not explicitly handled in TTCN but causes an INCONC due to default behaviour; an additional error code can be signalled in the common part of the ASP; |
| RachPreamble | NB_RachPreamble_Type | RACH preamble being sent by the UE NOTE 1: for NB-IoT there can be more than one repetition per preamble attempt (numRepetitionsPerPreambleAttempt-r13 in NPRACH-ConfigSIB-NB-DEFAULT in SystemInformationBlockType2-NB); nevertheless the SS shall raise only one indication per preamble attempt and indicate the number of attempts in the 'RepetitionsPerPreambleAttempt' field of the indication NOTE 2: The SS shall provide start of the attempt's first preamble as timing information of the primitive (in a lab environment in general this is the start time of the NPRACH preamble transmission according to clause 10.1.6.1 of TS 36.211 - depending on nprach-Periodicity and nprach-StartTime) |
| DPR | NB_DPR_Type | indicates MAC DPR control element (TS 36.321 clause 6.1.3.10) being contained in MAC PDU |
| HarqError | NB_HarqError_Type | indicates detection of HARQ error: 1. HARQ CRC error for UL data 2. HARQ NACK from the UE unless SS is configured to report HARQ ACK/NACK |
| UL_HARQ | HARQ_Type | to report the UL HARQ ACK/NACK; In case of HARQ repetitions (i.e. ack-NACK-NumRepetitions-Msg4-r13 OR ack-NACK-NumRepetitions-r13 > r1) the SS shall raise only one indication per logical HARQ, but not an indication per HARQ repetition (e.g. with ack-NACK-NumRepetitions-r13 = r8 in NPUSCH-ConfigDedicated-NB-r13 still only one HARQ indication shall be raised). The SS shall provide start of UL HARQ feedback as timing information of the primitive. |
| TimingAdvanceReport | MAC_CTRL_TimingAdvanceReport_Type | To get UE reported TA |

## F.1.3	Cell_Configuration

Specific Info for Cell Configuration Primitive

### F.1.3.1	Cell_Configuration_Common

Cell_Configuration_Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_ToRS_EPRE_Ratio_Type | integer (-35..0) | any-resource-element to NRS ratio in dB (e.g. NPDSCH-to-NRS ratio; see TS 36.213, clause 16.2.2) |

NB_CellConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_CellConfigRequest_Type |  |
| Comment |  |  |
| AddOrReconfigure | NB_CellConfigInfo_Type | for cell configuration: CellId : identifier of the cell to be configured RoutingInfo : None TimingInfo : Now (for initial configuration and for reconfiguration in general) ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Release | Null_Type | to remove a cell completely - CellId : identifier of the cell to be released; nbiot_Cell_NonSpecific, in case all cells shall be released RoutingInfo : None TimingInfo : Now ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

NB_CellConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CellConfigInfo_Type |  |  |
| Comment | common information for initial cell configuration or reconfiguration; in case of reconfiguration OMIT means 'keep configuration as it is' |  |  |
| Basic | NB_BasicCellConfig_Type | opt | basic information for a cell (e.g. broadcasting) |
| Active | NB_ActiveCellConfig_Type | opt | add. configuration for active cell (i.e. cell being capable to receive RACH preamble) |

NB_BasicCellConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_BasicCellConfig_Type |  |  |
| Comment |  |  |  |
| StaticCellInfo | NB_StaticCellInfo_Type | opt | Common information which does not change during a test |
| PhysicalLayerConfigDL | NB_PhysicalLayerConfigDL_Type | opt | default settings regarding DL physical control channels |
| InitialCellPower | InitialCellPower_Type | opt | reference cell power for the NRS of each antenna in DL NOTE 1: the power of the RS of an antenna may be reduced by antenna specific configuration NOTE 2: in general the power may be adjusted on a per resource element basis => all physical channel/signal power settings shall be adjusted relatively to the NRS; if there are more than one TX antennas each one may have its own attenuation; independently from those relative power settings the cell power can easily be adjusted by just changing the reference power |
| DownlinkBitmapConfig | NB_DownlinkBitmapConfig_Type | opt | Usage of bitmap or no bitmap to define valid NB-IoT DL subframes; omit means "keep as it is" |
| BcchConfig | NB_BcchConfig_Type | opt | configuration of BCCH/BCH; SS is triggered to configure RLC/MAC regardingly; BCCH data on the NPDSCH NPBCH: MIB; NPDSCH: scheduling and resource allocation; SIBs |
| PcchConfig | NB_PcchConfig_Type | opt | configuration of PCCH/PCH; SS is triggered to configure RLC/MAC regardingly; PCCH data on the NPDSCH is distinguished by the P-RNTI (needed even to modify SI => shall be configured for CELL_BROADCASTING) |

NB_ActiveCellConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_ActiveCellConfig_Type |  |  |
| Comment |  |  |  |
| C_RNTI | EUTRA_ASN1_C_RNTI_Type | opt | (pre-)configured C-RNTI; affects scrambling of NPDSCH/NPUSCH and CRC of NPDCCH(s); shall be used implicitly in RACH procedure (i.e. as CE in RAR) |
| PhysicalLayerConfigUL | NB_PhysicalLayerConfigUL_Type | opt | parameters for NPRACH, NPUSCH Omitted when no UL frequency is applied to the band |
| RachProcedureConfig | NB_RachProcedureConfig_Type | opt | to configure the SS's behaviour for the RACH procedure Omitted when no UL frequency is applied to the band |
| CcchDcchDtchConfig | NB_CcchDcchDtchConfig_Type | opt | Parameters related to CCCH/DCCH/DTCH in UL and DL |
| NonAnchorCarrier | NB_NonAnchorCarrier_Type | opt | configuration for non-Anchor Carrier ("None" if there is no non-Anchor Carrier) |
| CarrierConfigCommonDL | NB_CarrierConfigCommonDL_Type | opt | to configure multiple NonAnchorCarriers for Paging in accordance with TS 36.304 clause 7.1.1 (r14 and later) |
| CarrierConfigCommonUL | NB_CarrierConfigCommonUL_Type | opt | to configure multiple NonAnchorCarriers for RACH in accordance with TS 36.321 clause 5.1.1 (r14 and later) |
| NtnConfig | NB_NTN_Config_Type | opt | to configure parameters specifically needed for NTN operation |

NB_StaticCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_StaticCellInfo_Type |  |  |
| Comment | Common information which (normally) does not change during a test; therefore all fields are mandatory |  |  |
| Common | NB_CommonStaticCellInfo_Type |  | information common for UL and DL |
| Earfcn | NB_CarrierFreq_Type |  | DL or UL EARFCN as defined in TS 36.101 |

NB_CommonStaticCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CommonStaticCellInfo_Type |  |  |
| Comment | information common for UL and DL; all fields are mandatory |  |  |
| PhysicalCellId | EUTRA_ASN1_PhysCellId_Type |  | N(Ncell, ID): imported from core spec; -> narrowband reference signals -> scrambling of all DL physical channels: NPBCH, NPDCCH, and NPDSCH (together with nRNTI) |
| NB_Band | integer |  |  |
| CellTimingInfo | CellTimingInfo_Type |  |  |
| OperationModeInfo | NB_OperationModeInfo_Type |  |  |
| LteCellInfo | NB_CommonStaticLteCellInfo_Type | opt | Only used for non-standalone operation mode, omitted otherwise. Provides information on the associated E-UTRA cell within which the NBIOT cell is operating, to enable LTE CRS broadcast or skipping (see TS 36.523-3 cl. 7A.1). |

NB_CommonStaticLteCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CommonStaticLteCellInfo_Type |  |  |
| Comment |  |  |  |
| Earfcn | integer |  | E-UTRA cell DL EARFCN. |
| Bandwidth | EUTRA_ASN1_Dl_Bandwidth_Type |  | E-UTRA cell DL bandwidth. |
| CyclicPrefix | EUTRA_CyclicPrefix_Type |  | E-UTRA cell cyclic prefix. |
| PhysicalCellId | EUTRA_ASN1_PhysCellId_Type | opt | E-UTRA cell physical cell identity. If omitted, it is the same identity as the NBIOT cell identity. |
| EutraControlRegionSize | NB_EutraControlRegionSize_Type | opt | Only used for the in-band operation modes, omitted otherwise. Indicates the control region size of the E-UTRA cell. |

### F.1.3.2	Downlink_Physical_Layer_Configuration

Downlink physical layer configuration:
- DL antenna configuration
- control region (NPDCCH)
- primary/secondary sync signals
- power control for physical channels and signals

AntennaConfig_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | AntennaConfig_Type |
| Comment | This type specifies how many Tx antennas shall be used by the SS |
| AN1 | single Tx antenna shall be used by the SS |
| AN2 | two Tx antennas shall be used by the SS |

NB_PhysicalLayerConfigDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PhysicalLayerConfigDL_Type |  |  |
| Comment | all fields are declared as optional to allow single reconfigurations; in this case omit means "keep as it is" |  |  |
| Antenna | AntennaConfig_Type | opt |  |
| NPbch | NPbchConfig_Type | opt |  |
| NPdcch | NPdcchConfig_Type | opt |  |
| NPdsch | NPdschConfig_Type | opt |  |
| NPss | NB_PrimarySyncSignal_Type | opt |  |
| NSss | NB_SecondarySyncSignal_Type | opt |  |
| LteCrs | NB_LTE_CellSpecificReferenceSignal_Type | opt | if omitted in initial configuration the CRS shall be considered as being not present, i.e. shall not be transmitted by the SS |
| NWus | NB_WUS_Config_Type | opt | if omitted in initial configuration, the narrow band Wake-Up Signal shall be considered as being not present; if is present WUS shall be transmitted by the SS at the activation time given by WakeUpSignal trigger |

NB_DownlinkBitmapConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_DownlinkBitmapConfig_Type |  |
| Comment | to specify valid NB-IoT DL subframes according to TS 36.213 clause 16.4 |  |
| Bitmap | DL_Bitmap_Type | valid NB-IoT DL subframe specified by bitmap and does not contain NPSS/NSSS/NPBCH/NB-SIB1 transmission |
| None | Null_Type | valid NB-IoT DL subframe does not contain NPSS/NSSS/NPBCH/NB-SIB1 transmission |

NB_NonAnchorCarrier_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_NonAnchorCarrier_Type |  |
| Comment |  |  |
| Config | CarrierConfigDedicated_Type | SS shall apply given configuration of non-anchor-carrier in accordance with TS 36.331 clause 5.3.10.6 and TS 36.321 clause 5.1.1 |
| None | Null_Type |  |

#### F.1.3.2.1	Physical_Channels

Physical_Channels: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NPdcchSearchSpaceCandidateIndex_Type | integer (0..7) | Index of the search space candidate to be used for UL grant or DL assignment (corresponds to "u" in the calculations of clause 16.6 in TS 36.213) |

NPbchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPbchConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NB_ToRS_EPRE_Ratio_Type | opt | power ratio for NPBCH's resource elements relative to the NRS |

NPdcchFormat_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NPdcchFormat_Type |
| Comment | NPDCCH format according to 36.211 claues 10.2.5.1 => aggregation level according to 36.213 clause 16.6 |
| npdcchFormat0 |  |
| npdcchFormat1 |  |

NPdcchUESpecificSearchSpace_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdcchUESpecificSearchSpace_Type |  |  |
| Comment | parameters for UE specific search space (UESS, TS 36.213 clause 16.6); In addition the SS needs to consider the "DCISubFrameRepetitionNumber" in the DCI UL/DL configuration to determine the value of R (actual number of NPDCCH repetitions for the DCI) |  |  |
| NPdcchFormat | NPdcchFormat_Type |  | Aggregation level: According to tables 16.6-1, 16.6-2, 16.6-3 in TS 36.213 for typical search space configuration as per TS 36.508 NPDCCH format 1 is used i.e. there is just one search space candidate per NPDCCH transmission. Use cases for NPDCCH format 0 are FFS |
| NumRepetitions | NPDCCH_NumRepetitions_UESS_Type |  | Rmax for UE specific search space; TS 36.213 clause 16.6 |
| StartSF | NPDCCH_StartSF_UESS_Type |  | G for UE specific search space; TS 36.213 clause 16.6 |
| Offset | NPDCCH_Offset_UESS_Type |  | offset for UE specific search space; TS 36.213 clause 16.6 |
| SearchSpaceCandidateForDL | NPdcchSearchSpaceCandidateIndex_Type |  | index of the search space candidate to be used for DL assignment |
| SearchSpaceCandidateForUL | NPdcchSearchSpaceCandidateIndex_Type |  | index of the search space candidate to be used for UL grant |

NPdcchType1CommonSearchSpace_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdcchType1CommonSearchSpace_Type |  |  |
| Comment | parameters for Type1 common search space for paging (Type1CSS, TS 36.213 clause 16.6); In addition the SS needs to consider the "DCISubFrameRepetitionNumber" in the DCI DL configuration to determine the value of R (actual number of NPDCCH repetitions for the DCI) |  |  |
| NumRepetitions | NPDCCH_NumRepetitions_Type1CSS_Type |  | Rmax for Type1 common search space; TS 36.213 clause 16.6 |

NPdcchType2CommonSearchSpace_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdcchType2CommonSearchSpace_Type |  |  |
| Comment | parameters for Type2 common search space for RACH procedure (Type2CSS, TS 36.213 clause 16.6); In addition the SS needs to consider the "DCISubFrameRepetitionNumber" in the DCI DL configuration to determine the value of R (actual number of NPDCCH repetitions for the DCI); NOTE: NumRepetitions, StartSF and Offset are also included in each entry of the nprach-ParametersList of NPRACH_Config_Type (NB_PhysicalLayerConfigUL_Type); nevertheless NPRACH_Config_Type contains a list of NPRACH resources whereas the SS needs to consider only one search space corresponding to the NPRACH resource the UE shall select |  |  |
| NumRepetitions | NPDCCH_NumRepetitions_Type2CSS_Type |  | Rmax for Type2 common search space; TS 36.213 clause 16.6 |
| StartSF | NPDCCH_StartSF_Type2CSS_Type |  | G for Type2 common search space; TS 36.213 clause 16.6 |
| Offset | NPDCCH_Offset_Type2CSS_Type |  | offset for Type2 common search space; TS 36.213 clause 16.6 |
| SearchSpaceCandidateForRAR | NPdcchSearchSpaceCandidateIndex_Type |  | index of the search space candidate to be used for random access response |
| SearchSpaceCandidateForMsg4 | NPdcchSearchSpaceCandidateIndex_Type |  | index of the search space candidate to be used for Msg4 scheduling; this can be an DL transmission (contention resolution id based) or an UL transmission (C-RNTI based); in case of early contention resolution the SS shall use this index also for the subsequent DL transmission |

NPdcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdcchConfig_Type |  |  |
| Comment |  |  |  |
| UESS | NPdcchUESpecificSearchSpace_Type | opt | parameters for UE specific search space |
| Type1CSS | NPdcchType1CommonSearchSpace_Type | opt | parameters for Type1 common search space |
| Type2CSS | NPdcchType2CommonSearchSpace_Type | opt | parameters for Type2 common search space |
| RelativeTxPower | NB_ToRS_EPRE_Ratio_Type | opt | power ratio for NPDCCH's resource elements relative to the NRS |

NPdschRelativeTxPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdschRelativeTxPower_Type |  |  |
| Comment | FFS |  |  |
| RachResponse | NB_ToRS_EPRE_Ratio_Type | opt |  |
| BcchOnNPdsch | NB_ToRS_EPRE_Ratio_Type | opt |  |
| PcchOnNPdsch | NB_ToRS_EPRE_Ratio_Type | opt |  |
| CcchOnNPdsch | NB_ToRS_EPRE_Ratio_Type | opt |  |
| DcchDtchOnNPdsch | NB_ToRS_EPRE_Ratio_Type | opt |  |

NPdschConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdschConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NPdschRelativeTxPower_Type | opt |  |

#### F.1.3.2.2	Physical_Signals

NB_PrimarySyncSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PrimarySyncSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NB_ToRS_EPRE_Ratio_Type | opt | power ratio for NPSS's resource elements relative to the NRS (NOTE: applicable even though NRS is not in the same subframe) |

NB_SecondarySyncSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SecondarySyncSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NB_ToRS_EPRE_Ratio_Type | opt | power ratio for NPSS's resource elements relative to the NRS (NOTE: applicable even though NRS is not in the same subframe) |

NB_LTE_CellSpecificReferenceSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_LTE_CellSpecificReferenceSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NB_ToRS_EPRE_Ratio_Type | opt | power ratio for the LTE CRS in case of In-band transmission with same PCI |

NWUS_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NWUS_Config_Type |  |  |
| Comment |  |  |  |
| TimeOffset | TimeOffsetInFrames |  | Number of frames used to calculate start of the first PO to which the NWUS is associated as per 36.211 cl 10.2.6B.1 and 36.304 cl 7.4 |
| WUS_Duration | WUS_Durations_Type |  | Actual duration of NWUS in subframes, corresponds to M in 36.211 cl. 10.2.6B.1 |
| RelativeTxPower | WUS_PowerBoost_Type | opt | The ratio of NWUS EPRE to NRS EPRE. If omit, default value of 0 dB shall be used |

NB_WUS_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_WUS_Config_Type |  |  |
| Comment |  |  |  |
| NWUS_Config | NWUS_Config_Type | opt |  |

### F.1.3.3	Uplink_Physical_Layer_Configuration

Uplink physical channel configuration: NPRACH, NPUSCH and UL RS

NB_SubCarrierSpacingUL_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NB_SubCarrierSpacingUL_Type |
| Comment |  |
| subCarrierSpacing_15kHz |  |
| subCarrierSpacing_3_75kHz |  |

NPUSCH_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPUSCH_Configuration_Type |  |  |
| Comment |  |  |  |
| Common | NPUSCH_ConfigCommon_Type | opt |  |
| Dedicated | NPUSCH_ConfigDedicated_Type | opt |  |

NB_PhysicalLayerConfigUL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PhysicalLayerConfigUL_Type |  |  |
| Comment | NOTE: In general SS is required to keep the UE's UL power constant |  |  |
| NPrach | NPRACH_Config_Type | opt | parameters acc. TS 36.331, clause 6.7.3.2 |
| NPusch | NPUSCH_Configuration_Type | opt | parameters acc. TS 36.331, clause 6.7.3.2 (including configuration of RS) |
| TimingAdvance | SS_TimingAdvanceConfig_Type | opt | to adjust timing advance; at cell initialization absolute timing advance is configured as 0; absolute timing advance may be modified as part of the Random Access procedure configuration; in some MAC test cases timing advance may be configured to a non-zero (11 bit value) at the beginning and modified by (6 bit) timing advance commands during the test |
| SubCarrierSpacingUL | NB_SubCarrierSpacingUL_Type | opt | 15kHz or 3.75kHz UL carrier spacing; cyclic prefix as per 36.211 Table 10.1.5-1 accordingly |

### F.1.3.4	Common_MAC_Configuration

Transport channel and MAC related procedures and configuration

Common_MAC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NPDSCH_ImcsValue_Type | integer | Modulation and coding scheme index coding; TS 36.213 Table 16.4.1.5.1-1 |
| NPUSCH_ImcsValue_Type | integer | Modulation and coding scheme index coding; TS 36.213 Table 16.5.1.2-2 |

NB_PdcchDciFormat_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NB_PdcchDciFormat_Type |
| Comment |  |
| dci_N0 | physical layer parameters acc. TS 36.508 Table 8.1.3.6.1.1-1 (for NPUSCH) |
| dci_N1 | physical layer parameters acc. TS 36.508 Table 8.1.3.6.1.1-3 (for NPDSCH) |
| dci_N2 | physical layer parameters acc. TS 36.508 Table 8.1.3.6.1.1-3 (for Paging) |

NB_CRC_ErrorMode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NB_CRC_ErrorMode_Type |
| Comment |  |
| noError | SS shall not generate any CRC error |
| crcErrorWithRetransmission | SS shall generate CRC error for DL transmissions and schedule retransmissions for each HARQ NACK |
| crcErrorWithoutRetransmission | SS shall generate CRC error for DL transmissions but not do any retransmissions |

NB_DciDlInfoCommon_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DciDlInfoCommon_Type |  |  |
| Comment | Downlink control information according to 36.212 clause 6.4.3 |  |  |
| Format | NB_PdcchDciFormat_Type |  | NPDSCH: N1; Paging: N2 |
| RepetitionNumber | integer |  | Defined in TS 36.213, clause 16.4.1.3 |
| DCISubFrameRepetitionNumber | integer |  | Defined in TS 36.213, clause 16.6 |
| SchedulingDelay | integer | opt | Defined in TS 36.213, clause 16.4.1 Present for N1; not present for N2 (Paging) |
| HARQ_ACKResource | integer | opt | Defined in TS 36.213, clause 16.4.2 present for N1; not present for N2 (Paging) |
| CRC_ErrorMode | NB_CRC_ErrorMode_Type |  | no CRC error, CRC error with or without subsequent retransmission by the SS |
| HARQ_ProcessNumber | integer | opt | 1 bit to be added by the SS to the DCI in case of DCI format N1 being mapped onto the UE specific search space given by the C-RNTI (e.g. in case of RA_RNTI the SS shall skip this field even when it is present); if present, HARQ_ProcessNumber is 0 or 1 addressing the HARQ process to be used; see TS 36.212 clause 6.4.3.2 Note: When two HARQ processes are configured at the UE, this field is provided to the SS otherwise it is omit (-> PhysicalConfigDedicated_NB_r13.twoHARQ_ProcessesConfig_r14:=true) |

NB_DciDlInfoExplicit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DciDlInfoExplicit_Type |  |  |
| Comment | Downlink control information according to 36.212 clause 6.4.3 |  |  |
| Format | NB_PdcchDciFormat_Type |  | NPDSCH: N1; Paging: N2 |
| Imcs | NPDSCH_ImcsValue_Type |  | MCS index of TS 36.213 Table 16.4.1.5.1-1 |
| ResourceAssignment | integer |  | Defined in TS 36.213, clause 16.4.1.3: to determine the number of subframes |
| RepetitionNumber | integer |  | Defined in TS 36.213, clause 16.4.1.3 |
| DCISubFrameRepetitionNumber | integer |  | Defined in TS 36.213, clause 16.6 |
| SchedulingDelay | integer | opt | Defined in TS 36.213, clause 16.4.1 Present for N1; not present for N2 (Paging) |
| HARQ_ACKResource | integer | opt | Defined in clause 16.4.2 present for N1; not present for N2 (Paging) |
| CRC_ErrorMode | NB_CRC_ErrorMode_Type |  | no CRC error, CRC error with or without subsequent retransmission by the SS |
| HARQ_ProcessNumber | integer | opt | (see NB_DciDlInfoCommon_Type) |

NB_DciDlInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_DciDlInfo_Type |  |
| Comment |  |  |
| Auto | NB_DciDlInfoCommon_Type | SS shall choose the appropriate TBS |
| Explicit | NB_DciDlInfoExplicit_Type | used in MAC or RAB tests where exact TBS needs to be specified |

NB_AdaptiveHarqNack_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_AdaptiveHarqNack_Type |  |  |
| Comment |  |  |  |
| RedundancyVersion | integer |  | to be used in DCI for HARQ NACK to request UL restransmission |

NB_UL_TransRetransmission_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_UL_TransRetransmission_Type |  |
| Comment |  |  |
| NewTransmission | Null_Type | new transmission of data with redundancy version RV=0 (acc. to TS 36.321 clause 5.4.2.2); NDI is toggled |
| ReTransmissionAdaptive | NB_AdaptiveHarqNack_Type | DCI requesting retransmission (NDI not toggled) |

NB_UL_TransRetransmissionList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_UL_TransRetransmissionList_Type |
| Comment | to allow multiple retransmissions |
| record length (1..infinity) of NB_UL_TransRetransmission_Type |  |

NB_DciUlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DciUlInfo_Type |  |  |
| Comment | Downlink control information according to 36.212 clause 6.4.3 |  |  |
| SubCarrierIndication | integer |  | Defined in TS 36.213, clause 16.5.1.1 |
| ResourceAssignment | integer |  | Defined in TS 36.213, clause 16.5.1.2 |
| SchedulingDelay | integer |  | Defined in TS 36.213, clause 16.5.1 |
| Imcs | NPUSCH_ImcsValue_Type |  | Defined in TS 36.213, clause 16.5.1.2 |
| RedundancyVersion | integer |  | Defined in TS 36.213, clause 16.5.1.2 |
| RepetitionNumber | integer |  | Defined in TS 36.213, clause 16.5.1.1 |
| DCISubFrameRepetitionNumber | integer |  | Defined in TS 36.213, clause 16.6 |
| TransRetransmissionList | NB_UL_TransRetransmissionList_Type |  | for possible adaptive restransmissions: when there is more than one entry in the list the SS shall sent DCIs for the subsequent entries as ACK/NACK response according to TS 36.213 clause 16.5.2 |
| HARQ_ProcessNumber | integer | opt | 1 bit to be added by the SS to the DCI if the corresponding DCI format is mapped onto the UE specific search space given by the C-RNTI if present, HARQ_ProcessNumber is 0 or 1 addressing the HARQ process to be used; see TS 36.212 clause 6.4.3.1 Note: When two HARQ processes are configured at the UE, this field is provided to the SS otherwise it is omit (-> PhysicalConfigDedicated_NB_r13.twoHARQ_ProcessesConfig_r14:=true) |

NB_PeriodicGrant_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PeriodicGrant_Type |  |  |
| Comment | configuration of UL grants to be assigned periodically acc. to configuration of the UE specific search space; DL assignments have higher priority than UL grant schedules: when there is a DL transmission to be scheduled for some search space the UL grant shall be scheduled in the next search space which is not used for DL assignment |  |  |
| DciInfo | NB_DciUlInfo_Type |  | DCI format: N0 (TS 36.212, clause 6.4.3.1) |
| NoOfRepetitions | TransmissionRepetition_Type |  | number of UL Grants to be automatically transmitted or continuous repetition |
| Periodicity | integer | opt | 1 => every search space, 2 => every 2nd search space, 3 => every 3rd search space, etc. may be omitted if only one UL grant shall be assigned and shall be ignored by the SS in this case; the periodicity is related to beginning of the UL grant scheduling, i.e. independent from whether or single grant is postponed due to pending DL assignment |

NB_UESS_GrantScheduling_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_UESS_GrantScheduling_Type |  |
| Comment | Configuration of (periodic) UL grants for the UE specific search space; NOTE: UL grants for Type2 common search space during RACH procedure are configured as part of the RACH procedure configuration |  |
| None | Null_Type | no UL grant configured => UE needs to trigger RACH procedure to get UL resources; to used also to stop periodic UL grant scheduling when the UE goes back to IDLE |
| Periodic | NB_PeriodicGrant_Type | configuration of one or several grants |
| PeriodicAfterRachContResolution | NB_PeriodicGrant_Type | The SS shall schedule UL grants periodically after sending the DCI for contention resolution: a) contention resolution id based contention resolution: DCI is DL assignment of Msg4 sent to the UE b) C-RNTI based contention resolution: DCI is grant for UL transmission The periodicity starts from the search space of DCI used for contention resolution NOTE: for now it is not foreseen that the SS needs to evaluate BSRs for (automatic) UL grant assignments; FFS |

### F.1.3.5	Random_Access_Procedure

NB_RAR_UplinkGrant_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RAR_UplinkGrant_Type |  |  |
| Comment | TS 36.213, clause 16.3.3 First bit for subcarrier spacing shall be set according to SubCarrierSpacingUL in NB_PhysicalLayerConfigUL_Type |  |  |
| SubCarrierIndication | B6_Type |  | Defined in TS 36.213 clause 16.5.1.1 |
| SchedulingDelay | B2_Type |  | Defined in TS 36.213 clause 16.5.1 where NB-IoT DL subframe n is the last subframe in which the NPDSCH associated with the NB random access response grant is transmitted |
| Msg3RepetitionNumber | B3_Type |  | Defined in TS 36.213 clause 16.5.1.1 |
| MCSIndex | B3_Type |  | Indicating TBS, modulation and number of RUs for Msg3 according to TS 36.213 table 16.3.3-1 |

NB_CRNTI_ContentionResolutionCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_CRNTI_ContentionResolutionCtrl_Type |  |
| Comment | configuration for Random Access Procedure in RRC_CONNECTED (see TS 36.300, clause 10.1.5.1); when SS receives C-RNTI MAC element sent by the UE after Random Access Response, SS shall deal with the C-RNTI as specified in this structure |  |
| AutomaticGrant | NB_DciUlInfo_Type | before expiry of the contention resolution timer SS shall automatically send an UL grant to the UE by addressing NPDCCH using C-RNTI as sent by the UE in Msg3 but scheduled according to Type2 common search space; the UL grant is explicitly specified by NB_DciUlInfo_Type |
| None | Null_Type | Used in case of dedicated preamble transmission (contention free random access) or to simulate failure cases for contention based random access procedure; SS shall not address PDCCH using C-RNTI => in case of contention based random access procedure: expiry of contention resolution timer on UE side |

NB_ContentionResolutionCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_ContentionResolutionCtrl_Type |  |
| Comment | NOTE: SS only needs to consider one kind of contention resolution at one time; in the initial configuration of a cell TCRNTI_Based shall be configured; whether or not a UE triggers RACH procedures in RRC_CONNECTED depends on RRC signalling (logicalChannelSR-Prohibit, logicalChannelSR-ProhibitTimer); when the UE is supposed to trigger RACH procedures in RRC_CONNECTED the SS needs to be configured for C-RNTI based contention resolution after the temporary C-RNTI based contention resolution has been done at RRC connection establishment |  |
| TCRNTI_Based | TCRNTI_ContentionResolutionCtrl_Type | TCRNTI based contention resolution (e.g. initial access), hence involves inclusion contention resolution identity in DL message 4 of RACH procedure; in case of early contention resolution the SS shall schedule the subsequent DL transmission in the Type2 common search space; NOTE: The subsequent DL transmission can be a CCCH message (e.g. RRC Connection Setup) but also a DCCH message (RRC Connection Resume) in which case according to the NOTE in TS 36.331 clause 5.3.3.3a the UE uses the common search space until successful connection resumption |
| CRNTI_Based | NB_CRNTI_ContentionResolutionCtrl_Type | CRNTI based contention resolution (e.g. in case UE is being in RRC_CONNECTED) or contention free random access: in case of CRNTI based contention resolution the uplink message in step 3 (of RACH procedure) is followed by PDCCH transmission with UE C-RNTI to end procedure |

NB_RandomAccessResponseParameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RandomAccessResponseParameters_Type |  |  |
| Comment | paramenters to control content of RAR sent to the UE NOTE: in case SS detects a PRACH using preamble format 2 the SS shall use the MAC RAR as defined in TS 36.321 Figure 6.1.5-3c |  |  |
| RapId | RAR_RapIdCtrl_Type |  | to control Random Access Preamble Id to be sent back to the UE; used in RAR MAC sub-header |
| InitialGrant | NB_RAR_UplinkGrant_Type |  | initial UL grant |
| TimingAdvance | RACH_TimingAdvance_Type |  | timing advance: granularity of 0.52 micro sec (16*Ts); see TS 36.300, clause 5.2.7.3, TS 36.321, clause 6.1.3.5; NOTE: timing advance has impact not only on the RA procedure; SS in general needs to adjust its timing accordingly |
| TempC_RNTI | TempC_RNTI_Type |  | NOTE: For initial Random Access Procedure at network (SS) side there is no temporary C-RNTI: network assigns the C-RNTI which is used by any UE as being temporary; the UE which 'wins' the contention resolution keeps the (temporary) C-RNTI; other UEs need to repeat the RACH procedure; => at the SS the TempC_RNTI shall be 'SameAsC_RNTI' For Random Access Procedure in RRC_CONNECTED state the NW assigns a temporary C-RNTI which is replaced by the one stored at the UE; => TempC_RNTI may be 'SameAsC_RNTI' (in this case temp. C-RNTI and C-RNTI are equal what is not likely in a real network), or there is an explicit temp. C-RNTI what is used during RA procedure only (as in a real network) |

NB_RarList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_RarList_Type |
| Comment | in general MAC PDU may contain one or several RARs; normally only one RAR is contained |
| record of NB_RandomAccessResponseParameters_Type |  |

NB_RandomAccessResponse_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RandomAccessResponse_Type |  |
| Comment |  |  |
| None | Null_Type | used for unsuccessful RA procedure |
| List | NB_RarList_Type | normally one RAR to be sent to the UE; in general there can be more than one RAR |

NB_RandomAccessResponseCtrl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RandomAccessResponseCtrl_Type |  |  |
| Comment | configuration for Random Access Response mapped to DL-SCH mapped to NPDSCH TransmissionMode: single antenna mode when there is only one antenna configured, transmit diversity else; RNTI: RA-RNTI (TS 36.321, clause 7.1); if both RAR msg and backoff indicator are 'None' SS shall not respond on random access preamble |  |  |
| DciInfo | NB_DciDlInfoCommon_Type |  | DCI format N1 addressed by RA-RNTI |
| Rar | NB_RandomAccessResponse_Type |  | RAR to be sent to the UE |
| BackoffInd | RandomAccessBackoffIndicator_Type |  | possible backoff indicator; 'None' for normal cases |

NB_RandomAccessResponseConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RandomAccessResponseConfig_Type |  |
| Comment |  |  |
| Ctrl | NB_RandomAccessResponseCtrl_Type | contains information to control sending of RAR |
| None | Null_Type | to be used when there is no RAR to be sent at all |

NB_RachProcedure_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RachProcedure_Type |  |  |
| Comment |  |  |  |
| RAResponse | NB_RandomAccessResponseConfig_Type |  | control of how the SS shall react on RA preamble; this may be - the RAP id as expected by the UE - a RAP id not matching to the UE's RAP - a backoff indicator - nothing at all |
| ContentionResolutionCtrl | NB_ContentionResolutionCtrl_Type |  |  |

NB_RachProcedureList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_RachProcedureList_Type |
| Comment | to simulate RACH procedure with one or more than one attempt by the UE: 1. Normal cases: one single RandomAccessResponse is sent to the UE matching the UE's RACH preamble; contention resolution is successful immediately => list contains only one element which is used for any RA procedure (Even if a RACH procedure is repeated by the UE for any reason this element shall be used; e.g. it needs not to be handled as error when the UE sends another RACH preamble instead of the RRC connection request message) 2. Special cases: SS shall start with the first element in the list and use the RAR as specified in this element; if the RAR matches at the UE side the UE will send UL data and contention resolution is performed as configured for this element; if the RAR does not match the UE sends another RAP and SS continues with the next element in the list; in this case the contention resolution of the respective element is not used; if the end of the list is reached and further RACH preambles are sent by the UE SS shall repeatively apply the last element of the list (this is necessary because there might be not enough time to reconfigure SS after the end of the list has been reached and there shall be well-defined behaviour after the list has been processed); to change from a special mode to normal mode the RachProcedureList is reconfigured by TTCN to achieve transparency and readability of the code; NOTE: for NB-IoT there can be more than one repetition per preamble attempt (numRepetitionsPerPreambleAttempt-r13 in NPRACH-ConfigSIB-NB-DEFAULT in SystemInformationBlockType2-NB); the SS gets configured with the nprach-ParametersList (NPRACH_Config_Type in NB_PhysicalLayerConfigUL_Type) and it is up to SS implementation how to get out how many preamble repetitions an attempt has; => NB_PhysicalLayerConfigUL_Type contains one entry per attempt and the SS shall report the number of preambles per attempt in a RachPreamble indication (if configured) |
| record of NB_RachProcedure_Type |  |

NB_RachProcedureConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RachProcedureConfig_Type |  |  |
| Comment | parameters to control the random access procedure; TS 36.321, clause 5.1 |  |  |
| RACH_ConfigCommon | NB_RACH_ConfigCommon_Type | opt | acc. TS 36.331, clause 6.7.3.2; may not be necessary for SS; omit: "keep as it is" |
| RachProcedureList | NB_RachProcedureList_Type | opt | in normal cases there is one element which is used for any RA procedure; special cases are used in MAC test cases; omit means "keep as it is" |

NB_RA_NPDCCH_Order_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RA_NPDCCH_Order_Type |  |  |
| Comment | TS 36.212, clause 6.4.3.2 |  |  |
| NprachRepetitions | integer |  | TS 36.213, Table 16.3.2-1: 0, 1, 3 |
| SubcarrierIndex | integer |  | ra-PreambleIndex (acc. to 36.321 cl. 5.1.2) as indicated in subcarrier indication field of the DCI (36.213 cl. 16.3.2 and 36.213 cl. 6.4.3.2): NOTE: the corresponding RAPID gets calculated according to 36.321 cl. 5.1.2 |

### F.1.3.6	System_Information_Control

Primitive to configuration BCCH/BCH

NB_SiSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SiSchedul_Type |  |  |
| Comment | for specific SI scheduling and repetitions |  |  |
| Periodicity | NB_SiPeriodicity_Type | opt |  |
| RepetitionPattern | NB_SiRepetitionPattern_Type | opt | Indicates the starting radio frames within the SI window used for SI message transmission |
| SchedulingInfoSI | NB_SiTransportBlockSize_Type | opt | This field indicates the transport block size in number of bits used to broadcast the SI message |

NB_SiSchedulList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_SiSchedulList_Type |
| Comment |  |
| record length(1..maxSI_Message_NB_r13) of NB_SiSchedul_Type |  |

NB_AllSiSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_AllSiSchedul_Type |  |  |
| Comment | SI-message scheduling according to TS 36.331, clause 5.1.2.1a and clause 5.2.3a |  |  |
| WindowLength | NB_SiWindowLength_Type | opt | Common window size for all Sis to calculate start of each SI window acc. TS 36.331, clause 5.2.3a |
| RadioFrameOffset | integer | opt | Integer (0..15); Frame offset for scheduling of SI-messages (see TS 36.331, clause 5.2.3a) |
| SiList | NB_SiSchedulList_Type | opt | List of scheduling info for the SIs containing one or more SIBs |

SIScheduleConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SIScheduleConfig_Type |  |  |
| Comment | configuration for BCCH mapped to DL-SCH mapped to NPDSCH SIB1-NB: fixed scheduling in time domain according to TS 36.331 clause 5.2.1.2a (subframe 4, periodicity 2560ms, repetitions in every other frame in 160ms) |  |  |
| SchedulingInfoSIB1_NB | NB_SchedulingInfoSIB1_Type | opt | Integer (0..15) TBSize and Repetition number refer to TS 36.213 table 16.4.1.5.2-1 |
| SiSchedul | NB_AllSiSchedul_Type | opt | scheduling of SIs and TBSize in time domain |

NB_SI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_SI_List_Type |
| Comment | TS 36.331, clause 6.7.1 BCCH-DL-SCH-Message-NB and clause 6.7.2 SystemInformation-NB |
| record of BCCH_DL_SCH_Message_NB |  |

NB_BcchInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_BcchInfo_Type |  |  |
| Comment | all fields are declared as optional to allow modification of single field; acc. to TS 36.331, clause 9.1.1.1 "RRC will perform padding, if required due to the granularity of the TF signalling, as defined in 8.5."; therefore this needs to be done by the system simulator |  |  |
| MIB_NB | BCCH_BCH_Message_NB | opt | TS 36.331, clause 6.7.1 BCCH-BCH-Message_NB and clause 6.7.2 MasterInformationBlock-NB; NOTE: the sequence numbers included in MIB-NB need to be handled and maintained by the system simulator; that means that the sequence numbers being setup by TTCN will be overwritten by SS |
| SIB1_NB | BCCH_DL_SCH_Message_NB | opt | TS 36.331, clause 6.7.1 BCCH-DL-SCH-Message-NB and clause 6.7.2 SystemInformationBlockType1-NB |
| SIs | NB_SI_List_Type | opt | list of SIs corresponding to SiList of AllSiSchedul_Type (i.e. element i of AllSiSchedul_Type's SiList specifies the scheduling for SIs[i]) |

NB_BcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_BcchConfig_Type |  |  |
| Comment | all fields are optional to allow single modifications; activation time may be applied in the common part of the ASP; NOTE 1: mapping/scheduling and contents of the System Information in general is done in one go (i.e. there are no separate ports for SIB data and configuration) |  |  |
| SIConfig | SIScheduleConfig_Type | opt |  |
| BcchInfo | NB_BcchInfo_Type | opt |  |

### F.1.3.7	Paging_Control

Primitive to configuration PCCH/PCH

NB_PcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PcchConfig_Type |  |  |
| Comment | configuration for PCCH mapped to NPCH TransmissionMode: single antenna mode when there is only one antenna configured, transmit diversity else; RNTI: P-RNTI (TS 36.321, clause 7.1) NOTE: acc. to TS 36.331, clause 9.1.1.3 there is no PDCP and RLC/MAC are in TM |  |  |
| DciInfoPaging | NB_DciDlInfoCommon_Type | opt | DCI format: N2, Paging (TS 36.212, clause 6.4.3.3) |
| NoOfDciRepetitionsForDirectIndication | integer | opt | Absolute number of NPDCCH repetitions (R) for direct indication |

### F.1.3.8	NonAnchorCarrier_Common

configuration of non-anchor carrier for paging and RACH procedure according to rel-14 enhancements

NB_NonAnchorCarrier_SearchSpaceType1CSS_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_NonAnchorCarrier_SearchSpaceType1CSS_Type |  |
| Comment |  |  |
| SameAsAnchorCarrier | Null_Type | SS shall use the same Type1 common search space configuration as for the anchor carrier (-> NPdcchConfig_Type) |
| Explicit | NPdcchType1CommonSearchSpace_Type | to be used when test case requires different search space configuration for anchor and non-anchor configuration |

NB_NonAnchorCarrier_SearchSpaceType2CSS_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_NonAnchorCarrier_SearchSpaceType2CSS_Type |  |
| Comment |  |  |
| SameAsAnchorCarrier | Null_Type | SS shall use the same Type2 common search space configuration as for the anchor carrier (-> NPdcchConfig_Type)) |
| Explicit | NPdcchType2CommonSearchSpace_Type | to be used when test case requires different search space configuration for anchor and non-anchor configuration |

NB_NonAnchorCarrierCommonDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_NonAnchorCarrierCommonDL_Type |  |  |
| Comment |  |  |  |
| Id | integer |  | identifier to address the carrier to be used for paging (-> NB_PagingTrigger_Type) |
| ConfigCommon | NB_DL_ConfigCommon_NB_Type | opt | ASN.1 configuration as provided to the UE |
| Type1CSS | NB_NonAnchorCarrier_SearchSpaceType1CSS_Type | opt | Type1CSS search space configuration to be applied by the SS for the given non-anchor carrier |

NB_NonAnchorCarrierCommonDL_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_NonAnchorCarrierCommonDL_List_Type |
| Comment |  |
| record of NB_NonAnchorCarrierCommonDL_Type |  |

NB_CarrierConfigCommonDL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_CarrierConfigCommonDL_Type |  |
| Comment |  |  |
| Config | NB_NonAnchorCarrierCommonDL_List_Type |  |
| None | Null_Type |  |

NB_NpdcchCarrier_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_NpdcchCarrier_Type |  |
| Comment | to specify the DL carrier corresponding to the UL carrier on which the RACH preamble has been received |  |
| AnchorCarrier | Null_Type | NPDCCCH on anchor carrier |
| Id | integer | identifier of a DL non-anchor carrier carrying the NPDCCH |

NB_NonAnchorCarrierCommonUL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_NonAnchorCarrierCommonUL_Type |  |  |
| Comment |  |  |  |
| Id | integer |  | identifier to indicate the carrier on which the UE has sent a RACH preamble (-> NB_RachPreamble_Type) |
| ConfigCommon | NB_UL_ConfigCommon_NB_Type | opt | ASN.1 configuration as provided to the UE |
| Type2CSS | NB_NonAnchorCarrier_SearchSpaceType2CSS_Type | opt | Type2CSS search space configuration to be applied by the SS for the given non-anchor carrier |
| NpdcchCarrierId | NB_NpdcchCarrier_Type | opt | identifier of the corresponding DL carrier; NOTE 1: the DL carrier shall be the same as being addressed by npdcch-CarrierIndex in SIB22 NOTE 2: in general the number and order of carriers being configured at the SS and being contained in SIB22 does not need to be the same |

NB_NonAnchorCarrierCommonUL_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_NonAnchorCarrierCommonUL_List_Type |
| Comment |  |
| record of NB_NonAnchorCarrierCommonUL_Type |  |

NB_CarrierConfigCommonUL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_CarrierConfigCommonUL_Type |  |
| Comment |  |  |
| Config | NB_NonAnchorCarrierCommonUL_List_Type |  |
| None | Null_Type |  |

### F.1.3.9	UE_Specific_Channel_Configuration

NB_DrxCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_DrxCtrl_Type |  |
| Comment | DRX configuration for connected mode (TS 36.321, clause 5.7) |  |
| None | Null_Type | DRX not configured |
| Config | NB_DRX_Config_Type | DRX is configured as signalled to the UE; NOTE: the release branch of DRX-Config in general is not used for configuration of the SS |

NB_CcchDcchDtchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CcchDcchDtchConfig_Type |  |  |
| Comment |  |  |  |
| DL | NB_CcchDcchDtchConfigDL_Type | opt | Scheduling, parameters related to CCCH, DCCH and DTCH in DL |
| UL | NB_CcchDcchDtchConfigUL_Type | opt | Scheduling, parameters related to CCCH, DCCH and DTCH in UL |
| DrxCtrl | NB_DrxCtrl_Type | opt | DRX configuration as sent to the UE (or 'None' when the UE does not support connected mode DRX) |

#### F.1.3.9.1	UE_Specific_Channel_Configuration_DL

Scheduling and other information for CCCH/DCCH/DTCH mapped to DL-SCH mapped to NPDSCH

NB_DownlinkGapConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_DownlinkGapConfig_Type |  |
| Comment | to specify DL gaps for NPDSCH according to TS 36.211 clause 10.2.3.4 |  |
| DownlinkGapConfig | DL_GapConfig_Type | DL gaps shall be used according to the DL gap configuration (TS 36.331 clause 6.7.3.2) |
| None | Null_Type | there are no gaps in DL |

NB_CcchDcchDtchConfigDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CcchDcchDtchConfigDL_Type |  |  |
| Comment | configuration for CCCH/DCCH/DTCH mapped to DL-SCH mapped to NPDSCH RNTI: C-RNTI (TS 36.321, clause 7.1); all fields optional (omit = "keep as it is") since DCI format and modulation may be changed during a test; for initial configuration all fields are mandatory |  |  |
| DciInfo | NB_DciDlInfo_Type | opt |  |
| GapConfig | NB_DownlinkGapConfig_Type | opt | to specify the DL gap configuration for NPDSCH |

#### F.1.3.9.2	UE_Specific_Channel_Configuration_UL

Scheduling information for CCCH/DCCH/DTCH mapped to UL-SCH mapped to NPUSCH

NB_CcchDcchDtchConfigUL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CcchDcchDtchConfigUL_Type |  |  |
| Comment | scheduling for CCCH/DCCH/DTCH mapped to UL-SCH mapped to NPUSCH NOTE 1: for definition of the possible UL grants the location of the NPUSCH (TS 36.211, clause 10.1.3) and the NPRACH (TS 36.211, clause 10.1.6) need to be taken into account; NOTE 2: In contrast to the DL where the scheduling can be done (with consideration of some restrictions) by SS on a per need basis in the UL the scheduling depends on information provided by the UE: e.g. BSR (buffer status report) see TS 36.523-3 clause 7.2 for further information. |  |  |
| UplinkTimeAlignment_Synch | UplinkTimeAlignment_Synch_Type | opt | parameters to control automatic control of timing advance |
| UESS_GrantScheduling | NB_UESS_GrantScheduling_Type | opt | UL grant allocation to be applied |

### F.1.3.10	NB_NTN_Configuration

NB_NTN_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_NTN_K_Offset_Type | integer (0..1023) |  |
| NB_NTN_K_Mac_Type | integer (0..512) |  |

NB_NTN_TimingOffset_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_NTN_TimingOffset_Type |  |  |
| Comment |  |  |  |
| Koffset | NB_NTN_K_Offset_Type | opt | Koffset in ms as defined in TS 36.213 clause 16. If omit, keep as it is. |
| Kmac | NB_NTN_K_Mac_Type | opt | Kmac in ms as defined in TS 36.213 clause 16.6. If omit, keep as it is. |

NB_NTN_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_NTN_Config_Type |  |  |
| Comment |  |  |  |
| NtnTimingOffset | NB_NTN_TimingOffset_Type | opt | to configure PHY/MAC layers with NTN parameters Koffset and Kmac needed for NTN operation. See clause 7A.14.2.3 for further information. |

## F.1.4	Cell_Power_Attenuation

NB_CellAttenuationConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CellAttenuationConfig_Type |  |  |
| Comment |  |  |  |
| CellId | NBIOT_CellId_Type |  |  |
| Attenuation | Attenuation_Type |  |  |
| TimingInfo | TimingInfo_Type | opt |  |

NB_CellAttenuationList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_CellAttenuationList_Type |
| Comment |  |
| record of NB_CellAttenuationConfig_Type |  |

## F.1.5	Radio_Bearer_Configuration

Radio Bearer Configuration: SRBs/DRBs

NB_PDCP_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Transparent | Null_Type | requires PDCP to be configured as transparent => - TTCN uses AS ciphering in both directions, employing the dummy ciphering algorithm - SS does not interpret, insert or remove PDCP headers - SS does not maintain PDCP sequence numbers and state variables |

NB_PDCP_RbConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_RbConfig_Type |  |
| Comment |  |  |
| Srb | Null_Type |  |
| Drb | NB_PDCP_Config_Type | PDCP-Configuration acc. to TS 36.331, clause 6.7.3.2 |

NB_PDCP_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PDCP_ConfigInfo_Type |  |  |
| Comment |  |  |  |
| Rb | NB_PDCP_RbConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | NB_PDCP_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

NB_PDCP_Configuration_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_Configuration_Type |  |
| Comment |  |  |
| None | Null_Type | for SRB0 & SRB1bis no PDCP is configured; |
| Config | NB_PDCP_ConfigInfo_Type |  |

NB_RadioBearerConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RadioBearerConfigInfo_Type |  |  |
| Comment | semantics of omit: "keep as it is" |  |  |
| Pdcp | NB_PDCP_Configuration_Type | opt | for SRB0 & SRB1bis: "Pdcp.None:=true" mandatory for initial configuration; omit means "keep as it is" |
| Rlc | NB_RLC_Configuration_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| LogicalChannelId | LogicalChannelId_Type | opt | DRBs: DTCH-LogicalChannelIdentity as for rb-MappingInfo in DRB-ToAddModifyList (according to 36.331 a value of 3 shall not be used in DRB-ToAddModifyList); SRBs: for SRBs specified configurations acc. to TS 36.331, clause 9.1.2 shall be applied: SRB1: ul-LogicalChannel-Identity = dl-LogicalChannel-Identity = 1 SRB1bis: ul-LogicalChannel-Identity = dl-LogicalChannel-Identity = 3 for SRB0 being mapped to CCCH the LCID is '00000'B acc. to TS 36.321, clause 6.2.1; mandatory for initial configuration; omit means "keep as it is" |
| Mac | NB_MAC_Configuration_Type | opt |  |

NB_RadioBearerConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RadioBearerConfig_Type |  |
| Comment |  |  |
| AddOrReconfigure | NB_RadioBearerConfigInfo_Type | add / re-configure RB - CellId : identifier of the cell being configured RoutingInfo : None TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Release | Null_Type | release RB - CellId : identifier of the cell being configured RoutingInfo : None TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

NB_RadioBearer_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RadioBearer_Type |  |  |
| Comment |  |  |  |
| Id | NB_RadioBearerId_Type |  | either for SRB or DRB |
| L2TestMode | IndicationAndControlMode_Type | opt | to enable/disable L2 testmode: omit : "keep as it is"; "omit" in initial configuration means that L2 testmode is disabled enable : SS shall route uplink data to N_L2DATA port instead of N_SRB port disable : SS shall route uplink data to N_SRB port as usual |
| Config | NB_RadioBearerConfig_Type |  |  |

NB_RadioBearerList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_RadioBearerList_Type |
| Comment | array of SRBs and/or DRBs |
| record length (1..tsc_NB_MaxRB) of NB_RadioBearer_Type |  |

### F.1.5.1	RLC_Configuration

RLC configuration: radio bearer specific

RLC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_SS_RLC_TM_Type | Null_Type | TM to configure SRB0; no parameters to be defined |

NB_RLC_TestModeInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RLC_TestModeInfo_Type |  |
| Comment |  |  |
| TransparentMode_UMDwith5BitSN | Null_Type | shall be set when TTCN expects RLC PDUs as UMD in UL with an SN of 5 bits; valid only when the RLC is configured in TM |

NB_RLC_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RLC_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Info | NB_RLC_TestModeInfo_Type |  |

NB_SS_RLC_AM_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SS_RLC_AM_Type |  |  |
| Comment |  |  |  |
| Tx | NB_UL_AM_RLC_Type | opt | the UE's UL setting to be used in SS's tx direction |
| Rx | NB_DL_AM_RLC_Type | opt | the UE's DL setting to be used in SS's rx direction |

NB_RLC_RbConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RLC_RbConfig_Type |  |
| Comment |  |  |
| AM | NB_SS_RLC_AM_Type |  |
| TM | NB_SS_RLC_TM_Type | normally SRB0 only; may be used for test purposes also |

NB_RLC_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RLC_Configuration_Type |  |  |
| Comment |  |  |  |
| Rb | NB_RLC_RbConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | NB_RLC_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

### F.1.5.2	MAC_Configuration

MAC configuration: radio bearer specific configuration

NB_MAC_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_MAC_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Transparent | Null_Type | In UL and DL the SS' MAC layer is transparent i.e. SS does not add or remove any MAC header |

NB_MAC_LogicalChannelConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_MAC_LogicalChannelConfig_Type |  |  |
| Comment |  |  |  |
| Priority | integer |  | logical channel priority for the DL as described in TS 36.321, clause 5.4.3.1 for the UL |

NB_MAC_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_MAC_Configuration_Type |  |  |
| Comment |  |  |  |
| LogicalChannel | NB_MAC_LogicalChannelConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | NB_MAC_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is"; for none MAC tests "TestMode.None:=true" |

## F.1.6	AS_Security

Primitive for control of AS security

NB_PdcpSQN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PdcpSQN_Type |  |  |
| Comment |  |  |  |
| Format | NB_PdcpCountFormat_Type |  | 5 bit, 7 bit or 12 bit SQN |
| Value | integer |  | SQN value (5 bit, 7 bit or 12 bit SQN) NOTE: in TTCN the test case writer is responsible to deal with potential overflows (e.g. there shall be a "mod 32", "mod 128" or "mod 4096" according to the format) |

NB_PDCP_ActTime_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_ActTime_Type |  |
| Comment | The sequence number in UL and DL for SRB1 should be one more than the present SQN, as Ciphering starts in UL and DL soon after SMC and SMComp; For DRBs it should be the present SQN |  |
| None | Null_Type | No Activation time; to be used if Ciphering is not applied |
| SQN | NB_PdcpSQN_Type | PDCP sequence number |

NB_SecurityActTime_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SecurityActTime_Type |  |  |
| Comment |  |  |  |
| RadioBearerId | NB_RadioBearerId_Type |  |  |
| UL | NB_PDCP_ActTime_Type |  |  |
| DL | NB_PDCP_ActTime_Type |  |  |

NB_SecurityActTimeList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_SecurityActTimeList_Type |
| Comment |  |
| record length (1..tsc_NB_MaxRB) of NB_SecurityActTime_Type |  |

NB_AS_IntegrityInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_AS_IntegrityInfo_Type |  |  |
| Comment | for initial configuration activation time is not needed for integrity protection as all messages in DL after security activation are integrity protected; this means this ASP is invoked before transmission of Security mode command; if there is a integrity violation in UL SS shall set the IndicationStatus in the common ASP part to flag the integrity error (IndicationStatus.Error.Integrity.Pdcp := true); integrity to be provided for each SRB as per core spec |  |  |
| Algorithm | IntegrityProtAlgorithm_Type |  | IntegrityProtAlgorithm_Type being defined in RRC ASN.1 |
| KRRCint | B128_Key_Type |  |  |
| ActTimeList | NB_SecurityActTimeList_Type | opt | omit for initial configuration (i.e. all SRBs to be integrity protected immediately) |

NB_AS_CipheringInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_AS_CipheringInfo_Type |  |  |
| Comment |  |  |  |
| Algorithm | EUTRA_ASN1_CipheringAlgorithm_r12_Type |  | CipheringAlgorithm_Type being defined in RRC ASN.1 |
| KRRCenc | B128_Key_Type |  |  |
| KUPenc | B128_Key_Type |  | KUPenc is used by SS when DRBs are configured |
| ActTimeList | NB_SecurityActTimeList_Type |  |  |

NB_AS_SecStartRestart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_AS_SecStartRestart_Type |  |  |
| Comment |  |  |  |
| Integrity | NB_AS_IntegrityInfo_Type | opt | optional to allow separated activation of integrity and ciphering; omit: keep as it is |
| Ciphering | NB_AS_CipheringInfo_Type | opt | optional to allow separated activation of integrity and ciphering; omit: keep as it is |

NB_AS_Security_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_AS_Security_Type |  |
| Comment | Security mode command procedure (TS 36.331, clause 5.3.4): both SMC and SMComp are integrity protected (nevertheless SS shall be able to cope with unprotected SM reject); ciphering is started just after SMComp (acc. to TS 36.331, clause 5.3.4.3 and 5.3.1.1) |  |
| StartRestart | NB_AS_SecStartRestart_Type | information to start/restart AS security protection in the PDCP |
| Release | Null_Type | to release AS security protection in the PDCP |

## F.1.7	Paging_Trigger

NB_PagingMessage_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PagingMessage_Type |  |
| Comment |  |  |
| Paging | PCCH_Message_NB |  |
| DirectIndication | B8_Type |  |

NB_PagingTrigger_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PagingTrigger_Type |  |  |
| Comment | CellId : identifier of the cell where the UE is active RoutingInfo : None TimingInfo : Calculated paging occasion ControlInfo : CnfFlag:=false; FollowOnFlag:=false primitive to trigger transmission of a paging on the NPCCH at a calculated paging occasion (TS 36.304, clause 7); the paging occasion is calculated by TTCN and activation time is applied; as for BCCH Info acc. to TS 36.331, clause 9.1.1.3 "RRC will perform padding, if required due to the granularity of the TF signalling, as defined in 8.5."; therefore this needs to be done by the system simulator |  |  |
| Paging | NB_PagingMessage_Type |  | paging to be scheduled in the Type1 common search space beginning from the time given in the timing information of the common part of the ASP; the DCI is transmitted on the NPDCCH using P-RNTI |
| PagingCarrierId | integer | opt | When the UE supports paging on a non-anchor carrier and paging configuration for non-anchor carrier is provided in system information: Id of the DL non-anchor carrier (-> NB_NonAnchorCarrierCommonDL_Type.Id) on which the SS shall do the paging; the SS configuration of the carrier with the given Id corresponds to the entry in dl-ConfigList-r14 of SIB22 which has the same carrier frequency; NOTE: it is up to TTCN implementation to determine the carrier which fulfils the equation given in TS 36.304 clause 7.1 to determine the paging carrier; omit when paging happens on anchor-carrier |

## F.1.8	RLC_Counts

Primitives to enquire RLC Counts AM_VTS and AM_VRR

NB_RLC_CountsInfoList_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RLC_CountsInfoList_Type |  |  |
| Comment |  |  |  |
| AM_VTS | integer |  | sequnce number for DL |
| AM_VRR | integer |  | sequnce number for UL |

NB_RLC_CountsReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RLC_CountsReq_Type |  |
| Comment |  |  |
| Get | Null_Type | Request RLC Counts |
| Set | NB_RLC_CountsInfoList_Type | Set RLC Counts |

NB_RLC_CountsCnf_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RLC_CountsCnf_Type |  |
| Comment |  |  |
| Get | NB_RLC_CountsInfoList_Type | VTS and VRR |
| Set | Null_Type |  |

## F.1.9	PDCP_Count

Primitives to enquire PDCP COUNT

NB_PdcpCountFormat_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NB_PdcpCountFormat_Type |
| Comment |  |
| PdcpCount_Srb | 27 bit HFN; 5 bit SQN |
| PdcpCount_DrbShortSQN | 25 bit HFN; 7 bit SQN |

NB_PdcpCount_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PdcpCount_Type |  |  |
| Comment |  |  |  |
| Format | NB_PdcpCountFormat_Type |  |  |
| Value | PdcpCountValue_Type |  |  |

NB_PdcpCountInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PdcpCountInfo_Type |  |  |
| Comment | - at the start of the PDCP entity Next_PDCP_RX_SN resp. Next_PDCP_TX_SN are set to 0 - when receiving or sending a PDCP PDU the COUNT associated is established from the current values of Next_PDCP_RX_SN resp. Next_PDCP_TX_SN - Next_PDCP_RX_SN resp. Next_PDCP_TX_SN are incremented afterwards |  |  |
| RadioBearerId | NB_RadioBearerId_Type |  |  |
| UL | NB_PdcpCount_Type | opt | omit: keep as it is |
| DL | NB_PdcpCount_Type | opt | omit: keep as it is |

NB_PdcpCountInfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_PdcpCountInfoList_Type |
| Comment |  |
| record length (1..tsc_NB_MaxRB) of NB_PdcpCountInfo_Type |  |

NB_PdcpCountGetReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PdcpCountGetReq_Type |  |
| Comment |  |  |
| AllRBs | Null_Type | return COUNT values for all RBs being configured |
| SingleRB | NB_RadioBearerId_Type |  |

NB_PDCP_CountReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_CountReq_Type |  |
| Comment |  |  |
| Get | NB_PdcpCountGetReq_Type | Request PDCP count for one or all RBs being configured at the PDCP |
| Set | NB_PdcpCountInfoList_Type | Set PDCP count for one or all RBs being configured at the PDCP; list for RBs which's COUNT shall be manipulated |

NB_PDCP_CountCnf_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_CountCnf_Type |  |
| Comment |  |  |
| Get | NB_PdcpCountInfoList_Type | RBs in ascending order; SRBs first |
| Set | Null_Type |  |

## F.1.10	L1_MAC_Test_Mode

Primitive for control of L1/MAC Test Modes

NB_L1Mac_IndicationControl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_L1Mac_IndicationControl_Type |  |  |
| Comment |  |  |  |
| RachPreamble | IndicationAndControlMode_Type | opt | To enable/disable reporting of NPRACH preamble received |
| DPR | IndicationAndControlMode_Type | opt | To enable/disable reporting of MAC DPR control elements |
| HarqError | IndicationAndControlMode_Type | opt | To enable/disable reporting of HARQ errors |
| UL_HARQ | IndicationAndControlMode_Type | opt | To enable/disable reporting of HARQ ACK/NACK |
| TimingAdvanceReport | IndicationAndControlMode_Type | opt | To enable/disable reporting of timing advance |

## F.1.11	PTM_Configuration

configuration of Point To Multipoint (PTM) according to rel-14 enhancements

NBIOT_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_MaxSCMTCH_NB | integer | 64 | Maximum number of SC-MTCHs in one cell for NB-IoT |

PTM_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NPdcchType1ACommonSearchSpace_Type | NPDCCH_SC_MCCH_Config_NB_r14 | parameters for Type1A common search space for SC-MCCH (Type1ACSS, TS 36.213 clause 16.6) |

NPdcchType2ACommonSearchSpace_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdcchType2ACommonSearchSpace_Type |  |  |
| Comment | parameters for Type2A common search space for SC-MTCH (Type2ACSS, TS 36.213 clause 16.6); |  |  |
| NumRepetitions | NPDCCH_NumRepetitions_SC_MTCH_Type |  | Rmax for Type2A common search space; TS 36.213 clause 16.6 |
| StartSF | NPDCCH_StartSF_SC_MTCH_Type |  | G for Type2A common search space; TS 36.213 clause 16.6 |
| Offset | NPDCCH_Offset_SC_MTCH_Type |  | offset for Type2A common search space; TS 36.213 clause 16.6 |
| Npdsch_MaxTBS | NPDCCH_NPDSCH_MaxTBS_SC_MTCH_Type |  | Maximum NPDSCH TBS for the SC-MTCH |

NB_SC_MTCH_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SC_MTCH_Config_Type |  |  |
| Comment |  |  |  |
| GRnti | B16_Type |  |  |
| ScMtchCarrierConfig | NB_SC_MTCH_CarrierConfig_Type |  |  |
| ScMtchSchedulingInfo | SC_MTCH_SchedulingInfo_NB_r14 | opt |  |

NB_SC_MTCH_Info_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SC_MTCH_Info_Type |  |  |
| Comment |  |  |  |
| Type2ACSS | NPdcchType2ACommonSearchSpace_Type |  | parameters for Type2A common search space |
| ScMrbId | SC_MRB_Identity_Type |  |  |
| Config | NB_SC_MTCH_Config_Type |  |  |

NB_SC_MTCH_InfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_SC_MTCH_InfoList_Type |
| Comment |  |
| record length(1.. tsc_MaxSCMTCH_NB) of NB_SC_MTCH_Info_Type |  |

NB_SC_MCCH_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SC_MCCH_Config_Type |  |  |
| Comment |  |  |  |
| Type1ACSS | NPdcchType1ACommonSearchSpace_Type |  | parameters for Type1A common search space |
| ScMcchCarrierConfig | NB_SC_MCCH_CarrierConfig_Type |  |  |
| ScMcchRepetitionPeriod | NB_SC_MCCH_RepetitionPeriod_Type |  |  |
| ScMcchOffset | NB_SC_MCCH_Offset_Type |  |  |
| ScMcchModificationPeriod | NB_SC_MCCH_ModificationPeriod_Type |  |  |
| ScMcchSchedulingInfo | NB_SC_MCCH_SchedulingInfo_Type | opt |  |

NB_SCPTM_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SCPTM_Config_Type |  |  |
| Comment | all fields are optional to allow single modifications; in case of reconfiguration OMIT means 'keep configuration as it is' |  |  |
| ScMcchConfig | NB_SC_MCCH_Config_Type | opt | SC-MCCH scheduling configuration on NPDCCH as per SystemInformationBlockType20-NB acc. to TS 36.331 cl 9.1.1.7 there is no PDCP and SC-MCCH uses the RLC-UM mode configuration/scheduling and contents of the SC-MCCH Information is done in one go (i.e. there are no separate ports for SC-MCCH data and configuration) |
| ScptmConfiguration | SCPTMConfiguration_NB_r14 | opt | Message containing SC-MTCH configuration to be broadcasted on SC-MCCH |
| ScMtchInfoList | NB_SC_MTCH_InfoList_Type | opt | Configure/release SC-MTCH SC-MRB |

NB_PTM_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PTM_Config_Type |  |  |
| Comment |  |  |  |
| ScptmConfig | NB_SCPTM_Config_Type |  | PTM services are provided via SC-PTM |

## F.1.12	System_Interface

NB_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SYSTEM_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | NB_ReqAspCommonPart_Type |  | TimingInfo depends on respective primitive: |
| Request | NB_SystemRequest_Type |  | - Cell TimingInfo: 'now' (in general) - CellAttenuationList TimingInfo: 'now' (in general, but activation time may be used also) - RadioBearerList TimingInfo: 'now' in general; activation time may be used in special case for release and/or reconfiguration of one or several RBs; |

NB_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SYSTEM_CTRL_CNF |  |  |
| Comment |  |  |  |
| Common | NB_CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN (apart from EnquireTiming) => SS may set TimingInfo to "None" |
| Confirm | NB_SystemConfirm_Type |  |  |

NB_SYSTEM_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SYSTEM_IND |  |  |
| Comment |  |  |  |
| Common | NB_IndAspCommonPart_Type |  | The SS shall provide TimingInfo (HSFN + SFN + subframe number) depending on the respective indication: |
| Indication | NB_SystemIndication_Type |  |  |

NBIOT_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NBIOT_SYSTEM_PORT |  |
| Comment | NBIOT PTC: Port for system configuration |  |
| out | NB_SYSTEM_CTRL_REQ |  |
| in | NB_SYSTEM_CTRL_CNF |  |

NBIOT_SYSIND_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NBIOT_SYSIND_PORT |  |
| Comment | NBIOT PTC: Port for system indications |  |
| in | NB_SYSTEM_IND |  |

# F.2	NBIOT_ASP_SrbDefs

## F.2.1	SRB_DATA_ASPs

ASP Definitions to send/receive peer-to-peer messages on SRBs

NB_C_Plane_Request_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_C_Plane_Request_Type |  |  |
| Comment | RRC and/or NAS PDU to be send to the UE; Note: it may be necessary to allow more than one NAS PDU (-> "record of") |  |  |
| Rrc | NB_RRC_MSG_Request_Type | opt | omit: NAS message shall be present; NAS message shall be sent in DLInformationTransfer present: if NAS message is present also, (piggybacked) NAS PDU shall be security protected (if necessary) and inserted in RRC PDU's DedicatedInfoNAS |
| Nas | NAS_MSG_RequestList_Type | opt | omit: RRC message shall be present; RRC message does not contain (piggybacked) NAS PDU present: if RRC message is omitted => NAS message shall be sent embedded in DLInformationTransfer if RRC message is present => NAS message is piggybacked in RRC message in case of RRC message is sent on CCCH, NAS message shall be omitted |

NB_C_Plane_Indication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_C_Plane_Indication_Type |  |  |
| Comment | RRC and/or NAS PDU to be received from the UE; Note: it may be necessary to allow more than one NAS PDU (-> "record of") |  |  |
| Rrc | NB_RRC_MSG_Indication_Type | opt | omit: NAS message shall be present; NAS message is received in ULInformationTransfer present: if NAS message is present also, DedicatedInfoNAS contains unstructured and ciphered NAS message and the NAS message is the deciphered message in structured format |
| Nas | NAS_MSG_IndicationList_Type | opt | omit: RRC message shall be present; RRC message does not contain (piggybacked) NAS PDU present: if RRC message is omitted => NAS message has been received in ULInformationTransfer if RRC message is present => NAS message has been piggybacked in RRC message |

NB_SRB_COMMON_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SRB_COMMON_REQ |  |  |
| Comment | common ASP to send PDUs to SRB0, SRB1 or SRB1bis |  |  |
| Common | NB_ReqAspCommonPart_Type |  | CellId identifier of the cell RoutingInfo SRB0, SRB1, SRB1bis TimingInfo Now in normal cases; For latency tests TimingInfo can be set to the SFN/subframe in which the RRC messages shall be sent out (in this case and if the RRC PDU is too long to be sent in one TTI the TimingInfo corresponds to the first TTI) ControlInfo CnfFlag:=false; FollowOnFlag true: Indicates that the message(s) to be sent on the same TTI will follow NOTE 1: When FollowOnFlag is true, TimingInfo shall always be "Now". Otherwise SS shall produce an error NOTE 2: the follow on flag applies only for messages of the same SRB false: Indicates that no more message(s) will follow |
| Signalling | NB_C_Plane_Request_Type |  |  |

NB_SRB_COMMON_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SRB_COMMON_IND |  |  |
| Comment | common ASP to receive PDUs from SRB0, SRB1 or SRB1bis |  |  |
| Common | NB_IndAspCommonPart_Type |  | CellId identifier of the cell RoutingInfo SRB0, SRB1, SRB1bis TimingInfo time when message has been received (as received from the SS by the NAS emulator) |
| Signalling | NB_C_Plane_Indication_Type |  |  |

## F.2.2	Port_Definitions

NBIOT_SRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NBIOT_SRB_PORT |  |
| Comment | NBIOT PTC: Port for Sending/Receiving data on SRBs |  |
| out | NB_SRB_COMMON_REQ |  |
| in | NB_SRB_COMMON_IND |  |

NASEMU_NBIOT_SRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NASEMU_NBIOT_SRB_PORT |  |
| Comment | NASEMU PTC: Port for Sending/Receiving data on SRBs (interface to NBIOT PTC) |  |
| out | NB_SRB_COMMON_IND |  |
| in | NB_SRB_COMMON_REQ |  |

# F.3	NBIOT_ASP_L2DataDefs

## F.3.1	System_Interface

NB_L2_DATA_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_L2_DATA_REQ |  |  |
| Comment | common ASP to send L2 data |  |  |
| Common | NB_ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB/DRB id TimingInfo : starting point when to start sending sequence of data PDUs e.g. SFN = X, subframe number = x; L2Data.SubframeDataList[i].SubframeOffset := offset_i; => L2Data.SubframeDataList[i].PduSduList shall be scheduled in the next search space after SFN = X + ((x + offset_i) / 10); subframe number = (x + offset_i) % 10 ControlInfo : CnfFlag:=false; FollowOnFlag:=false |
| L2Data | L2Data_Request_Type |  |  |

NB_L2_DATA_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_L2_DATA_IND |  |  |
| Comment | common ASP to receive L2 data |  |  |
| Common | NB_IndAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB/DRB id TimingInfo : time when message has been received according to clause 7A.6 of TS 36.523-3 |
| L2Data | L2Data_Indication_Type |  |  |

NBIOT_L2DATA_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NBIOT_L2DATA_PORT |  |
| Comment |  |  |
| out | NB_L2_DATA_REQ |  |
| in | NB_L2_DATA_IND |  |

# F.4	EUTRA_NB_ASP_L2DataDefs

ASP interface for DRBs

## F.4.1	PDU_TypeDefs

### F.4.1.1	MAC_PDU

MAC_PDU: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| MAC_CTRL_C_RNTI_Type | C_RNTI | TS 36.321, clause 6.1.3.2 |
| MAC_CTRL_ContentionResolutionId_Type | ContentionResolutionId_Type | TS 36.321, clause 6.1.3.4 fix 48-bit size; consists of a single field defined UE Contention Resolution Identity (first 48 bits of the uplink CCCH SDU transmitted by MAC) |
| MAC_CTRL_TimingAdvance_Type | B8_Type | TS 36.321, clause 6.1.3.5 indicates the amount of timing adjustment in 0.5 ms that the UE has to apply; the length of the field is 8 bits |
| MAC_SDU_Type | octetstring |  |

MAC_PDU_Length_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_PDU_Length_Type |  |  |
| Comment | NOTE: since F and L field are either both present or both omitted they are put into this record; to allow homogeneous (direct) encoding the PDU length is not defined as union; TTCN-3 does allow length restrictions to one lenght or a range of length but not to two specific lengthes; further restriction may be achieved by appropriate templates (parameter either 7 or 15 bit) |  |  |
| Format | B1_Type |  | F: The Format field indicates the size of the Length field as indicated in table 6.2.1-3. There is one F field per MAC PDU subheader except for the last subheader and sub-headers corresponding to fixed-sized MAC control elements. The size of the F field is 1 bit. If the size of the MAC SDU or MAC control element is less than 128 bytes, the UE shall set the value of the F field to 0, otherwise the UE shall set it to 1 |
| Value | B7_15_Type |  | L: The Length field indicates the length of the corresponding MAC SDU or MAC control element in bytes. There is one L field per MAC PDU subheader except for the last subheader and sub-headers corresponding to fixed-sized MAC control elements. The size of the L field is indicated by the F field |

MAC_PDU_SubHeader_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_PDU_SubHeader_Type |  |  |
| Comment |  |  |  |
| Reserved | B2_Type |  | Reserved bits |
| Extension | B1_Type |  | E: The Extension field is a flag indicating if more fields are present in the MAC header or not. The E field is set to "1" to indicate another set of at least R/R/E/LCID fields. The E field is set to "0" to indicate that either a MAC SDU, a MAC control element or padding starts at the next byte |
| LCID | B5_Type |  | LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC control element or padding as described in tables 6.2.1-1 and 6.2.1-2 for the DL and UL-SCH respectively. There is one LCID field for each MAC SDU, MAC control element or padding included in the MAC PDU. The LCID field size is 5 bits; NOTE: In case of DRX command the sub-header corresponds to a control element of length zero (i.e. there is no control element) |
| Length | MAC_PDU_Length_Type | opt |  |

MAC_Header_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MAC_Header_Type |
| Comment |  |
| record of MAC_PDU_SubHeader_Type |  |

MAC_CTRL_ShortBSR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_ShortBSR_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.1 |  |  |
| LCG | B2_Type |  |  |
| Value | B6_Type |  |  |

MAC_CTRL_LongBSR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_LongBSR_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.1 |  |  |
| Value_LCG1 | B6_Type |  |  |
| Value_LCG2 | B6_Type |  |  |
| Value_LCG3 | B6_Type |  |  |
| Value_LCG4 | B6_Type |  |  |

MAC_CTRL_PowerHeadRoom_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_PowerHeadRoom_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.6 |  |  |
| Reserved | B2_Type |  |  |
| Value | B6_Type |  |  |

MAC_CTRL_ElementList_Type

| TTCN-3 Set Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_ElementList_Type |  |  |
| Comment | NOTE 1: for simplicication UL and DL are not distinguished even though the control elements are either UL or DL NOTE 2: type is defined as set: the ordering is not signifficant; nevertheless the ordering is well-defined by the sub-headers; for codec implementations it is in any case necessary to evaluate the sub-header information in order to encode/decode the payload |  |  |
| ShortBSR | MAC_CTRL_ShortBSR_Type | opt | UL only |
| LongBSR | MAC_CTRL_LongBSR_Type | opt | UL only |
| C_RNTI | MAC_CTRL_C_RNTI_Type | opt | UL only |
| ContentionResolutionID | MAC_CTRL_ContentionResolutionId_Type | opt | DL only |
| TimingAdvance | MAC_CTRL_TimingAdvance_Type | opt | DL only |
| PowerHeadRoom | MAC_CTRL_PowerHeadRoom_Type | opt | UL only |
| ScellActDeact | MAC_CTRL_ScellActDeact_Type | opt | DL only |
| ExtPowerHeadRoom | MAC_CTRL_ExtPowerHeadRoom_Type | opt | UL only. Only one among PowerHeadroom, ExtPowerHeadroom and DC_PowerHeadRoom may be present |
| DC_PowerHeadRoom | MAC_CTRL_DC_PowerHeadRoom_Type | opt | UL only. Only one among PowerHeadroom, ExtPowerHeadroom and DC_PowerHeadRoom may be present |
| TimingAdvanceReport | MAC_CTRL_TimingAdvanceReport_Type | opt | UL only |
| DifferentialKoffset | MAC_CTRL_DifferentialKoffset_Type | opt | DL only |

MAC_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MAC_SDUList_Type |
| Comment |  |
| record of MAC_SDU_Type |  |

MAC_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_PDU_Type |  |  |
| Comment |  |  |  |
| Header | MAC_Header_Type |  | list of MAC PDU SubHeaders corresponding to MAC control elements and MAC SDUs |
| CtrlElementList | MAC_CTRL_ElementList_Type | opt | Mac control elements; acc. to TS 36.321, clause 6.1.2 "MAC control elements, are always placed before any MAC SDU." |
| SduList | MAC_SDUList_Type | opt | MAC SDUs, which can typically be RLC PDUs |
| Padding | octetstring | opt | Octet aligned Padding if more than or equal to 2 bytes |

MAC_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MAC_PDUList_Type |
| Comment |  |
| record of MAC_PDU_Type |  |

### F.4.1.2	RLC_PDU

RLC_PDU: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_SDU_Type | octetstring |  |

RLC_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_PDU_Type |  |
| Comment |  |  |
| TMD | RLC_TMD_PDU_Type |  |
| UMD | RLC_UMD_PDU_Type |  |
| AMD | RLC_AMD_PDU_Type |  |
| AMD_Ext | RLC_AMD_PDU_Ext_Type |  |
| AMD_SegExt | RLC_AMD_PDU_SegExt_Type |  |
| Status | RLC_AM_StatusPDU_Type |  |
| Status_Ext | RLC_AM_StatusPDU_Ext_Type |  |

RLC_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_PDUList_Type |
| Comment |  |
| record of RLC_PDU_Type |  |

RLC_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_SDUList_Type |
| Comment |  |
| record of RLC_SDU_Type |  |

#### F.4.1.2.1	Common

RLC PDU definition: common AM/UM field definitions

Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_FramingInfo_Type | B2_Type | 00 - First byte of the Data field corresponds to the first byte of a RLC SDU. Last byte of the Data field corresponds to the last byte of a RLC SDU. 01 - First byte of the Data field corresponds to the first byte of a RLC SDU. Last byte of the Data field does not correspond to the last byte of a RLC SDU. 10 - First byte of the Data field does not correspond to the first byte of a RLC SDU. Last byte of the Data field corresponds to the last byte of a RLC SDU. 11 - First byte of the Data field does not correspond to the first byte of a RLC SDU. Last byte of the Data field does not correspond to the last byte of a RLC SDU. |

RLC_LengthIndicator_LI11_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_LengthIndicator_LI11_Type |  |  |
| Comment |  |  |  |
| Extension | B1_Type |  | 0 - Data field follows from the octet following the LI field following this E field 1 - A set of E field and LI field follows from the bit following the LI field following this E field |
| LengthIndicator | B11_Type |  | Length Indicator |

RLC_LengthIndicator_LI15_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_LengthIndicator_LI15_Type |  |  |
| Comment |  |  |  |
| Extension | B1_Type |  | 0 - Data field follows from the octet following the LI field following this E field 1 - A set of E field and LI field follows from the bit following the LI field following this E field |
| LengthIndicator | B15_Type |  | 15 bit Length Indicator |

RLC_LI11_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_LI11_List_Type |
| Comment |  |
| record of RLC_LengthIndicator_LI11_Type |  |

RLC_LI15_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_LI15_List_Type |
| Comment |  |
| record of RLC_LengthIndicator_LI15_Type |  |

RLC_LI_List_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_LI_List_Type |  |
| Comment |  |  |
| LI11 | RLC_LI11_List_Type |  |
| LI15 | RLC_LI15_List_Type |  |

RLC_PDU_Header_FlexPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_PDU_Header_FlexPart_Type |  |  |
| Comment | Flexible part of the header with a number of K LIs |  |  |
| LengthIndicator | RLC_LI_List_Type |  | List of E, LI fields |
| Padding | B4_Type | opt | optional 4 bit padding present in case of odd number of LI's |

#### F.4.1.2.2	TM_Data

RLC PDU definition: UM  (TS 36.322, clause 6.2.1.2)

TM_Data: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_TMD_PDU_Type | octetstring | TS 36.322, clause 6.2.1.2 |

#### F.4.1.2.3	UM_Data

RLC PDU definition: UM  (TS 36.322, clause 6.2.1.3)
NOTE:
To allow direct encoding the definition for RLC UM Data PDU is split into data PDU with 5/10 bit sequence number

UM_Data: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_DataField_Type | octetstring | restrictions imposed from LI size of 11 bits is not applicable when the LI's are not present |

RLC_UMD_Header_FixPartShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_Header_FixPartShortSN_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.3 Figure 6.2.1.3-1, 6.2.1.3-3 and 6.2.1.3-4); one octet |  |  |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bits FI |
| Extension | B1_Type |  | 1 bit E |
| SequenceNumber | B5_Type |  | 5 bits SN |

RLC_UMD_Header_FixPartLongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_Header_FixPartLongSN_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.3 Figure 6.2.1.3-2, 6.2.1.3-5 and 6.2.1.3-6); two octets |  |  |
| Reserved | B3_Type |  | 3 bits reserved |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bits FI |
| Extension | B1_Type |  | 1 bit E |
| SequenceNumber | B10_Type |  | 10 bits SN |

RLC_UMD_HeaderShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_HeaderShortSN_Type |  |  |
| Comment |  |  |  |
| FixPart | RLC_UMD_Header_FixPartShortSN_Type |  |  |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_UMD_HeaderLongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_HeaderLongSN_Type |  |  |
| Comment |  |  |  |
| FixPart | RLC_UMD_Header_FixPartLongSN_Type |  |  |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_DataFieldList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_DataFieldList_Type |
| Comment | One to one correspondence with sub headers (LengthIndicatorList_Type) |
| record of RLC_DataField_Type |  |

RLC_UMD_PDU_ShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_PDU_ShortSN_Type |  |  |
| Comment |  |  |  |
| Header | RLC_UMD_HeaderShortSN_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_UMD_PDU_LongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_PDU_LongSN_Type |  |  |
| Comment |  |  |  |
| Header | RLC_UMD_HeaderLongSN_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_UMD_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_UMD_PDU_Type |  |
| Comment |  |  |
| ShortSN | RLC_UMD_PDU_ShortSN_Type |  |
| LongSN | RLC_UMD_PDU_LongSN_Type |  |

#### F.4.1.2.4	AM_Data

RLC PDU definition: AM (TS 36.322, clause 6.2.1.4 and 6.2.1.5)

RLC_AMD_Header_FixPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_FixPart_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.4 Figure 6.2.1.4-1, 6.2.1.4-2 and 6.2.1.4-3); 2 or 4 octets (AMD PDU or AMD PDU segment) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| ReSeg | B1_Type |  | 0 - AMD PDU 1 - AMD PDU segment |
| Poll | B1_Type |  | 0 - Status report not requested 1 - Status report is requested |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bit FI |
| Extension | B1_Type |  | 1 bit E |
| SN | B10_Type |  | Sequence number |

RLC_AMD_Header_FixPartExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_FixPartExt_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.4 Figure 6.2.1.4-1a, 6.2.1.4-2a and 6.2.1.4-3a); 3 octets |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| ReSeg | B1_Type |  | 0 - AMD PDU 1 - AMD PDU segment |
| Poll | B1_Type |  | 0 - Status report not requested 1 - Status report is requested |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bit FI |
| Extension | B1_Type |  | 1 bit E |
| Reserved | B2_Type |  | 2 reserved bits |
| SN_Ext | B16_Type |  | Sequence number |

RLC_AMD_Header_FixPartSegExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_FixPartSegExt_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.5 Figure 6.2.1.5-1a); 5 octets |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| ReSeg | B1_Type |  | 0 - AMD PDU 1 - AMD PDU segment |
| Poll | B1_Type |  | 0 - Status report not requested 1 - Status report is requested |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bit FI |
| Extension | B1_Type |  | 1 bit E |
| LastSegmentFlag | B1_Type |  | 0 - Last byte of the AMD PDU segment does not correspond to the last byte of an AMD PDU 1 - Last byte of the AMD PDU segment corresponds to the last byte of an AMD PDU |
| Reserved | B1_Type |  | 1 reserved bit |
| SN_Ext | B16_Type |  | Sequence number |

RLC_AMD_Header_SegmentPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_SegmentPart_Type |  |  |
| Comment | AMD PDU segment related info in PDU header acc. TS 36.322, clause 6.2.1.5 |  |  |
| LastSegmentFlag | B1_Type |  | 0 - Last byte of the AMD PDU segment does not correspond to the last byte of an AMD PDU 1 - Last byte of the AMD PDU segment corresponds to the last byte of an AMD PDU |
| SegOffset | B15_Type |  | The SO field indicates the position of the AMD PDU segment in bytes within the original AMD PDU. Specifically, the SO field indicates the position within the Data field of the original AMD PDU to which the first byte of the Data field of the AMD PDU segment corresponds to. |

RLC_AMD_Header_SegmentPartExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_SegmentPartExt_Type |  |  |
| Comment | AMD PDU segment related info in PDU header acc. TS 36.322, clause 6.2.1.5 |  |  |
| SegOffset | B16_Type |  | The SO field indicates the position of the AMD PDU segment in bytes within the original AMD PDU. Specifically, the SO field indicates the position within the Data field of the original AMD PDU to which the first byte of the Data field of the AMD PDU segment corresponds to. |

RLC_AMD_Header_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_Type |  |  |
| Comment |  |  |  |
| FixPart | RLC_AMD_Header_FixPart_Type |  |  |
| SegmentPart | RLC_AMD_Header_SegmentPart_Type | opt | present in case of AMD Seg PDU only |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_AMD_HeaderExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_HeaderExt_Type |  |  |
| Comment |  |  |  |
| FixPartExt | RLC_AMD_Header_FixPartExt_Type |  |  |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_AMD_HeaderSegExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_HeaderSegExt_Type |  |  |
| Comment |  |  |  |
| FixPartSegExt | RLC_AMD_Header_FixPartSegExt_Type |  |  |
| SegmentPart | RLC_AMD_Header_SegmentPartExt_Type | opt | present in case of AMD Seg PDU only |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_AMD_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_PDU_Type |  |  |
| Comment |  |  |  |
| Header | RLC_AMD_Header_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_AMD_PDU_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_PDU_Ext_Type |  |  |
| Comment |  |  |  |
| HeaderExt | RLC_AMD_HeaderExt_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_AMD_PDU_SegExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_PDU_SegExt_Type |  |  |
| Comment |  |  |  |
| HeaderSegExt | RLC_AMD_HeaderSegExt_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

#### F.4.1.2.5	AM_Status

AM Status PDU (TS 36.322, clause 6.2.1.6)

AM_Status: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_Status_Padding_Type | bitstring length (1..7) | NOTE: in TTCN-3 length restriction cannot be done inline in record definition => explicit type definition necessary |

RLC_Status_ACK_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_ACK_Type |  |  |
| Comment |  |  |  |
| ACK_SN | B10_Type |  | Acknowledgement SN (TS 36.322, clause 6.2.2.14) |
| Extn1 | B1_Type |  | 0 - a set of NACK_SN, E1 and E2 does not follow. 1 - a set of NACK_SN, E1 and E2 follows. |

RLC_Status_ACK_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_ACK_Ext_Type |  |  |
| Comment |  |  |  |
| ACK_SN_Ext | B16_Type |  | Acknowledgement SN (TS 36.322, clause 6.2.2.14) |
| Extn1 | B1_Type |  | 0 - a set of NACK_SN, E1 and E2 does not follow. 1 - a set of NACK_SN, E1 and E2 follows. |

RLC_Status_SegOffset_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_SegOffset_Type |  |  |
| Comment |  |  |  |
| Start | B15_Type |  | SOstart field indicates the position of the first byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU |
| End | B15_Type |  | SOend field indicates the position of the last byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU. The special SOend value '111111111111111'B is used to indicate that the missing portion of the AMD PDU includes all bytes to the last byte of the AMD PDU |

RLC_Status_SegOffset_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_SegOffset_Ext_Type |  |  |
| Comment |  |  |  |
| Start | B16_Type |  | SOstart field indicates the position of the first byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU |
| End | B16_Type |  | SOend field indicates the position of the last byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU. The special SOend value '111111111111111'B is used to indicate that the missing portion of the AMD PDU includes all bytes to the last byte of the AMD PDU |

RLC_Status_NACK_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_NACK_Type |  |  |
| Comment |  |  |  |
| NACK_SN | B10_Type |  |  |
| Extn1 | B1_Type |  | 0 - A set of NACK_SN, E1 and E2 does not follow. 1 - A set of NACK_SN, E1 and E2 follows. |
| Extn2 | B1_Type |  | 0 - A set of SOstart and SOend does not follow for this NACK_SN. 1 - A set of SOstart and SOend follows for this NACK_SN. |
| SO | RLC_Status_SegOffset_Type | opt |  |

RLC_Status_NACK_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_Status_NACK_List_Type |
| Comment |  |
| record of RLC_Status_NACK_Type |  |

RLC_Status_NACK_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_NACK_Ext_Type |  |  |
| Comment |  |  |  |
| NACK_SN_Ext | B16_Type |  |  |
| Extn1 | B1_Type |  | 0 - A set of NACK_SN, E1 and E2 does not follow. 1 - A set of NACK_SN, E1 and E2 follows. |
| Extn2 | B1_Type |  | 0 - A set of SOstart and SOend does not follow for this NACK_SN. 1 - A set of SOstart and SOend follows for this NACK_SN. |
| SO | RLC_Status_SegOffset_Ext_Type | opt |  |

RLC_Status_NACK_Ext_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_Status_NACK_Ext_List_Type |
| Comment |  |
| record of RLC_Status_NACK_Ext_Type |  |

RLC_AM_StatusPDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AM_StatusPDU_Type |  |  |
| Comment | RLC STATUS PDU (TS 36.322, clause 6.2.1.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| CP_Type | B3_Type |  | 000 - STATUS PDU 001..111 - Reserved (=> PDU to be discarded by the receiving entity for this release of the protocol) |
| Ack | RLC_Status_ACK_Type |  | ACK_SN and E1 bit |
| NackList | RLC_Status_NACK_List_Type | opt | presence depends on Extn1 bit of Ack filed (RLC_Status_ACK_Type) |
| Padding | RLC_Status_Padding_Type | opt | 1..7 bit padding if needed for octet alignment |

RLC_AM_StatusPDU_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AM_StatusPDU_Ext_Type |  |  |
| Comment | RLC STATUS PDU (TS 36.322, clause 6.2.1.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| CP_Type | B3_Type |  | 000 - STATUS PDU 001..111 - Reserved (=> PDU to be discarded by the receiving entity for this release of the protocol) |
| Ack_Ext | RLC_Status_ACK_Ext_Type |  | ACK_SN and E1 bit |
| Nack_Ext_List | RLC_Status_NACK_Ext_List_Type | opt | presence depends on Extn1 bit of Ack field (RLC_Status_ACK_Type) |
| Padding | RLC_Status_Padding_Type | opt | 1..7 bit padding if needed for octet alignment |

### F.4.1.3	PDCP

PDCP user plane SDU and PDU definitions
NOTE:
To allow direct encoding the definition for PDCP Data PDU is split into data PDU with long/short sequence number

PDCP: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| PDCP_SDU_Type | octetstring |  |

PDCP_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PDCP_SDUList_Type |
| Comment |  |
| record of PDCP_SDU_Type |  |

PDCP_DataPdu_LongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_LongSN_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number (TS 36.323, clause 6.2.3) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| Reserved | B3_Type |  | 3 reserved bits |
| SequenceNumber | B12_Type |  | 12 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_ShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_ShortSN_Type |  |  |
| Comment | User plane PDCP Data PDU with short sequence number (TS 36.323, clause 6.2.4) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| SequenceNumber | B7_Type |  | 7 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_ExtSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_ExtSN_Type |  |  |
| Comment | User plane PDCP Data PDU with extended sequence number (TS 36.323, clause 6.2.9) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| SequenceNumber | B15_Type |  | 15 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_18bitSN_Type |  |  |
| Comment | User plane PDCP Data PDU with 18 bit sequence number (TS 36.323, clause 6.2.11) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PollingBit | B1_Type |  | 1 - indicates the UE is requested to send a PDCP status report or a LWA status report for LWA |
| Reserved | B4_Type |  | 4 reserved bits |
| SequenceNumber | B18_Type |  | 18 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_LongSN_UDC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_LongSN_UDC_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number for UDC (TS 36.323, clause 6.2.14) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| Reserved | B3_Type |  | 3 reserved bits |
| SequenceNumber | B12_Type |  | 12 bit sequence number |
| FU | B1_Type |  | 0 - Packet is not compressed using UDC protocol 1 - Packet is compressed using UDC protocol |
| FR | B1_Type |  | 0 - Compression buffer is not reset 1 - Compression buffer has been reset |
| Reserved2 | B2_Type |  | 2 reserved bits |
| Checksum | B4_Type |  | 4 validation bits for the compression buffer content |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_ExtSN_UDC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_ExtSN_UDC_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number for UDC (TS 36.323, clause 6.2.15) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| SequenceNumber | B15_Type |  | 15 bit sequence number |
| FU | B1_Type |  | 0 - Packet is not compressed using UDC protocol 1 - Packet is compressed using UDC protocol |
| FR | B1_Type |  | 0 - Compression buffer is not reset 1 - Compression buffer has been reset |
| Reserved2 | B2_Type |  | 2 reserved bits |
| Checksum | B4_Type |  | 4 validation bits for the compression buffer content |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_18bitSN_UDC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_18bitSN_UDC_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number for UDC (TS 36.323, clause 6.2.16) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PollingBit | B1_Type |  | 1 - indicates the UE is requested to send a PDCP status report or a LWA status report for LWA |
| Reserved | B4_Type |  | 4 reserved bits |
| SequenceNumber | B18_Type |  | 18 bit sequence number |
| FU | B1_Type |  | 0 - Packet is not compressed using UDC protocol 1 - Packet is compressed using UDC protocol |
| FR | B1_Type |  | 0 - Compression buffer is not reset 1 - Compression buffer has been reset |
| Reserved2 | B2_Type |  | 2 reserved bits |
| Checksum | B4_Type |  | 4 validation bits for the compression buffer content |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_SLRB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_SLRB_Type |  |  |
| Comment | User plane PDCP Data PDU for SLRB (TS 36.323, clause 6.2.10.1) |  |  |
| SDU_Type | B3_Type |  | 000 - IP 001 - ARP 010 - PC5 Signaling 011 - Non-IP 100 - 111 reserved |
| PGK_Index | B5_Type |  | 5 LSBs of PGK Identity (TS 33.303, clause 6.2.3) |
| PTK_Identity | B16_Type |  | PTK Identity (TS 33.303, clause 6.2.3) |
| SequenceNumber | B16_Type |  | 16 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_SLRB_1to1_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_SLRB_1to1_Type |  |  |
| Comment | User plane PDCP Data PDU for SLRB (TS 36.323, clause 6.2.10.2) |  |  |
| SDU_Type | B3_Type |  | 000 - IP 001 - ARP 010 - PC5 Signaling 011 - Non-IP 100 - 111 reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| KD_sess_ID | B16_Type |  | KD_sess Identity (TS 33.303, clause 6.5.3.1) |
| SequenceNumber | B16_Type |  | 16 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |
| MAC_I | O4_Type |  | MAC-I |

PDCP_Ctrl_ROHC_FB_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_ROHC_FB_PDU_Type |  |  |
| Comment | PDCP Control PDU for interspersed ROHC feedback packet (TS 36.323, clause 6.2.5) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B4_Type |  |  |
| ROHC_FB | octetstring |  | Contains one ROHC packet with only feedback, i.e. a ROHC packet that is not associated with a PDCP |

PDCP_Ctrl_StatusReport_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_StatusReport_Type |  |  |
| Comment | PDCP Control PDU for PDCP status report (TS 36.323, clause 6.2.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| FMS | B12_Type |  | PDCP SN of the first missing PDCP SDU. |
| Bitmap | octetstring | opt | The MSB of the first octet of the type "Bitmap" indicates whether or not the PDCP SDU with the SN (FMS + 1) modulo 4096 has been received and, optionally decompressed correctly. 0 - PDCP SDU with PDCP SN = (FMS + bit position) modulo 4096 is missing in the receiver. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. 1 - PDCP SDU with PDCP SN = (FMS + bit position) modulo 4096 does not need to be retransmitted. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. |

PDCP_Ctrl_StatusReportExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_StatusReportExt_Type |  |  |
| Comment | PDCP Control PDU for PDCP status report using a 15 bit SN (TS 36.323, clause 6.2.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| FMS_Ext | B15_Type |  | PDCP SN of the first missing PDCP SDU. |
| Bitmap | octetstring | opt | The MSB of the first octet of the type "Bitmap" indicates whether or not the PDCP SDU with the SN (FMS + 1) modulo (Maximum_PDCP_SN + 1) has been received and, optionally decompressed correctly. 0 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) is missing in the receiver. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. 1 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) does not need to be retransmitted. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. |

PDCP_Ctrl_StatusReport_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_StatusReport_18bitSN_Type |  |  |
| Comment | PDCP Control PDU for PDCP status report using a 18 bit SN (TS 36.323, clause 6.2.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B2_Type |  | 2 reserved bits |
| FMS_18bitSN | B18_Type |  | PDCP SN of the first missing PDCP SDU. |
| Bitmap | octetstring | opt | The MSB of the first octet of the type "Bitmap" indicates whether or not the PDCP SDU with the SN (FMS + 1) modulo (Maximum_PDCP_SN + 1) has been received and, optionally decompressed correctly. 0 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) is missing in the receiver. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. 1 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) does not need to be retransmitted. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. |

PDCP_Ctrl_LWA_StatusReport_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_StatusReport_Type |  |  |
| Comment | PDCP Control PDU for LWA status report (TS 36.323, clause 6.2.12) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| FMS | B12_Type |  | PDCP SN of the first missing PDCP SDU. |
| HRW | B12_Type |  | PDCP SN of the PDCP SDU received on WLAN with highest associated PDCP COUNT value. |
| NMP | B12_Type |  | Number of Missing PDCP PDUs with PDCP SNs below HRW starting from and including FMS. |

PDCP_Ctrl_LWA_StatusReportExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_StatusReportExt_Type |  |  |
| Comment | PDCP Control PDU for LWA status report (TS 36.323, clause 6.2.12) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| FMS_Ext | B15_Type |  | PDCP SN of the first missing PDCP SDU. |
| Reserved2 | B1_Type |  | 1 reserved bit |
| HRW_Ext | B15_Type |  | PDCP SN of the PDCP SDU received on WLAN with highest associated PDCP COUNT value. |
| Reserved3 | B1_Type |  | 1 reserved bit |
| NMP_Ext | B15_Type |  | Number of Missing PDCP PDUs with PDCP SNs below HRW starting from and including FMS. |

PDCP_Ctrl_LWA_StatusReport_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_StatusReport_18bitSN_Type |  |  |
| Comment | PDCP Control PDU for LWA status report (TS 36.323, clause 6.2.12) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B2_Type |  | 2 reserved bits |
| FMS_18bitSN | B18_Type |  | PDCP SN of the first missing PDCP SDU. |
| HRW_18bitSN | B18_Type |  | PDCP SN of the PDCP SDU received on WLAN with highest associated PDCP COUNT value. |
| Reserved2 | B4_Type |  | 4 reserved bits |
| NMP_18bitSN | B18_Type |  | Number of Missing PDCP PDUs with PDCP SNs below HRW starting from and including FMS. |

PDCP_Ctrl_LWA_EndMarker_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_EndMarker_Type |  |  |
| Comment | PDCP Control PDU for LWA end marker (TS 36.323, clause 6.2.13) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| LSN | B12_Type |  | PDCP SN of the last PDCP PDU for which the data part is ciphered with the key used before PDCP re-establishment. |

PDCP_Ctrl_LWA_EndMarkerExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_EndMarkerExt_Type |  |  |
| Comment | PDCP Control PDU for LWA end marker (TS 36.323, clause 6.2.13) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| LSN_Ext | B15_Type |  | PDCP SN of the last PDCP PDU for which the data part is ciphered with the key used before PDCP re-establishment. |

PDCP_Ctrl_LWA_EndMarker_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_EndMarker_18bitSN_Type |  |  |
| Comment | PDCP Control PDU for LWA end marker (TS 36.323, clause 6.2.13) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B2_Type |  | 2 reserved bits |
| LSN_18bitSN | B18_Type |  | PDCP SN of the last PDCP PDU for which the data part is ciphered with the key used before PDCP re-establishment. |

PDCP_Ctrl_UDC_FB_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_UDC_FB_PDU_Type |  |  |
| Comment | PDCP Control PDU for UDC feedback packet (TS 36.323, clause 6.2.17) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 100 - UDC feedback packet |
| FE | B1_Type |  | 0 - No Error 1 - Checksum Error Notification |
| Reserved | B3_Type |  |  |

PDCP_Ctrl_EHC_FB_PDU_ShortCID_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_EHC_FB_PDU_ShortCID_Type |  |  |
| Comment |  |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 101 - EHC feedback packet |
| Reserved | B4_Type |  | 4 bits reserved |
| Reserved2 | B1_Type |  | 1 bit reserved |
| CID | B7_Type |  | 7 bits CID |

PDCP_Ctrl_EHC_FB_PDU_LongCID_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_EHC_FB_PDU_LongCID_Type |  |  |
| Comment |  |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 101 - EHC feedback packet |
| Reserved | B4_Type |  | 4 bits reserved |
| Reserved2 | B1_Type |  | 1 bit reserved |
| CID | B15_Type |  | 15 bits CID |

PDCP_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_PDU_Type |  |
| Comment |  |  |
| DataLongSN | PDCP_DataPdu_LongSN_Type | user plane PDCP data PDU with 12 Bit Seq Number |
| DataShortSN | PDCP_DataPdu_ShortSN_Type | user plane PDCP data PDU with 7 Bit Seq Number |
| DataExtSN | PDCP_DataPdu_ExtSN_Type | user plane PDCP data PDU with 15 Bit Seq Number |
| Data_18bitSN | PDCP_DataPdu_18bitSN_Type | user plane PDCP data PDU with 18 Bit Seq Number |
| RohcFeedback | PDCP_Ctrl_ROHC_FB_PDU_Type | PDCP Control PDU for interspersed ROHC feedback packet |
| StatusReport | PDCP_Ctrl_StatusReport_Type | PDCP Control PDU for PDCP status report |
| StatusReportExt | PDCP_Ctrl_StatusReportExt_Type | PDCP Control PDU for PDCP status report using a 15 bit SN |
| StatusReport_18bitSN | PDCP_Ctrl_StatusReport_18bitSN_Type | PDCP Control PDU for PDCP status report using a 18 bit SN |
| LWA_StatusReport | PDCP_Ctrl_LWA_StatusReport_Type | PDCP Control PDU for LWA status report |
| LWA_StatusReportExt | PDCP_Ctrl_LWA_StatusReportExt_Type | PDCP Control PDU for LWA status report using a 15 bit SN |
| LWA_StatusReport_18bitSN | PDCP_Ctrl_LWA_StatusReport_18bitSN_Type | PDCP Control PDU for LWA status report using a 18 bit SN |
| DataSLRB | PDCP_DataPdu_SLRB_Type | user plane PDCP Data PDU for SLRB |
| DataSLRB_1to1 | PDCP_DataPdu_SLRB_1to1_Type | user plane PDCP Data PDU for SLRB one to one communication |
| LWA_EndMarker | PDCP_Ctrl_LWA_EndMarker_Type | PDCP Control PDU for LWA end marker |
| LWA_EndMarkerExt | PDCP_Ctrl_LWA_EndMarkerExt_Type | PDCP Control PDU for LWA end marker using a 15 bit SN |
| LWA_EndMarker_18bitSN | PDCP_Ctrl_LWA_EndMarker_18bitSN_Type | PDCP Control PDU for LWA end marker using a 18 bit SN |
| DataLongSN_UDC | PDCP_DataPdu_LongSN_UDC_Type | user plane PDCP data PDU with 12 Bit Seq Number for UDC |
| DataExtSN_UDC | PDCP_DataPdu_ExtSN_UDC_Type | user plane PDCP data PDU with 15 Bit Seq Number for UDC |
| Data18bitSN_UDC | PDCP_DataPdu_18bitSN_UDC_Type | user plane PDCP data PDU with 18 Bit Seq Number for UDC |
| UdcFeedback | PDCP_Ctrl_UDC_FB_PDU_Type | PDCP Control PDU for UDC feedback packet |
| EhcFeedback_ShortCID | PDCP_Ctrl_EHC_FB_PDU_ShortCID_Type | PDCP Control PDU for EHC feedback packet Short CID |
| EhcFeedback_LongCID | PDCP_Ctrl_EHC_FB_PDU_LongCID_Type | PDCP Control PDU for EHC feedback packet Long CID |

PDCP_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PDCP_PDUList_Type |
| Comment |  |
| record of PDCP_PDU_Type |  |

## F.4.2	DRB_Primitive_Definitions

Primitive definitions to send/receive data PDUs over DRB's

### F.4.2.1	DRB_Common

L2DataList_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | L2DataList_Type |  |
| Comment | MAC: acc. to rel-8 protocols there is not more than one MAC PDU per TTI; any MAC PDU is completely included in one subframe RLC: one or more RLC PDUs per TTI (e.g. RLC Data + Status PDU on a logical channel; more than one RLC Data PDU in one MAC PDU is valid too) any RLC PDU is completely included in one subframe PDCP: one or more PDUs per TTI; one PDCP PDU may be included in more than one subframe |  |
| MacPdu | MAC_PDUList_Type | SS configuration: RLC TM mode, MAC no header removal (PDCP is not configured) |
| RlcPdu | RLC_PDUList_Type | SS configuration: RLC TM mode, MAC header removal (PDCP is not configured) |
| PdcpPdu | PDCP_PDUList_Type | SS configuration: RLC AM/UM mode, PDCP no header removal |
| PdcpSdu | PDCP_SDUList_Type | SS configuration: RLC AM/UM mode, PDCP header removal |
| NrPdcpSdu | PDCP_SDUList_Type | SS configuration: RLC AM/UM mode, PDCP header removal |
| RlcSdu | RLC_SDUList_Type | SS configuration: RLC UM mode with no PDCP, for example MRB |
| SdapPdu | SDAP_PDUList_Type | SS configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header), no handling of SDAP header |
| SdapSdu | SDAP_SDUList_Type | SS configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header), automatic handling of SDAP header |

HarqProcessAssignment_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | HarqProcessAssignment_Type |  |
| Comment | in DL the HARQ process id may be specified by the test case or automatically assigned by SS |  |
| Id | HarqProcessId_Type | HARQ process as specified by the test case NOTE1: the scope of this type is only for data being sent in one TTI; if data needs more than one TTI the HarqProcessId is undefined for the 2nd TTI onward what shall be handled as an error at the SS; SS may send a SYSTEM_IND indicating an error in this case; NOTE2: The initial value of the NDI shall be the same for all HARQ processes and cells |
| Automatic | Null_Type | HARQ process id automatically assigned by SS |

### F.4.2.2	Downlink

DRB_DataPerSubframe_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DRB_DataPerSubframe_DL_Type |  |  |
| Comment | common definition for one or several PDUs/SDUs; in EUTRA the DL data is sent in the subframe given by the subframe offset; in NBIOT the DL transmission is scheduled in the search space candidate given by "SubframeOffset" for NBIOT it specifies the NOTE 1: For MAC and RLC PDUs a single PDU is always sent in one subframe; SS shall raise an error indication (using SYSTEM_IND) when that is not possible NOTE 2: For PDCP the data may be spread over more than one subframe (segmented by the RLC); the TTCN implemetation is responsible to calculate appropriate offsets accordingly; the exact timing depends on (and is exactly specified by) configuration of the DL scheduling; SS shall raise an error when there is any conflict |  |  |
| SubframeOffset | integer |  | EUTRA: Subframe offset relative to the absolute timing information given in the common part of the ASP; NBIOT: Offset of the search space candidate relative to the (default) search space candidate as configured for the NPDCCH UE specific search space; the search space candidate is in the next search space starting at or after the point in time given by the timing information of the common part of the ASP NOTE 1: Notes: Acc. to TS 36.523-3, clause 7.3.3 in case of TDD or half-duplex configuration only subframes available for DL are taken into consideration NOTE 2: if a PDCP PDU or SDU takes more than one subframe, SubframeOffset specifies the first TTI |
| HarqProcess | HarqProcessAssignment_Type | opt | HARQ process to be used: specific value (0..7) or automatically assigned by SS; in automatic mode SS chooses HARQ process out of the set configured by CcchDcchDtchConfigDL_Type.HarqProcessConfig NOTE 1: for PDCP SDUs or PDUs automatic mode shall be used; otherwise SS shall raise an error NOTE 2: for NB-IoT the HarqProcess shall be omitted by TTCN and ignored by the SS |
| PduSduList | L2DataList_Type |  | list of PDUs/SDUs to be sent in one TTI |

DRB_DataPerSubframeList_DL_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | DRB_DataPerSubframeList_DL_Type |
| Comment | list of user plane data to be sent in sub-frames given by the SubframeOffset in the single elements of the list; Timing: the start time for the whole sequence is given by the timing info of the ASP (common information); the timing for the respective data pdus is given by the SubframeOffset relative to the common timing info; design consideration: repetitions of this sequence are not foreseen (in which case the subframe offset could not be related to the timing info of the ASP) |
| record of DRB_DataPerSubframe_DL_Type |  |

L2Data_Request_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | L2Data_Request_Type |  |  |
| Comment | NOTE: formal type definition to allow later enhancements; L2Data_Request_Type defines a sequence of subframes in which data shall be sent |  |  |
| SubframeDataList | DRB_DataPerSubframeList_DL_Type |  |  |

### F.4.2.3	Uplink

DRB_DataPerSubframe_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DRB_DataPerSubframe_UL_Type |  |  |
| Comment | common definition for one or several PDUs/SDUs being received in one subframe or to receive one PDCP PDU or SDU being spread over more than one TTI; NOTE: There is a fix relation between HARQ process id and subframe in UL => it is not necessary to include HARQ process id for UL data |  |  |
| PduSduList | L2DataList_Type |  | list of PDUs/SDUs being received in one TTI; elements of the list appear in the same order as the PDUs/SDUs in the MAC PDU; for PDCP when a PDU or SDU takes more than one TTI the list only contains this PDU or SDU |
| NoOfTTIs | integer |  | in case of PDCP: number of TTIs the SDU or PDU has taken NOTE 1: for the time being the NoOfTTIs is not checked by TTCN-3 and may be set to 1 by SS; NOTE 2: the timing info in common part of the ASP refers to the last TTI NOTE 3: when NoOfTTIs > 1 => PduSduList shall only contain one PDCP PDU or SDU in case of MAC or RLC PDUs: NoOfTTIs shall always be 1 (acc. to TS 36.321 MAC is not doing segmentation of RLC PDUs and acc. to TS 36.322, clause 6.2.2.2 the maximum RLC data is calculated to fit into a MAC PDU and RLC does segmentation accordingly) |

L2Data_Indication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | L2Data_Indication_Type |  |  |
| Comment | NOTE: formal type definition to allow later enhancements; L2Data_Indication_Type defines data being received in a single subframe i.e. PDUs of subsequent TTIs are indicated in separated ASPs |  |  |
| SubframeData | DRB_DataPerSubframe_UL_Type |  |  |

# F.5	NasEmu_AspTypes_NBIOT

System interface between NAS emulation and system adaptor

## F.5.1	System_Interface

NBIOT_RRC_PDU_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NBIOT_RRC_PDU_REQ |  |  |
| Comment |  |  |  |
| Common | NB_ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB0, SRB1, SRB1bis TimingInfo : Now in normal cases; For latency tests TimingInfo can be set to the SFN/subframe in which the RRC messages shall be sent out NOTE 1: if the RRC PDU is too long to be sent in one TTI the TimingInfo corresponds to the first TTI NOTE 2: the TimingInfo is not changed by the NAS Emu (i.e. the timing info as coming from the test case (SRB_COMMON_REQ) is handed through by the NAS Emu) ControlInfo CnfFlag:=false; FollowOnFlag true: Indicates that the message(s) to be sent on the same TTI will follow NOTE 1: If the TimingInfo is not the same for messages to be sent on the same TTI, the SS shall produce an error NOTE 2: the follow on flag applies only for messages of the same SRB false: Indicates that no more message(s) will follow |
| RrcPdu | NB_RRC_MSG_Request_Type |  |  |

NBIOT_RRC_PDU_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NBIOT_RRC_PDU_IND |  |  |
| Comment | common ASP to receive PDUs from SRB0, SRB1, SRB1bis |  |  |
| Common | NB_IndAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB0, SRB1, SRB1bis TimingInfo : time when message has been received (frame and sub-frame number); this is handed through to the test case by the NAS emulation NOTE: normally an RRC PDU is expected in one TTI; nevertheless if it is spread over more than one TTIs TimingInfo shall refer to the end of the PDU i.e. to the last RLC PDU being received; Status : OK or RRC integrity error |
| RrcPdu | NB_RRC_MSG_Indication_Type |  |  |

NASEMU_NBIOT_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NASEMU_NBIOT_SYSTEM_PORT |  |
| Comment | NASEMU PTC: Port for Sending/Receiving data to/from the SYSTEM Interface |  |
| out | NBIOT_RRC_PDU_REQ |  |
| in | NBIOT_RRC_PDU_IND |  |

# F.6	NBIOT_CommonDefs

## F.6.1	NBIOT_Common_Types

NBIOT_CellId_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NBIOT_CellId_Type |
| Comment |  |
| nbiot_Cell_NonSpecific |  |
| nbiot_Cell1 |  |
| nbiot_Cell2 |  |
| nbiot_Cell3 |  |
| nbiot_Cell4 |  |
| nbiot_Cell5 |  |
| nbiot_Cell6 |  |
| nbiot_Cell10 |  |
| nbiot_Cell11 |  |
| nbiot_Cell12 |  |
| nbiot_Cell13 |  |
| nbiot_Cell14 |  |
| nbiot_Cell18 |  |
| nbiot_Cell23 |  |
| nbiot_Cell50 |  |
| nbiot_Cell51 |  |
| nbiot_Cell52 |  |
| nbiot_Cell53 |  |
| nbiot_Cell54 |  |
| nbiot_Cell55 |  |
| nbiot_Cell56 |  |
| nbiot_Cell57 |  |
| nbiot_Cell58 |  |
| nbiot_Cell59 |  |
| nbiot_Cell60 |  |
| nbiot_Cell61 |  |
| nbiot_Cell62 |  |
| nbiot_Cell63 |  |

NB_CarrierFreq_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CarrierFreq_Type |  |  |
| Comment |  |  |  |
| DL | CarrierFreq_NB_r13 |  |  |
| UL | CarrierFreq_NB_r13 |  |  |

NB_RRC_MSG_Request_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RRC_MSG_Request_Type |  |
| Comment | DL RRC PDU on CCCH or DCCH |  |
| Ccch | DL_CCCH_Message_NB |  |
| Dcch | DL_DCCH_Message_NB |  |

NB_RRC_MSG_Indication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RRC_MSG_Indication_Type |  |
| Comment | UL RRC PDU on CCCH or DCCH |  |
| Ccch | UL_CCCH_Message_NB |  |
| Dcch | UL_DCCH_Message_NB |  |

## F.6.2	NBIOT_RRC_Nested_Types

NBIOT_RRC_Nested_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_OperationModeInfo_Type | MasterInformationBlock_NB.operationModeInfo_r13 |  |
| NB_SchedulingInfoSIB1_Type | MasterInformationBlock_NB.schedulingInfoSIB1_r13 |  |
| NB_SiWindowLength_Type | SystemInformationBlockType1_NB.si_WindowLength_r13 |  |
| NB_EutraControlRegionSize_Type | SystemInformationBlockType1_NB.eutraControlRegionSize_r13 |  |
| NB_SiPeriodicity_Type | SchedulingInfoList_NB_r13[0].si_Periodicity_r13 |  |
| NB_SiRepetitionPattern_Type | SchedulingInfoList_NB_r13[0].si_RepetitionPattern_r13 |  |
| NB_SiTransportBlockSize_Type | SchedulingInfoList_NB_r13[0].si_TB_r13 |  |
| NPDCCH_NumRepetitions_UESS_Type | NPDCCH_ConfigDedicated_NB_r13.npdcch_NumRepetitions_r13 |  |
| NPDCCH_StartSF_UESS_Type | NPDCCH_ConfigDedicated_NB_r13.npdcch_StartSF_USS_r13 |  |
| NPDCCH_Offset_UESS_Type | NPDCCH_ConfigDedicated_NB_r13.npdcch_Offset_USS_r13 |  |
| NPDCCH_NumRepetitions_Type1CSS_Type | PCCH_Config_NB_r13.npdcch_NumRepetitionPaging_r13 |  |
| NPDCCH_NumRepetitions_Type2CSS_Type | NPRACH_Parameters_NB_r13.npdcch_NumRepetitions_RA_r13 |  |
| NPDCCH_StartSF_Type2CSS_Type | NPRACH_Parameters_NB_r13.npdcch_StartSF_CSS_RA_r13 |  |
| NPDCCH_Offset_Type2CSS_Type | NPRACH_Parameters_NB_r13.npdcch_Offset_RA_r13 |  |
| NB_SC_MCCH_CarrierConfig_Type | SystemInformationBlockType20_NB_r14.sc_mcch_CarrierConfig_r14 |  |
| NB_SC_MCCH_RepetitionPeriod_Type | SystemInformationBlockType20_NB_r14.sc_mcch_RepetitionPeriod_r14 |  |
| NB_SC_MCCH_Offset_Type | SystemInformationBlockType20_NB_r14.sc_mcch_Offset_r14 |  |
| NB_SC_MCCH_ModificationPeriod_Type | SystemInformationBlockType20_NB_r14.sc_mcch_ModificationPeriod_r14 |  |
| NB_SC_MCCH_SchedulingInfo_Type | SystemInformationBlockType20_NB_r14.sc_mcch_SchedulingInfo_r14 |  |
| NPDCCH_NumRepetitions_SC_MTCH_Type | SC_MTCH_Info_NB_r14.npdcch_NumRepetitions_SC_MTCH_r14 |  |
| NPDCCH_StartSF_SC_MTCH_Type | SC_MTCH_Info_NB_r14.npdcch_StartSF_SC_MTCH_r14 |  |
| NPDCCH_Offset_SC_MTCH_Type | SC_MTCH_Info_NB_r14.npdcch_Offset_SC_MTCH_r14 |  |
| NPDCCH_NPDSCH_MaxTBS_SC_MTCH_Type | SC_MTCH_Info_NB_r14.npdcch_NPDSCH_MaxTBS_SC_MTCH_r14 |  |
| NB_SC_MTCH_CarrierConfig_Type | SC_MTCH_Info_NB_r14.sc_mtch_CarrierConfig_r14 |  |

## F.6.3	NBIOT_ASP_CommonPart

Definition of ASP common parts for REQ-, CNF- and IND-ASPs

### F.6.3.1	NBIOT_ASP_CommonPart_Definitions

#### F.6.3.1.1	NBIOT_Routing_Info

NBIOT_CommonDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_NB_MaxRB | integer | 2 + 3 | 2 DRBs + 3 SRBs |
| tsc_SRB1bis | integer | 3 |  |

NBIOT_Routing_Info: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_SRB_Identity_Type | integer (tsc_SRB0, tsc_SRB1, tsc_SRB1bis) |  |

NB_RadioBearerId_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RadioBearerId_Type |  |
| Comment |  |  |
| Srb | NB_SRB_Identity_Type |  |
| Drb | EUTRA_ASN1_DRB_Identity_Type |  |
| ScMrb | SC_MRB_Identity_Type |  |

NB_RoutingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RoutingInfo_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| L2Test | Null_Type | to be used to let the NAS emulator route back encoded an RRC message back to the test case instead of forwarding it to the SS |
| RadioBearerId | NB_RadioBearerId_Type |  |

### F.6.3.2	REQ_ASP_CommonPart

NB_ReqAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_ReqAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | NBIOT_CellId_Type |  |  |
| RoutingInfo | NB_RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| ControlInfo | ReqAspControlInfo_Type |  |  |

### F.6.3.3	NBIOT_CNF_ASP_CommonPart

NB_CnfAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CnfAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | NBIOT_CellId_Type |  |  |
| RoutingInfo | NB_RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| Result | ConfirmationResult_Type |  |  |

### F.6.3.4	NBIOT_IND_ASP_CommonPart

NB_IndAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_IndAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | NBIOT_CellId_Type |  |  |
| RoutingInfo | NB_RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| Status | IndicationStatus_Type |  |  |

# F.7	NBIOT_Imported_EUTRA_ASN1_Types

NBIOT_Imported_EUTRA_ASN1_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| EUTRA_ASN1_DRB_Identity_Type | DRB_Identity |  |
| EUTRA_ASN1_PhysCellId_Type | PhysCellId |  |
| EUTRA_ASN1_C_RNTI_Type | C_RNTI |  |
| EUTRA_ASN1_CipheringAlgorithm_r12_Type | CipheringAlgorithm_r12 |  |
| EUTRA_ASN1_Dl_Bandwidth_Type | CarrierBandwidthEUTRA.dl_Bandwidth |  |

# F.8	NBIOT_ASP_VirtualNoiseDefs

ASP definitions for virtual noise generation in NBIOT cells.
The noise is configured for an already existing NBIOT cell.
For UEs with 2 antenna connectors the AWGN (Additive white Gaussian noise) signals applied to each receiver antenna connector shall be uncorrelated.

NBIOT_ASP_VirtualNoiseDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NBIOT_VngConfigConfirm_Type | Null_Type |  |

NBIOT_VngConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NBIOT_VngConfigInfo_Type |  |  |
| Comment |  |  |  |
| NocLevel | integer |  | Noc level; calculation is according to 36.523-3 cl 7.22 |

NBIOT_VngConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NBIOT_VngConfigRequest_Type |  |
| Comment | configure/activate noise for a given cell; NOTE: it is assumed the the associated NBIOT cell has been created beforehand |  |
| Configure | NBIOT_VngConfigInfo_Type | configuration of the virtual noise generator; regardless of the power level the noise generator is off before it gets activated for this cell; in case the configuration needs to be changed during a test, the noise generator shall be deactivated for this cell |
| Activate | Null_Type | noise is activated (switched on) for the given cell acc. to the previous configuration; while being active the configuration shall not be modified |
| Deactivate | Null_Type | deactivate noise for given cell |

NBIOT_VNG_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NBIOT_VNG_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | NB_ReqAspCommonPart_Type |  | CellId : as for the associated NBIOT cell RoutingInfo : None TimingInfo : Now ControlInfo : CnfFlag:=true; FollowOnFlag:=false |
| Request | NBIOT_VngConfigRequest_Type |  |  |

NBIOT_VNG_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NBIOT_VNG_CTRL_CNF |  |  |
| Comment |  |  |  |
| Common | NB_CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN (apart from EnquireTiming) => SS may set TimingInfo to "None" |
| Confirm | NBIOT_VngConfigConfirm_Type |  |  |

NBIOT_VNG_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NBIOT_VNG_PORT |  |
| Comment | NBIOT PTC: Port for virtual noise generator |  |
| out | NBIOT_VNG_CTRL_REQ |  |
| in | NBIOT_VNG_CTRL_CNF |  |

# F.9	CommonDefs

CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| B1_Type | bitstring length(1) |  |
| B2_Type | bitstring length(2) |  |
| B3_Type | bitstring length(3) |  |
| B4_Type | bitstring length(4) |  |
| B5_Type | bitstring length(5) |  |
| B6_Type | bitstring length(6) |  |
| B7_Type | bitstring length(7) |  |
| B7_15_Type | bitstring length(7..15) | NOTE: length restriction can only be a range but not two destinct lengths |
| B8_Type | bitstring length(8) |  |
| B10_Type | bitstring length(10) |  |
| B11_Type | bitstring length(11) |  |
| B12_Type | bitstring length(12) |  |
| B14_Type | bitstring length(14) |  |
| B15_Type | bitstring length(15) |  |
| B16_Type | bitstring length(16) |  |
| B18_Type | bitstring length(18) |  |
| B32_Type | bitstring length(32) |  |
| B128_Type | bitstring length(128) |  |
| B128_Key_Type | B128_Type | 128 bit security key |
| O4_Type | octetstring length(4) |  |
| Null_Type | boolean (true) | dummy type for 'typeless' fields in unions |
| PdcpCountValue_Type | B32_Type |  |

IndicationAndControlMode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IndicationAndControlMode_Type |
| Comment |  |
| enable |  |
| disable |  |

# F.10	CommonAspDefs

## F.10.1	Cell_Configuration_Common

CellTimingInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CellTimingInfo_Type |  |  |
| Comment | Cell Timing |  |  |
| TcOffset | integer (0..63) | opt | For NR according to TS 38.211 clause 4.1 Ts/Tc = 64 with Tc = 1/(480000 * 4096) and Ts = 1/(15000 * 2048) as for EUTRA; => for NR to specify granularity per Tc; for EUTRA to be set to 0 (and/or to be ignored by the SS) |
| Tcell | integer (0..307199) |  | frame duration Tf = 307200 * Ts = 10ms; System Time Unit Ts = 1/(15000 * 2048) |
| SfnOffset | integer (0..1023) |  |  |
| HsfnOffset | integer (0..1023) |  |  |

## F.10.2	MAC_Layer

TransmissionRepetition_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TransmissionRepetition_Type |  |
| Comment |  |  |
| Continuous | Null_Type |  |
| NumOfCycles | integer (1..infinity) |  |

RAR_RapIdCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RAR_RapIdCtrl_Type |  |
| Comment |  |  |
| Automatic | Null_Type | SS shall automatically use same RAPID as received from the UE |
| Unmatched | Null_Type | SS shall use RAPID being different from preamble sent by the UE; SS shall calculate this RAPID acc. to RAPID := (RAPID + 3..63) mod 64 if single RAR is transmitted in a MAC PDU then only 3 is added if multiple RAR's are transmitted in MAC PDU, then for first unmatched RAR 3 is added, second unmatched 4 is added, third unmatched 5 is added and so on |

## F.10.3	System_Indications

HARQ_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | HARQ_Type |
| Comment | ack represents HARQ ACK; nack represents HARQ NACK |
| ack |  |
| nack |  |

## F.10.4	ASP_CommonPart

Definition of ASP common parts for REQ-, CNF- and IND-ASPs

### F.10.4.1	ASP_CommonPart_Definitions

#### F.10.4.1.1	Routing_Info

CommonAspDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_SRB0 | integer | 0 |  |
| tsc_SRB1 | integer | 1 |  |

#### F.10.4.1.2	Timing_Info

Timing_Info: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SystemFrameNumber_Type | integer (0..1023) |  |
| SubFrameNumber_Type | integer (0..9) |  |
| SymbolNumber_Type | integer (0..13) |  |
| HyperSystemFrameNumberInfo_Type | SystemFrameNumberInfo_Type |  |

SubFrameInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SubFrameInfo_Type |  |
| Comment |  |  |
| Number | SubFrameNumber_Type |  |
| Any | Null_Type | no specific sub-frame, in which case the SS may choose the next available subframe (valid for REQ ASPs only) |

SystemFrameNumberInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SystemFrameNumberInfo_Type |  |
| Comment |  |  |
| Number | SystemFrameNumber_Type |  |
| Any | Null_Type | no specific frame number, in which case the SS may choose the next available SFN (valid for REQ ASPs only) |

SlotOffset_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SlotOffset_Type |  |
| Comment | slots per subframe according to TS 38.211 Table 4.3.2-1 |  |
| Numerology0 | Null_Type | mu=0; only one slot per subframe |
| Numerology1 | integer (0..1) | mu=1; 2 slots per subframe |
| Numerology2 | integer (0..3) | mu=2; 4 slots per subframe |
| Numerology3 | integer (0..7) | mu=3; 8 slots per subframe |
| Numerology4 | integer (0..15) | mu=4; 16 slots per subframe |

SlotTimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SlotTimingInfo_Type |  |
| Comment | EUTRA, NBIOT: REQ ASPs: TTCN shall set the SlotTimingInfo to "FirstSlot" for EUTRA, NBIOT addressing the whole subframe IND ASPs: TTCN shall ignore the SlotTimingInfo sent by the SS for EUTRA, NBIOT NR: REQ ASPs: Any:=true only if the slot number is not relevant, in which case the SS may choose the next available slot of the given subframe IND ASPs: Any:=true only if there is no slot information available for the particular kind of indication |  |
| SlotOffset | SlotOffset_Type | to address a particular slot in a subframe |
| FirstSlot | Null_Type | to address the first slot independent from the numerology (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT |
| Any | Null_Type | for IND ASPs in EUTRA and NBIOT or if slot number is not relevant or not available |

SymbolTimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SymbolTimingInfo_Type |  |
| Comment | EUTRA, NBIOT: REQ ASPs: TTCN shall set the SymbolTimingInfo to "FirstSymbol" for EUTRA, NBIOT addressing the whole subframe IND ASPs: TTCN shall ignore the SymbolTimingInfo sent by the SS for EUTRA, NBIOT NR: REQ ASPs: Any:=true only if the symbol number is not relevant, in which case the SS may choose the next available symbol of the given slot IND ASPs: Any:=true only if there is no symbol information available for the particular kind of indication |  |
| SymbolOffset | SymbolNumber_Type | to address a particular symbol in a slot |
| FirstSymbol | Null_Type | to address the first symbol (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT |
| Any | Null_Type | for IND ASPs in EUTRA and NBIOT or if symbol number is not relevant or not available |

SubFrameTiming_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SubFrameTiming_Type |  |  |
| Comment |  |  |  |
| SFN | SystemFrameNumberInfo_Type |  |  |
| Subframe | SubFrameInfo_Type |  |  |
| HSFN | HyperSystemFrameNumberInfo_Type |  |  |
| Slot | SlotTimingInfo_Type |  |  |
| Symbol | SymbolTimingInfo_Type |  |  |

TimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TimingInfo_Type |  |
| Comment |  |  |
| SubFrame | SubFrameTiming_Type |  |
| Now | Null_Type | to be used in REQ ASPs when there is no 'activation time' |
| None | Null_Type | only to be used in SYSTEM_CTRL_CNF or NR_SYSTEM_CTRL_CNF but not for EnquireTiming |

### F.10.4.2	REQ_ASP_CommonPart

ReqAspControlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ReqAspControlInfo_Type |  |  |
| Comment |  |  |  |
| CnfFlag | boolean |  | true => SS shall send CNF: when the REQ is with no timing information (no activation time), SS shall send the confirmation when the configuration is done, i.e. when the test case may continue. Example: when there is a configuration followed by a send event it shall not be necessary to have a wait timer in between but the CNF triggers the send event. If there are other triggers e.g. like the UE sending a message, CnfFlag shall be set to false by the test case to avoid racing conditions with the CNF and the signalling message. When there is an activation time SS shall send the CNF after the configuration has been scheduled; that means SS shall not wait until the activation time has been expired. When FollowOnFlag is true, CnfFlag shall be set to false. |
| FollowOnFlag | boolean |  | false => no further (related) information true: further related information will be sent to SS (semantics depending on respective ASP) |

### F.10.4.3	CNF_ASP_CommonPart

ConfirmationResult_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ConfirmationResult_Type |  |
| Comment |  |  |
| Success | Null_Type |  |
| Error | integer | may contain SS specific error code; this will not be evaluated by TTCN |

### F.10.4.4	IND_ASP_CommonPart

IntegrityErrorIndication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IntegrityErrorIndication_Type |  |  |
| Comment |  |  |  |
| Nas | boolean |  | NAS Integrity: set to true when received MAC does not match calculated MAC |
| Pdcp | boolean |  | PDCP Integrity: set to true when received MAC does not match calculated MAC |

ErrorIndication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ErrorIndication_Type |  |  |
| Comment |  |  |  |
| Integrity | IntegrityErrorIndication_Type |  | Integrity error: received MAC does not match calculated MAC |
| System | integer |  | any other error: may be SS specific error code; this will not be evaluated by TTCN; e.g. an error shall be raised when the UE requests retransmission of an RLC PDU |

IndicationStatus_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IndicationStatus_Type |  |
| Comment |  |  |
| Ok | Null_Type |  |
| Error | ErrorIndication_Type |  |

# F.11	EUTRA_NB_ASP_TypeDefs

Type definitions for configuration of the system simulator;
Common design principles:
Semantics of OMIT: for all TTCN-3 type definitions used in ASPs omit means "keep as it is" =>
- on initial configuration in general all fields shall be provided
- no default values for fields are foreseen
- if necessary non-existence of information shall be explicitly configured
  (e.g. with a union of "no configuration" and "configuration parameters"
- fields within structures imported from the core spec are excepted from this rule

## F.11.1	Cell_Configuration

Specific Info for Cell Configuration Primitive

### F.11.1.1	Cell_Configuration_Common

EUTRA_NB_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_CellAttenuation_Off | Attenuation_Type | {Off:=true} |  |

Cell_Configuration_Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| InitialAttenuation_Type | Attenuation_Type (tsc_CellAttenuation_Off) | Attenuation restricted to 'Off' |

Attenuation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | Attenuation_Type |  |
| Comment | attenuation of the reference power |  |
| Value | integer (0..149) | cell power reference power reduced by the given attenuation (value is in dB) |
| Off | Null_Type | even though in TS 36.508 -145dBm for LTE and -150dBm for NB-IoT is given for a non suitable cell we specify an explicit "Off" value here |

InitialCellPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | InitialCellPower_Type |  |  |
| Comment |  |  |  |
| MaxReferencePower | AbsoluteCellPower_Type |  | maximum value of cell reference power (RS EPRE in dBm/15kHz as per TS 36.508, clause 4.3.4.1); a cell is initialised with this reference power; its value is the upper bound of the cell power during the test case |
| Attenuation | InitialAttenuation_Type |  | initial attenuation |

EUTRA_CyclicPrefix_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | EUTRA_CyclicPrefix_Type |
| Comment | NOTE: in DL extended cyclic prefix depends on sub-carrier spacing |
| normal |  |
| extended |  |

### F.11.1.2	Downlink_Physical_Layer_Configuration

Downlink_Physical_Layer_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| WUS_Durations_Type | integer (1,2,4,8,16,32,64,128,256,512,1024) | In 36.213 Table 16.9-1 in cl 16.9 for NB-IoT and Table 17-1 in cl 17 for BL/CE UE, values are given in subframes |
| TimeOffsetInFrames | integer (4,8,16,24,100,200) | Corresponds to timeOffsetDRX/timeOffset-eDRX-Short-r15/ timeOffset-eDRX-Long-r15 as defined in WUS-Config-NB-r15 for NB-IoT or WUS-Config-r15 for BL/CE UE |

WUS_PowerBoost_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | WUS_PowerBoost_Type |
| Comment |  |
| db0 |  |
| dB1dot8 |  |
| dB3 |  |
| dB4dot8 |  |

### F.11.1.3	Uplink_Physical_Layer_Configuration

SS_TimingAdvanceConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SS_TimingAdvanceConfig_Type |  |
| Comment |  |  |
| InitialValue | RACH_TimingAdvance_Type | initial value corresponding to what is sent to the UE in RACH response (range acc. 11 bit value; 0 in normal cases) |
| Relative | TimingAdvanceIndex_Type | timing advance command to adjust changes of timing advance acc. to TS 36.213, clause 4.2.3; (range acc. 6 bit value: -31..32) |
| NtnTA | NTN_TimingAdvanceConfig_Type | timing advance configuration for NTN operation |

### F.11.1.4	Common_MAC_Configuration

Transport channel and MAC related procedures and configuration

Common_MAC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| TimingAdvanceIndex_Type | integer (0..63) | acc. to TS 36.321, clause 6.1.3.5 "Timing Advance Command MAC Control Element" and TS 36.213, clause 4.2.3 "Transmission timing adjustments" |
| TimingAdvance_Period_Type | integer (400, 600, 1020, 1530, 2040, 4090, 8190) | the values correspond to 80 % of TimeAlignmentTimer (acc. to TS 36.523-3, clause 7.2) (TS 36.331, clause 6.3.2: sf500, sf750, sf1280, sf1920, sf2560, sf5120, sf10240) rounded to nearest multiple of 10 |

UplinkTimeAlignment_AutoSynch_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UplinkTimeAlignment_AutoSynch_Type |  |  |
| Comment |  |  |  |
| TimingAdvance | TimingAdvanceIndex_Type |  |  |
| TA_Period | TimingAdvance_Period_Type |  | time period after which TA MAC control elements need to be automatically transmitted |
| TA_Repetition | TransmissionRepetition_Type |  | number of TA MAC control element repetitions to be automatically transmitted or 'Continuous' |

UplinkTimeAlignment_Synch_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UplinkTimeAlignment_Synch_Type |  |
| Comment |  |  |
| None | Null_Type | no PUCCH Synchronisation applied |
| Auto | UplinkTimeAlignment_AutoSynch_Type | SS automatically maintains PUCCH synchronization at UE If the cell is a - Rel 10 or earlier cell - or Rel 11 or later Pcell - or the Rel-11 or later scell with no STAG-ID configured, the TAG-ID is set to '00' i.e. P-TAG in Timing advance MCE If the cell is Rel 11 scell with STAG-ID configured, the configured stag-ID is used as TAG-ID in Timing advance MCE |

### F.11.1.5	Random_Access_Procedure

Random_Access_Procedure: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RACH_TimingAdvance_Type | integer (0..2047) | 11 bit timing advance as used in RACH response (absolute value) |

ContentionResolution_ContainedDlschSdu_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ContentionResolution_ContainedDlschSdu_Type |  |
| Comment |  |  |
| RlcPduCCCH | octetstring | octetstring of an RLC PDU containing e.g. the RRC Connection Setup; to be sent in the same MAC PDU as the MAC Contention Resolution Control Element (Msg4) |
| PdcpSduDCCH | octetstring | octetstring of a PDCP SDU containing e.g. the RRC Connection Resume; to be sent in the same MAC PDU as the MAC Contention Resolution Control Element (Msg4); the SS shall - apply integrity protection, - add a PDCP header accordingly, - add an AM RLC header, according to TS 36.523-3 clause 7A.8a and clause 7A.13 The logical channel id to be used for the MAC DL-SCH SDU shall be '00001'B corresponding to SRB1 |
| None | Null_Type | MAC PDU containing the MAC Contention Resolution Control Element does not contain an RLC PDU (i.e. RRC Connection Setup is sent in another PDU) |

ContentionResolution_ContainedId_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ContentionResolution_ContainedId_Type |  |
| Comment |  |  |
| XorMask | ContentionResolutionId_Type | When SS receives Contention Resolution ID from the UE, SS shall XOR it with the given mask and use this as Contention Resolution ID; this allows to get an unmatching Contention Resolution ID; in normal cases mask shall be set to tsc_ContentionResolutionId_Unchanged (i.e. the Contention Resolution ID remains unchanged) |
| None | Null_Type | MAC Contention Resolution Control Element is not contained in the MAC PDU sent out as response on Msg3 |

TCRNTI_ContentionResolutionMacPdu_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TCRNTI_ContentionResolutionMacPdu_Type |  |  |
| Comment | NOTE: Either ContainedId or ContainedRlcPdu (or both) shall not be 'none'; (if no Contention Resolution Mac Pdu shall be sent, TCRNTI_ContentionResolutionCtrl_Type.NoContResolID shall be used instead) |  |  |
| ContainedId | ContentionResolution_ContainedId_Type |  | Either the Contention Resolution ID as received from the UE or a modified Contention Resolution ID (XorMask != tsc_ContentionResolutionId_Unchanged) or no Contention Resolution ID at all |
| ContainedRlcPdu | ContentionResolution_ContainedDlschSdu_Type |  | the MAC PDU containing the MAC Contention Resolution Control Element may contain the RRC Connection Setup; in this case the RRC PDU shall be completely encoded been contained in an RLC PDU |

TCRNTI_ContentionResolutionCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TCRNTI_ContentionResolutionCtrl_Type |  |
| Comment | when the UE responds on a Random Access Response with a RRC Connection Request on CCCH and not with a C-RNTI SS shall assume initial Random Access Procedure (TS 36.300, clause 10.1.5.1), i.e. sends a ContentionResolutionId back to the UE |  |
| MacPdu | TCRNTI_ContentionResolutionMacPdu_Type | MAC PDU containing the Contention Resolution ID and optionally an RRC PDU (RRC Connection Setup) |
| MacPdu_CRC_Error | TCRNTI_ContentionResolutionMacPdu_Type | same as MacPdu (see above), but SS shall generate CRC error by toggling CRC bits; no retransmissions shall be made as UE shall not send a NACK |
| NoContResolID | Null_Type | SS shall not include contention resolution ID (i.e. no MAC PDU shall be sent); used for contention resolution fail case |

TempC_RNTI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TempC_RNTI_Type |  |
| Comment |  |  |
| SameAsC_RNTI | Null_Type | in the RA response SS shall use the same C-RNTI as configured in ActiveCellConfig_Type; this is useful for initial random access |
| Explicit | C_RNTI | in the RA response SS shall use different value as configured in ActiveCellConfig_Type; this can be used when the UE already is in RRC_CONNECTED to have a temporary C-RNTI different from the one used by the UE; NOTE: when the UE is not in RRC_CONNECTED there shall be no explicit temp. C-RNTI since then the UE would assume this value as C-RNTI |

RandomAccessBackoffIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RandomAccessBackoffIndicator_Type |  |
| Comment |  |  |
| None | Null_Type | normal case, no back off indicator included |
| Index | integer (0..15) | Backoff Parameter values acc. TS 36.321, clause 7.2; values 0..12 are defined, 13..15 may be used in error case |

### F.11.1.6	NTN_Configuration

NTN_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NTN_T_TA_Type | ServingSatelliteInfo_r17.nta_CommonParameters_17.nta_Common_r17 | acc. to TS 36.331 clause 6.3.1 |

NTN_SemistaticTA_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NTN_SemistaticTA_Type |  |  |
| Comment | see Table 7A.14.2.2-1 for further information |  |  |
| InitialValue | RACH_TimingAdvance_Type |  | initial value corresponding to what is sent to the UE in RACH response; (range acc. 11 bit value; 0 in normal cases) |
| NtnCommonTA | NTN_T_TA_Type |  | round trip time (RTT) between the UL time synchronisation reference point and the serving satellite |
| NtnUeSpecificTA | NTN_T_TA_Type |  | round trip time (RTT) between the UE and the serving satellite |

NTN_TimingAdvanceConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NTN_TimingAdvanceConfig_Type |  |
| Comment |  |  |
| Semistatic | NTN_SemistaticTA_Type | semi-static configuration of the NTN cell's Timing Advance (TA); SS shall not autonomously change the value of TA over time; TTCN may re-configure the TA value |

# F.12	EUTRA_NB_CommonDefs

## F.12.1	Common_Types

Common_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| LogicalChannelId_Type | integer (0..10) | acc. TS 36.331, clause 6.3.2 and clause 6.7.3.2 for DRBs DTCH-LogicalChannelIdentity is INTEGER (3..10); additionally we have 0..2 for the SRBs |
| HarqProcessId_Type | integer (0..14) | The values 0..7 represent the ID of HARQ process ID; value range 0..14 is for TDD |
| ContentionResolutionId_Type | bitstring length(48) | used in EUTRA_ASP_DrbDefs and EUTRA_ASP_Typedefs |
| AbsoluteCellPower_Type | integer (-150..0) | absolute cell power (dBm) |

## F.12.2	RRC_Nested_Types

RRC_Nested_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| IntegrityProtAlgorithm_Type | SecurityAlgorithmConfig.integrityProtAlgorithm |  |

## F.12.3	L2Data_CommonDefs

Common definitions of L2 control elements used in L2 messages (EUTRA_NB_ASP_L2DataDefs) as well as for SS configuration (EUTRA_ASP_TypeDefs)

L2Data_CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| MAC_CTRL_ScellActDeact_Type | ScellBitMap_Type | 36.321 clause 6.1.3.8 |

ScellBitMap_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ScellBitMap_Type |  |  |
| Comment |  |  |  |
| Value | B7_Type |  | 7 MSB bits the C Fields C7 to C1. 1 => the corresponding Scell is Active 0 => Inactive |
| Reserved | B1_Type |  | LSBit Reserved. Shall be set to 0 |

PH_Record_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PH_Record_Type |  |  |
| Comment | 36.321 clause 6.1.3.6a |  |  |
| P_Bit | B1_Type |  | P bit: 1 indicates the UE applies power backoff due to power management |
| V_Bit | B1_Type |  | V bit: Indicates if the PH value is based on a real transmission or a reference format. For Type 1 PH, V=0 indicates real transmission on PUSCH and V=1 indicates that a PUSCH reference format is used |
| Valve | B6_Type |  | The power headroom level. Ph Type 2 (if configured) for Pcell and Type 1 for Pcell and Scell |
| Reserved | B2_Type | opt | 2 reservid bits. Present if V=1 |
| PCMaxc | B6_Type | opt | Present if V=1 |

PH_RecordList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PH_RecordList_Type |
| Comment | If simultaneousPUCCH-PUSCH is not configured at least oneType 1 PH record for Pcell is present. Additional one record per Scell for which respective 'C' bit is set as 1. If simultaneousPUCCH-PUSCH is configured then one PH Type 2 record for P cell followed by PH Type 1 record for P cell is present. Additional one record per Scell for which respective 'C' bit is set as 1 |
| record length(1..9) of PH_Record_Type |  |

MAC_CTRL_ExtPowerHeadRoom_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_ExtPowerHeadRoom_Type |  |  |
| Comment |  |  |  |
| EPH_Octet1 | ScellBitMap_Type |  |  |
| PH_RecordList | PH_RecordList_Type |  | At least one record for Pcell is present. Additional one record per Scell for which respective 'C' bit is set as 1 |

DC_PH_RecordList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | DC_PH_RecordList_Type |
| Comment | If simultaneousPUCCH-PUSCH is not configured at least oneType 1 PH record for Pcell is present. Additional one record per PScell/Scell for which respective 'C' bit is set as 1. If simultaneousPUCCH-PUSCH is configured then one PH Type 2 record for Pcell followed one PH Type 2 record for PScell followed by PH Type 1 record for Pcell is present. Additional one record per PSCell/Scell for which respective 'C' bit is set as 1 |
| record length(1..10) of PH_Record_Type |  |

MAC_CTRL_DC_PowerHeadRoom_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_DC_PowerHeadRoom_Type |  |  |
| Comment |  |  |  |
| DC_PH_Octet1 | ScellBitMap_Type |  |  |
| DC_PH_RecordList | DC_PH_RecordList_Type |  | At least one record for Pcell is present. Additional one record per PScell/Scell for which respective 'C' bit is set as 1 |

MAC_CTRL_DifferentialKoffset_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_DifferentialKoffset_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.22 |  |  |
| Reserved | B2_Type |  | 2 bits are reserved |
| Value | B6_Type |  | the differential Koffset in subframes |

MAC_CTRL_TimingAdvanceReport_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_TimingAdvanceReport_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.20 |  |  |
| Reserved | B2_Type |  | 2 bits are reserved |
| Value | B14_Type |  | the least integer number of subframes greater than or equal to the Timing Advance value |

## F.12.4	SC_PTM_Types

SC_MRB_Identity_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SC_MRB_Identity_Type |  |  |
| Comment |  |  |  |
| MbmsSessionInfo | MBMSSessionInfo_r13 |  |  |

# F.13	References to TTCN-3

| References to TTCN-3 |  |  |
| --- | --- | --- |
| NBIOT_ASP_TypeDefs | Common/NBIOT_ASP_TypeDefs.ttcn | Rev 36614 |
| NBIOT_ASP_SrbDefs | Common/NBIOT_ASP_SrbDefs.ttcn | Rev 20533 |
| NBIOT_ASP_L2DataDefs | Common/NBIOT_ASP_L2DataDefs.ttcn | Rev 20533 |
| EUTRA_NB_ASP_L2DataDefs | Common4G/EUTRA_NB_ASP_L2DataDefs.ttcn | Rev 36615 |
| NasEmu_AspTypes_NBIOT | NasEmulation/NasEmu_AspTypes_NBIOT.ttcn | Rev 17143 |
| NBIOT_CommonDefs | Common/NBIOT_CommonDefs.ttcn | Rev 32163 |
| NBIOT_Imported_EUTRA_ASN1_Types | Common/NBIOT_Imported_EUTRA_ASN1_Types.ttcn | Rev 34389 |
| NBIOT_ASP_VirtualNoiseDefs | Common/NBIOT_ASP_VirtualNoiseDefs.ttcn | Rev 20533 |
| CommonDefs | Common/CommonDefs.ttcn | Rev 36499 |
| CommonAspDefs | Common/CommonAspDefs.ttcn | Rev 34526 |
| EUTRA_NB_ASP_TypeDefs | Common4G/EUTRA_NB_ASP_TypeDefs.ttcn | Rev 36032 |
| EUTRA_NB_CommonDefs | Common4G/EUTRA_NB_CommonDefs.ttcn | Rev 36616 |