3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230673

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3190 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new verified and e-mail agreed TTCN test cases in the TC lists in 38.523-3 (prose), Annex A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - New RAN5 e-mail agreed TTCN test cases have not been documented in the test case lists (cut-off date on 4th September 2023). Updating the approved TC list is required. - Annex D for the TTCN type definitions should be updated to document the changes in the TTCN delivery iwd-23wk37 (Friday 15th September 2023). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the new e-mail agreed TTCN test cases in the test suite lists. - Updated Annex D. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The new RAN approved test cases at this meeting will not be included in the ATSs in the upcoming version. The list will be incomplete. - The ASP changes will not be correctly documented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A, Annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex A (normative):
Test Suites

This annex references the approved Test Suites, which accompany the present document. The Test Suites have been produced using the Testing and Test Control Notation version 3 (TTCN-3) according to ES 201 873 [4].

# A.1	Baseline of specifications

Table A.1-1 lists the core specifications and test specifications, which the delivered Test Suites are based upon.

Table A.1-1: References of the test and core specifications

| Type | Specification | Release | Version |
| --- | --- | --- | --- |
| Core specifications | TS 38.321 [13] | Note 1 | Note 2 |
|  | TS 38.322 [14] | Note 1 | Note 2 |
|  | TS 38.323 [15] | Note 1 | Note 2 |
|  | TS 36.331 [17] | Note 1 | Note 2 |
|  | TS 38.331 [16] | Note 1 | Note 2 |
|  | TS 24.301 [18] | Note 1 | Note 2 |
| Test specifications | TS 36.508 [10] | Note 2 | Note 2 |
|  | TS 36.509 [11] | Note 1 | Note 2 |
|  | TS 38.508-1 [5] | Note 2 | Note 2 |
|  | TS 38.508-2 [6] | Note 2 | Note 2 |
|  | TS 38.509 [7] | Note 1 | Note 2 |
|  | TS 38.523-1 [8] | Note 2 | Note 2 |
|  | TS 38.523-2 [9] | Note 2 | Note 2 |
| NOTE 1: Latest release available, up to the release number of the present document. NOTE 2: Latest available. |  |  |  |

# A.2	5GS Test Suites

## A.2.1	EN-DC Test Suites

Table A.2.1-1 lists all approved test cases.

For a given test case, the following variants are distinguished (if applicable):

- FR1: FR1 NR frequency band(s) in the NR cell(s).

- FR2: FR2 NR frequency band(s) in the NR cell(s).

- FRx: Mix of FR1 NR frequency band(s) and FR2 NR frequency band(s) in the NR cell(s).

An "X" in columns FR1, FR2 or FRx indicates the test case is approved for the respective variant.

An "-" in columns FR1, FR2 or FRx indicates the test case is not applicable to the respective variant.

Table A.2.1-1: EN-DC TTCN test cases

| Test case |  | Description |  | FR1 |  | FR2 |  | FRx |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1.1.ENDC |  | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure |  | X |  | X |  | - |  |
| 7.1.1.1.1a.ENDC |  | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure |  | X |  | X |  | - |  |
| 7.1.1.1.2.ENDC |  | Random access procedure / Successful/ C-RNTI Based/Preamble selected by MAC itself |  | X |  | X |  | - |  |
| 7.1.1.2.1.ENDC |  | Correct Handling of DL MAC PDU / Assignment / HARQ process |  | X |  | X |  | - |  |
| 7.1.1.2.2.ENDC |  | Correct Handling of DL HARQ process PDSCH Aggregation |  | X |  |  |  | - |  |
| 7.1.1.3.1.ENDC |  | Correct Handling of UL MAC PDU / Assignment / HARQ process |  | X |  | X |  | - |  |
| 7.1.1.3.2.ENDC |  | Logical channel prioritization handling |  | X |  | X |  | - |  |
| 7.1.1.3.2b.ENDC |  | Logical channel prioritization handling with Mapping restrictions |  | X |  | X |  | - |  |
| 7.1.1.3.3.ENDC |  | Correct handling of MAC control information / Scheduling requests |  | X |  | X |  | - |  |
| 7.1.1.3.4.ENDC |  | Correct handling of MAC control information / Buffer status / UL data arrive in the UE Tx buffer / Regular BSR |  | X |  | X |  | - |  |
| 7.1.1.3.5.ENDC |  | Correct handling of MAC control information / Buffer Status / UL resources are allocated / Padding BSR |  | X |  | X |  | - |  |
| 7.1.1.3.6.ENDC |  | Correct handling of MAC control information / Buffer status / Periodic BSR timer expires |  | X |  | X |  | - |  |
| 7.1.1.3.7.ENDC |  | UE power headroom reporting / Periodic reporting / DL pathloss change reporting |  | X |  | X |  | - |  |
| 7.1.1.3.9.ENDC |  | Correct Handling of UL HARQ process/ PUSCH Aggregation |  | X |  | X |  | - |  |
| 7.1.1.3.8.1.ENDC |  | UE power headroom reporting / SCell activation / DL pathloss change reporting / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 7.1.1.3.8.2.ENDC |  | UE power headroom reporting / SCell activation / DL pathloss change reporting/ Inter-band CA |  | X |  |  |  |  |  |
| 7.1.1.4.1.1.ENDC |  | DL-SCH Transport Block Size selection / DCI format 1_0 |  | X |  |  |  | - |  |
| 7.1.1.4.2.1.ENDC |  | UL-SCH transport block size selection / DCI format 0_0 / Transform precoding disabled |  | X |  |  |  | - |  |
| 7.1.1.4.2.3.ENDC |  | UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / Transform precoding disabled |  | X |  |  |  | - |  |
| 7.1.1.4.2.4.ENDC |  | UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled |  | X |  |  |  | - |  |
| 7.1.1.4.2.5.ENDC |  | UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM |  | X |  |  |  | - |  |
| 7.1.1.5.1.ENDC |  | DRX operation / Short cycle not configured / Parameters configured by RRC |  | X |  | X |  | - |  |
| 7.1.1.5.2.ENDC |  | DRX operation / Short cycle not configured /Long DRX command MAC control element reception |  | X |  | X |  | - |  |
| 7.1.1.5.3.ENDC |  | DRX operation / Short cycle configured / Parameters configured by RRC |  | X |  | X |  | - |  |
| 7.1.1.5.4.ENDC |  | DRX Operation / Short cycle configured / DRX command MAC control element reception |  | X |  | X |  | - |  |
| 7.1.1.5.5.ENDC |  | DRX Operation / Short cycle configured / Long DRX command MAC control element reception |  | X |  | X |  | - |  |
| 7.1.1.6.1.ENDC |  | Correct handling of DL assignment / Semi-persistent case |  | X |  |  |  | - |  |
| 7.1.1.6.2.ENDC |  | Correct handling of UL grant / configured grant Type 1 |  | X |  |  |  | - |  |
| 7.1.1.6.3.ENDC |  | Correct handling of UL grant / configured grant Type 2 |  | X |  |  |  | - |  |
| 7.1.1.7.1.1.ENDC |  | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 7.1.1.7.1.2.ENDC |  | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Inter-band CA |  | X |  |  |  |  |  |
| 7.1.1.7.1.3.ENDC |  | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band non-Contiguous CA |  | X |  | X |  | - |  |
| 7.1.1.8.1.ENDC |  | Bandwidth Part (BWP) operation UL/DL |  | X |  |  |  | - |  |
| 7.1.1.9.1.ENDC |  | MAC Reset |  | X |  | X |  | - |  |
| 7.1.1.12.3.ENDC |  | DRX adaptation / UE wakeup indication |  | X |  |  |  | - |  |
| 7.1.2.2.1.ENDC |  | UM RLC / Segmentation and reassembly / 6-bit SN / Segmentation Info (SI) field |  | X |  | X |  | - |  |
| 7.1.2.2.2.ENDC |  | UM RLC / Segmentation and reassembly / 12-bit SN / Segmentation Info (SI) field |  | X |  | X |  | - |  |
| 7.1.2.2.3.ENDC |  | UM RLC / 6-bit SN / Correct use of sequence numbering |  | X |  | X |  | - |  |
| 7.1.2.2.4.ENDC |  | UM RLC / 12-bit SN / Correct use of sequence numbering |  | X |  | X |  | - |  |
| 7.1.2.2.5.ENDC |  | UM RLC / Receive Window operation and t-Reassembly expiry |  | X |  | X |  | - |  |
| 7.1.2.2.6.ENDC |  | UM RLC / RLC re-establishment procedure |  | X |  | X |  | - |  |
| 7.1.2.3.1.ENDC |  | AM RLC / 12-bit SN/Segmentation and reassembly / Segmentation Info (SI) field |  | X |  | X |  | - |  |
| 7.1.2.3.2.ENDC |  | AM RLC / 18-bit SN/Segmentation and reassembly / Segmentation Info (SI) field |  | X |  | X |  | - |  |
| 7.1.2.3.3.ENDC |  | AM RLC / 12-bit SN / Correct use of sequence numbering |  | X |  | X |  | - |  |
| 7.1.2.3.4.ENDC |  | AM RLC / 18-bit SN / Correct use of sequence numbering |  | X |  | X |  | - |  |
| 7.1.2.3.5.ENDC |  | AM RLC / Control of transmit window / Control of receive window |  | X |  | X |  | - |  |
| 7.1.2.3.6.ENDC |  | AM RLC / Polling for status |  | X |  | X |  | - |  |
| 7.1.2.3.7.ENDC |  | AM RLC / Receiver status triggers |  | X |  | X |  | - |  |
| 7.1.2.3.8.ENDC |  | AM RLC / Reconfiguration of RLC parameters by upper layers |  | X |  | X |  | - |  |
| 7.1.2.3.9.ENDC |  | AM RLC / Reassembling of AMD PDUs |  | X |  | X |  | - |  |
| 7.1.2.3.10.ENDC |  | AM RLC / Re-transmission of RLC PDU with and without re-segmentation |  | X |  | X |  | - |  |
| 7.1.2.3.11.ENDC |  | AM RLC / RLC re-establishment procedure |  | X |  | X |  | - |  |
| 7.1.3.1.1.ENDC |  | Maintenance of PDCP sequence numbers / User plane / 12-bit SN |  | X |  | X |  | - |  |
| 7.1.3.1.2.ENDC |  | Maintenance of PDCP sequence numbers / User plane / 18-bit SN |  | X |  | X |  | - |  |
| 7.1.3.2.1.ENDC |  | Integrity protection / Correct functionality of encryption algorithm SNOW3G / SRB / DRB |  | X |  | X |  | - |  |
| 7.1.3.2.2.ENDC |  | Integrity protection / Correct functionality of encryption algorithm AES / SRB / DRB |  | X |  | X |  | - |  |
| 7.1.3.2.3.ENDC |  | Integrity protection / Correct functionality of encryption algorithm ZUC / SRB / DRB |  | X |  | X |  | - |  |
| 7.1.3.3.1.ENDC |  | Ciphering and deciphering / Correct functionality of encryption algorithm SNOW3G / SRB / DRB |  | X |  | X |  | - |  |
| 7.1.3.3.2.ENDC |  | Ciphering and deciphering / Correct functionality of encryption algorithm AES / SRB / DRB |  | X |  | X |  | - |  |
| 7.1.3.3.3.ENDC |  | Ciphering and deciphering / Correct functionality of encryption algorithm ZUC / SRB / DRB |  | X |  | X |  | - |  |
| 7.1.3.4.1.ENDC |  | PDCP handover / Lossless handover / PDCP sequence number maintenance/PDCP status report to convey the information on missing or acknowledged PDCP SDUs at handover/ In-order delivery and duplicate elimination in the downlink |  | X |  | X |  | - |  |
| 7.1.3.4.2.ENDC |  | PDCP handover / Non-lossless handover / PDCP sequence number maintenance |  | X |  | X |  | - |  |
| 7.1.3.5.1.ENDC |  | PDCP Discard |  | X |  | X |  | - |  |
| 7.1.3.5.2.ENDC |  | PDCP Uplink Routing / Split DRB |  | X |  | X |  | - |  |
| 7.1.3.5.3.ENDC |  | PDCP Data Recovery |  | X |  | X |  | - |  |
| 7.1.3.5.4.ENDC |  | PDCP reordering / Maximum re-ordering delay below t-Reordering / t-Reordering timer operations |  | X |  | X |  | - |  |
| 7.1.3.5.5.ENDC |  | PDCP Duplication |  | X |  | X |  | - |  |
| 8.2.1.1.1.ENDC |  | UE capability transfer / Success / EN-DC |  | X |  | X |  | - |  |
| 8.2.2.1.1.ENDC |  | SRB3 Establishment, Reconfiguration and Release / NR addition, modification and release / EN-DC |  | X |  | X |  | - |  |
| 8.2.2.2.1.ENDC |  | Split SRB Establishment and Release / EN-DC |  | X |  | X |  | - |  |
| 8.2.2.3.1.ENDC |  | Simultaneous SRB3 and Split SRB / Sequential message flow on SRB3 and Split SRB with one UL path / EN-DC |  | X |  | X |  | - |  |
| 8.2.2.4.1.ENDC |  | PSCell addition, modification and release / SCG DRB / EN-DC |  | X |  | X |  | - |  |
| 8.2.2.5.1.ENDC |  | PSCell addition, modification and release / Split DRB / EN-DC |  | X |  | X |  | - |  |
| 8.2.2.6.1.ENDC |  | Bearer Modification / MCG DRB / SRB / PDCP version change / EN-DC |  | X |  | X |  | - |  |
| 8.2.2.7.1.ENDC |  | Bearer Modification / Handling for bearer type change without security key change / EN-DC |  | X |  | X |  | - |  |
| 8.2.2.8.1.ENDC |  | Bearer Modification / Handling for bearer type change with security key change / EN-DC |  | X |  | X |  | - |  |
| 8.2.2.9.1.ENDC |  | Bearer Modification / Uplink data path / Split DRB Reconfiguration / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.1.1.ENDC |  | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.2.1.ENDC |  | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells / RSRQ based measurements / EN-DC |  | X |  |  |  | - |  |
| 8.2.3.3.1.ENDC |  | Measurement configuration control and reporting / Inter-RAT measurements / Periodic reporting / Measurement of NR cells / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.4.1.ENDC |  | Measurement configuration control and reporting / Event A1 / Measurement of NR PSCell / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.5.1.ENDC |  | Measurement configuration control and reporting / Event A2 / Measurement of NR PSCell / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.6.1.ENDC |  | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cell / Intra-frequency measurements / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.6.1a.ENDC |  | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cell / Inter-frequency measurements / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.6.1b.ENDC |  | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cell / Inter-band measurements / EN-DC |  |  |  | X |  | - |  |
| 8.2.3.7.1.ENDC |  | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Intra-frequency measurements / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.7.1a.ENDC |  | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Inter-frequency measurements / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.7.1b.ENDC |  | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Inter-band measurements / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.8.1.ENDC |  | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Intra-frequency measurements / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.8.1a.ENDC |  | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Inter-frequency measurements / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.8.1b.ENDC |  | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Inter-band measurements / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.12.1.ENDC |  | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of NR cells / EN-DC |  | X |  |  |  | - |  |
| 8.2.3.13.1.ENDC |  | PCell Handover with SCG change / Reconfiguration with sync / SCG DRB / EN-DC |  | X |  |  |  | - |  |
| 8.2.3.14.1.ENDC |  | SCG change / Reconfiguration with sync / Split DRB / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.15.1.ENDC |  | Measurement configuration control and reporting / Two simultaneous events A2 and A3 (intra-frequency measurements) / Measurement of Neighbour NR cells / EN-DC |  | X |  |  |  | - |  |
| 8.2.3.16.1.ENDC |  | Measurement configuration control and reporting / SRB3 / Intra NR measurements / EN-DC |  | X |  | X |  | - |  |
| 8.2.3.17.1.ENDC |  | Measurement configuration control and reporting / SFTD / EN-DC |  | X |  |  |  | - |  |
| 8.2.4.1.1.1.ENDC |  | NR CA / NR SCell addition / modification / release / Success / EN-DC / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.1.1.2.ENDC |  | NR CA / NR SCell addition / modification / release / Success / EN-DC / Intra-band non-Contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.1.1.3.ENDC |  | NR CA / NR SCell addition / modification / release / Success / EN-DC / Inter-band CA |  | X |  |  |  | - |  |
| 8.2.4.1.1.4.ENDC |  | NR CA / NR SCell addition / modification / release / Success / EN-DC / Active SCG SCell addition / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.1.1.5.ENDC |  | NR CA / NR SCell addition / modification / release / Success / EN-DC / Active SCG SCell addition / Intra-band non-Contiguous CA |  | X |  |  |  | - |  |
| 8.2.4.1.1.6.ENDC |  | NR CA / NR SCell addition / modification / release / Success / EN-DC / Active SCG SCell addition / Inter-band CA |  | X |  |  |  |  |  |
| 8.2.4.2.1.1.ENDC |  | NR CA / Simultaneous PSCell and SCell addition / PSCell and SCell change / CA Release / EN-DC / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.2.1.2.ENDC |  | NR CA / Simultaneous PSCell and SCell addition / PSCell and SCell change / CA Release / EN-DC / Intra-band non-Contiguous CA |  | X |  |  |  | - |  |
| 8.2.4.2.1.3.ENDC |  | NR CA / Simultaneous PSCell and SCell addition / PSCell and SCell change / CA Release / EN-DC / Inter-band CA |  | X |  |  |  |  |  |
| 8.2.4.3.1.1.ENDC |  | NR CA / SCell change / Intra-NR measurement event A6 / SRB3 / EN-DC / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.3.1.2.ENDC |  | NR CA / SCell change / Intra-NR measurement event A6 / SRB3 / EN-DC / Intra-band non-Contiguous CA |  | X |  |  |  | - |  |
| 8.2.4.3.1.3.ENDC |  | NR CA / SCell change / Intra-NR measurement event A6 / SRB3 / EN-DC / Inter-band CA |  | X |  |  |  |  |  |
| 8.2.5.1.1.ENDC |  | Radio link failure / PSCell addition failure - random access problem / EN-DC |  | X |  | X |  |  |  |
| 8.2.5.2.1.ENDC |  | Radio link failure / PSCell out of sync indication / Radio link failure / EN-DC |  | X |  | X |  | - |  |
| 8.2.5.3.1.ENDC |  | Radio link failure / rlc-MaxNumRetx failure / EN-DC |  | X |  | X |  |  |  |
| 8.2.5.4.1.ENDC |  | Reconfiguration failure / SCG change failure / EN-DC |  | X |  | X |  | - |  |
| 8.2.6.2.1.ENDC |  | Processing delay / PSCell addition / SCG DRB / Success / Latency check / EN-DC |  | X |  | X |  | - |  |
| 10.2.1.1.ENDC |  | Default EPS bearer context activation |  | X |  | X |  | - |  |
| 10.2.1.2.ENDC |  | Dedicated EPS bearer context activation |  | X |  | X |  | - |  |
| 10.2.2.1.ENDC |  | EPS bearer resource allocation / modification |  | X |  | X |  | - |  |

## A.2.2	NR/5GC Test Suites

Table A.2.2-1 lists all approved test cases.

For a given test case, the following variants are distinguished (if applicable):

-	FR1: FR1 NR frequency band(s) in the NR cell(s).

-	FR2: FR2 NR frequency band(s) in the NR cell(s).

-	FRx: Mix of FR1 NR frequency band(s) and FR2 NR frequency band(s) in the NR cell(s).

An "X" in columns FR1 or FR2 indicates the test case is approved for the respective variant.

An "-" in columns FR1, FR2 or FRx indicates the test case is not applicable to the respective variant.

Table A.2.2-1: NR/5GC TTCN test cases

| Test case |  |  | Description |  |  |  | FR1 |  |  |  |  | FR2 |  |  |  |  | FRx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1.NR5GC |  |  | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.1.2.NR5GC |  |  | PLMN selection of "Other PLMN/access technology combinations" / Automatic mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.1.3.NR5GC |  |  | Cell reselection of ePLMN in manual mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.1.4.NR5GC |  |  | PLMN selection in shared network environment / Automatic mode |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 6.1.1.5.NR5GC |  |  | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.15a.NR5GC |  |  | Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.1.6.NR5GC |  |  | PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.1.7.NR5GC |  |  | PLMN selection of RPLMN or (E)HPLMN; Automatic mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.1.8.NR5GC |  |  | PLMN selection of RPLMN or (E)HPLMN; Manual mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.1.NR5GC |  |  | Cell Selection/Qrxlevmin & Cell Reselection (Intra NR) |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 6.1.2.2.NR5GC |  |  | Cell Selection/Qqualmin/Intra NR / Serving cell becomes non-suitable (Srxlev > 0, Squal < 0) |  |  |  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.3.NR5GC |  |  | Cell selection / Intra NR/ Serving cell becomes non-suitable (S<0 , MIB Indicated barred) |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 6.1.2.4.NR5GC |  |  | Cell reselection for interband operation |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.5.NR5GC |  |  | Cell reselection for interband operation using Pcompensation / Between FDD and TDD |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.7.NR5GC |  |  | Cell reselection / Equivalent PLMN |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.8.NR5GC |  |  | Cell reselection / Equivalent PLMN / Single Frequency operation |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 6.1.2.9.NR5GC |  |  | Cell reselection using Qhyst, Qoffset and Treselection |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 6.1.2.11.NR5GC |  |  | Area Specific SIBs using systemInformationAreaID |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.12.NR5GC |  |  | Cell reselection using cell status and cell reservations / cellReservedForOtherUse |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.13.NR5GC |  |  | Cell reselection using cell status and cell reservations / Access Identity 1 , 2 and 12 to 14 - cellReservedForOperatorUse |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.14.NR5GC |  |  | Cell reselection using cell status and cell reservations / Access Identity 11 or 15 - cellReservedForOperatorUse |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.15.NR5GC |  |  | Cell reselection in shared network environment |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 6.1.2.16.NR5GC |  |  | Inter-frequency cell reselection (equal priority) |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.17.NR5GC |  |  | Cell reselection / Cell-specific reselection parameters provided by the network in a neighbouring cell list |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 6.1.2.18.NR5GC |  |  | Cell reselection, Sintrasearch, Snonintrasearch |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.19.NR5GC |  |  | Speed Dependent Cell Reselection |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.20.NR5GC |  |  | Inter-frequency cell reselection according to cell reselection priority provided by SIBs |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.21.NR5GC |  |  | Cell reselection , SIntraSearchQ and SnonIntraSearchQ |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.22.NR5GC |  |  | Inter-frequency cell reselection based on common priority information with parameters ThreshX, HighQ, ThreshX, LowQ and ThreshServing, LowQ |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.23.NR5GC |  |  | Cell Reselection / MFBI |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.26.NR5GC |  |  | Cell Selection / RedCap |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.1.2.27.NR5GC |  |  | Cell reselection / inter-freguency / RedCap |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.1.1.NR5GC |  |  | Inter-RAT PLMN Selection / Selection of correct RAT for OPLMN / Automatic mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.1.2.NR5GC |  |  | Inter-RAT PLMN Selection / Selection of correct RAT for UPLMN / Automatic mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.1.3.NR5GC |  |  | Inter-RAT PLMN Selection / Selection of correct PLMN and RAT in shared network environment / Automatic mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.1.4.NR5GC |  |  | Inter-RAT PLMN Selection / Selection of correct RAT from the OPLMN list / Manual mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.1.5.NR5GC |  |  | Inter-RAT Background HPLMN Search / Search for correct RAT for HPLMN / Automatic Mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.2.1.NR5GC |  |  | Inter-RAT cell selection / From NR RRC_IDLE to EUTRA_Idle / Serving cell becomes non-suitable |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.2.2.NR5GC |  |  | Inter-RAT cell selection / From E-UTRA_Idle to NR RRC_IDLE / Serving cell becomes non-suitable |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.3.1.NR5GC |  |  | Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE (lower priority & higher priority , Srxlev based) |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.3.2.NR5GC |  |  | Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE (lower priority & higher priority , Squal based) |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.3.3.NR5GC |  |  | Inter-RAT cell reselection / From NR RRC_Idle to E-UTRA_IDLE (lower priority & higher priority , Srxlev based) |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.3.4.NR5GC |  |  | Inter-RAT cell reselection / From NR RRC_Idle to E-UTRA_IDLE (lower priority & higher priority , Squal based) |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.3.5.NR5GC |  |  | Inter-RAT cell reselection / From NR RRC_IDLE to E-UTRA_Idle according to RAT priority provided by dedicated signalling (RRCRelease) |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.3.6.NR5GC |  |  | Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE according to RAT priority provided by dedicated signalling (RRConnRelease) |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.3.7.NR5GC |  |  | Inter-RAT cell reselection / From NR RRC_IDLE to E-UTRA RRC_Idle, Snonintrasearch |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.3.8.NR5GC |  |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to NR RRC_Idle, Snonintrasearch |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.3.9.NR5GC |  |  | Speed Dependent Cell Reselection (NR RRC_IDLE to E-UTRA RRC_IDLE) |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.3.10.NR5GC |  |  | Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE / schedulingInfoList-v12j0 |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.2.3.11.NR5GC |  |  | Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE / schedulingInfoListExt-r12 |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.3.1.1.NR5GC |  |  | Steering of UE in roaming during registration/security check successful using List Type 1 |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.3.1.2.NR5GC |  |  | Steering of UE in roaming during registration/security check successful but SOR Transparent container indicates ACK has been NOT been requested |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.3.1.3.NR5GC |  |  | Steering of UE in roaming during registration/security check unsuccessful/Automatic mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.3.1.4.NR5GC |  |  | Steering of UE in roaming during registration/security check unsuccessful/Manual mode |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 6.3.1.5.NR5GC |  |  | Steering of UE in roaming during registration/UE configured to receive Steering of Roaming information but does not receive Steering of Roaming from Network |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.3.1.7.NR5GC |  |  | Steering of UE in roaming during registration/security check unsuccessful but emergency service pending to be activated |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 6.3.1.8.NR5GC |  |  | Steering of UE in roaming after registration/automatic plmn selection mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.3.1.9.NR5GC |  |  | Steering of UE in roaming after registration/manual plmn selection mode |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 6.3.1.10.NR5GC |  |  | Steering of UE in roaming during mobility update registration |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.4.1.1.NR5GC |  |  | PLMN Selection/Higher priority/HPLMN in Automatic PLMN Selection Mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.4.1.2.NR5GC |  |  | Cell reselection of ePLMN in manual mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.4.2.1.NR5GC |  |  | Cell Selection/Qrxlevmin & Cell Reselection (Intra NR in RRC_INACTIVE state) |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.4.2.2.NR5GC |  |  | Inter-frequency cell reselection according to cell reselection priority provided by SIBs in RRC_INACTIVE state |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.4.3.1.NR5GC |  |  | Inter-RAT cell reselection / From NR RRC_INACTIVE to E-UTRA RRC_IDLE (lower priority & higher priority , Srxlev based) |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.5.1.1.NR5GC |  |  | SNPN Selection in Manual Mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.5.1.2.NR5GC |  |  | SNPN Selection in Automatic Mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.5.2.1.NR5GC |  |  | CAG Selection in Manual Mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.5.2.2.NR5GC |  |  | CAG Selection in Automatic Mode |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.5.2.3.NR5GC |  |  | CAG / Limited Service / No Suitable cell |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.5.2.4.NR5GC |  |  | CAG / cell reselection / Within allowed CAG/ non-CAG cell to CAG cell |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 6.5.2.6.NR5GC |  |  | CAG/ Cell Reservation |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.1.1.NR5GC |  |  | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.1.1a.NR5GC |  |  | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.1.2.NR5GC |  |  | Random access procedure / Successful/ C-RNTI Based/Preamble selected by MAC itself |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.1.3.NR5GC |  |  | Random access procedure / Successful/SI request |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.1.6.NR5GC |  |  | Random access procedure / Successful/ Temporary C-RNTI Based/Preamble selected by MAC itself |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.1.16.NR5GC |  |  | Random access procedure / RedCap UE identification / Msg3-based / CCCH1 |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.2.1.NR5GC |  |  | Correct Handling of DL MAC PDU / Assignment / HARQ process |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.2.2.NR5GC |  |  | Correct Handling of DL HARQ process PDSCH Aggregation |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.2.3.NR5GC |  |  | Correct HARQ process handling / CCCH |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.2.4.NR5GC |  |  | Correct HARQ process handling / BCCH |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.3.1.NR5GC |  |  | Correct Handling of UL MAC PDU / Assignment / HARQ process |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.3.2.NR5GC |  |  | Logical channel prioritization handling |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.3.2b.NR5GC |  |  | Logical channel prioritization handling with Mapping restrictions |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.3.3.NR5GC |  |  | Correct handling of MAC control information / Scheduling requests |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.3.4.NR5GC |  |  | Correct handling of MAC control information / Buffer status / UL data arrive in the UE Tx buffer / Regular BSR |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.3.5.NR5GC |  |  | Correct handling of MAC control information / Buffer Status / UL resources are allocated / Padding BSR |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.3.6.NR5GC |  |  | Correct handling of MAC control information / Buffer status / Periodic BSR timer expires |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.3.7.NR5GC |  |  | UE power headroom reporting / Periodic reporting / DL pathloss change reporting |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.3.8.1.NR5GC |  |  | UE power headroom reporting / SCell activation / DL pathloss change reporting / Intra-band Contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.3.8.2.NR5GC |  |  | UE power headroom reporting / SCell activation / DL pathloss change reporting/ Inter-band CA |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1.1.3.9.NR5GC |  |  | Correct Handling of UL HARQ process/ PUSCH Aggregation |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.4.1.1.NR5GC |  |  | DL-SCH Transport Block Size selection / DCI format 1_0 |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.4.2.1.NR5GC |  |  | UL-SCH transport block size selection / DCI format 0_0 / Transform precoding disabled |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.4.2.3.NR5GC |  |  | UL-SCH transport block size selection / DCI format 1_1 / RA type 0 / RA Type 1 / Transform precoding disabled |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.4.2.4.NR5GC |  |  | UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.4.2.5.NR5GC |  |  | UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.5.1.NR5GC |  |  | DRX operation / Short cycle not configured / Parameters configured by RRC |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.5.2.NR5GC |  |  | DRX operation / Short cycle not configured /Long DRX command MAC control element reception |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.5.3.NR5GC |  |  | DRX operation / Short cycle configured / Parameters configured by RRC |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.5.4.NR5GC |  |  | DRX Operation / Short cycle configured / DRX command MAC control element reception |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.5.5.NR5GC |  |  | DRX Operation / Short cycle configured / Long DRX command MAC control element reception |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.6.1.NR5GC |  |  | Correct handling of DL assignment / Semi-persistent case |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.6.2.NR5GC |  |  | Correct handling of UL grant / configured grant Type 1 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1.1.6.3.NR5GC |  |  | Correct handling of UL grant / configured grant Type 2 |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.7.1.1.NR5GC |  |  | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band Contiguous CA |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.7.1.2.NR5GC |  |  | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Inter-band CA |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1.1.7.1.3.NR5GC |  |  | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band non-Contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.1.8.1.NR5GC |  |  | Bandwidth Part (BWP) operation UL/DL |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1.1.9.1.NR5GC |  |  | MAC Reset |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.10.1.NR5GC |  |  | DataInactivityTimer expiry |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.1.12.3.NR5GC |  |  | DRX adaptation / UE wakeup indication |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 7.1.2.2.1.NR5GC |  |  | UM RLC / Segmentation and reassembly / 6-bit SN / Segmentation Info (SI) field |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.2.2.NR5GC |  |  | UM RLC / Segmentation and reassembly / 12-bit SN / Segmentation Info (SI) field |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.2.3.NR5GC |  |  | UM RLC / 6-bit SN / Correct use of sequence numbering |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.2.4.NR5GC |  |  | UM RLC / 12-bit SN / Correct use of sequence numbering |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.2.5.NR5GC |  |  | UM RLC / Receive Window operation and t-Reassembly expiry |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.2.6.NR5GC |  |  | UM RLC / RLC re-establishment procedure |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.3.1.NR5GC |  |  | AM RLC / 12-bit SN/Segmentation and reassembly / Segmentation Info (SI) field |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.3.2.NR5GC |  |  | AM RLC / 18-bit SN/Segmentation and reassembly / Segmentation Info (SI) field |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.3.3.NR5GC |  |  | AM RLC / 12-bit SN / Correct use of sequence numbering |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.3.4.NR5GC |  |  | AM RLC / 18-bit SN / Correct use of sequence numbering |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.3.5.NR5GC |  |  | AM RLC / Control of transmit window / Control of receive window |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.3.6.NR5GC |  |  | AM RLC / Polling for status |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.3.7.NR5GC |  |  | AM RLC / Receiver status triggers |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.3.8.NR5GC |  |  | AM RLC / Reconfiguration of RLC parameters by upper layers |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.3.9.NR5GC |  |  | AM RLC / Reassembling of AMD PDUs |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.3.10.NR5GC |  |  | AM RLC / Re-transmission of RLC PDU with and without re-segmentation |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.2.3.11.NR5GC |  |  | AM RLC / RLC re-establishment procedure |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.1.1.NR5GC |  |  | Maintenance of PDCP sequence numbers / User plane / 12-bit SN |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.1.2.NR5GC |  |  | Maintenance of PDCP sequence numbers / User plane / 18-bit SN |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.2.1.NR5GC |  |  | Integrity protection / Correct functionality of encryption algorithm SNOW3G / SRB / DRB |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.2.2.NR5GC |  |  | Integrity protection / Correct functionality of encryption algorithm AES / SRB / DRB |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.2.3.NR5GC |  |  | Integrity protection / Correct functionality of encryption algorithm ZUC / SRB / DRB |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.3.1.NR5GC |  |  | Ciphering and deciphering / Correct functionality of encryption algorithm SNOW3G / SRB / DRB |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.3.2.NR5GC |  |  | Ciphering and deciphering / Correct functionality of encryption algorithm AES / SRB / DRB |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.3.3.NR5GC |  |  | Ciphering and deciphering / Correct functionality of encryption algorithm ZUC / SRB / DRB |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.4.1.NR5GC |  |  | PDCP handover / Lossless handover / PDCP sequence number maintenance/PDCP status report to convey the information on missing or acknowledged PDCP SDUs at handover/ In-order delivery and duplicate elimination in the downlink |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.4.2.NR5GC |  |  | PDCP handover / Non-lossless handover / PDCP sequence number maintenance |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.4.3.NR5GC |  |  | PDCP handover / DAPS handover / Status reporting / Intra-frequency |  |  | X |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
| 7.1.3.4.4.NR5GC |  |  | PDCP handover / DAPS handover with key change / Status reporting / Inter-frequency |  |  | X |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
| 7.1.3.5.1.NR5GC |  |  | PDCP Discard |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.3.5.2.NR5GC |  |  | PDCP Uplink Routing / Split DRB |  |  |  | - |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 7.1.3.5.3.NR5GC |  |  | PDCP Data Recovery |  |  |  |  |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 7.1.3.5.4.NR5GC |  |  | PDCP reordering / Maximum re-ordering delay below t-Reordering / t-Reordering timer operations |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.4.1.NR5GC |  |  | SDAP Data Transfer and PDU Header Handling UL/DL |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 7.1.4.2.NR5GC |  |  | SDAP Data Transfer handling without Header UL/DL |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.1.1.1.NR5GC |  |  | RRC / Paging for connection / Multiple paging records |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.1.1.2.NR5GC |  |  | RRC / Paging for connection / Shared network environment |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.1.2.1.NR5GC |  |  | RRC connection establishment / Return to idle state after T300 expiry |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.1.2.3.NR5GC |  |  | RRC connection establishment / RRC Reject with wait time |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.1.2.4.NR5GC |  |  | RRC connection establishment / Extended and spare fields in SI |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.1.3.1.NR5GC |  |  | RRC connection release / Redirection to another NR frequency |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.1.3.2.NR5GC |  |  | RRC connection release / Redirection from NR to E-UTRAN |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.1.3.3.NR5GC |  |  | RRC connection release / Success / With priority information |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.1.3.4.NR5GC |  |  | RRC connection release / Success / With priority information / E-UTRA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.1.3.7.NR5GC |  |  | RRC connection release / Success / Deprioritisation / Frequency / T325 expiry |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.1.3.7a.NR5GC |  |  | RRC connection release / Success / Deprioritisation / NR / T325 expiry |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.1.3.7b.NR5GC |  |  | RRC connection release / Success / Deprioritisation / Deletion of Stored deprioritisation request |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.1.4.1.NR5GC |  |  | RRC resume / Suspend-Resume / RNA update / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.1.4.2.NR5GC |  |  | RRC resume / Suspend-Resume / RRC setup / T319 expiry |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.2.1.1.NR5GC |  |  | RRC reconfiguration / DRB / SRB / Establishment / Modification / Release / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.2.1.2.NR5GC |  |  | RRC reconfiguration / RRC bearer establishment / uplinkTxDirectCurrentList |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.2.1.4.NR5GC |  |  | RRC reconfiguration / Dedicated RLF timer |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.2.1.5.1.NR5GC |  |  | NR CA / RRC reconfiguration / SCell addition / modification / release / Success / Intra-band Contiguous CA |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.2.1.5.2.NR5GC |  |  | NR CA / RRC reconfiguration / SCell addition / modification / release / Success / Inter-band CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.2.1.5.3.NR5GC |  |  | NR CA / RRC reconfiguration / SCell addition / modification / release / Success / Intra-band non-contiguous CA |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.1.NR5GC |  |  | Measurement configuration control and reporting / Intra NR measurements / Event A1 / Event A2 |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.2.NR5GC |  |  | Measurement configuration control and reporting / Event A3 / Measurement of Neighbor NR cell / Intra-frequency measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.3.NR5GC |  |  | Measurement configuration control and reporting / Event A3 / Measurement of Neighbor NR cell / Inter-frequency measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.4.NR5GC |  |  | Measurement configuration control and reporting / Event A3 / Measurement of Neighbor NR cell / Inter-band measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.5.NR5GC |  |  | Measurement configuration control and reporting / Event A4 / Measurement of Neighbor NR cell / Intra-frequency measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.6.NR5GC |  |  | Measurement configuration control and reporting / Event A4 / Measurement of Neighbor NR cell / Inter-frequency measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.7.NR5GC |  |  | Measurement configuration control and reporting / Event A4 / Measurement of Neighbor NR cell / Inter-band measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.8.NR5GC |  |  | Measurement configuration control and reporting / Event A5 / Measurement of Neighbor NR cell / Intra-frequency measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.9.NR5GC |  |  | Measurement configuration control and reporting / Event A5 / Measurement of Neighbor NR cell / Inter-frequency measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.10.NR5GC |  |  | Measurement configuration control and reporting / Event A5 / Measurement of Neighbor NR cell / Inter-band measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.11.NR5GC |  |  | Measurement configuration control and reporting / Intra NR measurements / Two simultaneous events A3 (intra and inter-frequency measurements) / RSRQ based measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.12.NR5GC |  |  | Measurement configuration control and reporting / Intra NR measurements / Two simultaneous events A5 (intra and inter-frequency measurements) / SINR based measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.15A.NR5GC |  |  | Measurement configuration control and reporting / Intra NR measurements / Exclude-listed cells |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.16.NR5GC |  |  | Measurement configuration control and reporting / Intra NR measurements / Allow-listed cells |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.17.1.NR5GC |  |  | NR CA / Measurement configuration control and reporting / Intra NR measurements / Event A6 / Intra-band Contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.17.2.NR5GC |  |  | NR CA / Measurement configuration control and reporting / Intra NR measurements / Event A6 / Inter-band CA |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.3.1.17.3.NR5GC |  |  | NR CA / Measurement configuration control and reporting / Intra NR measurements / Event A6 / Intra-band non-Contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.18.1.NR5GC |  |  | NR CA / Measurement configuration control and reporting / Intra NR measurements / Additional measurement reporting / Intra-band Contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.18.2.NR5GC |  |  | NR CA / Measurement configuration control and reporting / Intra NR measurements / Additional measurement reporting / Inter-band CA |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.3.1.18.3.NR5GC |  |  | NR CA / Measurement configuration control and reporting / Intra NR measurements / Additional measurement reporting / Intra-band non-Contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.20.NR5GC |  |  | Measurement configuration control and reporting / Measurement Gaps / gapFR1 |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.1.23.NR5GC |  |  | Measurement configuration control and reporting / Intra NR measurements / Continuation of the measurements after RRC Resume |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.2.1.NR5GC |  |  | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of E-UTRA cells |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.2.2.NR5GC |  |  | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of E-UTRA cells |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.2.3.NR5GC |  |  | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of E-UTRA cells / RSRQ based measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.2.4.NR5GC |  |  | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of E-UTRA cells / SINR based measurements |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.3.1.NR5GC |  |  | Measurement configuration control and reporting / CGI reporting of NR cell |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.3.3.2.NR5GC |  |  | Measurement configuration control and reporting / CGI reporting of E-UTRA cell |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.1.2.NR5GC |  |  | Intra NR handover / Success / Inter-frequency |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.1.5.NR5GC |  |  | Intra NR handover / Failure / Re-establishment successful |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.1.6.NR5GC |  |  | Intra NR handover / Failure / Re-establishment failure |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.1.7.1.NR5GC |  |  | NR CA / Intra NR handover / Success / PCell Change and SCell addition / SCell release / Intra-band Contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.1.7.2.NR5GC |  |  | NR CA / Intra NR handover / Success / PCell Change and SCell addition / SCell release / Inter-band CA |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.4.1.7.3.NR5GC |  |  | NR CA / Intra NR handover / Success / PCell Change and SCell addition / SCell release/ Intra-band non-contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.1.8.1.NR5GC |  |  | NR CA / Intra NR handover / Success / PCell Change / SCell no Change / Intra-band Contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.1.8.2.NR5GC |  |  | NR CA / Intra NR handover / Success / PCell Change / SCell no Change / Inter-band CA |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.4.1.8.3.NR5GC |  |  | NR CA / Intra NR handover / Success / PCell Change / SCell no Change / Intra-band non-contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.1.9.1.NR5GC |  |  | NR CA / Intra NR handover / Failure / Re-establishment successful / Intra-band Contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.1.9.2.NR5GC |  |  | NR CA / Intra NR handover / Failure / Re-establishment successful / Inter-band CA |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.4.1.9.3.NR5GC |  |  | NR CA / Intra NR handover / Failure / Re-establishment successful / Intra-band non-contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.2.1.1.NR5GC |  |  | Inter-RAT handover / From NR to E-UTRA / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.4.2.1.2.NR5GC |  |  | Inter-RAT handover / From NR to EN-DC / Success |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.2.2.1.NR5GC |  |  | Inter-RAT handover / From E-UTRA to NR / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.4.3.1.NR5GC |  |  | DAPS handover with key change / Success / Intra-frequency |  |  |  |  | X |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
| 8.1.4.3.2.NR5GC |  |  | DAPS handover / HO Failure and source link available / HO Success and RLF in source / Intra-frequency |  |  |  |  | X |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
| 8.1.4.3.4.NR5GC |  |  | DAPS handover with key change / Success / Inter-frequency |  |  |  | X |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
| 8.1.4.3.5.NR5GC |  |  | DAPS handover / HO Failure and source link available / HO Success and RLF in source / Inter-frequency |  |  |  | X |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
| 8.1.4.4.1.NR5GC |  |  | Conditional handover / Success / A3 / A5 / A3+A5 |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.4.2.NR5GC |  |  | Conditional handover / modify conditional handover configuration |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.4.3.NR5GC |  |  | Conditional handover / Failure |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.4.4.4.NR5GC |  |  | Conditional handover / legacy Handover / legacy Handover Failure |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.5.1.1.NR5GC |  |  | UE capability transfer / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.5.2.1.NR5GC |  |  | SI change / Notification of BCCH modification / Short message for SI update |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.5.2.2.NR5GC |  |  | SI change / Notification of BCCH modification / Short message for SI update in NR RRC_CONNECTED state |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.5.3.1.NR5GC |  |  | PWS notification / PWS reception in NR RRC_IDLE state |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.5.3.2.NR5GC |  |  | PWS notification / PWS reception in NR RRC_INACTIVE state |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.5.3.3.NR5GC |  |  | PWS notification / PWS reception in NR RRC_CONNECTED state |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.5.3.4.NR5GC |  |  | PWS notification / PWS reception using dedicatedSystemInformationDelivery |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.5.4.1.NR5GC |  |  | Counter check / Reception of CounterCheck message by the UE |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.5.5.1.NR5GC |  |  | Redirection to NR / From E-UTRA / Success |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.5.6.1.NR5GC |  |  | Radio link failure / RRC connection re-establishment success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.5.6.3.NR5GC |  |  | Radio link failure / T311 expiry |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.5.6.5.1.NR5GC |  |  | NR CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Intra-band Contiguous CA |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.5.6.5.2.NR5GC |  |  | NR CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Inter-band CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.5.6.5.3.NR5GC |  |  | NR CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Intra-band non-Contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.5.7.1.1.NR5GC |  |  | Failure information / RLC failure / MCG / Intra-band Contiguous CA |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.5.7.1.2.NR5GC |  |  | Failure information / RLC failure / MCG / Inter-band CA |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.5.8.1.NR5GC |  |  | Processing delay / RRC_Idle to RRC_Connected / RRC_Inactive to RRC_Connected / Success / Latency check |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.5.8.2.1.NR5GC |  |  | Processing delay / RRC_Inactive to RRC_Connected / Success / Latency check / SCell addition / Intra-band Contiguous CA |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.5.8.2.2.NR5GC |  |  | Processing delay / RRC_Inactive to RRC_Connected / Success / Latency check / SCell addition / Inter-band CA |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.5.8.2.3.NR5GC |  |  | Processing delay / RRC_Inactive to RRC_Connected / Success / Latency check / SCell addition / Intra-band non-Contiguous CA |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.5.9.1.NR5GC |  |  | RACS / UL Message Segment transfer / UECapabilityInformation |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.5.9.2.NR5GC |  |  | RRC reconfiguration / DL segment transfer |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.5.10.1.NR5GC |  |  | UE Assistance Information / Release Preference |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.5.10.4.NR5GC |  |  | UE Assistance Information / RRM measurement relaxation / RedCap |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.1.2.NR5GC |  |  | Immediate MDT / Measurement / Latency metrics for UL PDCP Packet Delay per DRB |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.1.1.NR5GC |  |  | Immediate MDT / Measurement reporting / Location information |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.1.NR5GC |  |  | Logged MDT / RRC_IDLE / Logging and reporting / Intra-frequency measurement |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.2.NR5GC |  |  | Logged MDT / RRC_INACTIVE / Logging and reporting / Inter-frequency measurement |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.6.1.2.3.NR5GC |  |  | Logged MDT / RRC_IDLE / Logging and reporting / Limiting area scope |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.4.NR5GC |  |  | Logged MDT/ RRC_IDLE / Logging and reporting / periodic measurement trigger |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.5.NR5GC |  |  | Logged MDT/ RRC_IDLE / Logging and reporting / event-based trigger |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.6.NR5GC |  |  | Logged MDT/ RRC_IDLE / Logging and reporting / event-based trigger / out-of-coverage |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.7.NR5GC |  |  | Logged MDT / RRC_IDLE / Logging and reporting / Reporting at NR re-establishment |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.8.NR5GC |  |  | Logged MDT / Logging and reporting / Reporting at RRC reconfiguration |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.9.NR5GC |  |  | Logged MDT / Location information |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.10.NR5GC |  |  | Logged MDT / Maintaining logged measurement configuration / UE mobility |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.11.NR5GC |  |  | Logged MDT / Maintaining logged measurement configuration / UE state transitions |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.12.NR5GC |  |  | Logged MDT / Release of logged MDT measurement configuration / Expire of duration timer |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.2.13.NR5GC |  |  | Logged MDT / Release of logged MDT measurement configuration / Reception of new logged measurement configuration |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.3.1.NR5GC |  |  | Radio Link Failure / Reporting of Intra-frequency measurements |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.3.2.NR5GC |  |  | Radio Link Failure / Reporting of Inter-frequency measurements |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.3.3.NR5GC |  |  | Radio Link Failure / Reporting at RRC connection establishment and reestablishment |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.3.4.NR5GC |  |  | Radio Link Failure / Reporting at NR handover |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.3.5.NR5GC |  |  | Radio Link Failure / Location information |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.3.6.NR5GC |  |  | Radio Link Failure / RACH failure report |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.3.7.NR5GC |  |  | Radio Link Failure / Logging and reporting / Reporting at intra NR handover / PLMN list |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.4.1.NR5GC |  |  | Connection Establishment Failure / Logging and reporting / T300 expiry |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.4.2.NR5GC |  |  | Connection Establishment Failure / Logging and reporting / RRC Resume |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.4.3.NR5GC |  |  | Connection Establishment Failure / Logging and reporting / Reporting at intra-NR handover |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.4.4.NR5GC |  |  | Connection Establishment Failure / Logging and reporting / Reporting at RRC connection re-establishment |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.4.5.NR5GC |  |  | Connection Establishment Failure / Logging and reporting / Location Information |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.4.6.NR5GC |  |  | Connection Establishment Failure / Logging and reporting / Reporting of Intra-frequency measurements |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.1.4.7.NR5GC |  |  | Connection Establishment Failure / Logging and reporting / Reporting of Inter-frequency measurements |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1.6.1.4.8.NR5GC |  |  | Connection Establishment Failure / Logging and reporting / RACH failure report |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.6.2.1.NR5GC |  |  | Inter-RAT MDT / Immediate MDT / Periodic reporting of E-UTRAN/ Location information |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.2.2.NR5GC |  |  | Inter-RAT MDT / Logged MDT / E-UTRA Inter-RAT measurement, logging and reporting |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.2.3.NR5GC |  |  | Inter-RAT MDT / Radio Link Failure / Reporting at E-UTRA Inter-RAT handover |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.2.4.NR5GC |  |  | Inter-RAT MDT / Connection Establishment Failure / Logging and reporting / Reporting of E-UTRA measurement |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.1.6.4.1.NR5GC |  |  | SON / RACH logging and reporting |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 8.1.7.1.1.NR5GC |  |  | Measurement configuration control and reporting / CGI reporting of NR NPN cell |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 8.2.2.1.2.NR5GC |  |  | SRB3 Establishment, Reconfiguration and Release / NR addition, modification and release / NR-DC |  |  |  |  |  |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 8.2.2.4.2.NR5GC |  |  | PSCell addition, modification and release / SCG DRB / NR-DC |  |  |  |  | - |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 8.2.2.5.2.NR5GC |  |  | PSCell addition, modification and release / Split DRB / NR-DC |  |  |  |  | - |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 8.2.2.9.2.NR5GC |  |  | Bearer Modification / Uplink data path / Split DRB Reconfiguration / NR-DC |  |  |  |  | - |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 8.2.3.14.2.NR5GC |  |  | SCG change / Reconfiguration with sync / Split DRB / NR-DC |  |  |  |  | - |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 8.2.3.16.2.NR5GC |  |  | Measurement configuration control and reporting / SRB3 / Intra NR measurements / NR-DC |  |  |  |  |  |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 8.2.5.1.2.NR5GC |  |  | Radio link failure / Random access problem / NR-DC |  |  |  |  | - |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 8.2.5.2.2.NR5GC |  |  | Radio link failure / PSCell out of sync indication / NR-DC |  |  |  |  | - |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 8.2.5.3.2.NR5GC |  |  | Radio link failure / rlc-MaxNumRetx failure / NR-DC |  |  |  |  | - |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 8.2.5.4.2.NR5GC |  |  | Reconfiguration failure / SCG change failure / NR-DC |  |  |  |  | - |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 8.2.6.2.2.NR5GC |  |  | Processing delay / Latency check / NR-DC |  |  |  |  |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 8.2.7.2.1.NR5GC |  |  | RRC Resume / Suspend-Resume / RRC reconfiguration / NR-DC / Resume with SCG |  |  |  |  |  |  |  |  |  | - |  |  |  |  | X |  |  |  |  |
| 9.1.1.1.NR5GC |  |  | EAP based primary authentication and key agreement / EAP-AKA' related procedures |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.1.2.NR5GC |  |  | EAP based primary authentication and key agreement / Reject |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.1.3.NR5GC |  |  | EAP based primary authentication and key agreement / EAP message transport / Abnormal |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.1.4.NR5GC |  |  | 5G AKA based primary authentication and key agreement / 5G-AKA related procedures |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.1.5.NR5GC |  |  | 5G AKA based primary authentication and key agreement / Reject |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| 9.1.1.6.NR5GC |  |  | 5G AKA based primary authentication and key agreement / Abnormal |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.2.1.NR5GC |  |  | NAS security mode command |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.2.2.NR5GC |  |  | Protection of initial NAS signalling messages |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.2.3.NR5GC |  |  | Integrity protection / Correct functionality of 5G NAS integrity algorithm / SNOW3G |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.2.4.NR5GC |  |  | Integrity protection / Correct functionality of 5G NAS integrity algorithm / AES |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.2.5.NR5GC |  |  | Integrity protection / Correct functionality of 5G NAS integrity algorithm / ZUC |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.2.6.NR5GC |  |  | Ciphering and deciphering / Correct functionality of 5G NAS encryption algorithm / SNOW3G |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.2.7.NR5GC |  |  | Ciphering and deciphering / Correct functionality of 5G NAS encryption algorithm / AES |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.2.8.NR5GC |  |  | Ciphering and deciphering / Correct functionality of 5G NAS encryption algorithm / ZUC |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.3.1.NR5GC |  |  | Identification procedure |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.4.1.NR5GC |  |  | Generic UE configuration update / New 5G-GUTI, NITZ, registration requested, Network slicing indication, New Allowed NSSAI / acknowledgement from the UE |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.1.NR5GC |  |  | Initial registration / Success / 5G-GUTI reallocation, Last visited TAI |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.2.NR5GC |  |  | Initial registration / 5GS services / Equivalent PLMN list handling |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.3.NR5GC |  |  | Initial registration / 5GS services / NSSAI handling |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.3a.NR5GC |  |  | Initial registration / 5GS services / NSSAI handling / NSSAI Storage |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.5.NR5GC |  |  | Initial registration / Abnormal / Failure after 5 attempts |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.6.NR5GC |  |  | Initial registration / Rejected / Illegal UE |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.8.NR5GC |  |  | Initial registration / Rejected / Serving network not authorized |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.9.NR5GC |  |  | Initial registration / Abnormal / Change of cell into a new tracking area |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.10.NR5GC |  |  | Initial registration / Rejected / PLMN not allowed |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.11.NR5GC |  |  | Initial registration / Rejected / Tracking area not allowed |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.12.NR5GC |  |  | Initial registration / Rejected / Roaming not allowed in this tracking area |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.13.NR5GC |  |  | Initial registration / Rejected / No suitable cells in tracking area |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.14.NR5GC |  |  | Initial registration / Rejected / Congestion / Abnormal cases / T3346 |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.1.15.NR5GC |  |  | Initial registration / Success / Extended and spare fields in UE network capability |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.2.1.NR5GC |  |  | Mobility registration update / TAI list handling |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.2.2.NR5GC |  |  | Periodic registration update / Accepted |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.2.4.NR5GC |  |  | Mobility registration update / The lower layer requests NAS signalling connection recovery |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.2.6.NR5GC |  |  | Mobility registration update / Registered slice(s) change |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.5.2.7.NR5GC |  |  | Mobility and periodic registration update / Rejected / UE identity cannot be derived by the network |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.2.8.NR5GC |  |  | Mobility and periodic registration update / Rejected / implicitly de-registered |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.5.2.9.NR5GC |  |  | Mobility and periodic registration update / Abnormal / Change of cell into a new tracking area, collision with generic UE configuration update procedure |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.6.1.1.NR5GC |  |  | UE-initiated de-registration / switch off / Abnormal / De-registration and 5GMM common procedure collision |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.6.1.2.NR5GC |  |  | UE-initiated de-registration / normal de-registration / Abnormal / Transmission failure without TAI change from lower layers, De-registration and 5GMM common procedure collision, T3521 timeout |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.6.1.3.NR5GC |  |  | UE-initiated de-registration / Abnormal / Change of cell into a new tracking area |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.6.1.4.NR5GC |  |  | UE-initiated de-registration / Abnormal / Transmission failure with TAI change from lower layers |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.6.2.1.NR5GC |  |  | Network-initiated de-registration / de-registration for 3GPP access / re-registration required |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.6.2.2.NR5GC |  |  | Network-initiated de-registration / de-registration for 3GPP access / re-registration not required |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.7.1.NR5GC |  |  | Service request / IDLE mode uplink user data transport / Rejected / Restricted service area, Abnormal / T3517 / T3525 |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.7.2.NR5GC |  |  | Service request / CONNECTED mode user data transport / Abnormal / T3517 |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.8.1.NR5GC |  |  | SMS over NAS / MO and MT SMS over NAS - Idle mode |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.8.2.NR5GC |  |  | SMS over NAS / Multiple MO and MT SMS over NAS - CONNECTED mode |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.9.1.NR5GC |  |  | RACS / Network assigned UE radio capability ID |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.9.2.NR5GC |  |  | RACS / UE configuration update / UE radio capability ID |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.9.3.NR5GC |  |  | RACS / PLMN change within registration area / From PLMN assigned to Manufacturer assigned UE Radio Capability ID |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.9.4.NR5GC |  |  | RACS / USIM change / Handling of URCID |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.9.5.NR5GC |  |  | RACS / Handling of delete indication for NW assigned UE radio capability ID |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.9.7.NR5GC |  |  | RACS / Inter-system mobility registration update / Handling of UE radio capability ID |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.9.6.NR5GC |  |  | RACS / Change in radio capability / NW assigned URCID |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.10.1.NR5GC |  |  | NSSAA / EAP message transport / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.10.2.NR5GC |  |  | "Network slice-specific authentication and authorization / EAP message transport / Abnormal |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.10.3.NR5GC |  |  | NSSAA / Initial registration / Rejected NSSAI, pending NSSAI |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.10.4.NR5GC |  |  | NSSAA / Initial registration / Reject |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.10.6.NR5GC |  |  | NSSAA / UE configuration update / Rejected NSSAI |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.1.11.1.NR5GC |  |  | SNPN / Initial registration / Rejected / Temporarily not authorized for this SNPN |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.11.2.NR5GC |  |  | SNPN / Initial registration / Rejected / Permanently not authorized for this SNPN |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.1.11.3.NR5GC |  |  | SNPN / EAP based primary authentication and key agreement / EAP-AKA' related procedures |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.3.1.1.NR5GC |  |  | Inter-system mobility registration update / Single-registration mode with N26 / 5GMM-IDLE / 5GC to EPC |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 9.3.1.2.NR5GC |  |  | Inter-system mobility registration update / Single-registration mode with N26 / 5GMM-IDLE / EPC to 5GC |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 9.3.1.3.NR5GC |  |  | Inter-system mobility and periodic registration update / Rejected / Single-registration mode with N26 / Handling of EPC relevant parameters |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 10.1.1.1.NR5GC |  |  | PDU session authentication and authorization / During the UE-requested PDU session procedure |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 10.1.1.2.NR5GC |  |  | PDU session authentication and authorization / After the UE-requested PDU session procedure |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 10.1.2.1.NR5GC |  |  | Network-requested PDU session modification / Accepted |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 10.1.2.2.NR5GC |  |  | Network-requested PDU session modification / Abnormal / PDU session in state PDU SESSION INACTIVE |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 10.1.3.2.NR5GC |  |  | Network-requested PDU session release / Accepted / Insufficient resources / T3396, Accepted / Insufficient resources for specific slice and DNN / T3584, Abnormal / No PDU session context active for the received PDU session ID |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 10.1.4.1.NR5GC |  |  | UE-requested PDU session establishment / Abnormal / T3580 |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 10.1.5.1.NR5GC |  |  | UE-requested PDU session modification |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 10.1.6.1.NR5GC |  |  | UE-requested PDU session release / Abnormal / collision with network-requested PDU session modification procedure |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 10.1.6.2.NR5GC |  |  | UE-requested PDU session release / Abnormal / collision with network-requested PDU session release procedure |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 10.1.8.1.NR5GC |  |  | NASC / PDU session establishment reject / Maximum number of PDU sessions reached / Back-off timer is neither zero nor deactivated |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 10.1.8.2.NR5GC |  |  | NASC / PDU session establishment reject / Maximum number of PDU sessions reached / Back-off timer is deactivated |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 10.1.8.5.NR5GC |  |  | NASC / Maximum number of PDU sessions reached / Emergency service |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.1.1.NR5GC |  |  | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.1.1a.NR5GC |  |  | MO MMTEL enhanced voice service call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.1.2.NR5GC |  |  | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode without N26 interface / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.1.3.NR5GC |  |  | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.1.3a.NR5GC |  |  | MO MMTEL enhanced voice service call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.1.4.NR5GC |  |  | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode with N26 interface / E-UTRAN cell selection using cell status barred / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.1.5.NR5GC |  |  | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell selection using cell status reservation / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.1.6.NR5GC |  |  | MT MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode without N26 interface / Success |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.1.7.NR5GC |  |  | Emergency call setup from NR RRC_IDLE / Emergency Services Fallback to EPS with redirection / Single registration mode with N26 interface / Success |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.1.8.NR5GC |  |  | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / voiceFallbackIndication |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.1.9.NR5GC |  |  | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / voiceFallbackIndication |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.3.1.NR5GC |  |  | UAC / Access Identity 0 / 0% access probability / MTSI MO speech call/SMSoIP |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.3.1a.NR5GC |  |  | UAC / Access Identity 0 / 0% access probability / Uplink User data transfer / RRC_INACTIVE |  |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| 11.3.2.NR5GC |  |  | UAC / Access Identity 0 / 0% access probability / Paging for MT Access/Emergency Call |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.3.3.NR5GC |  |  | UAC / Access Identity 0 / AC8 / RRC_INACTIVE / RNAUpdate/RRC Resume |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.3.4.NR5GC |  |  | UAC / Access Identity 0 / Registration procedure for mobility and periodic registration update / BarringPerPLMN/Implicit AC Barring List |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.3.5.NR5GC |  |  | UAC / Access Identity 1 / New cell not in the country of its HPLMN/EHPLMN 0% access probability/MPS indicator / HPLMN/0%/100% accessibility AC5/MMTEL-Video call |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.3.6.NR5GC |  |  | UAC / Access Identity 2 / New cell not in the country of its HPLMN/EHPLMN 0% access probability/MCS indicator / HPLMN/0%/100% accessibility AC7/RRC_INACTIVE |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.3.7.NR5GC |  |  | UAC / Access Identity 11..15 / High Priority Access / HPLMN/0% accessibility AC2/Emergency call |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.3.8.NR5GC |  |  | UAC / Access Identity 0 / NR RRC_IDLE / Cell re-selection while T390 is running |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.3.9.NR5GC |  |  | UAC / Access Identity 0 / ODAC / PLMN / RPLMN / not EPLMN |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.3.9a.NR5GC |  |  | UAC / Access Identity 0 / ODAC / SNPN / RSNPN / new SNPN |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.3.10.NR5GC |  |  | UAC / Access Identity 0 / AC9 / 0% access probability / SIP Re-registration |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.4.1.NR5GC |  |  | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / Network failing the authentication check (5G AKA) |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.4.2.NR5GC |  |  | 5GMM-DEREGISTERED.LIMITED-SERVICE / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services / Handling of forbidden PLMNs |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.4.3.NR5GC |  |  | 5GMM-DEREGISTERED.NO-SUPI / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.4.4.NR5GC |  |  | 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE T3346 running / Emergency call establishment / 5GMM-REGISTERED.NORMAL-SERVICE / Emergency call establishment before T3396 expiry |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.4.5.NR5GC |  |  | 5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.4.6.NR5GC |  |  | 5GMM-REGISTERED.NON-ALLOWED-SERVICE / Emergency call establishment and release / Handling of non-allowed tracking areas |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.4.7.NR5GC |  |  | Handling of Local and Extended emergency numbers / Mobility |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.4.8.NR5GC |  |  | Handling of Local and extended emergency numbers / Switch-off and maximum local numbers storage |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.4.9.NR5GC |  |  | 5GMM-DEREGISTERED.LIMITED-SERVICE No suitable cells in tracking area / Emergency call establishment and release |  |  |  |  | X |  |  |  |  | X |  |  |  |  | - |  |  |  |  |
| 11.4.11.NR5GC |  |  | 5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / S1 mode to N1 mode transfer of an existing emergency PDN connection |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.4.12.NR5GC |  |  | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Disabling N1 mode / Emergency call establishment over EPS / Success |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.4.14.NR5GC |  |  | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call /Deregistration upon emergency registration expiration |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.6.1.NR5GC |  |  | Data Off / MO Voice Call |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |
| 11.6.2.NR5GC |  |  | Data Off / MO Video Call |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.6.3.NR5GC |  |  | Data Off / SMSoIP |  |  |  |  | X |  |  |  |  |  |  |  |  |  | - |  |  |  |  |

Annex D (normative): TTCN-3 definitions

# D.0	Introduction

The present Annex D specifies the TTCN-3 type definitions used at the system interface to configure and control the SS.

In case of discrepancy between the content of the present Annex D and the equivalent TTCN-3 definitions / semantic requirements found in the TTCN modules provided as attachments to the present specification, the latter shall take precedence.

NOTE:	This annex is automatically generated from the TTCN-3 modules provided as attachment to the present specification and containing the listed TTCN-3 type definitions.

# D.1	NR_ASP_TypeDefs

Type definitions for configuration of the system simulator;
Common design principles:
Semantics of OMIT: unless specified otherwise, for all TTCN-3 type definitions used in ASPs omit means "keep as it is" =>
- on initial configuration in general all fields shall be provided
- no default values for fields are foreseen
- if necessary non-existence of information shall be explicitly configured
  (e.g. with a union of "no configuration" and "configuration parameters"
- fields within structures imported from the core spec are excepted from this rule
- if a sub-structure is explicitly excluded from this rule all fields and sub-fields shall be fully specified for each (re-)configuration

## D.1.1	ASN1_Container

Definitions containing ASN.1 types for backward compatibility

NR_ASN1_ARFCN_ValueNR_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_ARFCN_ValueNR_Type |  |
| Comment |  |  |
| R15 | ARFCN_ValueNR |  |

NR_ASN1_UL_AM_RLC_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_UL_AM_RLC_Type |  |
| Comment |  |  |
| R15 | UL_AM_RLC |  |

NR_ASN1_DL_AM_RLC_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_DL_AM_RLC_Type |  |
| Comment |  |  |
| R15 | DL_AM_RLC |  |

NR_ASN1_UL_UM_RLC_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_UL_UM_RLC_Type |  |
| Comment |  |  |
| R15 | UL_UM_RLC |  |

NR_ASN1_DL_UM_RLC_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_DL_UM_RLC_Type |  |
| Comment |  |  |
| R15 | DL_UM_RLC |  |

NR_ASN1_PDSCH_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_PDSCH_Config_Type |  |
| Comment |  |  |
| R15 | PDSCH_Config |  |

NR_ASN1_PDSCH_ConfigCommon_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_PDSCH_ConfigCommon_Type |  |
| Comment |  |  |
| R15 | PDSCH_ConfigCommon |  |

NR_ASN1_SPS_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_SPS_Config_Type |  |
| Comment |  |  |
| R15 | SPS_Config |  |
| R16 | SPS_Config_R16 |  |

NR_ASN1_TDD_UL_DL_ConfigCommon_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_TDD_UL_DL_ConfigCommon_Type |  |
| Comment |  |  |
| R15 | TDD_UL_DL_ConfigCommon |  |

NR_ASN1_TDD_UL_DL_SlotConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_TDD_UL_DL_SlotConfig_Type |  |
| Comment |  |  |
| R15 | TDD_UL_DL_SlotConfig |  |

NR_ASN1_FrequencyInfoDL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_FrequencyInfoDL_Type |  |
| Comment |  |  |
| R15 | FrequencyInfoDL |  |

NR_ASN1_FrequencyInfoUL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_FrequencyInfoUL_Type |  |
| Comment |  |  |
| R15 | FrequencyInfoUL |  |

NR_ASN1_BWP_UplinkCommon_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_BWP_UplinkCommon_Type |  |
| Comment |  |  |
| R15 | BWP_UplinkCommon |  |

NR_ASN1_BWP_UplinkDedicated_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_BWP_UplinkDedicated_Type |  |
| Comment |  |  |
| R15 | BWP_UplinkDedicated |  |

NR_ASN1_RACH_ConfigDedicated_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_RACH_ConfigDedicated_Type |  |
| Comment |  |  |
| R15 | RACH_ConfigDedicated |  |

NR_ASN1_SI_RequestConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_SI_RequestConfig_Type |  |
| Comment |  |  |
| R15 | SI_RequestConfig |  |

NR_ASN1_PDSCH_ServingCellConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_PDSCH_ServingCellConfig_Type |  |
| Comment |  |  |
| R15 | PDSCH_ServingCellConfig |  |

NR_ASN1_PUSCH_ServingCellConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_PUSCH_ServingCellConfig_Type |  |
| Comment |  |  |
| R15 | PUSCH_ServingCellConfig |  |

NR_ASN1_SearchSpace_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_ASN1_SearchSpace_Type |  |  |
| Comment |  |  |  |
| R15 | SearchSpace |  |  |
| R16 | SearchSpaceExt_r16 | opt |  |
| R17 | SearchSpaceExt_v1700 | opt |  |

NR_ASN1_ControlResourceSet_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_ControlResourceSet_Type |  |
| Comment |  |  |
| R15 | ControlResourceSet |  |

NR_ASN1_BWP_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_BWP_Type |  |
| Comment |  |  |
| R15 | BWP |  |

NR_ASN1_DRX_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_DRX_Config_Type |  |
| Comment |  |  |
| R15 | DRX_Config |  |

NR_ASN1_MeasGapConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_MeasGapConfig_Type |  |
| Comment |  |  |
| R15 | MeasGapConfig |  |

NR_ASN1_MAC_CellGroupConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_MAC_CellGroupConfig_Type |  |
| Comment |  |  |
| R15 | MAC_CellGroupConfig |  |

NR_ASN1_PhysicalCellGroupConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_PhysicalCellGroupConfig_Type |  |
| Comment |  |  |
| R15 | PhysicalCellGroupConfig |  |

NR_ASN1_RateMatchPattern_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_RateMatchPattern_Type |  |
| Comment |  |  |
| R15 | RateMatchPattern |  |

NR_ASN1_RateMatchPatternLTE_CRS_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_RateMatchPatternLTE_CRS_Type |  |
| Comment |  |  |
| R15 | RateMatchPatternLTE_CRS |  |

## D.1.2	System_Configuration

Formal ASP Definitions for system configuration

NR_SystemRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SystemRequest_Type |  |
| Comment |  |  |
| Cell | NR_CellConfigRequest_Type | configure/release a cell |
| CellAttenuationList | NR_CellAttenuationList_Type | power attenuation for one or several cells; all cells included in the list shall be changed at the same time; all cells in the list shall reach the new cell power within a maximum of 100ms (10 frames) as according to TS 38.523-3 clause 7.1.4.2 CellId: In the common ASP part the CellId shall be set - to the cell the timing information refers to if activation time shall be applied - to nr_Cell_NonSpecific when there is no activation time TimingInfo: 'Now' (in general, but activation time may be used also) |
| RadioBearerList | NR_RadioBearerList_Type | configure/release one or several SRBs and/or DRBs at an SpCell NOTE: RBs are not configured in an SCell |
| EnquireTiming | Null_Type | get current timing information for the given cell TimingInfo : 'Now' |
| AS_Security | NR_AS_Security_Type | StartRestart/Release of AS security |
| SystemIndCtrl | NR_System_IndicationControl_Type | to configure SS to generate system indications |
| PdcpCount | NR_PDCP_CountReq_Type | to set or enquire PDCP COUNT for one or more RBs |
| DciTrigger | NR_DCI_Trigger_Type | to trigger a specific DCI to be transmitted on PDCCH (e.g. PDCCH order) |
| Paging | NR_PagingTrigger_Type | to trigger SS to send paging at the given paging occasion (as calculated in TTCN) NOTE: The SS shall use the DCI configuration as provided by NR_PcchConfig_Type; the DCI may or may not carry a short message but it in any case it shall indicate presence |
| MacCommandTrigger | NR_MAC_ControlElementDL_Type | to trigger a specific MAC control element to be transmitted to the UE |
| L1_TestMode | NR_L1_TestMode_Type | to Set L1/MAC in special Test modes e.g. DL CRC etc. per default (at initial configuration) no test mode is activated |
| PdcpHandoverControl | NR_PDCP_HandoverControlReq_Type | to inform the target cell about the handover (or PSCell change) procedure. |
| DeltaValues | NR_Band_SsbForDelta_Type | to provide the primary and secondary frequency info for deriving the delta values |
| SpsCg | NR_SPS_CG_ActDeact_Type | to configure/activate or release/deactivate DL SPS assignment or UL configured grant type 2 scheduling |

NR_SystemConfirm_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SystemConfirm_Type |  |
| Comment | confirmations for system configuration; in general to be sent after the configuration has been done |  |
| Cell | Null_Type | (no further parameters from SS) |
| CellAttenuationList | Null_Type | (no further parameters from SS) NOTE 1: the confirmation shall be sent when all cells have changed power levels NOTE 2: for the CellId in the common ASP part the same rules are applied as for the SYSTEM REQ |
| RadioBearerList | Null_Type | (no further parameters from SS) |
| EnquireTiming | Null_Type | the cell's timing information is contained in the TimingInfo of the ASP's common part |
| AS_Security | Null_Type | (no further parameters from SS) |
| SystemIndCtrl | Null_Type | (no further parameters from SS) |
| PdcpCount | NR_PDCP_CountCnf_Type | as response to 'Get' a list is returned containing COUNT information for the requested RBs |
| DciTrigger | Null_Type | (no further parameters from SS) |
| MacCommandTrigger | Null_Type | (no further parameters from SS) |
| L1_TestMode | Null_Type | confirmation for L1 test mode |
| PdcpHandoverControl | Null_Type | confirmation for PDCP handover control |
| DeltaValues | UE_NR_DeltaValues_Type | Delta values to be used for primary and secondary band |
| SpsCg | Null_Type | confirmation for DL SPS or UL configured grant type 2 Activation/Deactivation |

NR_SystemIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SystemIndication_Type |  |
| Comment |  |  |
| Error | charstring | indicates an error situation in SS; is not explicitly handled in TTCN but causes an INCONC due to default behaviour; an additional error code can be signalled in the common part of the ASP; SS shall raise an error when in TS 38.523-3 or in any other ASP definitions |
| RlcDiscardInd | NR_RlcDiscardInd_Type | indicates discarded PDUs |
| MAC | NR_MAC_ControlElementUL_Type | indicates MAC control element being received from the UE |
| RachPreamble | NR_RachPreamble_Type | RACH preamble being sent by the UE |
| SchedReq | Null_Type | indication for scheduling request sent by the UE |
| UL_HARQ | HARQ_Type | to report the UL HARQ as received on PUCCH or PUSCH for corresponding DL transmission |
| HarqError | NR_HarqError_Type | indicates detection of HARQ error: 1. HARQ CRC error for UL data 2. HARQ NACK from the UE unless SS is configured to report HARQ ACK/NACK |
| UL_SRB0_LCID | NR_LogicalChannelId_Type | indicates the Logical Channel Id used in UL SRB0 MAC subPDU |

## D.1.3	Cell_Configuration

Specific Info for Cell Configuration Primitive

### D.1.3.1	Cell_Configuration_Common

NR_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_NR_CellAttenuation_Off | NR_Attenuation_Type | {Off:=true} |  |

Cell_Configuration_Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_InitialAttenuation_Type | NR_Attenuation_Type (tsc_NR_CellAttenuation_Off) | Attenuation restricted to 'Off' |

NR_CellConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_CellConfigRequest_Type |  |
| Comment |  |  |
| AddOrReconfigure | NR_CellConfigInfo_Type | for cell configuration: TimingInfo : 'Now' for initial configuration; specific TimingInfo may be used for reconfiguration ControlInfo : FollowOnFlag:=false (unless explicitly specified otherwise in TS 38.523-3 clause 7) |
| Release | Null_Type | to remove a cell completely - CellId : identifier of the cell to be released; nr_Cell_NonSpecific, in case all cells shall be released TimingInfo : 'Now' ControlInfo : FollowOnFlag:=false |

NR_CellConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellConfigInfo_Type |  |  |
| Comment | common information for initial cell configuration or reconfiguration; in case of reconfiguration omit means 'keep configuration as it is' |  |  |
| StaticResourceConfig | NR_SS_StaticCellResourceConfig_Type | opt | mandatory for the initial configuration; to be omitted afterwards |
| CellConfigCommon | NR_CellConfigCommon_Type | opt | common configuration parameters which are not specific to physical layer (or any other layer) |
| PhysicalLayer | NR_CellConfigPhysicalLayer_Type | opt | Physical layer configuration: physical channels, signals and BWPs for UL and DL; DCI |
| BcchConfig | NR_BcchConfig_Type | opt | configuration of BCCH/BCH; SS is triggered to configure RLC/MAC accordingly; BCCH data on the PDSCH is distinguished by the SI-RNTI PBCH: MIB; PDSCH: scheduling and resource allocation; SIBs |
| PcchConfig | NR_PcchConfig_Type | opt | configuration of PCCH/PCH; SS is triggered to configure RLC/MAC accordingly; PCCH data on the PDSCH is distinguished by the P-RNTI (needed even to modify SI => shall be configured for CELL_BROADCASTING) |
| RachProcedureConfig | NR_RachProcedureConfig_Type | opt | to configure the SS's behaviour for the RACH procedure; may be omitted at initial configuration e.g. in case that the cell shall not have an uplink; NOTE: there is no way to explicitly remove the RACH procedure configuration after it has been configured for a cell |
| DcchDtchConfig | NR_DcchDtchConfig_Type | opt | Parameters related to DCCH/DTCH in UL and DL |
| ServingCellConfig | NR_ServingCellConfig_Type | opt | To be configured at initial configuration of a cell: for non-CA scenarios it shall be either 'SpCell' or 'None' ('None' applies for pure neighbouring cells) |
| SidelinkScheduled | NR_SidelinkScheduled_Type | opt | To configure SS for sidelink resource allocation mode 1 |

NR_SS_StaticCellResourceConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SS_StaticCellResourceConfig_Type |  |  |
| Comment | capabilities of a cell according to the initial condition of a test case, to allow resource management at SS implementation (see TS 38.508-1 clause 6.3.3) |  |  |
| CA_Capability | NR_CellCA_Capability_Type |  | capability of a cell in context of carrier aggregation |

NR_CellCA_Capability_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_CellCA_Capability_Type |
| Comment | static information about the cell's capability for carrier aggregation, not being changed during a test case; may be used for resource management at the SS; according to TS 38.508-1 clause 6.3.3.2 |
| SpCell | Cell can be used as SpCell during a test case (primary cell of a master or secondary cell group; TS 37.340 clause 3.1); normal case i.e. applicable even when SpCell is the only cell of the cell group |
| Scell_Active | Active SCell: An NR cell that may become an SCell at any point of time during the test case and which, while being an SCell, may be activated |
| Scell_Inactive | Inactive SCell: An NR cell that may become an SCell at any point of time during the test case but is never activated while being an SCell |
| None | e.g. when a cell is not used for connected mode during a test case (pure neighbouring cell) |

NR_CellConfigCommon_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellConfigCommon_Type |  |  |
| Comment | common configuration parameters which are not specific to physical layer (or any other layer) |  |  |
| C_RNTI | RNTI_Value_Type | opt | (pre-)configured C-RNTI used by physical layer and by MAC layer; affects scrambling of PDSCH/PUSCH and CRC of PDCCH(s); shall be used implicitly in RACH procedure (i.e. as CE in RAR) |
| CellTimingInfo | CellTimingInfo_Type | opt |  |
| InitialCellPower | NR_InitialCellPower_Type | opt | reference cell power of each antenna in DL NOTE 1: the power of an antenna may be reduced by antenna specific configuration NOTE 2: in general the power may be adjusted on a per resource element basis => all physical channel/signal power settings shall be adjusted relatively to the reference cell power; if there are more than one TX antennas each one may have its own attenuation; independently from those relative power settings the cell power can easily be adjusted by just changing the reference power |

NR_Attenuation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_Attenuation_Type |  |
| Comment | attenuation of the reference power |  |
| Value | integer (0..149) | cell power reference power reduced by the given attenuation (value is in dB); corresponds to AbsoluteCellPower_Type |
| Off | Null_Type | =< -145dBm according to TS 38.508-1 Table 6.2.2.1-3 |

NR_InitialCellPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_InitialCellPower_Type |  |  |
| Comment |  |  |  |
| MaxReferencePower | NR_AbsoluteCellPower_Type |  | maximum value of cell reference power (in dBm/SCS as per TS 38.508-1, clause 6.2.2); a cell is initialised with this reference power; its value is the upper bound of the cell power during the test case |
| Attenuation | NR_InitialAttenuation_Type |  | initial attenuation |

### D.1.3.2	PhysicalLayer

NR_CellConfigPhysicalLayer_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellConfigPhysicalLayer_Type |  |  |
| Comment | Common configuration of physical channels, signals and BWPs |  |  |
| Common | NR_CellConfigPhysicalLayerCommon_Type | opt | Configuration common for UL and DL |
| Downlink | NR_CellConfigPhysicalLayerDownlink_Type | opt | DL configuration |
| Uplink | NR_CellConfigPhysicalLayerUplink_Type | opt | UL configuration; may be omitted at initial configuration e.g. in case that the cell shall not have an uplink; NOTE: there is no way to explicitly remove the uplink configuration after it has been configured for a cell |

#### D.1.3.2.1	PhysicalLayer_Common

NR_CellConfigPhysicalLayerCommon_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellConfigPhysicalLayerCommon_Type |  |  |
| Comment | Configuration common for UL and DL |  |  |
| PhysicalCellId | PhysCellId | opt | Physical-layer cell identity according to 38.211 clause 7.4.2.1; EN-DC: corresponds to ServingCellConfigCommon.physCellId |
| DuplexMode | NR_DuplexMode_Type | opt | FDD or TDD; FDD/TDD specific parameters |

NR_DuplexMode_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DuplexMode_Type |  |
| Comment | FDD/TDD and maybe other types of duplex mode; in general FDD/TDD mode is determined from the frequency band |  |
| FDD | NR_FDD_Info_Type |  |
| TDD | NR_TDD_Info_Type |  |

NR_FDD_Info_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_FDD_Info_Type |  |  |
| Comment | FDD (paired spectrum) |  |  |
| HalfDuplexFDD | boolean | opt | "true" for RedCap half-duplex FDD. When configured, the SS shall operate as per the HD-FDD SS requirements specified in 38.523-3 clause 7.3.2.2.2. May be omitted at initial configuration when it is not applied. |

NR_TDD_UL_DL_SlotConfigList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_TDD_UL_DL_SlotConfigList_Type |
| Comment | corresponds to ServingCellConfig.tdd-UL-DL-ConfigurationDedicated |
| record of NR_ASN1_TDD_UL_DL_SlotConfig_Type |  |

NR_TDD_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_TDD_Config_Type |  |  |
| Comment | Common and dedicated TDD configuration |  |  |
| Common | NR_ASN1_TDD_UL_DL_ConfigCommon_Type | opt | Common TDD configuration as used in TS 38.213 clause 11 corresponding to ServingCellConfigCommon.tdd-UL-DL-ConfigurationCommon; shall be present for TDD at initial configuration |
| Dedicated | NR_TDD_UL_DL_SlotConfigList_Type | opt | Dedicated TDD configuration for single slots over-ruling the flexible slots of the common configuration; corresponds to ServingCellConfig.tdd-UL-DL-ConfigurationDedicated; shall be present for TDD at initial configuration: the list is empty when there is no dedicated slot configuration; (omit means "keep as it is"); NOTE: The dedicated configuration can only exist together with common configuration as a single slot configuration is related to the periodicity given by the common configuration (see TDD-UL-DL-SlotConfig field description for slotIndex in TS 38.331) |

NR_TDD_Info_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_TDD_Info_Type |  |
| Comment | cell specific parameters for TDD (unpaired spectrum) |  |
| Config | NR_TDD_Config_Type | specific TDD configuration with sets of symbols for UL and DL and possibly flexible symbols which are not specified as UL or DL (corresponding to TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated according to TS 38.213 clause 11.1) |
| FullFlexible | Null_Type | No TDD configuration is provided to the UE: all slots and symbols are considered as flexible according to TS 38.213 clause 11.1 |

#### D.1.3.2.2	PhysicalLayer_Downlink

PhysicalLayer_Downlink: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_EPRE_Ratio_Type | integer | Energy per resource element relative to given reference signal or abstract reference cell power (dB) |

NR_CellConfigPhysicalLayerDownlink_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellConfigPhysicalLayerDownlink_Type |  |  |
| Comment | physical layer configuration at the SS for the downlink of a cell |  |  |
| FrequencyInfoDL | NR_ASN1_FrequencyInfoDL_Type | opt | carries information about location of SSB and reference resource block (point A) in frequency domain and about associated frequency bands (list of FreqBandIndicatorNR) |
| SSPbchBlock | NR_SSB_Config_Type | opt | Configuration of SS/PBCH-block transmission |
| PdschCellLevelConfig | NR_PDSCH_CellLevelConfig_Type | opt | Cell-level configuration of PDSCH being applicable independent from the BWP a PDSCH is associated to |
| BWPs | NR_DownlinkBWPs_Type | opt | Configuration of DL BWPs and their associated physical channels and signals |
| CsiConfig | NR_CSI_Config_Type | opt | Configuration of CSI Reference Signals |

##### D.1.3.2.2.1	SS_PBCH_Block

SS/PBCH block configuration according to TS 38.213 clause 4.1:
SS/PBCH block consists of synchronisation Signals (PSS and SSS) and PBCH (see e.g. TS 38.300 figure 5.2.4-1);
a demodulation reference signal (DM-RS) is frequency multiplexed on the PBCH symbols (TS 38.300 clause 5.2.4, TS 38.211 clause 7.4.1.4) and
the DM-RS sequence corresponds to the three LSBs of the SS/PBCH index (TS 38.213 clause 4.1 and TS 38.211 clause 7.4.1.4.1);
the SS/PBCH index needs to be maintained by the SS (as the system frame number);
the physical layer cell id is carried by PSS and SSS according to TS 38.211 clause 7.4.2

SS_PBCH_Block: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_SSB_Periodicity_Type | ServingCellConfigCommon.ssb_periodicityServingCell |  |
| NR_SSB_PositionsInBurst_Type | ServingCellConfigCommon.ssb_PositionsInBurst |  |

NR_SS_BlockPattern_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_SS_BlockPattern_Type |
| Comment | TS 38.101-1 Table 5.4.3.3-1 specifies for a given operating band and SS Block subcarrier spacing which case of TS 38.213 clause 4.1 to be applied => first symbol indexes for candidate SS/PBCH blocks and the size of the bitmap are determined accordingly |
| caseA | 15 kHz subcarrier spacing: 4 bits (<= 3GHz) or 8 bits (> 3GHz); first symbol indexes: {2,8} + 14*n |
| caseB | 30 kHz subcarrier spacing: 4 bits (<= 3GHz) or 8 bits (> 3GHz); first symbol indexes: {4,8,16,20} + 28*n |
| caseC | 30 kHz subcarrier spacing: 4 bits (<= 3GHz) or 8 bits (> 3GHz); first symbol indexes: {2,8} + 14*n |
| caseD | 120 kHz subcarrier spacing: 64 bits (> 6GHz); first symbol indexes: {4,8,16,20} + 28*n |
| caseE | 240 kHz subcarrier spacing: 64 bits (> 6GHz); first symbol indexes: {8,12,16,20,32,36,40,44} + 56*n |

NR_SSB_Beam_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SSB_Beam_Type |  |  |
| Comment |  |  |  |
| SsbIndex | integer | opt | SSB index starting at 0 according to TS 38.213 clause 4.1 |
| Attenuation | integer | opt | Beam power: reference power for SSB transmissions relative to the actual reference cell power (MaxReferencePower - Attenuation of cell power); the beam power is reduced by 'Attenuation' relative to the actual reference cell power; the attenuation may be negative in which case the power level of the SSB transmission is higher than the actual cell power |

NR_SSB_BeamArray_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SSB_BeamArray_Type |
| Comment |  |
| record of NR_SSB_Beam_Type |  |

NR_SSB_BurstConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SSB_BurstConfig_Type |  |  |
| Comment | To describe the SSB burst |  |  |
| BlockPattern | NR_SS_BlockPattern_Type | opt | case A..E according to 38.213 clause 4.1; mandatory for initial configuration, "keep as it is" in case of omit otherwise |
| PositionsInBurst | NR_SSB_PositionsInBurst_Type | opt | "SSB-transmitted" parameter as used by the UE to rate-match around SSBs acc. 38.214 cl. 5.1; 4, 8 or 64 bits; mandatory for initial configuration, "keep as it is" in case of omit otherwise |
| BeamArray | NR_SSB_BeamArray_Type | opt | beam specific configuration: if omit, all SSBs as configured in 'Bitmap' shall be transmitted with no attenuation (i.e. using the actual reference cell power); if present only the SSBs contained in the array shall be transmitted (with beam power as according to their entry in the array); when the array contains beams with an SSB index not included in 'Bitmap', the SS may raise an error |

NR_SSB_EPREs_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SSB_EPREs_Type |  |  |
| Comment | EPRE for PBCH and related signals relative to the reference power (EPRE_SSB#N) of an SSB (beam) given by SSB index N |  |  |
| PbchToDmrs | NR_EPRE_Ratio_Type | opt | transmit power for resource elements (REs) being occupied by PBCH; EPRE ratio of PBCH to PBCH DMRS |
| PssToSss | NR_EPRE_Ratio_Type | opt | Primary synchronization signal; 38.211 clause 7.4.2.2; EPRE ratio of PSS to SSS |
| SssToSsbBeam | NR_EPRE_Ratio_Type | opt | Secondary synchronization signal; 38.211 clause 7.4.2.3; EPRE ratio of SSS to EPRE_SSB#N; in general the SSS power is the same as the reference beam power, i.e. SssToSsbBeam = 0dB |
| DmrsToSss | NR_EPRE_Ratio_Type | opt | DM-RS associated to PBCH (Demodulation reference signals for PBCH; 38.211 clause 7.4.1.4); EPRE ratio of PBCH DMRS to SSS |

NR_SSB_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SSB_Config_Type |  |  |
| Comment | Synchronization signals and PBCH; TS 38.211 clause 7.4.3, TS 38.300 clause 5.2.4; NOTE: SSB location in frequency domain is specified by NR_CellConfigPhysicalLayerDownlink_Type.FrequencyInfoDL.absoluteFrequencySSB |  |  |
| SubCarrierSpacing | SubcarrierSpacing | opt | sub-carrier spacing for SS/PBCH block (as specified by ServingCellConfigCommon.subcarrierSpacing in case of non-initial access): According to comments for ServingCellConfigCommon.subcarrierSpacing "Only the values 15 or 30 kHz (<6GHz), 120 or 240 kHz (>6GHz) are applicable"; this corresponds to tables 13-1 .. 13.10 in TS 38.213 where only 15 or 30 kHz and 120 or 240 kHz are considered for SS/PBCH block sub-carrier spacing; and it corresponds to 38.211 clause 7.4.3.1 defining SS/PBCH block type A as numerology=0,1 and type B as numerology=3,4 (i.e. there is no numerology=2 for SS/PBCH block) NOTE 1: in contrast to SS/PBCH block sub-carrier spacing the sub-carrier spacing for SIB1, Msg.2/4 for initial access and broadcast SI-messages is restricted to 15kHz or 30kHz for carrier frequency <= 6 GHz and 60kHz or 120kHz for carrier frequency > 6 GHz (see comments for MIB.subCarrierSpacingCommon and TS 38.213 tables 13-1 .. 13.10) NOTE 2: As long as there is no sub-carrier spacing of 60kHz (numerology=2) for the SS/PBCH block acc. to TS 38.211 Table 4.2-1 there is no extended cyclic prefix either (even though Table 4.2-1 is mainly for BWP there is no indication for any extended cyclic prefix for SS/PBCH block) => there is no need to specify the cyclic prefix for SS/PBCH block configuration (normal cyclic prefix is assumed for all cases) |
| SubcarrierOffset | integer | opt | k_SSB as defined in TS 38.211 clause 7.4.3.1; needs to be consistent with absoluteFrequencySSB and absoluteFrequencyPointA as provided by FrequencyInfoDL to the UE and the SS |
| Periodicity | NR_SSB_Periodicity_Type | opt | in multiples of half frames (5ms) |
| HalfFrameOffset | integer | opt | to specify together the Periodicity the half-frames in which the SSB burst shall be transmitted: - for Periodicity = 5ms SSB burst in every half-frame - for a Periodicity >= 5ms SSB burst in frames with (SFN mod (Periodicity / 10)) = (HalfFrameOffset / 2) and in the - lower half-frame for (HalfFrameOffset mod 2) = 0 - higher half-frame for (HalfFrameOffset mod 2) = 1 Depending on the Periodicity the HalfFrameOffset has a range of 0 .. ((Periodicity / 5) - 1) Unless explicitly required by a test case the HalfFrameOffset is always 0 |
| BurstConfig | NR_SSB_BurstConfig_Type | opt | to specify the burst configuration and a bitmap for the SS/PBCH block candidates which are eventually used for transmission of SS/PBCH blocks in a half frame |
| RelativeTxPower | NR_SSB_EPREs_Type | opt | transmit power for PBCH and SS/PBCH signals NOTE: Parameter SS-PBCHBlockPower is provided to the UE in SIB1.ss-PBCH-BlockPower, ServingCellConfigCommon.ss-PBCH-BlockPower) as referenceSignalPower; The UE uses referenceSignalPower to determine the transmission power of the PRACH (TS 38.213 clause 7.4) => For signalling tests there seems to be no need to provide this parameter to the SS (nevertheless the value provided to the UE shall not conflict with the power settings for the SSB at the SS) |

##### D.1.3.2.2.2	CSI_Configuration

NR_CSI_RS_Periodicity_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_CSI_RS_Periodicity_Type |  |
| Comment | NOTE: may be extended with 'Aperiodic' branch if needed |  |
| PeriodicityAndOffset | CSI_ResourcePeriodicityAndOffset | periodicity and slot offset as used by NZP-CSI-RS-Resource for periodic and semi-persistent configuration; the periodicity is given in the number of slots whereas CSI-RS-Resource-Mobility.slotConfig specifies the periodicity in millseconds => to configure the CSI-RS configuration corresponding to CSI-RS-Resource-Mobility the periodicity needs to be converted in TTCN depending on the numerology |

NR_NZP_CSI_RS_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_NZP_CSI_RS_Config_Type |  |  |
| Comment | Channel-state information reference signal (CSI) according to TS 38.211 clause 7.4.1.5, TS 38.214 clause 5.1.6.1 and clause 5.2.2.3; the UE may be configured with non-zero-power (NZP) CSI-RS by a) NZP-CSI-RS-Resource (contained in CSI-MeasConfig) or b) CSI-RS-Resource-Mobility (contained in MeasConfig) (according to TS 38.211 clause 7.4.1.5.1) |  |  |
| ScramblingId | ScramblingId | opt | INTEGER(0..1023); Scrambling ID for CSI-RS as provided to the UE in NZP-CSI-RS-Resource.scramblingID or CSI-RS-Resource-Mobility.sequenceGenerationConfig |
| ResourceMapping | CSI_RS_ResourceMapping | opt | resource mapping as used in NZP-CSI-RS-Resource; there is the following mapping with CSI-RS-CellMobility - frequencyDomainAllocation: For CSI-RS-Resource-Mobility there are row1 or row2 only - nrofPorts: One port for CSI-RS-Resource-Mobility (according to TS 38.211 Table 7.4.1.5.3-1 for Row=1 and Row=2) - firstOFDMSymbolInTimeDomain (l0): same for CSI-RS-ResourceMapping and CSI-RS-Resource-Mobility - firstOFDMSymbolInTimeDomain2 (l1): Not present for CSI-RS-Resource-Mobility (there is no l1 for Row=1 and Row=2 in Table 7.4.1.5.3-1 of TS 38.211) - cdm-Type: No CDM for CSI-RS-Resource-Mobility (according to Row=1 and Row=2 in Table 7.4.1.5.3-1 of TS 38.211) - density: ENUMERATED {d1,d3} for CSI-RS-CellMobility to be mapped to CHOICE (one, three) of CSI-RS-ResourceMapping - freqBand: According to TS 38.331 a) CSI-FrequencyOccupation specifies the frequency range for CSI-RS relative to a given BWP (CSI-ResourceConfig.bwp-id) whereas b) CSI-RS-CellMobility specifies the position relative to point A (given by MeasObjectNR.refFreqCSI-RS) => As there is no equivalent to CSI-ResourceConfig.bwp-id for variant b), 'OffsetToFreqBand' is used to fully specify the frequency range (see below) |
| OffsetToFreqBand | integer | opt | The frequency band for CSI-RS starts at OffsetToFreqBand + ResourceMapping.freqBand.startingRB from point A. a) When the UE is configured with NZP-CSI-RS-Resource: ResourceMapping.freqBand shall be set to the same value as signalled to the UE and OffsetToFreqBand shall be set offsetToCarrier + BWP.RB_Start. (NOTE: in general RB_Start is 0 in BWP.locationAndBandwidth: i.e. BWP.RB_Start=0 => OffsetToFreqBand = offsetToCarrier) b) When the UE is configured with CSI-RS-CellMobility: OffsetToFreqBand and ResourceMapping.freqBand.startingRB shall be set so that OffsetToFreqBand + ResourceMapping.freqBand.startingRB = CSI-RS-CellMobility.csi-rs-MeasurementBW.startPRB. (NOTE: In general it shall be OffsetToFreqBand = offsetToCarrier and ResourceMapping.freqBand.startingRB = CSI-RS-CellMobility.csi-rs-MeasurementBW.startPRB - offsetToCarrier) |
| Periodicity | NR_CSI_RS_Periodicity_Type | opt |  |
| Attenuation | integer | opt | Beam power: reference power for CSI-RS transmissions relative to the actual reference cell power (MaxReferencePower - Attenuation of cell power); see NR_SSB_Beam_Type |

NR_NZP_CSI_RS_ConfigList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_NZP_CSI_RS_ConfigList_Type |
| Comment | Array with set of CSI reference signals; each CSI-RS may belong to a different beam |
| record of NR_NZP_CSI_RS_Config_Type |  |

NR_CSI_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_CSI_Config_Type |  |
| Comment | Primitive for configuration CSI at the SS NOTE: further branches may be added e.g. to support CSI-IM |  |
| CSI_RS | NR_NZP_CSI_RS_ConfigList_Type | Configuration of CSI reference signals: Configuration of CSI reference signals does not necessarily mean that there is CSI reporting but CSI-RS is also needed for - link monitoring (beam failure scenarios) and - neighbouring cell measurement. For these cases periodic CSI-RS is used and may be started immediately (TimingInfo=Now) or at a specific point in time with periodicity as specified by Periodicity.PeriodicityAndOffset. Semi-persistent CSI-RS could be realised in the same way as periodic CSI-RS (but with specific TimingInfo). Aperiodic CSI-RS is not considered so far (NOTE: For simple cases of aperiodic CSI-RS NR_CSI_RS_Periodicity_Type may be extended with an 'Aperiodic' branch but e.g. combination of periodic and aperiodic CSI-RS would require a more complex approach, e.g. with explicit trigger for the transmission of the CSI-RS) |
| None | Null_Type |  |

##### D.1.3.2.2.3	Cell_Level_Configuration_PDSCH

Cell_Level_Configuration_PDSCH: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_PDSCH_DMRS_TypeA_Position_Type | MIB.dmrs_TypeA_Position | same as ServingCellConfigCommon.dmrs-TypeA-Position |

NR_PDSCH_CellLevelConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDSCH_CellLevelConfig_Type |  |  |
| Comment | cell-level parameters for PDSCH: in contrast to BWP specific parameters the cell-level parameters apply to any PDSCH independent from the BWP a PDSCH is associated to |  |  |
| DMRS_TypeA_Position | NR_PDSCH_DMRS_TypeA_Position_Type | opt | Provided to the UE by MIB.dmrs-TypeA-Position or ServingCellConfigCommon.dmrs-TypeA-Position; dmrs-TypeA-Position defines position of the first DM-RS symbol in the sequence of DM-RS symbols according to TS 38.211 clause 7.4.1.1. |
| RateMatchPattern | NR_CellLevelRateMatchPattern_Type | opt | rate match pattern according to TS 38.214 clause 5.1.4 |
| ServingCellConfig | NR_ASN1_PDSCH_ServingCellConfig_Type | opt | PDSCH related parameters not being BWP-specific; corresponds to ServingCellConfig.pdsch-ServingCellConfig |

NR_RateMatchPatternList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_RateMatchPatternList_Type |
| Comment |  |
| record of NR_ASN1_RateMatchPattern_Type |  |

NR_RateMatchPatternLteCrsList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_RateMatchPatternLteCrsList_Type |
| Comment |  |
| record of NR_ASN1_RateMatchPatternLTE_CRS_Type |  |

NR_CellLevelRateMatchPattern_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellLevelRateMatchPattern_Type |  |  |
| Comment | configuration of rate match pattern on cell level (see TS 38.214 clause 5.1.4) |  |  |
| PatternList | NR_RateMatchPatternList_Type | opt | up to 4 cell-level RateMatchPattern according to TS 38.214 clause 5.1.4 (L1 RateMatchPattern); corresponds to ServingCellConfigCommon.rateMatchPatternToAddModList/rateMatchPatternToReleaseList; empty list per default (i.e. at initial configuration) |
| PatternListLteCrs | NR_RateMatchPatternLteCrsList_Type | opt | 0 or 1 LTE CRS pattern to rate match around (see TS 38.214 clause 5.1.4.2) corresponds to ServingCellConfigCommon.lte-CRS-ToMatchAround; empty list per default (i.e. at initial configuration) |

##### D.1.3.2.2.4	Downlink_BWP

NR_DownlinkBWP_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DownlinkBWP_Type |  |  |
| Comment | Configuration of single BWP at the SS |  |  |
| Id | BWP_Id | opt | Initial BWP and RedCap-specific initial BWP: 0 Dedicated BWP: 1..4 |
| BWP | NR_ASN1_BWP_Type | opt | Frequency domain location and bandwidth, subcarrier spacing, cyclic prefix |
| Pdcch | NR_BWP_PDCCH_Configuration_Type | opt |  |
| Pdsch | NR_BWP_PDSCH_Configuration_Type | opt |  |
| Sps | NR_ASN1_SPS_Config_Type | opt | BWP-DownlinkDedicated.SPS-Config |
| SL_Pdcch | NR_BWP_PDCCH_Configuration_Type | opt | For sidelink scheduled mode |

NR_DownlinkBWP_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_DownlinkBWP_List_Type |
| Comment | configuration of BWPs: each entry shall have a distinct Id, except for the initial DL BWP and the RedCap-specific initial DL BWP, which both use ID=0. NOTE 1: Even though in general the BWP-Id corresponds to the index of the element within the array of BWPs, the SS shall not take this as assumption |
| record of NR_DownlinkBWP_Type |  |

NR_DownlinkBWPs_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DownlinkBWPs_Type |  |  |
| Comment | configuration of downlink BWPs |  |  |
| ActiveBWP | BWP_Id | opt | Id of the currently active BWP (this does not need to be the same as the index) According to TS 38.211 clause 4.4.5: "A UE can be configured with up to four carrier bandwidth parts in the downlink with a single downlink carrier bandwidth part being active at a given time. The UE is not expected to receive PDSCH, PDCCH, CSI-RS, or TRS outside an active bandwidth part." |
| BwpArray | NR_DownlinkBWP_List_Type | opt | array of band width parts: initial BWP + up to 4 dedicated BWPs |
| InitialBWP_RedCap | NR_DownlinkBWP_Type | opt | RedCap-specific initial DL BWP. May be omitted at initial configuration when it is not applied (e.g. no RedCap-specific initial DL BWP to be configured), otherwise 'omit' means keep as it is. |

SPS_Config_R16

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SPS_Config_R16 |  |  |
| Comment |  |  |  |
| Sps_ConfigToAddModList_r16 | SPS_ConfigToAddModList_r16 | opt |  |
| Sps_ConfigToReleaseList_r16 | SPS_ConfigToReleaseList_r16 | opt |  |
| Sps_ConfigDeactivationStateList_r16 | SPS_ConfigDeactivationStateList_r16 | opt |  |

###### D.1.3.2.2.4.1	PDSCH_Configuration

Configuration of PDSCH and its related reference signals:
- DM-RS (Demodulation reference signal); TS 38.211 clause 7.4.1.1
- PT-RS (Phase-tracking reference signals for PDSCH); TS 38.211 clause 7.4.1.2

NR_BWP_PDSCH_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_BWP_PDSCH_Configuration_Type |  |  |
| Comment | PDSCH configuration at the SS for specific BWP |  |  |
| ConfigCommon | NR_ASN1_PDSCH_ConfigCommon_Type | opt |  |
| ConfigDedicated | NR_ASN1_PDSCH_Config_Type | opt |  |
| RelativeTxPower | NR_PDSCH_EPREs_Type | opt |  |

NR_PDSCH_EPREs_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDSCH_EPREs_Type |  |  |
| Comment | EPRE for PDSCH and related signals |  |  |
| PdschToCell | NR_EPRE_Ratio_Type | opt | transmit power relative to given reference cell power for resource elements (REs) being occupied by PDSCH |
| PdschToDmrs | NR_EPRE_Ratio_Type | opt | EPRE power ratio of PDSCH to DM-RS being associated to the PDSCH according to TS 38.214 clause 4.1 |
| PdschToPtrs | NR_EPRE_Ratio_Type | opt | EPRE power ratio of PDSCH to PT-RS being associated to the PDSCH according to TS 38.214 clause 4.1 NOTE: PT-RS need only to be considered when being present (TS 38.211 clause 7.4.1.2.2 according to TS 38.214 clause 4.1), i.e. the SS shall ignore the PT-RS's EPRE if no PT-RS is present |

###### D.1.3.2.2.4.2	PDCCH_Configuration

Configuration of PDSCH and its related reference signals:
- DM-RS (Demodulation reference signal); TS 38.211 clause 7.4.1.3

NR_BWP_PDCCH_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_BWP_PDCCH_Configuration_Type |  |  |
| Comment | PDCCH configuration at the SS for specific BWP; NOTE: There are no fields for PDCCH-Config's "downlinkPreemption", "slotFormatIndicator", "tpc-PUSCH" and "tpc-PUCCH": This information is related to triggering DCI formats 2_X an shall be configured there (NR_DCI_Trigger_Type) according to test case requirements |  |  |
| SearchSpaceArray | NR_BWP_SearchSpaceList_Type | opt |  |
| CoresetArray | NR_BWP_CoresetList_Type | opt |  |
| RelativeTxPower | NR_PDCCH_EPREs_Type | opt |  |
| Coreset0_OffsetRBs | integer | opt | Number of Offset RBs as specified in 38.213 clause 13. Applied to CORESET#0 only |

NR_BWP_CoresetList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_BWP_CoresetList_Type |
| Comment | list of CORESETs defined for a single BWP; according to TS 38.213 clause 10.1 "The control resource set configured for Type0-PDCCH common search space has control resource set index 0" NOTE: even though in general (array) index and controlResourceSetId are the same it is not clear what is meant by TS 38.213 |
| record of NR_ASN1_ControlResourceSet_Type |  |

NR_BWP_SearchSpaceList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_BWP_SearchSpaceList_Type |
| Comment | list of search spaces defined for a single BWP; according to TS 38.213 clause 10.1 "The Type0-PDCCH common search space has search space index 0" NOTE: even though in general (array) index and searchSpaceId are the same it is not clear what is meant by TS 38.213 |
| record of NR_BWP_SearchSpaceConfig_Type |  |

NR_PDCCH_EPREs_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCCH_EPREs_Type |  |  |
| Comment | EPRE for PDCCH and related signals |  |  |
| PdcchToCell | NR_EPRE_Ratio_Type | opt | transmit power relative to given reference cell power for resource elements (REs) being occupied by PDCCH |
| PdcchToDmrs | NR_EPRE_Ratio_Type | opt | EPRE power ratio of PDCCH to DM-RS being associated to the PDCCH; further DMRS parameters are given by ASN.1 PDCCH-Config as signalled to the UE or by additional parameters of the initial BWP configuration |

D.1.3.2.2.4.2.1	Search_Space_Configuration

Search_Space_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_SearchSpaceCandidatePriority_Type | UInt_Type | Priorities to be considered by the SS in order to choose the candidate of a search space not colliding with the candidate of some other search space; a value of 0 represents the highest priority, a value of 1 the second highest priority and so on. (see TS 38.523-3 clause 7.1.2.1.4 for further details) |

NR_SearchSpaceType_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_SearchSpaceType_Type |
| Comment | types of search spaces according to TS 38.213 clause 10.1; NOTE 1: In principle there could be more than one instance for the same kind of search space; if this is ever needed, this enumerated shall be enhanced by introduction of a second entry for a particular type. NOTE 2: For the USS UL and DL are distinguished as they may need different priority in terms of scheduling of DCIs |
| cssType0 | Type0-PDCCH common search space for scheduling of SIB1 (SI-RNTI); the SS shall scramble the DCI format's CRC with SI-RNTI; aggregation level and number of candidates correspond to TS 38.213 Table 10.1-1; For stand-alone options parameters of search space and associated CORESET are according MIB.pdcch-ConfigSIB1 (TS 38.213 clause 13) |
| cssType0A | Type0A-PDCCH common search space for scheduling of other system information (SI-RNTI); the SS shall scramble the DCI format's CRC with SI-RNTI |
| cssType1 | Type1-PDCCH common search space for scheduling of Msg2 or Msg4 of RACH procedure; the SS shall scramble the DCI format's CRC with RA-RNTI for Msg2 or with TC-RNTI for Msg4 (temporary C-RNTI) as configured for Msg4 (NR_TempC_RNTI_Type in NR_RAR_Payload_Type) |
| cssType2 | Type2-PDCCH common search space for scheduling of Paging messages or short messages (P-RNTI); the SS shall scramble the DCI format's CRC with P-RNTI |
| cssType2A | Type2A-PDCCH common search space for scheduling of Paging Early Indication messages the SS shall scramble the DCI format's CRC with PEI-RNTI |
| cssType3 | Type3-PDCCH common search space for other purpose DCIs (INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, C-RNTI, CS-RNTI(s)); the SS shall scramble the DCI format's CRC with the RNTI-value as according to the ASP triggering the DCI to be sent |
| ussDL | UE-specific search space (C-RNTI, CS-RNTI(s)); the SS shall scramble the DCI format's CRC with the C-RNTI value as stored for the cell (NR_CellConfigCommon_Type) |
| ussUL | UE-specific search space: as USS_DL but in general with lower priority to give DL assignments precedence over periodic UL grants |
| ussSL | UE-specific sidelink search space (SL-RNTI, SL-CS-RNTI); SS configured with NR_CellConfigInfo_Type.AddOrReconfigure.PhysicalLayer.Downlink.BWPs.BwpArray.SL_Pdcch. The SS shall scramble the DCI format's CRC with the SL-RNTI/SL-CS-RNTI value provided in NR_CellConfigInfo_Type.AddOrReconfigure.SidelinkScheduled.SL_DCI_Config.SearchSpaceAndDci.DciInfo.Format |

NR_SearchSpaceTypeAndPriority_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SearchSpaceTypeAndPriority_Type |  |  |
| Comment |  |  |  |
| Type | NR_SearchSpaceType_Type |  |  |
| CandidatePriority | NR_SearchSpaceCandidatePriority_Type |  | priority to be considered when candidates of a different or the same search space overlap the same CCEs |

NR_SearchSpaceTypeAndPriorityList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SearchSpaceTypeAndPriorityList_Type |
| Comment |  |
| record of NR_SearchSpaceTypeAndPriority_Type |  |

NR_PDCCH_CCE_AggregationLevel_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_PDCCH_CCE_AggregationLevel_Type |
| Comment | Aggregation level for a search space |
| AggregationLevel1 |  |
| AggregationLevel2 |  |
| AggregationLevel4 |  |
| AggregationLevel8 |  |
| AggregationLevel16 |  |

NR_BWP_SearchSpaceConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_BWP_SearchSpaceConfig_Type |  |  |
| Comment | configuration of a single search space at the SS: The position of a particular search space candidate in frequency and time domain depends on configuration of the search space and its associated CORESET (see TS 38.213 clause 10.1): - Common or UE-specific search space - aggregation level L - number of candidates per aggregation level - PDCCH monitoring periodicity and offset - frequency domain resources - number of symbols (time domain) in addition in case of UE-specific search space: - C-RNTI (as configured for the active cell) - carrier indicator field value (in case of cross carrier scheduling) all fields are mandatory as modification of a single field may cause inconsistencies |  |  |
| TypeAndPriorityList | NR_SearchSpaceTypeAndPriorityList_Type |  | list of search space types (according to TS 38.213 clause 10.1) to be mapped to the given search space configuration. NOTE 1: In general the lists of search space types for different search spaces shall be mutual exclusive; NOTE 2: TS 38.213 clause 10.1 could be read as if the different types of search spaces use distinct instances of search spaces; nevertheless RRC type definitions allow use of different types in one and the same search space |
| AggregationLevel | NR_PDCCH_CCE_AggregationLevel_Type |  | aggregation level to be applied for an actual PDCCH of the given search space |
| SearchSpaceConfigAtUE | NR_ASN1_SearchSpace_Type |  | search space configuration as sent to the UE; contains searchSpaceId and controlResourceSetId referring to the associated CORESET; furthermore the SS may need e.g. the number of candidates to detect error situations in context of candidate selection |

D.1.3.2.2.4.2.2	Search_Space_DCI_Assignment

NR_BWP_Id_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_BWP_Id_List_Type |
| Comment |  |
| record of BWP_Id |  |

NR_AssignedBWPs_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_AssignedBWPs_Type |  |  |
| Comment | definition of a set of BWPs being assigned e.g. to system information scheduling or RACH procedures NOTE 1: there is no error when e.g. "ActiveBWP" is set and the currently active BWP is contained in DedicatedBWPs too NOTE 2: it is up to use of this type in TTCN to ensure that a specific BWP assignment makes sense, in most cases multiple BWPs may not be applicable NOTE 3: "omit" means "not applied" |  |  |
| ActiveBWP | Null_Type | opt | SS shall assign the currently active BWP |
| InitialBWP | Null_Type | opt | SS shall assign the initial BWP |
| DedicatedBWPs | NR_BWP_Id_List_Type |  | SS shall assign all BWPs as listed (on top of active or initial BWP if set); an empty list indicates that no (additional) BWPs are assigned |
| InitialBWP_RedCap | Null_Type | opt | SS shall assign the RedCap-specific initial BWP |

NR_SearchSpaceDlDciAssignment_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SearchSpaceDlDciAssignment_Type |  |  |
| Comment | configuration of DCI for a specific search space; in general the configuration belongs to a transport channel configuration (e.g. BCH, PCH, DL-SCH): the DCI is applied for DL transmission on the respective channel and can be explicitly initiated by TTCN (e.g. PCH, DL-SCH) or automatically by the SS (e.g. BCH or Msg2/Msg4 scheduling of RACH procedure); all fields are mandatory for the first configuration of an instance for modifications "omit" means "keep as it is" |  |  |
| AssignedBWPs | NR_AssignedBWPs_Type | opt | BWP where given DCI shall be scheduled in given search space; NOTE 1: When there is no BWP according to configuration of AssignedBWPs, there is no DL assignment; this can be used e.g. to prevent automatic scheduling of system information; NOTE 2: In principle a DL assignment can happen simultaneously in more than one BWP e.g. if system information shall be scheduled in active and initial BWP; but as long as there is no use case for simultaneous DL assignments, configuration of AssignedBWPs shall be restricted to a maximum of one BWP |
| SearchSpaceType | NR_SearchSpaceType_Type | opt | search space to be used for sending of given DCI; when at the scheduled point in time of a DL transmission there is no such search space configured at the given BWP, the SS may raise an error |
| DciInfo | NR_DciDlInfo_Type | opt | DCI to be used |

NR_SearchSpaceUlDciAssignment_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SearchSpaceUlDciAssignment_Type |  |  |
| Comment | configuration of DCI for UL grants in the UL USS; in general the configuration belongs to a transport channel configuration (e.g. UL-SCH); all fields are mandatory for the first configuration of an instance for modifications "omit" means "keep as it is" |  |  |
| AssignedBWPs | NR_AssignedBWPs_Type | opt | BWP where given DCI shall be scheduled in given search space; AssignedBWPs shall specify exactly one BWP (ActiveBWP in general); the SS may raise an error otherwise |
| SearchSpaceType | NR_SearchSpaceType_Type | opt | search space to be used for sending of given DCI; when at the scheduled point in time of a UL grant transmission there is no such search space configured at the given BWP, the SS may raise an error |
| DciInfo | NR_DciUlInfo_Type | opt | DCI to be used |

#### D.1.3.2.3	PhysicalLayer_Uplink

Uplink physical layer configuration: UL channels and BWPs

NR_CellConfigPhysicalLayerUplink_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellConfigPhysicalLayerUplink_Type |  |  |
| Comment | physical layer configuration at the SS for the uplink of a cell |  |  |
| Uplink | NR_Uplink_Type | opt |  |
| SupplementaryUplink | NR_Uplink_Type | opt |  |
| TimingAdvance | NR_SS_TimingAdvanceConfig_Type | opt |  |
| PUSCH_ServingCellConfig | NR_ASN1_PUSCH_ServingCellConfig_Type | opt |  |
| PUSCH_ServingCellConfigSUL | NR_ASN1_PUSCH_ServingCellConfig_Type | opt |  |

NR_Uplink_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_Uplink_Type |  |
| Comment |  |  |
| Config | NR_UplinkConfig_Type |  |
| None | Null_Type | in case the uplink or supplementary uplink is not used/needed |

NR_SS_TimingAdvanceConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SS_TimingAdvanceConfig_Type |  |
| Comment |  |  |
| InitialValue | NR_RACH_TimingAdvance_Type | initial 12 bit value corresponding to Timing Advance Command field of the Random Access Response (TS 38.321 clause 6.2.3): value of 0..3846 according to TS 38.213 clause 4.2; 0 in normal cases) |
| Relative | NR_TimingAdvanceIndex_Type | timing advance command to adjust changes of timing advance acc. to TS 38.213 clause 4.2; (range acc. 6 bit value: -31..32) |

##### D.1.3.2.3.1	Uplink_BWP

NR_UplinkBWP_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_UplinkBWP_Type |  |  |
| Comment | Configuration of single BWP at the SS; NOTE: for rel-15 this is the same as the ASN.1 type "BWP-Uplink" but nevertheless the TTCN type may be useful for future extensions |  |  |
| Id | BWP_Id | opt | Initial BWP and RedCap-specific initial BWP: 0 Dedicated BWP: 1..4 |
| Common | NR_ASN1_BWP_UplinkCommon_Type | opt | contains common configuration for RACH, PUSCH, PUCCH configuration at the UE: - Initial BWP: -> ServingCellConfigCommon.uplinkConfigCommon.initialUplinkBWP ServingCellConfigCommon.supplementaryUplinkConfig.initialUplinkBWP -> SIB1.servingCellConfigCommon.uplinkConfigCommon.initialUplinkBWP SIB1.supplementaryUplink.uplinkConfigCommon.initialUplinkBWP - RedCap-Specific initial BWP (if configured): -> ServingCellConfigCommon.uplinkConfigCommon-v1700.initialUplinkBWP-RedCap-r17 -> SIB1.uplinkConfigCommon.initialUplinkBWP.initialUplinkBWP-RedCap-r17 - Dedicated BWP: -> ServingCellConfig.uplinkConfig.uplinkBWP-ToAddModList[-].bwp-Common ServingCellConfig.supplementaryUplink.uplinkBWP-ToAddModList[-].bwp-Common |
| Dedicated | NR_ASN1_BWP_UplinkDedicated_Type | opt | contains dedicated configuration for PUCCH, PUCCH, ConfiguredGrant, SRS, BeamFailureRecovery configuration at the UE: - Initial BWP: -> ServingCellConfig.uplinkConfig.initialUplinkBWP ServingCellConfig.supplementaryUplink.initialUplinkBWP - Dedicated BWP: -> ServingCellConfig.uplinkConfig.uplinkBWP-ToAddModList[-].bwp-Dedicated ServingCellConfig.supplementaryUplink.uplinkBWP-ToAddModList[-].bwp-Dedicated |

NR_UplinkBWP_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_UplinkBWP_List_Type |
| Comment | configuration of BWPs: each entry shall have a distinct Id, except for the initial UL BWP and the RedCap-specific initial UL BWP, which both use ID=0. NOTE: Even though in general the BWP-Id corresponds to the index of the element within the array of BWPs, the SS shall not take this as assumption |
| record of NR_UplinkBWP_Type |  |

NR_ActiveUplinkBWP_Id_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ActiveUplinkBWP_Id_Type |  |
| Comment |  |  |
| Explicit | BWP_Id | in case that BWP-Id of active UL-BWP (and/or active UL-BWP of supplementary UL) is different than BWP-Id of active DL-BWP |
| SameIdAsDL | Null_Type | same BWP-Id as of the active DL-BWP |

NR_UplinkBWPs_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_UplinkBWPs_Type |  |  |
| Comment | configuration of uplink BWPs |  |  |
| ActiveBWP | NR_ActiveUplinkBWP_Id_Type | opt | Id of the currently active BWP (this does not need to be the same as the index) |
| BwpArray | NR_UplinkBWP_List_Type | opt | array of band width parts: initial BWP + up to 4 dedicated BWPs |
| InitialBWP_RedCap | NR_UplinkBWP_Type | opt | RedCap-specific initial UL BWP. May be omitted at initial configuration when it is not applied (e.g. no RedCap-specific initial UL BWP to be configured), otherwise 'omit' means keep as it is. |

NR_UplinkConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_UplinkConfig_Type |  |  |
| Comment | configuration of a single uplink (uplink or supplementary uplink) |  |  |
| FrequencyInfoUL | NR_ASN1_FrequencyInfoUL_Type | opt | carries information about location of reference resource block (point A) in frequency domain and about associated frequency bands (list of FreqBandIndicatorNR) |
| BWPs | NR_UplinkBWPs_Type | opt |  |
| RACH_ConfigDedicated | NR_ASN1_RACH_ConfigDedicated_Type | opt | configuration at the UE: -> SpCellConfig.reconfigurationWithSync.rach-ConfigDedicated.uplink/supplementaryUplink |
| SI_RequestConfig | NR_ASN1_SI_RequestConfig_Type | opt | configuration of PRACH preamble(s) and PRACH resource(s) for si-RequestConfig configuration at the UE: -> in SIB1.si-SchedulingInfo.si-RequestConfig |

#### D.1.3.2.4	DCI_Configuration

Definitions for resource assignment and DCI according to TS 38.212 clause 7.3 and TS 38.214 clause 5.1.2 and 6.1.2

##### D.1.3.2.4.1	Common_Fields

Common type definitions for DCI fields being used for UL and DL assignments (format 0_X and 1_X);
NOTE: in general fields of DCIs are defined as union in the first place to allow backward compatible enhancements,
e.g. when a dynamic SS behaviour needs to replace the static value assignment

NR_DciCommon_CarrierIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciCommon_CarrierIndicator_Type |  |
| Comment | Carrier indicator field (CIF) of DCI formats 0_1 and 1_1 according to TS 38.212 and TS 38.213 clause 10.1 |  |
| None | Null_Type | no cell index to be indicated in Carrier indicator field |
| CellIndex | B3_Type | 3 bits cell index to be indicated in Carrier indicator field; applicable when the UE is configured with higher layer parameter CrossCarrierSchedulingConfig |

NR_DciCommon_BWPIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciCommon_BWPIndicator_Type |  |
| Comment | Bandwidth part indicator according to TS 38.212 Table 7.3.1.1.2-1; used to address RRC-configured BandwidthPart-Config; NOTE: in general the BWP configuration at the SS is static i.e. dedicated BWPs may be preconfigured in a test case preamble => a) BWP configuration at the SS is not always the same as at the UE and SS cannot determine the number of BWPs being configured at the UE from its BWP configuration. b) The number and order of BWPs may differ at SS and UE. c) The index used in the DCI's BWP indicator is not identical with the BWP-Id as the BWP indicator is the index in the UE's BWP array => in general the SS cannot determine the size and value of the DCI's BWP indicator but this need to be done in TTCN |  |
| Index | bitstring | 0, 1 or 2 bits |

NR_DciCommon_TpcCommand_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciCommon_TpcCommand_Type |  |
| Comment | TPC Command Field according to TS 38.213 Table 7.1.1-1 and Table 7.2.1-1 |  |
| Value | B2_Type | 2 bits; default value: '01'B (0 dB; accumulated TPC) |

NR_DciCommon_UL_SUL_Indicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciCommon_UL_SUL_Indicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.1/2: UL/SUL indicator |  |
| None | Null_Type | to be used when no SUL is configured |
| Value | B1_Type | UL/SUL indicator according to TS 38.212 Table 7.3.1.1.1-1 |

NR_DciCommon_VrbPrbMapping_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciCommon_VrbPrbMapping_Type |  |
| Comment | to specify how VRB-to-PRB mapping shall be controlled by DCI if applicable (see TS 38.212 Table 7.3.1.2.2-5 and where it is referred) |  |
| None | Null_Type | 0 bit, applicable for format 1_0 when only resource allocation type 0 is configured |
| Index | B1_Type | 1 bit, index in TS 38.212 Table 7.3.1.2.2-5 indicating non-interleaved or interleaved VRB-to-PRB mapping according to TS 38.211 clause 6.3.1.7 |

NR_DciCommon_TimeDomainResourceAssignment_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciCommon_TimeDomainResourceAssignment_Type |  |
| Comment | Common type definition for UL/DL Resource allocation in time domain according to TS 38.214 clause 5.1.2.1 and 6.1.2.1 |  |
| Index | bitstring | index of entry in SEQUENCE OF PUSCH/PDSCH-TimeDomainResourceAllocation provided e.g. by PUSCH/PDSCH-Config; number of bits in a particular DCI depends on the size of the SEQUENCE OF PUSCH/PDSCH-TimeDomainResourceAllocation (e.g. Time domain resource assignment is an empty bitstring (''B) when only one time-domain configuration is provided to the UE) NOTE: PDSCH/PUSCH-Config overrules list in PDSCH/PUSCH-ConfigCommon |

NR_DciFormat_X_1_SrsRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_X_1_SrsRequest_Type |  |
| Comment | TS 38.212 clause 7.3.1.2: SRS request |  |
| SingleUL | B2_Type | 2 bits: Index of the SRS resource set to be used according to TS 38.212 Table 7.3.1.1.2-24 |
| UL_SUL | B3_Type | 3 bits: Index of the SRS resource set to be used according to TS 38.212 Table 7.3.1.1.2-24 plus first bit to distinguish UL/SUL |

NR_DciCommon_DmrsSequenceInit_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciCommon_DmrsSequenceInit_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2 (format 0_1), clause 7.3.1.1.3 (format 0_2), clause 7.3.1.2.2 (format 1_1) and clause 7.3.1.2.3 (format 1_2): DMRS sequence initialization |  |
| None | Null_Type | 0 bit - for format 0_1 if transform precoder is enabled; - for format 0_2 if dmrs-SequenceInitializationDCI-0-2 is not configured or if transform precoder is enabled; - for format 1_2 if dmrs-SequenceInitializationDCI-1-2 is not configured or if transform precoder is enabled |
| Value | B1_Type | 1 bit else |

NR_DciCommon_PriorityIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciCommon_PriorityIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2 (format 0_1), clause 7.3.1.2.2 (format 1_1), clause 7.3.1.1.3 (format 0_2) and clause 7.3.1.2.3 (format 1_2): Priority indicator |  |
| None | Null_Type | 0 bit no Priority indicator, if higher layer parameter priorityIndicatorDCI-0-1/priorityIndicatorDCI-1-1/priorityIndicatorDCI-0-2/priorityIndicatorDCI-1-2 is not configured |
| Value | B1_Type | 1 bit as defined in Clause 9 in TS 38.213. |

NR_DciFormat_X_1_MinimumApplicableSchedulingOffset_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_X_1_MinimumApplicableSchedulingOffset_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2 (format 0_1) and clause 7.3.1.2.2 (format 1_1): Minimum applicable scheduling offset indicator |  |
| None | Null_Type | 0 bit no Minimum applicable scheduling offset indicator |
| Value | B1_Type | 1 bit if minimumSchedulingOffsetK2(format 0_1)/minimumSchedulingOffsetK0((format 1_1)) is configured. Set according to TS 38.212 according to TS 38.212 Table 7.3.1.1.2-33. |

NR_DciFormat_X_1_SCellDormancyIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_X_1_SCellDormancyIndication_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2 (format 0_1) and clause 7.3.1.2.2 (format 1_1): SCell dormancy indication |  |
| None | Null_Type | 0 bit no SCell dormancy indication |
| Value | bitstring | 1, 2, 3, 4 or 5 bits: determined according to dormancyGroupWithinActiveTime parameter |

NR_DciFormat_X_0_ChannelAccessCPext_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_X_0_ChannelAccessCPext_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.1 and clause 7.3.1.2.1: ChannelAccess-CPext |  |
| None | Null_Type | 0 bit no ChannelAccess-CPext |
| Value | B2_Type | 2 bits indicating combinations of channel access type and CP extension as defined in TS 38.212 Table 7.3.1.1.1-4 or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access. |

NR_DciFormat_X_2_CarrierIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_X_2_CarrierIndicator_Type |  |
| Comment | Carrier indicator field (CIF) of DCI formats 0_2 and 1_2 according to TS 38.212 and TS 38.213 clause 10.1 |  |
| None | Null_Type | no cell index to be indicated in Carrier indicator field |
| CellIndex | bitstring | 1, 2 or 3 bits cell index to be indicated in carrierIndicatorSize_r16.carrierIndicatorSizeDCI_1_2_r16/carrierIndicatorSize_r16.carrierIndicatorSizeDCI_0_2_r16 |

NR_DciFormat_X_2_SrsRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_X_2_SrsRequest_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.3 and clause 7.3.1.1.3: SRS Request |  |
| None | Null_Type | srs-RequestDCI-0-2-r16/srs-RequestDCI-1-2-r16 is not present |
| Value | bitstring | 1 or 2 or 3 as defined in 7.3.1.2.3/7.3.1.1.3 |

NR_DciCommon_SecondTpcCommand_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciCommon_SecondTpcCommand_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2 (Format 0_1) clause 7.3.1.1.3 (Format 0_2) clause 7.3.1.2.2 (Format 1_1) clause 7.3.1.2.3 (Format 1_2): Second TPC command for scheduled PUSCH |  |
| None | Null_Type | no second TPC command |
| SecondTpc | NR_DciCommon_TpcCommand_Type | 2 bits as defined in Clause 7.1.1 of TS 38.213 if higher layer parameter SecondTPCFieldDCI-0-X is configured |

NR_DciCommon_SrsOffsetIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciCommon_SrsOffsetIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2 (format 0_1), clause 7.3.1.1.3 (format 0_2), clause 7.3.1.2.2 (format 1_1) and clause 7.3.1.2.3 (format 1_2): SRS offset indicator depending on AvailableSlotOffset |  |
| None | Null_Type | no SRS offset indicator |
| Indicator | bitstring | According to Table 7.3.1.1.2-37 and Clause 6.2.1 of TS 38.214. Empty string if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperodic SRS resource set in the scheduled cell and the maximum number of entries of AvailableSlotOffset configured for all aperiodic SRS resource set(s) is 1 |

NR_DciCommon_PdcchMonitoringAdaptationIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciCommon_PdcchMonitoringAdaptationIndication_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2 (format 0_1), clause 7.3.1.2.2 (format 1_1), clause 7.3.1.1.3 (format 0_2) and clause 7.3.1.2.3 (format 1_2): PDCCH monitoring adaptation indication |  |
| None | Null_Type | PDCCH monitoring adaptation indication |
| Value | bitstring | 1 or 2 bits, depending on searchSpaceGroupIdList-r17 and pdcch-SkippingDurationList |

##### D.1.3.2.4.2	Resource_Allocation

Type definitions for resource allocation which do not correpond directly to DCI fields
but are used to configure how the SS maintains resource allocation for a given DCI

Resource_Allocation: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_ImcsValue_Type | integer (0..31) | Modulation and coding scheme index coding |
| NR_RedundancyVersion_Type | integer (0..255) | Redundancy Version (RV): - 2 bits if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1 - 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PUSCHs among all entries in pusch-TimeDomainAllocationListForMultiPUSCH (TS 38.212 clause 7.3.1.1.2). |

NR_ResourceAllocationType_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_ResourceAllocationType_Type |
| Comment | to specify the format of the resource allocation type being used for frequency domain resource assignment in DCI; NOTE 1: For DCI Format 0_0 and 1_0 only resourceAllocationType1 is applicable (TS 38.214 clause 5.1.2.2 and 6.1.2.2) NOTE 2: The SS needs to determine based on RRC configuration whether MSB of the frequency domain resource assignment needs to be used as discriminator for type 0/1 (see TS 38.212 clause 7.3.1.1.2 and clause 7.3.1.2.2 and PUSCH/PDSCH-Config.resourceAllocation) NOTE 3: The SS shall raise an error if the DCI configuration conflicts with the configuration given by PUSCH/PDSCH-Config.resourceAllocation |
| resourceAllocationType0 | resource allocation type 0 according to TS 38.214 clause 5.1.2.2.1 and 6.1.2.2.1: bitmap indicating the Resource Block Groups (RBGs) that are allocated; not applicable for DCI Format 0_0 and 1_0 |
| resourceAllocationType1 | resource allocation type 1 according to TS 38.214 clause 5.1.2.2.2 and 6.1.2.2.2: resource indication value (RIV) corresponding to a starting virtual resource block and a length in terms of contiguously allocated resource blocks |

NR_ModulationSchemePDSCH_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_ModulationSchemePDSCH_Type |
| Comment | Supported modulation schemes for PDSCH according to 38.211 Table 7.3.1.2-1 |
| qpsk |  |
| qam16 |  |
| qam64 |  |
| qam256 |  |

NR_FreqDomainSchedulExplicit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_FreqDomainSchedulExplicit_Type |  |  |
| Comment | type used for explicit DL scheduling; Nprb is the exact number of RBs whereas in NR_FreqDomainSchedulCommon_Type MaxRbCnt is the upper bound |  |  |
| FirstRbIndex | integer |  | index of the first resource block in frequency domain |
| Nprb | integer |  | number of resource blocks to be assigned |

NR_FreqDomainSchedulCommonDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_FreqDomainSchedulCommonDL_Type |  |  |
| Comment | common type to specify restrictions for frequency domain scheduling by a start index and a maximum range of RBs (similar to EUTRA, but for NR in general the frequency domain scheduling is not related to the whole frequency band but to a given band width part (BWP) |  |  |
| FirstRbIndex | integer |  | index of the first (virtual) resource block in frequency domain |
| MaxRbCnt | integer |  | maximum number of resource-blocks to be used for a transport block; SS shall not assigned more than the given resource blocks; FirstRbIndex + MaxRbCnt shall not exceed the total number of available resoource blocks in frequency domain; the SS shall raise an error otherwise |

NR_FreqDomainResourceAssignmentDL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_FreqDomainResourceAssignmentDL_Type |  |
| Comment |  |  |
| Automatic | NR_FreqDomainSchedulCommonDL_Type | The SS shall automatically do the resource assignment needed for a DL transmission based on TBS evaluation guideline given in Annex B.1 of 38.523-3 |
| Explicit | NR_FreqDomainSchedulExplicit_Type | Frequency domain resource assignment is given explicitly by TTCN; the SS needs to calculate the RIV (resource allocation type 1) or generate the bitmap (resource allocation type 0). In case of resource allocation type 0 the allocation shall also be in consecutive RBGs. NOTE: So far there is no requirement for signalling tests to use non-consecutive RBGs ((neither for LTE nor for NR) |

NR_RedundancyVersionList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_RedundancyVersionList_Type |
| Comment | There shall be as many entries in the list as re-transmissions are allowed; if there are not enough elements specified SS shall raise an error; In 4G at least in UL the array length corresponds to maxHARQ-Tx (i.e. up to 28 re-transmissions according to RRC ASN.1) |
| record of NR_RedundancyVersion_Type |  |

NR_TransportBlockSchedulingDL_Automatic_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_TransportBlockSchedulingDL_Automatic_Type |  |  |
| Comment | transport block information for a DL transmission and potential retransmission in automatic mode |  |  |
| TransportBlock1 | NR_ModulationSchemePDSCH_Type |  |  |
| TransportBlock2 | NR_ModulationSchemePDSCH_Type | opt | MCS for 2nd transport block (if any); 'omit' means that there is no 2nd transport block; presence corresponds to PDSCH-Config.maxNrofCodeWordsScheduledByDCI |
| RedundancyVersionList | NR_RedundancyVersionList_Type | opt | list of Redundancy versions to be used for DL transmission and possible retransmissions; not present, if the DL transmission does not make use of harq processing (e.g. paging); in automatic mode the same list of redundancy versions is used for both transport blocks; if there are not enough elements to achieve successful DL transmission, SS shall raise an error |

RetransmissionTiming_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RetransmissionTiming_Type |  |
| Comment | to specify the timing of potential retransmissions related to the initial transmission. Rules in case of necessary UL or DL retransmissions: - When a transmission is scheduled with TimingInfo=Now, then any retransmission of a previous transmission takes precedence over the new transmission - Re-transmissions take precedence over periodic UL grants - The SS shall raise an error indication when - a retransmission collides with another UL or DL transmisssion which is scheduled by TTCN with specific TimingInfo for the same slot as the retransmssion - a new DL transmission would take over an ongoing DL transmission - the retransmission is not possible at the given time for any other reason (e.g. due to slot format) |  |
| SlotOffset | integer | the kth retransmission shall be k * SlotOffset slots after the initial transmission: e.g. slots per subframe = N and initial transmission at subframeX and slotX => 1. retransmission at subframeX + (slotX + SlotOffset) / N and (slotX + SlotOffset) mod N 2. retransmission at subframeX + (slotX + 2*SlotOffset) / N and (slotX + 2*SlotOffset) mod N and so on |
| SubframeOffset | integer | the kth retransmission shall be k * SubframeOffset subframes after the initial transmission in the same slot of subframe as for the initial transmission: e.g. initial transmission at subframeX and slotX 1. retransmission at subframeX + SubframeOffset and slotX 2. retransmission at subframeX + 2*SubframeOffset and slotX and so on |
| AnyTime | Null_Type | the SS shall autonomuously determine the next possible Ocassion for the retransmission |

TransmissionTimingOffset_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TransmissionTimingOffset_Type |  |
| Comment | Timing information for retransmissions |  |
| None | Null_Type | initial transmission: no timing offset but timing info as according to common part of the ASP |
| Retransmission | RetransmissionTiming_Type | retransmission with timing offset relative to initial transmission |

NR_TransportBlockSingleTransmission_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_TransportBlockSingleTransmission_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.2.1 and 7.3.1.2.2: parameters for transmission (or re-transmission) of a single transport block; used for explicit mode of DL transmission and for UL grants |  |  |
| TimingOffset | TransmissionTimingOffset_Type |  | in general "None" in case of a new transmission (i.e. no timing offset) and "Retransmission" for any retransmission |
| ImcsValue | NR_ImcsValue_Type |  | Imcs value to be mapped to the Modulation and coding scheme field of DCI format 1_0 or 1_1 |
| RedundancyVersion | NR_RedundancyVersion_Type |  | Redundancy version for a single transmission or re-transmission |
| ToggleNDI | boolean |  | "true" for transmission of a new transport block, "false" for a re-transmission; the NDI (New data indicator) itself is maintained by the SS and therefore not provided as configuration parameter |

NR_TransportBlockRetransmissionList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_TransportBlockRetransmissionList_Type |
| Comment | list of (initial) transmission and potential retransmissions; used for explicit mode of DL transmission and for UL grants; in general the Imcs is the same for all (re-)transmissions and the NDI is not toggled for the retransmissions |
| record of NR_TransportBlockSingleTransmission_Type |  |

NR_TransportBlockSchedulingDL_Explicit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_TransportBlockSchedulingDL_Explicit_Type |  |  |
| Comment |  |  |  |
| TransportBlock1 | NR_TransportBlockRetransmissionList_Type |  | list of transmission and retransmissions for transport block 1 |
| TransportBlock2 | NR_TransportBlockRetransmissionList_Type | opt | 'omit' means that there is no 2nd transport block; Presence corresponds to PDSCH-Config.maxNrofCodeWordsScheduledByDCI |

NR_TransportBlockSchedulingDL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_TransportBlockSchedulingDL_Type |  |
| Comment |  |  |
| Automatic | NR_TransportBlockSchedulingDL_Automatic_Type |  |
| Explicit | NR_TransportBlockSchedulingDL_Explicit_Type |  |

NR_HarqProcessConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_HarqProcessConfig_Type |  |
| Comment | HARQ processes to be used automatically for UL grants or DL assignments |  |
| None | Null_Type | when there is no HARQ for the given DCI (paging) |
| Broadcast | Null_Type | when the broadcast process shall be used |
| AnyProcess | Null_Type | The SS may choose any process for scheduling of the UL/DL data transfer |
| SpecificSubset | NR_HarqProcessList_Type | only the HARQ processes of this list shall be used automatically, other processes are excluded from automatic assignments; nevertheless for DL any HARQ processes may be addressed explicitly by NR_DRB_DataPerSlot_DL_Type.HarqProcess |

##### D.1.3.2.4.3	PDSCH_Scheduling

NR_DciDlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciDlInfo_Type |  |  |
| Comment | scheduling for CCCH/DCCH/DTCH mapped to DL-SCH mapped to PDSCH; for all parameters: mandatory for initial configuration of an instance, omit means "keep as it is" afterwards; definition is applicable for DCI format 1_X |  |  |
| ResoureAssignment | NR_DciFormat_1_X_ResourceAssignment_Type | opt | resource assignment; to control setting of the following fields in DCI formats 1_X (TS 38.212 clause 7.3.1.2.X): Frequency domain resource assignment Time domain resource assignment Modulation and coding scheme New data indicator Redundancy version HARQ process number |
| VrbPrbMapping | NR_DciCommon_VrbPrbMapping_Type | opt | VRB-to-PRB mapping |
| Format | NR_DciFormat_1_X_SpecificInfo_Type | opt | DCI format and DCI format specific parameters |

NR_DciFormat_1_X_ResourceAssignment_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_1_X_ResourceAssignment_Type |  |  |
| Comment | Common definition to be used for resource scheduling in DL |  |  |
| ResourceAllocationType | NR_ResourceAllocationType_Type | opt | resource allocation type to be used for the frequency domain resource assignment |
| FreqDomain | NR_FreqDomainResourceAssignmentDL_Type | opt |  |
| TimeDomain | NR_DciCommon_TimeDomainResourceAssignment_Type | opt |  |
| TransportBlockScheduling | NR_TransportBlockSchedulingDL_Type | opt | information about MCS and RV for one or two transport blocks |
| HarqProcessConfig | NR_HarqProcessConfig_Type | opt | configures which HARQ processes the SS may use; corresponds to "HARQ process number" in TS 38.212 clause 7.3.1.2.1 and 7.3.1.2.2 |

NR_DciFormat_1_X_SpecificInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_SpecificInfo_Type |  |
| Comment |  |  |
| Format_1_0 | NR_DciFormat_1_0_SpecificInfo_Type |  |
| Format_1_0_P_RNTI | NR_DciFormat_1_0_P_RNTI_SpecificInfo_Type |  |
| Format_1_0_SI_RNTI | NR_DciFormat_1_0_SI_RNTI_SpecificInfo_Type |  |
| Format_1_0_RA_RNTI | NR_DciFormat_1_0_RA_RNTI_SpecificInfo_Type |  |
| Format_1_1 | NR_DciFormat_1_1_SpecificInfo_Type |  |
| Format_1_2 | NR_DciFormat_1_2_SpecificInfo_Type |  |

NR_DciFormat_1_0_SpecificInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_1_0_SpecificInfo_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.2.1: scheduling of PDSCH in one DL cell; default parameters according to TS 38.508-1 clause 4.3.6.1.2.1 |  |  |
| DAI | NR_DciFormat_1_0_DAI_Type | opt | downlink assignment index |
| TpcCommandPucch | NR_DciCommon_TpcCommand_Type | opt | TPC command for scheduled PUCCH; to be set to 1 per default (0 dB; accumulated TPC) |
| PucchResourceIndicator | NR_DciFormat_1_X_PucchResourceIndicator_Type | opt | PUCCH resource indicator |
| PdschHarqTimingIndicator | NR_DciFormat_1_X_PdschHarqTimingIndicator_Type | opt | PDSCH-to-HARQ_feedback timing indicator |
| ChannelAccessCPext | NR_DciFormat_X_0_ChannelAccessCPext_Type | opt | ChannelAccess-CPext |

NR_DciFormat_1_0_P_RNTI_SpecificInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_1_0_P_RNTI_SpecificInfo_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.2.1: scheduling of PDSCH with DCI scrambled by P-RNTI; default parameters according to TS 38.508-1 clause 4.3.6.1.2.1 |  |  |
| ShortMessageIndicator | B2_Type | opt | Short Message Indicator according to TS 38.212 Table 7.3.1.2.1-1 |
| ShortMessages | B8_Type | opt | Short Messages according to TS 38.331 Table 6.5-1 |
| TbScaling | B2_Type | opt | Scaling factor according to TS 38.214 Table 5.1.3.2-2 |
| TrsAvailabilityIndication | NR_DciFormat_1_0_P_RNTI_TrsAvailabilityIndication_Type | opt | TRS availability indication |

NR_DciFormat_1_0_SI_RNTI_SpecificInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_1_0_SI_RNTI_SpecificInfo_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.2.1: scheduling of PDSCH with DCI scrambled by SI-RNTI; default parameters according to TS 38.508-1 clause 4.3.6.1.2.1 |  |  |
| SystemInfoIndicator | B1_Type | opt | System information indicator according to TS 38.212 Table 7.3.1.2.1-2 |

NR_DciFormat_1_0_RA_RNTI_SpecificInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_1_0_RA_RNTI_SpecificInfo_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.2.1: scheduling of PDSCH with DCI scrambled by RA-RNTI/MSGB-RNTI; default parameters according to TS 38.508-1 clause 4.3.6.1.2.1. FFS whether this is applicable to MsgB-RNTI |  |  |
| TbScaling | B2_Type | opt | Scaling factor according to TS 38.214 Table 5.1.3.2-2 |
| LSBsOfSFN | NR_DciFormat_1_0_LSBsOfSFN_Type | opt | LSBs of SFN |

NR_DciFormat_1_1_SpecificInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_1_1_SpecificInfo_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.2.2: scheduling of PDSCH in one cell; default parameters according to TS 38.508-1 clause 4.3.6.1.2.2 For all fields: 'omit' means that the information shall not be contained in the DCI on PDCCH |  |  |
| CarrierIndicator | NR_DciCommon_CarrierIndicator_Type | opt | Carrier indicator - CIF value for Cross Carrier Scheduling; 'None' otherwise |
| BWPIndicator | NR_DciCommon_BWPIndicator_Type | opt | Bandwidth part indicator |
| PrbBundlingSizeIndicator | NR_DciFormat_1_X_PrbBundlingSizeIndicator_Type | opt | PRB bundling size indicator |
| RateMatchingIndicator | NR_DciFormat_1_X_RateMatchingIndicator_Type | opt | Rate matching indicator |
| ZP_CSI_RS_Trigger | NR_DciFormat_1_X_ZP_CSI_RS_Trigger_Type | opt | ZP CSI-RS trigger |
| DAI | NR_DciFormat_1_1_DAI_Type | opt | downlink assignment index |
| TpcCommandPucch | NR_DciCommon_TpcCommand_Type | opt | TPC command for scheduled PUCCH; to be set to 1 per default (0 dB; accumulated TPC) |
| SecondTpcCommand | NR_DciFormat_1_X_SecondTpcCommand_Type | opt | Second TPC command for scheduled PUCCH |
| PucchResourceIndicator | NR_DciFormat_1_X_PucchResourceIndicator_Type | opt | PUCCH resource indicator |
| PdschHarqTimingIndicator | NR_DciFormat_1_X_PdschHarqTimingIndicator_Type | opt | PDSCH-to-HARQ_feedback timing indicator |
| OneShotHarqAckRequest | NR_DciFormat_1_X_OneShotHarqAckRequest_Type | opt | One-shot HARQ-ACK request |
| EnhancedType3CodebookIndicator | NR_DciFormat_1_X_EnhancedType3CodebookIndicator_Type | opt | Enhanced Type 3 codebook indicator |
| PdschGroupIndex | NR_DciFormat_1_1_PdschGroupIndex_Type | opt | PDSCH group index |
| NewFeedbackIndicator | NR_DciFormat_1_1_NewFeedbackIndicator_Type | opt | New feedback indicator |
| NumberRequestedPdschGroup | NR_DciFormat_1_1_NumberRequestedPdschGroup_Type | opt | Number of requested PDSCH group |
| HarqAckRetransmissionIndicator | NR_DciFormat_1_X_HarqAckRetransmissionIndicator_Type | opt | HARQ-ACK retransmission indicator |
| AntennaPorts | NR_DciFormat_1_1_AntennaPorts_Type | opt | Antenna port(s) |
| TCI | NR_DciFormat_1_1_TCI_Type | opt | Transmission configuration indication |
| SrsRequest | NR_DciFormat_X_1_SrsRequest_Type | opt | SRS request |
| SrsOffsetIndicator | NR_DciCommon_SrsOffsetIndicator_Type | opt | SRS offset indicator |
| CBGTI | NR_DciFormat_1_1_CBGTI_Type | opt | CBG transmission information (CBGTI) |
| CBGFI | NR_DciFormat_1_1_CBGFI_Type | opt | CBG flushing out information (CBGFI) |
| DmrsSequenceInit | NR_DciCommon_DmrsSequenceInit_Type | opt | DMRS sequence initialization |
| PriorityIndicator | NR_DciCommon_PriorityIndicator_Type | opt | Priority indicator |
| ChannelAccessCPext | NR_DciFormat_1_X_ChannelAccessCPext_Type | opt | ChannelAccess-CPext |
| MinimumApplicableSchedulingOffset | NR_DciFormat_X_1_MinimumApplicableSchedulingOffset_Type | opt | Minimum applicable scheduling offset indicator |
| SCellDormancyIndication | NR_DciFormat_X_1_SCellDormancyIndication_Type | opt | SCell dormancy indication |
| PdcchMonitoringAdaptationIndication | NR_DciCommon_PdcchMonitoringAdaptationIndication_Type | opt | PDCCH monitoring adaptation indication |
| PucchCellIndicator | NR_DciFormat_1_X_PucchCellIndicator_Type | opt | PUCCH Cell indicator |

NR_DciFormat_1_2_SpecificInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_1_2_SpecificInfo_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.2.3: scheduling of PDSCH in one cell; default parameters according to TS 38.508-1 clause 4.3.6.1.2.3. For all fields: 'omit' means that the information shall not be contained in the DCI on PDCCH |  |  |
| CarrierIndicator | NR_DciFormat_X_2_CarrierIndicator_Type | opt | Carrier indicator - CIF value for Cross Carrier Scheduling; 'None' otherwise |
| BWPIndicator | NR_DciCommon_BWPIndicator_Type | opt | Bandwidth part indicator |
| PrbBundlingSizeIndicator | NR_DciFormat_1_X_PrbBundlingSizeIndicator_Type | opt | PRB bundling size indicator |
| RateMatchingIndicator | NR_DciFormat_1_X_RateMatchingIndicator_Type | opt | Rate matching indicator |
| ZP_CSI_RS_Trigger | NR_DciFormat_1_X_ZP_CSI_RS_Trigger_Type | opt | ZP CSI-RS trigger |
| DAI | NR_DciFormat_1_2_DAI_Type | opt | downlink assignment index |
| TpcCommandPucch | NR_DciCommon_TpcCommand_Type | opt | TPC command for scheduled PUCCH; to be set to 1 per default (0 dB; accumulated TPC) |
| SecondTpcCommand | NR_DciFormat_1_X_SecondTpcCommand_Type | opt | Second TPC command for scheduled PUCCH |
| PucchResourceIndicator | NR_DciFormat_1_2_PucchResourceIndicator_Type | opt | PUCCH resource indicator |
| PdschHarqTimingIndicator | NR_DciFormat_1_X_PdschHarqTimingIndicator_Type | opt | PDSCH-to-HARQ_feedback timing indicator |
| OneShotHarqAckRequest | NR_DciFormat_1_X_OneShotHarqAckRequest_Type | opt | One-shot HARQ-ACK request |
| EnhancedType3CodebookIndicator | NR_DciFormat_1_X_EnhancedType3CodebookIndicator_Type | opt | Enhanced Type 3 codebook indicator |
| HarqAckRetransmissionIndicator | NR_DciFormat_1_X_HarqAckRetransmissionIndicator_Type | opt | HARQ-ACK retransmission indicator |
| AntennaPorts | NR_DciFormat_1_2_AntennaPorts_Type | opt | Antenna port(s) |
| TCI | NR_DciFormat_1_2_TCI_Type | opt | Transmission configuration indication |
| SrsRequest | NR_DciFormat_X_2_SrsRequest_Type | opt | SRS request |
| SrsOffsetIndicator | NR_DciCommon_SrsOffsetIndicator_Type | opt | SRS offset indicator |
| DmrsSequenceInit | NR_DciCommon_DmrsSequenceInit_Type | opt | DMRS sequence initialization |
| PriorityIndicator | NR_DciCommon_PriorityIndicator_Type | opt | Priority indicator |
| PdcchMonitoringAdaptationIndication | NR_DciCommon_PdcchMonitoringAdaptationIndication_Type | opt | PDCCH monitoring adaptation indication |
| ChannelAccessCPext | NR_DciFormat_1_X_ChannelAccessCPext_Type | opt | ChannelAccess-CPext |
| PucchCellIndicator | NR_DciFormat_1_X_PucchCellIndicator_Type | opt | PUCCH Cell indicator |

NR_DciFormat_1_X_PucchResourceIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_PucchResourceIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.1/2: PUCCH resource indicator |  |
| Value | B3_Type | 3 bits as defined in TS 38.213 clause 9.2.3 or reserved bits; index to "PUCCH-ResourceSet" according to 38.213 clause 9.2.3 as provided by PUCCH-Config |

NR_DciFormat_1_X_PdschHarqTimingIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_PdschHarqTimingIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.1/2: PDSCH-to-HARQ_feedback timing indicator (TS 38.213 clause 9.2.3) |  |
| Value | bitstring | Format 1_0: 3 bits, addresses one of {1, 2, 3, 4, 5, 6, 7, 8} according to TS 38.213 clause 9.2.3; Format 1_1: 0..3 bits, addresses entry in table provided by PUCCH-Config.dl-DataToUL-ACK |

NR_DciFormat_1_0_DAI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_0_DAI_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.1: DAI (downlink assignment indicator) |  |
| Index | DAI_B2_Type | 2 bits according to TS 38.213 clause 9.1.3 or reserved bits |

NR_DciFormat_1_0_LSBsOfSFN_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_0_LSBsOfSFN_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.1: LSBs of SFN |  |
| None | Null_Type | no LSBs of SFN |
| Automatic | Null_Type | LSBs of SFN automatically assigned by SS; 2 bits |

NR_DciFormat_1_X_PrbBundlingSizeIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_PrbBundlingSizeIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.X: PRB bundling size indicator |  |
| None | Null_Type | no PRB bundling |
| Dynamic | B1_Type | when PDSCH-Config.prb-BundlingType is set to 'dynamicBundling'; indicates which set of PRG values to be used (see 38.214 clause 5.1.2.3) |

NR_DciFormat_1_X_RateMatchingIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_RateMatchingIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.X: Rate matching indicator; rateMatchPatternGroup1/rateMatchPatternGroup1DCI-1-2 and rateMatchPatternGroup2/rateMatchPatternGroup2DCI-1-2 configured by PDSCH-Config |  |
| Bitmap | bitstring | 0, 1, or 2 bits: bitmap indicating rateMatchPatternGroup1/rateMatchPatternGroup1DCI-1-2 and/or rateMatchPatternGroup2/rateMatchPatternGroup2DCI-1-2 to be applied size depending on whether rateMatchPatternGroup1/rateMatchPatternGroup1DCI-1-2, rateMatchPatternGroup2/rateMatchPatternGroup2DCI-1-2 or both are configured by PDSCH-Config at the UE |

NR_DciFormat_1_X_ZP_CSI_RS_Trigger_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_ZP_CSI_RS_Trigger_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.X: ZP CSI-RS trigger |  |
| Index | bitstring | 0, 1, or 2 bits: Index in list of ZP-CSI-RS-Resource as configured by PDSCH-Config at the UE |

NR_DciFormat_1_1_DAI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_1_DAI_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2: DAI (downlink assignment indicator) - 6 bits if more than one serving cell are configured in the DL and the higher layer parameter nfi-TotalDAI-Included = enable - 4 bits if only one serving cell are configured in the DL and the higher layer parameter nfi-TotalDAI-Included = enable - 4 bits if more than one serving cell are configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook=enhancedDynamic-r16, and nfi-TotalDAI-Included is not configured - 4 bits if only one serving cell is configured in the DL, and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, and the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback - 2 bits if only one serving cell is configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook=enhancedDynamic-r16, and nfi-TotalDAI-Included is not configured, when the UE is not configured with CORESETPoolIndex or the value of CORESETPoolIndex is the same for all CORESETs if CORESETPoolIndex is provided or the UE is not configured with ACKNACKFeedbackMode = JointFeedback |  |
| None | Null_Type | no DAI |
| TwoBits | DAI_B2_Type |  |
| FourBits | DAI_B4_Type |  |
| SixBits | DAI_B6_Type |  |

NR_DciFormat_1_1_AntennaPorts_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_1_AntennaPorts_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2: Antenna ports as defined by TS 38.212 Tables 7.3.1.2.2-1..4 and Tables 7.3.1.2.2-1A/2A/3A/4A |  |
| Index | bitstring | bitstring presentation of index to TS 38.212 Tables 7.3.1.2.2-1..4 and Tables 7.3.1.2.2-1A/2A/3A/4A |

NR_DciFormat_1_1_TCI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_1_TCI_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2: Transmission configuration indication (TCI) |  |
| None | Null_Type | if ControlResourceSet.tci-PresentInDCI does not indicate 'enabled' |
| Value | B3_Type | if ControlResourceSet.tci-PresentInDCI=='enabled': TCI according to TS 38.213 clause 5.1.5/6 |

NR_DciFormat_1_1_CBGTI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_1_CBGTI_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2: CBG transmission information (CBGTI) |  |
| Bitmap | bitstring | 0, 2, 4, 6, or 8 bits according to PDSCH-CodeBlockGroupTransmission.maxCodeBlockGroupsPerTransportBlock and PDSCH-Config.maxNrofCodeWordsScheduledByDCI and TS 38.214 clause 5.1.7.2 |

NR_DciFormat_1_1_CBGFI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_1_CBGFI_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2: CBG flushing out information (CBGFI) |  |
| None | Null_Type | no CBGFI |
| Flag | B1_Type | CBGFI flag, if codeBlockGroupTransmission is configured in PDSCH-ServingCellConfig with codeBlockGroupFlushIndicator set to true |

NR_DciFormat_1_X_ChannelAccessCPext_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_ChannelAccessCPext_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2: ChannelAccess-CPext |  |
| None | Null_Type | no ChannelAccess-CPext |
| Value | bitstring | 1, 2, 3 or 4 bits. Determined with ul-AccessConfigListDCI-1-X or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 or Table 7.3.1.2.2-6A are configured by the higher layer parameter ul-AccessConfigListDCI-1-1. |

DAI_B1_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DAI_B1_Type |  |
| Comment |  |  |
| Index | B1_Type | 1 bit DAI value |
| Automatic | Null_Type | DAI automatically assigned by SS according to clause 7.1.2.1.3.3 |

DAI_B2_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DAI_B2_Type |  |
| Comment |  |  |
| Index | B2_Type | 2 bits DAI value |
| Automatic | Null_Type | DAI automatically assigned by SS according to clause 7.1.2.1.3.3 |

DAI_B4_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DAI_B4_Type |  |
| Comment |  |  |
| Index | B4_Type | 4 bits DAI value |
| Automatic | Null_Type | DAI automatically assigned by SS according to clause 7.1.2.1.3.3 |

DAI_B6_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DAI_B6_Type |  |
| Comment |  |  |
| Index | B6_Type | 6 bits DAI value |
| Automatic | Null_Type | DAI automatically assigned by SS according to clause 7.1.2.1.3.3 |

NR_DciFormat_1_X_OneShotHarqAckRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_OneShotHarqAckRequest_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2: One-shot HARQ-ACK request |  |
| None | Null_Type | no One-shot HARQ-ACK request |
| Value | B1_Type | 1 bit if higher layer parameter pdsch-HARQ-ACK-OneShotFeedback-r16 or pdsch-HARQ-ACK-OneShotFeedback-r16 or pdsch-HARQ-ACK-EnhType3List or pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2 or pdsch-HARQ-ACK-EnhType3DCI-1-2 is configured |

NR_DciFormat_1_1_PdschGroupIndex_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_1_PdschGroupIndex_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2: PDSCH group index |  |
| None | Null_Type | no PDSCH group index |
| Value | B1_Type | 1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16 |

NR_DciFormat_1_1_NewFeedbackIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_1_NewFeedbackIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2: New feedback indicator |  |
| None | Null_Type | no New feedback indicator |
| Value | bitstring | 1 or 2 bits if pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16 |

NR_DciFormat_1_1_NumberRequestedPdschGroup_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_1_NumberRequestedPdschGroup_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2: Number of requested PDSCH group(s) |  |
| None | Null_Type | no Number of requested PDSCH group(s) |
| Value | B1_Type | if the higher layer parameter pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16 |

NR_DciFormat_1_2_DAI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_2_DAI_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.3: DAI (downlink assignment indicator) 0 bit if the higher layer parameter downlinkAssignmentIndexDCI-1-2 is not configured; 4 bits if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI 4 bits if one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI. 1 or 2 bits if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, when the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided or the UE is not configured with ackNackFeedbackMode = joint, where the 1 bit or 2 bits are the counter DAI. |  |
| None | Null_Type | no DAI |
| OneBit | DAI_B1_Type |  |
| TwoBits | DAI_B2_Type |  |
| FourBits | DAI_B4_Type |  |

NR_DciFormat_1_2_PucchResourceIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_2_PucchResourceIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.3: PUCCH resource indicator |  |
| None | Null_Type | Numberofbits-forPUCCHresourceindicator-DCIFormat1_2 is not present |
| Value | bitstring | 1 or 2 or 3 bits determined by parameter Numberofbits-forPUCCHresourceindicator-DCIFormat1_2 |

NR_DciFormat_1_2_AntennaPorts_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_2_AntennaPorts_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.3: Antenna ports as defined by TS 38.212 Tables 7.3.1.2.2-6..26 and Tables 7.3.1.2.2-1A/2A/3A/4A |  |
| None | Null_Type | antennaPortsFieldPresenceDCI-1-2 is not configured |
| Index | bitstring | 2, 3, 4, or 5 bits, bitstring presentation of index to TS 38.212 Tables 7.3.1.2.2-6..26 and Tables 7.3.1.2.2-1A/2A/3A/4A |

NR_DciFormat_1_2_TCI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_2_TCI_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.3: Transmission configuration indication |  |
| None | Null_Type | tci-PresentDCI-1-2-r16 is not present |
| Value | bitstring | 1 or 2 or 3 bits determined by parameter tci-PresentDCI-1-2-r16 |

NR_DciFormat_1_0_P_RNTI_TrsAvailabilityIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_0_P_RNTI_TrsAvailabilityIndication_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.1: TRS availability indication depending on TRS-ResourceSetConfig |  |
| None | Null_Type | no TRS availability indication |
| Value | bitstring | 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the TRS-ResourceSetConfig if configured |

NR_DciFormat_1_X_SecondTpcCommand_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_SecondTpcCommand_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2 (Format 1_1) 7.3.1.2.3 (Format 1_2): Second TPC command for scheduled PUCCH |  |
| None | Null_Type | No second TPC command for scheduled PUCCH |
| Value | B2_Type | 2 bits; as defined in Clause 7.2.1 of TS 38.213 if higher layer parameter SecondTPCFieldDCI-1-X is configured |

NR_DciFormat_1_X_EnhancedType3CodebookIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_EnhancedType3CodebookIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2 (Format 1_1) 7.3.1.2.3 (Format 1_2): Enhanced Type 3 codebook indicator depending on pdsch-HARQ-ACK-EnhType3DCI-Field and pdsch-HARQ-ACK-EnhType3DCI-Field-1-2 |  |
| None | Null_Type | No enhanced Type 3 codebook indicator |
| Value | bitstring | According to TS 38.213 clause 7.3.1.2.2 (Format 1_1) 7.3.1.2.3 (Format 1_2) |

NR_DciFormat_1_X_HarqAckRetransmissionIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_HarqAckRetransmissionIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2 (Format 1_1) 7.3.1.2.3 (Format 1_2): HARQ-ACK retransmission indicator |  |
| None | Null_Type | No HARQ-ACK retransmission indicator |
| Value | B1_Type | 1 bit if higher layer parameter pdsch-HARQ-ACK-Retx or pdsch-HARQ-ACK-retxDCI-1-2 is configured. |

NR_DciFormat_1_X_PucchCellIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_1_X_PucchCellIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.2.2 (Format 1_1) 7.3.1.2.3 (Format 1_2): PUCCH Cell indicator depending on pucch-sSCellDyn |  |
| None | Null_Type | No PUCCH Cell indicator |
| Value | B1_Type | 1 bit if higher layer parameter pucch-sSCellDyn is configured |

##### D.1.3.2.4.4	PUSCH_Scheduling

NR_DciUlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciUlInfo_Type |  |  |
| Comment | scheduling for CCCH/DCCH/DTCH mapped to UL-SCH mapped to PUSCH; for all parameters: mandatory for initial configuration of an instance, omit means "keep as it is" afterwards |  |  |
| ResoureAssignment | NR_DciFormat_0_X_ResourceAssignment_Type | opt | resource assignment; to control setting of the following fields in DCI formats 0_X (TS 38.212 clause 7.3.1.1.X): Frequency domain resource assignment Time domain resource assignment Modulation and coding scheme New data indicator Redundancy version HARQ process number |
| PuschHoppingCtrl | NR_DciFormat_0_X_PuschHoppingCtrl_Type | opt | control of frequency hopping in DCI formats (TS 38.212 Table 7.3.1.1.1-3) |
| TpcCommandPusch | NR_DciCommon_TpcCommand_Type | opt | TPC command for scheduled PUSCH; to be set to 1 per default (0 dB; accumulated TPC) |
| UL_SUL_Indicator | NR_DciCommon_UL_SUL_Indicator_Type | opt | to control use of supplementary UL by DCI |
| Format | NR_DciFormat_0_X_SpecificInfo_Type | opt | DCI format and DCI format specific parameters |

NR_DciFormat_0_X_ResourceAssignment_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_0_X_ResourceAssignment_Type |  |  |
| Comment | Common definition to be used for resource scheduling in UL |  |  |
| ResourceAllocationType | NR_ResourceAllocationType_Type | opt | resource allocation type to be used for the frequency domain resource assignment |
| FreqDomain | NR_FreqDomainSchedulExplicit_Type | opt | explicit resource assignment: first RB, number of RBs; corresponds to "Frequency domain resource assignment" in TS 38.212 clause 7.3.1.1.1 and 7.3.1.1.2 |
| TimeDomain | NR_DciCommon_TimeDomainResourceAssignment_Type | opt | corresponds to "Time domain resource assignment" in TS 38.212 clause 7.3.1.1.1 and 7.3.1.1.2 |
| TransportBlockScheduling | NR_TransportBlockRetransmissionList_Type | opt | information about MCS and RV for transport block transmission and possible re-transmissions; corresponds to "Modulation and coding scheme", "New data indicator" and "Redundancy version" in TS 38.212 clause 7.3.1.1.1 and 7.3.1.1.2 |
| HarqProcessConfig | NR_HarqProcessConfig_Type | opt | HARQ process to be used for the scheduled UL data transfer corresponds to "HARQ process number" in TS 38.212 clause 7.3.1.1.1 and 7.3.1.1.2; NOTE: for 5G there is no synchronous HARQ anymore but HARQ is asynchronous even for UL (TS 38.300 clause 5.3.5.4; TS 38.212 clause 7.3.1: 4-bit HARQ process number included in all DCI Formats for UL) |

NR_DciFormat_0_X_SpecificInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_SpecificInfo_Type |  |
| Comment |  |  |
| Format_0_0 | NR_DciFormat_0_0_SpecificInfo_Type |  |
| Format_0_1 | NR_DciFormat_0_1_SpecificInfo_Type |  |
| Format_0_2 | NR_DciFormat_0_2_SpecificInfo_Type |  |

NR_DciFormat_0_0_SpecificInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_0_0_SpecificInfo_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.1: scheduling of PUSCH in one cell; default parameters according to TS 38.508-1 clause 4.3.6.1.1.1 |  |  |
| ChannelAccessCPext | NR_DciFormat_X_0_ChannelAccessCPext_Type | opt | ChannelAccess-CPext |

NR_DciFormat_0_1_SpecificInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_0_1_SpecificInfo_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.2: scheduling of PUSCH in one cell; default parameters according to TS 38.508-1 clause 4.3.6.1.1.2; for all parameters: mandatory for initial configuration of an instance, omit means "keep as it is" afterwards |  |  |
| CarrierIndicator | NR_DciCommon_CarrierIndicator_Type | opt | Carrier indicator - CIF value for Cross Carrier Scheduling; 'None' otherwise |
| DfiFlag | NR_DciFormat_0_1_DfiFlag_Type | opt | DFI flag |
| BWPIndicator | NR_DciCommon_BWPIndicator_Type | opt | Bandwidth part indicator |
| FirstDAI | NR_DciFormat_0_X_FirstDAI_Type | opt | 1st downlink assignment index |
| SecondDAI | NR_DciFormat_0_X_SecondDAI_Type | opt | 2nd downlink assignment index |
| ThirdDAI | NR_DciFormat_0_X_ThirdDAI_Type | opt | 3rd downlink assignment index |
| SecondTpcCommandPusch | NR_DciCommon_SecondTpcCommand_Type | opt | Second TPC command for scheduled PUSCH |
| SrsResourceSetIndicator | NR_DciFormat_0_X_SrsResourceSetIndicator_Type | opt | SRS resource set indicator |
| SrsResourceIndicator | NR_DciFormat_0_X_SrsResourceIndicator_Type | opt | SRS resource indicator |
| SecondSrsResourceIndicator | NR_DciFormat_0_X_SecondSrsResourceIndicator_Type | opt | Second SRS resource indicator |
| PrecodingInfo | NR_DciFormat_0_X_PrecodingInfo_Type | opt | Precoding information and number of layers |
| SecondPrecodingInfo | NR_DciFormat_0_X_SecondPrecodingInfo_Type | opt | Second Precoding information |
| AntennaPorts | NR_DciFormat_0_X_AntennaPorts_Type | opt | Antenna ports |
| SrsRequest | NR_DciFormat_X_1_SrsRequest_Type | opt | SRS request |
| SrsOffsetIndicator | NR_DciCommon_SrsOffsetIndicator_Type | opt | SRS offset indicator |
| CsiRequest | NR_DciFormat_0_X_CsiRequest_Type | opt | CSI request |
| CBGTI | NR_DciFormat_0_1_CBGTI_Type | opt | CBG transmission information (CBGTI) |
| PtrsDmrsAssociation | NR_DciFormat_0_X_PtrsDmrsAssociation_Type | opt | PTRS-DMRS association |
| SecondPtrsDmrsAssociation | NR_DciFormat_0_X_SecondPtrsDmrsAssociation_Type | opt | Second PTRS-DMRS association |
| BetaOffsetIndicator | NR_DciFormat_0_X_BetaOffsetIndicator_Type | opt | beta_offset indicator |
| DmrsSequenceInit | NR_DciCommon_DmrsSequenceInit_Type | opt | DMRS sequence initialization |
| UlschIndicator | NR_DciFormat_0_X_UlschIndicator_Type | opt | UL-SCH indicator |
| ChannelAccessCPextCAPC | NR_DciFormat_0_X_ChannelAccessCPextCAPC_Type | opt | ChannelAccess-CPext-CAPC |
| OpenLoopPowerControl | NR_DciFormat_0_X_OpenLoopPowerControl_Type | opt | Open-loop power control parameter set indication |
| PriorityIndicator | NR_DciCommon_PriorityIndicator_Type | opt | Priority indicator |
| InvalidSymbolPatternIndicator | NR_DciFormat_0_X_InvalidSymbolPatternIndicator_Type | opt | Invalid symbol pattern indicator |
| MinimumApplicableSchedulingOffset | NR_DciFormat_X_1_MinimumApplicableSchedulingOffset_Type | opt | Minimum applicable scheduling offset indicator |
| SCellDormancyIndication | NR_DciFormat_X_1_SCellDormancyIndication_Type | opt | SCell dormancy indication |
| SidelinkAssignmentIndex | NR_DciFormat_0_1_SidelinkAssignmentIndex_Type | opt | Sidelink assignment index |
| PdcchMonitoringAdaptationIndication | NR_DciCommon_PdcchMonitoringAdaptationIndication_Type | opt | PDCCH monitoring adaptation indication |

NR_DciFormat_0_2_SpecificInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_0_2_SpecificInfo_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.1.3: scheduling of PUSCH in one cell; default parameters according to TS 38.508-1 clause 4.3.6.1.1.3; for all parameters: mandatory for initial configuration of an instance, omit means "keep as it is" afterwards |  |  |
| CarrierIndicator | NR_DciFormat_X_2_CarrierIndicator_Type | opt | Carrier indicator - CIF value for Cross Carrier Scheduling; 'None' otherwise |
| BWPIndicator | NR_DciCommon_BWPIndicator_Type | opt | Bandwidth part indicator |
| FirstDAI | NR_DciFormat_0_X_FirstDAI_Type | opt | 1st downlink assignment index |
| SecondDAI | NR_DciFormat_0_X_SecondDAI_Type | opt | 2nd downlink assignment index |
| ThirdDAI | NR_DciFormat_0_X_ThirdDAI_Type | opt | 3rd downlink assignment index |
| SecondTpcCommandPusch | NR_DciCommon_SecondTpcCommand_Type | opt | Second TPC command for scheduled PUSCH |
| SrsResourceSetIndicator | NR_DciFormat_0_X_SrsResourceSetIndicator_Type | opt | SRS resource set indicator |
| SrsResourceIndicator | NR_DciFormat_0_X_SrsResourceIndicator_Type | opt | SRS resource indicator |
| SecondSrsResourceIndicator | NR_DciFormat_0_X_SecondSrsResourceIndicator_Type | opt | Second SRS resource indicator |
| PrecodingInfo | NR_DciFormat_0_X_PrecodingInfo_Type | opt | Precoding information and number of layers |
| SecondPrecodingInfo | NR_DciFormat_0_X_SecondPrecodingInfo_Type | opt | Second Precoding information |
| AntennaPorts | NR_DciFormat_0_X_AntennaPorts_Type | opt | Antenna ports |
| SrsRequest | NR_DciFormat_X_2_SrsRequest_Type | opt | SRS request |
| SrsOffsetIndicator | NR_DciCommon_SrsOffsetIndicator_Type | opt | SRS offset indicator |
| CsiRequest | NR_DciFormat_0_X_CsiRequest_Type | opt | CSI request |
| PtrsDmrsAssociation | NR_DciFormat_0_X_PtrsDmrsAssociation_Type | opt | PTRS-DMRS association |
| SecondPtrsDmrsAssociation | NR_DciFormat_0_X_SecondPtrsDmrsAssociation_Type | opt | Second PTRS-DMRS association |
| BetaOffsetIndicator | NR_DciFormat_0_X_BetaOffsetIndicator_Type | opt | beta_offset indicator |
| DmrsSequenceInit | NR_DciCommon_DmrsSequenceInit_Type | opt | DMRS sequence initialization |
| UlschIndicator | NR_DciFormat_0_X_UlschIndicator_Type | opt | UL-SCH indicator |
| ChannelAccessCPextCAPC | NR_DciFormat_0_X_ChannelAccessCPextCAPC_Type | opt | ChannelAccess-CPext-CAPC |
| OpenLoopPowerControl | NR_DciFormat_0_X_OpenLoopPowerControl_Type | opt | Open-loop power control parameter set indication |
| PriorityIndicator | NR_DciCommon_PriorityIndicator_Type | opt | Priority indicator |
| InvalidSymbolPatternIndicator | NR_DciFormat_0_X_InvalidSymbolPatternIndicator_Type | opt | Invalid symbol pattern indicator |
| PdcchMonitoringAdaptationIndication | NR_DciCommon_PdcchMonitoringAdaptationIndication_Type | opt | PDCCH monitoring adaptation indication |

NR_DciFormat_0_X_PuschHoppingCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_PuschHoppingCtrl_Type |  |
| Comment | TS 38.212 7.3.1.1.1 (format 0_0) and 7.3.1.1.2 (format 0_1) |  |
| None | Null_Type | DCI format 0_1 only: 0 bit if only resource allocation type 0 is configured or PUSCH-Config.frequencyHopping is not configured |
| Flag | B1_Type | 1 bit if resource allocation type 1 is configured (or type 0 and type 1) '1'B to indicate frequency hopping according to TS 38.214 clause 6.3 |

NR_DciFormat_0_X_FirstDAI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_FirstDAI_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: First DAI (downlink assignment indicator) depending on PhysicalCellGroupConfig.pdsch-HARQ-ACK-Codebook |  |
| SemiStaticCodebook | DAI_B1_Type | 1 bit according to TS 38.213 clause 9.1.2.2 for Type-1 HARQ-ACK (pdsch-HARQ-ACK-codebook=semi-static) |
| DynamicCodebook | DAI_B2_Type | 2 bits according to TS 38.213 Table 9.1.3-2 for Type-2 HARQ-ACK (pdsch-HARQ-ACK-codebook=dynamic or for enhanced dynamic HARQ-ACK codebook without UL-TotalDAI-Included configured) |
| EnhancedDynamicCodebook | DAI_B4_Type | Not applicable for DCI 0_2 - 4 bits for enhanced dynamic HARQ-ACK codebook and with UL-TotalDAI-Included = true. |

NR_DciFormat_0_X_SecondDAI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_SecondDAI_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: Second DAI (downlink assignment indicator) depending on PhysicalCellGroupConfig.pdsch-HARQ-ACK-Codebook |  |
| None | Null_Type | no 2nd DAI |
| DynamicCodebook | DAI_B2_Type | 2 bits according to TS 38.213 Table 9.1.3-2 for Type-2 HARQ-ACK (pdsch-HARQ-ACK-codebook=dynamic or for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and without UL-TotalDAI-Included configured) |
| EnhancedDynamicCodebook | DAI_B4_Type | Not applicable for DCI 0_2 - 4 bits for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and with UL-TotalDAI-Included = true |

NR_DciFormat_0_X_SrsResourceIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_SrsResourceIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: SRS resource indicator depending on PUSCH-Config.txConfig |  |
| NonCodeBook | bitstring | txConfig==NonCodeBook: bitstring of 0..3 bits according to TS 38.212 Tables 7.3.1.1.2-28/29/30/31 (according to TS 38.331 clause 6.3.2 "SRS-Config" there are at most 4 entries with usage==nonCodebook) |
| CodeBook | bitstring | txConfig==CodeBook: - bitstring of 0 or 1 bits according to TS 38.212 Table 7.3.1.1.2-32 - bitstring of 4 bits according to TS 38.212 Table 7.3.1.1.2-32A/32B if ul-FullPowerTransmission is configured (according to TS 38.331 clause 6.3.2 "SRS-Config" there are at most 2 entries with usage==codebook) |

NR_DciFormat_0_X_PrecodingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_PrecodingInfo_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: Precoding information and number of layers depending on PUSCH-Config.txConfig |  |
| NonCodeBook | Null_Type | txConfig==NonCodeBook: 0 bits |
| CodeBook | bitstring | txConfig==CodeBook: bitstring according to TS 38.212 Tables 7.3.1.1.2-2..5A; empty string for one antenna port only |

NR_DciFormat_0_X_AntennaPorts_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_AntennaPorts_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: Antenna ports depending (mainly) on - PUSCH-Config.transformPrecoder, - DMRS-UplinkConfig.dmrs-Type, - DMRS-UplinkConfig.maxLength, - dmrs-UplinkTransformPrecoding, - tp-pi2BPSK |  |
| Index | bitstring | bitstring presentation of index to Tables 7.3.1.1.2-6..23 |
| None | Null_Type | antennaPortsFieldPresenceDCI-0-2 is not present Not applicable for DCI format 0_1 |

NR_DciFormat_0_X_CsiRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_CsiRequest_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: CSI request |  |
| Index | bitstring | 0, 1, 2, 3, 4, 5, or 6 bits determined by CSI-MeasConfig.reportTriggerSize/CSI-MeasConfig.reportTriggerSizeDCI-0-2; TS 38.214 clause 5.2.1.5.1) |

NR_DciFormat_0_1_CBGTI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_1_CBGTI_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2: CBG transmission information (CBGTI) |  |
| Index | bitstring | 0, 2, 4, 6, or 8 bits determined by PUSCH-CodeBlockGroupTransmission.maxCodeBlockGroupsPerTransportBlock configured by PUSCH-ServingCellConfig |

NR_DciFormat_0_X_PtrsDmrsAssociation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_PtrsDmrsAssociation_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: PTRS-DMRS association |  |
| None | Null_Type | 0 bit PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1 |
| Value | B2_Type | 2 bits according to TS 38.212 Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 |

NR_DciFormat_0_X_BetaOffsetIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_BetaOffsetIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: beta_offset indicator |  |
| None | Null_Type | 0 bit if uci-on-PUSCH != dynamic (ConfiguredGrantConfig.uci-OnPUSCH |
| Value | B2_Type | 2 bits according to TS 38.213 Table 9.3-3 and Table 9.3-3A |

NR_DciFormat_0_X_UlschIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_UlschIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: UL-SCH indicator |  |
| None | Null_Type | 0 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1 Not applicable for DCI format 0_2 |
| Value | B1_Type | 1 bit: "1" indicates UL-SCH shall be transmitted on the PUSCH, "0" indicates UL-SCH shall not be transmitted on the PUSCH. Except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, a UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0" and CSI request of all zero(s). |

NR_DciFormat_0_1_DfiFlag_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_1_DfiFlag_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2: DFI flag indicator |  |
| None | Null_Type | 0 bit no DFI flag indicator |
| Flag | B1_Type | 1 bit: if the UE is configured to monitor DCI format 0_X with CRC scrambled by CS-RNTI and for operation in a cell with shared spectrum channel access. |

NR_DciFormat_0_X_ChannelAccessCPextCAPC_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_ChannelAccessCPextCAPC_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2 (Format 0_1) clause 7.3.1.1.3 (Format 0_2): ChannelAccess-CPext-CAPC |  |
| None | Null_Type | 0 bit: no ChannelAccess-CPext-CAPC |
| Value | bitstring | 1, 2, 3, 4, 5 or 6 bits. One or more entries from Table 7.3.1.1.2-35 or Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-X. |

NR_DciFormat_0_X_OpenLoopPowerControl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_OpenLoopPowerControl_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: Open-loop power control parameter set indication |  |
| None | Null_Type | 0 bit no Open-loop power control parameter set indication |
| Value | bitstring | 1 or 2 bits - 1 bit if SRS resource indicator is present in the DCI format 0_X; - 1 or 2 bits as determined by higher layer parameter olpc-ParameterSetDCI-0-X if SRS resource indicator is not present in the DCI format 0_X |

NR_DciFormat_0_X_InvalidSymbolPatternIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_InvalidSymbolPatternIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: Invalid symbol pattern indicator |  |
| None | Null_Type | 0 bit no Invalid symbol pattern indicator |
| Value | B1_Type | 1 bit as defined in Clause 6.1.2.1 in TS 38.214 |

NR_DciFormat_0_1_SidelinkAssignmentIndex_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_1_SidelinkAssignmentIndex_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2: Sidelink assignment index |  |
| None | Null_Type | 0 bit no Sidelink assignment index |
| SemiStaticCodebook | DAI_B1_Type | 1 bit according to TS 38.213 clause 16.5.1 for Type-1 HARQ-ACK (pdsch-HARQ-ACK-codebook=semi-static) and SS configured with NR_CellConfigInfo_Type.SidelinkScheduled |
| DynamicCodebook | DAI_B2_Type | 2 bits according to TS 38.213 clause 16.5.2 for Type-2 HARQ-ACK (pdsch-HARQ-ACK-codebook=dynamic) and SS configured with NR_CellConfigInfo_Type.SidelinkScheduled |

NR_DciFormat_0_X_ThirdDAI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_ThirdDAI_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: Third DAI (downlink assignment indicator) depending on PhysicalCellGroupConfig.pdsch-HARQ-ACK-Codebook |  |
| None | Null_Type | no 3rd DAI |
| SemiStaticCodebook | DAI_B1_Type | 1 bit for semi-static HARQ-ACK codebook for multicast if the higher layer parameter fdmed-Reception-Multicast is configured |
| DynamicCodebook | DAI_B2_Type | 2 bits for the dynamic HARQ-ACK codebook for multicast if the higher layer parameter pdsch-HARQ-ACK-Codebook-Multicast = dynamic is configured; |

NR_DciFormat_0_X_SrsResourceSetIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_SrsResourceSetIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2 (Format 0_1) clause 7.3.1.1.3 (Format 0_2): SRS resource set indicator depending on PUSCH-Config.txConfig and two SRS resource sets |  |
| None | Null_Type | No SRS resource set indicator |
| TwoSrsResourceSet | B2_Type | 2 bits according to Table 7.3.1.1.2-36 if - txConfig==nonCodeBook, and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with the usage of value 'nonCodeBook', or - txConfig==codebook, and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with usage of value 'codebook'; |

NR_DciFormat_0_X_SecondSrsResourceIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_SecondSrsResourceIndicator_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.X: Second SRS resource indicator depending on PUSCH-Config.txConfig and SRS resource set indicator field is present |  |
| None | Null_Type |  |
| NonCodeBook | bitstring | txConfig==nonCodebook and SRS resource set indicator field is present: according to TS 38.212 Tables 7.3.1.1.2-28/29/30/31 |
| CodeBook | bitstring | txConfig==codebook and SRS resource set indicator field is present: according to Tables 7.3.1.1.2-32, 7.3.1.1.2-32A and 7.3.1.1.2-32B |

NR_DciFormat_0_X_SecondPrecodingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_SecondPrecodingInfo_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2 (Format 0_1) clause 7.3.1.1.3 (Format 0_2): Second Precoding information: depending on PUSCH-Config.txConfig and SRS resource set indicator is present |  |
| None | Null_Type | No Second Second Precoding information |
| NonCodeBook | Null_Type | txConfig==nonCodebook and SRS resource set indicator field is present: 0 bits |
| Codebook | bitstring | txConfig==CodeBook: bitstring according to TS 38.212 Tables 7.3.1.1.2-2C..5A; empty string for one antenna port only |

NR_DciFormat_0_X_SecondPtrsDmrsAssociation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_0_X_SecondPtrsDmrsAssociation_Type |  |
| Comment | TS 38.212 clause 7.3.1.1.2 (Format 0_1) clause 7.3.1.1.3 (Format 0_2): Second PTRS-DMRS association |  |
| None | Null_Type | 0 bit - No Second PTRS-DMRS association |
| Value | B2_Type | 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRank>2. According to TS 38.212 Table 7.3.1.1.2-25 and 7.3.1.1.2-26. |

### D.1.3.3	MAC_Layer

Configuration for MAC procedures according to TS 38.321 clause 5 and related physical layer configuration

#### D.1.3.3.1	MAC_Layer_Common

MAC_Layer_Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_TimingAdvanceIndex_Type | integer (0..63) | acc. to TS 38.321, clause 6.1.3.4 "Timing Advance Command MAC CE" and TS 38.213 clause 4.2 "Transmission timing adjustments" |
| NR_TimingAdvance_Period_Type | integer (400, 600, 1020, 1530, 2040, 4090, 8190) | the values correspond to 80 % of TimeAlignmentTimer (acc. to TS 38.523-3, clause 6.3.2): ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240 rounded to nearest multiple of 10 |

NR_UplinkTimeAlignment_AutoSynch_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_UplinkTimeAlignment_AutoSynch_Type |  |  |
| Comment | Parameters for automatic synchronisation of UL time alignment; The SS shall periodically transmit TA MAC control elements according to 38.321 clause 6.1.3.4 with - TAG-Id=0 for the SpCell - TAG-Id as configured for an SCell the transmission shall be continuously until being stopped |  |  |
| TimingAdvance | NR_TimingAdvanceIndex_Type |  | amount of timing adjustment that MAC entity has to apply |
| TA_Period | NR_TimingAdvance_Period_Type |  | time period after which TA MAC control elements need to be automatically transmitted |

NR_UplinkTimeAlignment_Synch_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_UplinkTimeAlignment_Synch_Type |  |
| Comment | Configuration of Time Alignment of the UL |  |
| None | Null_Type | no PUCCH Synchronisation applied |
| Auto | NR_UplinkTimeAlignment_AutoSynch_Type | SS automatically maintains PUCCH synchronization at UE |

#### D.1.3.3.2	Random_Access_Procedure

NR_RachProcedureConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RachProcedureConfig_Type |  |  |
| Comment | parameters to control the random access procedure; TS 38.321, clause 5.1. When SUL is configured, SS shall monitor UE RACH transmissions on both uplink and supplementary uplink carriers. NOTE: RACH-ConfigCommon and RACH-ConfigDedicated are contained in NR_UplinkBWP_Type already (RACH-ConfigCommon as part of BWP-UplinkCommon). When supplementary uplink is configured, RACH-ConfigCommon and RACH-ConfigDedicated are contained in the fields Uplink and SupplementaryUplink. |  |  |
| RachProcedureList | NR_RachProcedureList_Type | opt | in normal cases there is one element which is used for any RA procedure |

NR_RachProcedureList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_RachProcedureList_Type |
| Comment | to simulate RACH procedure with one or more than one attempt by the UE: There is one element in the list per PRACH Preamble attempt |
| record of NR_RachProcedure_Type |  |

NR_RachProcedure_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RachProcedure_Type |  |  |
| Comment | for a 2Step Rach only MsgB is present. MsgB + ContentionResolution are present for 2step->4Step Rach fallback with Msg3 transmission by UE after MsgB. RandomAccessResponse + ContentionResolution are present for 4 Step RACH procedure |  |  |
| MsgB | NR_2StepMsgB_Config_Type | opt | configures how the SS shall react on a PRACH Preamble and MsgA attempt, in general: - a MAC subheader and fallbackRAR; - a MAC subheader and successRAR; - a MAC subheader and MAC SDU for CCCH or DCCH; - an Absolute Timing advance MCE; - BackoffIndicator - no response at all |
| RandomAccessResponse | NR_RandomAccessResponseConfig_Type | opt | configures how the SS shall react on a PRACH Preamble attempt, in general: - RAR with RAPID matching the RAPID of the UE's PRACH Preamble - RAR with RAPID not matching the RAPID of the UE's PRACH Preamble - BackoffIndicator - no response at all |
| ContentionResolution | NR_ContentionResolutionCtrl_Type | opt | Random Access Procedure may be 1. Contention free (Non-contention based) => no contention resolution 2. Contention based (see TS 38.321 clause 5.1.5): 2a) C-RNTI based: Msg3 contains MAC C-RNTI control element and in general contention resolution is done by assignment of an UL grant for this C-RNTI 2b) UE Contention Resolution Identity based: Msg3 contains RRC message to setup or restore RRC connection => contention resolution is done by sending of Msg4 with UE Contention Resolution Identity MAC CE |

##### D.1.3.3.2.1	Random_Access_Response

Random_Access_Response: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_RACH_TimingAdvance_Type | integer (0..3846) | 12 bit value corresponding to Timing Advance Command field of the Random Access Response (TS 38.321 clause 6.2.3): 0..3846 according to TS 38.213 clause 4.2) |
| NR_RAR_BackoffIndicator_Type | integer (0..15) | MAC subPDU for Backoff Indicator only according to TS 38.321 clause 6.1.5 |

NR_RAR_UplinkGrant_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RAR_UplinkGrant_Type |  |  |
| Comment | 27 bits according to TS 38.213 Table 8.2-1 and TS 38.321 Figure 6.2.3-1 |  |  |
| HoppingFlag | B1_Type |  | Hopping flag |
| Msg3FrequencyResourceAllocation | B14_Type |  | Msg3 PUSCH frequency resource allocation: RIV value as defined in TS 38.213 clause 8.2 and TS 38.214 clause 6.1.2.2.2 |
| Msg3TimeResourceAllocation | B4_Type |  | Msg3 PUSCH time resource allocation similar to the Time-domain resource assignment field of DCI format 0_0: index addressing an entry of the applicable PUSCH time domain resource allocation table as specified in TS 38.214 clause 6.1.2.1.1; the timing between the RAR and Msg3 is given by K2 (as in the time domain resource allocation table) and a delta value as according to Table 6.1.2.1.1-5 in TS 38.214. It is the responsibility of SS implementation to send the RAR in an appropriate DL slot so that the corresponding Msg3 transmission is scheduled for a valid UL slot; it is responsibility of test specification to provide a correct K2 value for the given numerology. |
| MCS | B4_Type |  | Modulation and Coding Scheme: first sixteen indices of the applicable MCS index table for PUSCH (TS 38.214 Table 6.1.4.1-1) |
| TPC_Command | B3_Type |  | TPC command for Msg3 PUSCH |
| CQI_Req | B1_Type |  | CQI request |

NR_TempC_RNTI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_TempC_RNTI_Type |  |
| Comment |  |  |
| SameAsC_RNTI | Null_Type | in the RA response SS shall use the same C-RNTI as configured in NR_CellConfigCommon_Type; this is useful for initial random access |
| Explicit | RNTI_Value_Type | in the RA response SS shall use different value as configured in NR_CellConfigCommon_Type; this can be used when the UE already is in RRC_CONNECTED to have a temporary C-RNTI different from the one used by the UE; NOTE: when the UE is not in RRC_CONNECTED there shall be no explicit temp. C-RNTI since then the UE would assume this value as C-RNTI |

NR_RAR_Payload_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RAR_Payload_Type |  |  |
| Comment | MAC payload for Random Access Response according to TS 38.321 clause 6.2.3 |  |  |
| TimingAdvance | NR_RACH_TimingAdvance_Type |  | timing advance: TS 38.321 clause 6.2.3 and TS 38.213 clause 4.2 NOTE: timing advance has impact not only on the RA procedure; SS in general needs to adjust its timing accordingly |
| UplinkGrant | NR_RAR_UplinkGrant_Type |  | initial UL grant |
| TempC_RNTI | NR_TempC_RNTI_Type |  | NOTE: In general for initial Random Access Procedure TempC_RNTI shall be 'SameAsC_RNTI' For Random Access Procedure in RRC_CONNECTED state the NW assigns a temporary C-RNTI which is replaced by the one stored at the UE; => TempC_RNTI may be 'SameAsC_RNTI' (in this case temp. C-RNTI and C-RNTI are equal what is not likely in a real network), or there is an explicit temp. C-RNTI what is used during RA procedure only (as in a real network) |

NR_RAR_RapIdOnly_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RAR_RapIdOnly_Type |  |  |
| Comment | MAC subPDU for RAPID only (acknowledgment for SI request) according to TS 38.321 clause 6.1.5 |  |  |
| RapId | RAR_RapIdCtrl_Type |  |  |

NR_RAR_RapIdAndPayload_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RAR_RapIdAndPayload_Type |  |  |
| Comment | MAC subPDU for RAPID and RAR payload according to TS 38.321 clause 6.1.5 |  |  |
| RapId | RAR_RapIdCtrl_Type |  |  |
| Payload | NR_RAR_Payload_Type |  |  |

NR_RAR_SubPdu_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RAR_SubPdu_Type |  |
| Comment | Random Access Response sub-PDU according to TS 38.321 clause 6.1.5 |  |
| BackoffIndicator | NR_RAR_BackoffIndicator_Type |  |
| RapIdOnly | NR_RAR_RapIdOnly_Type |  |
| RapIdAndPayload | NR_RAR_RapIdAndPayload_Type |  |

NR_RAR_SubPduList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_RAR_SubPduList_Type |
| Comment | list of MAC subPDUs; if a Backoff Indicator is included it has to be the first element (TS 38.321 clause 6.1.5) |
| record of NR_RAR_SubPdu_Type |  |

NR_RAR_MacPdu_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RAR_MacPdu_Type |  |  |
| Comment |  |  |  |
| SubPduList | NR_RAR_SubPduList_Type | opt | list of Backoff Indicator (optional) and random access responses; empty list if no RAR shall be sent at all (omit means "keep as it is" as usual) |
| CrcError | boolean | opt | if set, MAC PDU shall transmitted with CRC bits (0-3) being toggled |

NR_RandomAccessResponseConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RandomAccessResponseConfig_Type |  |  |
| Comment | configuration for Random Access Response mapped to DL-SCH mapped to PDSCH |  |  |
| SearchSpaceAndDci | NR_SearchSpaceDlDciAssignment_Type | opt | in general a RACH procedure is expected at the BWP currently being configured as active BWP at the SS; Type1-PDCCH common search space is used for scheduling of the Random Access Response (Msg2) |
| MacPdu | NR_RAR_MacPdu_Type | opt | MAC PDU to be sent automatically by the SS when there has been a RACH preamble |

##### D.1.3.3.2.2	Contention_Resolution

NR_ContentionResolutionCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ContentionResolutionCtrl_Type |  |
| Comment |  |  |
| None | Null_Type | no contention resolution: e.g. in case of contention free random access procedure or for special cases of contention based random access procedure |
| CRNTI_Based | NR_SearchSpaceUlDciAssignment_Type | contention resolution based on C-RNTI on PDCCH: The SS assigns UL grant on PDCCH; the UL grant is scrambled by C-RNTI and associated with the UE-specific search space |
| Msg4_Based | NR_RachProcedureMsg4_Type | contention resolution based on UE Contention Resolution Identity on DL-SCH |

NR_RachProcedureMsg4_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RachProcedureMsg4_Type |  |  |
| Comment | Msg4 of the RACH procedure in case of contention resolution based on UE Contention Resolution Identity |  |  |
| SearchSpaceAndDci | NR_SearchSpaceDlDciAssignment_Type | opt | DCI to be used for Msg4; the DCI is scrambled by the temporary C-RNTI and associated with the Type1-PDCCH common search space |
| ContentionResolutionId | NR_ContentionResolutionId_Type | opt | Contention Resolution Id contained in MAC PDU of Msg4 |
| RrcPdu | NR_RachProcedureMsg4RrcMsg_Type | opt | RRC message to be contained in Msg4 of the RACH procedure |
| CrcError | boolean | opt | if set, MAC PDU shall transmitted with CRC bits (0-3) being toggled |

NR_ContentionResolutionId_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ContentionResolutionId_Type |  |
| Comment |  |  |
| XorMask | B48_Type | When SS receives Contention Resolution ID from the UE, SS shall XOR it with the given mask and use this as Contention Resolution ID; this allows to get an unmatching Contention Resolution ID; in normal cases mask shall be set to tsc_ContentionResolutionId_Unchanged (i.e. the Contention Resolution ID remains unchanged) NOTE: In case of UL_CCCH1_Message the contention resolution id shall be cut down to the first 48 bits according to TS 38.321 clause 6.1.3.3 |
| None | Null_Type | MAC Contention Resolution Control Element is not contained in the MAC PDU sent out as response for Msg3 |

NR_RachProcedureMsg4RrcMsg_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RachProcedureMsg4RrcMsg_Type |  |
| Comment |  |  |
| RrcCcchMsg | octetstring | encoded RRC message for CCCH; LCID=000000 for CCCH |
| RrcDcchMsg | octetstring | encoded RRC message for DCCH; the SS shall - apply integrity protection, - add a PDCP header accordingly, - add an AM RLC header, - use LCID=000001 corresponding to SRB1 as logical channel id |
| None | Null_Type | Msg4 does not contain any RRC message, e.g. when RRC message is sent stand-alone in separate DL transmission |

##### D.1.3.3.2.3	TwoStepRACH

NR_2StepMsgB_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_2StepMsgB_Config_Type |  |
| Comment |  |  |
| None | Null_Type | No Msg transmitted |
| MsgbRNTI | NR_MsgB_Config_Type | The DCI is scrambled by MSGB-RNTI, C-RNTI MCE not included in MSGA |
| CRNTI | NR_MsgB_CRNTI_Config_Type | The DCI is scrambled by C-RNTI, C-RNTI MCE included in MSGA |

NR_MsgB_CRNTI_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MsgB_CRNTI_Config_Type |  |  |
| Comment | configuration for MsgB C-RNTI scrambled mapped to DL-SCH mapped to PDSCH |  |  |
| SearchSpaceAndDci | NR_SearchSpaceDlDciAssignment_Type | opt | in general a RACH procedure is expected at the BWP currently being configured as active BWP at the SS; Type1-PDCCH common search space is used for scheduling of the MsgB |
| MacPdu | NR_MsgB_CRNTI_MacPdu_Type | opt | MAC PDU to be sent automatically by the SS when there has been a RACH preamble |

NR_MsgB_CRNTI_MacPdu_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MsgB_CRNTI_MacPdu_Type |  |  |
| Comment |  |  |  |
| SubPduList | NR_MsgB_CRNTI_SubPduList_Type | opt | list of MsgB sub PDU's; empty list if no MsgB SubPDU shall be sent at all (omit means "keep as it is" as usual), MAC PDU with Padding only is sent. |
| CrcError | boolean | opt | if set, MAC PDU shall be transmitted with CRC bits (0-3) being toggled |

NR_MsgB_CRNTI_SubPdu_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_MsgB_CRNTI_SubPdu_Type |  |
| Comment | MsgB C-RNTI scrambled sub-PDU |  |
| AbsTA | NR_RACH_TimingAdvance_Type | Figure 6.1.3.4a-1 Absolute Timing Advance Command MAC CE. NOTE: timing advance has impact not only on the RA procedure; SS in general needs to adjust its timing accordingly |

NR_MsgB_CRNTI_SubPduList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_MsgB_CRNTI_SubPduList_Type |
| Comment | list of MAC subPDUs |
| record of NR_MsgB_CRNTI_SubPdu_Type |  |

NR_MsgB_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MsgB_Config_Type |  |  |
| Comment | configuration for MsgB mapped to DL-SCH mapped to PDSCH |  |  |
| SearchSpaceAndDci | NR_SearchSpaceDlDciAssignment_Type | opt | in general a RACH procedure is expected at the BWP currently being configured as active BWP at the SS; Type1-PDCCH common search space is used for scheduling of the MsgB |
| MacPdu | NR_MsgB_MacPdu_Type | opt | MAC PDU to be sent automatically by the SS when there has been a RACH preamble |

NR_MsgB_MacPdu_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MsgB_MacPdu_Type |  |  |
| Comment |  |  |  |
| SubPduList | NR_MsgB_SubPduList_Type | opt | list of Backoff Indicator (optional) and MsgB sub PDU's; empty list if no MsgB SubPDU shall be sent at all (omit means "keep as it is" as usual), MAC PDU with Padding only is sent. |
| CrcError | boolean | opt | if set, MAC PDU shall be transmitted with CRC bits (0-3) being toggled |

NR_MsgB_SubPdu_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_MsgB_SubPdu_Type |  |
| Comment | MsgB sub-PDU according to TS 38.321 clause 6.1.5a |  |
| BackoffIndicator | NR_RAR_BackoffIndicator_Type |  |
| FallbackRAR | NR_RAR_RapIdAndPayload_Type | Figure 6.1.5a-2: FallbackRAR MAC subheader is same as RAPId & Figure 6.2.3a-1: fallbackRAR Payload is same as RAR |
| SuccessRAR | NR_RAR_SuccessPayload_Type |  |
| RrcPdu | NR_RachProcedureMsg4RrcMsg_Type | RRC message to be contained in MsgB of the 2 Step RACH procedure |

NR_RAR_SuccessPayload_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RAR_SuccessPayload_Type |  |  |
| Comment | MAC subPDU Success RAR according to TS 38.321 Figure 6.2.3a-2 |  |  |
| ContentionResolutionId | NR_ContentionResolutionId_Type |  | Contention Resolution Id contained in MAC PDU of MsgB This field contains the UL CCCH SDU. If the UL CCCH SDU is longer than 48 bits, this field contains the first 48 bits of the UL CCCH SDU |
| ChAccess_CPext | B2_Type |  | The field is only present when the MSGB HARQ feedback is to be transmitted with shared spectrum channel access as specified in TS 37.213 [18]. Otherwise, the field is not present and R bits (set to 0) are present instead |
| TPC_Command | B3_Type |  | TPC command for Msg3 PUSCH |
| MsgB_HARQ_TimInd | B3_Type |  | The PDSCH-to-HARQ feedback timing indicator field for MSGB HARQ feedback as specified in TS 38.213 |
| PUCCH_ResInd | B4_Type |  | The PUCCH resource indicator for HARQ feedback for MSGB, as specified in TS 38.213 |
| TimingAdvance | NR_RACH_TimingAdvance_Type |  | timing advance: TS 38.321 clause 6.2.3 and TS 38.213 clause 4.2 NOTE: timing advance has impact not only on the RA procedure; SS in general needs to adjust its timing accordingly |
| C_RNTI | NR_TempC_RNTI_Type |  | NOTE: In general for C_RNTI shall be 'SameAsC_RNTI' stored in SS. explicit C-RNTI can be used for error cases |

NR_MsgB_SubPduList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_MsgB_SubPduList_Type |
| Comment | list of MAC subPDUs; if a Backoff Indicator is included it has to be the first element (TS 38.321 clause 6.1.5a) |
| record of NR_MsgB_SubPdu_Type |  |

### D.1.3.4	System_Information_Control

Primitive to configuration scheduling of system information on BCCH/BCH

System_Information_Control: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_BcchToPbchConfig_Type | Null_Type | place holder for BCCH mapped to BCH mapped to PBCH: MIB using fixed periodicity (80ms) and repetitions (80ms) according to TS 38.331 clause 5.2.1; the position of SS/PBCH blocks in frequency and time domain is specified in TS 38.211 clause 7.4.3 and TS 38.213 clause 4.1 the SS configuration for SS/PBCH blocks is defined by NR_SSB_Config_Type as part of physical layer configuration of a cell |

NR_Sib1Schedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_Sib1Schedul_Type |  |  |
| Comment | Scheduling of SIB1 (RMSI - Remaining Minimum SI, according to TS 38.300 clause 7.3): fixed scheduling in time domain according to TS 38.331 clause 5.2.1 (periodicity: 160ms according to TS 38.331 clause 5.2.1) |  |  |
| SearchSpaceAndDci | NR_SearchSpaceDlDciAssignment_Type | opt | in general SIB scheduling is assigned to the initial BWP's Type0-PDCCH common search space (searchSpaceZero); in principle SIB scheduling can be configured to happen simultaneously in more than one BWP (e.g. initial BWP and active BWP); SIB1 scheduling may be stopped by not assigning any BWP (AssignedBWPs being empty) |
| Periodicity | integer | opt | SIB1 repetition transmission period in milliseconds. According to TS 38.331 clause 5.2.1 the SIB1 repetition transmission period is 20 ms in case of SSB and CORESET multiplexing pattern 1; for SSB and CORESET multiplexing pattern 2/3 it is the same as the SSB period |
| SlotOffsetList | IntegerList_Type | opt | List of slot offsets for SIB1: For single beam configuration there is exactly one offset corresponding to the SSB index of the beam. For multiple beam configuration with N beams, SIB1 may be scheduled per beam so that there can be up-to N SIB1-transmission per repetition period. The offsets correspond to the SSB index of the respective beam as according to TS 38.213 clause 13: For a repetition period of 20ms the offset is relative to start of frames with even SFN (i.e. SFN mod 2 = 0) => in terms of TS 38.213 clause 13 for a given SSB index the slot offset is n0 when nO refers to a frame with even SFN n0 + (number of slots per frame) when nO refers to a frame with odd SFN A repetition period other than 20ms is not considered as long as there is no test requirement. NOTE: the SIB1 scheduling for a given offset shall match the Type0-PDCCH common search space configuration as configured in the SearchSpaceArray of the BWP |

NR_SingleSiSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SingleSiSchedul_Type |  |  |
| Comment | Scheduling for a single SI in its SI-window within a BWP (or even within several BWPs) |  |  |
| SearchSpaceAndDci | NR_SearchSpaceDlDciAssignment_Type | opt | in general SIB scheduling is assigned to the initial BWP's Type0A-PDCCH common search space; nevertheless in principle scheduling can be configured to happen simultaneously in more than one BWP (e.g. initial BWP and active BWP) |
| SlotOffset | integer | opt | slot-offset within the SI-window |

NR_OtherSiSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_OtherSiSchedul_Type |  |  |
| Comment | Scheduling of other SI ("other SI" according to TS 38.300 clause 7.3): specifies for a specific SI the scheduling and repetitions within its SI window |  |  |
| Periodicity | NR_SiPeriodicity_Type | opt |  |
| Window | record of NR_SingleSiSchedul_Type | opt | one or more repetitions within the si-window |

NR_OtherSiSchedulList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_OtherSiSchedulList_Type |
| Comment |  |
| record of NR_OtherSiSchedul_Type |  |

NR_PosSiSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PosSiSchedul_Type |  |  |
| Comment | Scheduling of other SI ("other SI" according to TS 38.300 clause 7.3) including posSIBs: specifies for a specific SI the scheduling and repetitions within its SI window |  |  |
| PosSI_Periodicity | NR_PosSI_Periodicity_Type | opt |  |
| Window | record of NR_SingleSiSchedul_Type | opt | one or more repetitions within the si-window |

NR_PosSiSchedulList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_PosSiSchedulList_Type |
| Comment |  |
| record of NR_PosSiSchedul_Type |  |

PosSiSchedulInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PosSiSchedulInfo_Type |  |  |
| Comment |  |  |  |
| OffsetToSI_Used | boolean | opt | If true: indicates that all the SI messages containing posSIBs are scheduled with an offset of 8 radio frames compared to SI messages provided in NR_AllOtherSiSchedul_Type. SiList and NR_AllOtherSiSchedul_Type.SegmentedSiList. If false: the SI messages containing posSIBs are scheduled after the other SI messages provided in NR_AllOtherSiSchedul_Type.SiList and NR_AllOtherSiSchedul_Type.SegmentedSiList. Corresponding to ASN.1 PosSchedulingInfo-r16.offsetToSI-Used-r16. |
| PosSiList | NR_PosSiSchedulList_Type | opt | list of scheduling info for the posSIs containing one or more posSIBs |

NR_AllOtherSiSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_AllOtherSiSchedul_Type |  |  |
| Comment | Scheduling of (all) other SI (i.e. SIB2 and above according to TS 38.300 clause 7.3) |  |  |
| WindowLength | NR_SiWindowLength_Type | opt | to calculate start of each SI window according to TS 38.331 clause 5.2.2.3.2 |
| SiList | NR_OtherSiSchedulList_Type | opt | list of scheduling info for the SIs containing one or more SIBs |
| SegmentedSiList | NR_OtherSiSchedulList_Type | opt | list of scheduling info for segmented SIs (e.g. SI containing SIB7 or SIB8 or SIB12); corresponds to SegmentedSIs in NR_BcchInfo_Type: SS shall subsequently schedule the elements of the corresponding SegmentedSIs (NR_BcchInfo_Type); SegmentedSiList[i] provides scheduling info for BcchInfo_Type's SegmentedSIs[i] The kth element of SegmentedSIs[i] is sent at the following slot number from T0: SlotOffset + ((N+i)* WindowLength) + k*Periodicity with k : kth element in SegmentedSIs[i], i.e. SegmentedSIs[i][k] N : number of SI provided in SIs in NR_BcchInfo_Type T0: start of the modification period SlotOffset, Periodicity: scheduling info as given by SegmentedSiList[i] - in slots WindowLength: provided in NR_AllOtherSiSchedul_Type - in slots |
| PosSiSchedulInfo | PosSiSchedulInfo_Type | opt | list of scheduling info for SIs containing one or more posSIBs |

NR_BcchToPdschConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_BcchToPdschConfig_Type |  |  |
| Comment | configuration for BCCH mapped to DL-SCH mapped to PDSCH |  |  |
| Sib1Schedul | NR_Sib1Schedul_Type | opt | SIB1 scheduling |
| SiSchedul | NR_AllOtherSiSchedul_Type | opt | scheduling of other SI |

NR_SI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SI_List_Type |
| Comment | list of system information messages |
| record of BCCH_DL_SCH_Message |  |

NR_SegmentedSI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SegmentedSI_List_Type |
| Comment | Each element is a list of segments; used for segmented SIBs Used for SIB7/SIB8/SIB12 segmentation |
| record of NR_SI_List_Type |  |

NR_PosSI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PosSI_Type |  |
| Comment |  |  |
| PosSI | BCCH_DL_SCH_Message | None of the AssistanceDataElement in the posSIBs in the SI are segmented. |
| SegmentedPosSIs | NR_SI_List_Type | List of SI containing posSIBs containing segments of AssistanceDataElement, according to TS 37.355 clause 7.3. SS shall schedule one element of the list (SegmentedPosSIs) in the si-window; PosSiSchedulInfo[i] provides scheduling info for PosSIs[i].SegmentedPosSIs |

NR_PosSI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_PosSI_List_Type |
| Comment | Each element is an SI including posSIBs or a list of SI containing posSIBs containing segments of AssistanceDataElement. |
| record of NR_PosSI_Type |  |

NR_BcchInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_BcchInfo_Type |  |  |
| Comment | Configuration of system information message contents to be scheduled at the SS |  |  |
| MIB | BCCH_BCH_Message | opt | TS 38.331, clause 6.2.1 BCCH-BCH-Message and clause 6.2.2 MIB; NOTE: the system frame number included in MIB needs to be handled and maintained by the system simulator; that means that the system frame number being setup by TTCN shall be ignored and overwritten by the SS |
| SIB1 | BCCH_DL_SCH_Message | opt | TS 38.331, clause 6.2.1 BCCH-DL-SCH-Message and clause 6.2.2 SIB1 |
| SIs | NR_SI_List_Type | opt | list of SIs corresponding to SiList of NR_AllOtherSiSchedul_Type (i.e. element i of NR_AllOtherSiSchedul_Type's SiList specifies the scheduling for SIs[i]) |
| SegmentedSIs | NR_SegmentedSI_List_Type | opt | list of SIs containing segmented SIBs; corresponds to SegmentedSiList in NR_AllOtherSiSchedul_Type |
| PosSIs | NR_PosSI_List_Type | opt | list of SIs containing posSIBs corresponding to PosSiSchedulInfo of NR_AllOtherSiSchedul_Type (i.e. element i of NR_AllOtherSiSchedul_Type's PosSiSchedulInfo specifies the scheduling for posSIs[i]) |

NR_BcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_BcchConfig_Type |  |  |
| Comment | Configuration of system information scheduling and content at the SS |  |  |
| Pbch | NR_BcchToPbchConfig_Type | opt |  |
| Pdsch | NR_BcchToPdschConfig_Type | opt |  |
| BcchInfo | NR_BcchInfo_Type | opt |  |

### D.1.3.5	Paging_Control

Primitive to configuration PCCH/PCH

NR_PcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PcchConfig_Type |  |  |
| Comment | configuration for PCCH mapped to PCH mapped to PDSCH |  |  |
| SearchSpaceAndDci | NR_SearchSpaceDlDciAssignment_Type | opt | in general Paging happens at the BWP currently being configured as active BWP at the SS and Type2-PDCCH common search space is used for scheduling; NOTE 1: there is no use case to schedule Paging simultaneously in multiple BWPs NOTE 2: the DCI may or may not carry a short message but it in any case it shall indicate presence of the paging (to send only a short message the "DciTrigger" primitive shall be used) |

### D.1.3.6	CCCH_DCCH_DTCH_Configuration

NR_DcchDtchConfigDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DcchDtchConfigDL_Type |  |  |
| Comment | configuration for DCCH/DTCH mapped to DL-SCH mapped to DL-SCH mapped to PDSCH; NOTE: in contrast to EUTRA the configuration of HARQ processes to be used is done as part of DCI configuration |  |  |
| SearchSpaceAndDci | NR_SearchSpaceDlDciAssignment_Type | opt | in general DCCH/DTCH transmissions happen at the BWP currently being the active BWP at the SS and the UE specific search space is used for scheduling; DCI configuration for Msg2 of the RACH procedure is done as part of the RACH procedure configuration (NR_RandomAccessResponseConfig_Type) |

NR_DcchDtchConfigUL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DcchDtchConfigUL_Type |  |  |
| Comment | scheduling for DCCH/DTCH mapped to UL-SCH mapped to PUSCH |  |  |
| SearchSpaceAndDci | NR_SearchSpaceUlDciAssignment_Type | opt | in general DCCH/DTCH transmissions happen at the BWP currently being the active BWP at the SS and the UE specific search space is used for scheduling |
| PUCCH_Synch | NR_UplinkTimeAlignment_Synch_Type | opt | parameters for automatic control of timing advance |
| GrantConfig | UL_GrantConfig_Type | opt | configuration how UL grant allocation is done (as response to scheduling request, periodically, etc. |

NR_DrxCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DrxCtrl_Type |  |
| Comment | DRX configuration for connected mode (TS 38.321, clause 5.7) |  |
| None | Null_Type | DRX not configured |
| Config | NR_ASN1_DRX_Config_Type | DRX is configured as signalled to the UE |

NR_MeasGapCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_MeasGapCtrl_Type |  |
| Comment | support of measurement gap configuration |  |
| None | Null_Type | no measurement gap configuration |
| Config | NR_ASN1_MeasGapConfig_Type | measurement gap configuration acc. to TS 38.331, clause 5.5.2.9; NOTE: the release branch of MeasGapConfig in general is not used for configuration of the SS |

NR_DcchDtchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DcchDtchConfig_Type |  |  |
| Comment |  |  |  |
| DL | NR_DcchDtchConfigDL_Type | opt | Scheduling, parameters related to DCCH and DTCH in DL |
| UL | NR_DcchDtchConfigUL_Type | opt | Scheduling, parameters related to DCCH and DTCH in UL |
| DrxCtrl | NR_DrxCtrl_Type | opt | DRX configuration as sent to the UE (or 'None' when the UE does not support connected mode DRX) |
| MeasGapCtrl | NR_MeasGapCtrl_Type | opt | to tell the SS when no assignments/grants shall be assigned to the UE |

### D.1.3.7	Cell_Group_Configuration

Configuration of cell group(s) in terms of dual connectivity and carrier aggregation

Cell_Group_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_ServingCellIndex_Type | integer | corresponds to ASN.1 (v15.1.0) definitions SCellIndex (1..31) and ServCellIndex (0..maxNrofServingCells-1): According to ASN.1 "The value range is shared across the Cell Groups" and "the PCell of the Master Cell Group uses ID = 0" |

NR_ServingCellConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ServingCellConfig_Type |  |
| Comment | serving cell capabilities of a cell |  |
| SpCell | NR_SpCellConfig_Type | parameters specific for an SpCell and cell group configuration |
| SCell | NR_SCellConfig_Type | parameters specific for an SCell |
| None | Null_Type | the is no serving cell at all (e.g. neighbouring cell only) |

NR_SCellConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SCellConfig_Type |  |  |
| Comment | cell parameters specific for an SCell; NOTE: the corresponding SpCell can be derived from the SpCell's SCellList; further parameters may be added according to test requirements for CA test cases |  |  |
| ServingCellIndex | NR_ServingCellIndex_Type | opt |  |
| TAG_Id | TAG_Id | opt | Id of the Timing Advance Group the SCell belongs to (according to TS 38.321 clause 6.1.3.4 the SpCell has the TAG Identity 0); the SS shall use the given TAG_Id e.g. for automatic time alignment in UL (see NR_UplinkTimeAlignment_AutoSynch_Type) |

NR_SpCellConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SpCellConfig_Type |  |  |
| Comment | cell parameters specific for an SpCell (PCell of the MCG or the PSCell of the SCG) and additionall parameters of the cell group; further parameters may be added according to test requirements for CA test cases |  |  |
| ServingCellIndex | NR_ServingCellIndex_Type | opt |  |
| CellGroupConfig | NR_SpCell_CellGroupConfig_Type | opt | parameters of the cell group of which the cell is SpCell (PCell or PSCell): assigned to SpCell as in many test cases the cell group consists of the SpCell only and on the other hand every cell group has to have at least an SpCell |

NR_SpCell_CellGroupConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SpCell_CellGroupConfig_Type |  |  |
| Comment | Configuration of an NR cell group; NOTE 1: The type of cell group (MCG, SCG) may be derived from the CellGroupId (CellGroupId==0 => MCG, CellGroupId>0 => SCG) NOTE 2: Further cell group specific MAC and PHY parameters may be added corresponding to ASN.1 MAC-CellGroupConfig or may need to be added (but e.g. the DRX configuration is assigned to NR_CcchDcchDtchConfig_Type already i.e. there is no need to configure it here) |  |  |
| CellGroupId | CellGroupId | opt | 0 for MCG (i.e. EUTRA in EN-DC), 1 for SCG (NR in EN-DC); see comments to ASN.1 definition of CellGroupId (v15.1.0) |
| SCellList | NR_CellIdList_Type | opt | list of SCells belonging to the SpCell's cell group; shall be initialised as empty list |
| MAC_CellGroupConfig | NR_ASN1_MAC_CellGroupConfig_Type | opt | Cell group specific MAC parameters as sent to the UE in CellGroupConfig.MAC-CellGroupConfig |
| PhysicalCellGroupConfig | NR_ASN1_PhysicalCellGroupConfig_Type | opt | Cell group specific physical layer parameters as sent to the UE in CellGroupConfig.PhysicalCellGroupConfig |

### D.1.3.8	Sidelink_ResourceAllocationMode1

Configuration of sidelink resource allocation mode 1

NR_SidelinkScheduled_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SidelinkScheduled_Type |  |  |
| Comment | Configuration of sidelink esource allocation mode 1 for a cell; all fields are mandatory for the first configuration of an instance for modifications "omit" means "keep as it is" |  |  |
| SL_MAC_CellGroupConfig | SL_MAC_CellGroupConfig_Type | opt |  |
| SL_DCI_Config | SL_DCI_Config_Type | opt |  |

SL_MAC_CellGroupConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_MAC_CellGroupConfig_Type |  |  |
| Comment |  |  |  |
| SL_BSR_Config | BSR_Config | opt |  |
| SL_LogicalChannelConfig | SL_LogicalChannelConfig_r16 | opt |  |

SL_DCI_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_DCI_Config_Type |  |  |
| Comment |  |  |  |
| SearchSpaceAndDci | NR_SearchSpaceSlDciAssignment_Type | opt |  |
| GrantConfig | SL_GrantConfig_Type | opt |  |

NR_SearchSpaceSlDciAssignment_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SearchSpaceSlDciAssignment_Type |  |  |
| Comment |  |  |  |
| AssignedBWPs | NR_AssignedBWPs_Type | opt | BWP where given DCI shall be scheduled in given search space; AssignedBWPs shall specify exactly one BWP (ActiveBWP in general); the SS may raise an error otherwise |
| SearchSpaceType | NR_SearchSpaceType_Type | opt | search space to be used for sending of given DCI; when at the scheduled point in time of a UL grant transmission there is no such search space configured at the given BWP, the SS may raise an error |
| DciInfo | NR_DciSlInfo_Type | opt | DCI to be used |

NR_DciSlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciSlInfo_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.4.1: scheduling of NR PSCCH and NR PSSCH in one cell; default parameters according to TS 38.508-1 clause 4.3.6.1.4.1; for all parameters: mandatory for initial configuration of an instance, omit means "keep as it is" afterwards |  |  |
| ResourceAssignment | NR_DciFormat_3_0_ResourceAssignment_Type | opt |  |
| SlHarqViaUu | NR_DciFormat_3_0_SlHarqViaUu_Type | opt |  |
| Format | NR_DciFormat_3_0_SpecificInfo_Type | opt |  |

NR_DciFormat_3_0_ResourceAssignment_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_3_0_ResourceAssignment_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.4.1: common fields for resource scheduling of SL |  |  |
| ResourcePoolIndex | NR_DciFormat_3_0_ResourcePoolIndex | opt | Resource Pool Index |
| TimeGap | NR_DciFormat_3_0_TimeGap | opt | Time gap |
| HarqProcess | NR_DciFormat_3_0_HarqProcess | opt | HARQ process number |
| NDI | NR_DciFormat_3_0_NDI | opt | New data indicator |
| LowestSubchTx | NR_DciFormat_3_0_LowestSubchTx | opt | Lowest index of the subchannel allocation to the initial transmission |
| Sci1_A | NR_DciFormat_3_0_Sci1_A | opt | SCI format 1-A fields according to TS 38.212 clause 8.3.1.1 |
| CounterSAI | NR_DciFormat_3_0_CounterSAI | opt | Counter sidelink assignment index |

NR_DciFormat_3_0_ResourcePoolIndex

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_3_0_ResourcePoolIndex |  |
| Comment | TS 38.212 clause 7.3.1.4.1: index to one of the resourece pools in sl-TxPoolScheduling, if configured, and sl-DiscTxPoolScheduling, if configured |  |
| None | Null_Type |  |
| Index | bitstring |  |

NR_DciFormat_3_0_TimeGap

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_3_0_TimeGap |  |  |
| Comment | entry of sl-DCI-ToSL-Trans to be used |  |  |
| Index | B3_Type |  |  |

NR_DciFormat_3_0_HarqProcess

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_3_0_HarqProcess |  |
| Comment |  |  |
| HarqProcessCGtype2 | NR_DciFormat_3_0_HarqProcess_CGtype2 |  |
| Value | NR_HarqProcessConfig_Type |  |

NR_DciFormat_3_0_HarqProcess_CGtype2

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_3_0_HarqProcess_CGtype2 |  |
| Comment |  |  |
| Activate | B3_Type | set to '000'B for activation of configured grant type 2; |
| Deactivate | B3_Type | set to '111'B for deactivation of configured grant type 2 |

NR_DciFormat_3_0_NDI

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_3_0_NDI |  |
| Comment |  |  |
| ToggleNDI | boolean | "true" for transmission of a new transport block, "false" for a re-transmission; the NDI (New data indicator) itself is maintained by the SS and therefore not provided as configuration parameter |
| CG_Type2 | B1_Type | to be set to '0'B when activating/releasing SL configured grant Type 2. |

NR_DciFormat_3_0_LowestSubchTx

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_3_0_LowestSubchTx |  |  |
| Comment |  |  |  |
| Index | bitstring |  |  |

NR_DciFormat_3_0_SlHarqViaUu_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_3_0_SlHarqViaUu_Type |  |  |
| Comment |  |  |  |
| PfschToHarqTiming | NR_DciFormat_3_0_PfschToHarqTiming | opt |  |
| PucchResourceIndicator | NR_DciFormat_3_0_PucchResourceIndicator | opt |  |

NR_DciFormat_3_0_PfschToHarqTiming

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_3_0_PfschToHarqTiming |  |
| Comment |  |  |
| None | Null_Type |  |
| Index | B3_Type |  |

NR_DciFormat_3_0_PucchResourceIndicator

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_3_0_PucchResourceIndicator |  |  |
| Comment |  |  |  |
| Index | B3_Type |  |  |

NR_DciFormat_3_0_SpecificInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_3_0_SpecificInfo_Type |  |
| Comment |  |  |
| Format3_0_DG | NR_DciFormat_3_0_DG |  |
| Format3_0_CG | NR_DciFormat_3_0_CG |  |

NR_DciFormat_3_0_DG

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_3_0_DG |  |  |
| Comment | Configuration of dynamic grant |  |  |
| SL_RNTI | RNTI_Value |  |  |

NR_DciFormat_3_0_CG

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_3_0_CG |  |  |
| Comment | Configuration of configured grant |  |  |
| SL_CS_RNTI | RNTI_Value |  |  |
| ConfigIndex | NR_DciFormat_3_0_ConfigIndex | opt | Configuration index |

NR_DciFormat_3_0_ConfigIndex

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_3_0_ConfigIndex |  |  |
| Comment |  |  |  |
| Index | B3_Type |  | as defined in TS 38.214 clause 8.1.2; index to entry of sl-ConfiguredGrantConfigList to be used |

NR_DciFormat_3_0_CounterSAI

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_3_0_CounterSAI |  |  |
| Comment |  |  |  |
| Index | B2_Type |  |  |

NR_DciFormat_3_0_Sci1_A

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_3_0_Sci1_A |  |  |
| Comment |  |  |  |
| FreqResourceAssignment | NR_SL_FreqRessourceAssignment_Type | opt |  |
| TimeResourceAssignment | integer | opt |  |

SL_GrantConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SL_GrantConfig_Type |  |
| Comment |  |  |
| OnSL_BSR_Reception | Null_Type | SS transmits SL Grant as configured by NR_DciSlInfo_Type at every reception of SL_BSR or sl-SchedulingRequestId-r16 when different than SchedulingRequestId; |
| Periodic | SL_PeriodicGrant_Type | SS transmits SL Grant as configured by NR_DciSlInfo_Type periodically; to be used in L2 tests; MAC tests testing Grants might set the period as infinite and num grant as 1 |
| PeriodicOnSR_Reception | SL_PeriodicGrant_Type | SS transmits SL Grant as configured by NR_DciSlInfo_Type periodically; the periodic grant transmission is started/restarted on reception of a SL_BSR from UE |
| Retransmission | Null_Type | Acc to 38.321, for retransmission of configured grant type 1 and type 2 - SL-CS-RNTI |
| None | Null_Type | disable any grant transmission |

SL_PeriodicGrant_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SL_PeriodicGrant_Type |  |  |
| Comment |  |  |  |
| Period | SLGrant_Period_Type |  | time period after which SL Grant need to be automatically transmitted or 'OnlyOnce' |
| NoOfRepetitions | TransmissionRepetition_Type |  | number of SL Grant repetitions to be automatically transmitted or continuous repetition |

SLGrant_Period_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SLGrant_Period_Type |  |
| Comment |  |  |
| OnlyOnce | Null_Type | grant is sent out only once; no period |
| Duration | integer (1..infinity) | duration of the grant period in number of slots |

NR_SL_FreqRessourceAssignment_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_FreqRessourceAssignment_Type |  |  |
| Comment | TS 38.214 clause 8.1.5 The NR-SS-UE shall automatically do the resource assignment needed for a transmission |  |  |
| FirstSubchannel | integer |  | The starting sub-channel index of the first resource |
| MaxSubchannels | integer |  | maximum number of sub-channels to be used |

## D.1.4	Cell_Power_Attenuation

NR_CellAttenuationConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CellAttenuationConfig_Type |  |  |
| Comment |  |  |  |
| CellId | NR_CellId_Type |  |  |
| Attenuation | NR_Attenuation_Type |  |  |
| TimingInfo | TimingInfo_Type | opt |  |

NR_CellAttenuationList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_CellAttenuationList_Type |
| Comment |  |
| record of NR_CellAttenuationConfig_Type |  |

## D.1.5	Radio_Bearer_Configuration

Radio Bearer Configuration: SRBs/DRBs
NOTE: Type definitions for PDCP configuration are in NR_PDCP_TypeDefs

NR_RlcBearerConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RlcBearerConfigInfo_Type |  |  |
| Comment | RLC bearer as defined in TS 37.340: RLC and MAC logical channel configuration of a radio bearer in one cell group |  |  |
| Rlc | NR_RLC_Configuration_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| LogicalChannelId | NR_LogicalChannelId_Type | opt | DRBs: DTCH-LogicalChannelIdentity as for rb-MappingInfo in DRB-ToAddModifyList; SRBs: for SRBs specified configurations acc. to 38.331 clause 9.2.1 shall be applied: SRB1: ul-LogicalChannel-Identity = dl-LogicalChannel-Identity = 1 SRB2: ul-LogicalChannel-Identity = dl-LogicalChannel-Identity = 2 SRB3: ul-LogicalChannel-Identity = dl-LogicalChannel-Identity = 3 for SRB0 being mapped to CCCH the SS shall apply - LCID=0 for DL according to TS 38.321 Table 6.2.1-1 - LCID=0, 52, 35 or 36 for UL according to TS 38.321 Table 6.2.1-2 depending on whether it is a CCCH1 or a CCCH message; For DRBs and SRBs - except SRB0: mandatory for initial configuration; omit means "keep as it is" For SRB0 LogicalChannelId is always omit and the SS shall apply the LCIDs according to TS 38.321 |
| Mac | NR_MAC_Configuration_Type | opt |  |
| DiscardULData | boolean | opt | if omitted: initial configuration: data is handed over to TTCN as usual re-configuration: "keep as it is" if set: true - SS shall discard any data in UL for this radio bearer false - (re)configuration back to normal mode NOTE: typically applicable for UM DRBs only |

NR_RlcBearerConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RlcBearerConfig_Type |  |
| Comment | configuration of RLC bearer below NR-PDCP |  |
| Config | NR_RlcBearerConfigInfo_Type | "normal" configuration: there is an RLC bearer configured for the cell which is linked to the PDCP being configured at the cell (the PDCP can be either 'RBTerminating' or 'Proxy') |
| None | Null_Type | No RLC bearer is configured at NR below the NR-PDCP: NR_PDCP_Configuration_Type shall be 'RBTerminating' with LinkToOtherCellGroup indicating cell with RLC bearer to be used |

NR_RadioBearerConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RadioBearerConfigInfo_Type |  |  |
| Comment |  |  |  |
| Sdap | SDAP_Configuration_Type | opt | omitted for EN-DC, otherwise mandatory for initial configuration; omit means "keep as it is" for SRBs: Sdap.None:=true |
| Pdcp | NR_PDCP_Configuration_Type | opt | for SRB0: "Pdcp.None:=true" mandatory for initial configuration; omit means "keep as it is" |
| RlcBearer | NR_RlcBearerConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

NR_RadioBearerConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RadioBearerConfig_Type |  |
| Comment |  |  |
| AddOrReconfigure | NR_RadioBearerConfigInfo_Type | add / re-configure RB |
| Release | Null_Type | release RB |

NR_RadioBearer_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RadioBearer_Type |  |  |
| Comment |  |  |  |
| Id | NR_RadioBearerId_Type |  | either for SRB or DRB |
| Config | NR_RadioBearerConfig_Type |  |  |

NR_RadioBearerList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_RadioBearerList_Type |
| Comment | array of SRBs and/or DRBs. TimingInfo: 'Now' in general; activation time may be used in special case for release and/or reconfiguration of one or several RBs; the following rules shall be considered: - release/Reconfiguration of an RB shall not be scheduled earlier than 5ms after a previous data transmission on this RB - subsequent release and reconfiguration(s) shall be scheduled with an interval of at least 5ms - a subsequent data transmission on an RB shall not be scheduled earlier than 5ms after the last reconfiguration of the RB the configuration shall be performed exactly at the given time ControlInfo : FollowOnFlag:=false (unless explicitly specified otherwise in TS 38.523-3 clause 7) |
| record of NR_RadioBearer_Type |  |

### D.1.5.1	RLC_Configuration

RLC configuration: radio bearer specific

RLC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_SS_RLC_TM_Type | Null_Type | TM to configure SRB0; no parameters to be defined |

NR_RLC_ACK_Prohibit_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_RLC_ACK_Prohibit_Type |
| Comment | As per 38.523-3 cl. 7.1.6.1 |
| Prohibit | cause SS RLC layer to stop any ACK transmission for UL PDUs received from UE |
| Continue | bring back the SS RLC in normal mode, where ACK/NACK are transmitted at polling |

NR_RLC_NotACK_NextRLC_PDU_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_RLC_NotACK_NextRLC_PDU_Type |
| Comment | As per 38.523-3 cl. 7.1.6.1 |
| Start | cause SS RLC layer not to ACK the next received RLC PDU; this is done regardless of whether the poll bit is set or not; Example [from UMTS]: when the UE gets new security information in a SECURITY MODE COMMAND the response (SECURITY MODE COMPLETE) sent by the UE is not acknowledged at the RLC level; this causes the UE to continue using the "old" security information |

NR_RLC_TransparentMode

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RLC_TransparentMode |  |
| Comment |  |  |
| Umd | SN_FieldLengthUM | SN-FieldLengthUM ::= ENUMERATED {size6, size12} TS 38.331 |
| Amd | SN_FieldLengthAM | SN-FieldLengthAM ::= ENUMERATED {size12, size18} TS 38.331 |

NR_RLC_TestModeInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RLC_TestModeInfo_Type |  |
| Comment |  |  |
| AckProhibit | NR_RLC_ACK_Prohibit_Type | valid only when the RLC is configured in AM |
| NotACK_NextRLC_PDU | NR_RLC_NotACK_NextRLC_PDU_Type | valid only when the RLC is configured in AM |
| TransparentMode | NR_RLC_TransparentMode |  |

NR_RLC_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RLC_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Info | NR_RLC_TestModeInfo_Type |  |

NR_SS_RLC_AM_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SS_RLC_AM_Type |  |  |
| Comment |  |  |  |
| Tx | NR_ASN1_UL_AM_RLC_Type | opt | the UE's UL setting to be used in SS's tx direction |
| Rx | NR_ASN1_DL_AM_RLC_Type | opt | the UE's DL setting to be used in SS's rx direction |

NR_SS_RLC_UM_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SS_RLC_UM_Type |  |  |
| Comment |  |  |  |
| Tx | NR_ASN1_UL_UM_RLC_Type | opt | the UE's UL setting to be used in SS's tx direction |
| Rx | NR_ASN1_DL_UM_RLC_Type | opt | the UE's DL setting to be used in SS's rx direction |

NR_RLC_RbConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RLC_RbConfig_Type |  |
| Comment |  |  |
| AM | NR_SS_RLC_AM_Type |  |
| UM | NR_SS_RLC_UM_Type |  |
| TM | NR_SS_RLC_TM_Type | normally SRB0 only; may be used for test purposes also |

NR_RLC_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_Configuration_Type |  |  |
| Comment |  |  |  |
| Rb | NR_RLC_RbConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | NR_RLC_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

### D.1.5.2	MAC_Configuration

MAC configuration: radio bearer specific configuration

MAC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_LogicalChannelId_Type | UInt_Type |  |

NR_MAC_Test_DLLogChID_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_MAC_Test_DLLogChID_Type |  |
| Comment |  |  |
| LogChId | NR_LogicalChannelId_Type | Specifies to over write the logical channel ID in MAC header in all the DL messages sent on the configured logical channel |
| ConfigLchId | Null_Type | Specifies that the normal mode of correct logical channel ID to be used in DL MAc header. This will be the default mode, when SS is initially configured. |

NR_MAC_Test_SCH_NoHeaderManipulation_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_MAC_Test_SCH_NoHeaderManipulation_Type |
| Comment |  |
| NormalMode | MAC header is fully controlled by the SS |
| DL_SCH_Only | TTCN can submit a final MAC PDU including header and payloads; SS does not do anything with this MAC PDU i.e. no header is added for the DL SCH transport channel. It is possible that data belonging to multiple DRBs is sent in one MAC PDU and from one special RB configured. NOTE: SRBs shall work as in normal mode and data can be sent/received on SRBs but sending on SRBs shall be in different TTIs than sending data PDUs. |
| DL_UL_SCH | In UL and DL the SS' MAC layer is transparent i.e. SS does not add or remove any MAC header |

NR_MAC_TestModeInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_TestModeInfo_Type |  |  |
| Comment | Parameters/Configuration for MAC tests |  |  |
| DiffLogChId | NR_MAC_Test_DLLogChID_Type |  | to be used in test cases 7.1.1.1 and 7.1.1.2 for using a different logical channel ID in MAC-header on DL-SCH channel |
| No_HeaderManipulation | NR_MAC_Test_SCH_NoHeaderManipulation_Type |  | to configure mode for no header manipulation in SS MAC layer for DL/UL SCH |

NR_MAC_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_MAC_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Info | NR_MAC_TestModeInfo_Type |  |

NR_MAC_LogicalChannelConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_LogicalChannelConfig_Type |  |  |
| Comment |  |  |  |
| Priority | integer |  | logical channel priority for the DL as described in TS 38.321, clause 5.4.3.1 for the UL |
| PrioritizedBitRate | NR_PrioritizedBitRate_Type |  | PBR as described for the UL; probably not needed at SS |

NR_MAC_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_Configuration_Type |  |  |
| Comment |  |  |  |
| LogicalChannel | NR_MAC_LogicalChannelConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | NR_MAC_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is"; for none MAC tests "TestMode.None:=true" |

## D.1.6	AS_Security

Primitive for control of AS security

NR_PdcpSQN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PdcpSQN_Type |  |  |
| Comment |  |  |  |
| Format | NR_PdcpCountFormat_Type |  | 12 bit or 18 bit SQN |
| Value | integer |  | SQN value (12 bit or 18 bit SQN) NOTE: in TTCN the test case writer is responsible to deal with potential overflows (e.g. there shall be a "mod 32", "mod 128" or "mod 4096" according to the format) |

NR_PDCP_ActTime_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_ActTime_Type |  |
| Comment | The sequence number (SQN) in UL and DL for SRB1 should be one more than the present SQN, as Ciphering starts in UL and DL soon after SMC and SMComp; For other SRB/DRB it should be the present SQN. UL: (TS 38.323 cl.7.1) RX_NEXT = [RX_NEXT_HFN, RX_NEXT_SN]. The receiving PDCP entity in the SS shall start/restart the AS security protection when RX_NEXT_SN becomes equal to or higher than SQN; DL: (TS 38.323 cl.7.1) TX_NEXT = [TX_NEXT_HFN, TX_NEXT_SN]. The transmitting PDCP entity in the SS shall start/restart the AS security protection when TX_NEXT_SN becomes equal to or higher than SQN. |  |
| None | Null_Type | No Activation time; to be used if Ciphering is not applied |
| SQN | NR_PdcpSQN_Type | PDCP sequence number |

NR_SecurityActTime_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SecurityActTime_Type |  |  |
| Comment |  |  |  |
| RadioBearerId | NR_RadioBearerId_Type |  |  |
| UL | NR_PDCP_ActTime_Type |  |  |
| DL | NR_PDCP_ActTime_Type |  |  |

NR_SecurityActTimeList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SecurityActTimeList_Type |
| Comment |  |
| record of NR_SecurityActTime_Type |  |

NR_AS_IntegrityInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_AS_IntegrityInfo_Type |  |  |
| Comment | for initial configuration activation time is not needed for integrity protection as all messages in DL after security activation are integrity protected; this means this ASP is invoked before transmission of Security mode command; if there is a integrity violation in UL SS shall set the IndicationStatus in the common ASP part to flag the integrity error (IndicationStatus.Error.Integrity.Pdcp := true); integrity to be provided for each SRB as per core spec |  |  |
| Algorithm | IntegrityProtAlgorithm |  | IntegrityProtAlgorithm being defined in RRC ASN.1 |
| KRRCint | B128_Key_Type |  |  |
| KUPint | B128_Key_Type | opt | Not used when UE connected to EPS (i.e. set to omit for EPS, unless UPIP) |
| ActTimeList | NR_SecurityActTimeList_Type | opt | omit for initial configuration (i.e. all SRBs to be integrity protected immediately); in HO scenarios activation time may be needed e.g. for SRB1 |

NR_AS_CipheringInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_AS_CipheringInfo_Type |  |  |
| Comment |  |  |  |
| Algorithm | CipheringAlgorithm |  | CipheringAlgorithm_Type being defined in RRC ASN.1 |
| KRRCenc | B128_Key_Type |  |  |
| KUPenc | B128_Key_Type |  | KUPenc is mandatory; and SS uses it when DRB are configured |
| ActTimeList | NR_SecurityActTimeList_Type |  |  |

NR_AS_SecStartRestart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_AS_SecStartRestart_Type |  |  |
| Comment |  |  |  |
| Integrity | NR_AS_IntegrityInfo_Type | opt | optional to allow separated activation of integrity and ciphering; omit: keep as it is |
| Ciphering | NR_AS_CipheringInfo_Type | opt | optional to allow separated activation of integrity and ciphering; omit: keep as it is |

NR_AS_Security_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_AS_Security_Type |  |
| Comment | Security mode command procedure (TS 38.331, clause 5.3.4) |  |
| StartRestart | NR_AS_SecStartRestart_Type | information to start/restart AS security protection in the PDCP TimingInfo : 'Now' (in general) NOTE: "activation time" may be specified in the primitive based on PDCP SQN |
| Release | Null_Type | to release AS security protection in the PDCP (if any; if there is no AS security the SS does not need to do anything) TimingInfo : 'Now' (in general) NOTE: "activation time" may be specified in the primitive based on PDCP SQN |

## D.1.7	Paging_Trigger

NR_SlotOffsetList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SlotOffsetList_Type |
| Comment |  |
| record length (1..infinity) of integer |  |

NR_PagingTrigger_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PagingTrigger_Type |  |  |
| Comment | to trigger transmission of a paging message on the PCCH at a calculated paging occasion (TS 38.304, clause 7); the paging occasion is calculated by TTCN and activation time is applied TimingInfo : Calculated paging occasion |  |  |
| Paging | PCCH_Message |  | paging to be send out at paging occasion and being announced on PDCCH using P-RNTI SS shall add the necessary padding bits as specified in TS 38.331, clause 8.5 |
| SlotOffsetList | NR_SlotOffsetList_Type | opt | list of slot offsets relative to the absolute timing information given in the common part of the ASP; if present, multiple pagings are sent out at all occasions given by the list; if omitted only a single paging is sent at the occasion given by the timing information in the common part of the ASP |

## D.1.8	Delta_Value_Trigger

NR_Band_SsbForDelta_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_Band_SsbForDelta_Type |  |  |
| Comment | Primary and secondary bands for requesting Delta values CellId: nr_Cell_NonSpecific TimingInfo : 'Now' |  |  |
| DeltaPrimary | Band_SsbInfo_Type |  |  |
| DeltaSecondary | Band_SsbInfo_Type |  |  |

Band_SsbInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Band_SsbInfo_Type |  |  |
| Comment | Band and SSB Frequency value for which Delta values are requested |  |  |
| DeltaBand | FreqBandIndicatorNR |  |  |
| Ssb_NRf1 | NR_ASN1_ARFCN_ValueNR_Type | opt |  |
| Ssb_NRf2 | NR_ASN1_ARFCN_ValueNR_Type | opt |  |
| Ssb_NRf3 | NR_ASN1_ARFCN_ValueNR_Type | opt |  |
| Ssb_NRf4 | NR_ASN1_ARFCN_ValueNR_Type | opt |  |

## D.1.9	System_Indication_Control

Primitive for control of system indications for special purposes

NR_System_IndicationControl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_System_IndicationControl_Type |  |  |
| Comment | Initially all indications apart from "Error" are disabled in SS (i.e. it shall not be necessary in 'normal' test cases to use this primitive but only if a specific indication is needed); omit means indication mode is not changed. TimingInfo : 'Now' (in general) |  |  |
| RLC_Discard | IndicationAndControlMode_Type | opt | To enable/disable reporting of discarded RLC PDUs |
| MAC_BSR | IndicationAndControlMode_Type | opt | To enable/disable reporting of short/long BSR |
| MAC_PHR | IndicationAndControlMode_Type | opt | To enable/disable reporting of short/long PHR |
| RachPreamble | IndicationAndControlMode_Type | opt | To enable/disable reporting of PRACH preamble |
| SchedReq | IndicationAndControlMode_Type | opt | To enable/disable reporting of Scheduling Request |
| UL_HARQ | IndicationAndControlMode_Type | opt | To enable/disable reporting of reception of HARQ ACK/NACK |
| HarqError | IndicationAndControlMode_Type | opt | To enable/disable reporting of HARQ errors |
| RecommendedBitRate | IndicationAndControlMode_Type | opt | To enable/disable reporting of Recommended bit rate MAC CE received in UL MAC PDU |
| UL_SRB0_LCID | IndicationAndControlMode_Type | opt | To enable/disable reporting of Logical Channel Id used in UL SRB0 MAC subPDU |
| MAC_SL_BSR | IndicationAndControlMode_Type | opt | To enable/disable reporting of sidelink BSR |
| MAC_SL_CG_Confirmation | IndicationAndControlMode_Type | opt | To enable/disable reporting of sidelink Configured Grand Confirmation |
| MAC_PosMeasGap | IndicationAndControlMode_Type | opt | To enable/disable reporting of Positioning Measurement Gap Activation/Deactivation request |

## D.1.10	PDCP_Count

Primitives to enquire PDCP COUNT

NR_PdcpCountFormat_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_PdcpCountFormat_Type |
| Comment |  |
| PdcpCount_Srb | 20 bit HFN; 12 bit SQN |
| PdcpCount_DrbSQN12 | 20 bit HFN; 12 bit SQN |
| PdcpCount_DrbSQN18 | 14 bit HFN; 18 bit SQN |

NR_PdcpCount_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PdcpCount_Type |  |  |
| Comment |  |  |  |
| Format | NR_PdcpCountFormat_Type |  |  |
| Value | PdcpCountValue_Type |  |  |

NR_PdcpCountInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PdcpCountInfo_Type |  |  |
| Comment |  |  |  |
| RadioBearerId | NR_RadioBearerId_Type |  |  |
| UL | NR_PdcpCount_Type | opt | omit: keep as it is |
| DL | NR_PdcpCount_Type | opt | omit: keep as it is |

NR_PdcpCountInfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_PdcpCountInfoList_Type |
| Comment |  |
| record of NR_PdcpCountInfo_Type |  |

NR_PdcpCountGetReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PdcpCountGetReq_Type |  |
| Comment |  |  |
| AllRBs | Null_Type | return COUNT values for all RBs being configured with Pdcp.RBTerminating in the NR Cell. |
| SingleRB | NR_RadioBearerId_Type |  |

NR_PDCP_CountReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_CountReq_Type |  |
| Comment |  |  |
| Get | NR_PdcpCountGetReq_Type | Request PDCP count for one or all RBs being configured at the PDCP TimingInfo: 'Now' |
| Set | NR_PdcpCountInfoList_Type | Set PDCP count for one or all RBs being configured at the PDCP; list for RBs which's COUNT shall be manipulated TimingInfo: 'Now' (in general) activation time may be used in case of CA inter cell handover to set the PdcpCount |

NR_PDCP_CountCnf_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_CountCnf_Type |  |
| Comment |  |  |
| Get | NR_PdcpCountInfoList_Type | RBs in ascending order; SRBs first |
| Set | Null_Type |  |

## D.1.11	PDCP_Handover

Primitives to control PDCP regarding handover

NR_PDCP_HandoverInit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_HandoverInit_Type |  |  |
| Comment |  |  |  |
| SourceCellId | NR_CellId_Type |  |  |

NR_PDCP_HandoverControlReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_HandoverControlReq_Type |  |
| Comment |  |  |
| HandoverInit | NR_PDCP_HandoverInit_Type | to inform SS that a handover (or PSCell change) will follow, for SS handling of PDCP context from the source cell to the target cell. CellId : In the common ASP part the CellId shall be set to the id of the target cell TimingInfo : 'Now' |
| HandoverComplete | Null_Type | to inform SS that the handover (or PSCell change) has successfully been performed by the UE; this shall trigger the SS to send a PDCP Status Report on AM DRB(s) to the UE; CellId : the CellId shall be set to the id of the target cell TimingInfo : 'Now' |

## D.1.12	L1_Test_Mode

Primitive for control of L1 Test Modes

NR_L1_TestMode_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_L1_TestMode_Type |  |  |
| Comment | TimingInfo : depends on the test mode; activation time is used e.g. for manipulation of the CRC |  |  |
| DL_SCH_CRC | NR_DL_SCH_CRC_Type |  | Manipulation of CRC bit generation for DL-SCH |

NR_MAC_Test_DL_SCH_CRC_Mode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_MAC_Test_DL_SCH_CRC_Mode_Type |
| Comment |  |
| Normal | default mode, the CRC generation is correct |
| Erroneous | SS shall generate CRC error by toggling CRC bits; the CRC error shall be applied for all PDUs of the given RNTI and their retransmission until SS is configured back to 'normal' operation |
| Error1AndNormal | the SS generates wrong CRC for first transmission and correct CRC on first retransmission. Later SS operates in normal mode. The retransmission is automatically triggered by reception of HARQ NACK |

NR_DL_SCH_CRC_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DL_SCH_CRC_Type |  |
| Comment | NOTE: CRC error mode for RA_RNTI is not addressed as it will be configured in RACHProcedureConfig |  |
| C_RNTI | NR_MAC_Test_DL_SCH_CRC_Mode_Type | to configure mode for CRC bit for all MAC PDUs for which C-RNTI is used in PDCCH transmission |
| SI_RNTI | NR_MAC_Test_DL_SCH_CRC_Mode_Type | to configure mode for CRC bit for all MAC PDUs for which SI-RNTI is used in PDCCH transmission |
| CS_RNTI | NR_MAC_Test_DL_SCH_CRC_Mode_Type | to configure mode for CRC bit for all MAC PDUs for which CS-RNTI is used in PDCCH transmission |

## D.1.13	DCI_Trigger

Primitive to trigger SS to send specific DCI (e.g. PDCCH order) which is not associated with any PDSCH or PUSCH transmission

NR_DCI_Trigger_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DCI_Trigger_Type |  |  |
| Comment | TimingInfo : 'Now' or specific activation time |  |  |
| AssignedBWPs | NR_AssignedBWPs_Type |  | BWP which shall be used to schedule the DCI |
| SearchSpaceType | NR_SearchSpaceType_Type |  | search space to be used for sending of given DCI; the SS may raise an error when there is no such search space at the scheduled point in time |
| DciFormat | NR_DCI_TriggerFormat_Type |  |  |

NR_DCI_TriggerFormat_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DCI_TriggerFormat_Type |  |
| Comment |  |  |
| PdcchOrder | NR_PDCCH_Order_Type |  |
| ShortMessage | NR_DciWithShortMessageOnly_Type |  |
| DciFormat_2_0 | NR_DciFormat_2_0_Type |  |
| DciFormat_2_1 | NR_DciFormat_2_1_Type |  |
| DciFormat_2_2 | NR_DciFormat_2_2_Type |  |
| DciFormat_2_3 | NR_DciFormat_2_3_Type |  |
| DciFormat_2_6 | NR_DciFormat_2_6_Type |  |
| DciFormat_2_7 | NR_DciFormat_2_7_Type |  |

NR_DciWithShortMessageOnly_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciWithShortMessageOnly_Type |  |  |
| Comment | "stand-alone" DCI with CRC scrambled by P-RNTI with no PCCH-Message being associated; => all fields apart from the ones listed in this record are reserved (see TS 38.212 clause 7.3.1.2.1) |  |  |
| ShortMessageIndicator | B2_Type |  | Short Message Indicator according to TS 38.212 Table 7.3.1.2.1-1; to be set to '10'B indicating that only short message is present in the DCI |
| ShortMessages | B8_Type |  | Short Messages according to TS 38.331 Table 6.5-1 |
| SlotOffsetList | NR_SlotOffsetList_Type | opt | list of slot offsets relative to the absolute timing information given in the common part of the ASP; if present, multiple short messages are sent out at all occasions given by the list; if omitted only a single short message is sent at the occasion given by the timing information in the common part of the ASP |

NR_PDCCH_Order_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCCH_Order_Type |  |  |
| Comment | PDCCH order accordingt to TS 38.212 clause 7.3.1.2.1 to initiate RA procedure (TS 38.321, clause 5.1.1): DCI format 1_0 with CRC scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones |  |  |
| RA_PreambleIndex | B6_Type |  | ra-PreambleIndex according to TS 38.321 clause 5.1.2 |
| UL_SUL_Indicator | NR_DciCommon_UL_SUL_Indicator_Type |  | indicates which UL carrier in the cell to transmit the PRACH if the UE is configured with SUL in the cell and RA_PreambleIndex != '000000'B; "None" otherwise |
| SSB_Index | B6_Type |  | indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission if RA_PreambleIndex != '000000'B; '000000'B (reserved) otherwise |
| PrachMaskIndex | B4_Type |  | indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to TS 38.321 clause 5.1.1; '0000'B (reserved) otherwise |

NR_DciFormat_2_0_SfiList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_DciFormat_2_0_SfiList_Type |
| Comment | list of SFI-indexes as used as slotFormatCombinationId in SlotFormatCombination (see TS 38.331): Each index addresses the SlotFormatCombination for a serving cell in SlotFormatCombinationsPerCell (the position is given by 'positionInDCI'); the size of each SFI-index depends on the maximum value of slotFormatCombinationIds (maxSFIindex) in the sequence of SlotFormatCombinations: SFI-index-length = MAX(CEIL(log2(maxSFIindex+1)), 1); see TS 38.213 clause 11.1.1; !!!! NR OPEN ISSUE: it is not fully clear whether the maximum slotFormatCombinationId is determined per Cell or over all cells !!!! !!!! ASSUMPTION: it is per cell and therefore different SFI-indexes may use bitstrings of different length !!!! |
| record of bitstring |  |

NR_DciFormat_2_0_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_2_0_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.3.1: for notifying the slot format; default parameters according to TS 38.508-1 clause 4.3.6.1.3.1 |  |  |
| SfiList | NR_DciFormat_2_0_SfiList_Type |  |  |

NR_DciFormat_2_1_IntValueList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_DciFormat_2_1_IntValueList_Type |
| Comment | list of 14 bit INT values per serving cell (see DownlinkPreemption, INT-ConfigurationPerServingCell in TS 38.331 and TS 38.213 clause 11.2) |
| record length (1..9) of B14_Type |  |

NR_DciFormat_2_1_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_2_1_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.3.2: notifying the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE; default parameters according to TS 38.508-1 clause 4.3.6.1.3.2 |  |  |
| IntValueList | NR_DciFormat_2_1_IntValueList_Type |  |  |

NR_DciFormat_2_2_ClosedLoopIndicator_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_2_2_ClosedLoopIndicator_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.3.3: Closed loop indicator |  |  |
| None | Null_Type |  | 0 bit if the UE is not configured with higher layer parameter twoPUSCH-PC-AdjustmentStates |
| Index | B1_Type |  | 1 bit otherwise |

NR_DciFormat_2_2_TpcBlock_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_2_2_TpcBlock_Type |  |
| Comment | TS 38.212 clause 7.3.1.3.3: single TPC block |  |
| ClosedLoopIndicator | NR_DciFormat_2_2_ClosedLoopIndicator_Type |  |
| TpcCommand | NR_DciCommon_TpcCommand_Type |  |

NR_DciFormat_2_2_TpcBlockList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_DciFormat_2_2_TpcBlockList_Type |
| Comment | list of TPC blocks according to TS 38.212 clause 7.3.1.3.3; beginning of each block is configured at the UE by tpc-IndexPCell or tpc-IndexPUCCH-SCell for PUCCH and by tpc-Index or tpc-IndexSUL for PUCCH (see PUCCH-TPC-CommandConfig and PUSCH-TPC-CommandConfig in TS 38.331 and TS 38.213 clause 11.3) |
| record of NR_DciFormat_2_2_TpcBlock_Type |  |

NR_DciFormat_2_2_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_2_2_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.3.3: for the transmission of TPC commands for PUCCH and PUSCH; default parameters according to TS 38.508-1 clause 4.3.6.1.3.3 |  |  |
| TpcBlockList | NR_DciFormat_2_2_TpcBlockList_Type |  |  |

NR_DciFormat_2_3_SrsRequest_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_2_3_SrsRequest_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.3.4: according to TS 38.213 clause 11.4 the presence of SRS request(s) depends on fieldTypeFormat2-3 (see SRS-TPC-CommandConfig in TS 38.331) |  |  |
| None | Null_Type |  | 0 bit: no SRS request |
| SrsRequestValue | B2_Type |  | 2 bits: Index of the SRS resource set to be used according to TS 38.212 Table 7.3.1.1.2-24 |

NR_DciCommon_TpcCommandList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_DciCommon_TpcCommandList_Type |
| Comment |  |
| record of NR_DciCommon_TpcCommand_Type |  |

NR_DciFormat_2_3_TypeA_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_2_3_TypeA_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.3.4: srs-TPC-PDCCH-Group = typeA (see SRS-CarrierSwitching in TS 38.331) |  |  |
| SrsRequest | NR_DciFormat_2_3_SrsRequest_Type |  |  |
| TpcCommandList | NR_DciCommon_TpcCommandList_Type |  |  |

NR_DciFormat_2_3_SingleBlockTypeB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_2_3_SingleBlockTypeB_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.3.4: srs-TPC-PDCCH-Group = typeB (see SRS-CarrierSwitching in TS 38.331) |  |  |
| SrsRequest | NR_DciFormat_2_3_SrsRequest_Type |  |  |
| TpcCommand | NR_DciCommon_TpcCommand_Type |  |  |

NR_DciFormat_2_3_TypeB_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_DciFormat_2_3_TypeB_Type |
| Comment |  |
| record of NR_DciFormat_2_3_SingleBlockTypeB_Type |  |

NR_DciFormat_2_3_TypeA_B_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_2_3_TypeA_B_Type |  |
| Comment |  |  |
| TypeA | NR_DciFormat_2_3_TypeA_Type | Type-A as indicated in srs-TPC-PDCCH-Group (see SRS-CarrierSwitching in TS 38.331) |
| TypeB | NR_DciFormat_2_3_TypeB_Type | Type-B as indicated in srs-TPC-PDCCH-Group (see SRS-CarrierSwitching in TS 38.331) |

NR_DciFormat_2_3_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_2_3_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.3.4: for the transmission of a group of TPC commands for SRS transmissions by one or more UEs; default parameters according to TS 38.508-1 clause 4.3.6.1.3.4 |  |  |
| TypeA_B | NR_DciFormat_2_3_TypeA_B_Type |  |  |

NR_DciFormat_2_6_Block_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_2_6_Block_Type |  |  |
| Comment |  |  |  |
| WakeupIndication | B1_Type |  |  |
| SCellDormacyIndication | NR_DciFormat_X_1_SCellDormancyIndication_Type |  |  |

NR_DciFormat_2_6_Block_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_DciFormat_2_6_Block_List_Type |
| Comment |  |
| record of NR_DciFormat_2_6_Block_Type |  |

NR_DciFormat_2_6_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_2_6_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.3.7: DCI format 2_6 is used for notifying the power saving information outside DRX Active Time for one or more UEs |  |  |
| BlockList | NR_DciFormat_2_6_Block_List_Type |  |  |

NR_DciFormat_2_7_TRS_Indication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_DciFormat_2_7_TRS_Indication_Type |  |
| Comment |  |  |
| None | Null_Type | 0 bit no TRS availability indication |
| Value | bitstring | 1, 2, 3, 4, 5, or 6 bits, being the number of bits equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig if configured |

NR_DciFormat_2_7_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DciFormat_2_7_Type |  |  |
| Comment | TS 38.212 clause 7.3.1.3.8: DCI format 2_7 is used for notifying the paging early indication and TRS availability indication for one or more UEs |  |  |
| PagingIndication | bitstring |  |  |
| TRSAvailabilityIndication | NR_DciFormat_2_7_TRS_Indication_Type |  |  |
| PayloadSizeDCI_2_7 | integer |  | If the number of information bits obtained from PagingIndication + TRSAvailabilityIndication is less than the size of format 2_7, the SS shall set the remaining bits as reserved bits (set to 0). |

## D.1.14	Configured_Scheduling

Primitive for control of DL SPS assignment and UL Configured Grant Type 2 configuration

NR_SPS_CG_ActDeact_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SPS_CG_ActDeact_Type |  |
| Comment |  |  |
| Activate | NR_SpsCgActivateInfo_Type | CellId : identifier of the cell where the UE is active RoutingInfo : None RlcBearerRouting : 'None' TimingInfo : activation time for DL SPS assignment or UL configured grant type 2 transmission; ControlInfo : CnfFlag:=false; FollowOnFlag:=false |
| Deactivate | NR_SpsCgDeactivateInfo_Type | CellId : identifier of the cell where the UE is active RoutingInfo : None RlcBearerRouting : 'None' TimingInfo : activation time for DL SPS assignment or UL configured grant type 2 release indicated by PDCCH transmission or SS local deactivation ControlInfo : CnfFlag:=false; FollowOnFlag:=false |

NR_SpsCgActivateInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SpsCgActivateInfo_Type |  |  |
| Comment | Even though DL SPS assignment or UL configured grant type 2 is pre-configured at the UE, it needs to be activated by L1 signalling => SS shall 'activate' DL SPS assignment or UL configured grant type 2 by sending appropriate assignments/grants to the UE; this shall be done with an activation time. If DL SPS assignment or UL configured grant type 2 is already configured and new Activate command is received, with ConfigIndex not included (i.e. Rel-15 operation), at the activation time SS locally deactivates old configuration, sends UE a PDCCH assignment for new DL SPS assignment or UL configured grant type 2 and locally activates new SPS configuration. In DL, in addition to SS SPS assignment configuration with activation time 'T', TTCN writer shall also schedule a DL MAC PDU with same activation time 'T' and at every DL assignment periodicity (NOTE: in general it is an error when TTCN does not provide data for a periodicity; SS shall send no data in this case). The SS sends the MAC PDU in 'T+ PDSCH slot offset (K0)' |  |  |
| UplinkConfiguredGrant | NR_UL_ConfiguredGrant_Type | opt |  |
| DownlinkSpsAssignment | NR_DL_SpsAssignment_Type | opt |  |

NR_UL_ConfiguredGrant_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_UL_ConfiguredGrant_Type |  |  |
| Comment | information to assign UL configured grant type schedules in UL |  |  |
| DciInfo | NR_DciUlInfo_Type | opt | to apply an UL grant |
| SetNDI_1 | Null_Type | opt | if present then NDI is set as 1 indicating a retransmission; If absent then NDI is set as 0 indicating a new transmission |
| ConfiguredGrantConfigIndex | ConfiguredGrantConfigIndex_r16 | opt | if not present SS shall follow Rel-15 operation. If present indicates Rel-16 or later operation and as per TS 38.213 clause 10.2 SS shall use this value in HARQ process ID field in DCI |

NR_DL_SpsAssignment_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DL_SpsAssignment_Type |  |  |
| Comment | information to assign semi-persistent scheduling in DL |  |  |
| DciInfo | NR_DciDlInfo_Type | opt | to apply a DL assignment |
| SetNDI_1 | Null_Type | opt | if present then NDI is set as 1 indicating a retransmission; If absent then NDI is set as 0 indicating a new transmission |
| SPS_ConfigIndex | SPS_ConfigIndex_r16 | opt | if not present SS shall follow Rel-15 operation. If present indicates Rel-16 or later operation and as per TS 38.213 clause 10.2 SS shall use this value in HARQ process ID field in DCI |

NR_SPS_CG_ReleasePdcchDciFormat_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_SPS_CG_ReleasePdcchDciFormat_Type |
| Comment | DCI format to be used for DL SPS assignment and UL configured grant type 2 acc. to TS 38.213, Table 10.2-X |
| dci_0_0 |  |
| dci_0_1 |  |
| dci_0_2 |  |
| dci_1_0 |  |
| dci_1_1 |  |
| dci_1_2 |  |

NR_SpsCgDeactivateInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SpsCgDeactivateInfo_Type |  |
| Comment |  |  |
| LocalRelease | Null_Type | DL SPS assignment and UL configured grant type 2 configuration shall be released at the SS, that means as well that the SS shall not address CS-RNTI anymore from the given TimingInfo onward; NOTE: there is no DL SPS or UL configured grant type 2 release to be signalled on PDCCH (this is done with PdcchExplicitRelease - see below) |
| PdcchExplicitRelease | NR_SPS_CG_ReleasePdcchDciFormat_Type | SS transmits PDCCH content indicating DL SPS assignment or UL configured grant type 2 release but holds the local configuration until it is locally released |
| PdcchExplicitRelease_Extn | NR_PdcchExplicitRelease_Extn_Type | Indicates Rel-16 or later operation. SS transmits PDCCH content indicating DL SPS assignment or UL configured grant type 2 release but holds the local configuration until it is locally released |

NR_PdcchExplicitRelease_Extn_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PdcchExplicitRelease_Extn_Type |  |  |
| Comment |  |  |  |
| DciFormat | NR_SPS_CG_ReleasePdcchDciFormat_Type |  |  |
| SPS_ConfigIndex | SPS_ConfigIndex_r16 | opt | Explicit release of DL SPS grant with config index; SS shall use this config index as HARQ process ID in DCI, ref TS 38.213 clause 10.2 |
| ConfiguredGrantConfigIndex | ConfiguredGrantConfigIndex_r16 | opt | Explicit release of UL configured grant type 2 with config index; SS shall use this config index as HARQ process ID in DCI, ref TS 38.213 clause 10.2 |

## D.1.15	System_Indications

Primitives for System indications

NR_HarqProcessInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_HarqProcessInfo_Type |  |  |
| Comment |  |  |  |
| Id | NR_HarqProcessId_Type |  |  |

NR_HarqError_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_HarqError_Type |  |
| Comment |  |  |
| UL | NR_HarqProcessInfo_Type | indicates HARQ error detected at the SS side (error at UL transmission) |
| DL | NR_HarqProcessInfo_Type | indicates HARQ NACK sent by the UE (error at DL transmission) |

NR_RachPreamble_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RachPreamble_Type |  |  |
| Comment |  |  |  |
| RAPID | integer |  | indicates the RAPID of the preamble (integer (0..63)) |

NR_RlcDiscardInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RlcDiscardInd_Type |  |  |
| Comment | SS shall send this indication if it discards a received RLC AMD PDU as specified in TS 38.322 cl. 5.2.3.2.2 |  |  |
| SequenceNumber | integer |  | sequence number of the PDU being discarded |

## D.1.16	System_Interface

NR_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SYSTEM_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | NR_ReqAspCommonPart_Type |  | Unless specified otherwise for a particular primitive, the following applies: CellId: identifier of the cell RoutingInfo : 'None' RlcBearerRouting : 'None' MacBearerRouting : 'omit' TimingInfo : 'Now' or specific activation time, depends on respective primitive ControlInfo : CnfFlag: depends on TimingInfo; in general 'false' when specific activation time is used, 'true' for 'Now' FollowOnFlag 'false' |
| Request | NR_SystemRequest_Type |  |  |

NR_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SYSTEM_CTRL_CNF |  |  |
| Comment |  |  |  |
| Common | NR_CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN (apart from EnquireTiming) => SS may set TimingInfo to "None" |
| Confirm | NR_SystemConfirm_Type |  |  |

NR_SYSTEM_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SYSTEM_IND |  |  |
| Comment |  |  |  |
| Common | NR_IndAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : 'none' RoutingInfoSUL: 'omit' RlcBearerRouting : 'none' MacBearerRouting : 'omit' TimingInfo : The SS shall provide TimingInfo depending on the respective indication (see below) |
| Indication | NR_SystemIndication_Type |  | - Error TimingInfo: related to the error (if available) - RlcDiscardInd TimingInfo: slot in which the RLC PDU has been received - MAC TimingInfo: slot in which the MAC PDU has been received containing the MAC CE being indicated - RachPreamble TimingInfo: start of the RACH preamble - SchedReq TimingInfo: slot containing the SR - UL_HARQ TimingInfo: slot containing the UL HARQ - HarqError TimingInfo: slot containing the UL HARQ - UL_SRB0_LCID TimingInfo: slot in which the MAC PDU has been received containing the UL SRB0 MAC subPDU |

NR_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NR_SYSTEM_PORT |  |
| Comment | NR PTC: Port for system configuration |  |
| out | NR_SYSTEM_CTRL_REQ |  |
| in | NR_SYSTEM_CTRL_CNF |  |

NR_SYSIND_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NR_SYSIND_PORT |  |
| Comment | NR PTC: Port for system indications |  |
| in | NR_SYSTEM_IND |  |

# D.2	NR_ASP_DrbDefs

ASP interface for DRBs

## D.2.1	PDU_TypeDefs

### D.2.1.1	MAC_PDU

NR_MAC_PDU_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_PDU_DL_Type |  |  |
| Comment |  |  |  |
| CE_SubPDUList | NR_MAC_CE_SubPDU_DL_List_Type | opt | list of subPDUs with MAC CE |
| SDU_SubPDUList | NR_MAC_SDU_SubPDU_List_Type | opt | list of subPDUs with MAC SDU |
| Padding_SubPDU | NR_MAC_Padding_SubPDU_Type | opt | subPDU with padding |

NR_MAC_PDU_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_PDU_UL_Type |  |  |
| Comment |  |  |  |
| SDU_SubPDUList | NR_MAC_SDU_SubPDU_List_Type | opt | list of subPDUs with MAC SDU |
| CE_SubPDUList | NR_MAC_CE_SubPDU_UL_List_Type | opt | list of subPDUs with MAC CE |
| Padding_SubPDU | NR_MAC_Padding_SubPDU_Type | opt | subPDU with padding |

NR_MAC_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_MAC_PDU_Type |  |
| Comment |  |  |
| DL | NR_MAC_PDU_DL_Type |  |
| UL | NR_MAC_PDU_UL_Type |  |

NR_MAC_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_MAC_PDUList_Type |
| Comment |  |
| record of NR_MAC_PDU_Type |  |

#### D.2.1.1.1	MAC_PDU_SubPDU

MAC subPDU (TS 38.321 clause 6.1.2)

MAC_PDU_SubPDU: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| B8_16_Type | bitstring length(8..16) | NOTE: length restriction can only be a range but not two distinct lengths |
| NR_MAC_SDU_Type | octetstring |  |

NR_MAC_PDU_SubHeader_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_PDU_SubHeader_Type |  |  |
| Comment | MAC PDU SubHeader (TS 38.321 clause 6.1.2) |  |  |
| Reserved | B1_Type |  | Reserved bit |
| Format | B1_Type |  | F: The Format field indicates the size of the Length field according to TS 38.321 clause 6.2.1: value 0 => 8 bits, value 1 => 16 bits. In case of MAC subheader for fixed sized MAC CE or padding (R/LCID MAC subheader) this field is reserved (i.e. treated as another R field) |
| LCID | B6_Type |  | LCID: Logical Channel ID field according to TS 38.321 Tables 6.2.1-1 and 6.2.1-2 |
| ELCID | B8_16_Type | opt | eLCID: The extended Logical Channel ID field according to TS 38.321 Tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits. |
| Length | B8_16_Type | opt | Either omit (fixed-sized MAC CE) or 8 bits (F=0) or 16 bits (F=1) |

NR_MAC_CE_SubPDU_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SubPDU_DL_Type |  |  |
| Comment | MAC DL subPPU with MAC subheader only (in case of MAC CE with fixed size of zero bits) or MAC subheader + MAC CE |  |  |
| SubHeader | NR_MAC_PDU_SubHeader_Type |  |  |
| ControlElement | NR_MAC_ControlElementDL_Type | opt | omit if MAC CE has fixed size of zero bits |

NR_MAC_CE_SubPDU_DL_List_Type

| TTCN-3 Set of Type |  |
| --- | --- |
| Name | NR_MAC_CE_SubPDU_DL_List_Type |
| Comment |  |
| set of NR_MAC_CE_SubPDU_DL_Type |  |

NR_MAC_CE_SubPDU_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SubPDU_UL_Type |  |  |
| Comment | MAC UL subPPU with MAC subheader only (in case of MAC CE with fixed size of zero bits) or MAC subheader + MAC CE |  |  |
| SubHeader | NR_MAC_PDU_SubHeader_Type |  |  |
| ControlElement | NR_MAC_ControlElementUL_Type | opt | omit if MAC CE has fixed size of zero bits |

NR_MAC_CE_SubPDU_UL_List_Type

| TTCN-3 Set of Type |  |
| --- | --- |
| Name | NR_MAC_CE_SubPDU_UL_List_Type |
| Comment |  |
| set of NR_MAC_CE_SubPDU_UL_Type |  |

NR_MAC_SDU_SubPDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_SDU_SubPDU_Type |  |  |
| Comment | MAC subPPU with MAC subheader + MAC SDU |  |  |
| SubHeader | NR_MAC_PDU_SubHeader_Type |  |  |
| SDU | NR_MAC_SDU_Type |  |  |

NR_MAC_SDU_SubPDU_List_Type

| TTCN-3 Set of Type |  |
| --- | --- |
| Name | NR_MAC_SDU_SubPDU_List_Type |
| Comment |  |
| set of NR_MAC_SDU_SubPDU_Type |  |

NR_MAC_Padding_SubPDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_Padding_SubPDU_Type |  |  |
| Comment | MAC subPPU with MAC subheader + Padding |  |  |
| SubHeader | NR_MAC_PDU_SubHeader_Type |  |  |
| Padding | octetstring |  | 0 or more octets padding |

#### D.2.1.1.2	MAC_ControlElements

MAC Control Elements (CEs) (TS 38.321 clause 6.1.3)

NR_MAC_ControlElementDL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_MAC_ControlElementDL_Type |  |
| Comment | TimingInfo : 'Now' or specific activation time |  |
| ContentionResolutionID | NR_MAC_CE_ContentionResolutionId_Type | TS 38.321 clause 6.1.3.3 |
| TimingAdvance | NR_MAC_CE_TimingAdvance_Type | TS 38.321 clause 6.1.3.4 |
| SCellActDeact | NR_MAC_CE_SCellActDeact_Type | TS 38.321 clause 6.1.3.10 |
| DuplicationActDeact | NR_MAC_CE_DuplicationActDeact_Type | TS 38.321 clause 6.1.3.11 |
| SP_ResourceSetActDeact | NR_MAC_CE_SP_ResourceSetActDeact_Type | TS 38.321 clause 6.1.3.12 |
| CSI_TriggerStateSubselection | NR_MAC_CE_CSI_TriggerStateSubselection_Type | TS 38.321 clause 6.1.3.13 |
| TCI_StatesActDeact | NR_MAC_CE_TCI_StatesActDeact_Type | TS 38.321 clause 6.1.3.14 |
| TCI_StateIndication | NR_MAC_CE_TCI_StateIndication_Type | TS 38.321 clause 6.1.3.15 |
| SP_CSI_ReportingActDeact | NR_MAC_CE_SP_CSI_ReportingActDeact_Type | TS 38.321 clause 6.1.3.16 |
| SP_SRS_ActDeact | NR_MAC_CE_SP_SRS_ActDeact_Type | TS 38.321 clause 6.1.3.17 |
| PUCCH_SpatialRelationActDeact | NR_MAC_CE_PUCCH_SpatialRelationActDeact_Type | TS 38.321 clause 6.1.3.18 |
| SP_ZP_ResourceSetActDeact | NR_MAC_CE_SP_ZP_ResourceSetActDeact_Type | TS 38.321 clause 6.1.3.19 |
| RecommendatdBitrate | NR_MAC_CE_RecommendedBitrate_Type | TS 38.321 clause 6.1.3.20 |
| RlcDuplicationActDeact | NR_MAC_CE_RLC_DuplicationActDeact_Type | TS 38.321 clause 6.1.3.32 |
| PosMeasGapActDeactCmd | NR_MAC_CE_POS_MeasGapActDeact_Type | TS 38.321 clause 6.1.3.41 |
| PPWActDeactCmd | NR_MAC_CE_PPWActDeactCmd_Type | TS 38.321 clause 6.1.3.42 |

NR_MAC_ControlElementUL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_MAC_ControlElementUL_Type |  |
| Comment |  |  |
| ShortBSR | NR_MAC_CE_ShortBSR_Type | TS 38.321 clause 6.1.3.1 |
| LongBSR | NR_MAC_CE_LongBSR_Type | TS 38.321 clause 6.1.3.1 |
| C_RNTI | RNTI_B16_Type | TS 38.321 clause 6.1.3.2 |
| SingleEntryPHR | NR_MAC_CE_SingleEntryPHR_Type | TS 38.321 clause 6.1.3.8 |
| MultiEntryPHR | NR_MAC_CE_MultiEntryPHR_Type | TS 38.321 clause 6.1.3.9 |
| RecommendedBitrate | NR_MAC_CE_RecommendedBitrate_Type | TS 38.321 clause 6.1.3.20 |
| SL_BSR | NR_MAC_CE_SL_BSR_List_Type | TS 38.321 clause 6.1.3.33 |
| SL_CG_Confirmation | O1_Type | TS 38.321 clause 6.1.3.34 |
| PosMeasGapActDeactReq | NR_MAC_CE_POS_MeasGapActDeact_Type | TS 38.321 clause 6.1.3.40 |

##### D.2.1.1.2.1	MAC_ControlElement_Common

NR_MAC_CE_SCellFlags_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SCellFlags_Type |  |  |
| Comment | bitmap to indicate presence of SCell with SCellIndex according to TS 38.331 either SCellIndex7_1 is present only or all octets are present |  |  |
| SCellIndex7_1 | B8_Type |  | leftmost bit corresponds to SCellIndex7, 2nd bit from the right corresponds to SCellIndex1, rightmost bit is reserved |
| SCellIndex15_8 | B8_Type | opt | leftmost bit corresponds to SCellIndex15, rightmost bit corresponds to SCellIndex8 |
| SCellIndex23_16 | B8_Type | opt | leftmost bit corresponds to SCellIndex23, rightmost bit corresponds to SCellIndex16 |
| SCellIndex31_24 | B8_Type | opt | leftmost bit corresponds to SCellIndex31, rightmost bit corresponds to SCellIndex24 |

NR_MAC_CE_ServCellId_BwpId_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_ServCellId_BwpId_Type |  |  |
| Comment | Common definition for first octet of CEs defined in TS 38.321 clause 6.1.3.12 .. TS 38.321 clause 6.1.3.19 |  |  |
| Field1 | B1_Type |  | A/D field for NR_MAC_CE_SP_ResourceSetActDeact_Type and NR_MAC_CE_SP_SRS_ActDeact_Type or CORESET Pool ID for NR_MAC_CE_TCI_StatesActDeact_Type reserved (set to 0) otherwise |
| ServCellId | B5_Type |  | identity of the Serving Cell for which the MAC CE applies |
| BwpId | B2_Type |  | BWP-Id (as specified in TS 38.331) of the uplink/downlink bandwidth part for which the MAC CE applies |

##### D.2.1.1.2.2	MAC_ControlElement_BSR

TS 38.321 clause 6.1.3.1 (Buffer Status Report MAC CEs)

MAC_ControlElement_BSR: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_MAC_LongBSR_BufferSize_Type | O1_Type |  |

NR_MAC_CE_ShortBSR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_ShortBSR_Type |  |  |
| Comment | Short BSR and Short Truncated BSR MAC CE according to TS 38.321 Figure 6.1.3.1-1 |  |  |
| LCG | B3_Type |  |  |
| BufferSize | B5_Type |  |  |

NR_MAC_LongBSR_BufferSizeList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_MAC_LongBSR_BufferSizeList_Type |
| Comment |  |
| record length (0..8) of NR_MAC_LongBSR_BufferSize_Type |  |

NR_MAC_CE_LongBSR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_LongBSR_Type |  |  |
| Comment | Long BSR, Long Truncated BSR and Pre-emptive BSR MAC CE according to TS 38.321 Figure 6.1.3.1-2 |  |  |
| LCG_Presence | B8_Type |  | '1' indicates that the Buffer Size field for a logical channel group i is reported, with i = 7 for the leftmost bit and i = 0 for the rightmost |
| BufferSizeList | NR_MAC_LongBSR_BufferSizeList_Type |  | According to TS 38.321 clause 6.1.3.1 the Buffer Size fields are included in ascending order based on the LCGi |

##### D.2.1.1.2.3	MAC_ControlElement_ContentionResolutionId

TS 38.321 clause 6.1.3.3 (UE Contention Resolution Identity MAC CE)

MAC_ControlElement_ContentionResolutionId: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_MAC_CE_ContentionResolutionId_Type | B48_Type | TS 38.321 Figure 6.1.3.3-1; fix size of 48 bits |

##### D.2.1.1.2.4	MAC_ControlElement_TimingAdvance

TS 38.321 clause 6.1.3.4 (Timing Advance Command MAC CE)

NR_MAC_CE_TimingAdvance_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_TimingAdvance_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.4-1 |  |  |
| TAG_ID | B2_Type |  | TAG Identity of the addressed TAG |
| TimingAdvanceCommand | B6_Type |  | index value TA (0..63) used to control the amount of timing adjustment that MAC entity has to apply (as specified in TS 38.213) |

##### D.2.1.1.2.5	MAC_ControlElement_PHR

TS 38.321 clause 6.1.3.8 (Single Entry PHR) and 6.1.3.9 (Multiple Entry PHR)

MAC_ControlElement_PHR: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_MAC_CE_SingleEntryPHR_Type | NR_MAC_CE_PH_Record_Type | TS 38.321 Figure 6.1.3.8-1 |

NR_MAC_CE_PH_Record_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_PH_Record_Type |  |  |
| Comment |  |  |  |
| P_Bit | B1_Type |  | P bit: 1 indicates the UE applies power backoff due to power management; For Single Entry PHR MAC CE: If mpe-Reporting is configured indicates the applied power backoff to meet MPE requirements. |
| V_Bit | B1_Type |  | V bit: Indicates when the PH value is based on a real transmission or a reference format; reserved (R = '0'B) for Single Entry PHR MAC CE |
| Value | B6_Type |  | The power headroom level. Ph Type 2 (if configured) for PCell and Type 1 for PCell and SCell |
| MPE_or_R | B2_Type | opt | 2 reserved bits or MPE (Maximum Permissible Exposure); present if V=0 |
| PCMaxc | B6_Type | opt | present if V=0 |

NR_MAC_CE_MultiEntryPHR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_MultiEntryPHR_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.9-1 and Figure 6.1.3.9-2 |  |  |
| PHFieldPresentForSCell | NR_MAC_CE_SCellFlags_Type |  | to indicate presence of PH field for particular SCell |
| PH_Record | record of NR_MAC_CE_PH_Record_Type |  | list of PH_Records for PCell and SCells as described in TS 38.321 clause 6.1.3.9 |

##### D.2.1.1.2.6	MAC_ControlElement_SCellActivationDeactivation

TS 38.321 clause 6.1.3.10 (SCell Activation/Deactivation MAC CEs)

MAC_ControlElement_SCellActivationDeactivation: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_MAC_CE_SCellActDeact_Type | NR_MAC_CE_SCellFlags_Type | TS 38.321 Figure 6.1.3.10-1 and Figure 6.1.3.10-2 |

##### D.2.1.1.2.7	MAC_ControlElement_DuplicationActivationDeactivation

TS 38.321 clause 6.1.3.11 (Duplication Activation/Deactivation MAC CE)

MAC_ControlElement_DuplicationActivationDeactivation: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_MAC_CE_DuplicationActDeact_Type | B8_Type | TS 38.321 Figure 6.1.3.11-1 |

##### D.2.1.1.2.8	MAC_ControlElement_SP_ResourceSetActivationDeactivation

TS 38.321 clause 6.1.3.12 (SP CSI-RS / CSI-IM Resource Set Activation/Deactivation MAC CE)

NR_MAC_CE_SP_ResourceSetActDeact_Octet2_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SP_ResourceSetActDeact_Octet2_Type |  |  |
| Comment |  |  |  |
| Reserved | B1_Type |  |  |
| IM | B1_Type |  | indicates whether or not octet 3 is present |
| CSI_RS_ResourcesetId | B6_Type |  |  |

NR_MAC_CE_SP_ResourceSetActDeact_Octet3_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SP_ResourceSetActDeact_Octet3_Type |  |  |
| Comment |  |  |  |
| Reserved | B2_Type |  |  |
| CSI_IM_ResourcesetId | B6_Type |  |  |

NR_MAC_CE_SP_ResourceSetActDeact_TciStateId_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SP_ResourceSetActDeact_TciStateId_Type |  |  |
| Comment |  |  |  |
| Reserved | B1_Type |  |  |
| Id | B7_Type |  |  |

NR_MAC_CE_SP_ResourceSetActDeact_TciStateIdList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_MAC_CE_SP_ResourceSetActDeact_TciStateIdList_Type |
| Comment |  |
| record of NR_MAC_CE_SP_ResourceSetActDeact_TciStateId_TypeeId_Type |  |

NR_MAC_CE_SP_ResourceSetActDeact_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SP_ResourceSetActDeact_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.12-1 |  |  |
| Octet1 | NR_MAC_CE_ServCellId_BwpId_Type |  | Field1: A/D field |
| Octet2 | NR_MAC_CE_SP_ResourceSetActDeact_Octet2_Type |  |  |
| Octet3 | NR_MAC_CE_SP_ResourceSetActDeact_Octet3_Type | opt | present if IM=1 in octet 2 |
| IdList | NR_MAC_CE_SP_ResourceSetActDeact_TciStateIdList_Type |  |  |

##### D.2.1.1.2.9	MAC_ControlElement_CSI_TriggerStateSubselection

TS 38.321 clause 6.1.3.13 (Aperiodic CSI Trigger State Subselection MAC CE)

NR_MAC_CE_CSI_TriggerStateSubselection_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_CSI_TriggerStateSubselection_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.13-1 |  |  |
| Octet1 | NR_MAC_CE_ServCellId_BwpId_Type |  | Field1: reserved |
| Selection | B8_List_Type |  |  |

##### D.2.1.1.2.10	MAC_ControlElement_TCI_StatesActivationDeactivation

TS 38.321 clause 6.1.3.14 (TCI States Activation/Deactivation for UE-specific PDSCH MAC CE)

NR_MAC_CE_TCI_StatesActDeact_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_TCI_StatesActDeact_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.14-1 |  |  |
| Octet1 | NR_MAC_CE_ServCellId_BwpId_Type |  | Field1: CORESET Pool ID field |
| Status | B8_List_Type |  |  |

##### D.2.1.1.2.11	MAC_ControlElement_TCI_StateIndication

TS 38.321 clause 6.1.3.15 (TCI State Indication for UE-specific PDCCH MAC CE)

NR_MAC_CE_TCI_StateIndication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_TCI_StateIndication_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.15-1 |  |  |
| ServCellId | B5_Type |  | identity of the Serving Cell for which the MAC CE applies |
| CoresetId | B4_Type |  | ControlResourceSetId for which the TCI State is being indicated |
| TciStateId | B7_Type |  | TCI-StateId applicable to the Control Resource Set identified by CORESET ID field |

##### D.2.1.1.2.12	MAC_ControlElement_SP_CSI_ReportingActivationDeactivation

TS 38.321 clause 6.1.3.16 (SP CSI reporting on PUCCH Activation/Deactivation MAC CE)

NR_MAC_CE_SP_CSI_ReportingActDeact_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SP_CSI_ReportingActDeact_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.16-1 |  |  |
| Octet1 | NR_MAC_CE_ServCellId_BwpId_Type |  | Field1: reserved |
| Reserved | B4_Type |  |  |
| ConfigState | B4_Type |  |  |

##### D.2.1.1.2.13	MAC_ControlElement_SP_SRS_ActivationDeactivation

NR_MAC_CE_SP_SRS_ActDeact_Octet2_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SP_SRS_ActDeact_Octet2_Type |  |  |
| Comment | TS 38.321 clause 6.1.3.17 (SP SRS Activation/Deactivation MAC CE) |  |  |
| Reserved | B2_Type |  |  |
| C | B1_Type |  |  |
| SUL | B1_Type |  |  |
| SRS_ResourcesetId | B4_Type |  |  |

NR_MAC_CE_SP_SRS_ActDeact_ResourceId_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SP_SRS_ActDeact_ResourceId_Type |  |  |
| Comment |  |  |  |
| F | B1_Type |  |  |
| Id | B7_Type |  |  |

NR_MAC_CE_SP_SRS_ActDeact_ResourceIdList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_MAC_CE_SP_SRS_ActDeact_ResourceIdList_Type |
| Comment |  |
| record of NR_MAC_CE_SP_SRS_ActDeact_ResourceId_Typee |  |

NR_MAC_CE_SP_SRS_ActDeact_ResourceInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SP_SRS_ActDeact_ResourceInfo_Type |  |  |
| Comment |  |  |  |
| Reserved | B1_Type |  |  |
| ServingCellId | B5_Type |  |  |
| BwpId | B2_Type |  |  |

NR_MAC_CE_SP_SRS_ActDeact_ResourceInfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_MAC_CE_SP_SRS_ActDeact_ResourceInfoList_Type |
| Comment |  |
| record of NR_MAC_CE_SP_SRS_ActDeact_ResourceInfo_Typeype |  |

NR_MAC_CE_SP_SRS_ActDeact_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SP_SRS_ActDeact_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.17-1 |  |  |
| Octet1 | NR_MAC_CE_ServCellId_BwpId_Type |  | Field1: A/D field |
| Octet2 | NR_MAC_CE_SP_SRS_ActDeact_Octet2_Type |  |  |
| ResourceIdList | NR_MAC_CE_SP_SRS_ActDeact_ResourceIdList_Type |  |  |
| ResourceInfoList | NR_MAC_CE_SP_SRS_ActDeact_ResourceInfoList_Type |  | empty list when C=0 |

##### D.2.1.1.2.14	MAC_ControlElement_PUCCH_SpatialRelationActivationDeactivation

TS 38.321 clause 6.1.3.18 (PUCCH spatial relation Activation/Deactivation MAC CE)

NR_MAC_CE_PUCCH_SpatialRelationActDeact_Octet2_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_PUCCH_SpatialRelationActDeact_Octet2_Type |  |  |
| Comment |  |  |  |
| Reserved | B1_Type |  |  |
| ResourceId | B7_Type |  |  |

NR_MAC_CE_PUCCH_SpatialRelationActDeact_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_PUCCH_SpatialRelationActDeact_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.18-1 |  |  |
| Octet1 | NR_MAC_CE_ServCellId_BwpId_Type |  | Field1: reserved |
| Octet2 | NR_MAC_CE_PUCCH_SpatialRelationActDeact_Octet2_Type |  |  |
| ActivationStatus | B8_Type |  |  |

##### D.2.1.1.2.15	MAC_ControlElement_ZP_ResourceSetActivationDeactivation

TS 38.321 clause 6.1.3.19 (SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE)

NR_MAC_CE_SP_ZP_ResourceSetActDeact_Octet2_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SP_ZP_ResourceSetActDeact_Octet2_Type |  |  |
| Comment |  |  |  |
| Reserved | B4_Type |  |  |
| Id | B4_Type |  |  |

NR_MAC_CE_SP_ZP_ResourceSetActDeact_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SP_ZP_ResourceSetActDeact_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.19-1 |  |  |
| Octet1 | NR_MAC_CE_ServCellId_BwpId_Type |  | Field1: A/D field |
| Octet2 | NR_MAC_CE_SP_ZP_ResourceSetActDeact_Octet2_Type |  |  |

##### D.2.1.1.2.16	MAC_ControlElement_RecommendedBitrate

TS 38.321 clause 6.1.3.20 (Recommended bit rate MAC CE)

NR_MAC_CE_RecommendedBitrate_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_RecommendedBitrate_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.20-1 |  |  |
| LCID | B6_Type |  |  |
| UL_DL | B1_Type |  |  |
| Bitrate | B6_Type |  |  |
| X | B1_Type |  | Bit rate multiplier |
| Reserved | B2_Type |  |  |

##### D.2.1.1.2.17	MAC_ControlElement_CE_RLC_DuplicationActDeact

NR_MAC_CE_RLC_DuplicationActDeact_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_RLC_DuplicationActDeact_Type |  |  |
| Comment |  |  |  |
| DrbId | B5_Type |  |  |
| Rlc2 | B1_Type |  |  |
| Rlc1 | B1_Type |  |  |
| Rlc0 | B1_Type |  |  |

##### D.2.1.1.2.18	MAC_ControlElement_SL_BSR

TS 38.321 clause 6.1.3.33 (Sidellink Buffer Status Report MAC CEs)

NR_MAC_CE_SL_BSR_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_MAC_CE_SL_BSR_List_Type |
| Comment | SL-BSR and Truncated SL-BSR MAC CE according to TS 38.321 Figure 6.1.3.33-1 |
| record of NR_MAC_CE_SL_BSR_Type |  |

NR_MAC_CE_SL_BSR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_SL_BSR_Type |  |  |
| Comment |  |  |  |
| DestinationIndex | B5_Type |  |  |
| LCG | B3_Type |  |  |
| BufferSize | O1_Type |  |  |

##### D.2.1.1.2.19	MAC_ControlElement_CE_POS_MeasGapActDeact

TS 38.321 clause 6.1.3.40 (Positioning Measurement Gap Activation/Deactivation Request MAC CE);
TS 38.321 clause 6.1.3.41 (Positioning Measurement Gap Activation/Deactivation Command MAC CE)

NR_MAC_CE_POS_MeasGapActDeact_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_POS_MeasGapActDeact_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.40-1; TS 38.321 Figure 6.1.3.41-1 |  |  |
| Reserved | B3_Type |  |  |
| Field2 | B1_Type |  | A/D field; set to 1 for activation, set to 0 for deactivation |
| PosMG_Id | B4_Type |  | Positioning MG ID |

##### D.2.1.1.2.20	MAC_ControlElement_CE_PPWActDeactCmd

TS 38.321 clause 6.1.3.42 (PPW Activation/Deactivation Command MAC CE)

PpwInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PpwInfo_Type |  |  |
| Comment |  |  |  |
| ServingCellId | B5_Type |  | Identity of the Serving Cell for which the MAC CE applies |
| PpwId | B2_Type |  | Index of the PPW configured on active DL BWP of the Serving Cell |
| Field3 | B1_Type |  | A/D field; set to 1 for activation, set to 0 for deactivation |

PpwInfo_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PpwInfo_List_Type |
| Comment |  |
| record of PpwInfo_Type |  |

NR_MAC_CE_PPWActDeactCmd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_MAC_CE_PPWActDeactCmd_Type |  |  |
| Comment | TS 38.321 Figure 6.1.3.42-1 |  |  |
| Reserved | B6_Type |  |  |
| NumEntry | B2_Type |  | Number of entries N-1 in the MAC CE. 00 indicates that N equals to 2; 01 indicates that N equals to 3 and so on |
| PpwInfo_List | PpwInfo_List_Type |  | List of PPW info |

### D.2.1.2	RLC_PDU

RLC_PDU: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_RLC_SDU_Type | octetstring |  |

NR_RLC_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RLC_PDU_Type |  |
| Comment |  |  |
| TMD | NR_RLC_TMD_PDU_Type |  |
| UMD | NR_RLC_UMD_PDU_Type |  |
| AMD | NR_RLC_AMD_PDU_Type |  |
| Status | NR_RLC_AM_StatusPDU_Type |  |

NR_RLC_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_RLC_PDUList_Type |
| Comment |  |
| record of NR_RLC_PDU_Type |  |

NR_RLC_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_RLC_SDUList_Type |
| Comment |  |
| record of NR_RLC_SDU_Type |  |

#### D.2.1.2.1	Common

RLC PDU definition: common AM/UM field definitions

Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_RLC_SegmentationInfo_Type | B2_Type | Segmentation Info (SI) field (TS 38.322, clause 6.2.3.4) 00 - Data field contains all bytes of an RLC SDU 01 - Data field contains the first segment of an RLC SDU 10 - Data field contains the last segment of an RLC SDU 11 - Data field contains neither the first nor last segment of an RLC SDU |
| NR_RLC_SegmentOffset_Type | B16_Type | Segment Offset (SO) field (TS 38.322, 6.2.3.5) |

#### D.2.1.2.2	TM_Data

RLC PDU definition: UM  (TS 38.322, clause 6.2.2.2)

TM_Data: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_RLC_TMD_PDU_Type | octetstring | TS 38.322, clause 6.2.2.2 |

#### D.2.1.2.3	UM_Data

RLC PDU definition: UM  (TS 38.322, clause 6.2.2.3)

UM_Data: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_RLC_UMD_Data_Type | octetstring | TS 38.322, clause 6.2.2.3 |

NR_RLC_UMD_HeaderNoSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_UMD_HeaderNoSN_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.3 (Figure 6.2.2.3-1; one octet |  |  |
| SegmentationInfo | NR_RLC_SegmentationInfo_Type |  |  |
| Reserved | B6_Type |  |  |

NR_RLC_UMD_PduNoSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_UMD_PduNoSN_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.3 (Figure 6.2.2.3-1); one octet |  |  |
| Header | NR_RLC_UMD_HeaderNoSN_Type |  |  |
| Data | NR_RLC_UMD_Data_Type |  |  |

NR_RLC_UMD_HeaderSN6Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_UMD_HeaderSN6Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.3 (6.2.2.3-2, 6.2.2.3-4); one octet |  |  |
| SegmentationInfo | NR_RLC_SegmentationInfo_Type |  | 2 bits SI |
| SequenceNumber | B6_Type |  | 6 bits SN |
| SegmentOffset | NR_RLC_SegmentOffset_Type | opt | 16 bits SO; included in case of segmentation but not for the first segment (TS 38.322 clause 6.2.2.3) |

NR_RLC_UMD_PduSN6Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_UMD_PduSN6Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.3 (6.2.2.3-2, 6.2.2.3-4); one octet |  |  |
| Header | NR_RLC_UMD_HeaderSN6Bit_Type |  |  |
| Data | NR_RLC_UMD_Data_Type |  |  |

NR_RLC_UMD_HeaderSN12Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_UMD_HeaderSN12Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.3 (Figure 6.2.2.3-3, 6.2.2.3-5); two octets |  |  |
| SegmentationInfo | NR_RLC_SegmentationInfo_Type |  | 2 bits SI |
| Reserved | B2_Type |  | 2 bits reserved |
| SequenceNumber | B12_Type |  | 12 bits SN |
| SegmentOffset | NR_RLC_SegmentOffset_Type | opt | 16 bits SO; included in case of segmentation but not for the first segment (TS 38.322 clause 6.2.2.3) |

NR_RLC_UMD_PduSN12Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_UMD_PduSN12Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.3 (Figure 6.2.2.3-3, 6.2.2.3-5); two octets |  |  |
| Header | NR_RLC_UMD_HeaderSN12Bit_Type |  |  |
| Data | NR_RLC_UMD_Data_Type |  |  |

NR_RLC_UMD_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RLC_UMD_PDU_Type |  |
| Comment |  |  |
| NoSN | NR_RLC_UMD_PduNoSN_Type |  |
| SN6Bit | NR_RLC_UMD_PduSN6Bit_Type |  |
| SN12Bit | NR_RLC_UMD_PduSN12Bit_Type |  |

#### D.2.1.2.4	AM_Data

RLC PDU definition: AM  (TS 38.322, clause 6.2.2.4)

AM_Data: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_RLC_AMD_Data_Type | octetstring | TS 38.322, clause 6.2.2.4 |

NR_RLC_AMD_HeaderSN12Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_AMD_HeaderSN12Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.4 (Figure 6.2.2.4-1, 6.2.2.4-3) |  |  |
| D_C | B1_Type |  | 1 bit, '1'B for Data PDU |
| Poll | B1_Type |  | 1 bit, '0'B - Status report not requested '1'B - Status report is requested |
| SegmentationInfo | NR_RLC_SegmentationInfo_Type |  | 2 bits SI |
| SequenceNumber | B12_Type |  | 12 bits SN |
| SegmentOffset | NR_RLC_SegmentOffset_Type | opt | 16 bits SO; included in case of segmentation but not for the first segment (TS 38.322 clause 6.2.2.4) |

NR_RLC_AMD_PduSN12Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_AMD_PduSN12Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.4 (Figure 6.2.2.4-1, 6.2.2.4-3) |  |  |
| Header | NR_RLC_AMD_HeaderSN12Bit_Type |  |  |
| Data | NR_RLC_AMD_Data_Type |  |  |

NR_RLC_AMD_HeaderSN18Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_AMD_HeaderSN18Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.4 (Figure 6.2.2.4-2, 6.2.2.4-4) |  |  |
| D_C | B1_Type |  | 1 bit, '1'B for Data PDU |
| Poll | B1_Type |  | 1 bit, 0 - Status report not requested 1 - Status report is requested |
| SegmentationInfo | NR_RLC_SegmentationInfo_Type |  | 2 bits SI |
| Reserved | B2_Type |  | 2 bits reserved |
| SequenceNumber | B18_Type |  | 18 bits SN |
| SegmentOffset | NR_RLC_SegmentOffset_Type | opt | 16 bits SO; included in case of segmentation but not for the first segment (TS 38.322 clause 6.2.2.4) |

NR_RLC_AMD_PduSN18Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_AMD_PduSN18Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.4 Figure 6.2.2.4-2, 6.2.2.4-4) |  |  |
| Header | NR_RLC_AMD_HeaderSN18Bit_Type |  |  |
| Data | NR_RLC_AMD_Data_Type |  |  |

NR_RLC_AMD_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RLC_AMD_PDU_Type |  |
| Comment | TS 38.322, clause 6.2.2.4 |  |
| SN12Bit | NR_RLC_AMD_PduSN12Bit_Type |  |
| SN18Bit | NR_RLC_AMD_PduSN18Bit_Type |  |

#### D.2.1.2.5	AM_Status

AM Status PDU (TS 36.322, clause 6.2.1.6)

AM_Status: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_RLC_Status_ExtensionBit1_Type | B1_Type | TS 38.322, clause 6.2.3.11 Extension bit 1 (E1) field: '0'B A set of NACK_SN, E1, E2 and E3 does not follow. '1'B A set of NACK_SN, E1, E2 and E3 follows. |
| NR_RLC_Status_ExtensionBit2_Type | B1_Type | TS 38.322, clause 6.2.3.13 Extension bit 2 (E2) field: '0'B A set of SOstart and SOend does not follow for this NACK_SN. '1'B A set of SOstart and SOend follows for this NACK_SN. |
| NR_RLC_Status_ExtensionBit3_Type | B1_Type | TS 38.322, clause 6.2.3.16 Extension bit 3 (E3) field: '0'B A set of NACK_SN, E1, E2 and E3 follows. '1'B NACK range field follows for this NACK_SN. |

NR_RLC_Status_NackSN12Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_Status_NackSN12Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.5 (Figure 6.2.2.5-1) |  |  |
| SequenceNumberNACK | B12_Type |  | 12 bits SN |
| E1 | NR_RLC_Status_ExtensionBit1_Type |  | 1 bit E1 field; set if further NACK set follows |
| E2 | NR_RLC_Status_ExtensionBit2_Type |  | 1 bit E2 field |
| E3 | NR_RLC_Status_ExtensionBit3_Type |  | 1 bit E3 field |
| Reserved | B1_Type |  | 1 bit reserved |
| SOstart | NR_RLC_SegmentOffset_Type | opt | segment offset (start), present only if E2 is set to '1'B |
| SOstop | NR_RLC_SegmentOffset_Type | opt | segment offset (stop), present only if E2 is set to '1'B |
| NACKrange | B8_Type | opt | NACK range, present only if E3 is set to '1'B |

NR_RLC_Status_NackListSN12Bit_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_RLC_Status_NackListSN12Bit_Type |
| Comment |  |
| record of NR_RLC_Status_NackSN12Bit_Type |  |

NR_RLC_StatusPduSN12Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_StatusPduSN12Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.5 (Figure 6.2.2.5-1) |  |  |
| D_C | B1_Type |  | 1 bit, '0'B for Control PDU |
| CPT | B3_Type |  | 3 bits, TS 38.322, clause 6.2.3.9 Control PDU Type (CPT) field: '000'B STATUS PDU ELSE reserved |
| SequenceNumberACK | B12_Type |  | 12 bits SN |
| E1 | NR_RLC_Status_ExtensionBit1_Type |  | 1 bit E1 field |
| Reserved | B7_Type |  | 7 bits reserved |
| NackList | NR_RLC_Status_NackListSN12Bit_Type | opt | zero or more sets of a NACK_SN, E1, E2 and E3 and possibly a pair of SOstart/SOend or NACK range field for each NACK_SN |

NR_RLC_Status_NackSN18Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_Status_NackSN18Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.5 (Figure 6.2.2.5-1) |  |  |
| SequenceNumberNACK | B18_Type |  | 18 bits SN |
| E1 | NR_RLC_Status_ExtensionBit1_Type |  | 1 bit E1 field; set if further NACK set follows |
| E2 | NR_RLC_Status_ExtensionBit2_Type |  | 1 bit E2 field |
| E3 | NR_RLC_Status_ExtensionBit3_Type |  | 1 bit E3 field |
| Reserved | B3_Type |  | 3 bits reserved |
| SOstart | NR_RLC_SegmentOffset_Type | opt | segment offset (start), present only if E2 is set to '1'B |
| SOstop | NR_RLC_SegmentOffset_Type | opt | segment offset (stop), present only if E2 is set to '1'B |
| NACKrange | B8_Type | opt | NACK range, present only if E3 is set to '1'B |

NR_RLC_Status_NackListSN18Bit_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_RLC_Status_NackListSN18Bit_Type |
| Comment |  |
| record of NR_RLC_Status_NackSN18Bit_Type |  |

NR_RLC_StatusPduSN18Bit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_RLC_StatusPduSN18Bit_Type |  |  |
| Comment | TS 38.322, clause 6.2.2.5 (Figure 6.2.2.5-1) |  |  |
| D_C | B1_Type |  | 1 bit, '0'B for Control PDU |
| CPT | B3_Type |  | 3 bits, TS 38.322, clause 6.2.3.9 Control PDU Type (CPT) field: '000'B STATUS PDU ELSE reserved |
| SequenceNumberACK | B18_Type |  | 18 bits SN |
| E1 | NR_RLC_Status_ExtensionBit1_Type |  | 1 bit E1 field |
| Reserved | B1_Type |  | 1 bit reserved |
| NackList | NR_RLC_Status_NackListSN18Bit_Type | opt | zero or more sets of a NACK_SN, E1, E2 and E3 and possibly a pair of SOstart/SOend or NACK range field for each NACK_SN |

NR_RLC_AM_StatusPDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RLC_AM_StatusPDU_Type |  |
| Comment | TS 38.322, clause 6.2.254 |  |
| SN12Bit | NR_RLC_StatusPduSN12Bit_Type |  |
| SN18Bit | NR_RLC_StatusPduSN18Bit_Type |  |

## D.2.2	DRB_Primitive_Definitions

Primitive definitions to send/receive data PDUs over DRB's

### D.2.2.1	DRB_Common

NR_L2DataList_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_L2DataList_Type |  |
| Comment | MAC: acc. to rel-15 protocols there is exactly one MAC PDU per TB but in case of spatial multiplexing there can be one or more TB per HARQ process; any MAC PDU is completely included in one slot (TTI) RLC: one or more RLC PDUs per slot (TTI) (e.g. RLC Data + Status PDU on a logical channel; more than one RLC Data PDU in one MAC PDU is valid too) any RLC PDU is completely included in one slot (TTI) PDCP: one or more PDUs per slot (TTI); one PDCP PDU may be included in more than one slot (TTI) |  |
| MacPdu | NR_MAC_PDUList_Type | SS configuration: RLC TM mode, MAC no header removal (PDCP is not configured) |
| RlcPdu | NR_RLC_PDUList_Type | SS configuration: RLC TM mode, MAC header removal (PDCP is not configured) |
| RlcSdu | NR_RLC_SDUList_Type | SS configuration: RLC UM mode with no PDCP |
| PdcpPdu | NR_PDCP_PDUList_Type | SS configuration: RLC AM/UM mode, no handling of PDCP header |
| PdcpSdu | NR_PDCP_SDUList_Type | SS configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header) |
| SdapPdu | SDAP_PDUList_Type | SS configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header), no handling of SDAP header |
| SdapSdu | SDAP_SDUList_Type | SS configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header), automatic handling of SDAP header |

NR_HarqProcessAssignment_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_HarqProcessAssignment_Type |  |
| Comment | in DL the HARQ process id may be specified by the test case or automatically assigned by SS |  |
| Id | NR_HarqProcessId_Type | HARQ process as specified by the test case NOTE1: the scope of this type is only for data being sent in one slot (TTI); if data needs more than one slot (TTI) the HarqProcessId is undefined for the 2nd TTI onward what shall be handled as an error at the SS; SS may send a SYSTEM_IND indicating an error in this case; NOTE2: The initial value of the NDI shall be the same for all HARQ processes and cells |
| Automatic | Null_Type | HARQ process id automatically assigned by SS |

### D.2.2.2	Downlink

NR_DRB_DataPerSlot_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DRB_DataPerSlot_DL_Type |  |  |
| Comment | common definition for one or several PDUs/SDUs; in NR the DL data is sent in the slot given by the slot offset NOTE 1: For MAC and RLC PDUs a single PDU is always sent in one slot; SS shall raise an error indication (using SYSTEM_IND) when that is not possible NOTE 2: For PDCP the data may be spread over more than one slot (segmented by the RLC); the TTCN implementation is responsible to calculate appropriate offsets accordingly; the exact timing depends on (and is exactly specified by) configuration of the DL scheduling; SS shall raise an error when there is any conflict |  |  |
| SlotOffset | integer |  | NR: Slot offset relative to the absolute timing information given in the common part of the ASP; NOTE: if a PDCP PDU or SDU takes more than one slot, SlotOffset specifies the first slot (TTI) |
| HarqProcess | NR_HarqProcessAssignment_Type | opt | HARQ process to be used: specific value or automatically assigned by SS; in automatic mode SS chooses HARQ process out of the set configured by DcchDtchConfig.DL.SearchSpaceAndDci.DciInfo.ResoureAssignment.HarqProcessConfig NOTE: for PDCP SDUs or PDUs automatic mode shall be used; otherwise SS shall raise an error |
| PduSduList | NR_L2DataList_Type |  | list of PDUs/SDUs to be sent in one slot (TTI) |

NR_DRB_DataPerSlotList_DL_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_DRB_DataPerSlotList_DL_Type |
| Comment | list of user plane data to be sent in slots given by the SlotOffset in the single elements of the list; Timing: the start time for the whole sequence is given by the timing info of the ASP (common information); the timing for the respective data pdus is given by the SlotOffset relative to the common timing info; design consideration: repetitions of this sequence are not foreseen (in which case the slot offset could not be related to the timing info of the ASP) |
| record of NR_DRB_DataPerSlot_DL_Type |  |

NR_L2Data_Request_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_L2Data_Request_Type |  |  |
| Comment | NOTE: formal type definition to allow later enhancements |  |  |
| SlotDataList | NR_DRB_DataPerSlotList_DL_Type |  |  |

### D.2.2.3	Uplink

NR_DRB_DataPerSlot_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DRB_DataPerSlot_UL_Type |  |  |
| Comment | common definition for one or several PDUs/SDUs being received in one slot or to receive one PDCP PDU or SDU being spread over more than one slot (TTI); NOTE: There is a fix relation between HARQ process id and slot in UL => it is not necessary to include HARQ process id for UL data |  |  |
| PduSduList | NR_L2DataList_Type |  | list of PDUs/SDUs being received in one TTI; elements of the list appear in the same order as the PDUs/SDUs in the MAC PDU; for PDCP when a PDU or SDU takes more than one TTI the list only contains this PDU or SDU |
| NoOfTTIs | integer |  | in case of PDCP: number of TTIs the SDU or PDU has taken NOTE 1: for the time being the NoOfTTIs is not checked by TTCN-3 and may be set to 1 by SS; NOTE 2: the timing info in common part of the ASP refers to the last TTI NOTE 3: when NoOfTTIs > 1 => PduSduList shall only contain one PDCP PDU or SDU in case of MAC or RLC PDUs: NoOfTTIs shall always be 1 |

NR_L2Data_Indication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_L2Data_Indication_Type |  |  |
| Comment | NOTE: formal type definition to allow later enhancements; L2Data_Indication_Type defines data being received in a single slot i.e. PDUs of subsequent TTIs are indicated in separated ASPs |  |  |
| SlotData | NR_DRB_DataPerSlot_UL_Type |  |  |

## D.2.3	System_Interface

NR_DRB_COMMON_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DRB_COMMON_REQ |  |  |
| Comment | common ASP to send PDUs to DRBs |  |  |
| Common | NR_ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : QosFlow when SDAP is configured in non-transparent mode at the SS, else DRB id RlcBearerRouting : TTCN provides the id of the cell hosting the RLC bearer, which the SS shall use to send the data out to the UE (for non-split bearers in general same as CellId) MacBearerRouting : If NR CA is configured, TTCN provides the id of the cell hosting the physical layer, which the SS shall use to send the data out to the UE. Else 'omit' TimingInfo : starting point when to start sending sequence of data PDUs e.g. SFN = X, subframe number = x; slot number = slot_i depending on numerology U_Plane.SubframeDataList[i].SlotOffset := offset_i; => U_Plane.SubframeDataList[0].PduSduList shall be sent out at (X, x, i) U_Plane.SubframeDataList[i].PduSduList shall be sent out offset_i slots after U_Plane.SubframeDataList[0].PduSduList (depending on the numerology) NOTE: In case of K0>0 (K0 according to 38.214 clause 5.1.2.1) the timing specifies the PDCCH assignment (rather than the PDSCH transmission) ControlInfo : CnfFlag:=false; FollowOnFlag:=false |
| U_Plane | NR_L2Data_Request_Type |  |  |
| SuppressPdcchForC_RNTI | Null_Type | opt | By default all DRB_COMMON_REQ scheduled DL PDU's are associated with an appropriate explicit configured or SS selected DL assignment allocation on PDCCH. For SuppressPdcch:=true in the sub frame in which DL PDU's are transmitted, there is no associated DL assignment allocation for configured C-RNTI. This will be used for SPS assignment based transmission or in any error scenarios; NOTE: this flag has no impact on PDCCH messages required for SPS activation |

NR_DRB_COMMON_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_DRB_COMMON_IND |  |  |
| Comment | common ASP to receive PDUs from DRBs |  |  |
| Common | NR_IndAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : QosFlow when SDAP is configured in non-transparent mode at the SS, else DRB id RoutingInfoSUL : The SS shall include this field when data is received from SUL carrier. Else 'omit'. RlcBearerRouting : The SS shall provide the id of the cell hosting the RLC bearer in which the data has been received from the UE MacBearerRouting : If NR CA is configured, the SS shall provide the id of the cell hosting the physical layer in which the data has been received from the UE. Else 'omit' TimingInfo : time when message has been received NOTE 1: For MAC and RLC PDUs per definition L2Data_Indication_Type corresponds to exactly one slot => TimingInfo refers to this slot NOTE 2: For PDCP a single PDU or SDU may take more than one TTI => TimingInfo refers to the end of the PDU/SDU and the length is given by NoOfTTIs in L2Data_Indication_Type (the end of the PDU/SDU is the last RLC PDU being received; in case of retransmissions this is not necessarily the RLC PDU with the last SN) |
| U_Plane | NR_L2Data_Indication_Type |  |  |

NR_DRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NR_DRB_PORT |  |
| Comment |  |  |
| out | NR_DRB_COMMON_REQ |  |
| in | NR_DRB_COMMON_IND |  |

# D.3	NR_ASP_SrbDefs

## D.3.1	SRB_DATA_ASPs

ASP Definitions to send/receive peer-to-peer messages on SRBs

NR_C_Plane_Request_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_C_Plane_Request_Type |  |  |
| Comment | RRC and/or NAS PDU to be send to the UE; Note: it may be necessary to allow more than one NAS PDU (-> "record of") |  |  |
| Rrc | NR_RRC_MSG_Request_Type | opt | omit: NAS message shall be present; NAS message shall be sent in DLInformationTransfer present: if NAS message is present also, (piggybacked) NAS PDU shall be security protected (if necessary) and inserted in RRC PDU's DedicatedInfoNAS |
| Nas | NG_NAS_MSG_RequestList_Type | opt | omit: RRC message shall be present; RRC message does not contain (piggybacked) NAS PDU present: if RRC message is omitted => NAS message shall be sent embedded in DLInformationTransfer if RRC message is present => NAS message is piggybacked in RRC message in case of RRC message is sent on CCCH, NAS message shall be omitted |

NR_C_Plane_Indication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_C_Plane_Indication_Type |  |  |
| Comment | RRC and/or NAS PDU to be received from the UE; Note: it may be necessary to allow more than one NAS PDU (-> "record of") |  |  |
| Rrc | NR_RRC_MSG_Indication_Type | opt | omit: NAS message shall be present; NAS message is received in ULInformationTransfer present: if NAS message is present also, DedicatedInfoNAS contains unstructured and ciphered NAS message and the NAS message is the deciphered message in structured format |
| Nas | NG_NAS_MSG_IndicationList_Type | opt | omit: RRC message shall be present; RRC message does not contain (piggybacked) NAS PDU present: if RRC message is omitted => NAS message has been received in ULInformationTransfer if RRC message is present => NAS message has been piggybacked in RRC message NOTE: even though currently (DEC 08 ASN.1) there is no RRC PDU in UL containing more than one DedicatedInfoNAS we provide a list to allow extendability |

NR_SRB_COMMON_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SRB_COMMON_REQ |  |  |
| Comment | common ASP to send PDUs to SRB0, SRB1 or SRB2 |  |  |
| Common | NR_ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB0, SRB1, SRB2, SRB3 RlcBearerRouting : TTCN provides the id of the cell hosting the RLC bearer which the SS shall use to send the data out to the UE (for non-split bearers in general same as CellId) MacBearerRouting : If NR CA is configured, TTCN provides the id of the cell hosting the physical layer, which the SS shall use to send the data out to the UE. Else 'omit' TimingInfo : Now in normal cases; For latency tests TimingInfo can be set to the SFN/subframe in which the RRC messages shall be sent out (in this case and if the RRC PDU is too long to be sent in one TTI the TimingInfo corresponds to the first TTI) ControlInfo CnfFlag:=false; FollowOnFlag true: Indicates that the message(s) to be sent on the same TTI will follow NOTE 1: When FollowOnFlag is true, TimingInfo shall always be "Now". Otherwise SS shall produce an error NOTE 2: the FollowOnFlag applies only for messages of the same SRB false: Indicates that no more message(s) will follow |
| Signalling | NR_C_Plane_Request_Type |  |  |

NR_SRB_COMMON_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SRB_COMMON_IND |  |  |
| Comment | common ASP to receive PDUs from SRB0, SRB1 or SRB2 |  |  |
| Common | NR_IndAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB0, SRB1, SRB2, SRB3 RoutingInfoSUL : The SS shall include this field when data is received from SUL carrier. Else 'omit'. RlcBearerRouting : The SS shall provide the id of the cell hosting the RLC bearer in which the data has been received from the UE MacBearerRouting : If NR CA is configured, the SS shall provide the id of the cell hosting the physical layer in which the data has been received from the UE. Else 'omit' TimingInfo : time when message has been received (as received from the SS by the NAS emulator) |
| Signalling | NR_C_Plane_Indication_Type |  |  |

## D.3.2	Port_Definitions

NR_SRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NR_SRB_PORT |  |
| Comment | NR PTC: Port for Sending/Receiving data on SRBs |  |
| out | NR_SRB_COMMON_REQ |  |
| in | NR_SRB_COMMON_IND |  |

NASEMU_NR_SRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NASEMU_NR_SRB_PORT |  |
| Comment | NASEMU PTC: Port for Sending/Receiving data on SRBs (interface to NR PTC) |  |
| out | NR_SRB_COMMON_IND |  |
| in | NR_SRB_COMMON_REQ |  |

# D.4	NR_CommonDefs

## D.4.1	Common_Types

Common_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_HarqProcessId_Type | integer | HARQ process id; NOTE: there seems to be no need for any value restriction |
| NR_AbsoluteCellPower_Type | integer (-150..0) | absolute cell power (dBm) |

NR_RRC_MSG_Request_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RRC_MSG_Request_Type |  |
| Comment | DL RRC PDU on CCCH or DCCH |  |
| Ccch | DL_CCCH_Message |  |
| Dcch | DL_DCCH_Message |  |

NR_RRC_MSG_Indication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RRC_MSG_Indication_Type |  |
| Comment | UL RRC PDU on CCCH or DCCH |  |
| Ccch | UL_CCCH_Message |  |
| Ccch1 | UL_CCCH1_Message |  |
| Dcch | UL_DCCH_Message |  |

NR_HarqProcessList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_HarqProcessList_Type |
| Comment | list of HARQ processes: each element shall be unique; NOTE: there seems to be no need for any length restriction |
| record of NR_HarqProcessId_Type |  |

## D.4.2	RRC_Nested_Types

RRC_Nested_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_PrioritizedBitRate_Type | LogicalChannelConfig.ul_SpecificParameters.prioritisedBitRate |  |
| NR_SiPeriodicity_Type | SchedulingInfo.si_Periodicity |  |
| NR_PosSI_Periodicity_Type | PosSchedulingInfo_r16.posSI_Periodicity_r16 |  |
| NR_SiWindowLength_Type | SI_SchedulingInfo.si_WindowLength |  |

## D.4.3	ASP_CommonPart

Definition of ASP common parts for REQ-, CNF- and IND-ASPs

### D.4.3.1	ASP_CommonPart_Definitions

#### D.4.3.1.1	Routing_Info

Routing_Info: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_RoutingInfoSUL_Type | Null_Type |  |

NR_RadioBearerId_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RadioBearerId_Type |  |
| Comment |  |  |
| Srb | SRB_Identity_Type |  |
| Drb | DRB_Identity |  |

NR_RoutingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_RoutingInfo_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| RadioBearerId | NR_RadioBearerId_Type | routing of signalling and user plane data when SDAP is not configured at the SS for the DRB or it is configured in transparent mode |
| QosFlow | QosFlow_Identification_Type | routing of user plane data with SDAP being configured in non-transparent mode at the SS |

MacBearerRouting_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MacBearerRouting_Type |  |  |
| Comment | Physical layer cell data routing. Applicable for NR CA, multi-RAT Dual Connectivity (MR-DC) with NR CA as well as single-RAT Dual Connectivity with NR CA |  |  |
| NR | NR_CellId_Type |  |  |

### D.4.3.2	REQ_ASP_CommonPart

NR_ReqAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_ReqAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | NR_CellId_Type |  |  |
| RoutingInfo | NR_RoutingInfo_Type |  |  |
| RlcBearerRouting | RlcBearerRouting_Type |  |  |
| MacBearerRouting | MacBearerRouting_Type | opt |  |
| TimingInfo | TimingInfo_Type |  |  |
| ControlInfo | ReqAspControlInfo_Type |  |  |

### D.4.3.3	CNF_ASP_CommonPart

NR_CnfAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_CnfAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | NR_CellId_Type |  |  |
| RoutingInfo | NR_RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| Result | ConfirmationResult_Type |  |  |

### D.4.3.4	IND_ASP_CommonPart

NR_IndAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_IndAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | NR_CellId_Type |  |  |
| RoutingInfo | NR_RoutingInfo_Type |  |  |
| RoutingInfoSUL | NR_RoutingInfoSUL_Type | opt |  |
| RlcBearerRouting | RlcBearerRouting_Type |  |  |
| MacBearerRouting | MacBearerRouting_Type | opt |  |
| TimingInfo | TimingInfo_Type |  |  |
| Status | IndicationStatus_Type |  |  |

# D.5	IP_ASP_TypeDefs

General Notes:
NOTE 1:
In general the handling of IP data shall be independent from the RAT being used on lower layers.
NOTE 2:
It shall be possible for SS implementation to reuse existing IP stack implementations in the system adaptor;
therefore the well-known concept of socket programming shall be supported
(regardless of whether those are used in the system adaptor implementation or not)
NOTE 3:
Since in general at the network side there are several different IP addresses the SS needs to simulate more than one IP address;
that can be based on a concept of multiple virtual network adaptors
NOTE 4:
There is no easy way to control the routing of IP data for an IP connection from above the IP stack
i.e. there are no parameters at the socket interface to determine e.g. cell id and DRB id
=> another independent logical entity (DRB-MUX) is needed below the IP stack which is responsible to control the routing of IP packets from/to DRBs in different cells of different RATs

Reference:
An introduction to socket programming can be found in
UNIX Network Programming Volume 1, Third Edition: The Sockets Networking API
by W. Richard Stevens, Bill Fenner, Andrew M. Rudoff

## D.5.1	IP_Common

IP_Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| PortNumber_Type | UInt16_Type |  |

IPv4_AddrInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPv4_AddrInfo_Type |  |  |
| Comment | IPv4 specific info of the socket addr (AF_INET) |  |  |
| Addr | charstring |  | IP Address as string (IP v4 dot notation) to be converted to 32-bit unsigned integer |

IPv6_AddrInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPv6_AddrInfo_Type |  |  |
| Comment | IPv6 specific info of the socket addr (AF_INET6); NOTE: sin6_flowinfo can be ignored and set to 0 |  |  |
| Addr | charstring |  | to be converted to sin6_addr |
| ScopeId | UInt32_Type | opt | sin6_scope_id in general an IPv6 address is like "fe80::1%eth0" with eth0 being the network adaptor mapped to a scope id (Unix) assumption: for UE conformance testing it is not necessary to distinguish different scopes and the scope id in general can be determined by the system adaptor => omit |

IP_AddrInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_AddrInfo_Type |  |
| Comment |  |  |
| V4 | IPv4_AddrInfo_Type |  |
| V6 | IPv6_AddrInfo_Type |  |

IP_Socket_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_Socket_Type |  |  |
| Comment | Socket |  |  |
| IpAddr | IP_AddrInfo_Type | opt | IP address |
| Port | PortNumber_Type | opt | port number |

InternetProtocol_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | InternetProtocol_Type |
| Comment |  |
| udp |  |
| tcp |  |
| icmp |  |
| icmpv6 |  |

IP_Connection_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_Connection_Type |  |  |
| Comment | A connection between peer-to-peer entities is unambiguously defined by the protocol (udp/tcp/icmp/icmpv4), the local socket and the remote socket |  |  |
| Protocol | InternetProtocol_Type |  |  |
| Local | IP_Socket_Type | opt |  |
| Remote | IP_Socket_Type | opt |  |

## D.5.2	IP_Config

Configuration of the routing table managed be the system adaptor's DRB-MUX:
foreach IP connection it is specified which
- RAT
- Cell
- DRB
to be used.
The IP connection does not need to be fully specified depending on the role SS plays (e.g. in case of a server role the port number of the remote side is not known in advance).
The configurations of DRBs within the same cell shall be mutual exclusive.

With the configuration of the IP routing the DRB is configured either in IP or in raw mode:
either there are entries for the DRB in the routing table (IP mode) or not (raw mode)
=> It is not necessary to reconfigure this for the respective RAT.

Behaviour of the DRB-MUX in UL:
- SS gets data packet from the lower layers (e.g. PDCP SDU)
- SS checks whether there is any IP connection configured for this DRB (identified by {RAT, CellId, DrbId})
  if YES => packet is routed to the IP stack (IP mode)
  if NO  => packet is handed over to the DRB port (raw mode)
NOTE 1:
If there is any entry for a DRB in the routing table this DRB is considered as being in IP mode and all UL IP packets are sent to the IP stack regardless of whether their addresses match the DRB's routing entries or not (in general 'unknown' packets are discarded by the IP stack)
=> a DRB can be either in IP or in raw mode
NOTE 2:
=> The SS does not need to evaluate any IP headers to decide whether data shall be routed to the DRB port or to the IP stack (i.e. there is no conflict with unstructured loopback data)

Behaviour of the DRB-MUX in DL:
- SS gets IP packets from the IP stack for an IP connection
- SS compares the IP connection (protocol, local/remote IP Addr) against the IP routing table and
  checks whether the corresponding protocol stack is configured at the lower layers  =>
  1. no match:
     no entry in the routing table fits to the address in the IP packet
     or the corresponding RB is not configured
     => SS shall raise an error (DRBMUX_COMMON_IND_CNF.Error)
  2. one match:
     There is exactly one possibility to route the IP packet
     => SS shall send the packet to this RB
  3. several matches:
     There are more than one DRBs, cells or RATs to which the packet may be routed
     => SS shall raise an error if there is more than one DRB in one cell matching;
        if the DRBs belong to different cells or RATs SS shall send the data to all of them
        (whether this may occur in test cases is FFS)

General notes:
NOTE 1:
SS may use the information of the routing table to determine which network adaptors it needs to simulate (implementation dependent);
in general there will be more than one IP address at the network side.
NOTE 2:
In general the routing table is a simplified DL TFT implementation
NOTE 3:
When the routing table is empty all DRBs are in raw mode; this shall be the initial condition at the DRB-MUX;
=> for L2 testing in general (and apart from the preamble) there is no need to use/configure the IP_PTC; the configuration of the RAT specific U-plane stacks is not affected

IP_DataMode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IP_DataMode_Type |
| Comment |  |
| discard |  |
| loopbackRTP |  |
| loopbackRTCP |  |
| IPsecTunnel |  |

IP_RoutingInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_RoutingInfo_Type |  |  |
| Comment |  |  |  |
| IpInfo | IP_Connection_Type |  | IP connection tuple: protocol, local socket, remote socket depending on the role the SS plays the following information may be provided (informative; even less information can be sufficient): 1. TCP/UDP server - local IP addr -- provided - local port -- provided - remote IP addr -- omit - remote port -- omit 2. TCP/UDP client - local IP addr -- provided (to inform SS about the local IP addr for this service) - local port -- omit; for UDP a well-defined port may be defined (protocol dependent, e.g. DHCP) - remote IP addr -- provided - remote port -- provided 3. ICMP (in general ICMP may be mapped only to a single DRB) - local IP addr -- provided (to inform SS about the local IP addr for this service) - local port -- n/a (shall be set to omit) - remote IP addr -- omit - remote port -- n/a (shall be set to omit) NOTE: In case of broadcasts in UL the broadcast address shall match any local IP address; in DL for broadcast services typically no remote IP address is specified in the routing table |
| DRB | IP_DrbInfo_Type |  |  |
| DataMode | IP_DataMode_Type | opt | present when IP packets matching this entry shall be discarded or be looped back to the UE as defined for RTP or RTCP |

IP_RoutingTable_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | IP_RoutingTable_Type |
| Comment | NOTE: configurations of DRBs within the same cell shall be mutual exclusive |
| record of IP_RoutingInfo_Type |  |

## D.5.3	IPsec_Config

IP_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_IPsec_SPI_Max | integer | 4294967295 |  |

IPsec_Config: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| IPsec_SPI_Type | integer (0..tsc_IPsec_SPI_Max) | security parameter index for IPsec; According to RFC 2406, SPI values from 0 to 255 are reserved |

IPsec_IntegrityAlgorithm_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IPsec_IntegrityAlgorithm_Type |
| Comment |  |
| hmac_md5_96 |  |
| hmac_sha_1_96 |  |

IPsec_CipheringAlgorithm_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IPsec_CipheringAlgorithm_Type |
| Comment |  |
| des_ede3_cbc |  |
| aes_cbc |  |
| nociph | no ciphering |

IPsec_SecurityKeys_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPsec_SecurityKeys_Type |  |  |
| Comment | to install the security keys |  |  |
| MD5_96Key | bitstring length (128) |  |  |
| SHA_1_96Key | bitstring length (160) |  |  |
| DES_EDE3_CBCKey | bitstring length (192) |  |  |
| AES_CBCKey | bitstring length (128) |  |  |

IPsec_SecurityAssociation_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPsec_SecurityAssociation_Type |  |  |
| Comment | single security association (SA); for configuration of an SA at the SS all fields are mandatory; to release an SA the optional information is omitted |  |  |
| SPI | IPsec_SPI_Type |  |  |
| SrcAddress | charstring |  |  |
| DestAddress | charstring |  |  |
| SrcPort | UInt16_Type |  |  |
| DestPort | UInt16_Type |  |  |
| IntegrityAlgorithm | IPsec_IntegrityAlgorithm_Type | opt | mandatory to set-up an SA |
| CipheringAlgorithm | IPsec_CipheringAlgorithm_Type | opt | mandatory to set-up an SA |

IPsec_SecurityAssociationList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | IPsec_SecurityAssociationList_Type |
| Comment |  |
| record of IPsec_SecurityAssociation_Type |  |

IPsec_Configure_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPsec_Configure_Type |  |  |
| Comment | add new security associations; existing SAs are not affected |  |  |
| SA_List | IPsec_SecurityAssociationList_Type |  |  |
| SecurityKeys | IPsec_SecurityKeys_Type |  |  |

IPsec_Release_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IPsec_Release_Type |  |  |
| Comment | release security associations; NOTE: in context with multiple PDNs it cannot be ensured that all SPIs are unique; e.g. the UE may use the same SPI values in different PDNs in which case uniqueness cannot be achieved furthermore it depends on the system implementation how entries in the IPsec SAD and SPD are administrated => to release SAs the SS gets the same information as for configuration but without the security algorithms |  |  |
| SA_List | IPsec_SecurityAssociationList_Type |  |  |

## D.5.4	IP_SocketHandling

Handling of IP data and IP connections
NOTE 1:
In general IP connections are distinguished by the tuple {protocol, local socket, remote socket};
this information is used at the interface between TTCN and the system adaptor.
It is up the system adaptor implementation to associate the IP connection with the internal socket (file descriptor; implementation dependent)
NOTE 2:
In general the association of the IP connections to (internal) sockets and the routing table for the DRB mapping (as configured with IP_RoutingTable_Type) are independent from each other

### D.5.4.1	Socket_Common

IP_SockOpt_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_SockOpt_Type |  |
| Comment | socket options acc. to the setsockopt system call (i.e. for level=SOL_SOCKET in case of Berkeley socket API); NOTE: only options being relevant for a specific applications (upon a socket) are configured by TTCN other options (e.g. SO_REUSEADDR) are out of TTCN and therefore a matter of system adaptor implementation |  |
| SO_BROADCAST | boolean | set to true when IP broadcast messages shall be allowed for a port; this is required e.g. in case of DHCP |
| IP_MTU_SIZE | integer | MTU size to be used for IP data; NOTEs: - Even though the MTU size is defined as socket option it shall be the same for all sockets of a given interface (i.e. at least within one PDN the MTU size shall be the same) - in general a PIXIT is used as constant value for all sockets |

IP_SockOptList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | IP_SockOptList_Type |
| Comment |  |
| record of IP_SockOpt_Type |  |

IP_SocketError_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_SocketError_Type |  |
| Comment | used to indicate errors related to sockets; the IP_Connection shall contain as much address information as available at the system adaptor |  |
| InvalidAddress | Null_Type | TTCN error: e.g. invalid or incomplete address information |
| System | integer | system error caused by system call; the integer value may be used for validation but shall not be evaluated by TTCN |

### D.5.4.2	Socket_Datagram

Socket_Datagram: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| Datagram_Content_Type | octetstring | data as sent/received with sendto()/recvfrom() on UDP or ICMP socket; NOTE: For ICMP the data may depend on the socket options; in general it does not include the IP header and the checksum of the ICMP packet needs to be calculated/checked in TTCN |

Datagram_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Datagram_DL_Type |  |  |
| Comment | datagram to be sent at a UDP or ICMP socket |  |  |
| Buffer | Datagram_Content_Type |  | content of the IP packet |

Datagram_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | Datagram_UL_Type |  |  |
| Comment | datagram as received on a UDP or ICMP socket |  |  |
| Buffer | Datagram_Content_Type |  | content of the IP packet |
| DrbInfo | IP_DrbInfo_Type | opt | "interface id" where the data comes from in case of broadcast or multicast packets: for the LTE test model this is the DRB on which the IP packet has been received; the information is necessary when the SS cannot resolve an IP address being assigned to that DRB. => when the SS provides a broadcast or multicast address as local address in the ConnectionId of the ASP, the SS shall provide the DRB information in this field When the ConnectionId of the ASP is fully specified and unique (unicast address at least for local address) the DrbId is ignored by TTCN |

### D.5.4.3	TCP_Socket

TCP primitives used on the IP port

TCP_Socket: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| TCP_Data_Type | octetstring | data as sent/received with send()/recv() on a TCP socket |

InternetApplication_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | InternetApplication_Type |
| Comment | as TCP is stream oriented SS may need information about which criteria to be applied to get start/end of an application message |
| ims |  |
| http |  |
| msrp |  |

TLS_CIPHER_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | TLS_CIPHER_Type |
| Comment | Cipher suite to be used for TLS |
| TLS_PSK_WITH_RC4_128_SHA | RFC 4279 |
| TLS_PSK_WITH_3DES_EDE_CBC_SHA | RFC 4279 |
| TLS_PSK_WITH_AES_128_CBC_SHA | RFC 4279 |
| TLS_PSK_WITH_AES_256_CBC_SHA | RFC 4279 |
| TLS_PSK_WITH_AES_128_CBC_SHA256 | RFC 5487 |
| TLS_RSA_WITH_NULL_MD5 | RFC 5246 |
| TLS_RSA_WITH_NULL_SHA | RFC 5246 |
| TLS_RSA_WITH_NULL_SHA256 | RFC 5246 |
| TLS_RSA_WITH_RC4_128_MD5 | RFC 5246 |
| TLS_RSA_WITH_RC4_128_SHA | RFC 5246 |
| TLS_RSA_WITH_3DES_EDE_CBC_SHA | RFC 5246 |
| TLS_RSA_WITH_AES_128_CBC_SHA | RFC 5246 |
| TLS_RSA_WITH_AES_256_CBC_SHA | RFC 5246 |
| TLS_RSA_WITH_AES_128_CBC_SHA256 | RFC 5246 |
| TLS_RSA_WITH_AES_256_CBC_SHA256 | RFC 5246 |

PSK_BootstrappingInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PSK_BootstrappingInfo_Type |  |  |
| Comment | bootstrapping information as defined in 24.109 |  |  |
| BTid | charstring |  |  |
| Ks_NAF | bitstring |  |  |

TLS_PSK_Info_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TLS_PSK_Info_Type |  |  |
| Comment | configuration information for PSK TLS |  |  |
| IdentityHint | charstring |  |  |
| BootstrappingInfo | PSK_BootstrappingInfo_Type |  |  |

TLS_CipherSuiteInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TLS_CipherSuiteInfo_Type |  |
| Comment |  |  |
| psk | TLS_PSK_Info_Type |  |

TLSPSKInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TLSPSKInfo_Type |  |  |
| Comment |  |  |  |
| cipherSuite | TLS_CIPHER_Type |  | Cipher suite to be used |
| cipherSuiteInfo | TLS_CipherSuiteInfo_Type |  | parameters for the respective cipher suite |

TLSCertificateInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TLSCertificateInfo_Type |  |  |
| Comment |  |  |  |
| certificate | charstring |  | name or file location of the certificate to be used by the SS; in general provided by a PIXIT: the PIXIT contains the SS implementation specific information for the SS to identify the certificate to be used for a particular TLS connection; in case of only one certificate being used by TTCN, the string may be empty |
| cipherSuite | TLS_CIPHER_Type |  | (non-PSK) cipher suite to be used for the TLS tunnel; the SS shall raise an error when it detects that the UE does not support the configured cipher suite |

TLSConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TLSConfig_Type |  |
| Comment |  |  |
| pskInfo | TLSPSKInfo_Type | Used in the case of PSK |
| certificateInfo | TLSCertificateInfo_Type | Used in the case of certificate based TLS |

TCP_ConnectRequest_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TCP_ConnectRequest_Type |  |  |
| Comment | TCP client: -> 'connect' system call |  |  |
| SockOptList | IP_SockOptList_Type |  | when there are no options to configure the list is empty |
| Application | InternetApplication_Type |  | to specify start/end criteria for application messages |

TCP_Listen_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TCP_Listen_Type |  |  |
| Comment | TCP server: -> 'listen' system call |  |  |
| SockOptList | IP_SockOptList_Type |  | when there are no options to configure the list is empty |
| Application | InternetApplication_Type |  | to specify start/end criteria for application messages |
| TLSConfig | TLSConfig_Type | opt | to support TLS for HTTP server implementation |

TCP_CtrlRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TCP_CtrlRequest_Type |  |
| Comment |  |  |
| ConnectReq | TCP_ConnectRequest_Type | request a 'connect' to a remote server system calls (informative) socket -- get file descriptor (setsockopt) -- normally not needed bind -- assign local IP addr (to cope with multiple IP addresses) and dedicated port number (if local port is given) connect -- connect to the client IP_Connection: protocol -- tcp local IP addr -- mandatory to distinguish different network adaptors local port -- omit (ephemeral port will be assigned by the system) or specific port to be used for this connection (e.g. to bind a given port number to the IMS client) remote IP addr -- mandatory remote port -- mandatory |
| Listen | TCP_Listen_Type | establish a server at the local (SS) side system calls (informative) socket -- get file descriptor (setsockopt) -- if needed bind -- assign local IP addr and port listen -- await incoming connection IP_Connection: protocol -- tcp local IP addr -- mandatory to distinguish different network adaptors local port -- mandatory remote IP add -- omit remote port -- omit |
| Close | Null_Type | close a connection system calls (informative): close IP_Connection: protocol -- tcp local IP addr -- mandatory local port -- mandatory remote IP addr -- mandatory for TCP connections, omit for TCP server remote port -- mandatory for TCP connections, omit for TCP server |

TCP_DataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TCP_DataRequest_Type |  |
| Comment |  |  |
| Send | TCP_Data_Type | send data system calls (informative): send or write IP_Connection: protocol -- tcp local IP addr -- mandatory local port -- mandatory remote IP addr -- mandatory remote port -- mandatory |

TCP_CtrlIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TCP_CtrlIndication_Type |  |
| Comment |  |  |
| ConnectCnf | Null_Type | confirm a 'connect' to a remote server system calls (informative): getsockname -- get local port (ephemeral port assigned by the system) IP_Connection: protocol -- tcp local IP addr -- mandatory (as in corresponding TCP_ConnectRequest) local port -- mandatory (if there is more than one connection to the same server the local port is necessary to distinguish the connections) remote IP addr -- mandatory (as in corresponding TCP_ConnectRequest) remote port -- mandatory (as in corresponding TCP_ConnectRequest) |
| Accept | Null_Type | sent by the SS when it 'accepts' an incoming connection system calls (informative): accept IP_Connection: protocol -- tcp local IP addr -- mandatory (as in corresponding TCP_ListenRequest) local port -- mandatory (as in corresponding TCP_ListenRequest) remote IP addr -- mandatory (as gotten from 'accept') remote port -- mandatory (as gotten from 'accept') |
| Close | Null_Type | indicate 'close' by the remote side system calls (informative): indicated by recv or read IP_Connection: protocol -- tcp local IP addr -- mandatory local port -- mandatory remote IP addr -- mandatory remote port -- mandatory |
| CloseCnf | Null_Type | Confirmation for 'close' request; necessary since for TCP there are IP packets to release the connection system calls (informative): close IP_Connection: protocol -- tcp local IP addr -- mandatory local port -- mandatory remote IP addr -- mandatory for TCP connections, omit for TCP server remote port -- mandatory for TCP connections, omit for TCP server |

TCP_DataIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TCP_DataIndication_Type |  |
| Comment |  |  |
| Recv | TCP_Data_Type | receive data system calls (informative): recv or read IP_Connection: protocol -- tcp local IP addr -- mandatory local port -- mandatory remote IP addr -- mandatory remote port -- mandatory |

### D.5.4.4	UDP_Socket

UDP primitives used on the IP port
NOTE:
In principle a UDP socket may communicate with different remote entities;
therefore the system adaptor may associate the socket handle with the local socket only
(local IP address and local port)

UDP_SocketReq_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UDP_SocketReq_Type |  |  |
| Comment | to establish a UDP server or to bind local port number |  |  |
| SockOptList | IP_SockOptList_Type |  | e.g. to allow broadcast messages; when there are no options to configure the list is empty |

UDP_CtrlRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UDP_CtrlRequest_Type |  |
| Comment |  |  |
| SocketReq | UDP_SocketReq_Type | request the system adaptor to bind a socket to a local address; this is needed in general when the system adaptor acts as 1. UDP server 2. UDP client when it uses a well-known port rather than an ephemeral port (this is e.g. for DHCP) 3. UDP client when a local address needs to be bond (e.g. when there are several local addresses) system calls (informative): socket -- get file descriptor (setsockopt) -- needed e.g. to allow broad cast message bind -- assign local IP address (to cope with multiple IP addresses) and local port (in case of well-known local port) IP_Connection: protocol -- udp local IP addr -- mandatory (to distinguish multiple IP addresses) local port -- optional (mandatory in case of a UDP server) remote IP addr -- omit remote port -- omit |
| Close | Null_Type | release local socket system calls (informative): close IP_Connection: protocol -- udp local IP addr -- mandatory (to identify local socket) local port -- mandatory (to identify local socket) remote IP addr -- omit remote port -- omit |

UDP_DataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UDP_DataRequest_Type |  |
| Comment |  |  |
| SendTo | Datagram_DL_Type | send data to (any) remote socket; NOTE: To simplify implementation of the system adaptor the local socket shall be bond in any case (using 'SocketReq') to specify the local IP address before sending data; (in general the sendto system call can be used without explicitly binding the socket before; in this case the port gets implicitly bond to an ephemeral port and the default IP address is used) system calls (informative): sendto IP_Connection: protocol -- udp local IP addr -- mandatory (to identify local socket) local port -- mandatory (to identify local socket) remote IP addr -- mandatory (to address remote socket) remote port -- mandatory (to address remote socket) |

UDP_CtrlIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UDP_CtrlIndication_Type |  |
| Comment |  |  |
| SocketCnf | Null_Type | confirm 'SocketReq' and tell TTCN about assignment of ephemeral port; system calls (informative): getsockname -- get local port (ephemeral port assigned by the system; not needed if local port is well-known) IP_Connection: protocol -- udp local IP addr -- mandatory local port -- mandatory (well-known or ephemeral port assigned by the system) remote IP addr -- omit remote port -- omit |

UDP_DataIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UDP_DataIndication_Type |  |
| Comment |  |  |
| RecvFrom | Datagram_UL_Type | receive data; system calls (informative): recvfrom -- get data and src addr IP_Connection: protocol -- udp local IP addr -- mandatory (see note) local port -- mandatory remote IP addr -- mandatory (as gotten from recvfrom) remote port -- mandatory (as gotten from recvfrom) NOTE: The UE may send a UDP packet as broadcast (IP Addr 255.255.255.255 - e.g. in case of DHCP) or multicast (e.g. ICMPv6) SS shall consider a broadcast address as matching every IP for UL and DL; the SS shall not replace the broadcast/multicast address by the local unicast address, but shall provide DRB information in RecvFrom; example: - SS gets DHCPDISCOVER with DEST_Addr=255.255.255.255 DEST_Port=67, SRC_Addr=0.0.0.0 SRC_Port=68 - TTCN gets DHCPDISCOVER with local Addr=(255.255.255.255 Port=67), remote Addr=(0.0.0.0 Port=68), DrbId=(LTE, cell1, DRB1) - TTCN sends DHCPOFFER with local Addr=(local IP Addr Port=67), remote Addr=(255.255.255.255 Port=68) |

### D.5.4.5	ICMP_Socket

ICMP primitives used on the IP port
NOTE:
the local side is identified by the protocol and in general by the local IP address

ICMP_SocketReq_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ICMP_SocketReq_Type |  |  |
| Comment | to establish a raw socket to send/receive ICMP packets |  |  |
| SockOptList | IP_SockOptList_Type |  | e.g. to set the IP_HDRINCL socket option (to include the IP header in the data buffer) when there are no options to configure the list is empty |

ICMP_CtrlRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ICMP_CtrlRequest_Type |  |
| Comment |  |  |
| SocketReq | ICMP_SocketReq_Type | request the system adaptor to open a raw socket (IPv4 or IPv6) system calls (informative): socket -- get file descriptor (IPPROTO_ICMP or IPPROTO_IPV6); (setsockopt) -- optional; to set socket options bind -- assign local IP address (to cope with multiple IP addresses) IP_Connection: protocol -- icmp or icmpv6 local IP addr -- mandatory (to distinguish multiple IP addresses) local port -- omit (not applicable for ICMP) remote IP addr -- omit remote port -- omit (not applicable for ICMP) |
| Close | Null_Type | release local socket system calls (informative): close IP_Connection: protocol -- icmp or icmpv6 local IP addr -- mandatory (to identify local socket) local port -- omit remote IP addr -- omit remote port -- omit |

ICMP_DataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ICMP_DataRequest_Type |  |
| Comment |  |  |
| SendTo | Datagram_DL_Type | send datagram system calls (informative): sendto IP_Connection: protocol -- icmp or icmpv6 local IP addr -- mandatory (to identify local socket) local port -- omit remote IP addr -- mandatory remote port -- omit |

ICMP_CtrlIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ICMP_CtrlIndication_Type |  |
| Comment |  |  |
| SocketCnf | Null_Type | confirm 'SocketReq' system calls (informative): (SocketCnf is sent when all system calls for SocketReq have been successful) IP_Connection: protocol -- icmp or icmpv6 local IP addr -- mandatory local port -- omit remote IP addr -- omit remote port -- omit |

ICMP_DataIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ICMP_DataIndication_Type |  |
| Comment |  |  |
| RecvFrom | Datagram_UL_Type | receive datagram system calls (informative): recvfrom -- get data and src addr IP_Connection: protocol -- icmp or icmpv6 local IP addr -- mandatory (see note) local port -- omit remote IP addr -- mandatory (as gotten from recvfrom) remote port -- omit NOTE: As for UDP there may be multicast/broadcast packets. In this case - as for UDP - the SS shall provide the DRB information in RecvFrom. |

### D.5.4.6	Socket_Primitives

IP_CtrlRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_CtrlRequest_Type |  |
| Comment |  |  |
| TCP | TCP_CtrlRequest_Type |  |
| UDP | UDP_CtrlRequest_Type |  |
| ICMP | ICMP_CtrlRequest_Type |  |

IP_DataRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_DataRequest_Type |  |
| Comment |  |  |
| TCP | TCP_DataRequest_Type |  |
| UDP | UDP_DataRequest_Type |  |
| ICMP | ICMP_DataRequest_Type |  |

IP_CtrlIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_CtrlIndication_Type |  |
| Comment |  |  |
| TCP | TCP_CtrlIndication_Type |  |
| UDP | UDP_CtrlIndication_Type |  |
| ICMP | ICMP_CtrlIndication_Type |  |
| Error | IP_SocketError_Type |  |

IP_DataIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_DataIndication_Type |  |
| Comment |  |  |
| TCP | TCP_DataIndication_Type |  |
| UDP | UDP_DataIndication_Type |  |
| ICMP | ICMP_DataIndication_Type |  |

## D.5.5	System_Interface

DRBMUX_CONFIG_REQ

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DRBMUX_CONFIG_REQ |  |
| Comment | NOTE 1: There is just one primitive to configure the whole routing table. It is not foreseen to add, remove or manipulate single entries but the table is managed in TTCN and completely configured on any change; (otherwise it might get complicated to identify single entries) NOTE 2: the SS's routing table shall be empty at the beginning and can be cleared by an empty record (DRBMUX_CONFIG_REQ.RoutingInfo = {}) NOTE 3: In general a reconfiguration of the routing table during a test case would be necessary only if an ephemeral port is needed to distinguish different routing (e.g. when there are several TCP connections of the same service routed to different DRBs) |  |
| RoutingInfo | IP_RoutingTable_Type |  |

DRBMUX_COMMON_IND_CNF

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | DRBMUX_COMMON_IND_CNF |  |
| Comment |  |  |
| Confirm | Null_Type | confirm DRBMUX_CONFIG_REQ |
| Error | Null_Type | indication of errors at the DRB-MUX: An Error shall be raised by the DRB-MUX e.g. in the following cases: - in DL when there are IP packets which cannot be routed to any DRB i.e. the IP packet does not match to any entry in the routing table or the corresponding RB is not configured - in DL when there are several DRBs possible for routing in the same cell |

IPSEC_CONFIG_REQ

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IPSEC_CONFIG_REQ |  |
| Comment |  |  |
| Configure | IPsec_Configure_Type |  |
| Release | IPsec_Release_Type |  |

IPSEC_CONFIG_CNF

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IPSEC_CONFIG_CNF |  |
| Comment |  |  |
| Confirm | Null_Type | confirm IPSEC_CONFIG_REQ |
| Error | Null_Type | to indicate invalid configuration of IPsec |

IP_SOCKET_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_SOCKET_CTRL_REQ |  |  |
| Comment |  |  |  |
| ConnectionId | IP_Connection_Type |  |  |
| Req | IP_CtrlRequest_Type |  |  |

IP_SOCKET_DATA_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_SOCKET_DATA_REQ |  |  |
| Comment |  |  |  |
| ConnectionId | IP_Connection_Type |  |  |
| Req | IP_DataRequest_Type |  |  |

IP_SOCKET_CTRL_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_SOCKET_CTRL_IND |  |  |
| Comment |  |  |  |
| ConnectionId | IP_Connection_Type | opt | shall be present at the system interface; maybe omit only intenally in inter-PTC coordination |
| Ind | IP_CtrlIndication_Type | opt | shall be present at the system interface; maybe omit only intenally in inter-PTC coordination |

IP_SOCKET_DATA_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_SOCKET_DATA_IND |  |  |
| Comment |  |  |  |
| ConnectionId | IP_Connection_Type |  |  |
| Ind | IP_DataIndication_Type |  |  |

IP_SOCKET_REQ

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_SOCKET_REQ |  |
| Comment |  |  |
| CTRL | IP_SOCKET_CTRL_REQ |  |
| DATA | IP_SOCKET_DATA_REQ |  |

IP_SOCKET_IND

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_SOCKET_IND |  |
| Comment |  |  |
| CTRL | IP_SOCKET_CTRL_IND |  |
| DATA | IP_SOCKET_DATA_IND |  |

IP_CONTROL_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | IP_CONTROL_PORT |  |
| Comment |  |  |
| out | DRBMUX_CONFIG_REQ |  |
| in | DRBMUX_COMMON_IND_CNF |  |

IPSEC_CONTROL_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | IPSEC_CONTROL_PORT |  |
| Comment |  |  |
| out | IPSEC_CONFIG_REQ |  |
| in | IPSEC_CONFIG_CNF |  |

IP_SOCKET_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | IP_SOCKET_PORT |  |
| Comment |  |  |
| out | IP_SOCKET_REQ |  |
| in | IP_SOCKET_IND |  |

# D.6	NR_PDCP_TypeDefs

## D.6.1	NR_PDCP_Config_Parameters

Parameters defined in or related to NR ASN.1 type PDCP-Config

NR_PDCP_SN_Size_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_PDCP_SN_Size_Type |
| Comment | PDCP Sequence Number |
| PDCP_SNLength12 | TS 38.323 clause 6.2.2.1 and clause 6.2.2.2 |
| PDCP_SNLength18 | TS 38.323 clause 6.2.2.3 |

NR_PDCP_DRB_HeaderCompression_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_DRB_HeaderCompression_Type |  |
| Comment | place holder for header compression |  |
| None | Null_Type |  |

NR_PDCP_DRB_Config_Parameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_DRB_Config_Parameters_Type |  |  |
| Comment | parameters corresponding to NR ASN.1 PDCP-Config.drb; the following parameter are not included and may be added if needed: - integer DiscardTimer (timer value in milliseconds) - boolean StatusReportRequired |  |  |
| SN_SizeUL | NR_PDCP_SN_Size_Type |  | PDCP-Config.drb.pdcp-SN-SizeUL |
| SN_SizeDL | NR_PDCP_SN_Size_Type |  | PDCP-Config.drb.pdcp-SN-SizeDL |
| HeaderCompression | NR_PDCP_DRB_HeaderCompression_Type |  | PDCP-Config.drb.headerCompression |
| IntegrityProtectionEnabled | boolean |  | PDCP-Config.drb.integrityProtection |
| OutOfOrderDelivery | boolean |  | PDCP-Config.drb.outOfOrderDelivery Set to true by default (regardless of the UE configuration), unless specifically specified otherwise in a test case. |

NR_PDCP_RB_Config_Parameters_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_RB_Config_Parameters_Type |  |
| Comment |  |  |
| Srb | Null_Type | no SRB specific parameters in NR ASN.1 PDCP-Config |
| Drb | NR_PDCP_DRB_Config_Parameters_Type | DRB specific parameters corresponding to NR ASN.1 PDCP-Config.drb |

NR_PDCP_Config_Parameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_Config_Parameters_Type |  |  |
| Comment | parameters corresponding to NR ASN.1 PDCP-Config: the following parameter are not included and may be added if needed: - integer TReorderingTimer (timer value in milliseconds) |  |  |
| Rb | NR_PDCP_RB_Config_Parameters_Type |  |  |
| DuplicateDiscarding | boolean | opt | if omitted: initial configuration: duplicate discarding function operates as normal re-configuration: "keep as it is" if set: false - SS shall disable the duplicate discarding function of the receiving PDCP entity: a PDCP PDU, using the same SN as a previously received PDCP PDU, shall not be discarded and the resulting PDCP SDU shall be delivered to the upper layer. true - (re)configuration of duplicate discarding function back to normal operation. |

## D.6.2	NR_PDCP_Configuration

NR_PDCP_TransparentMode

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_TransparentMode |  |  |
| Comment |  |  |  |
| SN_Size | NR_PDCP_SN_Size_Type |  |  |

NR_PDCP_RbConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_RbConfig_Type |  |
| Comment |  |  |
| Params | NR_PDCP_Config_Parameters_Type | PDCP configuration parameters corresponding to UE configuration |
| TransparentMode | NR_PDCP_TransparentMode | PDCP configuration for transparent (test) mode: used for PDCP tests (TS 38.523-3, cl. 5.1.2.1): the SS does not apply ciphering, not apply integrity protection and does not maintain PDCP sequence numbers and state variables; ROHC is not not applied by the SS. Note: a reconfiguration of a RB from transparent mode to 'normal' mode is not foreseen (i.e. there is no mechanism to restore Ciphering, PDCP sequence numbers and state variables at the SS) (in UL PDCP PDUs are decoded depending on SN_Size) |

NR_PDCP_RBTerminating_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_RBTerminating_Type |  |  |
| Comment | RB terminating PDCP configuration: the PDCP may be linked - to a local RLC bearer: RLC bearer is configured for the same cell - to the RLC bearer of some other cell group: LinkToOtherCellGroup is not "None" - the both (in case of split bearer): RLC bearer is configured for the same cell and LinkToOtherCellGroup is not "None" |  |  |
| RbConfig | NR_PDCP_RbConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| LinkToOtherCellGroup | RlcBearerRouting_Type | opt | mandatory for initial configuration; omit means "keep as it is" None: no link to other cell group (normal case, non-split bearer) RAT/cellId: PDCP is linked to RLC bearer of another cell group (same or other RAT): split bearer or PDCP and RLC bearer being configured at different cells NOTE: applicable also for PDCP split bearer test cases when PDCP is in transparent mode => test case body may be implemented at one PTC |

NR_PDCP_Proxy_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_Proxy_Type |  |  |
| Comment | proxy to route PDCP data between terminating PDCP entity and RLC bearer of another cell (group) |  |  |
| LinkToOtherNode | RlcBearerRouting_Type |  | RAT/cellId to address the radio bearer terminating node (PDCP) (None is not applicable) |

NR_PDCP_Configuration_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_Configuration_Type |  |
| Comment |  |  |
| None | Null_Type | for SRB0 no PDCP is configured; furthermore the PDCP may not be configured e.g. for DRBs tested in RLC or MAC test cases |
| RBTerminating | NR_PDCP_RBTerminating_Type | PDCP entity at the terminating node: handling of PDCP protocol for the given bearer (normal or split beaerer) |
| Proxy | NR_PDCP_Proxy_Type | proxy to be configured above RLC instead of a normal PDCP entity when the RLC bearer is not in the same cell (group) as the terminating PDCP entity |

## D.6.3	NR_PDCP_DrbDefs

NR_PDCP_DrbDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_PDCP_SDU_Type | octetstring |  |
| NR_PDCP_CtrlPduType_Type | B3_Type | PDU type according to TS 38.323 clause 6.3.8: 000 PDCP status report 001 Interspersed ROHC feedback 010 EHC feedback 011 UDC feedback 100-111 Reserved |

NR_PDCP_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_PDCP_SDUList_Type |
| Comment |  |
| record of NR_PDCP_SDU_Type |  |

NR_PDCP_DataPduSN12Bits_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_DataPduSN12Bits_Type |  |  |
| Comment | Data PDU for DRBs with 12 bits PDCP SN (TS 38.323, clause 6.2.2.2) |  |  |
| D_C | B1_Type |  | 1 bit, '1'B for Data PDU |
| Reserved | B3_Type |  | 3 bits reserved |
| SequenceNumber | B12_Type |  | 12 bits sequence number |
| SDU | NR_PDCP_SDU_Type |  | content (octetstring) |
| MAC_I | B32_Type | opt | message authentication code according to TS 38.323, clause 6.3.4; MAC-I field is present only when the DRB is configured with integrity protection; in this case it is up to TTCN to provide the valid MAC_I in DL and to check it in UL |

NR_PDCP_DataPduSN18Bits_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_DataPduSN18Bits_Type |  |  |
| Comment | Data PDU for DRBs with 18 bits PDCP SN (TS 38.323, clause 6.2.2.3) |  |  |
| D_C | B1_Type |  | 1 bit, '1'B for Data PDU |
| Reserved | B5_Type |  | 5 bits reserved |
| SequenceNumber | B18_Type |  | 18 bits sequence number |
| SDU | NR_PDCP_SDU_Type |  | content (octetstring) |
| MAC_I | B32_Type | opt | message authentication code according to TS 38.323, clause 6.3.4; MAC-I field is present only when the DRB is configured with integrity protection; in this case it is up to TTCN to provide the valid MAC_I in DL and to check it in UL |

NR_PDCP_CtrlPduStatus_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_CtrlPduStatus_Type |  |  |
| Comment | Control PDU for PDCP status report (TS 38.323, clause 6.2.3.1) |  |  |
| D_C | B1_Type |  | 1 bit, '0'B for Ctrl PDU |
| PDU_Type | NR_PDCP_CtrlPduType_Type |  | 3 bits, '000'B for PDCP status report |
| Reserved | B4_Type |  | 4 bits reserved |
| FirstMissingCount | B32_Type |  | 32 bits, TS 38.323, clause 6.3.9 FMC |
| Bitmap | octetstring | opt | Bitmap according to TS 38.323, clause 6.3.10 |

NR_PDCP_CtrlPduRohcFeedback_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_CtrlPduRohcFeedback_Type |  |  |
| Comment | Control PDU for Interspersed ROHC feedback (TS 38.323, clause 6.2.3.2) |  |  |
| D_C | B1_Type |  | 1 bit, '0'B for Ctrl PDU |
| PDU_Type | NR_PDCP_CtrlPduType_Type |  | 3 bits, '001'B for Interspersed ROHC feedback |
| Reserved | B4_Type |  | 4 bits reserved |
| RohcFeedback | octetstring |  | ROHC packet that is not associated with a PDCP SDU |

NR_PDCP_CtrlPduEhcFeedbackCID7Bits_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_CtrlPduEhcFeedbackCID7Bits_Type |  |  |
| Comment |  |  |  |
| D_C | B1_Type |  | 1 bit, '0'B for Ctrl PDU |
| PDU_Type | NR_PDCP_CtrlPduType_Type |  | 3 bits, '101'B for EHC feedback |
| Reserved | B4_Type |  | 4 bits reserved |
| Reserved2 | B1_Type |  | 1 bit reserved |
| CID | B7_Type |  | 7 bits CID |

NR_PDCP_CtrlPduEhcFeedbackCID15Bits_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_CtrlPduEhcFeedbackCID15Bits_Type |  |  |
| Comment |  |  |  |
| D_C | B1_Type |  | 1 bit, '0'B for Ctrl PDU |
| PDU_Type | NR_PDCP_CtrlPduType_Type |  | 3 bits, '101'B for EHC feedback |
| Reserved | B4_Type |  | 4 bits reserved |
| Reserved2 | B1_Type |  | 1 bit reserved |
| CID | B15_Type |  | 15 bits CID |

NR_PDCP_CtrlPduUdcFeedback_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_CtrlPduUdcFeedback_Type |  |  |
| Comment |  |  |  |
| D_C | B1_Type |  | 1 bit, '0'B for Ctrl PDU |
| PDU_Type | NR_PDCP_CtrlPduType_Type |  | 3 bits, '011'B for UDC feedback |
| FE | B1_Type |  | 1 bit, '0' for NoError, '1' for Checksum Error Notification |
| Reserved | B3_Type |  | 3 bit reserved |

NR_PDCP_DataPduSN12Bits_UDC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_DataPduSN12Bits_UDC_Type |  |  |
| Comment | TS 38.323 clause 5.14 |  |  |
| D_C | B1_Type |  | 1 bit, '1'B for Data PDU |
| Reserved | B3_Type |  | 3 bits reserved |
| SequenceNumber | B12_Type |  |  |
| FU | B1_Type |  | 1 Bit: Indication of whether this packet is compressed by UDC protocol or not. Value '1' means the packet is compressed by UDC protocol. |
| FR | B1_Type |  | 1 Bit: Indication of whether UDC compression buffer is reset or not. Value '1' means this is the first compressed packet after UDC buffer reset. |
| Reserved2 | B2_Type |  | 2 bits: Reserved |
| Checksum | B4_Type |  | 4 bits |
| SDU | NR_PDCP_SDU_Type |  | content (octetstring) |
| MAC_I | B32_Type | opt | message authentication code according to TS 38.323, clause 6.3.4; MAC_I field only when the DRB is configured with integrity protection; in this case it is up to TTCN to provide the valid MAC_I in DL and to check it in UL |

NR_PDCP_DataPduSN18Bits_UDC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PDCP_DataPduSN18Bits_UDC_Type |  |  |
| Comment | TS 38.323 clause 5.14 |  |  |
| D_C | B1_Type |  | 1 bit, '1'B for Data PDU |
| Reserved | B5_Type |  | 3 bits reserved |
| SequenceNumber | B18_Type |  |  |
| FU | B1_Type |  | 1 Bit: Indication of whether this packet is compressed by UDC protocol or not. Value '1' means the packet is compressed by UDC protocol. |
| FR | B1_Type |  | 1 Bit: Indication of whether UDC compression buffer is reset or not. Value '1' means this is the first compressed packet after UDC buffer reset. |
| Reserved2 | B2_Type |  | 2 bits: Reserved |
| Checksum | B4_Type |  | 4 bits |
| SDU | NR_PDCP_SDU_Type |  | content (octetstring) |
| MAC_I | B32_Type | opt | message authentication code according to TS 38.323, clause 6.3.4; MAC_I field only when the DRB is configured with integrity protection; in this case it is up to TTCN to provide the valid MAC_I in DL and to check it in UL |

NR_PDCP_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PDCP_PDU_Type |  |
| Comment |  |  |
| DataPduSN12Bits | NR_PDCP_DataPduSN12Bits_Type |  |
| DataPduSN18Bits | NR_PDCP_DataPduSN18Bits_Type |  |
| CtrlPduStatus | NR_PDCP_CtrlPduStatus_Type |  |
| CtrlPduRohcFeedback | NR_PDCP_CtrlPduRohcFeedback_Type |  |
| CtrlPduEhcFeedbackCID7Bits | NR_PDCP_CtrlPduEhcFeedbackCID7Bits_Type |  |
| CtrlPduEhcFeedbackCID15Bits | NR_PDCP_CtrlPduEhcFeedbackCID15Bits_Type |  |
| CtrlPduUdcFeedback | NR_PDCP_CtrlPduUdcFeedback_Type |  |
| DataPduSN12Bits_UDC | NR_PDCP_DataPduSN12Bits_UDC_Type |  |
| DataPduSN18Bits_UDC | NR_PDCP_DataPduSN18Bits_UDC_Type |  |

NR_PDCP_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_PDCP_PDUList_Type |
| Comment |  |
| record of NR_PDCP_PDU_Type |  |

# D.7	SDAP_TypeDefs

## D.7.1	SDAP_Configuration

SDAP_Header_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | SDAP_Header_Type |
| Comment |  |
| Present |  |
| Absent |  |

QFI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | QFI_List_Type |
| Comment |  |
| record of integer |  |

SdapConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SdapConfig_Type |  |  |
| Comment |  |  |  |
| Pdu_SessionId | integer |  |  |
| Sdap_HeaderDL | SDAP_Header_Type | opt | mandatory for initial configuration; omit means "keep as it is" when set to 'Present', unless specifically triggered otherwise, the SS shall set the RDI and RQI fields to 0 |
| Sdap_HeaderUL | SDAP_Header_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| MappedQoS_Flows | QFI_List_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

SdapTransparentMode_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SdapTransparentMode_Type |  |  |
| Comment |  |  |  |
| Sdap_HeaderUL | SDAP_Header_Type |  | Indicates to the SS if the UE has been configured to include SDAP UL header |

SdapConfigInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SdapConfigInfo_Type |  |
| Comment |  |  |
| SdapConfig | SdapConfig_Type | SDAP configuration parameters for the DRB |
| TransparentMode | SdapTransparentMode_Type | SDAP configuration for transparent (test) mode, used for SDAP tests: SS does not add any SDAP headers in DL and does not remove any SDAP headers in UL |

SDAP_Configuration_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SDAP_Configuration_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Config | SdapConfigInfo_Type |  |

## D.7.2	SDAP_DrbDefs

SDAP_DrbDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SDAP_SDU_Type | octetstring |  |

SDAP_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SDAP_SDUList_Type |
| Comment |  |
| record of SDAP_SDU_Type |  |

SDAP_DL_PduHeader_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SDAP_DL_PduHeader_Type |  |  |
| Comment | TS 37.324 Figure 6.2.2.2-1 |  |  |
| RDI | B1_Type |  | TS 37.324 Figure 6.3.7: The RDI bit indicates whether QoS flow to DRB mapping rule should be updated 1 bit: '0'B No action '1'B To store QoS flow to DRB mapping rule |
| RQI | B1_Type |  | TS 37.324 clause 6.3.6: The RQI bit indicates whether NAS should be informed of the updated of SDF to QoS flow mapping rules 1 bit: '0'B No action '1'B To inform NAS that RQI bit is set to 1 |
| QFI | B6_Type |  | TS 37.324 clause 6.3.4: The QFI field indicates the ID of the QoS flow to which the SDAP SDU belongs |

SDAP_UL_PduHeader_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SDAP_UL_PduHeader_Type |  |  |
| Comment | TS 37.324 Figure 6.2.2.3-1 or 6.2.3-1 |  |  |
| DC | B1_Type |  | TS 37.324 clause 6.3.3: The D/C bit indicates whether the SDAP PDU is an SDAP Data PDU or an SDAP Control PDU 1 bit: '0'B Control PDU '1'B Data PDU |
| R | B1_Type |  | TS 37.324 clause 6.3.5: Reserved. In this version of the specification reserved bits shall be set to 0. Reserved bits shall be ignored by the receiver |
| QFI | B6_Type |  | QFI (UL) or PQFI (SL) TS 37.324 clause 6.3.4: The QFI field indicates the ID of the QoS flow to which the SDAP SDU belongs, or TS 37.324 clause 6.3.8: The PQFI field indicates the ID of the PC5 QoS flow to which the SDAP PDU belongs |

SDAP_PDU_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SDAP_PDU_DL_Type |  |  |
| Comment | TS 37.324 clause 6.2.2 Data PDU |  |  |
| Header | SDAP_DL_PduHeader_Type | opt | present for DL Data PDU with SDAP header (clause 6.2.2.2), omitted for Data PDU without SDAP header (clause 6.2.2.1) |
| Data | SDAP_SDU_Type |  |  |

SDAP_PDU_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SDAP_PDU_UL_Type |  |  |
| Comment | TS 37.324 clause 6.2.2 Data PDU or clause 6.2.3 End-Marker Control PDU |  |  |
| Header | SDAP_UL_PduHeader_Type | opt | present for: UL Data PDU in default DRB or UL Data PDU with SDAP header in a non-default DRB (clause 6.2.2.3) or End-Marker Control PDU (clause 6.2.3); omitted for UL Data PDU without SDAP header in a non-default DRB (clause 6.2.2.1) |
| Data | SDAP_SDU_Type | opt | present for UL Data PDU (clause 6.2.2.1 or 6.2.2.3), omitted for End-Marker Control PDU (clause 6.2.3) |

SDAP_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SDAP_PDU_Type |  |
| Comment | TS 37.324 clause 6.2.2 Data PDU or 6.2.3 End-Marker Control PDU |  |
| DL | SDAP_PDU_DL_Type | Data PDU in DL |
| UL | SDAP_PDU_UL_Type | Data PDU in UL or End-Marker Control PDU |

SDAP_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | SDAP_PDUList_Type |
| Comment |  |
| record of SDAP_PDU_Type |  |

# D.8	NR_ASP_VirtualNoiseDefs

ASP definitions for virtual noise generation in NR cells:
- A VNG instance, upon creation, is always associated with an already configured NR cell. This VNG instance therefore operates on the same frequency as the associated NR cell's frequency.
- A VNG instance generates AWGN (Additive white Gaussian noise):
  - Frequency domain: on the whole carrier bandwidth defined for the frequency of the associated NR cell.
  - Time domain: in every slot.
- The AWGN power level is defined as an EPRE in dBm/SCS, corresponding to the 'Noc' value provided in the test case prose.

NR_ASP_VirtualNoiseDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_VngConfigConfirm_Type | Null_Type |  |

NR_VngConfigInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_VngConfigInfo_Type |  |
| Comment |  |  |
| CellNocLevel | integer | CellNocLevel - EPRE level in dBm/SCS for AWGN transmission to model a noise source. The SS shall apply AWGN to the whole carrier bandwidth of the associated NR cell and in every slot |

NR_VngConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_VngConfigRequest_Type |  |
| Comment | configure/activate noise for a given cell; NOTE: it is assumed the associated NR cell has been created beforehand |  |
| Configure | NR_VngConfigInfo_Type | configuration of the virtual noise generator; regardless of the power level the noise generator is off before it gets activated for this cell; in case the configuration needs to be changed during a test, the noise generator shall be deactivated for this cell |
| Activate | Null_Type | noise is activated (switched on) for the given cell acc. to the previous configuration; while being active the configuration shall not be modified |
| Deactivate | Null_Type | deactivate noise for given cell |

NR_VNG_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_VNG_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | NR_ReqAspCommonPart_Type |  | CellId : as for the associated NR cell RoutingInfo : None RlcBearerRouting : None MacBearerRouting : 'omit' TimingInfo : Now ControlInfo : CnfFlag:=true; FollowOnFlag:=false |
| Request | NR_VngConfigRequest_Type |  |  |

NR_VNG_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_VNG_CTRL_CNF |  |  |
| Comment |  |  |  |
| Common | NR_CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN => SS may set TimingInfo to "None" |
| Confirm | NR_VngConfigConfirm_Type |  |  |

NR_VNG_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NR_VNG_PORT |  |
| Comment | NR BASE PTC: Port for virtual noise generator |  |
| out | NR_VNG_CTRL_REQ |  |
| in | NR_VNG_CTRL_CNF |  |

# D.9	CommonDefs

CommonDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_UInt16Max | integer | 65535 |  |
| tsc_UInt32Max | integer | 4294967295 |  |

CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| B1_Type | bitstring length(1) |  |
| B2_Type | bitstring length(2) |  |
| B3_Type | bitstring length(3) |  |
| B4_Type | bitstring length(4) |  |
| B5_Type | bitstring length(5) |  |
| B6_Type | bitstring length(6) |  |
| B7_Type | bitstring length(7) |  |
| B8_Type | bitstring length(8) |  |
| B12_Type | bitstring length(12) |  |
| B14_Type | bitstring length(14) |  |
| B15_Type | bitstring length(15) |  |
| B16_Type | bitstring length(16) |  |
| B18_Type | bitstring length(18) |  |
| B24_Type | bitstring length(24) |  |
| B32_Type | bitstring length(32) |  |
| B48_Type | bitstring length(48) |  |
| B128_Type | bitstring length(128) |  |
| B128_Key_Type | B128_Type | 128 bit security key |
| O1_Type | octetstring length(1) |  |
| Null_Type | boolean (true) | dummy type for 'typeless' fields in unions |
| UInt_Type | integer (0 .. infinity) |  |
| UInt16_Type | integer (0 .. tsc_UInt16Max) |  |
| UInt32_Type | integer (0 .. tsc_UInt32Max) |  |
| IP_DrbId_Type | integer | DRB identity type common for all RATs: - for EUTRA it corresponds to the ASN.1 type DRB-Identity - for UTRAN it corresponds to the ASN.1 type RB-Identity and values are as defined in TS 34.123-3 Table 8.2.4.1 - for GERAN the NSAPI value (type record NSAPI) may be used NOTE: this is introduced to simplify the dependencies (i.e. to keep IP_ASP_TypeDefs independent from any RAT specific type definitions) |
| PdcpCountValue_Type | B32_Type |  |
| RNTI_Value_Type | UInt16_Type | corresponds to NR ASN.1: RNTI-Value ::= INTEGER (0..65535) |
| RNTI_B16_Type | B16_Type |  |

B8_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | B8_List_Type |
| Comment |  |
| record of B8_Type |  |

IntegerList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | IntegerList_Type |
| Comment |  |
| record of integer |  |

IndicationAndControlMode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IndicationAndControlMode_Type |
| Comment |  |
| enable |  |
| disable |  |

NR_CellId_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_CellId_Type |
| Comment |  |
| nr_Cell_NonSpecific |  |
| nr_Cell1 |  |
| nr_Cell2 |  |
| nr_Cell3 |  |
| nr_Cell4 |  |
| nr_Cell6 |  |
| nr_Cell10 |  |
| nr_Cell11 |  |
| nr_Cell12 |  |
| nr_Cell13 |  |
| nr_Cell14 |  |
| nr_Cell23 |  |
| nr_Cell28 |  |
| nr_Cell29 |  |
| nr_Cell30 |  |
| nr_Cell31 |  |

NR_CellIdList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_CellIdList_Type |
| Comment | NOTE: there seems to be no need for any length restriction |
| record of NR_CellId_Type |  |

EUTRA_CellId_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | EUTRA_CellId_Type |
| Comment |  |
| eutra_Cell_NonSpecific |  |
| eutra_Cell1 |  |
| eutra_Cell2 |  |
| eutra_Cell3 |  |
| eutra_Cell4 |  |
| eutra_Cell6 |  |
| eutra_Cell10 |  |
| eutra_Cell11 |  |
| eutra_Cell12 |  |
| eutra_Cell13 |  |
| eutra_Cell14 |  |
| eutra_Cell23 |  |
| eutra_Cell28 |  |
| eutra_Cell29 |  |
| eutra_Cell30 |  |
| eutra_Cell31 |  |
| eutra_CellA |  |
| eutra_CellB |  |
| eutra_CellC |  |
| eutra_CellD |  |
| eutra_CellE |  |
| eutra_CellG |  |
| eutra_CellH |  |
| eutra_CellI |  |
| eutra_CellJ |  |
| eutra_CellK |  |
| eutra_CellL |  |
| eutra_CellM |  |

UE_NR_DeltaValues_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UE_NR_DeltaValues_Type |  |  |
| Comment |  |  |  |
| DeltaPrimaryBand | DeltaValues_Type |  |  |
| DeltaSecondaryBand | DeltaValues_Type |  |  |

DeltaValues_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DeltaValues_Type |  |  |
| Comment | Delta value for each frequency, by default for FR1 or when the frequency is not available the Delta value is set to 0. |  |  |
| DeltaNRf1 | integer |  |  |
| DeltaNRf2 | integer |  |  |
| DeltaNRf3 | integer |  |  |
| DeltaNRf4 | integer |  |  |

IP_EUTRA_DrbInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_EUTRA_DrbInfo_Type |  |  |
| Comment |  |  |  |
| CellId | EUTRA_CellId_Type |  | data is routed to a specific cell regardless of whether the same DRB is configured in any other cell |
| DrbId | IP_DrbId_Type | opt | mandatory at the system interface |

IP_UTRAN_GERAN_DrbInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_UTRAN_GERAN_DrbInfo_Type |  |  |
| Comment |  |  |  |
| CellId | integer |  |  |
| DrbId | IP_DrbId_Type | opt | mandatory at the system interface |

IP_WLAN_DrbInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_WLAN_DrbInfo_Type |  |  |
| Comment |  |  |  |
| CellId | integer |  |  |

IP_ePDG_IPsecTunnelInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_ePDG_IPsecTunnelInfo_Type |  |  |
| Comment |  |  |  |
| PdnId | PDN_Index_Type |  | 'index name' of PDN associated to the IPsec tunnel, e.g. for SS to distinguish routing of IP packets in case of more than one IPsec tunnel NOTE: In general only 'ePDG_XXX' values shall be used |

QosFlow_Identification_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | QosFlow_Identification_Type |  |  |
| Comment |  |  |  |
| PDU_SessionId | integer |  | TS 24.007 clause 11.2.3.1b |
| QFI | integer |  | TS 24.501 Table 11.2.3.1c.1 |

IP_NR_QosFlowInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_NR_QosFlowInfo_Type |  |  |
| Comment |  |  |  |
| CellId | NR_CellId_Type |  | data is routed to a specific cell regardless of whether the same DRB is configured in any other cell |
| QosFlow | QosFlow_Identification_Type | opt | mandatory at the system interface |

IP_EUTRA_QosFlowInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_EUTRA_QosFlowInfo_Type |  |  |
| Comment |  |  |  |
| CellId | EUTRA_CellId_Type |  | data is routed to a specific cell regardless of whether the same DRB is configured in any other cell |
| QosFlow | QosFlow_Identification_Type | opt | mandatory at the system interface |

IP_NR_DrbInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IP_NR_DrbInfo_Type |  |  |
| Comment |  |  |  |
| CellId | NR_CellId_Type |  |  |
| DrbId | IP_DrbId_Type | opt | mandatory at the system interface |

IP_DrbInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IP_DrbInfo_Type |  |
| Comment |  |  |
| Eutra | IP_EUTRA_DrbInfo_Type |  |
| Eutra5GC | IP_EUTRA_QosFlowInfo_Type | used when SDAP is configured in non-transparent mode at the SS |
| Utran | IP_UTRAN_GERAN_DrbInfo_Type |  |
| Geran | IP_UTRAN_GERAN_DrbInfo_Type |  |
| Nr5GC | IP_NR_QosFlowInfo_Type | used when SDAP is configured in non-transparent mode at the SS |
| NrEPC | IP_NR_DrbInfo_Type | used when NR is connected to EPC (S1-U interface) or when SDAP is configured in transparent mode at the SS |
| Wlan | IP_WLAN_DrbInfo_Type |  |
| IPsecTunnel | IP_ePDG_IPsecTunnelInfo_Type |  |

PDN_Index_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | PDN_Index_Type |
| Comment | 'index name' associated to a PDN: The major purpose is to associate a PDN specific set of IP addresses to the given PDN (e.g. UE addresses, P-CSCF address etc.); in general there are one or two PDNs configured at the same time and - from TTCN point of view - the IMS PDN is considered to be the first one; a second PDN may be configured in case of emergency call or e.g. for XCAP signalling; the 'internet PDN' (according to TS 36.508 clause 4.5.2) is considered as (optional) second PDN during initial registration and gets released after initial registration; in case of WLAN a separate group of index names is used to distinguish the different configuration of the emulated IP network |
| PDN_1 | "default" PDN being kept connected to during a test case (in case of LTE in general the IMS PDN) |
| PDN_2 | second PDN: during initial registration (TS 36.508 clause 4.5.2) for LTE and "multiple PDN' this is the internet PDN; after initial registration it is used if needed according to the test purpose (e.g. emergency call) |
| PDN_2a | used for the special case when the UE IP address of the second PDN changes in a test case |
| PDN_Internet | mainly used as alias for PDN2 during initial registration |
| ePDG_IMS1 | WLAN: PDN for 'normal' IMS |
| ePDG_IMS2 | WLAN: PDN for emergency IMS (in general) |
| ePDG_XCAP | WLAN: PDN for XCAP in case of XCAP server being part of 3GPP-network NOTE: In contrast to LTE for WLAN there is a different IP architecture to be consider by TTCN for XCAP and IMS emergency |
| ePDG_Internet | place-holder for WLAN-offload scenarios |
| nrPDN_Internet | mainly used as alias during initial registration in ENDC |
| PDN_3 | used in ENDC and 5GC for the third PDN |
| PDN_4 | used in 5GC for the URLLC PDN |
| PDN_5 | used in 5GC for the MIoT PDN |
| PDN_6 | used in 5GC for the V2X PDN |

# D.10	CommonAspDefs

## D.10.1	Cell_Configuration_Common

CellTimingInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CellTimingInfo_Type |  |  |
| Comment | Cell Timing |  |  |
| TcOffset | integer (0..63) | opt | For NR according to TS 38.211 clause 4.1 Ts/Tc = 64 with Tc = 1/(480000 * 4096) and Ts = 1/(15000 * 2048) as for EUTRA; => for NR to specify granularity per Tc; for EUTRA to be set to 0 (and/or to be ignored by the SS) |
| Tcell | integer (0..307199) |  | frame duration Tf = 307200 * Ts = 10ms; System Time Unit Ts = 1/(15000 * 2048) |
| SfnOffset | integer (0..1023) |  |  |
| HsfnOffset | integer (0..1023) |  |  |

## D.10.2	MAC_Layer

ULGrant_Period_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ULGrant_Period_Type |  |
| Comment |  |  |
| OnlyOnce | Null_Type | grant is sent out only once; no period |
| Duration | integer (1..infinity) | duration of the grant period in number of sub-frames (1ms) for EUTRA and number of slots for NR |

TransmissionRepetition_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TransmissionRepetition_Type |  |
| Comment |  |  |
| Continuous | Null_Type |  |
| NumOfCycles | integer (1..infinity) |  |

PeriodicGrant_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PeriodicGrant_Type |  |  |
| Comment |  |  |  |
| Period | ULGrant_Period_Type |  | time period after which UL Grant need to be automatically transmitted or 'OnlyOnce' |
| NoOfRepetitions | TransmissionRepetition_Type |  | number of UL Grant repetitions to be automatically transmitted or continuous repetition |

UL_GrantConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UL_GrantConfig_Type |  |
| Comment |  |  |
| OnSR_Reception | Null_Type | SS transmits UL Grant as configured by DciInfoUL_Type at every reception of SR; to be used in non L2 Test |
| Periodic | PeriodicGrant_Type | SS transmits UL Grant as configured by DciInfoUL_Type periodically; to be used in L2 tests; MAC tests testing Grants might set the period as infinite and num grant as 1 |
| PeriodicOnSR_Reception | PeriodicGrant_Type | SS transmits UL Grant as configured by DciInfoUL_Type periodically; the periodic grant transmission is started/restarted on reception of a SR from UE to be used in non L2 Test to enable large UL data transmission for lower category UEs (Cat<=1) |
| None | Null_Type | disable any grant transmission |

RAR_RapIdCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RAR_RapIdCtrl_Type |  |
| Comment |  |  |
| Automatic | Null_Type | SS shall automatically use same RAPID as received from the UE |
| Unmatched | Null_Type | SS shall use RAPID being different from preamble sent by the UE; SS shall calculate this RAPID acc. to RAPID := (RAPID + 3..63) mod 64 if single RAR is transmitted in a MAC PDU then only 3 is added if multiple RAR's are transmitted in MAC PDU, then for first unmatched RAR 3 is added, second unmatched 4 is added, third unmatched 5 is added and so on |

## D.10.3	System_Indications

HARQ_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | HARQ_Type |
| Comment | ack represents HARQ ACK; nack represents HARQ NACK |
| ack |  |
| nack |  |

## D.10.4	ASP_CommonPart

Definition of ASP common parts for REQ-, CNF- and IND-ASPs

### D.10.4.1	ASP_CommonPart_Definitions

#### D.10.4.1.1	Routing_Info

CommonAspDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_SRB0 | integer | 0 |  |
| tsc_SRB1 | integer | 1 |  |
| tsc_SRB2 | integer | 2 |  |
| tsc_SRB3 | integer | 3 |  |
| tsc_SRB4 | integer | 4 |  |

Routing_Info: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SRB_Identity_Type | integer (tsc_SRB0, tsc_SRB1, tsc_SRB2, tsc_SRB3, tsc_SRB4) |  |

RlcBearerRouting_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RlcBearerRouting_Type |  |
| Comment | data routing e.g. in case of split bearer (split DRB or split SRB); applicable for multi-RAT Dual Connectivity (MR-DC) as well as single-RAT Dual Connectivity |  |
| EUTRA | EUTRA_CellId_Type |  |
| NR | NR_CellId_Type |  |
| None | Null_Type | normal case: PDCP and RLC are configured at the same cell |

#### D.10.4.1.2	Timing_Info

Timing_Info: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SystemFrameNumber_Type | integer (0..1023) |  |
| SubFrameNumber_Type | integer (0..9) |  |
| SymbolNumber_Type | integer (0..13) |  |
| HyperSystemFrameNumberInfo_Type | SystemFrameNumberInfo_Type |  |

SubFrameInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SubFrameInfo_Type |  |
| Comment |  |  |
| Number | SubFrameNumber_Type |  |
| Any | Null_Type | no specific sub-frame, in which case the SS may choose the next available subframe (valid for REQ ASPs only) |

SystemFrameNumberInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SystemFrameNumberInfo_Type |  |
| Comment |  |  |
| Number | SystemFrameNumber_Type |  |
| Any | Null_Type | no specific frame number, in which case the SS may choose the next available SFN (valid for REQ ASPs only) |

SlotOffset_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SlotOffset_Type |  |
| Comment | slots per subframe according to TS 38.211 Table 4.3.2-1 |  |
| Numerology0 | Null_Type | mu=0; only one slot per subframe |
| Numerology1 | integer (0..1) | mu=1; 2 slots per subframe |
| Numerology2 | integer (0..3) | mu=2; 4 slots per subframe |
| Numerology3 | integer (0..7) | mu=3; 8 slots per subframe |
| Numerology4 | integer (0..15) | mu=4; 16 slots per subframe |

SlotTimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SlotTimingInfo_Type |  |
| Comment | EUTRA, NBIOT: REQ ASPs: TTCN shall set the SlotTimingInfo to "FirstSlot" for EUTRA, NBIOT addressing the whole subframe IND ASPs: TTCN shall ignore the SlotTimingInfo sent by the SS for EUTRA, NBIOT NR: REQ ASPs: Any:=true only if the slot number is not relevant, in which case the SS may choose the next available slot of the given subframe IND ASPs: Any:=true only if there is no slot information available for the particular kind of indication |  |
| SlotOffset | SlotOffset_Type | to address a particular slot in a subframe |
| FirstSlot | Null_Type | to address the first slot independent from the numerology (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT |
| Any | Null_Type | for IND ASPs in EUTRA and NBIOT or if slot number is not relevant or not available |

SymbolTimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SymbolTimingInfo_Type |  |
| Comment | EUTRA, NBIOT: REQ ASPs: TTCN shall set the SymbolTimingInfo to "FirstSymbol" for EUTRA, NBIOT addressing the whole subframe IND ASPs: TTCN shall ignore the SymbolTimingInfo sent by the SS for EUTRA, NBIOT NR: REQ ASPs: Any:=true only if the symbol number is not relevant, in which case the SS may choose the next available symbol of the given slot IND ASPs: Any:=true only if there is no symbol information available for the particular kind of indication |  |
| SymbolOffset | SymbolNumber_Type | to address a particular symbol in a slot |
| FirstSymbol | Null_Type | to address the first symbol (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT |
| Any | Null_Type | for IND ASPs in EUTRA and NBIOT or if symbol number is not relevant or not available |

SubFrameTiming_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SubFrameTiming_Type |  |  |
| Comment |  |  |  |
| SFN | SystemFrameNumberInfo_Type |  |  |
| Subframe | SubFrameInfo_Type |  |  |
| HSFN | HyperSystemFrameNumberInfo_Type |  |  |
| Slot | SlotTimingInfo_Type |  |  |
| Symbol | SymbolTimingInfo_Type |  |  |

TimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TimingInfo_Type |  |
| Comment |  |  |
| SubFrame | SubFrameTiming_Type |  |
| Now | Null_Type | to be used in REQ ASPs when there is no 'activation time' |
| None | Null_Type | only to be used in SYSTEM_CTRL_CNF or NR_SYSTEM_CTRL_CNF but not for EnquireTiming |

### D.10.4.2	REQ_ASP_CommonPart

ReqAspControlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ReqAspControlInfo_Type |  |  |
| Comment |  |  |  |
| CnfFlag | boolean |  | true => SS shall send CNF: when the REQ is with no timing information (no activation time), SS shall send the confirmation when the configuration is done, i.e. when the test case may continue. Example: when there is a configuration followed by a send event it shall not be necessary to have a wait timer in between but the CNF triggers the send event. If there are other triggers e.g. like the UE sending a message, CnfFlag shall be set to false by the test case to avoid racing conditions with the CNF and the signalling message. When there is an activation time SS shall send the CNF after the configuration has been scheduled; that means SS shall not wait until the activation time has been expired. When FollowOnFlag is true, CnfFlag shall be set to false. |
| FollowOnFlag | boolean |  | false => no further (related) information true: further related information will be sent to SS (semantics depending on respective ASP) |

### D.10.4.3	CNF_ASP_CommonPart

ConfirmationResult_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ConfirmationResult_Type |  |
| Comment |  |  |
| Success | Null_Type |  |
| Error | integer | may contain SS specific error code; this will not be evaluated by TTCN |

### D.10.4.4	IND_ASP_CommonPart

IntegrityErrorIndication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IntegrityErrorIndication_Type |  |  |
| Comment |  |  |  |
| Nas | boolean |  | NAS Integrity: set to true when received MAC does not match calculated MAC |
| Pdcp | boolean |  | PDCP Integrity: set to true when received MAC does not match calculated MAC |

ErrorIndication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ErrorIndication_Type |  |  |
| Comment |  |  |  |
| Integrity | IntegrityErrorIndication_Type |  | Integrity error: received MAC does not match calculated MAC |
| System | integer |  | any other error: may be SS specific error code; this will not be evaluated by TTCN; e.g. an error shall be raised when the UE requests retransmission of an RLC PDU |

IndicationStatus_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IndicationStatus_Type |  |
| Comment |  |  |
| Ok | Null_Type |  |
| Error | ErrorIndication_Type |  |

# D.11	NR_SideLink_ASP_TypeDefs

## D.11.1	NR_SL_CommonPart

NR_SideLink_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_SL_SRB0 | integer | 0 |  |
| tsc_SL_SRB1 | integer | 1 |  |
| tsc_SL_SRB2 | integer | 2 |  |
| tsc_SL_SRB3 | integer | 3 |  |

NR_SL_CommonPart: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_SL_SRB_Identity_Type | integer (tsc_SL_SRB0, tsc_SL_SRB1, tsc_SL_SRB2, tsc_SL_SRB3) |  |

NR_SL_ReqAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_ReqAspCommonPart_Type |  |  |
| Comment |  |  |  |
| SS_UE_Id | SS_UE_Id_Type |  |  |
| RoutingInfo | NR_SL_RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| ControlInfo | ReqAspControlInfo_Type |  |  |

NR_SL_CnfAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_CnfAspCommonPart_Type |  |  |
| Comment |  |  |  |
| SS_UE_Id | SS_UE_Id_Type |  |  |
| RoutingInfo | NR_SL_RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| Result | NR_SL_SystemConfirm_Type |  |  |

NR_SL_IndAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_IndAspCommonPart_Type |  |  |
| Comment |  |  |  |
| SS_UE_Id | SS_UE_Id_Type |  |  |
| RoutingInfo | NR_SL_RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| Layer2Id | NR_SL_Layer2Ids_Type |  | Source and Destination Layer-2 ids received together with the data |
| Status | IndicationStatus_Type |  |  |

NR_SL_Layer2Ids_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_Layer2Ids_Type |  |  |
| Comment |  |  |  |
| SourceLaye2Id | B24_Type |  |  |
| DestinationLaye2Id | B24_Type |  |  |

NR_SL_RoutingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_RoutingInfo_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| RadioBearerId | NR_SL_RadioBearerId_Type |  |
| SL_QosFlow | NR_SL_QosFlow_Identification_Type | routing of data with SDAP being configured in non-transparent mode at the NR-SS-UE |

NR_SL_RadioBearerId_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_RadioBearerId_Type |  |
| Comment | Transparent mode FFS |  |
| SL_Srb | NR_SL_SRB_Identity_Type | routing for SL-SRB 0-3 for unicast |
| SL_Drb | integer | routing for SL-DRBs |

NR_SL_QosFlow_Identification_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_QosFlow_Identification_Type |  |  |
| Comment |  |  |  |
| SdapId | integer |  |  |
| PQFI | integer | opt | Optional for broadcast and groupcast - Mandatory for unicast |

## D.11.2	NR_SidelinkUE_Configuration

### D.11.2.1	NR_SL_System_Configuration

NR_SL_SystemRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SystemRequest_Type |  |
| Comment |  |  |
| SS_UE_Config | NR_SL_ConfigRequest_Type | Configure/release a NR-SS-UE |
| EnquireTiming | Null_Type | Get DFN/subframe/slot number for this NR-SS-UE |
| SystemIndCtrl | NR_SL_System_IndicationControl_Type | Configure NR-SS-UE to generate system indications |
| RadioBearerList | NR_SL_RadioBearerList_Type | Configure/release one or several SL-SRBs and/or SL-DRB |
| SL_Security | NR_SL_Security_Type | Start/Restart/Release of SLRB security - Applicable for unicast only |
| PdcpCount | NR_SL_PDCP_CountReq_Type | Set or enquire PDCP COUNT for one or more SL-RBs |
| SciTrigger | NR_SL_SCI_Trigger_Type | To trigger a specific SCI format 1-A and SCI format 2-A or 2-B to be transmitted respectively on PSCCH and PSSCH |
| CongestionConfig | NR_SL_CongestionConfig_Type | Configure NR Sidelink congestion |

NR_SL_SystemConfirm_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SystemConfirm_Type |  |
| Comment | Confirmations for NR_SL_SYSTEM_CTRL_REQ system configuration; in general to be sent after the configuration has been done |  |
| SS_UE_Config | Null_Type | Confirmation for NR-SS-UE configuration |
| EnquireTiming | Null_Type | Confirmation for EnquireTiming |
| SystemIndCtrl | Null_Type | Confirmation for SystemIndCtrl |
| RadioBearerList | Null_Type | Confirmation for SystemIndCtrl |
| SL_Security | Null_Type | (no further parameters from NR-SS-UE) |
| PdcpCount | NR_SL_PDCP_CountCnf_Type | as response to 'Get' a list is returned containing COUNT information for the requested SL-RBs |
| SciTrigger | Null_Type | Confirmation for SystemIndCtrl |

NR_SL_SystemIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SystemIndication_Type |  |
| Comment |  |  |
| Error | charstring | indicates an error situation in NR-SS-UE; is not explicitly handled in TTCN but causes an INCONC due to default behaviour; an additional error code can be signalled in the common part of the ASP; NR-SS-UE shall raise an error in case of - Invalid TimingInfo - Invalid data scheduling (NOTE: additional cases may occur) |
| SyncRefReporting | NR_SL_SyncRefReporting_Type | Indication for S-SS/MasterInformationBlockSidelink sent by UE |
| PSSCH_SciReporting | NR_SL_PSSCH_SciReporting_Type | Indication for reception of SCI format 2-A or SCI format 2-B sent by UE |
| MAC_CE | NR_SL_MAC_ControlElement_Type | Indication for reception of MAC control element sent by UE |

### D.11.2.2	NR_SL_SS_UE_Config

NR_SL_SS_UE_Config: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_SL_SCS_SpecificCarrierList_Type | SL_FreqConfigCommon_r16.sl_SCS_SpecificCarrierList_r16 |  |
| NR_FrequencyShift7p5khzSL_Type | SL_FreqConfigCommon_r16.frequencyShift7p5khzSL_r16 |  |
| NR_ValueN_Type | SL_FreqConfigCommon_r16.valueN_r16 |  |
| NR_SL_PSCCH_Config_Type | SL_ResourcePool_r16.sl_PSCCH_Config_r16 |  |
| NR_SL_PSSCH_Config_Type | SL_ResourcePool_r16.sl_PSSCH_Config_r16 |  |
| NR_SL_PSFCH_Config_Type | SL_ResourcePool_r16.sl_PSFCH_Config_r16 |  |
| NR_SL_SubchannelSize_Type | SL_ResourcePool_r16.sl_SubchannelSize_r16 |  |
| NR_SL_StartRB_Subchannel_Type | SL_ResourcePool_r16.sl_StartRB_Subchannel_r16 |  |
| NR_SL_NumSubchannel_Type | SL_ResourcePool_r16.sl_NumSubchannel_r16 |  |
| NR_SL_Additional_MCS_Table_Type | SL_ResourcePool_r16.sl_Additional_MCS_Table_r16 |  |
| NR_SL_PTRS_Config_Type | SL_ResourcePool_r16.sl_PTRS_Config_r16 |  |
| NR_SL_RB_Number_Type | SL_ResourcePool_r16.sl_RB_Number_r16 |  |
| NR_SL_TimeResource_Type | SL_ResourcePool_r16.sl_TimeResource_r16 |  |
| NR_SL_X_Overhead_Type | SL_ResourcePool_r16.sl_X_Overhead_r16 |  |
| NR_SL_MaxNumPerReserve_Type | SL_UE_SelectedConfigRP_r16.sl_MaxNumPerReserve_r16 |  |
| NR_SL_MultiReserveResource_Type | SL_UE_SelectedConfigRP_r16.sl_MultiReserveResource_r16 |  |
| NR_SL_ResourceReservePeriodList_Type | SL_UE_SelectedConfigRP_r16.sl_ResourceReservePeriodList_r16 |  |
| NR_ASN1_SL_LatencyBoundCSI_Report_Type | RRCReconfigurationSidelink_r16_IEs.sl_LatencyBoundCSI_Report_r16 |  |
| NR_SidelinkChannelPower_Type | integer | see TS 38.213, clause 16.2 |
| SL_SSId_Type | SL_SyncConfig_r16.sl_SSID_r16 |  |

NR_SL_ConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_ConfigRequest_Type |  |
| Comment |  |  |
| AddOrReconfigure | NR_SL_ConfigInfo_Type | For one NR-SS-UE configuration: SS_UE_Id : identifier of the NR-SS-UE to be configured RoutingInfo : None TimingInfo : 'Now' for initial configuration; specific TimingInfo may be used for reconfiguration ControlInfo : FollowOnFlag:=false |
| Release | Null_Type | To remove all NR-SS-UEs completely SS_UE_Id : ss_UE_NonSpecific RoutingInfo : None TimingInfo : Now ControlInfo : FollowOnFlag:=false |

NR_SL_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_ConfigInfo_Type |  |  |
| Comment |  |  |  |
| TimingSyncSource | NR_SynchronisationTiming_Type | opt | Mandatory at initial NR-SS-UE configuration |
| Power | NR_SS_UE_Power_Type | opt | Reference power for the RS of each antenna in Tx Initially configured as switched OFF |
| SL_BWP_PoolConfigList | NR_SL_BWP_PoolConfigList_Type | opt | Mandatory at initial NR-SS-UE configuration. In release 16 only one entry can be configured |

NR_SS_UE_Power_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SS_UE_Power_Type |  |  |
| Comment |  |  |  |
| MaxReferencePower | NR_AbsoluteCellPower_Type | opt | Maximum value of NR-SS-UE reference power (in dBm/15kHz as per TS 38.508-1 clause 6.2.1.3 ) NR-SS-UE is initialised with this reference power; its value can't be reconfigured during test execution, attenuation is used to change NR-SS-UE power level its value is the upper bound of the NR-SS-UE power during the test case |
| Attenuation | NR_Attenuation_Type |  | Initial attenuation: Off |

NR_SL_BWP_PoolConfigList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_BWP_PoolConfigList_Type |
| Comment |  |
| record of NR_SL_BWP_PoolConfig_Type |  |

NR_SL_BWP_PoolConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_BWP_PoolConfig_Type |  |  |
| Comment |  |  |  |
| SL_FrequencyInfo | NR_SL_Frequency_Type | opt | mandatory at first configuration |
| SL_BWP_ConfigList | NR_SL_BWP_ConfigList_Type | opt | mandatory at first configuration |
| SyncConfig | NR_SL_SyncConfig_Type | opt | mandatory at first configuration |

NR_SL_Frequency_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_Frequency_Type |  |  |
| Comment |  |  |  |
| Band | integer | opt |  |
| SL_SCS_SpecificCarrierList | NR_SL_SCS_SpecificCarrierList_Type | opt |  |
| SL_AbsoluteFrequencyPointA | NR_ASN1_ARFCN_ValueNR_Type | opt |  |
| SL_AbsoluteFrequencySSB | NR_ASN1_ARFCN_ValueNR_Type | opt |  |
| FrequencyShift7p5khzSL | NR_FrequencyShift7p5khzSL_Type | opt | As per TS 38.101-1 clause 5.4E.2.1 |
| ValueN | NR_ValueN_Type | opt |  |

NR_SL_BWP_ConfigList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_BWP_ConfigList_Type |
| Comment |  |
| record of NR_SL_BWP_Config_Type |  |

NR_SL_BWP_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_BWP_Config_Type |  |  |
| Comment |  |  |  |
| SL_BWP_Id | BWP_Id |  |  |
| ActiveBwp | boolean | opt | Indicates to NR-SS-UE which BWP is currently used - only one active at a time |
| SL_PreconfigGeneral | NR_ASN1_SL_PreconfigGeneral | opt | mandatory at first configuration |
| SL_BWP_Generic | NR_ASN1_SL_BWP_Generic_Type | opt | mandatory at first configuration |
| SL_RxPool | NR_SL_RxPoolConfig_Type | opt |  |
| SL_TxPool | NR_SL_TxPoolConfig_Type | opt |  |

NR_ASN1_SL_PreconfigGeneral

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_ASN1_SL_PreconfigGeneral |  |  |
| Comment |  |  |  |
| R16 | SL_PreconfigGeneral_r16 |  |  |

NR_SL_RxPoolConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_RxPoolConfig_Type |  |  |
| Comment |  |  |  |
| SL_ResourcePool | NR_SL_ResourcePool_Type | opt |  |
| SL_CSI_RS | NR_SL_CSI_RS_ConfigRx_Type | opt | Configuration for sidelink CSI reception |

NR_SL_TxPoolConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_TxPoolConfig_Type |  |  |
| Comment |  |  |  |
| SL_ResourcePool | NR_SL_ResourcePool_Type | opt |  |
| SL_SCI | NR_SL_SCI_Tx_Type | opt |  |
| RelativePscchPsschTxPower | NR_SL_RelativeTxPower_Type | opt |  |
| SL_CSI_RS | NR_SL_CSI_RS_ConfigTx_Type | opt | Configuration for sidelink CSI transmission |

NR_ASN1_SL_BWP_Generic_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_SL_BWP_Generic_Type |  |
| Comment |  |  |
| R16 | SL_BWP_Generic_r16 |  |

NR_ASN1_SL_PSSCH_TxParameters_r16

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_ASN1_SL_PSSCH_TxParameters_r16 |  |  |
| Comment |  |  |  |
| R16 | SL_PSSCH_TxParameters_r16 |  |  |

NR_SL_ResourcePool_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_ResourcePool_Type |  |  |
| Comment |  |  |  |
| SL_PSCCH_Config | NR_SL_PSCCH_Config_Type | opt |  |
| SL_PSSCH_Config | NR_SL_PSSCH_Config_Type | opt |  |
| SL_PSSCH_TxParameters | NR_ASN1_SL_PSSCH_TxParameters_r16 | opt | In reception: set according to UE applied transmission parameters. In transmission: set to the NR-SS-UE transmission parameters to be selected acc to TS 38.321 clause 5.22.1.1 |
| SL_PSFCH_Config | NR_SL_PSFCH_Config_Type | opt |  |
| SL_SubchannelSize | NR_SL_SubchannelSize_Type | opt |  |
| SL_StartRB_Subchannel | NR_SL_StartRB_Subchannel_Type | opt |  |
| SL_NumSubchannel | NR_SL_NumSubchannel_Type | opt |  |
| SL_Additional_MCS_Table | NR_SL_Additional_MCS_Table_Type | opt |  |
| SL_PTRS_Config | NR_SL_PTRS_Config_Type | opt |  |
| SL_MultiReserveResource | NR_SL_MultiReserveResource_Type | opt |  |
| SL_MaxNumPerReserve | NR_SL_MaxNumPerReserve_Type | opt |  |
| SL_ResourceReservePeriodList | NR_SL_ResourceReservePeriodList_Type | opt |  |
| SL_RB_Number | NR_SL_RB_Number_Type | opt |  |
| SL_TimeResource | NR_SL_TimeResource_Type | opt |  |
| SL_X_Overhead | NR_SL_X_Overhead_Type | opt |  |

NR_SL_SCI_Tx_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SCI_Tx_Type |  |  |
| Comment |  |  |  |
| PSCCH_Sci | NR_SL_PSCCH_Sci_Tx_Type | opt | 1st-stage SCI formats |
| PSSCH_Sci | NR_SL_PSSCH_Sci_Tx_Type | opt | 2nd-stage SCI formats |

NR_SL_PSCCH_Sci_Tx_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PSCCH_Sci_Tx_Type |  |
| Comment |  |  |
| SciFormat1A | NR_SL_SciFormat1A_Type | Acc to TS 38.212 cl 8.3.1.1 SCI format 1-A |

NR_SL_ResourceReservationPeriod_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_ResourceReservationPeriod_Type |  |
| Comment | TS 38.212 clause 8.3.1.1 |  |
| None | Null_Type | 0 bit no Resource reservation period, if higher layer parameter sl-MultiReserveResource is not configured |
| Value | integer | number of bits depending on the number of entries in the higher layer parameter sl-ResourceReservePeriodList |

NR_SL_AdditionalMCS_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_AdditionalMCS_Type |  |
| Comment | TS 38.212 clause 8.3.1.1 Additional MCS table indicator |  |
| None | Null_Type | 0 bit if 2 bits if two MCS tables is not configured by higher layer parameter sl- Additional-MCS-Table |
| Value | integer | as defined in clause 8.1.3.1 of TS 38.214: 1 bit if one MCS table is configured by higher layer parameter sl-Additional-MCS-Table; 2 bits if two MCS tables are configured by higher layer parameter sl- Additional-MCS-Table |

NR_SL_PSFCH_OverheadIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PSFCH_OverheadIndication_Type |  |
| Comment | TS 38.212 cl 8.3.1.1 PSFCH overhead indication |  |
| None | Null_Type | 0 bit if sl-PSFCH-Period different from 2 and 4 |
| Value | integer | 1 bit as defined clause 8.1.3.2 of TS 38.214 if higher layer parameter sl-PSFCH-Period = 2 or 4 |
| Automatic | Null_Type | 1 bit as defined in TS 38.214 clause 8.1.3.2. Applicable when PSFCH is configured in the resource pool. Value set automatically by NR-SS-UE according to TS 38.214 clause 8.1.3.2 and TS 38.213 clause 16.3. |

NR_SL_PSFCH_ConflictInfoReceiverFlag_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PSFCH_ConflictInfoReceiverFlag_Type |  |
| Comment | TS 38.212 clause 8.3.1.1: Conflict information receiver flag - 0 or 1 bit |  |
| None | Null_Type |  |
| Value | B1_Type | 1 bit - if higher layer parameter indicationUEBScheme2 is configured to 'Enabled', where the bit value of 0 indicates that the UE cannot be a UE to receive conflict information and the bit value of 1 indicates that the UE can be a UE to receive conflict information as defined in Clause 16.3.0 of TS 38.213 |

NR_SL_SciFormat1A_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SciFormat1A_Type |  |  |
| Comment | Acc to TS 38.212 cl 8.3.1.1 SCI format 1-A For all fields: 'omit' means that the information shall not be contained in the SCI |  |  |
| Priority | B3_Type | opt | Priority - 3 bits as specified in clause 5.4.3.3 of TS 23.287 and clause 5.22.1.3.1 of TS 38.321. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on. |
| FreqRessourceAssignment | NR_SL_FreqRessourceAssignment_Type | opt | Frequency resource assignment - Number of bits depending on sl-MaxNumPerReserve |
| TimeResourceAssignment | integer | opt | Time resource assignment |
| ResourceReservationPeriod | NR_SL_ResourceReservationPeriod_Type | opt | Resource reservation period - present if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise. |
| Dmrs_Pattern | integer | opt | DMRS pattern |
| SecondStageSCI_Format | B2_Type | opt | 2nd-stage SCI format - 2 bits as defined in Table 8.3.1.1-1. |
| Beta_offset | B2_Type | opt | Beta_offset indicator - 2 bits as provided by higher layer parameter sl-BetaOffsets2ndSCI and Table 8.3.1.1-2. |
| NumberDMRS_Port | B1_Type | opt | Number of DMRS port - 1bit as defined in Table 8.3.1.1-3. |
| AdditionalMCS | NR_SL_AdditionalMCS_Type | opt | Additional MCS table indicator |
| PSFCH_OverheadIndication | NR_SL_PSFCH_OverheadIndication_Type | opt | PSFCH overhead indication |
| PSFCH_ConflictInfoReceiverFlag | NR_SL_PSFCH_ConflictInfoReceiverFlag_Type | opt | Conflict information receiver flag |

NR_SL_PSSCH_Sci_Tx_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PSSCH_Sci_Tx_Type |  |
| Comment |  |  |
| SciFormat2A | NR_SL_SciFormat2A_Type | Acc to TS 38.212 cl 8.4.1.1 SCI format 2-A |
| SciFormat2B | NR_SL_SciFormat2B_Type | Acc to TS 38.212 cl 8.4.1.2 SCI format 2-B |

NR_SL_SciFormat2A_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SciFormat2A_Type |  |  |
| Comment | Acc to TS 38.212 cl 8.4.1.1 SCI format 2-A Source ID and Destination ID provided in NR_SL_MAC_LogicalChannelConfig_Type |  |  |
| HarqProcessConfig | NR_HarqProcessConfig_Type | opt | configures which HARQ processes the NR-SS-UE may use; corresponds to HARQ process number - 4 bits. Acc to TS 38.321 clause 5.22.1.3.1: The maximum number of transmitting Sidelink processes associated with the Sidelink HARQ Entity is 16. |
| ToggleNDI | boolean | opt | for NR-SS-UE configured in Tx else NA "true" for transmission of a new transport block, "false" for a re-transmission; Not present for automatic mode: the NDI (New data indicator) itself is maintained by the NR-SS-UE and therefore not provided as configuration parameter |
| RedundancyVersionList | NR_RedundancyVersionList_Type | opt | Redundancy version - 2 bits as defined in TS 38.212 Table 7.3.1.1.1-2. list of Redundancy versions to be used for DL transmission and possible retransmissions; if there are not enough elements to achieve successful DL transmission, NR-SS-UE shall raise an error |
| HarqFeedback | B1_Type | opt | HARQ feedback enabled/disabled indicator - 1 bit as defined in TS 38.213 clause 16.3 |
| CastType | B2_Type | opt | Cast type indicator - 2 bits as defined in TS 38.212 Table 8.4.1.1-1 and in TS 38.214 clause 8.1 |
| CSI_Request | B1_Type | opt | CSI request - 1 bit as defined in TS 38.214 cl 8.2.1 and 8.1. |

NR_SL_SciFormat2B_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SciFormat2B_Type |  |  |
| Comment | Ac to TS 38.212 clause 8.4.1.2 SCI format 2-B Source ID and Destination ID provided in NR_SL_MAC_LogicalChannelConfig_Type |  |  |
| HarqProcessConfig | NR_HarqProcessConfig_Type | opt | configures which HARQ processes the NR-SS-UE may use; corresponds to HARQ process number - 4 bits. Acc to TS 38.321 clause 5.22.1.3.1: The maximum number of transmitting Sidelink processes associated with the Sidelink HARQ Entity is 16. |
| ToggleNDI | boolean | opt | for NR-SS-UE configured in Tx else NA "true" for transmission of a new transport block, "false" for a re-transmission; Not present for automatic mode: the NDI (New data indicator) itself is maintained by the NR-SS-UE and therefore not provided as configuration parameter |
| RedundancyVersionList | NR_RedundancyVersionList_Type | opt | Redundancy version - 2 bits as defined in TS 38.212 Table 7.3.1.1.1-2. list of Redundancy versions to be used for DL transmission and possible retransmissions; if there are not enough elements to achieve successful DL transmission, NR-SS-UE shall raise an error |
| HarqFeedback | B1_Type | opt | HARQ feedback enabled/disabled indicator - 1 bit as defined in TS 38.213 clause 16.3 |
| ZoneID | B12_Type | opt | Zone ID - 12 bits as defined in clause 5.8.11 of TS 38.331. |
| CommunicationRangeReq | B4_Type | opt | Communication range requirement - 4 bits determined by higher layer parameter sl-ZoneConfigMCR-Index |

NR_SL_RelativeTxPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_RelativeTxPower_Type |  |  |
| Comment | Power allocation for OFDM symbols and reference signals - TS 38.508-1 Table 6.2.1.3-1 |  |  |
| SssbRelativeTxPower | NR_SL_SSSB_EPREs_Type | opt |  |
| PscchRelativeTxPower | NR_SL_PSCCH_EPREs_Type | opt |  |
| PsschRelativeTxPower | NR_SL_PSSCH_EPREs_Type | opt |  |
| SlCsiRsRelativeTxPower | NR_SL_CSIRS_EPREs_Type | opt |  |

NR_SL_SSSB_EPREs_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SSSB_EPREs_Type |  |  |
| Comment |  |  |  |
| SsssToNRSSUE | NR_EPRE_Ratio_Type | opt | EPRE ratio of S-SSS to EPRE of NR-SS-UE |
| SpssToSsss | NR_EPRE_Ratio_Type | opt | EPRE ratio of S-PSS to S-SSS |
| PsbchDmrsToSsss | NR_EPRE_Ratio_Type | opt | EPRE ratio of PSBCH DMRS to S-SSS |
| PsbchToPsbchDmrs | NR_EPRE_Ratio_Type | opt | EPRE ratio of PSBCH to PSBCH DMRS |

NR_SL_PSCCH_EPREs_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_PSCCH_EPREs_Type |  |  |
| Comment |  |  |  |
| PscchToNRSSUE | NR_EPRE_Ratio_Type | opt | EPRE ratio of PSCCH DMRS to EPRE or NR-SS-UE |
| PscchToDmrs | NR_EPRE_Ratio_Type | opt | EPRE ratio of PSCCH to PSCCH DMRS |

NR_SL_PSSCH_EPREs_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_PSSCH_EPREs_Type |  |  |
| Comment |  |  |  |
| PsschToNRSSUE | NR_EPRE_Ratio_Type | opt | EPRE ratio of PSSCH DMRS to EPRE of NR-SS-UE |
| PsschToDmrs | NR_EPRE_Ratio_Type | opt | EPRE ratio of PSSCH to PSSCH DMRS |
| PsschTpPtrs | NR_EPRE_Ratio_Type | opt | EPRE ratio of SL PT-RS to PSSCH |
| PsfchToPssch | NR_EPRE_Ratio_Type | opt | EPRE ratio of PSFCH to PSSCH |

NR_SL_CSIRS_EPREs_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_CSIRS_EPREs_Type |  |  |
| Comment |  |  |  |
| SlSciRsToNRSSUE | NR_EPRE_Ratio_Type |  | EPRE ratio of SL CSI-RS to EPRE of NR-SS-UE |

NR_SL_CSI_RS_ConfigRx_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_CSI_RS_ConfigRx_Type |  |
| Comment | NR-SS-UE configured for reception of CSI-RS signal, CSI report trigger in SCI format 2-A and for transmission of sidelink CSI report. |  |
| AddOrReconfigure | NR_SL_CSI_RS_ConfigParamRx_Type | Add/re-configure SL-CSI reception |
| Release | Null_Type | Stop transmission or reception |
| None | Null_Type |  |

NR_SL_CSI_RS_ConfigParamRx_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_CSI_RS_ConfigParamRx_Type |  |  |
| Comment | According to TS 38.321 clause 5.22.1.7 and TS 38.214 clause 8.5. The MAC CE containing sidelink CSI Report (SidelinkCSI_Reporting) shall be transmitted by NR-SS-UE after reception of SCI format 2-A with CSI request set to '1'B. The Sidelink CSI report shall be transmitted at LatencyBoundCSI_Report slots after reception of the SCI format 2-A. |  |  |
| CSI_RS_Config | NR_ASN1_SL_CSI_RS_Config |  |  |
| LatencyBoundCSI_Report | NR_ASN1_SL_LatencyBoundCSI_Report_Type |  | Offset between the SCI format 2-A with CSI request set to '1'B and the transmitted sidelink CSI report provided in SidelinkCSI_Reporting |
| SidelinkCSI_Reporting | NR_SL_MAC_CE_SidelinkCSI_Reporting_Type |  | Sidelink CSI report to be transmitted by NR-SS-UE - TS 38.321 clause 6.1.3.35 |

NR_SL_CSI_RS_ConfigTx_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_CSI_RS_ConfigTx_Type |  |
| Comment |  |  |
| AddOrReconfigure | NR_SL_CSI_RS_ConfigParamTx_Type | Add/re-configure sidelink CSI for transmission |
| Release | Null_Type | Stop transmission or reception |
| None | Null_Type |  |

NR_SL_CSI_RS_ConfigParamTx_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_CSI_RS_ConfigParamTx_Type |  |  |
| Comment | According to TS 38.214 clause 8.5.2.2. CSI-RS shall be transmitted when SCI format 2-A with SCI request set to '1' is transmitted. SlCsiRsRelativeTxPower shall be provided in NR_SL_SSSB_EPREs_Type. |  |  |
| SL_CSI_RS_Config | NR_ASN1_SL_CSI_RS_Config |  |  |

NR_ASN1_SL_CSI_RS_Config

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_ASN1_SL_CSI_RS_Config |  |
| Comment |  |  |
| R16 | SL_CSI_RS_Config_r16 |  |

NR_SynchronisationTiming_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SynchronisationTiming_Type |  |
| Comment | To configure/reconfigure NR-SS-UE with the timing for synchronisation |  |
| InCoverage | NR_InCoverageConfig_Type | To configure NR-SS-UE to synchronise on NR Cell |
| UTC_GNSS_Synchronised | NR_UTC_SynchronisedConfig_Type | To configure NR-SS-UE to synchronise on UTC of a GNSS signal |
| OutOfCoverageTiming | NR_OutOfCoverageTiming_Type | To configure NR-SS-UE out of coverage - no synchronisation source |
| SyncRefUE | Null_Type | NR-SS-UE synchronises using UE under test - NR-SS-UE configured in SyncConfigRxMode |

NR_InCoverageConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_InCoverageConfig_Type |  |  |
| Comment |  |  |  |
| NR_CellId | NR_CellId_Type |  | NR cell Identity on which NR-SS-UE shall synchronise |

NR_UTC_SynchronisedConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_UTC_SynchronisedConfig_Type |  |  |
| Comment | NR-SS-UE selects GNSS as the synchronization reference source - As per TS 38.331 clause 5.8.12 |  |  |
| SL_OffsetDFN | integer |  |  |

NR_OutOfCoverageTiming_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_OutOfCoverageTiming_Type |  |  |
| Comment |  |  |  |
| TcOffset | integer (0..63) | opt | For NR according to TS 38.211 clause 4.1 Ts/Tc = 64 with Tc = 1/(480000 * 4096) and Ts = 1/(15000 * 2048); |
| Tcell | integer (0..307199) | opt | Frame duration Tf = 307200 * Ts = 10ms; System Time Unit Ts = 1/(15000 * 2048) |
| DfnOffset | integer (0..1023) | opt |  |

NR_SL_SyncConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SyncConfig_Type |  |  |
| Comment | To configure NR-SS-UE for SLSS and MasterInformationBlockSidelink transmission/reception |  |  |
| SyncConfigTxMode | NR_SL_SyncConfigTxModeList_Type | opt | A list needed in case one NR-SS-UE is configured to transmit different PSBCH |
| SyncConfigRxMode | NR_SL_SyncConfigRxMode_Type | opt |  |

NR_SL_SyncConfigTxModeList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_SyncConfigTxModeList_Type |
| Comment |  |
| record of NR_SL_SyncConfigTxMode_Type |  |

NR_SL_SyncConfigTxMode_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SyncConfigTxMode_Type |  |
| Comment |  |  |
| None | Null_Type | NR-SS-UE not configured for S-SS/PSBCH transmission - Used only for initial configuration when no transmission is required |
| SyncConfigTx | NR_SL_SyncConfigTx_Type |  |

NR_SL_SyncConfigTx_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SyncConfigTx_Type |  |
| Comment | To configure/reconfigure NR-SS-UE to start or stop transmitting SLSS and MasterInformationBlockSidelink |  |
| AddOrReconfigure | NR_SL_SyncTx_Type | Add/re-configure S-SS/PSBCH |
| Release | Null_Type | Stop transmission |

NR_SL_SyncTx_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SyncTx_Type |  |  |
| Comment | To configure NR-SS-UE to transmit Reference signal, S-SS and MasterInformationBlockSidelink acc to TS 38.211 clause 8.4, TS 38.212 clause 8.1, TS 38.213 clause 8.4.3.1. Acc to TS 38.213 cl 16.1 S-PSS symbol, a S-SSS symbol, and a PSBCH symbol have a same transmission power NR-SS-UE is configured in transmission mode with the associated SL_TxPool |  |  |
| TxSyncConfig | NR_TxSyncRefParam_Type | opt |  |
| SL_TDD_Configuration | TDD_UL_DL_ConfigCommon | opt | Corresponding SL-ResourcePool-r16.sl-RxParametersNcell-r16.sl-TDD-Configuration-r16 |
| Spss | NR_SidelinkPrimarySyncSignal_Type | opt | SL primary synchronization signals (S-PSS) |
| Ssss | NR_SidelinkSecondarySyncSignal_Type | opt | SL secondary synchronization signals (S-SSS) |
| Psbch | NR_PsbchConfig_Type | opt | PSBCH blocks |

NR_TxSyncRefParam_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_TxSyncRefParam_Type |  |  |
| Comment |  |  |  |
| SL_SSID | SL_SyncConfig_r16.sl_SSID_r16 |  |  |
| MasterInformationBlockSidelink | MasterInformationBlockSidelink | opt | directFrameNumber and slotIndex set to a dummy value - to be set by NR-SS-UE MasterInformationBlockSidelink using fixed periodicity (16ms) to TS 38.213 clause 16.1; The position of S-SS/PSBCH blocks in frequency and time domain is specified in TS 38.211 clause 8.4.3 and TS 38.213 clause 16.1. The NR-SS-UE configuration for S-SS/PSBCH blocks is defined by SL_SSB_TimeAllocation_r16 and NR_SL_Frequency_Type |
| SL_SSB_TimeAllocation | SL_SSB_TimeAllocation_r16 | opt |  |

NR_SidelinkPrimarySyncSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SidelinkPrimarySyncSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NR_SidelinkChannelPower_Type | opt | Power for S-PSS relative to the NR-SS-UE power level - value in dB |

NR_SidelinkSecondarySyncSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SidelinkSecondarySyncSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NR_SidelinkChannelPower_Type | opt | Power for S-SSS relative to the NR-SS-UE power level - value in dB |

NR_PsbchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_PsbchConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NR_SidelinkChannelPower_Type | opt | Power for PSBCH relative to the NR-SS-UE power level - value in dB |

NR_SL_SyncConfigRxMode_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SyncConfigRxMode_Type |  |
| Comment |  |  |
| None | Null_Type | NR-SS-UE not configured for S-SS/PSBCH reception - Used only for initial configuration when no reception is required |
| SyncConfigRx | NR_SL_SyncConfigRx_Type |  |

NR_SL_SyncConfigRx_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SyncConfigRx_Type |  |
| Comment | to configure/reconfigure NR-SS-UE to start or stop receiving S-SS/PSBCH - FFS is a list is needed |  |
| AddOrReconfigure | NR_SL_SyncRx_Type | Add/re-configure reception of SLSSID |
| Release | Null_Type | Stop SLSS and SBCCH reception |

NR_SL_SyncRx_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SyncRx_Type |  |  |
| Comment | To configure NR-SS-UE for reception of S-SS/PSBCH with one or two synchronisation SSB Time allocation sources. |  |  |
| SL_SSID | NR_SL_SSID_Type | opt |  |
| SL_SSB_TimeAllocation | NR_SL_SSB_TimeAllocation_Type | opt | List of the SSB_TimeAllocation to be used by UE |

NR_SL_SSB_TimeAllocation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SSB_TimeAllocation_Type |  |
| Comment |  |  |
| SL_SSB_TimeAllocationList | NR_SL_SSB_TimeAllocationList_Type | List of the SSB_TimeAllocation to be used by UE |
| AnyValue | Null_Type | Any value is expected |

NR_SL_SSB_TimeAllocationList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_SSB_TimeAllocationList_Type |
| Comment |  |
| record length(1..2) of SL_SSB_TimeAllocation_r16 |  |

NR_SL_SSID_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SSID_Type |  |
| Comment |  |  |
| SL_SSId_Value | SL_SSId_Type | As per TS 38.211 clause 8.4.2.1 |
| AnyValue | Null_Type | Any value is expected |

NR_SL_System_IndicationControl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_System_IndicationControl_Type |  |
| Comment | Initially all indications are disabled in NR-SS-UE (i.e. it shall not be necessary in 'normal' test cases to use this primitive but only if a specific indication is needed); omit means indication mode is not changed. TimingInfo : 'Now' (in general) |  |
| SyncRefReporting | IndicationAndControlMode_Type | To enable/disable reporting of start and stop of reception of NR sidelink S-SS/PSBCH. NOTE: this is applicable when NR-SS-UE is configured with SyncConfigRx NR-SS-UE reports an indication every time when SLSSID/MasterInformationBlockSidelink reception from UE has toggled in the subframes configured for SLSS/MasterInformationBlockSidelink reception |
| PSSCH_SciReporting | IndicationAndControlMode_Type | To enable/disable reporting of reception of NR PSSCH SCI-2A or SCI-2B NOTE: NR-SS-UE reports an indication every time an SCI format 2-A or SCI format 2-B is received |
| SidelinkCSIReportingMAC_CE | IndicationAndControlMode_Type | To enable/disable reporting of Sidelink CSI Reporting MAC CE received from UE |

### D.11.2.3	NR_SL_RadioBearerConfig

#### D.11.2.3.1	NR_SL_RadioBearerConfigGeneric

NR_SL_RadioBearerConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_RadioBearerConfigInfo_Type |  |  |
| Comment |  |  |  |
| Sdap | NR_SL_SDAP_Configuration_Type | opt | mandatory for initial configuration; omit means "keep as it is" for SL-SRBs: Sdap.None:=true |
| Pdcp | NR_SL_PDCP_Configuration_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| RlcBearer | NR_SL_RlcBearerConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

NR_SL_RadioBearerConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_RadioBearerConfig_Type |  |
| Comment |  |  |
| AddOrReconfigure | NR_SL_RadioBearerConfigInfo_Type | add / re-configure SL-RB |
| Release | Null_Type | release SL-RB |

NR_SL_RadioBearer_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_RadioBearer_Type |  |  |
| Comment |  |  |  |
| Id | NR_SL_RadioBearerId_Type |  |  |
| Config | NR_SL_RadioBearerConfig_Type |  |  |

NR_SL_RadioBearerList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_RadioBearerList_Type |
| Comment | array of SL-SRBs and/or SL-DRBs. TimingInfo: 'Now' in general; activation time may be used in special case for release and/or reconfiguration of one or several SL-RBs; the following rules shall be considered: - release/Reconfiguration of an SL-RB shall not be scheduled earlier than 5ms after a previous data transmission on this SL-RB - subsequent release and reconfiguration(s) shall be scheduled with an interval of at least 5ms - a subsequent data transmission on an SL-RB shall not be scheduled earlier than 5ms after the last reconfiguration of the SL-RB the configuration shall be performed exactly at the given time ControlInfo : FollowOnFlag:=false |
| record of NR_SL_RadioBearer_Type |  |

#### D.11.2.3.2	SDAP_Configuration

SDAP_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_SL_SDAP_Header_Type | SL_SDAP_Config_r16.sl_SDAP_Header_r16 |  |

PQFI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PQFI_List_Type |
| Comment |  |
| record of integer |  |

NR_SL_SdapConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SdapConfig_Type |  |  |
| Comment |  |  |  |
| SdapId | integer |  | Sdap identity used in NR-SS-UE as route identity the SL-DRB and route data |
| Sdap_Header | NR_SL_SDAP_Header_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| MappedQoS_Flows | PQFI_List_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

NR_SL_SdapTransparentMode_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SdapTransparentMode_Type |  |  |
| Comment |  |  |  |
| Sdap_Header | NR_SL_SDAP_Header_Type |  | Indicates to the NR-SS-UE if the UE has been configured to include SDAP header |

NR_SL_SdapConfigInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SdapConfigInfo_Type |  |
| Comment |  |  |
| SdapConfig | NR_SL_SdapConfig_Type | SDAP configuration parameters for the SL-DRB |
| TransparentMode | NR_SL_SdapTransparentMode_Type | SDAP configuration for transparent (test) mode, used for SDAP tests: NR-SS-UE does not add any SDAP headers in transmission and does not remove any SDAP headers in reception |

NR_SL_SDAP_Configuration_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SDAP_Configuration_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Config | NR_SL_SdapConfigInfo_Type |  |

#### D.11.2.3.3	PDCP_Configuration

PDCP_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_SL_PDCP_SN_Size_Type | SL_PDCP_Config_r16.sl_PDCP_SN_Size_r16 |  |
| NR_SL_PDCP_SDU_Type | B3_Type | Acc to TS 38.323 cl 6.3.12 |

NR_CastType_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_CastType_Type |
| Comment |  |
| broadcast |  |
| groupcast |  |
| unicast |  |

NR_SL_PDCP_DRB_Config_Parameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_PDCP_DRB_Config_Parameters_Type |  |  |
| Comment | parameters corresponding to NR ASN.1 SL-PDCP-Config |  |  |
| SN_Size | NR_SL_PDCP_SN_Size_Type |  | SL-PDCP-Config.sl-PDCP-SN-Size-r16 |
| PDCP_SDU_Data | NR_SL_PDCP_SDU_Type | opt | Only applicable to SL-DRB Corresponds to the V2X data type: IP/non-IP |
| CastType | NR_CastType_Type | opt | Only applicable to SL-DRB |

NR_SL_PDCP_TransparentMode

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_PDCP_TransparentMode |  |  |
| Comment |  |  |  |
| SN_Size | NR_SL_PDCP_SN_Size_Type |  |  |

NR_SL_PDCP_RbConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PDCP_RbConfig_Type |  |
| Comment |  |  |
| Params | NR_SL_PDCP_DRB_Config_Parameters_Type | PDCP configuration parameters corresponding to UE configuration |
| TransparentMode | NR_SL_PDCP_TransparentMode | PDCP configuration for transparent (test) mode: used for PDCP tests (TS 38.523-3, cl. 5.3.2.2): NR-SS-UE applies NULL integrity algorithm and NULL ciphering, header compression is not configured. Note: a reconfiguration of a SL-RB from transparent mode to 'normal' mode is not foreseen |

NR_SL_PDCP_Configuration_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PDCP_Configuration_Type |  |
| Comment |  |  |
| None | Null_Type | the PDCP may not be configured e.g. for SL-DRBs tested in RLC or MAC test cases |
| SLRB | NR_SL_PDCP_RbConfig_Type |  |

#### D.11.2.3.4	RLC_Configuration

RLC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_SL_UM_RLC_Type | SL_RLC_Config_r16.sl_UM_RLC_r16 |  |
| NR_SL_AM_RLC_Type | SL_RLC_Config_r16.sl_AM_RLC_r16 |  |

NR_SL_RLC_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_RLC_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Info | NR_RLC_TestModeInfo_Type |  |

NR_SL_AM_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_AM_Config_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| AM | NR_SL_AM_RLC_Type |  |

NR_SL_RLC_AM_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_RLC_AM_Type |  |  |
| Comment |  |  |  |
| Tx | NR_SL_AM_Config_Type | opt |  |
| Rx | NR_SL_AM_Config_Type | opt |  |

NR_SL_UM_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_UM_Config_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| UM | NR_SL_UM_RLC_Type | For groupcast and broadcast of NR sidelink communication, only 6 bit SN length is configured |

NR_SL_RLC_UM_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_RLC_UM_Type |  |  |
| Comment |  |  |  |
| Tx | NR_SL_UM_Config_Type | opt |  |
| Rx | NR_SL_UM_Config_Type | opt |  |

NR_SL_RLC_RbConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_RLC_RbConfig_Type |  |
| Comment | For groupcast and broadcast only uni-directional UM mode is supported |  |
| RlcAM | NR_SL_RLC_AM_Type |  |
| RlcUM | NR_SL_RLC_UM_Type |  |

NR_SL_RLC_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_RLC_Configuration_Type |  |  |
| Comment |  |  |  |
| Rb | NR_SL_RLC_RbConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | NR_SL_RLC_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

NR_SL_LogicalChannelIdentity_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_LogicalChannelIdentity_Type |  |
| Comment |  |  |
| LCID | integer | Refer to 38.321 Table 6.2.4-1 |
| AnyValue | Null_Type | Any value allowed from 4 to 19 - Used for NR-SS-UE reception - Acc to 38.331 clause 9.1.1.5: Selected by the transmitting UE, up to UE implementation |

NR_SL_RlcBearerConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_RlcBearerConfig_Type |  |  |
| Comment |  |  |  |
| Rlc | NR_SL_RLC_Configuration_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| LogicalChannelId | NR_SL_LogicalChannelIdentity_Type | opt | As per TS 38.321 Table 6.2.4-1 Values of LCID for SL-SCH |
| Mac | NR_SL_MAC_Configuration_Type | opt |  |

#### D.11.2.3.5	MAC_Configuration

MAC configuration: radio bearer specific configuration

MAC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NR_SL_Priority_Type | SL_LogicalChannelConfig_r16.sl_Priority_r16 |  |
| NR_SL_PrioritisedBitRate_Type | SL_LogicalChannelConfig_r16.sl_PrioritisedBitRate_r16 |  |
| NR_SL_HARQ_FeedbackEnabled_Type | SL_LogicalChannelConfig_r16.sl_HARQ_FeedbackEnabled_r16 |  |

NR_SL_MAC_Test_NoHeaderManipulation_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_SL_MAC_Test_NoHeaderManipulation_Type |
| Comment |  |
| normalMode | MAC header is fully controlled by the NR-SS-UE |
| transmission_Only | TTCN can submit a final MAC PDU including header and payloads; NR-SS-UE does not do anything with this MAC PDU i.e. no header is added for the PSSCH transport channel. It is possible that data belonging to multiple SL-DRBs is sent in one MAC PDU and from one special RB configured. NOTE: SL-SRBs shall work as in normal mode and data can be sent/received on SL-SRBs but sending on SL-SRBs shall be in different sidelink slots than sending data PDUs. |
| transmissionReception | In reception and transmission the NR-SS-UE MAC layer is transparent i.e. NR-SS-UE does not add or remove any MAC header. |

NR_SL_MAC_TestModeInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_MAC_TestModeInfo_Type |  |  |
| Comment | Parameters/Configuration for MAC test mode |  |  |
| No_HeaderManipulation | NR_SL_MAC_Test_NoHeaderManipulation_Type |  | to configure mode for no header manipulation in NR-SS-UE MAC layer for transmission/reception on PSSCH. |

NR_SL_MAC_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_MAC_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Info | NR_SL_MAC_TestModeInfo_Type |  |

NR_SL_Layer2Id_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_Layer2Id_Type |  |
| Comment |  |  |
| AnyValue | Null_Type | Any value. Used only when NR-SS-UE is configured in reception. |
| Value | B24_Type | Mandatory when NR-SS-UE is configured in transmission. |

NR_SL_MAC_LogicalChannelConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_MAC_LogicalChannelConfig_Type |  |  |
| Comment |  |  |  |
| Priority | NR_SL_Priority_Type | opt | Logical channel priority for the DL as described in TS 38.321, clause 5.4.3.1 for the Tx. |
| PrioritizedBitRate | NR_SL_PrioritisedBitRate_Type | opt | PBR as described for the Tx; probably not needed at NR-SS-UE. |
| HARQ_FeedbackEnabled | NR_SL_HARQ_FeedbackEnabled_Type | opt | Indicates the HARQ feedback enabled/disabled restriction in LCP for this sidelink logical channel. According to TS 38.321 clause 5.22.1.1 and 5.22.1.3. |
| NR_SS_UE_Layer2Identity | NR_SL_Layer2Id_Type | opt | The Layer-2 Identity of the NR-SS-UE - Corresponding to the source Layer-2 Id for NR-SS-UE transmission and to the destination Layer-2 Id for NR-SS-UE reception. |
| UE_Layer2Identity | NR_SL_Layer2Id_Type | opt | The UE under test Layer 2 Identity - Corresponding to the source Layer-2 Id for NR-SS-UE reception and to the destination Layer-2 Id for NR-SS-UE transmission. |

NR_SL_MAC_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_MAC_Configuration_Type |  |  |
| Comment |  |  |  |
| LogicalChannelConfig | NR_SL_MAC_LogicalChannelConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | NR_SL_MAC_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is"; for non MAC tests "TestMode.None:=true" |

### D.11.2.4	NR_SL_Security

NR_SL_PDCP_ActTime_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PDCP_ActTime_Type |  |
| Comment |  |  |
| None | Null_Type | No Activation time; to be used if Null Ciphering is applied |
| SQN | NR_PdcpSQN_Type | PDCP sequence number |

NR_SL_SecurityActTime_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SecurityActTime_Type |  |  |
| Comment |  |  |  |
| RadioBearerId | NR_SL_RadioBearerId_Type |  |  |
| Tx | NR_SL_PDCP_ActTime_Type |  |  |
| Rx | NR_SL_PDCP_ActTime_Type |  |  |

NR_SL_SecurityActTimeList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_SecurityActTimeList_Type |
| Comment |  |
| record of NR_SL_SecurityActTime_Type |  |

NR_SL_IntegrityProtAlgorithm

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_SL_IntegrityProtAlgorithm |
| Comment | Defined in TS 24.587 cl. 8.4.18 sl5G_IA0 is applied |
| sl5G_IA0 |  |
| sl128_5G_IA1 |  |
| sl128_5G_IA2 |  |
| sl128_5G_IA3 |  |
| sl5G_IA4 |  |
| sl5G_IA5 |  |
| sl5G_IA6 |  |
| sl5G_IA7 |  |

NR_SL_IntegrityInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_IntegrityInfo_Type |  |  |
| Comment | for initial configuration activation time is not needed for integrity protection as all messages in Tx and Rx after security activation are integrity protected; this means this ASP is invoked before transmission of Direct link security mode command; if there is a integrity violation in Rx NR-SS-UE shall set the IndicationStatus in the common ASP part to flag the integrity error (IndicationStatus.Error.Integrity.Pdcp := true); integrity to be provided for each SL-SRB and SL-DRB as per core spec Null integrity alogrithm is applied and therefore no keys are provided to NR-SS-UE |  |  |
| Algorithm | NR_SL_IntegrityProtAlgorithm |  | Integrity Algorithm being defined in TS 24.587 |
| ActTimeList | NR_SL_SecurityActTimeList_Type | opt | omit for initial configuration (i.e. all SL-SRBs and SL-DRBs to be integrity protected immediately) |

NR_SL_CipheringAlgorithmtypes

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NR_SL_CipheringAlgorithmtypes |
| Comment | Defined in TS 24.587 cl. 8.4.18 sl5G_EA0 is applied |
| sl5G_EA0 |  |
| sl128_5G_EA1 |  |
| sl128_5G_EA2 |  |
| sl128_5G_EA3 |  |
| sl5G_EA4 |  |
| sl5G_EA5 |  |
| sl5G_EA6 |  |
| sl5G_EA7 |  |

NR_SL_CipheringInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_CipheringInfo_Type |  |  |
| Comment | Null integrity algorithm is applied and therefore no keys are provided to NR-SS-UE |  |  |
| Algorithm | NR_SL_CipheringAlgorithmtypes |  | Ciphering Algorithm being defined in TS 24.587 |
| ActTimeList | NR_SL_SecurityActTimeList_Type |  |  |

NR_SL_SecStartRestart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SecStartRestart_Type |  |  |
| Comment |  |  |  |
| Integrity | NR_SL_IntegrityInfo_Type | opt | optional to allow separated activation of integrity and ciphering; omit: keep as it is |
| Ciphering | NR_SL_CipheringInfo_Type | opt | optional to allow separated activation of integrity and ciphering; omit: keep as it is |

NR_SL_Security_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_Security_Type |  |
| Comment | Security mode command procedure (TS 25.587, clause 6.1.2.7) - Applied for unicast only |  |
| StartRestart | NR_SL_SecStartRestart_Type | information to start/restart security protection in the PDCP TimingInfo : 'Now' (in general) NOTE: "activation time" may be specified in the primitive based on PDCP SN |
| Release | Null_Type | to release security protection in the PDCP (if any; if there is no security the NR-SS-UE does not need to do anything) TimingInfo : 'Now' (in general) NOTE: "activation time |

### D.11.2.5	NR_SL_PDCP_Count

NR_SL_PdcpCountInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_PdcpCountInfo_Type |  |  |
| Comment |  |  |  |
| RadioBearerId | NR_SL_RadioBearerId_Type |  |  |
| Tx | NR_PdcpCount_Type | opt | omit: keep as it is |
| Rx | NR_PdcpCount_Type | opt | omit: keep as it is |

NR_SL_PdcpCountInfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_PdcpCountInfoList_Type |
| Comment |  |
| record of NR_SL_PdcpCountInfo_Type |  |

NR_SL_PdcpCountGetReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PdcpCountGetReq_Type |  |
| Comment |  |  |
| AllSL_RBs | Null_Type | return COUNT values for all SL-RBs being configured |
| SingleRB | NR_SL_RadioBearerId_Type |  |

NR_SL_PDCP_CountReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PDCP_CountReq_Type |  |
| Comment |  |  |
| Get | NR_SL_PdcpCountGetReq_Type | Request PDCP count for one or all SL-RBs being configured at the PDCP TimingInfo: 'Now' |
| Set | NR_SL_PdcpCountInfoList_Type | Set PDCP count for one or all SL-RBs being configured at the PDCP; list for RBs which's COUNT shall be manipulated TimingInfo: 'Now' (in general) |

NR_SL_PDCP_CountCnf_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PDCP_CountCnf_Type |  |
| Comment |  |  |
| Get | NR_SL_PdcpCountInfoList_Type | SL-RBs in ascending order; SL-SRBs first |
| Set | Null_Type |  |

### D.11.2.6	NR_SL_SystemIndication

NR_SL_SyncRefReporting_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SyncRefReporting_Type |  |
| Comment | Start is triggered in NR-SS-UE when - the first S-SS associated with a MasterInformationBlockSidelink is received in the same subframe by NR-SS-UE since enabling of reporting - the previous indication was 'stop' or 'spurious' and an S-SS associated with a MasterInformationBlockSidelink is received by SS-UE Stop is triggered in NR-SS-UE when - no S-SS/MasterInformationBlockSidelink is transmitted in the period - the previous indication was 'start' or 'spurious' and no S-SS/MasterInformationBlockSidelink is transmitted in the period Spurious is triggered in NR-SS-UE when - the first S-SS is received without MasterInformationBlockSidelink or MasterInformationBlockSidelink is received without S-SS since enabling of SLSS reporting - the previous indication was 'start' or 'stop' and S-SS is received without MasterInformationBlockSidelink or MasterInformationBlockSidelink is received without S-SS |  |
| SyncRef_Start | NR_SL_SyncRefParamInd_Type |  |
| SyncRef_Stop | Null_Type |  |
| SyncRef_Spurious | Null_Type |  |

NR_SL_SyncRefParamInd_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SyncRefParamInd_Type |  |  |
| Comment |  |  |  |
| SL_SSId_Value | SL_SSId_Type |  |  |
| MasterInformationBlockSidelink | MasterInformationBlockSidelink |  |  |

NR_SL_PSSCH_SciReporting_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PSSCH_SciReporting_Type |  |
| Comment |  |  |
| SciFormat2A | NR_SL_SciFormat2A_Reporting_Type | Acc to TS 38.212 cl 8.4.1.1 SCI format 2-A |
| SciFormat2B | NR_SL_SciFormat2B_Reporting_Type | Acc to TS 38.212 cl 8.4.1.2 SCI format 2-B |

NR_SL_SciFormat2A_Reporting_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SciFormat2A_Reporting_Type |  |  |
| Comment | Acc to TS 38.212 cl 8.4.1.1 SCI format 2-A |  |  |
| HarqProcessNumber | integer |  |  |
| NewDataIndicator | integer |  |  |
| RedundancyVersion | integer |  |  |
| SourceID | integer |  |  |
| DestinationID | integer |  |  |
| HarqFeedback | integer |  |  |
| CastTypeIndicator | integer |  |  |
| CSI_Request | integer |  |  |

NR_SL_SciFormat2B_Reporting_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SciFormat2B_Reporting_Type |  |  |
| Comment | Ac to TS 38.212 clause 8.4.1.2 SCI format 2-B |  |  |
| HarqProcessNumber | integer |  |  |
| NewDataIndicator | integer |  |  |
| RedundancyVersion | integer |  |  |
| SourceID | integer |  |  |
| DestinationID | integer |  |  |
| HarqFeedback | integer |  |  |
| ZoneID | integer |  |  |
| CommunicationRangeRequirement | integer |  |  |

### D.11.2.7	NR_SL_SCI_Trigger

NR_SL_SCI_Trigger_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SCI_Trigger_Type |  |  |
| Comment |  |  |  |
| Layer2Ids | NR_SL_Layer2Ids_Type |  |  |
| SL_BWP_Id | BWP_Id |  |  |
| SL_PSCCH_SciTrigger | NR_SL_PSCCH_Sci_Tx_Type |  |  |
| SL_PSSCH_SciTrigger | NR_SL_PSSCH_Sci_Tx_Type |  |  |
| CSI_RS_Signal | boolean |  | when set to true the CSI-RS signal shall be transmitted by NR-SS-UE using NR_SL_TxPoolConfig_Type.SL_CSI_RS |

### D.11.2.8	NR_SL_CongestionConfig

NR_SL_CongestionConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_CongestionConfig_Type |  |
| Comment | Configure NR-SS-UE to start or stop generating congestion in all RBs of specific subchannels on specific slots. NOTE: NR-SS-UE already configured in Tx (Timing reference, power, frequency and subchannels are already configured). The configured SCI grant for transmission does apply. |  |
| Activate | NR_SL_SubchannelSlotsConfig_Type | For one NR-SS-UE configuration: SS_UE_Id : identifier of the NR-SS-UE to be configured RoutingInfo : None TimingInfo : activation time from when the SS starts transmission of random data. ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Deactivate | Null_Type | For one NR-SS-UE configuration: SS_UE_Id : identifier of the NR-SS-UE to be configured RoutingInfo : None TimingInfo : activation time from when the SS stops transmitting random data. ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

NR_SL_SubchannelList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_SubchannelList_Type |
| Comment |  |
| record of integer |  |

NR_SL_Subchannels_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_Subchannels_Type |  |
| Comment |  |  |
| All | Null_Type | Bitmap applies to all the subchannels |
| Specific | NR_SL_SubchannelList_Type | List of specific subchannels on which the bitmap applies |

NR_SL_Pattern_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_Pattern_Type |  |  |
| Comment |  |  |  |
| Subchannels | NR_SL_Subchannels_Type |  | Subchannels on which the bitmap applies |
| SlotBitmap | bitstring |  | Bitmap indicating the slots on which congestion is transmitted. Value 0 in the bitmap indicates that the corresponding slot is invalid for congestion transmission. Value 1 in the bitmap indicates that the corresponding slot is valid for congestion transmission. NOTE: The size of the SlotBitmap depends on the SCS. For example, for SCS=15 kHz, it will be 10 bits long. |

NR_SL_PatternPerFrameList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_PatternPerFrameList_Type |
| Comment |  |
| record of NR_SL_Pattern_Type |  |

NR_SL_SubchannelSlotsConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SubchannelSlotsConfig_Type |  |  |
| Comment | NR-SS-UE shall transmit, over PSSCH, QPSK modulated uncorrelated pseudo random data in all RBs on the provided subchannel list and in slots indicated as '1' in the SlotsBitmap The congestion is repeated a number of frames or continuously |  |  |
| CongestionRepetition | CongestionRepetition_Type |  |  |
| PatternPerFrameList | NR_SL_PatternPerFrameList_Type |  | Congestion configuration for one frame |

## D.11.3	NR_Sidelink_System_Interface

NR_SL_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SYSTEM_CTRL_REQ |  |  |
| Comment | ASP to request/control NR Sidelink system configuration |  |  |
| Common | NR_SL_ReqAspCommonPart_Type |  | Unless specified otherwise for a particular primitive, the following applies: SS_UE_Id: identifier of the NR-SS-UE RoutingInfo : 'None' TimingInfo : 'Now' or specific activation time, depends on respective primitive ControlInfo : CnfFlag: depends on TimingInfo; in general 'false' when specific activation time is used, 'true' for 'Now' FollowOnFlag 'false' - not applicable for NR-SS-UE |
| Request | NR_SL_SystemRequest_Type |  | - NR_SS_UE_Config TimingInfo: depends on the request - EnquireTiming TimingInfo: 'now' |

NR_SL_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SYSTEM_CTRL_CNF |  |  |
| Comment | ASP to confirm a Sidelink system configuration request |  |  |
| Common | NR_SL_CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN (apart from EnquireTiming): NR-SS-UE may set TimingInfo to "None" - EnquireTiming TimingInfo InCoverage: DFN/Subframe/Slot |
| Confirm | NR_SL_SystemConfirm_Type |  |  |

NR_SL_SYSTEM_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SYSTEM_IND |  |  |
| Comment | ASP to report errors or system indications |  |  |
| Common | NR_SL_IndAspCommonPart_Type |  | The NR-SS-UE shall provide TimingInfo (SFN + subframe number) depending on the respective indication: |
| Indication | NR_SL_SystemIndication_Type |  | - Error TimingInfo: related to the error (if available) - SyncRefReporting TimingInfo: SFN/Subframe/Slot or DFN/DirectSubframe/Slot related to the S-SS/MasterInformationBlockSidelink received - PSSCH_SciReporting TimingInfo: SFN/Subframe/Slot or DFN/DirectSubframe/Slot related to the received PSSCH SCI - MAC_CE TimingInfo: SFN/Subframe/Slot or DFN/DirectSubframe/Slot related to the received MAC_CE |

NR_SL_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SYSTEM_PORT |  |
| Comment | NR Sidelink UE PTC: Port for system configuration |  |
| out | NR_SL_SYSTEM_CTRL_REQ |  |
| in | NR_SL_SYSTEM_CTRL_CNF |  |

NR_SL_SYSIND_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SYSIND_PORT |  |
| Comment | NR Sidelink UE PTC: Port for system indications |  |
| in | NR_SL_SYSTEM_IND |  |

## D.11.4	NR_Sidelink_Data

NR_SL_U_PlaneTx_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_U_PlaneTx_Type |  |  |
| Comment | NOTE: formal type definition to allow later enhancements |  |  |
| SlotDataList | NR_SL_DRB_DataPerSlotList_Tx_Type |  |  |

NR_SL_DRB_DataPerSlotList_Tx_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_DRB_DataPerSlotList_Tx_Type |
| Comment | list of PC5 user plane data to be sent in slots given by the SlotOffset in the single elements of the list; Timing: the start time for the whole sequence is given by the timing info of the ASP (common information); the timing for the respective data pdus is given by the SlotOffset relative to the common timing info; design consideration: repetitions of this sequence are not foreseen (in which case the slot offset could not be related to the timing info of the ASP) |
| record of NR_SL_DRB_DataPerSlot_Tx_Type |  |

NR_SL_DRB_DataPerSlot_Tx_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_DRB_DataPerSlot_Tx_Type |  |  |
| Comment | common definition for one or several PDUs/SDUs; in NR the DL data is sent in the slot given by the slot offset |  |  |
| SlotOffset | integer |  | NR: Slot offset relative to the absolute timing information given in the common part of the ASP; NOTE: if a PDCP PDU or SDU takes more than one slot, SlotOffset specifies the first slot (TTI) |
| HarqProcess | NR_HarqProcessAssignment_Type | opt | HARQ process to be used: specific value or automatically assigned by NR-SS-UE; in automatic mode NR-SS-UE chooses HARQ process out of the set configured by SciFormat2A_Type.HarqProcessConfig |
| PduSduList | NR_SidelinkDataList_Type |  | list of PDUs/SDUs to be sent in one slot (TTI) |

NR_SL_U_PlaneRx_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_U_PlaneRx_Type |  |  |
| Comment | NOTE: formal type definition to allow later enhancements |  |  |
| SlotDataList | NR_SidelinkDataList_Type |  | It is currently FFS if SCI 1A/2A or 2B should be also reported |

NR_SidelinkDataList_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SidelinkDataList_Type |  |
| Comment |  |  |
| MacPdu | NR_SL_SCH_MAC_PDUList_Type | NR-SS-UE configuration: RLC TM mode, MAC no header removal (PDCP and SDAP are not configured) |
| RlcPdu | NR_RLC_PDUList_Type | NR-SS-UE configuration: RLC TM mode, MAC header removal (PDCP and SDAP are not configured) |
| RlcSdu | NR_RLC_SDUList_Type | NR-SS-UE configuration: RLC UM mode (PDCP and SDAP are not configured) |
| PdcpPdu | NR_SL_PDCP_PDUList_Type | NR-SS-UE configuration: RLC AM/UM mode, no handling of PDCP header, SDAP is not configured |
| PdcpSdu | NR_PDCP_SDUList_Type | NR-SS-UE configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header), SDAP is not configured |
| SdapPdu | NR_SL_SDAP_PDUList_Type | NR-SS-UE configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header), no handling of SDAP header |
| SdapSdu | SDAP_SDUList_Type | NR-SS-UE configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header), automatic handling of SDAP header |

NR_SL_SDAP_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_SDAP_PDUList_Type |
| Comment |  |
| record of NR_SL_SDAP_PDU_Type |  |

NR_SL_SDAP_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SDAP_PDU_Type |  |  |
| Comment | TS 37.324 clause 6.2.2.4 Data PDU |  |  |
| Header | NR_SL_SDAP_PduHeader_Type | opt | present for SL Data PDU with SDAP header configured, omitted for SL Data PDU without SDAP header |
| Data | SDAP_SDU_Type |  |  |

NR_SL_SDAP_PduHeader_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SDAP_PduHeader_Type |  |  |
| Comment | TS 37.324 Figure 6.2.2.4-1 |  |  |
| DC | B1_Type |  | TS 37.324 clause 6.3.3: The D/C bit indicates whether the SDAP PDU is an SDAP Data PDU or an SDAP Control PDU 1 bit: '0'B Control PDU '1'B Data PDU |
| R | B1_Type |  | TS 37.324 clause 6.3.5: Reserved. In this version of the specification reserved bits shall be set to 0. Reserved bits shall be ignored by the receiver |
| PQFI | B6_Type |  | PQFI (SL) TS 37.324 clause 6.3.8: The PQFI field indicates the ID of the PC5 QoS flow to which the SDAP PDU belongs |

NR_SL_SCH_MAC_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_SCH_MAC_PDUList_Type |
| Comment |  |
| record of NR_SL_SCH_MAC_PDU_Type |  |

NR_SL_SCH_MAC_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SCH_MAC_PDU_Type |  |  |
| Comment | Acc to 38.321 clause 6.1.6 No field from SL-SCH MAC subheader is needed: V is set to '0000'B SRC and DST are already provided in NR_SL_MAC_LogicalChannelConfig_Type.NR_SS_UE_Layer2Identity and NR_SL_MAC_LogicalChannelConfig_Type.UE_Layer2Identity |  |  |
| SDU_SubPDUList | NR_MAC_SDU_SubPDU_List_Type | opt | list of subPDUs with MAC SDU |
| CE_SubPDUList | NR_MAC_CE_SubPDU_UL_List_Type | opt | list of subPDUs with MAC CE |
| Padding_SubPDU | NR_MAC_Padding_SubPDU_Type | opt | subPDU with padding |

NR_SL_MAC_ControlElement_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_MAC_ControlElement_Type |  |
| Comment |  |  |
| SidelinkCSI_Reporting | NR_SL_MAC_CE_SidelinkCSI_Reporting_Type | TS 38.321 clause 6.1.3.35 |

NR_SL_MAC_CE_SidelinkCSI_Reporting_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_MAC_CE_SidelinkCSI_Reporting_Type |  |  |
| Comment | TS 38.321 clause 6.1.3.35 |  |  |
| RI | B1_Type |  |  |
| CQI | B4_Type |  |  |
| Reserved | B3_Type |  |  |

NR_SL_PDCP_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NR_SL_PDCP_PDUList_Type |
| Comment |  |
| record of NR_SL_PDCP_PDU_Type |  |

NR_SL_PDCP_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SL_PDCP_PDU_Type |  |
| Comment |  |  |
| DataPduGroupBroadcast | NR_SL_PDCP_DataPduGroupBroadcast_Type |  |
| DataPduUnicastSN12Bits | NR_SL_PDCP_DataPduUnicastSN12Bits_Type |  |
| DataPduUnicastSN18Bits | NR_SL_PDCP_DataPduUnicastSN18Bits_Type |  |
| CtrlPduStatus | NR_PDCP_CtrlPduStatus_Type |  |

NR_SL_PDCP_DataPduGroupBroadcast_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_PDCP_DataPduGroupBroadcast_Type |  |  |
| Comment | Data PDU for sidelink DRBs for groupcast and broadcast (TS 38.323, clause 6.2.2.4) |  |  |
| SduType | B3_Type |  | 3 bits sdu type |
| Reserved | B1_Type |  | 1 bit reserved |
| SequenceNumber | B12_Type |  | 12 bits sequence number |
| SDU | NR_PDCP_SDU_Type |  | content (octetstring) |

NR_SL_PDCP_DataPduUnicastSN12Bits_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_PDCP_DataPduUnicastSN12Bits_Type |  |  |
| Comment | Data PDU for sidelink DRBs for unicast with 12 bits PDCP SN (TS 38.323, clause 6.2.2.6) |  |  |
| D_C | B1_Type |  | 1 bit, '1'B for Data PDU |
| SduType | B3_Type |  | 3 bits sdu type |
| SequenceNumber | B12_Type |  | 12 bits sequence number |
| K_NRPsessID | B16_Type |  | 16 bits K_NRPsess identity; set to "0" for SLRB that does not need integrity and ciphering protection |
| SDU | NR_PDCP_SDU_Type |  | content (octetstring) |
| MAC_I | B32_Type | opt | message authentication code according to TS 38.323, clause 6.3.4; MAC-I field is present only when the DRB is configured with integrity protection; in this case it is up to TTCN to provide the valid MAC_I in DL and to check it in UL |

NR_SL_PDCP_DataPduUnicastSN18Bits_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_PDCP_DataPduUnicastSN18Bits_Type |  |  |
| Comment | Data PDU for sidelink DRBs for unicast with 18 bits PDCP SN (TS 38.323, clause 6.2.2.7) |  |  |
| D_C | B1_Type |  | 1 bit, '1'B for Data PDU |
| SduType | B3_Type |  | 3 bits sdu type |
| Reserved | B2_Type |  | 2 bits reserved |
| SequenceNumber | B18_Type |  | 18 bits sequence number |
| K_NRPsessID | B16_Type |  | 16 bits K_NRPsess identity; set to "0" for SLRB that does not need integrity and ciphering protection |
| SDU | NR_PDCP_SDU_Type |  | content (octetstring) |
| MAC_I | B32_Type | opt | message authentication code according to TS 38.323, clause 6.3.4; MAC-I field is present only when the DRB is configured with integrity protection; in this case it is up to TTCN to provide the valid MAC_I in DL and to check it in UL |

NR_SL_DATA_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_DATA_REQ |  |  |
| Comment | Common ASP to send PDUs on PC5 channels SL-DCH |  |  |
| Common | NR_SL_ReqAspCommonPart_Type |  | Unless specified otherwise for a particular primitive, the following applies: SS_UE_Id: identifier of the NR-SS-UE RoutingInfo : SL-SRB or SL-DRB or SL-QosFlow TimingInfo : 'Now' or specific activation time, depends on respective primitive ControlInfo : CnfFlag:=false; FollowOnFlag:=false |
| U_Plane | NR_SL_U_PlaneTx_Type |  |  |

NR_SL_DATA_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_DATA_IND |  |  |
| Comment | Common ASP to receive PDUs from PC5 channels SL-DCH |  |  |
| Common | NR_SL_IndAspCommonPart_Type |  | SS_UE_Id : identifier of the SL UE RoutingInfo : SL-SRB or SL-DRB or SL-QosFlow TimingInfo : SFN/Subframe or DFN/DirectSubframe when message has been received |
| U_Plane | NR_SL_U_PlaneRx_Type |  |  |

NR_SL_DRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NR_SL_DRB_PORT |  |
| Comment |  |  |
| out | NR_SL_DATA_REQ |  |
| in | NR_SL_DATA_IND |  |

## D.11.5	NR_Sidelink_SRBs

ASP Definitions to send/receive peer-to-peer messages on SL-SRBs

NR_PC5_RRC_Message_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_PC5_RRC_Message_Type |  |
| Comment | PC5-RRC PDU on SCCH |  |
| Scch | SCCH_Message |  |

NR_SRB_Request_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SRB_Request_Type |  |
| Comment |  |  |
| PC5_S | NG_V2X_Message_Type | PC5-S message to be sent to UE on SL-SRB0/1/2 |
| PC5_RRC | NR_PC5_RRC_Message_Type | PC5-RRC message to be sent on SL-SRB3 |

NR_SRB_Indication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NR_SRB_Indication_Type |  |
| Comment |  |  |
| PC5_S | NG_V2X_Message_Type | PC5-S message received from UE on SL-SRB0/1/2 |
| PC5_RRC | NR_PC5_RRC_Message_Type | PC5-RRC message received from UE on SL-SRB3 |

NR_SL_SRB_COMMON_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SRB_COMMON_REQ |  |  |
| Comment | common ASP to send PDUs to SL_SRB0, SL_SRB1, SL_SRB2, SL_SRB3 |  |  |
| Common | NR_SL_ReqAspCommonPart_Type |  | SS_UE_Id: identifier of the NR-SS-UE RoutingInfo : SL_SRB0, SL_SRB1, SL_SRB2, SL_SRB3 TimingInfo : Now in normal cases; TimingInfo can be set to specific activation time in which the Sidelink message shall be sent out (in this case and if the PDU is too long to be sent in one slot the TimingInfo corresponds to the first slot) ControlInfo CnfFlag:=false; FollowOnFlag=false; |
| Signalling | NR_SRB_Request_Type |  |  |

NR_SL_SRB_COMMON_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NR_SL_SRB_COMMON_IND |  |  |
| Comment | common ASP to receive PDUs from SL_SRB0, SL_SRB1, SL_SRB2, SL_SRB3 |  |  |
| Common | NR_SL_IndAspCommonPart_Type |  | SS_UE_Id: identifier of the NR-SS-UE RoutingInfo : SL_SRB0, SL_SRB1, SL_SRB2, SL_SRB3 TimingInfo : time when message has been received |
| Signalling | NR_SRB_Indication_Type |  |  |

NR_SL_SRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NR_SL_SRB_PORT |  |
| Comment | NR SL PTC: Port for Sending/Receiving data on SL-SRBs |  |
| out | NR_SL_SRB_COMMON_REQ |  |
| in | NR_SL_SRB_COMMON_IND |  |

# D.12	SidelinkUE_Common_TypeDefs

SS_UE_Id_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | SS_UE_Id_Type |
| Comment | Simulated UE-ID |
| ss_UE_NonSpecific | Represents all Simulated UEs |
| ss_UE_Id1 |  |
| ss_UE_Id2 |  |
| ss_UE_Id3 |  |
| ss_UE_Id4 |  |
| ss_UE_Id5 |  |

CongestionRepetition_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | CongestionRepetition_Type |  |
| Comment |  |  |
| Continuous | Null_Type | Send random data until Deactivation command is received |
| NumberOfFrames | integer | Number of consecutive frames the congestion is repeated |

# D.13	References to TTCN-3

| References to TTCN-3 |  |  |
| --- | --- | --- |
| NR_ASP_TypeDefs | NR_Defs/NR_ASP_TypeDefs.ttcn | Rev 36636 |
| NR_ASP_DrbDefs | NR_Defs/NR_ASP_DrbDefs.ttcn | Rev 36706 |
| NR_ASP_SrbDefs | NR_Defs/NR_ASP_SrbDefs.ttcn | Rev 32690 |
| NR_CommonDefs | NR_Defs/NR_CommonDefs.ttcn | Rev 36364 |
| IP_ASP_TypeDefs | IP_PTC/IP_ASP_TypeDefs.ttcn | Rev 32541 |
| NR_PDCP_TypeDefs | Common4G5G/NR_PDCP_TypeDefs.ttcn | Rev 36711 |
| SDAP_TypeDefs | Common4G5G/SDAP_TypeDefs.ttcn | Rev 29170 |
| NR_ASP_VirtualNoiseDefs | NR_Defs/NR_ASP_VirtualNoiseDefs.ttcn | Rev 30633 |
| CommonDefs | Common/CommonDefs.ttcn | Rev 36499 |
| CommonAspDefs | Common/CommonAspDefs.ttcn | Rev 34526 |
| NR_SideLink_ASP_TypeDefs | NR_Defs/NR_SideLink_ASP_TypeDefs.ttcn | Rev 36579 |
| SidelinkUE_Common_TypeDefs | Common4G5G/SidelinkUE_Common_TypeDefs.ttcn | Rev 35279 |