3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230679

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3191 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC eCall over IMS testcase 11.5.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 11.5.5 to the NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.5.5 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1, 38.508-1, 34.229-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

2.4	Change 4	6

3	Branches executed	7

4	Execution Log Files	7

4.1	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies	7

5	References	7

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.5.5 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.5.5

ATS Version:	iwd-TTCN3-B2022-09_D23wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.

Verification Status:	PASS

# Corrections required

In addition to the below changes, the changes of R5s230480_MCC160Comments.docx are also required and implemented.

## Change 1

| Function name | f_TC_11_5_5_NR5GC_IMS2() |
| --- | --- |
| Reason for change | - eCall setup will be without emergency IMS registration, so condition A6 should be used - Triggers to NR5GC PTC will not match with the expected. |
| Summary of change | - Pass new context parameter to function f_ECall_CallSetup_A_23() to indicate context for INVITE (see change 2) Note : Associated prose CRs on 38.523-1 and 34.229-5 will be raised at RAN5#101 - Add trigger text to match with the expectation in functions f_NR5GC_EmergencyCallLimitedService and f_NR5GC_EmergencyCallReleaseMT() |
| TTCN module |  |
| MCC160 Comment |  |

Before Change:

function f_TC_11_5_5_NR5GC_IMS2()runs on IMS_PTC { var SipUrl v_ContactUrlUE; // Emergency PDU session f_IMS_PTC_Init(PDN_2); f_ECall_CallSetup_A_23(MANUAL); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Step 5" siclog@ v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

After Change:

function f_TC_11_5_5_NR5GC_IMS2()runs on IMS_PTC { var SipUrl v_ContactUrlUE; // Emergency PDU session f_IMS_PTC_Init(PDN_2); f_ECall_CallSetup_A_23(MANUAL, A_2_1_A6); f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Emergency Call Established")); f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Step 5" siclog@ v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); }

## Change 2

| Function name | f_ECall_CallSetup_A_23() |
| --- | --- |
| Reason for change | For this test case, eCall setup will be without emergency IMS registration, so condition A6 should be used for checking the received INVITE |
| Summary of change | Add new parameter to indicate context for INVITE Note : Associated prose CRs on 38.523-1 and 34.229-5 will be raised at RAN5#101 |
| TTCN module | ECall_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_ECall_CallSetup_A_23(ECall_Type p_ECallType) runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_SDP_AudioCodecList_Type v_AudioCodecList := { EVS_16000 }; v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(p_ECallType); // @sic R5s230115, R5s230480 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // @sic R5s230480 sic@ f_ECall_CallSetup_Steps2_3 (v_InviteRequestWithSdp, v_AudioCodecList); // Split Invite into a separate function @sic R5-227567 sic@ f_IMS_IPCAN_WaitForTrigger(IPCAN, "Voice Bearer Added"); // @sic R5s230480 sic@ }

After Change:

function f_ECall_CallSetup_A_23(ECall_Type p_ECallType) runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_SDP_AudioCodecList_Type v_AudioCodecList := { EVS_16000 }; v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(p_ECallType); // @sic R5s230115, R5s230480 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // @sic R5s230480 sic@ f_ECall_CallSetup_Steps2_3 (v_InviteRequestWithSdp, v_AudioCodecList); // Split Invite into a separate function @sic R5-227567 sic@ f_IMS_IPCAN_WaitForTrigger(IPCAN, "Voice Bearer Added"); // @sic R5s230480 sic@ }

## Change 3

| Function name | f_TC_11_5_5_NR5GC() |
| --- | --- |
| Reason for change | Following the test body the UE will be in limited service state following the release of emergency call so the postamble state STATE_EMERGENCY_RELEASE_IDLE should be used. |
| Summary of change | Correct postamble state. |
| TTCN module | ECall_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_11_5_5_NR5GC() runs on NR5GC_PTC { //eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted / 5GS f_NR5GC_Init(NR_1); //the eCall capable UE is equipped with 'eCall only' enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-26 f_UT_USIM_Insert(UT, "USIM configuration 26 specified in table 6.4.1-26 TS 38.508-1"); f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_004_31); // @sic R5-227244 sic@ //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); // @sic R5-227244 sic@ f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0A); // @sic R5-227244 sic@ f_NR_TestBody_Set(true); fl_TC_11_5_5_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); // @sic R5-227244 sic@ }

After Change:

function f_TC_11_5_5_NR5GC() runs on NR5GC_PTC { //eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted / 5GS f_NR5GC_Init(NR_1); //the eCall capable UE is equipped with 'eCall only' enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-26 f_UT_USIM_Insert(UT, "USIM configuration 26 specified in table 6.4.1-26 TS 38.508-1"); f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_004_31); // @sic R5-227244 sic@ //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); // @sic R5-227244 sic@ f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0A); // @sic R5-227244 sic@ f_NR_TestBody_Set(true); fl_TC_11_5_5_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_EMERGENCY_RELEASE_IDLE); // @sic R5-227244 sic@ }

## Change 4

| Function name | f_GetTestcaseAttrib_NullIntegrityAlgorithmAllowed() |
| --- | --- |
| Reason for change | eCall in limited service will use NULL integrity algorithm |
| Summary of change | Allow NULL integrity algorithm for this test case |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment |  |

Before Change:

f_GetTestcaseAttrib_NullIntegrityAlgorithmAllowed(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_4_2_NR5GC") { return true; } case ("TC_11_4_3_NR5GC") { return true; } case ("TC_11_4_5_NR5GC") { return true; } case ("TC_11_4_9_NR5GC") { return true; } } return false; }

After Change:

f_GetTestcaseAttrib_NullIntegrityAlgorithmAllowed(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_4_2_NR5GC") { return true; } case ("TC_11_4_3_NR5GC") { return true; } case ("TC_11_4_5_NR5GC") { return true; } case ("TC_11_4_9_NR5GC") { return true; } case ("TC_11_5_5_NR5GC") { return true; } } return false; }

# Branches executed

The test case was executed on NR band n78 using ciphering algorithm nea1 and integrity algorithm nia1

# Execution Log Files

## Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies

The Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_5_5_LOG.html

- PICS/PIXIT parameter file:

TC_11_5_5_PIXIT.xml

# References

| [1] | R5s230680: Supporting information for addition of NR5GC eCall over IMS testcase 11.5.5 |
| --- | --- |