3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230687

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0934 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections In User Initiated USSI test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current implementation of USSD Data Template in TTCN declares the IE elems list as an empty string. Additionally, table Table 8.40.3.3-1 from TS 34.229-5 specifies that only ussd-string and language are required in the USSD string response from UE. Therefore, elems list can be empty or OMIT. But with current TTCN Template, elems list set to OMIT is not being handled accordingly. Therefore, template needs to be updated to handle ANY or OMIT values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update template (present) UssdData_Type cr_UssdData to support handling of OMIT value for elems list IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.40.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:                      Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | template (present) UssdData_Type cr_UssdData |
| --- | --- |
| Reason for change | Current implementation of USSD Data Template in TTCN declares the IE elems list as an empty string. Additionally, table Table 8.40.3.3-1 from TS 34.229-5 specifies that only ussd-string and language are required in the USSD string response from UE. Therefore, elems list can be empty or OMIT. But with current TTCN Template, elems list set to OMIT is not being handled accordingly. Therefore, template needs to be updated to handle ANY or OMIT values. |
| Summary of change | Update template (present) UssdData_Type cr_UssdData to support handling of OMIT value for elems list IE |
| TTCN module | IMS_Procedures_SupplementaryServices.ttcn |
| MCC160 Comment |  |

Before Change

type Ussd_data UssdData_Type with { encode "XML"}; /* @status APPROVED (IMS) */ template (value) MessageBody cs_MessageBody_USSD(template (value) UssdData_Type p_UssdData) := cs_MessageBody_XML_Bitstring(encvalue(p_UssdData)); /* @status APPROVED (IMS) */ template (present) UssdData_Type cr_UssdData(template (present) XSD.String p_Language := ?, template (present) XSD.String p_UssdString) := { /* @status APPROVED (IMS) */ attr := *, language_ := p_Language, ussd_string := p_UssdString, error_code := omit, anyExt := *, elem_list := {} };

After Change

type Ussd_data UssdData_Type with { encode "XML"}; /* @status APPROVED (IMS) */ template (value) MessageBody cs_MessageBody_USSD(template (value) UssdData_Type p_UssdData) := cs_MessageBody_XML_Bitstring(encvalue(p_UssdData)); /* @status APPROVED (IMS) */ template (present) UssdData_Type cr_UssdData(template (present) XSD.String p_Language := ?, template (present) XSD.String p_UssdString) := { /* @status APPROVED (IMS) */ attr := *, language_ := p_Language, ussd_string := p_UssdString, error_code := omit, anyExt := *, elem_list := * };