3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230707

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4762 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NB-IoT NTN test case 22.5.23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add verified NB-IOT NTN testcase 22.5.23 to NBIOT testsuite |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 22.5.23 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to NBIOT testsuite |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required for test case 22.5.23	4

2.1	Correction to f_TC_22_5_23_NBIOT	4

3	Branches executed in test case 22.5.23	5

4	Execution Log Files	5

4.1	Qualcomm 9205S	5

4.2	MediaTek 6825	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of NB-IoT NTN test case 22.5.23 which is part of the NBIOT test suite in iwd-TTCN3-B2022-09_D23wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

## Verification Test Summary

Test Case: 	22_5_23_NBIOT

ATS Version:	iwd-TTCN3-B2022-09_D23wk37

System Simulator used:	R&S® CMW 500 Protocol Conformance Test platform

UE used:	MediaTek MTK 6825, Qualcomm 9205S

Verification Status:	PASS

# Corrections required for test case 22.5.23

## Correction to f_TC_22_5_23_NBIOT

| Function name | f_TC_22_5_23_NBIOT |
| --- | --- |
| Reason for change | In this test cases cell 50 and cell 62 are on different plmn |
| Summary of change | Updated the init variable to pass the 3rd parameter LTE_CellsOnDifferentPLMN |
| TTCN module | NBIOT_CellInfo |
| MCC160 Comment |  |

Before Change:

function f_TC_22_5_23_NBIOT() runs on NBIOT_PTC { // NB-IoT / NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" var template (value) NB_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NB_CellPowerList_Type v_CellPowerList_AtT1; // var template (present) NAS_UL_Message_Type v_AttachReq; var template (present) NAS_UL_Message_Type v_ESMMessage; var template (present) B3_Type v_PDNType := (f_GetPdnType(), '101'B); var NAS_MSG_Indication_Type v_NasInd; // var SystemInformationBlockType1_NB v_SIB1; // var SystemInformationBlockType31_NB_r17 v_SIB31; v_CellPowerList_AtT0 := { cs_NB_CellPower(nbiot_Cell50, tsc_ServingCellRS_EPRE), cs_NB_CellPower(nbiot_Cell62, tsc_NonSuitableOffNBIOTCellRS_EPRE) }; v_CellPowerList_AtT1 := { cs_NB_CellPower(nbiot_Cell50, tsc_ServingCellRS_EPRE), cs_NB_CellPower(nbiot_Cell62, -73) }; //Initialise all cell, security and mobile parameters f_NBIOT_Init(c9); f_NBIOT_CellInfo_InitMaxReferencePower(nbiot_Cell62, -73);

After Change:

function f_TC_22_5_23_NBIOT() runs on NBIOT_PTC { // NB-IoT / NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" var template (value) NB_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NB_CellPowerList_Type v_CellPowerList_AtT1; // var template (present) NAS_UL_Message_Type v_AttachReq; var template (present) NAS_UL_Message_Type v_ESMMessage; var template (present) B3_Type v_PDNType := (f_GetPdnType(), '101'B); var NAS_MSG_Indication_Type v_NasInd; // var SystemInformationBlockType1_NB v_SIB1; // var SystemInformationBlockType31_NB_r17 v_SIB31; v_CellPowerList_AtT0 := { cs_NB_CellPower(nbiot_Cell50, tsc_ServingCellRS_EPRE), cs_NB_CellPower(nbiot_Cell62, tsc_NonSuitableOffNBIOTCellRS_EPRE) }; v_CellPowerList_AtT1 := { cs_NB_CellPower(nbiot_Cell50, tsc_ServingCellRS_EPRE), cs_NB_CellPower(nbiot_Cell62, -73) }; //Initialise all cell, security and mobile parameters f_NBIOT_Init(c9,-,LTE_CellsOnDifferentPLMN);// cells are on different plmn f_NBIOT_CellInfo_InitMaxReferencePower(nbiot_Cell62, -73);

# Branches executed in test case 22.5.23

The test case implementation was executed with Integrity and ciphering enabled using NTN bands 255 and 256.

# Execution Log Files

## Qualcomm 9205S

The Qualcomm 9205S device passed this test case on Rohde & Schwarz CMW500. The documentation below is enclosed as evidence of the successful test case run [1]:

Test Case Execution log file: 
tc_22_5_23_QC.log 
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

## MediaTek 6825

The MediaTek 6825 device passed this test case on Rohde & Schwarz CMW500. The documentation below is enclosed as evidence of the successful test case run [1]:

Test Case Execution log file: 
tc_22_5_23_MTK.log 
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

n the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s23700.zip This archive comprises Execution log files, PICS/PIXIT files |
| --- | --- |