3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230712

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3204 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_TC_8_1_4_3_2And5_TestBody for NR5GC DAPS testcases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Acc. TS 38.331, 5.3.5.8.3 ( if T304 of the MCG expires) the UE shall, for each SRB, re-establish the RLC entity for the source PCell but this has to be done when T304 of the MCG expires not just immediately after requesting SS to send the rrcReconfiguration message at STEP 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved the entire related code in t_T304Min timout block. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.3.2 and 8.1.4.3.5 NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	4

2.1	Function fl_TC_8_1_4_3_2And5_TestBody ()	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function fl_TC_8_1_4_3_2And5_TestBody ()

| Function name | function fl_TC_8_1_4_3_2And5_TestBody |
| --- | --- |
| Reason for change | 1. Acc. TS 38.331, 5.3.5.8.3 ( if T304 of the MCG expires) the UE shall, for each SRB, re-establish the RLC entity for the source PCell but this has to be done when T304 of the MCG expires not just immediately after requesting SS to send the rrcReconfiguration message at STEP 1. |
| Summary of change | 1. Moved the entire related code in t_T304Min timout block. |
| TTCN module | RRC_DAPS_Handover_NR5GC |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_4_3_2And5_TestBody(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) DL_DCCH_Message v_RRCReconfiguration; var SubFrameTiming_Type v_Timing; var NR_PdcpCountInfoList_Type v_PdcpCountInfoList1; //@sic R5s221242 sic@ var NR_PdcpCountInfoList_Type v_PdcpCountInfoList2; //@sic R5s221242 sic@ var NR_PdcpCountInfoList_Type v_PdcpCountInfoList; //@sic R5s221242 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; //@sic R5s230362 sic@ var NR_SecurityParams_Type v_RRC_SecurityParams; timer t_Wait_Reestab := 5.0; //@sic R5-234093 sic@ timer t_Wait_Failure := 5.0; //@sic R5-234093 sic@ timer t_T304Min; //@sic R5-234093 sic@ v_CellPowerList_AtT0 := {cs_NR_CellPower(p_SourceCellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // TC n/a for FR2 cs_NR_CellPower(p_TargetCellId, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 v_CellPowerList_AtT2 := {cs_NR_CellPower(p_TargetCellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 v_CellPowerList_AtT3 := {cs_NR_CellPower(p_SourceCellId, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 //Set power level to T0 f_NR_SetCellPowerList (v_CellPowerList_AtT0); // Configuration of Target Cell for no RACH response transmission) f_NR_SS_RachProcedureConfig(p_TargetCellId, cs_NR_RachProcedureConfig_NoResponse, cs_TimingInfo_Now); //@siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2. // DRB#n of the first PDU session is configured as DAPS bearer. v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); v_RACH_ConfigGeneric.preambleTransMax := n200; // Table 8.1.4.3.2.3.3-4 v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount), cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourceCellId, cs_TimingInfo_Now, v_RRCReconfiguration)); //@siclog "Step 2" siclog@ // Void @sic R5-234093 sic@ //Acc. 38.331, 5.3.5.8.3: the UE shall, for each SRB, re-establish the RLC entity for the source PCell. When we release an SRB in the SS, the PDCP count is lost, so we need to set it to the original value once the SRB is configured again. //@sic R5s221242 sic@ v_PdcpCountInfoList1 := f_NR_SS_PdcpCount_Get(p_SourceCellId,tsc_NR_RbId_SRB1); v_PdcpCountInfoList2 := f_NR_SS_PdcpCount_Get(p_SourceCellId,tsc_NR_RbId_SRB2); v_PdcpCountInfoList := v_PdcpCountInfoList1 & v_PdcpCountInfoList2; f_NR_SS_SRBs_DRBs_Release(p_SourceCellId, cs_TimingInfo_Now, {}, tsc_NoCnfReq); f_NR_SS_SRBs_DRBs_Config(p_SourceCellId, cs_TimingInfo_Now, omit, tsc_NoCnfReq); //Reapply security keys for SRBs in the source cell - @sic R5s230387 sic@ v_RRC_SecurityParams := f_NR_Security_Get(); v_RRC_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_SourceCellId, false, -, v_PdcpCountInfoList); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SourceCellId, v_RRC_SecurityParams.AS_Integrity, v_RRC_SecurityParams.AS_Ciphering); //@sic R5s221242, R5s230387 sic@ f_NR_SS_PdcpCount_Set(p_SourceCellId, v_PdcpCountInfoList, cs_TimingInfo_Now); //EXCEPTION: In parallel with step 3, parallel behaviour defined in table 8.1.4.3.2.3.2-3 is executed repeatedly until T304 expiry. //@siclog "Step 3-4" siclog@ // Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1 within 5s? // Check: Does the UE transmit a FailureInformation message with failureType-r16 set to daps-failure on NR Cell 1 within 5s of T304 expiry? t_Wait_Reestab.start; //@sic R5-234093 sic@ t_T304Min.start(f_NR_SetTimerToleranceMin(p_TargetCellId, rrcTimer, 1.0)); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND (p_SourceCellId, cr_38508_RRCReestablishmentRequest(?, ?))){ t_Wait_Reestab.stop; t_T304Min.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "RRCReestablishmentRequest received on NR Cell 1"); } [] t_T304Min.timeout { //No need to reconfigure RACH as it is not needed until step 9 of the main procedure t_Wait_Failure.start; alt { [] t_Wait_Reestab.timeout {} [] SRB.receive(car_NR_SRB0_RrcPdu_IND (p_SourceCellId, cr_38508_RRCReestablishmentRequest(?, ?))){ t_Wait_Reestab.stop; t_Wait_Failure.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "RRCReestablishmentRequest received on NR Cell 1"); } [] SRB.receive(car_NR_SRB1_RrcPdu_IND (p_SourceCellId, cr_38508_FailureInformation(-, cr_FailureInformation_v1610({daps_failure})))){ //@sic R5s221242 sic@ t_Wait_Reestab.stop; t_Wait_Failure.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@sic R5s221242 sic@ } [] t_Wait_Failure.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "FailureInformation not received on NR Cell 1 within 5s of T304 expiry"); } } } } //@siclog "Step 5" siclog@ f_NR5GC_DAPS_HO_CheckDataPath(p_SourceCellId, "Step 5"); //@siclog "Step 6" siclog@ // The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList (v_CellPowerList_AtT2); //@siclog "Step 7" siclog@ //v_Timing := f_NR_GetNextSendOccasion_DL(p_SourceCellId, 700); // 700 ms; @sic R5s230320 sic@: v_Timing is an out parameter set inside f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig v_NR_PdcpCountInfoList_DRB := f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(p_SourceCellId, p_TargetCellId, v_Timing); //@sic R5s230362 sic@ f_Delay(0.8); //@sic R5s230362, R5s230387 sic@ //@siclog "Step 8" siclog@ // The SS changes the power level setting according to the row "T3" and waits 300ms to ensure T310 expires in NR Cell 1. f_NR_SetCellPowerList (v_CellPowerList_AtT3); f_Delay(0.3); //@sic R5s221242 sic@ f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList_DRB, cs_TimingInfo_Now); //Set count for DAPS DRB; TS 38.523-3, 7.3.5.3.5, Step 10A @sic R5s230362 sic@ //@siclog "Steps 9-10" siclog@ // The SS transmits Random Access Response to respond to the latest preamble on NR Cell 2. // Check: Does UE transmit an RRCReconfigurationComplete message in NR Cell 2? f_NR5GC_DAPS_HO_RACHProcedure_RRCReconfigurationComplete(p_TargetCellId, "Step 10", -, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); //@siclog "Step 11" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB#n of the first PDU session on NR Cell 2? f_NR5GC_DAPS_HO_CheckDataPath(p_TargetCellId, "Step 11"); //@siclog "Steps 12-13" siclog@ // The SS transmits an RRCReconfiguration message with condition DAPS_HO_ReleaseSource in NR Cell 2. // Check: Does the UE transmits an RRCReconfigurationComplete message in NR Cell 2. f_NR5GC_DAPS_HO_SourceRelease(p_SourceCellId, p_TargetCellId, "Step 13", false); //@sic R5-230116, R5s230362 sic@ f_NR5GC_DAPS_HO_SourceRelease_SSConfig(p_SourceCellId); } }

After Change:

function fl_TC_8_1_4_3_2And5_TestBody(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) DL_DCCH_Message v_RRCReconfiguration; var SubFrameTiming_Type v_Timing; var NR_PdcpCountInfoList_Type v_PdcpCountInfoList1; //@sic R5s221242 sic@ var NR_PdcpCountInfoList_Type v_PdcpCountInfoList2; //@sic R5s221242 sic@ var NR_PdcpCountInfoList_Type v_PdcpCountInfoList; //@sic R5s221242 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; //@sic R5s230362 sic@ var NR_SecurityParams_Type v_RRC_SecurityParams; timer t_Wait_Reestab := 5.0; //@sic R5-234093 sic@ timer t_Wait_Failure := 5.0; //@sic R5-234093 sic@ timer t_T304Min; //@sic R5-234093 sic@ v_CellPowerList_AtT0 := {cs_NR_CellPower(p_SourceCellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // TC n/a for FR2 cs_NR_CellPower(p_TargetCellId, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 v_CellPowerList_AtT2 := {cs_NR_CellPower(p_TargetCellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 v_CellPowerList_AtT3 := {cs_NR_CellPower(p_SourceCellId, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // TC n/a for FR2 //Set power level to T0 f_NR_SetCellPowerList (v_CellPowerList_AtT0); // Configuration of Target Cell for no RACH response transmission) f_NR_SS_RachProcedureConfig(p_TargetCellId, cs_NR_RachProcedureConfig_NoResponse, cs_TimingInfo_Now); //@siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2. // DRB#n of the first PDU session is configured as DAPS bearer. v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); v_RACH_ConfigGeneric.preambleTransMax := n200; // Table 8.1.4.3.2.3.3-4 v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount), cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourceCellId, cs_TimingInfo_Now, v_RRCReconfiguration)); //@siclog "Step 2" siclog@ // Void @sic R5-234093 sic@ //EXCEPTION: In parallel with step 3, parallel behaviour defined in table 8.1.4.3.2.3.2-3 is executed repeatedly until T304 expiry. //@siclog "Step 3-4" siclog@ // Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1 within 5s? // Check: Does the UE transmit a FailureInformation message with failureType-r16 set to daps-failure on NR Cell 1 within 5s of T304 expiry? t_Wait_Reestab.start; //@sic R5-234093 sic@ t_T304Min.start(f_NR_SetTimerToleranceMin(p_TargetCellId, rrcTimer, 1.0)); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND (p_SourceCellId, cr_38508_RRCReestablishmentRequest(?, ?))){ t_Wait_Reestab.stop; t_T304Min.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "RRCReestablishmentRequest received on NR Cell 1"); } [] t_T304Min.timeout { //Acc. 38.331, 5.3.5.8.3: the UE shall, for each SRB, re-establish the RLC entity for the source PCell. When we release an SRB in the SS, the PDCP count is lost, so we need to set it to the original value once the SRB is configured again. //@sic R5s221242 sic@ v_PdcpCountInfoList1 := f_NR_SS_PdcpCount_Get(p_SourceCellId,tsc_NR_RbId_SRB1); v_PdcpCountInfoList2 := f_NR_SS_PdcpCount_Get(p_SourceCellId,tsc_NR_RbId_SRB2); v_PdcpCountInfoList := v_PdcpCountInfoList1 & v_PdcpCountInfoList2; f_NR_SS_SRBs_DRBs_Release(p_SourceCellId, cs_TimingInfo_Now, {}, tsc_NoCnfReq); f_NR_SS_SRBs_DRBs_Config(p_SourceCellId, cs_TimingInfo_Now, omit, tsc_NoCnfReq); //Reapply security keys for SRBs in the source cell - @sic R5s230387 sic@ v_RRC_SecurityParams := f_NR_Security_Get(); v_RRC_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_SourceCellId, false, -, v_PdcpCountInfoList); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SourceCellId, v_RRC_SecurityParams.AS_Integrity, v_RRC_SecurityParams.AS_Ciphering); //@sic R5s221242, R5s230387 sic@ f_NR_SS_PdcpCount_Set(p_SourceCellId, v_PdcpCountInfoList, cs_TimingInfo_Now); //No need to reconfigure RACH as it is not needed until step 9 of the main procedure t_Wait_Failure.start; alt { [] t_Wait_Reestab.timeout {} [] SRB.receive(car_NR_SRB0_RrcPdu_IND (p_SourceCellId, cr_38508_RRCReestablishmentRequest(?, ?))){ t_Wait_Reestab.stop; t_Wait_Failure.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "RRCReestablishmentRequest received on NR Cell 1"); } [] SRB.receive(car_NR_SRB1_RrcPdu_IND (p_SourceCellId, cr_38508_FailureInformation(-, cr_FailureInformation_v1610({daps_failure})))){ //@sic R5s221242 sic@ t_Wait_Reestab.stop; t_Wait_Failure.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@sic R5s221242 sic@ } [] t_Wait_Failure.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "FailureInformation not received on NR Cell 1 within 5s of T304 expiry"); } } } } //@siclog "Step 5" siclog@ f_NR5GC_DAPS_HO_CheckDataPath(p_SourceCellId, "Step 5"); //@siclog "Step 6" siclog@ // The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList (v_CellPowerList_AtT2); //@siclog "Step 7" siclog@ //v_Timing := f_NR_GetNextSendOccasion_DL(p_SourceCellId, 700); // 700 ms; @sic R5s230320 sic@: v_Timing is an out parameter set inside f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig v_NR_PdcpCountInfoList_DRB := f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig(p_SourceCellId, p_TargetCellId, v_Timing); //@sic R5s230362 sic@ f_Delay(0.8); //@sic R5s230362, R5s230387 sic@ //@siclog "Step 8" siclog@ // The SS changes the power level setting according to the row "T3" and waits 300ms to ensure T310 expires in NR Cell 1. f_NR_SetCellPowerList (v_CellPowerList_AtT3); f_Delay(0.3); //@sic R5s221242 sic@ f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList_DRB, cs_TimingInfo_Now); //Set count for DAPS DRB; TS 38.523-3, 7.3.5.3.5, Step 10A @sic R5s230362 sic@ //@siclog "Steps 9-10" siclog@ // The SS transmits Random Access Response to respond to the latest preamble on NR Cell 2. // Check: Does UE transmit an RRCReconfigurationComplete message in NR Cell 2? f_NR5GC_DAPS_HO_RACHProcedure_RRCReconfigurationComplete(p_TargetCellId, "Step 10", -, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId)))); //@siclog "Step 11" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB#n of the first PDU session on NR Cell 2? f_NR5GC_DAPS_HO_CheckDataPath(p_TargetCellId, "Step 11"); //@siclog "Steps 12-13" siclog@ // The SS transmits an RRCReconfiguration message with condition DAPS_HO_ReleaseSource in NR Cell 2. // Check: Does the UE transmits an RRCReconfigurationComplete message in NR Cell 2. f_NR5GC_DAPS_HO_SourceRelease(p_SourceCellId, p_TargetCellId, "Step 13", false); //@sic R5-230116, R5s230362 sic@ f_NR5GC_DAPS_HO_SourceRelease_SSConfig(p_SourceCellId); } }