3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230714

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3206 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the template cr_UE_NR_Capability_v1610_IEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the current TTCN implementation of the template cr_UE_NR_Capability_v1610_IEs, IE : son_Parameters_r16 is expected as a mandatory one - But UE can choose to set this IE as omit as per 38.331 son-Parameters-r16 SON-Parameters-r16 OPTIONAL - Hence it is proposed to check the contents of son_Parameters_r16 only when it is present in UE capability sent by UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It is proposed to check the contents of son_Parameters_r16 only when it is present in UE capability sent by UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC, 8.2.1.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to the template cr_UE_NR_Capability_v1610_IEs	3

# Corrections required

## Correction to the template cr_UE_NR_Capability_v1610_IEs

| Template name | cr_UE_NR_Capability_v1610_IEs |
| --- | --- |
| Reason for change | - In the current TTCN implementation of the template cr_UE_NR_Capability_v1610_IEs, IE : son_Parameters_r16 is expected as a mandatory one - But UE can choose to set this IE as omit as per 38.331 son-Parameters-r16 SON-Parameters-r16 OPTIONAL Hence it is proposed to check the contents of son_Parameters_r16 only when it is present in UE capability sent by UE |
| Summary of change | - It is proposed to check the contents of son_Parameters_r16 only when it is present in UE capability sent by UE |
| TTCN module | ttcn/develop/Common/NR/NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

template UE_NR_Capability_v1610 cr_UE_NR_Capability_v1610_IEs (template UE_NR_Capability_v1640 p_UE_NR_Capability_v1640_IEs := *) := // @sic R5s211246 R5-225384 R5s221155 R5-227571 R5-227572 R5-227573 sic@ { /* @status APPROVED (ENDC, NR5GC) */ << SKIPPED CODE >> son_Parameters_r16 := { rach_Report_r16 := f_Derive_rach_Report_r16() ifpresent, // pc_rachReport_r16 rlfReportCHO_r17 := *, rlfReportDAPS_r17 := *, success_HO_Report_r17 := *, twoStepRACH_Report_r17 := *, pscell_MHI_Report_r17 := *, onDemandSI_Report_r17 := f_Derive_onDemandSI_Report_r17() ifpresent // pc_onDemandSI_Report_r17 }, onDemandSIB_Connected_r16 := *, nonCriticalExtension := p_UE_NR_Capability_v1640_IEs } ifpresent;

After Change:

template UE_NR_Capability_v1610 cr_UE_NR_Capability_v1610_IEs (template UE_NR_Capability_v1640 p_UE_NR_Capability_v1640_IEs := *) := // @sic R5s211246 R5-225384 R5s221155 R5-227571 R5-227572 R5-227573 sic@ { /* @status APPROVED (ENDC, NR5GC) */ << SKIPPED CODE >> son_Parameters_r16 := { rach_Report_r16 := f_Derive_rach_Report_r16() ifpresent, // pc_rachReport_r16 rlfReportCHO_r17 := *, rlfReportDAPS_r17 := *, success_HO_Report_r17 := *, twoStepRACH_Report_r17 := *, pscell_MHI_Report_r17 := *, onDemandSI_Report_r17 := f_Derive_onDemandSI_Report_r17() ifpresent // pc_onDemandSI_Report_r17 }ifpresent, //WA#WI=1197525 onDemandSIB_Connected_r16 := *, nonCriticalExtension := p_UE_NR_Capability_v1640_IEs } ifpresent;