3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230733

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3217 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 38.213 sec 12: For unpaired spectrum operation, a DL BWP from the set of configured DL BWPs with index provided by BWP-Id is linked with an UL BWP from the set of configured UL BWPs with index provided by BWP-Id when the DL BWP index and the UL BWP index are same. For unpaired spectrum operation, a UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP. Hence, before Step 9, the local end configuration cannot be switched only for DL BWP The same reason as highlighted above holds true for the local end configuration before Step 11A where only the UL is switched. The same reason as highlighted above holds true for the lcoal end configuration after Step 22A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Both the DL and UL BWP configuration is switched before Step 9. - The configuration to switch only the UL before Step 11A is removed as having a configuration to switch both DL and UL would cause the PDCCH Order not to be transmitted to the UE at Step 12. - Both the DL and UL BWP configuration is switched after Step 22A before the start of the Postamble. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1.NR5GC, 7.1.1.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_8_1_NR_CommonTestBody |
| --- | --- |
| Reason for change | According to TS 38.213 sec 12: For unpaired spectrum operation, a DL BWP from the set of configured DL BWPs with index provided by BWP-Id is linked with an UL BWP from the set of configured UL BWPs with index provided by BWP-Id when the DL BWP index and the UL BWP index are same. For unpaired spectrum operation, a UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP. Hence, before Step 9, the local end configuration cannot be switched only for DL BWP The same reason as highlighted above holds true for the local end configuration before Step 11A where only the UL is switched. The same reason as highlighted above holds true for the lcoal end configuration after Step 22A |
| Summary of change | - Both the DL and UL BWP configuration is switched before Step 9. - The configuration to switch only the UL before Step 11A is removed as having a configuration to switch both DL and UL would cause the PDCCH Order not to be transmitted to the UE at Step 12. - Both the DL and UL BWP configuration is switched after Step 22A before the start of the Postamble. |
| TTCN module | MAC_BWP_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 sic@ <<SKIPPED CODE>> //@sic R5s230220 change 9.2 sic@ if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); } // @siclog "Step 9" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> //@sic R5-2134599 R5s230634 sic@ if (v_BWP_SameNumerology_upto4) { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3; } else { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1; } f_NR_SS_ConfigureBWPs_UL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); // Configure UL BWP to receive data at step 13D <<SKIPPED CODE>> // @siclog "Steps 22-22A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); } }

After Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 sic@ <<SKIPPED CODE>> //@sic R5s230220 change 9.2 sic@ if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -,tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); //WA# } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); //WA# } // @siclog "Step 9" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> //@sic R5-2134599 R5s230634 sic@ if (v_BWP_SameNumerology_upto4) { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3; } else { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1; } // REMOVED f_NR_SS_ConfigureBWPs_UL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); // Configure UL BWP to receive data at step 13D WA# <<SKIPPED CODE>> // @siclog "Steps 22-22A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); WA# } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); WA# } }