3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230735

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3218 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for altstep a_NR_RacingCond_AwaitRrcMessageAndTwoSysInd |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In altstep a_NR_RacingCond_AwaitRrcMessageAndTwoSysInd , the check for received SYSTEM_IND1 is using variable p_SysIndByRef2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct check for “RRC message and/or SYSTEM_IND1 has not been received yet” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 7.1.1.1.17.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | altstep a_NR_RacingCond_AwaitRrcMessageAndTwoSysInd |
| --- | --- |
| Reason for change | Check for received SYSTEM_IND1 is using variable p_SysIndByRef2 |
| Summary of change | Correct check for “RRC message and/or SYSTEM_IND1 has not been received yet” |
| TTCN module | NR_Timing.ttcn |
| MCC160 Comment |  |

Before Change:

altstep a_NR_RacingCond_AwaitRrcMessageAndTwoSysInd(NR_CellId_Type p_NR_CellId, RacingCond_ExpectedSequence_Type p_ExpectedSequence, inout template (omit) NR_SRB_COMMON_IND p_SrbIndByRef, inout template (omit) NR_SYSTEM_IND p_SysIndByRef1, inout template (omit) NR_SYSTEM_IND p_SysIndByRef2, template (present) NR_SRB_COMMON_IND p_ExpectedSrbInd := car_NR_SRB_IND_Any, template (present) NR_SYSTEM_IND p_ExpectedSysInd1 := car_NR_SYSTEM_IND_Any, template (present) NR_SYSTEM_IND p_ExpectedSysInd2 := car_NR_SYSTEM_IND_Any) runs on NR_BASE_PTC { /* note: since this is an altstep 'interleave' cannot be used */ /* note: since the altstep uses repeat to receive the second message the altstep is left and re-entered again * => local variables are uninitialised; the only way to store information of subsequent messages is by the inout parameters * (which for that reason have to be "inout"; "out" would not work */ var NR_SRB_COMMON_IND v_SRB_COMMON_IND; var NR_SYSTEM_IND v_SYSTEM_IND; var SubFrameTiming_Type v_Timing_SysInd1; var SubFrameTiming_Type v_Timing_SysInd2; var SubFrameTiming_Type v_Timing_SrbInd; [not isvalue(p_SrbIndByRef)] SRB.receive (p_ExpectedSrbInd) -> value v_SRB_COMMON_IND { p_SrbIndByRef := v_SRB_COMMON_IND; if (not (isvalue(p_SysIndByRef1) and isvalue(p_SysIndByRef2))) {// both SYSTEM_INDs not been received yet repeat; // => wait for SYSTEM_IND1 and/or 2 } else { v_Timing_SrbInd := valueof(p_SrbIndByRef.Common.TimingInfo.SubFrame); v_Timing_SysInd1 := valueof(p_SysIndByRef1.Common.TimingInfo.SubFrame); v_Timing_SysInd2 := valueof(p_SysIndByRef2.Common.TimingInfo.SubFrame); fl_NR_RacingCond_CheckSequence(p_NR_CellId, p_ExpectedSequence, v_Timing_SrbInd, v_Timing_SysInd1, v_Timing_SysInd2); } } [not isvalue(p_SysIndByRef1)] SYSIND.receive (p_ExpectedSysInd1) -> value v_SYSTEM_IND { p_SysIndByRef1 := v_SYSTEM_IND; if (not (isvalue(p_SrbIndByRef) and isvalue(p_SysIndByRef2))) { // RRC message and/or SYSTEM_IND2 has not been received yet repeat; // => wait for RRC PDU and/or SYSTEM_IND2 } else { v_Timing_SrbInd := valueof(p_SrbIndByRef.Common.TimingInfo.SubFrame); v_Timing_SysInd1 := valueof(p_SysIndByRef1.Common.TimingInfo.SubFrame); v_Timing_SysInd2 := valueof(p_SysIndByRef2.Common.TimingInfo.SubFrame); fl_NR_RacingCond_CheckSequence(p_NR_CellId, p_ExpectedSequence, v_Timing_SrbInd, v_Timing_SysInd1, v_Timing_SysInd2); } } [not isvalue(p_SysIndByRef2)] SYSIND.receive (p_ExpectedSysInd2) -> value v_SYSTEM_IND { p_SysIndByRef2 := v_SYSTEM_IND; if (not (isvalue(p_SrbIndByRef) and isvalue(p_SysIndByRef2))) { // RRC message and/or SYSTEM_IND1 has not been received yet repeat; // => wait for RRC PDU } else { v_Timing_SrbInd := valueof(p_SrbIndByRef.Common.TimingInfo.SubFrame); v_Timing_SysInd1 := valueof(p_SysIndByRef1.Common.TimingInfo.SubFrame); v_Timing_SysInd2 := valueof(p_SysIndByRef2.Common.TimingInfo.SubFrame); fl_NR_RacingCond_CheckSequence(p_NR_CellId, p_ExpectedSequence, v_Timing_SrbInd, v_Timing_SysInd1, v_Timing_SysInd2); } } }

After Change:

altstep a_NR_RacingCond_AwaitRrcMessageAndTwoSysInd(NR_CellId_Type p_NR_CellId, RacingCond_ExpectedSequence_Type p_ExpectedSequence, inout template (omit) NR_SRB_COMMON_IND p_SrbIndByRef, inout template (omit) NR_SYSTEM_IND p_SysIndByRef1, inout template (omit) NR_SYSTEM_IND p_SysIndByRef2, template (present) NR_SRB_COMMON_IND p_ExpectedSrbInd := car_NR_SRB_IND_Any, template (present) NR_SYSTEM_IND p_ExpectedSysInd1 := car_NR_SYSTEM_IND_Any, template (present) NR_SYSTEM_IND p_ExpectedSysInd2 := car_NR_SYSTEM_IND_Any) runs on NR_BASE_PTC { /* note: since this is an altstep 'interleave' cannot be used */ /* note: since the altstep uses repeat to receive the second message the altstep is left and re-entered again * => local variables are uninitialised; the only way to store information of subsequent messages is by the inout parameters * (which for that reason have to be "inout"; "out" would not work */ var NR_SRB_COMMON_IND v_SRB_COMMON_IND; var NR_SYSTEM_IND v_SYSTEM_IND; var SubFrameTiming_Type v_Timing_SysInd1; var SubFrameTiming_Type v_Timing_SysInd2; var SubFrameTiming_Type v_Timing_SrbInd; [not isvalue(p_SrbIndByRef)] SRB.receive (p_ExpectedSrbInd) -> value v_SRB_COMMON_IND { p_SrbIndByRef := v_SRB_COMMON_IND; if (not (isvalue(p_SysIndByRef1) and isvalue(p_SysIndByRef2))) {// both SYSTEM_INDs not been received yet repeat; // => wait for SYSTEM_IND1 and/or 2 } else { v_Timing_SrbInd := valueof(p_SrbIndByRef.Common.TimingInfo.SubFrame); v_Timing_SysInd1 := valueof(p_SysIndByRef1.Common.TimingInfo.SubFrame); v_Timing_SysInd2 := valueof(p_SysIndByRef2.Common.TimingInfo.SubFrame); fl_NR_RacingCond_CheckSequence(p_NR_CellId, p_ExpectedSequence, v_Timing_SrbInd, v_Timing_SysInd1, v_Timing_SysInd2); } } [not isvalue(p_SysIndByRef1)] SYSIND.receive (p_ExpectedSysInd1) -> value v_SYSTEM_IND { p_SysIndByRef1 := v_SYSTEM_IND; if (not (isvalue(p_SrbIndByRef) and isvalue(p_SysIndByRef2))) { // RRC message and/or SYSTEM_IND2 has not been received yet repeat; // => wait for RRC PDU and/or SYSTEM_IND2 } else { v_Timing_SrbInd := valueof(p_SrbIndByRef.Common.TimingInfo.SubFrame); v_Timing_SysInd1 := valueof(p_SysIndByRef1.Common.TimingInfo.SubFrame); v_Timing_SysInd2 := valueof(p_SysIndByRef2.Common.TimingInfo.SubFrame); fl_NR_RacingCond_CheckSequence(p_NR_CellId, p_ExpectedSequence, v_Timing_SrbInd, v_Timing_SysInd1, v_Timing_SysInd2); } } [not isvalue(p_SysIndByRef2)] SYSIND.receive (p_ExpectedSysInd2) -> value v_SYSTEM_IND { p_SysIndByRef2 := v_SYSTEM_IND; if (not (isvalue(p_SrbIndByRef) and isvalue(p_SysIndByRef1))) { // RRC message and/or SYSTEM_IND1 has not been received yet repeat; // => wait for RRC PDU } else { v_Timing_SrbInd := valueof(p_SrbIndByRef.Common.TimingInfo.SubFrame); v_Timing_SysInd1 := valueof(p_SysIndByRef1.Common.TimingInfo.SubFrame); v_Timing_SysInd2 := valueof(p_SysIndByRef2.Common.TimingInfo.SubFrame); fl_NR_RacingCond_CheckSequence(p_NR_CellId, p_ExpectedSequence, v_Timing_SrbInd, v_Timing_SysInd1, v_Timing_SysInd2); } } }