3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230758

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3231 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC UAC test case 11.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following triggering the UE to establish a PDU session at step 10B, the call to f_NR5GC_PDUSessionEstablishment() has the parameter p_ExpectedNumberOfNewPDUConnections set to the default value equal to pc_noOf_PDUsSameConnection. This can cause the test case to fail in this function if this PICS is set to greater than 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the value of p_ExpectedNumberOfNewPDNConnections such that the function f_NR5GC_PDUSessionEstablishment() will expect only 1 PDU session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_11_3_5_TestBody() |
| --- | --- |
| Reason for change | Following triggering the UE to establish a PDU session at step 10B, the call to f_NR5GC_PDUSessionEstablishment() has the parameter p_ExpectedNumberOfNewPDUConnections set to the default value equal to pc_noOf_PDUsSameConnection. This can cause the test case to fail in this function if this PICS is set to greater than 1. |
| Summary of change | Set the value of p_ExpectedNumberOfNewPDNConnections such that the function f_NR5GC_PDUSessionEstablishment() will expect only 1 PDU session. |
| TTCN module | UAC_NR5GC.ttcn. |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell12); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_GutiParameters_Type v_GutiParams_NRCell1 := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI_NR_Cell1 := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_NRCell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (nr_Cell12); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); /*Skipping the code */ //@siclog "Steps 10Fa1-10Fa8" siclog@ // @sic R5-223436 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0000'B)) { // @sic R5s230103 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); // @sic R5s221172 sic@ f_NR5GC_PDUSessionEstablishment (nr_Cell12, -, tsc_NR_RbId_SRB2, -, - , v_DNN, -, v_DNN); // @sic R5s230200 R5s230376 sic@ //@siclog "Steps 10H" siclog@ // @sic R5-223436 sic@ //The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4] /*Skipping the code */

After Change:

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell12); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_GutiParameters_Type v_GutiParams_NRCell1 := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI_NR_Cell1 := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_NRCell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (nr_Cell12); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); /*Skipping the code */ //@siclog "Steps 10Fa1-10Fa8" siclog@ // @sic R5-223436 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0000'B)) { // @sic R5s230103 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); // @sic R5s221172 sic@ f_NR5GC_PDUSessionEstablishment (nr_Cell12, 1, tsc_NR_RbId_SRB2, -, - , v_DNN, -, v_DNN); // @sic R5s230200 R5s230376 sic@ //@siclog "Steps 10H" siclog@ // @sic R5-223436 sic@ //The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4] /*Skipping the code */