3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230768

# Online, , 13th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3235 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-17 eCPSOR test case 6.3.2.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 6.3.2.6 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.3.2.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR…TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	12

3	Branches executed	15

4	Execution Log Files	15

4.1	Mediatek MT6815	15

5	References	15

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.3.2.6 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.3.2.6

ATS Version:	iwd-TTCN3-B2022-09_D23wk37

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6815

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_TC_6_3_2_6_TestBody () |
| --- | --- |
| Reason for change | - On SOR_Header template one argument was missing due to that Datatype was setting to 1 instead 0 and Additional parameters (AP) value was setting to 0 instead 1 while prose is expecting additional parameter ‘Criterion type’ so it should be set to 1. - Current TTCN is not implemented SOR-CMCI container correctly. As per 24.501 Figure: 9.11.3.51.2A it should be like below 8; 7; 6; 5; 4; 3; 2; 1. SOR transparent container IEI; octet 1. Length of SOR transparent container contents; octet 2 octet 3. SOR header; octet 4. SOR-MAC-IAUSF; octet 5 octet 20. CounterSOR; octet 21 octet 22. Length of PLMN ID and access technology list; octet 23*. PLMN ID and access technology list; octet 24* octet m*. 0 Spare; 0 Spare; 0 Spare; 0 Spare; 0 Spare; SSSI; SSCMI; SI; octet o. SOR-CMCI 12oct; octet (o+1)* octet p*. SOR-CMCI is as below Length of SOR-CMCI contents 2oct; octet o+1 octet o+2. SOR-CMCI rule 1 Length of SOR-CMCI rule contents 2oct; octet q+1 octet q+2. Tsor-cm timer value 1oct 21; octet q+3. Criterion type 1oct 01; octet q+4. Criterion value 6oct; octet (q+5)* octet r*...; octet (o+1)* octet p*. SOR-SNPN-SI; octet (p+1)* octet u. Figure 9.11.3.51.2A: SOR transparentcontainer information element for list type with value "1", SOR datatype with value "0", additional parameters with value "1" So updated variables and v_data variable according to figure Figure 9.11.3.51.2A. - At step 27 co-ordination message for "Emergency Call Established" missing which is leading to failure to IMS ecall on later steps30. - As per proposed prose CR : According to below requirements, the UE shall still be in 5GMM-REGISTERED state after step 30A, so the generic procedure 4.9.5 (MRU) rather than 4.5.2.2-2 (initial registration) shall be performed in afterwards steps. The UE which has an emergency PDU session, receives a request from the upper layers to establish an emergency PDU session or perform emergency services fallback, registers for emergency services, or is configured for high priority access in the selected PLMN or SNPN is not required to enter idle mode if the last running Tsor-cm timer for any PDU session or service stops or expires. In this case, the UE shall attempt to perform the PLMN or SNPN selection after the emergency PDU session or the high priority service is released and after the UE enters idle mode or 5GMM-CONNECTED mode with RRC inactive indication (see 3GPP TS 24.501 [64]). #draft proseCR would be raised for RAN5#101 meeting |
| Summary of change | - Corrected argument list for template cs_NG_SOR_Header - Added variables for Length of PLMN ID and access technology list, Length of SOR-CMCI contents, Length of SOR-CMCI rule contents and modified v_Data - Added co-ordination message for "Emergency Call Established". - Replaced generic procedure 4.5.2.2-2 by 4.9.5.2.2-1 using f_NR5GC_MobilityRegistration for cell11. |
| TTCN module | SOR_CMCI_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_6_3_2_6_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var octetstring v_NAS_TxMsgContainer_SOR; var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var TypeOfRegistration_Type v_TypeOfRegistration; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O2_Type v_Counter :='0001'O; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'1'B); //@sic R5-233461 sic@ var O1_Type v_Octet_O :='03'O; var octetstring v_Criterion_type:= 'FF'O; var octetstring v_Tsor_cm_timer:= '21'O; timer t_Tsor:= 60.0; timer t_Tmax; //@siclog "Steps 1" siclog@ //Void @sic R5-233461 sic@ //@siclog "Steps 2-21a1" siclog@ //Void @sic R5-233461 sic@ //@siclog "Step 22" siclog@ //The SS transmits an DLInformationTransfer message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception. v_Data := v_PlmnID & v_AccessTechId & v_Octet_O & v_Tsor_cm_timer & v_Criterion_type; //@sic R5-233461 sic@ v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_Counter, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_Counter, v_Data, omit, omit))); // @sic R5s200639 sic@ v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR)); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,v_PduToSend.Msg))); //@siclog "Step 23" siclog@ //The SS starts timer of Tsor-cm timer value t_Tsor.start; //@siclog "Step 24" siclog@ //The SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3) t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 480.0)); //@sic R5-233461 sic@ //@siclog "Step 25" siclog@ //The UE transmits an ULInformationTransfer message carrying acknowledgement of successful reception of the steering of roaming information SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerSORMsg), omit)) { } //@siclog "Step 26" siclog@ //Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911. (Note 4) f_UT_RequestIMSEmergencyCall (UT, "112"); //@siclog "Step 27" siclog@ //Generic procedure fo IMS Emergency call establishment in 5GC with IMS emergency registration as described in TS 38.508-1 [4] Table 4.9.11.2.2-1 is performed f_NR5GC_EmergencyCallPDUSession(nr_Cell12); //@sic R5-233461 sic@ //@siclog "Step 28" siclog@ //The SS waits for timer of Tsor-cm timer expiry. t_Tsor.timeout; //@siclog "Step 29" siclog@ //Make the UE release the emergency call. //@siclog "Step 30" siclog@ //The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. f_NR5GC_EmergencyCallRelease(nr_Cell12); //@siclog "Step 30A" siclog@ //The SS releases the RRC connection on NR cell 12. f_NR_RRCRelease(nr_Cell12); //@sic R5-233461 sic@ //@siclog "Step 31" siclog@ //Check: Does the UE transmit an RRCSetupRequest on NR Cell 11 before tmax expires?(Note 1, 2 and 3) alt { //@sic R5s201466 sic@ [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 31 PASS"); } [] t_Tmax.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 31 Failed"); } } //@siclog "Steps 32 - 33" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. (Note 4) f_NR_RRCSetup_Def (nr_Cell11); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell11); //@siclog "Steps 35-50a1" siclog@ // Steps 5 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 11. f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell11, v_ReceivedMsg, TESTMode_OFF,-, v_TypeOfRegistration); //@sic R5-233461 sic@ }//End of fl_TC_6_2_3_6_TestBody

After Change

function fl_TC_6_3_2_6_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var octetstring v_NAS_TxMsgContainer_SOR; var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var TypeOfRegistration_Type v_TypeOfRegistration; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O2_Type v_Counter :='0001'O; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B,'1'B); //@sic R5-233461 sic@ var O1_Type v_Octet_O :='03'O; var octetstring v_Criterion_type:= 'FF'O; var octetstring v_Tsor_cm_timer:= '21'O; var O1_Type v_Length_SOR_CMCI_rule_contents :=int2oct(2,2); var O1_Type v_Length_SOR_CMCI_contents :=int2oct(4,2); var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1); timer t_Tsor:= 60.0; timer t_Tmax; //@siclog "Steps 1" siclog@ //Void @sic R5-233461 sic@ //@siclog "Steps 2-21a1" siclog@ //Void @sic R5-233461 sic@ //@siclog "Step 22" siclog@ //The SS transmits an DLInformationTransfer message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception. //v_Data := v_PlmnID & v_AccessTechId & v_Octet_O & v_Tsor_cm_timer & v_Criterion_type; //@sic R5-233461 sic@ v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_Octet_O & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_Tsor_cm_timer & v_Criterion_type; v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_Counter, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_Counter, v_Data, omit, omit))); // @sic R5s200639 sic@ v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR)); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,v_PduToSend.Msg))); //@siclog "Step 23" siclog@ //The SS starts timer of Tsor-cm timer value t_Tsor.start; //@siclog "Step 24" siclog@ //The SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3) t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 480.0)); //@sic R5-233461 sic@ //@siclog "Step 25" siclog@ //The UE transmits an ULInformationTransfer message carrying acknowledgement of successful reception of the steering of roaming information SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerSORMsg), omit)) { } //@siclog "Step 26" siclog@ //Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911. (Note 4) f_UT_RequestIMSEmergencyCall (UT, "112"); //@siclog "Step 27" siclog@ //Generic procedure fo IMS Emergency call establishment in 5GC with IMS emergency registration as described in TS 38.508-1 [4] Table 4.9.11.2.2-1 is performed f_NR5GC_EmergencyCallPDUSession(nr_Cell12); //@sic R5-233461 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Emergency Call Established"); // @sic R5-225362 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "step 27 IMS eCall Established"); //@siclog "Step 28" siclog@ //The SS waits for timer of Tsor-cm timer expiry. t_Tsor.timeout; //@siclog "Step 29" siclog@ //Make the UE release the emergency call. //@siclog "Step 30" siclog@ //The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place. f_NR5GC_EmergencyCallRelease(nr_Cell12); //@siclog "Step 30A" siclog@ //The SS releases the RRC connection on NR cell 12. f_NR_RRCRelease(nr_Cell12); //@sic R5-233461 sic@ //@siclog "Step 31" siclog@ //Check: Does the UE transmit an RRCSetupRequest on NR Cell 11 before tmax expires?(Note 1, 2 and 3) alt { //@sic R5s201466 sic@ [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 31 PASS"); } [] t_Tmax.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 31 Failed"); } } //@siclog "Steps 32 - 33" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. (Note 4) f_NR5GC_MobilityRegistration(nr_Cell11); }//End of fl_TC_6_2_3_6_TestBody

## Change 2

| Function name | f_TC_6_3_2_6_NR5GC() |
| --- | --- |
| Reason for change | Cell12 was not powered up for the testbody. So cell should be powered up after preamble step. |
| Summary of change | Added f_NR_SetCellPower for both cells after preamble. |
| TTCN module | SOR_CMCI_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_6_3_2_6_NR5GC() runs on NR5GC_PTC { //Steering of UE in roaming after registration / SOR-CMCI rule / match all / Emergency call / DL NAS transport f_NR5GC_Init(NR_4); //Set Sys info and cell info for cell 11& 12 to be used f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell12, tsc_NR_HPLMN_002_81);//PLMN13 //Create and configure cell f_NR_CellConfig_Def(nr_Cell11); f_NR_CellConfig_Def(nr_Cell12); // Equip UE with the USIM configuration 21 specified in table 6.4.1-21 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 21 specified in table 6.4.1-21 TS 38.508-1"); f_UT_SwitchOnUE(UT, false, CNF_REQUIRED); f_UT_AutomaticPLMN_Select (UT); f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); f_NR5GC_Preamble (nr_Cell12, STATE_CONNECTED_3A); //@sic R5-233461 sic@ f_NR_TestBody_Set(true); fl_TC_6_3_2_6_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); }

After Change

function f_TC_6_3_2_6_NR5GC() runs on NR5GC_PTC { //Steering of UE in roaming after registration / SOR-CMCI rule / match all / Emergency call / DL NAS transport f_NR5GC_Init(NR_4); //Set Sys info and cell info for cell 11& 12 to be used f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell12, tsc_NR_HPLMN_002_81);//PLMN13 //Create and configure cell f_NR_CellConfig_Def(nr_Cell11); f_NR_CellConfig_Def(nr_Cell12); // Equip UE with the USIM configuration 21 specified in table 6.4.1-21 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 21 specified in table 6.4.1-21 TS 38.508-1"); f_UT_SwitchOnUE(UT, false, CNF_REQUIRED); f_UT_AutomaticPLMN_Select (UT); f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); f_NR5GC_Preamble (nr_Cell12, STATE_CONNECTED_3A); //@sic R5-233461 sic@ f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_TestBody_Set(true); fl_TC_6_3_2_6_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); }

# Branches executed

The test case was executed on NR band n28 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Mediatek MT6815

The Mediatek MT6815 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_3_2_6_LOG.html

- PICS/PIXIT parameter file:

TC_6_3_2_6_PIXIT.xml

# References

| [1] | R5s230769: NR5GC FR1: Supporting information for addition of Rel-17 eCPSOR test case 6.3.2.6 |
| --- | --- |