3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230780

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3242 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC idle mode test case 6.3.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the optional step 17Aa1 which takes place if 5GS registration type is set as Initial Registration in step 6, TTCN handling is incorrect. If the UE sends a 2nd PDU SESSION ESTABLISHMENT REQUEST after sending the PDU SESSION ESTABLISHMENT REQUEST with type “emergency”, this will be stored inside the default handler a_NR5GC_AdditionalPDUSessionEstablishmentReq but will never be processed due to p_ExpectedNumberOfNewPDUSessions being passed as 1 (from f_NR5GC_EmergencyCallPDUSession() to f_NR5GC_PDUSessionEstablishment()). Since the SS will never send a PDU SESSION ESTABLISHMENT ACCEPT for this request, the UE may re-send the PDU SESSION ESTABLISHMENT REQUEST message once the emergency call is released, causing test case failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes are proposed to handle both PDU Session Estalishment Requests for Emergency and IMS session in parallel. Note : The solution is based on the MCC TF160 comments for R5s230685, and the function f_NR5GC_MsgInDefault_RemoveOneNASMessageReq() is defined in R5s230685_MCC160Comments_r1 . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_6_3_1_7_TestBody() |
| --- | --- |
| Reason for change | For the optional step 17Aa1 which takes place if 5GS registration type is set as Initial Registration in step 6, TTCN handling is incorrect. If the UE sends a 2nd PDU SESSION ESTABLISHMENT REQUEST after sending the PDU SESSION ESTABLISHMENT REQUEST with type “emergency”, this will be stored inside the default handler a_NR5GC_AdditionalPDUSessionEstablishmentReq but will never be processed due to p_ExpectedNumberOfNewPDUSessions being passed as 1 (from f_NR5GC_EmergencyCallPDUSession() to f_NR5GC_PDUSessionEstablishment()). Since the SS will never send a PDU SESSION ESTABLISHMENT ACCEPT for this request, the UE may re-send the PDU SESSION ESTABLISHMENT REQUEST message once the emergency call is released, causing test case failure. |
| Summary of change | Changes are proposed to handle both PDU Session Estalishment Requests for Emergency and IMS session in parallel. Note : The solution is based on the MCC TF160 comments for R5s230685, and the function f_NR5GC_MsgInDefault_RemoveOneNASMessageReq() is defined in R5s230685_MCC160Comments_r1 . |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC { var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11); //@sic R5-204410 sic@ var O2_Type v_AccessTechId :='0800'O; var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); var NR_SRB_COMMON_IND v_ReceivedAsp; var default v_MyDefaultVar := null; //@sic R5-231404 sic@ //@siclog "Step 1" siclog@ //Power on the UE //@siclog "Step 1A1-1A13" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@sic R5-204410 sic@ f_NR5GC_Preamble_Steps1_13(nr_Cell11); //@siclog "Step 1B" siclog@ //SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. //@sic R5-204410 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B))))); //@siclog "Step 1C" siclog@ //The SS releases the RRC connection //@sic R5-204410 sic@ f_NR_RRCRelease(nr_Cell11); //@siclog "Step 2" siclog@ //The UE is made to establish an emergency call. This can be done by an AT/MMI command f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 3" siclog@ // The UE transmits an RRCSetupRequest message on NR Cell 11 with establishmentCause set to 'emergency'. SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_emergency)))); //@sic R5s201466 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "RRCSetupRequest received"); //@sic R5s200305 sic@ //@siclog "Steps 4-5" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@siclog "Step 6" siclog@ //The UE transmit a REGISTRATION REQUEST message with IE 5GS registration type set to "Emergency registration" or "Initial registration"//@sic R5-210807 R5s201466 sic@ //@siclog "Steps 7-14" siclog@ //Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing //steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception f_NR5GC_SorRegistration(nr_Cell11,v_Data,v_Header,true,Initial_Secure, tsc_SHT_IntegrityProtected);//@sic R5s210236 R5s201455 R5-205361 sic@//using Initial_Secure to indicate the security context state. //@siclog "Step 16" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container f_NR5GC_Registration_Complete(nr_Cell11); //@siclog "Step 17" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 13 //@siclog "Step 17Aa1" siclog@Exception // The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2, //takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. if (pc_noOf_PDUsSameConnection > 0 ){//@sic R5-227239 sic@ SRB.check(receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp); if (f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType('0001'B), omit,omit,cr_NG_Request_Type('001'B))) { f_NR5GC_PDUSessionEstablishment (nr_Cell11,-,tsc_NR_RbId_SRB1); } } //siclog "Step 18-22" siclog@//@sic R5-216165 sic@ //Steps 9 to 13 of the procedure of "IMS Emergency call establishment in 5GC with IMS emergency registration" described in TS 38.508-1 [4] Table 4.9.11.2.2-1 are performed. f_NR5GC_EmergencyCallPDUSession(nr_Cell11,tsc_NR_RbId_SRB1);//@sic R5s211479 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Emergency Call Established"); // Emergency call now active @sic R%s220982, R5-225362 sic@ //@siclog "Step 23" siclog@//@sic R5-216165 sic@ //Generic procedure of "Test Procedure for IMS MO Emergency call release" described in TS 38.508-1 [4] Table 4.9.12A.2.2-1 are performed. f_NR5GC_EmergencyCallRelease(nr_Cell11);//@sic R5s211479 sic@ // The above function won't finish until IMS messages have been sent and PDU session has been released @sic R5s220982 sic@ //@siclog "Step 24" siclog@//@sic R5-216165 sic@ //The SS releases the RRC connection. v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell11)); //@sic R5-231404 sic@ f_NR_RRCRelease(nr_Cell11); deactivate(v_MyDefaultVar); //@sic R5-231404 sic@ }

After Change:

*/ function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC { var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11); //@sic R5-204410 sic@ var O2_Type v_AccessTechId :='0800'O; var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); var NR_SRB_COMMON_IND v_ReceivedAsp; var default v_MyDefaultVar := null; //@sic R5-231404 sic@ //@siclog "Step 1" siclog@ //Power on the UE var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var boolean v_InitialRxd := false; var boolean v_EmgRxd := false; //@siclog "Step 1A1-1A13" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@sic R5-204410 sic@ f_NR5GC_Preamble_Steps1_13(nr_Cell11); //@siclog "Step 1B" siclog@ //SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. //@sic R5-204410 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B))))); //@siclog "Step 1C" siclog@ //The SS releases the RRC connection //@sic R5-204410 sic@ f_NR_RRCRelease(nr_Cell11); //@siclog "Step 2" siclog@ //The UE is made to establish an emergency call. This can be done by an AT/MMI command f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 3" siclog@ // The UE transmits an RRCSetupRequest message on NR Cell 11 with establishmentCause set to 'emergency'. SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_emergency)))); //@sic R5s201466 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "RRCSetupRequest received"); //@sic R5s200305 sic@ //@siclog "Steps 4-5" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@siclog "Step 6" siclog@ //The UE transmit a REGISTRATION REQUEST message with IE 5GS registration type set to "Emergency registration" or "Initial registration"//@sic R5-210807 R5s201466 sic@ //@siclog "Steps 7-14" siclog@ //Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing //steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception f_NR5GC_SorRegistration(nr_Cell11,v_Data,v_Header,true,Initial_Secure, tsc_SHT_IntegrityProtected);//@sic R5s210236 R5s201455 R5-205361 sic@//using Initial_Secure to indicate the security context state. //@siclog "Step 16" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container f_NR5GC_Registration_Complete(nr_Cell11); //@siclog "Step 17" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 13 //@siclog "Step 17Aa1" siclog@Exception // The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2, //takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. if(not pc_noOf_PDUsSameConnection > 0) { v_InitialRxd := true; } // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@ // Then activate store for any new msgs received - this works for Modification Reqs too v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(nr_Cell11)); while (not v_InitialRxd or not v_EmgRxd) { if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > 0) { v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(1); } else { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; f_NR5GC_MsgInDefault_StoreNASMessageReq (v_ReceivedAsp); } if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType, cr_NG_Request_Type('001'B))) { // initial v_InitialRxd := true; f_NR5GC_PDUSessionEstablishment (nr_Cell11, 1, v_SRB, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, false, 1); } else if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType, cr_NG_Request_Type('011'B))) { // emergency v_EmgRxd := true; //@siclog "Step 18-22" siclog@//@sic R5-216165 sic@ //Steps 9 to 13 of the procedure of "IMS Emergency call establishment in 5GC with IMS emergency registration" described in TS 38.508-1 [4] Table 4.9.11.2.2-1 are performed. f_NR5GC_EmergencyCallPDUSession(nr_Cell11, v_SRB, 1);//@sic R5s211479 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Emergency Call Established"); // Emergency call now active @sic R%s220982, R5-225362 sic@ } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Wrong Request Type"); } v_SRB := tsc_NR_RbId_SRB2; // Can remove the message that's been processed so that there's only 1 when calling f_NR5GC_PDUSessionEstablishment f_NR5GC_MsgInDefault_RemoveOneNASMessageReq(); } //@siclog "Step 23" siclog@//@sic R5-216165 sic@ //Generic procedure of "Test Procedure for IMS MO Emergency call release" described in TS 38.508-1 [4] Table 4.9.12A.2.2-1 are performed. f_NR5GC_EmergencyCallRelease(nr_Cell11);//@sic R5s211479 sic@ // The above function won't finish until IMS messages have been sent and PDU session has been released @sic R5s220982 sic@ deactivate(v_MyDefaultVar); //@sic R5-231404 sic@ //@siclog "Step 24" siclog@//@sic R5-216165 sic@ //The SS releases the RRC connection. v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell11)); //@sic R5-231404 sic@ f_NR_RRCRelease(nr_Cell11); deactivate(v_MyDefaultVar); //@sic R5-231404 sic@ }