3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230786

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3246 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.3.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation after Step 16, the Step 17a1 is called based on setting of pc_noOf_PDUs_SameConnection, and whether the UE has in Step 6 set the Registration Type as Initial Registration. Such a device shall trigger two PDU Session Establishment REQs after Step 16. In case the device is set as a MultiPDU device, it would trigger 3 PDU Session Establishment REQs after Step 16. This causes an issue. If the UE triggers a UL NAS Transport with request type set to “initial emergency request” (‘011’B) then this condition is not met and the test flow proceeds to emergency call establishment. The other PDU Session Establishment REQs (for IMS and Internet) remain unhandled. This causes TC failure. At present, the current TTCN will work only for a device that is IMS enabled, and which sets the the request type in Registration REQ at Step 6 to “initial emergency req”. When run in manual mode, Steps 18- 23s can easily take around 15s to complete, followed by rrc release. This causes problem for a device that sets Registration type to “initial registration” and which is configrured for IMS and/or Internet. For such a device, when the PDU Session Establishment for IMS and/or Internet is happening, the rrc connection gets released (Step 24) in middle. Thus, such a device will immediately initiate a new rrc connection to try and establsih the PDU Session(s) for IMS and/or Internet, which is not handled or acommodated. This needs to be addressed. Lastly, if the UE did not perform IMS registration (Step 17Aa1), then at end of the TC it will be in state STATE_EMERGENCY_RELEASE_IDLE. But if this UE performed IMS registration , then at end of the TC the state will be STATE_IDLE_1A. At present the TTCN only considers the possibility of STATE_EMERGENCY_RELEASE_IDLE. This also needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The whole TTCN implementation after Step 16 is re-worked. It is now checked if the Registration REQ in Step 6 is “initial registration”, and this in combination with pc_noOf_PDUsSameConnection is used to receive the PDU Session Establishment REQ(s). It is proposed that rather than progressing the non-emergency PDU Sessions in parallel with Emergency PDU Sessions (Step 17Aa1), these are handled after the emergency call is released on a new rrc connection (After Step 24). This will improve the testcase stability. Hence the Step 17Aa1 is voided, and new TTCN is implemented for newly introduced Steps 25a1-25a8. A Prose CR will be raised for this. Based on the test flow, the function fl_TC_6_3_1_7_TestBody returns a Boolean that is set to TRUE if the UE performed IMS registration. Based on this within the function f_TC_6_3_1_7_NR5GC postamble functions are called. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	12

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_6_3_1_7_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation after Step 16, the Step 17a1 is called based on setting of pc_noOf_PDUs_SameConnection, and whether the UE has in Step 6 set the Registration Type as Initial Registration. Such a device shall trigger two PDU Session Establishment REQs after Step 16. In case the device is set as a MultiPDU device, it would trigger 3 PDU Session Establishment REQs after Step 16. This causes an issue. If the UE triggers a UL NAS Transport with request type set to “initial emergency request” (‘011’B) then this condition is not met and the test flow proceeds to emergency call establishment. The other PDU Session Establishment REQs (for IMS and Internet) remain unhandled. This causes TC failure. At present, the current TTCN will work only for a device that is IMS enabled, and which sets the the request type in Registration REQ at Step 6 to “initial emergency req”. When run in manual mode, Steps 18- 23s can easily take around 15s to complete, followed by rrc release. This causes problem for a device that sets Registration type to “initial registration” and which is configrured for IMS and/or Internet. For such a device, when the PDU Session Establishment for IMS and/or Internet is happening, the rrc connection gets released (Step 24) in middle. Thus, such a device will immediately initiate a new rrc connection to try and establsih the PDU Session(s) for IMS and/or Internet, which is not handled or acommodated. This needs to be addressed. |
| Summary of change | The whole TTCN implementation after Step 16 is re-worked. It is now checked if the Registration REQ in Step 6 is “initial registration”, and this in combination with pc_noOf_PDUsSameConnection is used to receive the PDU Session Establishment REQ(s). It is proposed that rather than progressing the non-emergency PDU Sessions in parallel with Emergency PDU Sessions (Step 17Aa1), these are handled after the emergency call is released on a new rrc connection (After Step 24). This will improve the testcase stability. Hence the Step 17Aa1 is voided, and new TTCN is implemented for newly introduced Steps 25a1-25a8. A Prose CR will be raised for this. |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC { var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11); //@sic R5-204410 sic@ var O2_Type v_AccessTechId :='0800'O; var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); var NR_SRB_COMMON_IND v_ReceivedAsp; var default v_MyDefaultVar := null; //@sic R5-231404 sic@ //@siclog "Step 1" siclog@ //Power on the UE //@siclog "Step 1A1-1A13" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@sic R5-204410 sic@ f_NR5GC_Preamble_Steps1_13(nr_Cell11); //@siclog "Step 1B" siclog@ //SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. //@sic R5-204410 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B))))); //@siclog "Step 1C" siclog@ //The SS releases the RRC connection //@sic R5-204410 sic@ f_NR_RRCRelease(nr_Cell11); //@siclog "Step 2" siclog@ //The UE is made to establish an emergency call. This can be done by an AT/MMI command f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 3" siclog@ // The UE transmits an RRCSetupRequest message on NR Cell 11 with establishmentCause set to 'emergency'. SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_emergency)))); //@sic R5s201466 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "RRCSetupRequest received"); //@sic R5s200305 sic@ //@siclog "Steps 4-5" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@siclog "Step 6" siclog@ //The UE transmit a REGISTRATION REQUEST message with IE 5GS registration type set to "Emergency registration" or "Initial registration"//@sic R5-210807 R5s201466 sic@ //@siclog "Steps 7-14" siclog@ //Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing //steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception f_NR5GC_SorRegistration(nr_Cell11,v_Data,v_Header,true,Initial_Secure, tsc_SHT_IntegrityProtected);//@sic R5s210236 R5s201455 R5-205361 sic@//using Initial_Secure to indicate the security context state. //@siclog "Step 16" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container f_NR5GC_Registration_Complete(nr_Cell11); //@siclog "Step 17" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 13 //@siclog "Step 17Aa1" siclog@Exception // The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2, //takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. if (pc_noOf_PDUsSameConnection > 0 ){//@sic R5-227239 sic@ SRB.check(receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp); if (f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType('0001'B), omit,omit,cr_NG_Request_Type('001'B))) { f_NR5GC_PDUSessionEstablishment (nr_Cell11,-,tsc_NR_RbId_SRB1); } } //@siclog "Step 18-22" siclog@//@sic R5-216165 sic@ //Steps 9 to 13 of the procedure of "IMS Emergency call establishment in 5GC with IMS emergency registration" described in TS 38.508-1 [4] Table 4.9.11.2.2-1 are performed. f_NR5GC_EmergencyCallPDUSession(nr_Cell11,tsc_NR_RbId_SRB1);//@sic R5s211479 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Emergency Call Established"); // Emergency call now active @sic R%s220982, R5-225362 sic@ //@siclog "Step 23" siclog@//@sic R5-216165 sic@ //Generic procedure of "Test Procedure for IMS MO Emergency call release" described in TS 38.508-1 [4] Table 4.9.12A.2.2-1 are performed. f_NR5GC_EmergencyCallRelease(nr_Cell11);//@sic R5s211479 sic@ // The above function won't finish until IMS messages have been sent and PDU session has been released @sic R5s220982 sic@ //@siclog "Step 24" siclog@//@sic R5-216165 sic@ //The SS releases the RRC connection. v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell11)); //@sic R5-231404 sic@ f_NR_RRCRelease(nr_Cell11); deactivate(v_MyDefaultVar); //@sic R5-231404 sic@ }//End of fl_TC_6_3_1_7_TestBody

After Change:

import from Parameters all; //WA#6_3_1_7 import from NAS_AuxiliaryDefsAndFunctions all; //WA#6_3_1_7 . . . . function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC return boolean //WA#6_3_1_7 { var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11); //@sic R5-204410 sic@ var O2_Type v_AccessTechId :='0800'O; var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); var NR_SRB_COMMON_IND v_ReceivedAsp; var default v_MyDefaultVar := null; //@sic R5-231404 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp_IMS; //WA#6_3_1_7 var NR_SRB_COMMON_IND v_ReceivedAsp_Internet; //WA#6_3_1_7 var boolean v_Received_IMS := false; //WA#6_3_1_7 var boolean v_Received_Internet := false; //WA#6_3_1_7 var boolean v_Received_Emergency := false; //WA#6_3_1_7 var NR_SRB_COMMON_IND v_ReceivedAsp_Emergency; //WA#6_3_1_7 var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //WA#6_3_1_7 var NG_NAS_MSG_Indication_Type v_ReceivedMsg; //WA#6_3_1_7 var TypeOfRegistration_Type v_TypeOfRegistration; //WA#6_3_1_7 var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; //WA#6_3_1_7 //@siclog "Step 1" siclog@ //Power on the UE log("Parikshit : 6317 Bytecode #1 based on 23wk37"); //@siclog "Step 1A1-1A13" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@sic R5-204410 sic@ f_NR5GC_Preamble_Steps1_13(nr_Cell11); //@siclog "Step 1B" siclog@ //SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. //@sic R5-204410 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B))))); //@siclog "Step 1C" siclog@ //The SS releases the RRC connection //@sic R5-204410 sic@ f_NR_RRCRelease(nr_Cell11); //@siclog "Step 2" siclog@ //The UE is made to establish an emergency call. This can be done by an AT/MMI command f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 3" siclog@ // The UE transmits an RRCSetupRequest message on NR Cell 11 with establishmentCause set to 'emergency'. SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_emergency)))); //@sic R5s201466 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "RRCSetupRequest received"); //@sic R5s200305 sic@ //@siclog "Steps 4-5" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@siclog "Step 6" siclog@ //The UE transmit a REGISTRATION REQUEST message with IE 5GS registration type set to "Emergency registration" or "Initial registration"//@sic R5-210807 R5s201466 sic@ //@siclog "Steps 7-14" siclog@ //Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing //steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception //WA#6_3_1_7 COMMENTED OUT f_NR5GC_SorRegistration(nr_Cell11,v_Data,v_Header,true,Initial_Secure, tsc_SHT_IntegrityProtected);//@sic R5s210236 R5s201455 R5-205361 sic@//using Initial_Secure to indicate the security context state. //WA#6_3_1_7 : Change start v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell11, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);//@sic R5s201455 sic@ select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) { case (tsc_NG_RegistrationEmergency) { v_TypeOfRegistration := Emergency; } case (tsc_NG_RegistrationInitial) { v_TypeOfRegistration := Initial_Secure; } case else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect"); } } if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_TypeOfRegistration)) {//@sic R5s210236 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //Call f_NR5GC_SorRegistration_Step5_13 f_NR5GC_SorRegistration_Step5_13 (nr_Cell11, v_GMM_MobilityInfo, v_ReceivedMsg, v_Data, v_Header, v_TypeOfRegistration, true); //WA#6_3_1_7 : Change Stop //@siclog "Step 16" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container f_NR5GC_Registration_Complete(nr_Cell11); //@siclog "Step 17" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 13 //@siclog "Step 17Aa1" siclog@Exception // The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2, //takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. /*WA#6_3_1_7 COMMENTED OUT if (pc_noOf_PDUsSameConnection > 0 ){//@sic R5-227239 sic@ SRB.check(receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp); if (f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType('0001'B), omit,omit,cr_NG_Request_Type('001'B))) { f_NR5GC_PDUSessionEstablishment (nr_Cell11,-,tsc_NR_RbId_SRB1); } } */ //WA#6_3_1_7 : Change start if (v_TypeOfRegistration == Initial_Secure) { //Expect a UL NAS Transport/PDU Session Establishment REQ for IMS and Emergency call if Registration REQ is for Initial registration if (pc_noOf_PDUsSameConnection == 1) { //UE is only IMS enabled while ((v_Received_IMS == false) or (v_Received_Emergency == false)) { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue == '001'B) { v_Received_IMS := true; //Received UL NAS Transport/PDU Session Establishment REQ for IMS v_ReceivedAsp_IMS := v_ReceivedAsp; }; if (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue == '011'B) { v_Received_Emergency := true; //Received UL NAS Transport/PDU Session Establishment REQ for EmergencyIMS v_ReceivedAsp_Emergency := v_ReceivedAsp; }; } } else { //UE is IMS enabled and also Internet enabled while ((v_Received_IMS == false) or (v_Received_Emergency == false) or (v_Received_Internet == false)) { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (ispresent(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.dnn.dnnValue)) { if (match (valueof(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.dnn.dnnValue), f_DomainName_Encode(pc_APN_ID_IMS))) { v_Received_IMS := true; //Received UL NAS Transport/PDU Session Establishment REQ for IMS v_ReceivedAsp_IMS := v_ReceivedAsp; }; } if (ispresent(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.dnn.dnnValue)) { if (match (valueof(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.dnn.dnnValue), f_DomainName_Encode(pc_APN_ID_Internet))) { v_Received_Internet := true; //Received UL NAS Transport/PDU Session Establishment REQ for Internet v_ReceivedAsp_Internet := v_ReceivedAsp; }; } if (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue == '011'B) { v_Received_Emergency := true; //Received UL NAS Transport/PDU Session Establishment REQ for Emergency v_ReceivedAsp_Emergency := v_ReceivedAsp; }; } } } else { //Expect a UL NAS Transport/PDU Session Establishment REQ for Emergency call if Registration REQ is for Emergency SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp_Emergency; v_Received_Emergency := true; } //WA#6_3_1_7 : Change Stop //@siclog "Step 18-22" siclog@//@sic R5-216165 sic@ //Steps 9 to 13 of the procedure of "IMS Emergency call establishment in 5GC with IMS emergency registration" described in TS 38.508-1 [4] Table 4.9.11.2.2-1 are performed. //WA#6_3_1_7 COMMENTED OUT f_NR5GC_EmergencyCallPDUSession(nr_Cell11,tsc_NR_RbId_SRB1);//@sic R5s211479 sic@ //WA#6_3_1_7 : Change Start f_NR5GC_PDUSessionEstablishment_InnerLoop(nr_Cell11, v_ReceivedAsp_Emergency, SRB1, omit, -, -,crs_SSC_Mode(-, '001'B), omit, f_BuildDefaultQoSRules("2"), -, -, -,? ,- ,- ,crs_SSC_Mode('A'H, '001'B), - ,- , ? ,- ,- ,- ,- , cr_NG_Request_Type('011'B),- ,- ,- ,- ,- ,false ,"2"); v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Add Voice Bearer"); f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell11, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Voice Bearer Added")); //WA#6_3_1_7 : Change Stop f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Emergency Call Established"); // Emergency call now active @sic R%s220982, R5-225362 sic@ //@siclog "Step 23" siclog@//@sic R5-216165 sic@ //Generic procedure of "Test Procedure for IMS MO Emergency call release" described in TS 38.508-1 [4] Table 4.9.12A.2.2-1 are performed. f_NR5GC_EmergencyCallRelease(nr_Cell11);//@sic R5s211479 sic@ // The above function won't finish until IMS messages have been sent and PDU session has been released @sic R5s220982 sic@ //@siclog "Step 24" siclog@//@sic R5-216165 sic@ //The SS releases the RRC connection. v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell11)); //@sic R5-231404 sic@ f_NR_RRCRelease(nr_Cell11); deactivate(v_MyDefaultVar); //@sic R5-231404 sic@ //WA#R&S change start to implement the Step 17Aa1 if (v_TypeOfRegistration == Initial_Secure) { v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell11, ?); f_NR_RRC_ActivateSecurity (nr_Cell11, v_ReceivedMsg.SecurityProtection.NasCount); SRB.send (cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg()))); f_NR5GC_PDUSessionEstablishment (nr_Cell11, -, tsc_NR_RbId_SRB1); f_NR_RRCRelease(nr_Cell11); } //WA#R&S Change stop return v_Received_IMS; //WA#6_3_1_7 }//End of fl_TC_6_3_1_7_TestBody

## Change 2

| Function name | f_TC_6_3_1_7_NR5GC |
| --- | --- |
| Reason for change | If the UE did not perform IMS registration (Step 17Aa1), then at end of the TC it will be in state STATE_EMERGENCY_RELEASE_IDLE. But if this UE performed IMS registration , then at end of the TC the state will be STATE_IDLE_1A. At present the TTCN only considers the possibility of STATE_EMERGENCY_RELEASE_IDLE. This also needs to be addressed. |
| Summary of change | Based on the test flow, the function fl_TC_6_3_1_7_TestBody returns a Boolean that is set to TRUE if the UE performed IMS registration. Based on this within the function f_TC_6_3_1_7_NR5GC postamble functions are called. |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_3_1_7_NR5GC() runs on NR5GC_PTC { //Steering of UE in roaming during registration/security check unsuccessful but emergency service pending to be activated //@sic R5-204410 sic@ Cell change f_NR5GC_Init(NR_1);//@sic R5s201152 sic@ need to update to the default configuration //@sic R5-217805 sic@ f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//@sic R5-204410 sic@ //Create and configure cell f_NR_CellConfig_Def(nr_Cell11); // Equip UE with the USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1"); f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); //UE's Last Registered PLMN (RPLMN) needs to be cleared by using Registration Reject procedure as defined in TS 38.508-1 [4] Subclause 4.9.8 //@sic R5-204410 sic@ f_NR5GC_Preamble(nr_Cell11,STATE_OFF_0A); f_NR_TestBody_Set(true); fl_TC_6_3_1_7_TestBody(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell11, STATE_EMERGENCY_RELEASE_IDLE);//@sic R5s211479 sic@ }

After Change:

function f_TC_6_3_1_7_NR5GC() runs on NR5GC_PTC { //Steering of UE in roaming during registration/security check unsuccessful but emergency service pending to be activated var boolean v_IMS_Registered := false; //WA#6_3_1_7 //@sic R5-204410 sic@ Cell change f_NR5GC_Init(NR_1);//@sic R5s201152 sic@ need to update to the default configuration //@sic R5-217805 sic@ f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//@sic R5-204410 sic@ //Create and configure cell f_NR_CellConfig_Def(nr_Cell11); // Equip UE with the USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1"); f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); //UE's Last Registered PLMN (RPLMN) needs to be cleared by using Registration Reject procedure as defined in TS 38.508-1 [4] Subclause 4.9.8 //@sic R5-204410 sic@ f_NR5GC_Preamble(nr_Cell11,STATE_OFF_0A); f_NR_TestBody_Set(true); v_IMS_Registered := fl_TC_6_3_1_7_TestBody(); //WA#6_3_1_7 f_NR_TestBody_Set(false); // Finish if (v_IMS_Registered) { f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); //WA#6_3_1_7 } else { f_NR_Postamble(nr_Cell11, STATE_EMERGENCY_RELEASE_IDLE);//@sic R5s211479 sic@ } }