3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230789

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3248 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding local confirmation of UT commands before starting a timer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are many occasions where an AT command is sent to the UE, but the UE is NOT expected to send any signalling for a period of time. 36.523-3 clause 5.2 notes that a confirmation shall be requested in cases when there the MMI/AT command doesn’t result in signalling from the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added an optional local confirmation, LOCAL_CNF_REQUIRED, which can then be requested if the next TTCN operation is to start a timer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non conformant UEs may pass if timers are started too early in the TTCN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | UpperTesterFunctions, 9.1.10.3, 9.1.10.4, 9.1.10.6, 9.1.12.1, 9.1.12.2, 9.1.12.3, 9.1.12.4, 9.1.12.5, 10.1.3.2, 10.1.8.1, 10.1.8.2, 10.1.8.4, 11.3.1, 11.3.2, 11.3.5, 11.3.7, 11.3.9, 11.4.2, 11.4.5, 11.4.6, 11.4.9, 11.5.3, 11.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## Change 1

| Function name | function f_UT_RequestPDUSession |
| --- | --- |
| Reason for change | If a timer is started immediately after sending an AT/MMI command, it may be started before the AT/MMI command has been sent |
| Summary of change | Add an optional parameter to wait for a local confirmation if the function is called before starting a timer. Also implemented in: f_UT_IMS_TriggerSMS f_UT_RequestIMSEmergencyCall f_UT_InitiateECall |
| TTCN module | UpperTesterFunctions.ttcn |

function f_UT_RequestPDUSession(UT_PTC_MTC_PORT p_Port,

integer p_SessionId,

charstring p_DNN := "", // @sic R5s100466 sic@

boolean p_EmergencyBearer := false,  // @sic R5-120403 sic@

integer p_SSCMode := 1,

charstring p_S_NSSAI_ForATCmd := "",

boolean p_Access3GPP := true,

template (omit) boolean p_RQos := omit,

template (omit) boolean p_MH6 := omit,

template (omit) boolean p_AlwaysOn := omit,

integer p_NSLPI := -1,

charstring p_SpecialType := "",

Ut_CnfReq_Type p_CnfRequired := CNF_REQUIRED)

{

var charstring v_Cmd := fl_BuildPDNRequest(p_DNN, p_EmergencyBearer);

var charstring v_CommasToReqType := ",,,,,";

var charstring v_CommasToNSLPI := ",,,";

var charstring v_CommasToSSC := ",,,,,,";

if (not p_EmergencyBearer) { // add commas to Request Type

v_Cmd := v_Cmd & v_CommasToReqType;

}

// Then add commas to NSLPI

v_Cmd := v_Cmd & v_CommasToNSLPI;

if (p_NSLPI >= 0) {

v_Cmd := v_Cmd & int2str(p_NSLPI);

}

// Then add commas to SSC

v_Cmd := v_Cmd & v_CommasToSSC;

v_Cmd := v_Cmd & int2str(p_SSCMode-1) & ",";

v_Cmd := v_Cmd & """" & p_S_NSSAI_ForATCmd & """,";  //@sic R5s200501, R5s200963 sic@

if (p_Access3GPP) {

v_Cmd := v_Cmd & "0";

} else {

v_Cmd := v_Cmd & "1";

}

if (isvalue(p_RQos)) {

if(valueof(p_RQos)) {

v_Cmd := v_Cmd & ",1";

} else {

v_Cmd := v_Cmd & ",0";

}

}

if (isvalue(p_MH6)) {

if (not isvalue(p_RQos)) { // if previous field not included, need to add another comma

v_Cmd := v_Cmd & ",";

}

if(valueof(p_MH6)) {

v_Cmd := v_Cmd & ",1";

} else {

v_Cmd := v_Cmd & ",0";

}

}

if (isvalue(p_AlwaysOn)) {

if (not isvalue(p_RQos)) { // if previous field not included, need to add another comma

v_Cmd := v_Cmd & ",";

}

if (not isvalue(p_MH6)) { // if previous field not included, need to add another comma

v_Cmd := v_Cmd & ",";

}

if(valueof(p_AlwaysOn)) {

v_Cmd := v_Cmd & ",1";

} else {

v_Cmd := v_Cmd & ",0";

}

}

// Add comma if field is there or not

v_Cmd := v_Cmd & ",";

fl_RequestPDPContext(p_Port, p_SessionId, v_Cmd, p_SpecialType);  // @sic R5-227473 sic@

fl_UT_ApplyCommand(p_Port, cas_UT_Req(ACTIVATE_BEARER, p_CnfRequired, cs_PDP_Cid(int2str(p_SessionId))));

}

## Change 2

| Function name | functions fl_TC_10_1_3_2_Body (4 places) fl_TC_10_1_8_1_Body (3 places) fl_TC_10_1_8_2_Body (1 place) fl_TC_10_1_8_4_Body (1 place) fl_TC_10_1_8_5_Body (1 place) |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_RequestPDUSession is followed by a timer starting |
| TTCN module | GSM_NR5GC.ttcn |

function fl_TC_10_1_3_2_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var O1_Type v_PDUSessionIdToRelease; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (omit) DNN v_DNN; var integer v_InternetPduSessionID; var NR_SRB_COMMON_IND v_ReceivedAsp; var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB); var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; // @sic R5-206355 sic@ timer t_T3396 := 120.0; // @sic R5-201629 sic@ timer t_T3584 := 120.0; // @sic R5-201629 sic@ timer t_Wait := 5.0; v_InternetPduSessionID := f_NR5GC_MobileInfo_GetNoOfSessions()+1; //@siclog "Steps 1-2" siclog@ //AT command to trigger a PDU session WITHOUT DNN (APN) f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_InternetPduSessionID, -, omit); // @sic R5s210148 sic@ v_PDUSessionIdToRelease := f_NR_GetSessionId_ForLastPDUSession(); // @sic R5s210417 sic@ //@siclog "Steps 3-4" siclog@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00010'B)); //T3396 = 2 minutes // @sic R5-201629 sic@ // @sic R5-225052 sic@ //@siclog "Steps 5" siclog@ f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI , -, -, -, -, -, -, LOCAL_CNF_REQUIRED); //AT command to trigger a PDU session WITHOUT DNN (APN) //@siclog "Steps 6" siclog@ t_T3396.start; // @sic R5s200585 sic@ alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) { t_T3396.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); } []t_T3396.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } } //@siclog "Steps 7-9" siclog@ //AT command to trigger a PDU session WITHOUT DNN (APN) f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_InternetPduSessionID, -, omit); // @sic R5s200585, R5s201659, R5s210148 sic@ v_PDUSessionIdToRelease := f_NR_GetSessionId_ForLastPDUSession(); // @sic R5s210417 sic@ //@siclog "Steps 10-11" siclog@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00000'B)); //T3396 = 0 minutes // @sic R5-201629 sic@ //@siclog "Steps 12-14" siclog@ //AT command to trigger a PDU session WITHOUT DNN (APN) f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_InternetPduSessionID, -, omit); // @sic R5s200585, R5s201659, R5s210148 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Steps 15-16" siclog@ v_PDUSessionIdToRelease := f_NR_GetSessionId_ForLastPDUSession(); // @sic R5s210585, R5s210417 sic@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources cs_GPRS_Timer2_3_IEI('37'O, '111'B, '00000'B)); //T3396 = Deactivated //@siclog "Steps 17" siclog@ f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI , -, -, -, -, -, -, LOCAL_CNF_REQUIRED); //AT command to trigger a PDU session WITHOUT DNN (APN) //@siclog "Steps 18" siclog@ t_Wait.start; alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18"); } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); } } //@siclog "Step 19" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_CONNECTED_3A); //@siclog "Step 20" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //The UE is Switched ON @sic R5s211299 sic@ //@siclog "Step 21" siclog@ //Steps from Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA, -, noRrcConnectionRelease); // @sic R5s200585 sic@ //@siclog "Steps 22-24" siclog@ //AT command to trigger a PDU session WITHOUT DNN (APN) f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_InternetPduSessionID, -, omit); // @sic R5s200585, R5s201659, R5s210148 sic@ v_InternetPduSessionID := v_InternetPduSessionID +1; //@siclog "Steps 25-26" siclog@ //AT command to trigger a PDU session with S-NSSAI and DNN // @sic R5s201659 sic@ f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_InternetPduSessionID, -, tsc_APN1_String, cs_S_NSSAI_SST1eMBB, ?); // @sic R5s200585, R5s201659, R5s210148 sic@ v_PDUSessionIdToRelease := f_NR_GetSessionId_ForLastPDUSession(); // @sic R5s210417 sic@ //Check that DNN is present in the PDU session established at Step 26 v_DNN := f_Get_DNNForPDUSessionId(f_NR5GC_MobileInfo_GetSessionInfoList(), v_PDUSessionIdToRelease); //Store it for further use if (not isvalue(v_DNN)){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DNN being sent in Step 26"); } //@siclog "Steps 27-28" siclog@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '01000011'B), //5GSM cause Insufficient resources for specific slice and DNN cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00010'B)); //T3584 = 2 minutes // @sic R5-201629 sic@ // @sic R5-225052 sic@ //@siclog "Steps 29" siclog@ f_UT_RequestPDUSession(UT, v_InternetPduSessionID, tsc_APN1_String, -, -, v_NSSAI , -, -, -, -, -, -, LOCAL_CNF_REQUIRED); //AT command to trigger a PDU session with S-NSSAI and DNN // @sic R5s201659 sic@ //@siclog "Steps 30" siclog@ t_T3584.start; // @sic R5s200585 sic@ alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) { t_T3584.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 30"); } []t_T3584.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 30"); } } //@siclog "Steps 31-33" siclog@ //AT command to trigger a PDU session with S-NSSAI and DNN // @sic R5s201659 sic@ f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_InternetPduSessionID, -, tsc_APN1_String, cs_S_NSSAI_SST1eMBB, cs_S_NSSAI_SST1eMBB_WithIEI); // @sic R5s210148 sic@ v_PDUSessionIdToRelease := f_NR_GetSessionId_ForLastPDUSession(); // @sic R5s210417 sic@ //@siclog "Steps 34-35" siclog@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '01000011'B), //5GSM cause Insufficient resources for specific slice and DNN cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00000'B)); //T3584 = 0 minutes //@siclog "Steps 36-38" siclog@ //AT command to trigger a PDU session with S-NSSAI and DNN // @sic R5s201659 sic@ f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_InternetPduSessionID, -, tsc_APN1_String, cs_S_NSSAI_SST1eMBB, cs_S_NSSAI_SST1eMBB_WithIEI); // @sic R5s200585, R5s201659, R5s210148 sic@ v_PDUSessionIdToRelease := f_NR_GetSessionId_ForLastPDUSession(); // @sic R5s210417 sic@ //@siclog "Steps 39-40" siclog@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '01000011'B), //5GSM cause Insufficient resources for specific slice and DNN cs_GPRS_Timer2_3_IEI('37'O, '111'B, '00000'B)); //T3584 = Deactivated //@siclog "Steps 41" siclog@ f_UT_RequestPDUSession(UT, v_InternetPduSessionID, tsc_APN1_String, -, -, v_NSSAI , -, -, -, -, -, -, LOCAL_CNF_REQUIRED); //AT command to trigger a PDU session with S-NSSAI and DNN // @sic R5s201659 sic@ //@siclog "Steps 42" siclog@ t_Wait.start; alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 42"); } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 42"); } } //@siclog "Steps 43" siclog@ //Build the PDU Session Release v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100100'B), omit); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @sic R5-206355 sic@ //@siclog "Steps 44" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5-206355 sic@ if (not f_Check_NG_GSMStatus (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease, ?, cr_GMM_GSM_Cause(omit, tsc_NR5GCCause_InvalidPDUSessionIdentity))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 44"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 44"); //@siclog "Step 45" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_CONNECTED_3A); //@siclog "Step 46" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s211299 sic@ //@siclog "Step 47" siclog@ //Steps from Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA, -, noRrcConnectionRelease); // @sic R5s200585 sic@ //@siclog "Steps 48-50" siclog@ //AT command to trigger a PDU session with S-NSSAI and DNN // @sic R5s201659 sic@ f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_InternetPduSessionID, -, tsc_APN1_String, cs_S_NSSAI_SST1eMBB, cs_S_NSSAI_SST1eMBB_WithIEI); // @sic R5s210148 sic@ } //End of test body of TC_10_1_3_2

## Change 3

| Function name | Functions fl_TC_11_3_9_TestBody (1 place) fl_TC_11_3_5_TestBody (2 places) |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_RequestPDUSession is followed by a timer starting |
| TTCN module | UAC_NR5GC.ttcn |

function fl_TC_11_3_9_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_RadioBearerId_Type v_SRB:= tsc_NR_RbId_SRB2; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (omit) DNN v_DNN; var boolean v_ReceiveRRCSetupRequest; var default v_MyDefaultVar:= null; // @sic R5-224717 sic@ timer t_Wait := 11.2; // @sic R5-213507 sic@ timer t_WaitTime := 5.0; // @sic R5-214783 sic@ v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_DNNValue)); // @sic R5s210307 sic@ //@siclog "Step 3" siclog@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue, -, -, -, -, -, -, -, -, -, -, LOCAL_CNF_REQUIRED); //@siclog "Step 4" siclog@ // Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is the same value in UL NAS TRANSPORT message in preamble within 20s? t_Wait.start; alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // @sic R5s210307 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } } // @siclog "Step 5" siclog@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.9.3.2-1/2. v_CellPowerList_T1 := { //@sic R5-205388 sic@ cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; f_NR_SetCellPowerList(v_CellPowerList_T1); if( f_GetTestcaseAttrib_NR_SNPN(testcasename())) //@sic R5-233351 sic@ { //@siclog "Step 4-22a1" siclog@ //Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR5GC_MobileInfo_Init(); // Need to 'reset' all stored lists as all DRBs and PDU sessions do not exist in the new SNPN f_NR5GC_RRC_IdleState1N_Def (nr_Cell12); } else { // @siclog "Step 6" siclog@ //The mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 12 and release RRC connection (Note 1) //Note 1: SS ignore the PDU SESSION ESTABLISHMENT REQUEST sent before SS release RRC connection. @sic R5-224717 sic@ v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(nr_Cell12)); f_NR5GC_MobilityRegistration(nr_Cell12, -, -, -, -, noRrcConnectionRelease); f_NR_RRCRelease(nr_Cell12); deactivate(v_MyDefaultVar); } // @sic R5-214783 sic@ // @siclog "Steps 6A - 6Bb1" siclog@ t_WaitTime.start; alt{ [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_MoData, ?))) { t_WaitTime.stop; f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221054 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Ba1"); } [] t_WaitTime.timeout { //@siclog "Step 6Bb2" siclog@ // Make the UE attempt to establish a PDU session for the DNN defined in Table 11.3.9.3.3-1. This can be done by an AT/MMI command. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Step 6Bb3" siclog@ //Check: Does the UE transmit a RRCSetupRequest message and establishmentCause is set to mo-Data? v_ReceiveRRCSetupRequest := f_NR_RRC_SetupRequest_Common( nr_Cell12, t_Wait, cr_NR_EstablishmentCause_MoData); if (v_ReceiveRRCSetupRequest) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Bb3"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6Bb3"); } } } //@siclog "Steps 9 - 14" siclog@ //Steps 3-8 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0000'B)) { // @sic R5s200654 R5s210307 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); //@siclog "Steps 15 - 16" siclog@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is set to the same value with the DNN value of Operator-defined access category definitions defined in Table 11.3.9.3.3-1 //The SS transmits PDU SESSION ESTABLISHMENT ACCEPT message. f_NR5GC_PDUSessionEstablishment (nr_Cell12, 1, -, -, - , v_DNN, -, v_DNN); }

## Change 4

| Function name | Functions fl_TC_9_1_10_3_TestBody (1 place) fl_TC_9_1_10_4_TestBody (1 place) fl_TC_9_1_10_6_TestBody (1 place) |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_RequestPDUSession is followed by a timer starting |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |

function fl_TC_9_1_10_3_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NG_NAS_UL_Message_Type v_GSMMsg; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_s; var ProcedureTransactionIdentifier v_PTI := '00'O; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; //@sic R5s220948 sic@ var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); var template (value) RejectedNSSAI v_RejectedNSSAI := { iei := '11'O, iel := '02'O, //@sic R5-224165 sic@ rejectS_NSSAI := {cs_RejectedS_NSSAI('0001'B, '0010'B, '01'O, omit)}}; //@sic R5-222008 R5-223429 R5-224165 sic@ var template (value) NSSAI v_AllowedNSSAI:= { // @sic R5s221179 Baseline Moving sic@ iei := '15'O, lvPart := { iel := '06'O, s_nssai := {cs_S_NSSAI('01'O, '02'O, omit), cs_S_NSSAI('01'O, '03'O, omit), cs_S_NSSAI('01'O, '04'O, omit)} //@sic R5-223429 sic@ }}; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_NAS_DL_Pdu_Type v_MToSend; var charstring v_AT_Response; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI('01'O, '01'O, omit)); var template (value) NSSAI v_ConfiguredNSSAI := { // @sic R5s221179 Baseline Moving sic@ iei := '31'O, lvPart := { iel := '02'O, s_nssai := {cs_S_NSSAI('01'O, '01'O, omit)} }}; var template (value) NSSAI v_PendingNSSAI := { // @sic R5s221179 Baseline Moving sic@ iei := '39'O, lvPart := { iel := '02'O, s_nssai := {cs_S_NSSAI('01'O, '01'O, omit)} }}; var template (value) GPRS_Timer3 v_T3512 := cs_GPRS_Timer2_3_IEI('5E'O, '101'B, '00001'B); //@siclog "Step 0A" siclog@ f_NR5GC_DeleteAllStoredNSSAI(v_NGC_NASCellA); // @sic R5-227438 sic@ //@siclog "Step 0B" siclog@ f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_CONNECTED_3A); // @sic R5-227438 sic@ //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 12" siclog@ //The SS transmits a REGISTRATION ACCEPT message including Rejected S-NSSAI=1 v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, //@sic R5-222008 sic@ v_RejectedNSSAI, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); v_SRB := tsc_NR_RbId_SRB2; //@sic R5s220948 sic@ } //@siclog "Step 14A" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@sic R5-232339 sic@ //@siclog "Step 15" siclog@ //Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15"); } //@siclog "Step 16" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=1 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI , -, -, -, -, -, -, -, LOCAL_CNF_REQUIRED); //@siclog "Step 17" siclog@ //Check: Does the UE transmit a RRCSetupRequest message within 5 seconds? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellA, 5.0); //@sic R5-232339 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); //@siclog "Step 18" siclog@ //Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed. f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_IDLE_1A, //@sic R5-232339 sic@ v_SRB); //@sic R5s220948 sic@ //@siclog "Step 19" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-222008 sic@ ...

## Change 5

| Function name | Functions fl_TC_9_1_12_1_TestBody (2 places) fl_TC_9_1_12_2_TestBody (1 place) fl_TC_9_1_12_3_TestBody (1 place) fl_TC_9_1_12_4_TestBody (1 place) fl_TC_9_1_12_5_TestBody (1 place) |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_RequestPDUSession is followed by a timer starting |
| TTCN module | NSAC_NR5GC.ttcn |

function fl_TC_9_1_12_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); var template (value) NSSAI v_AllowedNSSAI_Step12:= { iei := '15'O, lvPart := { iel := '02'O, s_nssai := {cs_S_NSSAI('01'O, '03'O, omit)} }}; var template (value) NSSAI v_ConfiguredNSSAI := { iei := '31'O, lvPart := { iel := '04'O, s_nssai := {cs_S_NSSAI('01'O, '01'O, omit), cs_S_NSSAI('01'O, '02'O, omit)} }}; var template (value) PartialXtdRejectedNSSAI v_PartialXtdRejectedNSSAI1 := f_BuildNR5GC_PartialXtdRejectedNSSAIType1({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, omit, omit, omit)}, '82'O); //@sic R5-233378 sic@ var template (value) PartialXtdRejectedNSSAI v_PartialXtdRejectedNSSAI2 := f_BuildNR5GC_PartialXtdRejectedNSSAIType1({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '02'O, omit, omit, omit)}, '21'O); //@sic R5-233378 sic@ var template (value) ExtdRejectedNSSAI v_ExtdRejectedNSSAI := f_BuildNR5GC_ExtdRejectedNSSAI({v_PartialXtdRejectedNSSAI1, v_PartialXtdRejectedNSSAI2}); var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var charstring v_AT_Response; var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI('01'O, '01'O, omit)); var charstring v_NSSAI2 := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI('01'O, '02'O, omit)); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); timer t_T3526 := 720.0; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ //Check: Does UE transmit a REGISTRATION REQUEST message including ER-NSSAI bit? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_ERNSSAI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 12" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI_Step12, -, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_ExtdRejectedNSSAI); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); t_T3526.start; //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, v_SRB); v_SRB := tsc_NR_RbId_SRB2; // SRB2 is established during PDU Session Establishment, hence further NAS signalling will happen on SRB2 } //@siclog "Step 15" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 16" siclog@ //Check: Are S-NSSAI=1 and S-NSSAI=2 in the Rejected NSSAI list with cause 'S-NSSAI not available due to maximum number of UEs reached' associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadXtdRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s230702 sic@ if (f_MatchXtdRejectedNSSAI_WithATCommand({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, omit, omit, omit), cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '02'O, omit, omit, omit)}, v_AT_Response)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 16"); } //@siclog "Step 17" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=1 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI , -, -, -, -, -, -, -, LOCAL_CNF_REQUIRED); //@siclog "Step 18" siclog@ //Check: Does the UE transmit an RRCSetupRequest message within T3526 associated with S-NSSAI=1 in the next 15 seconds? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellA, 15.0); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); //@siclog "Step 19" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=2 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI2 , -, -, -, -, -, -, -, LOCAL_CNF_REQUIRED); //@siclog "Step 20" siclog@ //Check: Does the UE transmit an RRCSetupRequest message within T3526 associated with S-NSSAI=2 in the next 15 seconds? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellA, 15.0); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); //@siclog "Step 21" siclog@ t_T3526.timeout; //@siclog "Step 22" siclog@ //Check: Using AT/MMI command (+C5GNSSAIRDP) verify that S-NSSAI=1 is not in the Rejected NSSAI list associated with current PLMN v_AT_Response := f_UT_ReadXtdRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s230702 sic@ if (f_MatchXtdRejectedNSSAI_WithATCommand({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '01'O, omit, omit, omit)}, v_AT_Response)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 22") } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); } //@sic R5-233378 sic@ //@siclog "Step 23" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_IDLE_1A); //@siclog "Step 24" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-231553 sic@ //@siclog "Step 25" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 26" siclog@ v_AT_Response := f_UT_ReadXtdRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); // @sic R5s230702 sic@ if (f_MatchXtdRejectedNSSAI_WithATCommand({cs_XtdRejectedS_NSSAI('0001'B, '0011'B, '02'O, omit, omit, omit)}, v_AT_Response)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 26") } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 26"); } //@sic R5-233378 sic@ //delete step 27-29 }//End of fl_TC_9_1_12_1_Body

## Change 6

| Function name | Function fl_TC_11_3_1_TestBody |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_IMS_TriggerSMS is followed by a timer starting |
| TTCN module | UAC_NR5GC.ttcn |

function fl_TC_11_3_1_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var float v_TimerValue := 21.0; //@sic R5-221459 sic@ timer t_WaitTime; //@sic R5s211658 sic@ f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, //@sic R5-221459 sic@ cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(4,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); // @sic R5-216192 sic@ //@siclog "Step 13" siclog@ //Make the UE attempt a MMTel voice call. f_UT_RequestIMSCall(UT); //@siclog "Step 14" siclog@ //Check: Does the UE transmit RRCSetupRequest message including mo-VoiceCall with within 21 s? t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mo_VoiceCall,?))) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } [] t_WaitTime.timeout { } } //@siclog "Step 15" siclog@ //SS changes SIB1 according to Table 11.3.1.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. (Note 1) f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(6,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS //@sic R5-221459 sic@ //@siclog "Step 15A" siclog@ t_WaitTime.start(20.0); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mo_VoiceCall,?))) { t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15Aa1"); f_NR_ULGrantConfiguration_Start(nr_Cell1); // @sic R5s220817 sic@ } [] t_WaitTime.timeout { //@siclog "Step 15Ab2 - 15Ab3" siclog@ //Make the UE attempt another MO MTSI MMTEL Voice session. //Check: Does the UE send NR RRCSetupRequest with EstablishmentCause set to 'mo-VoiceCall'? f_UT_RequestIMSCall(UT); f_NR_RRC_SetupRequest_Def(nr_Cell1, mo_VoiceCall); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15Ab3"); } } //@siclog "Steps 16-17" siclog@ // void //@siclog "Steps 18-28" siclog@ //Step 3 to step 13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place, //Send RRC Connection Setup on SRB0 (CCCH): done automatically by SS in Msg4 of the RA procedure // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)), '0001'B)) { // @sic R5s200654 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); // done on IMS PTC // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Add Voice Bearer"); // @sic R5-225362 sic@ f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell1, valueof(v_GSM_MobilityInfo)); if (not pc_Preconditions) { // @sic R5s220579 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Voice Bearer Added")); // @sic R5-225362 sic@ } f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Voice Call Established"); // @sic R5-225362 sic@ //@siclog "Steps 29-31" siclog@ //Step 1 to step 5 from test procedure for test procedure for IMS MO speech call release as described in TS 38.508-1 [4] Table 4.9.17.2.2-1 take place f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall); //@siclog "Step 31A" siclog@ //The SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell1); //@sic R5-216192 sic@ //@siclog "Step 32" siclog@ //AT command to make the UE attempt to send SMS over IP f_UT_IMS_TriggerSMS(UT, LOCAL_CNF_REQUIRED); //@siclog "Step 33" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with 'establishmentCause' set to 'mo-SMS'? // @sic R5-210356 sic@ t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_SMS))) //@sic R5-221459 sic@ { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33"); } [] t_WaitTime.timeout {} } //@siclog "Step 34" siclog@ //SS changes SIB1 according to Table 11.3.1.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. (Note 1) f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); //@sic R5-221459 sic@ //@siclog "Step 34A" siclog@ t_WaitTime.start(20.0); alt { //@siclog "Step 34Aa1" siclog@ //Check: Does the UE send NR RRCSetupRequest with EstablishmentCause set to 'mo-SMS'? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest( mo_SMS,?))) { t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 34Aa1"); f_NR_ULGrantConfiguration_Start(nr_Cell1); // @sic R5s220817 sic@ } [] t_WaitTime.timeout { //@siclog "Step 34Ab2" siclog@ //AT command to make the UE attempt to send another SMS over IP f_UT_IMS_TriggerSMS(UT); f_NR_RRC_SetupRequest_Def(nr_Cell1, mo_SMS); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 34Ab3"); } } //@siclog "Steps 36-43" siclog@ //Steps 3-16 from generic procedure IMS MO SMS in 5GS as described in TS 38.508-1 [4] Table 4.9.19.2.2-1 take place. // Send RRC Connection Setup on SRB0 (CCCH): done automatically by SS in Msg4 of the RA procedure // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)), '0001'B)) { // @sic R5s200654 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //@sic R5s211658 sic@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); }

## Change 7

| Function name | Functions fl_TC_11_3_2_TestBody (1 place) fl_TC_11_3_7_TestBody (1 place) |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_RequestIMSEmergencyCall is followed by a timer starting |
| TTCN module | UAC_NR5GC.ttcn |

function fl_TC_11_3_2_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var RejectWaitTime v_RejectWaitTime := 16; var float v_WaitForConnectionMin := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_RejectWaitTime)); var float v_TimerValue := 30.0; timer t_WaitTime; timer t_Wait:= 15.0; //@siclog "Step 1" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) f_UT_RequestIMSEmergencyCall (UT , -, LOCAL_CNF_REQUIRED); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' within 30 s'? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_TimerValue, cr_NR_EstablishmentCause_emergency, "Step 2"); //@sic R5-213505 sic@ //@siclog "Step 2A1" siclog@ //IF pc_SwitchOnOff THEN switch off UE, ELSE IF pc_USIM_Removal THEN remove the USIM, ELSE power off UE in which case steps 2A2 to 2A4 are not performed if(pc_SwitchOnOff){ f_UT_SwitchOffUE(UT, true); //@siclog "Step 2A2" siclog@ //SS starts timer1 = 5 sec to see if UE performs optional Step 10a3a1 (Note 5) t_WaitTime.start(15.0);//@sic R5-216191 sic@ //@siclog "Steps 2A3a1-2A3b1" siclog@ //@sic R5-217001 sic@ alt{ [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp { t_WaitTime.stop; f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), tsc_SHT_IntegrityProtected); f_NR5GC_OptionalIMSDeregisterOnSwitchOff(nr_Cell1, v_ReceivedMsg); f_NR_RRCRelease(nr_Cell1); } [] t_WaitTime.timeout{ //@sic R5-216191 sic@ } } } //@siclog "Step 2A5" siclog@ //ELSE power off UE else { f_UT_PowerOffUE (UT); } f_NR5GC_MobileInfo_Init(); //@sic R5s210757 sic@ //@siclog "Step 2A5" siclog@ //Switch on the UE f_NR5GC_SwitchOnAndResetIMS(); //@sic R5s210757 sic@ //@siclog "Step 2A6" siclog@ //The UE performs Registration procedure as specified in TS 38.508-1 [4] subclause 4.5.2. f_NR5GC_RRC_IdleState1N_Def(nr_Cell1); //@siclog "Step 3" siclog@ //SS changes SIB1 according to Table 11.3.2.3.3-2 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect and T390 expire. (Note 1) // Table 11.3.2.3.3-2: SIB1 for NR Cell 1 f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_7Entry, //@sic R5-205321 sic@ -, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 4" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 5" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'mt-access' ? f_NR_RRC_SetupRequest_Def(nr_Cell1, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //@siclog "Steps 6-12" siclog@ // Steps 3 to 9a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0010'B)) { // @sic R5s200654 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); f_NR_PreliminaryPass(__FILE__, __LINE__, "Check 5GC RRC_IDLE"); f_NR_RRCRelease(nr_Cell1); //@siclog "Step 13" siclog@ // SS changes SIB1 according to 38.508-1 [4] Table 4.6.1-28 and sends Short Message on PDCCH using P-RNTI. Wait for 2.1* modification period to allow the new system information to take effect. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Steps 14-15" siclog@ //Steps 1-2 of the NR RRC_CONNECTED procedure in table 4.5.4.2-3 in TS 38.508-1[4] are performed. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 16" siclog@ //The SS responds with RRCReject message with IE waitTime set to 10s. SRB.send(cas_NR_SRB0_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReject(v_RejectWaitTime))); t_WaitTime.start(v_WaitForConnectionMin); //@siclog "Step 17" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) //NOTE 1: This could be done by e.g. MMI or AT command. f_UT_RequestIMSEmergencyCall (UT); t_WaitTime.stop; //@sic R5s210804 sic@ //@sic R5-216191 sic@ //EXCEPTION : Steps 21a1 - 21b6 describe the UE behaviour that depends on UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported //@siclog "Steps 21a1 - 21a12" siclog@ //IF pc_NR_5GC_EmergencyServices THEN Step 2-13 for IMS Emergency call establishment are performed as specified in TS 38.508-1 [4], subclause 4.9.11 on NR Cell 1. if (pc_NR_5GC_EmergencyServices){ //@sic R5-221456 sic@ f_NR5GC_EmergencyCallWithRegistration(nr_Cell1); //@siclog "Step 21a13 - 21a14" siclog@ //Make the UE release the emergency call. //Test procedure for IMS MO Emergency call release is performed as specified in TS 38.508-1 [4], subclause 4.9.12A on NR Cell 1. f_NR5GC_EmergencyCallRelease(nr_Cell1); f_NR_RRCRelease(nr_Cell1); f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_2)); } else{ //@siclog "Steps 21b1- 21b4" siclog@ //ELSE Step 2- 5 for IMS Emergency call establishment are performed as specified in TS 38.508-1 [4], subclause 4.9.11 on NR Cell 1. f_NR_RRC_SetupRequest_Common(nr_Cell1, t_Wait, cr_NR_EstablishmentCause_emergency); f_NR_RRCSetup_Def(nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR_RRC_ActivateSecurity (nr_Cell1, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); //@siclog "Steps 21b5" siclog@ //IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM. if(pc_SwitchOnOff){ f_UT_SwitchOffUE(UT, true); //@sic R5s220418 sic@ //@siclog "Steps 21b6" siclog@ t_WaitTime.start(15.0); alt{ //@siclog "Steps 21b6a1" siclog@ [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp{ t_WaitTime.stop; if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } //@siclog "Steps 21b6a2" siclog@ f_NR_RRCRelease(nr_Cell1); } [] t_WaitTime.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration failed"); } } } else{ //@siclog "Steps 21b6b1" siclog@ f_UT_PowerOffUE (UT); } } }

## Change 8

| Function name | Function fl_TC_11_5_5_TestBody |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_InitiateECall is followed by a timer starting |
| TTCN module | ECall_NR5GC.ttcn |

function fl_TC_11_5_5_TestBody() runs on NR5GC_PTC { var float v_Wait:= 120.0; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ f_Delay(60.0); f_NR5GC_RRC_IdleState1N_Def(nr_Cell1); // @sic R5-227244 sic@ //@siclog "Step 3" siclog@ f_UT_InitiateECall(UT, MANUAL, -, LOCAL_CNF_REQUIRED); //@siclog "Step 4A-4R" siclog@ f_NR5GC_EmergencyCallLimitedService(nr_Cell1); // @sic R5-227244 sic@ //@siclog "Step 5" siclog@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); f_NR5GC_EmergencyCallReleaseMT(nr_Cell1); // @sic R5-227244 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); //@siclog "Step 7" siclog@ f_NR_RRCRelease(nr_Cell1); // @sic R5-227244 sic@ //@siclog "Step 8" siclog@ f_Delay(5.0); //@siclog "Step 9" siclog@ f_UT_InitiateECall(UT, TEST , -, LOCAL_CNF_REQUIRED); //@siclog "Step 10" siclog@ //Check: Does UE send an RRCConnectionRequest on NGC Cell A within 120 seconds? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_Wait, -, "Step 10: RRCSetupRequest received"); // @sic R5-227244 sic@ }

## Change 9

| Function name | Function fl_TC_11_5_3_TestBody |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_InitiateECall is followed by a timer starting |
| TTCN module | ECall_IRAT_NR5GC_NR.ttcn |

function fl_TC_11_5_3_NR5GC_TestBody() runs on NR_BASE_PTC { //@siclog "Step 1" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);//@sic R5-235303 sic@ f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); // @siclog "Step 2" siclog@ // The UE is Switched ON f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state. f_Delay(60.0); // @siclog "Step 4" siclog@ //An automatic eCall is initiated. (Note 1) f_UT_InitiateECall(UT, AUTOMATIC); //@siclog "Step 29" siclog@ //The UE is switched on. f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Change Power Level"); f_UT_SwitchOnUE(UT); //@siclog "Step 30" siclog@ //Wait for 60s. f_Delay(60.0); //@siclog "Step 31" siclog@ //An automatic eCall is initiated. (Note 1) f_UT_InitiateECall(UT, AUTOMATIC , -, LOCAL_CNF_REQUIRED); // @siclog "Step 32" siclog@ // Check: Does the UE transmit an RRCConnectionRequest message in the next 30 sec?// prose CR needed. f_NR_CheckNoRRCSetupReq (nr_Cell1, 30.0,-,"step 32 failed"); }

## Change 10

| Function name | Function fl_TC_11_3_1_TestBody |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_RequestIMSCall is followed by a timer starting |
| TTCN module | UAC_NR5GC.ttcn |

function fl_TC_11_3_1_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var float v_TimerValue := 21.0; //@sic R5-221459 sic@ timer t_WaitTime; //@sic R5s211658 sic@ f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, //@sic R5-221459 sic@ cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(4,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); // @sic R5-216192 sic@ //@siclog "Step 13" siclog@ //Make the UE attempt a MMTel voice call. f_UT_RequestIMSCall(UT , -, LOCAL_CNF_REQUIRED); //@siclog "Step 14" siclog@ //Check: Does the UE transmit RRCSetupRequest message including mo-VoiceCall with within 21 s? t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mo_VoiceCall,?))) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } [] t_WaitTime.timeout { } } //@siclog "Step 15" siclog@ ...

## Change 11

| Function name | Functions fl_TC_11_4_2_TestBody (1 place) fl_TC_11_4_5_NR5GC_TestBody (1 place) fl_TC_11_4_6_NR5GC_TestBody (1 place) fl_TC_11_4_9_NR5GC_TestBody (1 place) |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_RequestIMSCall is followed by a timer starting |
| TTCN module | Emergency_NR5GC.ttcn |

function fl_TC_11_4_2_NR5GC_TestBody() runs on NR5GC_PTC { //@siclog "Step 1" siclog@ //The SS configures:- "NR Cell 1 as "Serving cell". f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 2" siclog@ //Switch the UE on. f_UT_SwitchOnUE(UT); //@siclog "Step 3" siclog@ //Make the UE attempt an IMS emergency call dialling a number which is stored on the ME. f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 4" siclog@ //Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12? f_NR5GC_EmergencyCallLimitedService(nr_Cell1); fl_ReleaseCallWithOptionalDeregisterOrRelease(nr_Cell1, 6.0);// @sic R5-223439 R5-235301 sic@ //@siclog "Step 9" siclog@ //The SS releases RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 9A" siclog@ //@sic R5-216198 sic@ // Make the UE attempt an IMS none-emergency call. f_UT_RequestIMSCall(UT , -, LOCAL_CNF_REQUIRED); //@siclog "Step 10" siclog@ //@sic R5-216198 sic@ //Check: Does the UE transmit an RRCSetupRequest message in the next 30sec? f_NR_CheckNoRRCSetupReq (nr_Cell1, 10.0); }