3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230790

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4769 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding local confirmation of UT commands before starting a timer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are many occasions where an AT command is sent to the UE, but the UE is NOT expected to send any signalling for a period of time. 36.523-3 clause 5.2 notes that a confirmation shall be requested in cases when there the MMI/AT command doesn’t result in signalling from the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added an optional local confirmation, LOCAL_CNF_REQUIRED, which can then be requested if the next TTCN operation is to start a timer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non conformant UEs may pass if timers are started too early in the TTCN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.2.1b, 9.2.3.2.1b, 11.3.4, 11.3.6, 13.5.1, 13.5.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## Change 1

| Function name | Function f_TC_13_5_1_EUTRA |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_RequestIMSCall is followed by a timer starting |
| TTCN module | AccessControl.ttcn |

function f_TC_13_5_1_EUTRA() runs on EUTRA_PTC { // MTSI MO speech Call / SSAC / 0% access probability for MTSI MO speech call var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS sic@ */ f_EUTRA_CellConfig_Def(eutra_Cell1); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS moved to f_EUTRA_NAS_Init sic@ */ f_EUTRA_CellInfo_SetSIB2_SSAC_BarringForMMTEL_Voice (eutra_Cell1,cs_AC_BarringConfig(p00, s64, '11111'B)); f_EUTRA_SS_ConfigureSysinfo (eutra_Cell1); f_EUTRA_Preamble (eutra_Cell1, STATE2_IDLEUPDATE); f_EUTRA_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); // @siclog "Step 2" siclog@ // Check: Does the UE transmit a RRCConnectionRequest message within 10 s?. f_Delay(10.0); // @siclog "Step 3" siclog@ f_EUTRA_CellInfo_SetSIB2_SSAC_BarringForMMTEL_Voice (eutra_Cell1, omit); f_EUTRA_ModifySysinfo (eutra_Cell1); // @siclog "Step 4" siclog@ // Wait for 15 s (Note 1) to allow the new system information to take effect f_Delay(15.0); // @siclog "Step 5" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); // @siclog "Step 6 - 7" siclog@ // 34.229-1 TC 12.18: To verify that the UE does not send INVITE message within 10s because the timer Ty is running. f_Delay(59.0); // @siclog "Step 8 - 21" siclog@ // Steps 1 to 14 of the generic test procedure for MTSI MO Speech call establishment in EUTRA: Normal Service (TS 36.508 4.5A.6.3-1). f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); // @sic R5s0928 sic@ f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell1, cr_EstablishmentCause_MoData); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // Steps 6 to 18 - MTSI MO call for EPS // @siclog "Step 6 - 18" siclog@ v_NasInd := f_EUTRA_ServiceReq_WithoutRrcConnReq (eutra_Cell1, v_RRC_TI); f_EUTRA_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, v_RRC_TI, v_NasInd.SecurityProtection.NasCount); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, v_RRC_TI); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_TestBody_Set(false); f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED); }

## Change 2

| Function name | Function f_TC_13_5_1a_EUTRA |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_RequestIMSCall is followed by a timer starting |
| TTCN module | AccessControl_R12.ttcn |

function f_TC_13_5_1a_EUTRA() runs on EUTRA_PTC { // MTSI MO speech Call / SSAC in Connected mode / 0% access probability for MTSI MO speech call f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS sic@ */ f_EUTRA_CellInfo_SetSIB2_SSAC_BarringForMMTEL_Voice (eutra_Cell1,cs_AC_BarringConfig(p00, s64, '11111'B)); // @sic R5s160935 sic@ f_EUTRA_CellConfig_Def(eutra_Cell1); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS moved to f_EUTRA_NAS_Init sic@ */ f_EUTRA_Preamble(eutra_Cell1, STATE2_IDLEUPDATE); f_EUTRA_RbEst_Def (eutra_Cell1); // @sic R5-185126 sic@ f_EUTRA_TestBody_Set(true); // @siclog "Step 5" siclog@ // Make the UE attempt a MTSI MO Speech call f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); // @siclog "Step 6" siclog@ // Check: Does the UE transmit an INVITE message within 10 s? f_Delay(10.0); // @siclog "Step 6AA" siclog@ // @sic R5-162789 sic@ f_EUTRA_CellInfo_SetSIB2_SSAC_BarringForMMTEL_Voice (eutra_Cell1, omit); f_EUTRA_ModifySysinfo (eutra_Cell1,-,RRC_CONNECTED); // @sic R5s160609 sic@ // @siclog "Step 6AB" siclog@ // Wait for 15 s (Note 1) to allow the new system information to take effect f_Delay(15.0); // @sic R5-162789 sic@ // @siclog "Step 6A" siclog@ // Make the UE attempt a MTSI MO Speech call f_UT_RequestIMSCall(UT, px_IMS_CalleeUri, LOCAL_CNF_REQUIRED); // @sic R5-160920 sic@ // @siclog "Step 6B" siclog@ // Wait for 10 s (Note 1) to allow the new system information to take effect. f_Delay(10.0); // @sic R5-160920 sic@ // @siclog "Step 7" siclog@ // Check: Does the UE transmit an INVITE message with in 10s? f_Delay(49.0); // @sic R5-160920 sic@ @sic R5-162789 sic@ // @siclog "Step 8" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @siclog "Step 13 - 15" siclog@ // Steps 12 to 14 of the generic procedure for MTSI MO speech call establishment (TS 36.508 table 4.5A.6.3-1). f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @siclog "Step 23" siclog@ f_EUTRA_508Check_ConnectedState (eutra_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_TestBody_Set(false); f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED); }

## Change 3

| Function name | Function fl_EUTRA_9_2_1_2_1b_Body |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_RequestIMSCall is followed by a timer starting |
| TTCN module | NAS_Attach_UG.ttcn |

function fl_EUTRA_9_2_1_2_1b_Body() runs on EUTRA_PTC { ... //@siclog "Step 11b2" siclog@ //Check: Does the UE respond to paging on cell A with S-TMSI in GUTI-2 for PS domain? //(Generic Procedure TS36.508 6.4.2.4) f_EPS_508Check_NAS_Paging(eutra_CellA, ps); if (pc_VoLTE) { // @sic R5-122012 sic@ //@siclog "Step 11b3a1" siclog@ // An IMS call is initiated f_UT_RequestIMSCall (UT, -, LOCAL_CNF_REQUIRED); //@siclog "Step 11b3a2" siclog@ // Check: Does the UE send RRC Connection Request message? (F, 4) @sic R5-120644 sic@ v_TimerValue := f_EUTRA_SetTimerToleranceMin(eutra_CellA, nonProtocolTimer, 5.0); if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_CellA, v_TimerValue)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11b3a2"); } } // send trigger to other RAT as test now finished f_EUTRA_SendTriggerToOtherRAT (); // @sic R5-112638 sic@ } }

## Change 4

| Function name | Function fl_EUTRA_9_2_3_2_1b_Body |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_RequestIMSCall is followed by a timer starting |
| TTCN module | NAS_TrackingArea_UG.ttcn |

function fl_TC_9_2_3_2_1b_Body() runs on EUTRA_PTC { ... //@siclog "Step 6b2" siclog@ //Check: Does the UE respond to paging on cell B with S-TMSI in GUTI-2 for PS domain? //(Generic Procedure TS36.508 6.4.2.4) f_EPS_508Check_NAS_Paging(eutra_CellB, ps); if (pc_VoLTE) { // @sic R5-123651 sic@ //@siclog "Step 6b3a1" siclog@ // An IMS call is initiated f_UT_RequestIMSCall (UT, -, LOCAL_CNF_REQUIRED); //@siclog "Step 6b3a2" siclog@ // Check: Does the UE send RRC Connection Request message? (F, 3) if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_CellA, f_EUTRA_SetTimerToleranceMin( eutra_CellA, nonProtocolTimer, 5.0 ))) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6b3a2"); } } // Postamble f_EUTRA_TestBody_Set(false); f_EUTRA_Postamble(eutra_CellB, E1_IDLE, Combined); // @sic R5s110176 sic@ // send trigger to UTRAN/GERAN side since Test case is now finished on E-UTRA side f_EUTRA_SendTriggerToOtherRAT(); // @sic R5-112638 sic@ } }// end function fl_TC_9_2_3_2_1b_Body

## Change 5

| Function name | Function fl_TC_11_3_4_Body |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_InitiateECall is followed by a timer starting |
| TTCN module | IMS_ECall_UG.ttcn |

function fl_TC_11_3_4_Body() runs on EUTRA_PTC { if (pc_CS_Em_Call_in_UTRA and px_RATComb_Tested == EUTRA_UTRA) { f_EUTRA_SendCoOrdToOtherRAT("UTRAN", cms_IRAT_Trigger); f_EUTRA_WaitForTriggerFromOtherRAT ("UTRAN"); } else if(pc_CS_Em_Call_in_GERAN and px_RATComb_Tested == EUTRA_GERAN) { f_EUTRA_SendCoOrdToOtherRAT("GERAN", cms_IRAT_Trigger); f_EUTRA_WaitForTriggerFromOtherRAT("GERAN"); } //@siclog "Step 6" siclog@ f_UT_SwitchOnUE (UT, -, CNF_REQUIRED); //@siclog "Step 7" siclog@ f_Delay (15.0); //@siclog "Step 8" siclog@ f_UT_InitiateECall(UT, AUTOMATIC , -, LOCAL_CNF_REQUIRED); //@siclog "Step 9" siclog@ if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_CellA, 30.0, emergency)) { // @sic R5-230590 sic@ f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 9"); } }

## Change 6

| Function name | Function fl_TC_11_3_6_Body |
| --- | --- |
| Reason for change | A confirmation shall be requested in cases when there the MMI/AT command is followed by the starting of a timer |
| Summary of change | Requested a LOCAL_CNF when the function f_UT_InitiateECall is followed by a timer starting |
| TTCN module | IMS_ECall.ttcn |

function fl_TC_11_3_6_Body() runs on EUTRA_PTC { var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (); var NAS_MSG_Indication_Type v_NasInd; var NAS_AttDetValue_Type v_DetachType; var GutiParameters_Type v_Guti1Params := f_EUTRA_CellInfo_GetGuti(eutra_CellA); var float v_EcallDeregisteredInactive := 60.0; var boolean v_Detached := false; timer t_EcallNoRRCConnReq := 120.0; timer t_Watchdog := 5.0; //@siclog "Step 1" siclog@ // The UE is switched on. f_EUTRA_SwitchOnUEandStartIP(eutra_CellA, -, -, -, PDN_2, tsc_DRB2);// @sic R5s230115 sic@ //@siclog "Step 2" siclog@ // Wait for 60s for the UE to enter EMM-DEREGISTERED.eCALL-INACTIVE state f_Delay(v_EcallDeregisteredInactive); //@siclog "Step 3" siclog@ // An eCall to URI for test service is initiated. f_UT_InitiateECall(UT, TEST , -, LOCAL_CNF_REQUIRED); //@siclog "Step 4" siclog@ // Check: Does UE send an RRCConnectionRequest on Cell A within 120 seconds? t_EcallNoRRCConnReq.start; alt { [] t_EcallNoRRCConnReq.timeout {} // Continue the test [] SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2 - Unexpected RRCConnectionRequest"); } } //@siclog "Step 5" siclog@ // A manual eCall is initiated. f_UT_InitiateECall(UT, MANUAL); //@siclog "Step 6" siclog@ // Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? f_EUTRA_RRC_ConnectionRequest_Def(eutra_CellA, emergency); //@siclog "Step 7" siclog@ // SS transmits an RRCConnectionSetup message. f_EUTRA_RRC_ConnectionSetup_Def (eutra_CellA, -, tsc_RRC_TI_Def); //@siclog "Step 8" siclog@ // Check: Does the UE transmit an ATTACH REQUEST message to attach for emergency bearer services, together with a PDN CONNECTIVITY REQUEST message with 'Request type' set to 'emergency? v_NasInd := f_EUTRA_RRCConnectionSetupComplete_Def(eutra_CellA, cr_508_RRCConnectionSetupComplete(tsc_RRC_TI_Def, ?), cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected), cr_508_ATTACH_REQUEST(tsc_EAttachType_Emergency, v_AdditionalUpdateType), cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_Emergency, f_GetPdnType()))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8"); //@siclog "Step 9-18" siclog@ // Steps 5 to 14 of the generic test procedure for IMS Emergency call establishment in EUTRA: Limited Service (TS 36.508 subclause 4.5A.5.3-1). f_EUTRA_508_IMSEmergencyCallLimitedService_Steps7_16(eutra_CellA, v_NasInd); // @sic R5-227236 sic@ //EXCEPTION: In parallel to the events described in steps 19 to 23 below, the behaviour in table 11.3.6.2-2 occurs. //@siclog "Step 19-23" siclog@ // Steps 15 to 19 of the generic test procedure for IMS emergency call establishment in EUTRA: Limited Service (TS 36.508 subclause 4.5A.5.3-1). f_EUTRA_508_IMSEmergencyCallLimitedService_Steps17_19(eutra_CellA); // @sic R5-227236 sic@ //@siclog "Step 24-27" siclog@ // Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33 @sic R5-196019 sic@ //IMS call has been released. f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // The SS deactivates the EPS bearer. [This is the rest of C.33] f_EUTRA_DeactivateEPS_BearerContext (eutra_CellA, tsc_EpsDedicatedBearerId2); // there is only one PDN //EXCEPTION: Step 27Aa1 and 27Aa2 describes optional behaviour that depends on the UE implementation. v_DetachType := f_GetEDetachType(EPS_Only); t_Watchdog.start; alt { [] t_Watchdog.timeout {} // Continue the test [] //@siclog "Step 27Aa1" siclog@ // The UE transmits a DETACH REQUEST message with the Detach type IE indicating "EPS detach" to regain normal service. SRB.receive(car_SRB2_NasPdu_IND(eutra_CellA, //@sic R5-227236, @sic R5s230115 sic@ cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_508_DETACH_REQUEST_MO(?, f_GutiParameters2MobileIdentity(omit, v_Guti1Params), cr_DetachType( '0'B, v_DetachType))))) { t_Watchdog.stop; // @sic R5s230115 sic@ //@siclog "Step 27Aa2" siclog@ // The SS responds the DETACH ACCEPT message. SRB.send (cas_SRB2_NasPdu_REQ (eutra_CellA, //@sic R5-227236, @sic R5s230115 sic@ cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_508_DETACH_ACCEPT))); v_Detached := true; // @sic R5s230115 sic@ }; } //@siclog "Step 29" siclog@ // The SS releases the RRC connectionafter 5secs of Step 27.. f_EUTRA_RRC_ConnectionRelease(eutra_CellA); f_EUTRA_TestBody_Set(false); // @sic R5s230115 sic@ if( v_Detached == false) { f_EUTRA_Postamble(eutra_CellA, E1_IDLE, EPS_Only); } else { f_EUTRA_Postamble(eutra_CellA, E4_DEREGISTERED); } }